| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0424-v2 |
| --- | --- |

| Title: | Mandatory film grain |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Davood Saffar Martin Pettersson Mitra Damghanian | Email: | rickard.sjoberg@ericsson.com davood.saffar@ericsson.com martin.m.pettersson@ericsson.com mitra.damghanian@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes to add normative film grain generation to VVC, within conformance. When enabled, the generated film grain is proposed to be applied to the decoded picture before output, specified in Annex C, similar to how picture cropping is handled.

The film grain is proposed to be signalled using the APS and to be applied to decoded picture before output, similar to when pictures are cropped. The contribution proposes the following specification text elements:

- Add an sps_film_grain_enabled_flag and a film_grain_seed_len_minus1 syntax element to the SPS

- Add an APS type, FILM_GRAIN_APS, to hold film grain model syntax elements

- Add a pic_film_grain_present_flag, a pic_film_grain_aps_is and a film_grain_seed syntax element to the picture header, where the length of the film_grain_seed syntax element is equal to  film_grain_seed_len_minus1+1

- Derive the seed to use as equal to slice_pic_order_cnt_lsb << (film_grain_seed_len_minus1 + 1) + film_grain_seed

- Reduce the maximum DPB size by 1 when sps_film_grain_enabled_flag is equal to 1

- Add a film grain generation process in Annex C stating the film grain shall be applied when a picture is output if sps_film_grain_enabled_flag is equal to 1

It is proposed to use film grain syntax, semantics and processes similar to the design used in AV1. The proponents claim that this makes the implementation burden low for decoder implementations that support both VVC and AV1.

A presentation deck is attached with the v2 of this contribution.

# Introduction

Film grain generation is supported in AVC, HEVC and in the VVC JVET-P2007-v3.docx draft by SEI messages. The SEI message in VVC, “Film grain characteristics SEI”, carries a parametrized model for film grain synthesis in the decoder. In the Film Grain Technology – specification in SMPTE RDD 5 (2006), a seed derivation method is specified that derives a seed value to be used for the Film grain characteristics SEI process.

The AV1 video codec format supports film grain generation. The film grain is applied when a picture is output. The film grain model syntax elements are signaled last in the frame_header_obu() in a syntax table called film_grain_params().

# Problem statement

Basing the intended output video quality on an SEI message such as a film grain SEI message is problematic since decoders are not required to implement them. This means that an encoder that considers using the film grain SEI message to improve the video quality risks the consumer being provided with a decoded video sequence without film grain resulting in bad experience. History has shown that this has rendered the SEI method unusable.

# Proposal

Figure 1 – The current VVC SEI method vs the proposed method

Fig 1. illustrates the proposal. Instead of using the SEI method, we propose to make film grain generation normative and inside conformance. We propose that film grain, when enabled, is generated and applied to the decoded picture before output, specified in Annex C, similar to how picture cropping is handled.

The proposed solution consists of decoding film grain model syntax elements from the adaptation parameter set. A seed syntax element is decoded from the picture header. The length of the seed syntax element is configurable and decoded from a SPS length syntax element. The seed to use is derived from the signalled seed value and the value of slice_pic_order_cnt_lsb

Proposed changes to the syntax, semantics and process are shown below with added text on top of JVET-Q0041-v2_DraftText.docx shown in yellow and deleted text shown using yellow strikethrough text.

The proponents aim at providing complete proposed specification text in either a future revision of this proposal or as a separate input contribution.

  - film grain generation: A filtering process that is applied on a decoded picture and is controlled by parameters conveyed in an APS.

  - film grain APS: An APS that controls the film grain generation process.

#### 7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_sao_enabled_flag | u(1) |
| sps_alf_enabled_flag | u(1) |
| sps_film_grain_enable_flag | u(1) |
| if( sps_film_grain_enable_flag ) |  |
| film_grain_seed_len_minus1 | u(4) |
| ... |  |

#### 7.3.2.5 Adaptation parameter set syntax

| adaptation_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| adaptation_parameter_set_id | u(5) |
| aps_params_type | u(3) |
| if( aps_params_type = = ALF_APS ) |  |
| alf_data( ) |  |
| else if( aps_params_type = = LMCS_APS ) |  |
| lmcs_data( ) |  |
| else if( aps_params_type = = SCALING_APS ) |  |
| scaling_list_data( ) |  |
| else if( aps_params_type = = FILM_GRAIN_APS ) |  |
| film_grain_data( ) |  |
| aps_extension_flag | u(1) |
| if( aps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| aps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

#### 7.3.2.6 Picture header RBSP syntax

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( sps_scaling_list_enabled_flag ) { |  |
| pic_scaling_list_present_flag | u(1) |
| if( pic_scaling_list_present_flag ) |  |
| pic_scaling_list_aps_id | u(3) |
| } |  |
| if( sps_film_grain_enable_flag ) { |  |
| pic_film_grain_present_flag | u(1) |
| if( pic_film_grain_present_flag ) { |  |
| pic_film_grain_aps_id | u(3) |
| film_grain_seed | u(v) |
| } |  |
| } |  |
| if( picture_header_extension_present_flag ) { |  |
| ph_extension_length | ue(v) |
| for( i = 0; i < ph_extension_length; i++) |  |
| ph_extension_data_byte[ i ] | u(8) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

#### 7.3.2.19 Film grain data syntax

| film_grain_data( ) { | Descriptor |
| --- | --- |
| parameters |  |
| } |  |

#### 7.3.3.2 General constraint information syntax

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| ... |  |
| no_alf_constraint_flag | u(1) |
| no_film_grain_constraint_flag | u(1) |
| ... | u(1) |
| } |  |

#### 7.4.3.3 Sequence parameter set RBSP semantics

sps_film_grain_enable_flag equal to 1 specifies that film grain may be applied to pictures in the CLVS. sps_film_grain_enable_flag equal to 0 specifies that film grain shall not be applied to any picture in the CLVS.

film_grain_seed_len_minus1 plus 1 specifies the length, in bits, of the film_grain_seed syntax element.

#### 7.4.3.5 Adaptation parameter set semantics

Each APS RBSP shall be available to the decoding process prior to it being referenced, included in at least one AU with TemporalId less than or equal to the TemporalId of the coded slice NAL unit that refers it or provided through external means.

All APS NAL units with a particular value of adaptation_parameter_set_id and a particular value of aps_params_type within a PU, regardless of whether they are prefix or suffix APS NAL units, shall have the same content.

adaptation_parameter_set_id provides an identifier for the APS for reference by other syntax elements.

When aps_params_type is equal to ALF_APS, or SCALING_APS, or FILM_GRAIN_APS, the value of adaptation_parameter_set_id shall be in the range of 0 to 7, inclusive.

When aps_params_type is equal to LMCS_APS, the value of adaptation_parameter_set_id shall be in the range of 0 to 3, inclusive.

aps_params_type specifies the type of APS parameters carried in the APS as specified in Table 6.

Table 6 – APS parameters type codes and types of APS parameters

| aps_params_type | Name of aps_params_type | Type of APS parameters |
| --- | --- | --- |
| 0 | ALF_APS | ALF parameters |
| 1 | LMCS_APS | LMCS parameters |
| 2 | SCALING_APS | Scaling list parameters |
| 3 | FILM_GRAIN_APS | Film grain parameters |
| 34..7 | Reserved | Reserved |

#### 7.4.3.6 Picture header RBSP semantics

pic_film_grain_present_flag equal to 1 specifies that the film grain generation process as specified in clause C.3.5 shall be invoked, with the decoded picture associated with this picture header as input, when the decoded picture is output. pic_film_grain_present_flag equal to 0 specifies that the film grain generation process shall not be invoked for the decoded picture. When not present, the value of pic_film_grain_present_flag is inferred to be equal to 0.

pic_film_grain_aps_id specifies the adaptation_parameter_set_id of the film grain APS. The TemporalId of the APS NAL unit having aps_params_type equal to FILM_GRAIN_APS and adaptation_parameter_set_id equal to pic_film_grain_aps_id shall be less than or equal to the TemporalId of the picture associated with PH.

film_grain_seed specifies the seed value to use for the picture to generate film grain. The length of the film_grain_seed syntax element is film_grain_seed_len_minus1 + 1 bits.

#### 7.4.3.19 Film grain data semantics

parameters semantics

#### 7.4.4.2 General constraint information semantics

no_film_grain_constraint_flag equal to 1 specifies that sps_film_grain_enabled_flag shall be equal to 0.  no_film_grain_constraint_flag equal to 0 does not impose such a constraint.

#### 7.4.8.1 General slice header semantics

slice_pic_order_cnt_lsb specifies the picture order count modulo MaxPicOrderCntLsb for the current picture. The length of the slice_pic_order_cnt_lsb syntax element is log2_max_pic_order_cnt_lsb_minus4 + 4 bits. The value of the slice_pic_order_cnt_lsb shall be in the range of 0 to MaxPicOrderCntLsb − 1, inclusive.

When the current picture is a GDR picture, the variable RpPicOrderCntVal is derived as follows:

RpPicOrderCntVal = PicOrderCntVal + recovery_poc_cnt	(114)

When pic_film_grain_present_flag is equal to 1, the parameter FilmGrainSeed is derived as follows:

FilmGrainSeed = slice_pic_order_cnt_lsb << (film_grain_seed_len_minus1 + 1) + film_grain_seed	(X)

### A.4.2 Profile-specific level limits

...

When the specified level is not level 8.5, the value of max_dec_pic_buffering_minus1[ Htid ] + 1 shall be less than or equal to MaxDpbSize, which is derived as follows:

if( PicSizeInSamplesY  <=  ( MaxLumaPs  >>  2 ) )
	MaxDpbSize = Min( 4 * maxDpbPicBuf, 16 – sps_film_grain_enable_flag )
else if( PicSizeInSamplesY  <=  ( MaxLumaPs  >>  1 ) )
	MaxDpbSize = Min( 2 * maxDpbPicBuf, 16 – sps_film_grain_enable_flag)	(A.1)
else if( PicSizeInSamplesY  <=  ( ( 3 * MaxLumaPs )  >>  2 ) )
	MaxDpbSize = Min( ( 4 * maxDpbPicBuf ) / 3, 16 – sps_film_grain_enable_flag)
else
	MaxDpbSize = maxDpbPicBuf – sps_film_grain_enable_flag

where MaxLumaPs is specified in Table A.1, and maxDpbPicBuf is equal to 8.

### ...

### C.3.3 Picture output

The output of the current picture is specified as follows:

–	If PictureOutputFlag is equal to 1 and DpbOutputTime[ n ] is equal to CpbRemovalTime[ n ], the current picture is output.

–	Otherwise, if PictureOutputFlag is equal to 0, the current picture is not output, but will be stored in the DPB as specified in clause C.3.4.

–	Otherwise (PictureOutputFlag is equal to 1 and DpbOutputTime[ n ] is greater than CpbRemovalTime[ n ] ), the current picture is output later and will be stored in the DPB (as specified in clause C.3.4) and is output at time DpbOutputTime[ n ] unless indicated not to be output by NoOutputOfPriorPicsFlag equal to 1.

When output, the picture is cropped, using the conformance cropping window for the picture. If pic_film_grain_present_flag is equal to 1, film grain is applied to the picture as specified in clause C.3.5.

When picture n is a picture that is output and is not the last picture of the bitstream that is output, the value of the variable DpbOutputInterval[ n ] is derived as follows:

DpbOutputInterval[ n ] = DpbOutputTime[ nextPicInOutputOrder ] − DpbOutputTime[ n ]	qq(C.16)

where nextPicInOutputOrder is the picture that follows picture n in output order and has PictureOutputFlag equal to 1.

...

### C.3.5 Film grain generation process

This process is applied to a picture before cropping and output.

Inputs to this process is the reconstructed picture after any in-loop filter process, i.e., the array recPictureL and, when ChromaArrayType is not equal to 0, the arrays recPictureCb and recPictureCr.

Outputs of this process is the output picture with film grain applied, i.e., the array outPictureL and, when ChromaArrayType is not equal to 0, the arrays outPictureCb and outPictureCr.

[Ed note: Add process here]

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).