| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0431 |
| --- | --- |

| Title: | Interaction between Affine and SBT |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Le Léannec K. Naser T. Poirier P. Bordes | Tel: Email: | fabrice.leleannec@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposed to deactivate the Sub Block Transform (SBT) in case of a CU coded in Affine Merge mode. More precisely, SBT is disallowed in case of sub-block merge mode when the merge candidate used is different from SbTMVP.

The proposed mutual exclusion reportedly leads to an average BD rate change of +0.01% in JVET CTC, in random access and 0.00% in low delay configurations. Encoding time is 98% in random access.

A second proposed method 2 deactivates SBT for all affine modes, leading +0.04% BD rate increase both in RA and LDB.

It is proposed to adopt the proposed deactivation of SBT in case of Affine merge mode into next VTM version.

# Introduction

It has been observed that the sub-block transform (SBT) partitioning of a CU is rarely used in the case of a CU predicted with affine motion compensation.

Therefore, a significant amount of encoder side RD checks related to the usage of SBT in combination with affine mode, with a very low usage rate.

# Proposal

In a first proposed method 1, this contribution proposes to infer the cu_sbt_flag to zero when the affine merge mode is on. This leads to the modified coding_unit syntax table of JVET-P2001 as shown by the following Table 1.

As can be seen, the cu_sbt_flag is now signaled if the following conditions are fulfilled:

- The CU prediction mode is INTER

- SBT is allowed on the SPS level

- Combined Intra/Inter prediction is not used in the considered CU

- Triangle Partition Mode is not used in the considered CU

- The considered CU is in not affine merge mode or SbTMVP merge candidate is not used

On Table 1, the variable enabledSbTMVP is equal to 1 is SbTMVP is allowed in the considered slice.

Table 1: modified coding_unit syntax table to disable SBT in combination with affine merge. (Method 1)

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| if( CuPredMode[ chType ][ x0 ][ y0 ] != MODE_INTRA && !pred_mode_plt_flag && general_merge_flag[ x0 ][ y0 ] = = 0 ) |  |
| cu_cbf | ae(v) |
| if( cu_cbf ) { |  |
| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_sbt_enabled_flag && !ciip_flag[ x0 ][ y0 ] && !MergeTriangleFlag[ x0 ][ y0 ] && ( merge_subblock_flag[ x0 ][ y0 ] = =0 ¦¦ ( enabledSbTMVP && merge_subblock_idx[ x0 ][ y0 ] = = 0 ) ) && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) { |  |
| allowSbtVerH = cbWidth >= 8 |  |
| allowSbtVerQ = cbWidth >= 16 |  |
| allowSbtHorH = cbHeight >= 8 |  |
| allowSbtHorQ = cbHeight >= 16 |  |
| if( allowSbtVerH ¦ ¦ allowSbtHorH ) |  |
| cu_sbt_flag | ae(v) |
| if( cu_sbt_flag ) { |  |
| if( ( allowSbtVerH ¦ ¦ allowSbtHorH ) && ( allowSbtVerQ ¦ ¦ allowSbtHorQ ) ) |  |
| cu_sbt_quad_flag | ae(v) |
| if( ( cu_sbt_quad_flag && allowSbtVerQ && allowSbtHorQ ) ¦ ¦ ( !cu_sbt_quad_flag && allowSbtVerH && allowSbtHorH ) ) |  |
| cu_sbt_horizontal_flag | ae(v) |
| cu_sbt_pos_flag | ae(v) |
| } |  |
| } |  |
| } |  |

In a method 2, the SBT mode is inferred to 0 if the CU is coded in affine mode, i.e. eiter in affine merge or affine amvp mode. The spec change associated to method 2 is given in the following

Table 2: modified coding_unit syntax table to disable SBT in combination with any affine mode. (Method 2)

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| if( CuPredMode[ chType ][ x0 ][ y0 ] != MODE_INTRA && !pred_mode_plt_flag && general_merge_flag[ x0 ][ y0 ] = = 0 ) |  |
| cu_cbf | ae(v) |
| if( cu_cbf ) { |  |
| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_sbt_enabled_flag && !ciip_flag[ x0 ][ y0 ] && !MergeTriangleFlag[ x0 ][ y0 ] && ( merge_subblock_flag[ x0 ][ y0 ] = =0 ¦¦ ( enabledSbTMVP && merge_subblock_idx[ x0 ][ y0 ] = = 0 ) ) && inter_affine_flag[ x0 ][ y0 ]==0 && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) { |  |
| allowSbtVerH = cbWidth >= 8 |  |
| allowSbtVerQ = cbWidth >= 16 |  |
| allowSbtHorH = cbHeight >= 8 |  |
| allowSbtHorQ = cbHeight >= 16 |  |
| if( allowSbtVerH ¦ ¦ allowSbtHorH ) |  |
| cu_sbt_flag | ae(v) |
| if( cu_sbt_flag ) { |  |
| if( ( allowSbtVerH ¦ ¦ allowSbtHorH ) && ( allowSbtVerQ ¦ ¦ allowSbtHorQ ) ) |  |
| cu_sbt_quad_flag | ae(v) |
| if( ( cu_sbt_quad_flag && allowSbtVerQ && allowSbtHorQ ) ¦ ¦ ( !cu_sbt_quad_flag && allowSbtVerH && allowSbtHorH ) ) |  |
| cu_sbt_horizontal_flag | ae(v) |
| cu_sbt_pos_flag | ae(v) |
| } |  |
| } |  |
| } |  |

# Experimental Results

The proposed mutual exclusion between affine merge mode and SBT has been tested on top of the VTM-7.0 under the JVET SDR CTC. It provides the performances results depicted by the followingTable 3. As can be seen, the BD-rate impact due to the removed combination is minor: +0.04% loss in RA and LDB.

Table 3: results obtained with the proposed Affine Merge / SBT mutual exclusion (Method 1)

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | 0.00% | 100% | 102% |
| Class A2 | 0.00% | -0.01% | 0.00% | 99% | 102% |
| Class B | 0.01% | 0.02% | 0.07% | 99% | 100% |
| Class C | 0.02% | 0.00% | 0.03% | 96% | 94% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.01% | 0.03% | 98% | 99% |
| Class D | -0.01% | -0.09% | -0.11% | 95% | 91% |
| Class F | -0.02% | 0.00% | -0.05% | 103% | 103% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.29% | -0.40% | 100% | 101% |
| Class C | 0.00% | -0.30% | 0.03% | 101% | 100% |
| Class E | -0.06% | 0.22% | -0.32% | 99% | 100% |
| Overall | 0.00% | -0.17% | -0.24% | 100% | 100% |
| Class D | -0.03% | 0.37% | 0.15% | 100% | 100% |
| Class F | -0.05% | -0.11% | -0.07% | 100% | 92% |

Table 4: results obtained with the proposed Affine /SBT mutual exclusion (Method 2)

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.01% | -0.01% | 98% | 101% |
| Class A2 | 0.05% | 0.05% | 0.06% | 97% | 102% |
| Class B | 0.05% | 0.00% | 0.11% | 98% | 103% |
| Class C | 0.05% | -0.03% | -0.03% | 98% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.00% | 0.04% | 98% | 101% |
| Class D | 0.04% | -0.07% | 0.10% | 98% | 104% |
| Class F | 0.02% | -0.01% | -0.03% | 97% | 107% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.07% | -0.27% | 0.12% | 99% | 99% |
| Class C | 0.07% | -0.06% | 0.01% | 98% | 99% |
| Class E | -0.03% | 0.22% | -0.62% | 98% | 100% |
| Overall | 0.04% | -0.08% | -0.10% | 99% | 100% |
| Class D | 0.04% | 0.10% | -0.04% | 99% | 100% |
| Class F | 0.00% | -0.09% | 0.59% | 99% | 101% |

# Conclusion

The contribution proposes to normatively disallow the combination of SBT and Affine merge modes. The proposed modification to VTM-7.0 brings  0.01% BD rate increase in Random Access and 0.00% Low-Delay B configuration.

The proposed method 2 normatively deactivates SBT for both affine merge and affine amvp modes. It leads to +0.04% BD rate increase both in RA and LDB configurations.

It is proposed to adopt  method 1 into the next version 8 of the VTM.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).