| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0434-v2 |
| --- | --- |

| Title: | Pre-Sample Adaptive Offset Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wang-Q Lim, Christian Helmrich, Johannes Erfurt, Heiko Schwarz, Detlev Marpe, and Thomas Wiegand | Email: | wang.lim@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI, Einsteinufer 37, 10587 Berlin |  |  |

_____________________________

Changes since v1: renamed PRE-SAO to Pre-SAO

# Abstract

This contribution proposes a tool referred to as Pre-Sample Adaptive Offset Filter (Pre-SAO). Pre-SAO operates jointly with the deblocking filter (DBF) process to further improve the coding efficiency of the in-loop filter. The tool is controlled by information in the bit-stream, and this information includes both (a) offset coefficients for luma and each chroma component and (b) threshold values for its classification. The offset coefficients and threshold values are both signalled at the slice level.

The proposed approach was evaluated using the CTC and reportedly results in the following average BD-rate improvement relative to VTM 7.1:

AI:	−0.64% (Y), −0.81% (Cb), −0.71% (Cr) at 101% enc. and approximately 101% dec. time;
RA:	−0.43% (Y), −0.83% (Cb), −0.81% (Cr) at 100% enc. and approximately 101% dec. time;
LB:	−0.54% (Y), −0.47% (Cb), −0.42% (Cr) at 100% enc. and approximately 100% dec. time;
LP:	−0.41% (Y), −0.30% (Cb), −0.37% (Cr) at 101% enc. and approximately 99% dec. time.

In addition, results for an alternative design, signalling the offset coefficients and threshold values on the CTU level, are reported. For this version, which more closely resembles the operation of the existing SAO in-loop filter, the following average results are reported:

AI:	−0.49% (Y), −0.71% (Cb), −0.62% (Cr) at 100% enc. and approximately 103% dec. time;
RA:	−0.xx% (Y), −0.xx% (Cb), −0.xx% (Cr) at 10x% enc. and approximately 10x% dec. time;
LB:	−0.xx% (Y), −0.xx% (Cb), −0.xx% (Cr) at 10x% enc. and approximately 10x% dec. time;
LP:	−0.xx% (Y), −0.xx% (Cb), −0.xx% (Cr) at 10x% enc. and approximately 10x% dec. time.

# Proposal

The contribution proposes a tool called Pre-Sample Adaptive Offset (Pre-SAO). Pre-SAO operates by applying two SAO-like filtering operations called SAOV and SAOH and they are applied jointly with the deblocking filter (DBF) before applying the existing (legacy) SAO, as illustrated in Fig. 1. The first SAO-like filter SAOV operates as applying SAO to the input picture Y2 after the deblocking filter for the vertical edges (DBFV) is applied – see Fig. 1.

Y3i=Clip1Y2`i + d1∙fi>T ?1 :0 - d2∙(fi<-T ?1 :0),

where T is a predetermined positive constant and d1 and d2 are offset coefficients associated with two classes based on the sample-wise difference between Y1(i) and Y2(i) given by

fi= Y1i-Y2(i).

The first class for d1 is given as taking all sample locations i such that fi>T while the second class for d2 is given by fi<-T. The offset coefficients d1 and d2 are calculated at the encoder so that the mean square error between output picture Y3 of SAOV and the original picture X is minimized, in the same way as in the existing SAO process. After SAOV is applied, the second SAO-like filter SAOH operates as applying SAO to Y4, after SAOV has been applied, with a classification based on the sample-wise difference between Y3(i) and Y4(i), the output picture of the deblocking filter for the horizontal edges (DBFH) – see Fig. 1. The same procedure as SAOV is applied for SAOH with  Y3i-Y4(i) instead of  Y1i-Y2i for its classification. The two offset coefficients, a predetermined threshold value T and an enabling flag for each of SAOH and SAOV are signalled at the slice level. SAOH and SAOV are applied for luma and the two chroma components independently.

Note that both SAOV and SAOH operate only on the picture samples affected by the respective deblocking (DBFV or DBFH). Hence, unlike with the existing SAO process, only a subset of all samples in the given spatial region (picture, or CTU in case of legacy SAO) are being processed by the Pre-SAO, which keeps the resulting increase in decoder-side mean operations per picture sample low (two or three comparisons and two additions per sample in the worst-case scenario according to preliminary estimates). It should also be noted that Pre-SAO only needs samples used by the deblocking filter without storing additional samples at the decoder.

Y1                                            Y2               Y3                                          Y4

DBFH DBFV SAOH SAOV

Fig. 1. Decoder block diagram for deblocking filter (DBF) combined with proposed SAO filterings SAOV and SAOH: The deblocking filter (DBF) consists of DBFV and DBH.

# Experimental Results

The proposed method was implemented on top of VTM 7.1 (with the additional merge request !1166 fixing decoder runtime issue #748) and evaluated for JVET’s SDR Common Test Conditions [1].  A summary of the results is provided below.  As can be seen, the tool provides coding efficiency improvements in all three channels for the All Intra (AI), Random Access (RA), Low Delay B (LB), and Low Delay P (LP) configurations. Furthermore, the gains range up to approximately 0.8% for AI classes A1 and B.  Detailed results can be found in the corresponding Excel file contained in the Zip file of this contribution.

Table 1. Results for slice-level Pre-SAO:

|  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM 7.1 (Git rev. 98ff0062) |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,78% | -0,85% | -0,38% | 103% | 101% |
| Class A2 | -0,64% | -0,82% | -0,92% | 100% | 100% |
| Class B | -0,79% | -0,90% | -0,90% | 101% | 102% |
| Class C | -0,47% | -0,89% | -0,94% | 101% | 104% |
| Class E | -0,47% | -0,47% | -0,22% | 100% | 98% |
| Overall | -0,64% | -0,81% | -0,71% | 101% | 101% |
| Class D | -0,42% | -0,72% | -0,69% | 100% | 101% |
| Class F | -0,34% | -0,87% | -1,02% | 103% | 100% |
|  |  | Random Access |  |  |  |
|  |  | Over VTM 7.1 (Git rev. 98ff0062) |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,33% | -0,65% | -0,38% | 99% | 103% |
| Class A2 | -0,56% | -0,48% | -0,63% | 101% | 97% |
| Class B | -0,51% | -0,99% | -1,00% | 99% | 101% |
| Class C | -0,33% | -1,01% | -1,02% | 101% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0,43% | -0,83% | -0,81% | 100% | 101% |
| Class D | -0,24% | -0,81% | -0,88% | 100% | 102% |
| Class F | -0,30% | -1,21% | -1,32% | 102% | 103% |
|  |  | Low Delay B |  |  |  |
|  |  | Over VTM 7.1 (Git rev. 98ff0062) |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.62% | -0.34% | -0.66% | 100% | 101% |
| Class C | -0,46% | -0,46% | -0,55% | 100% | 97% |
| Class E | -0,52% | -0,69% | 0,15% | 102% | 101% |
| Overall | -0.54% | -0.47% | -0.42% | 100% | 100% |
| Class D | -0,31% | -0,22% | 0,35% | 102% | 102% |
| Class F | -0.43% | -1.00% | -0.59% | 101% | 100% |
|  |  | Low Delay P |  |  |  |
|  |  | Over VTM 7.1 (Git rev. 98ff0062) |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.52% | -0.22% | -0.56% | 101% | 100% |
| Class C | -0,36% | -0,45% | -0,43% | 101% | 98% |
| Class E | -0,28% | -0,22% | 0,01% | 102% | 97% |
| Overall | -0.41% | -0.30% | -0.37% | 101% | 99% |
| Class D | -0,26% | -0,32% | -0,52% | 100% | 100% |
| Class F | -0,27% | -0,19% | -0,47% | 101% | 99% |

With (8Y+Cb+Cr) / 10 luma-chroma averaging, the following overall results are obtained:

All Intra: –0.66%, Random Access: –0.51%, Low Delay B: –0.52%, Low Delay P: –0.40%

# Additional CTU-wise Configuration

An alternative configuration of the Pre-SAO proposal, signalling the offset coefficients, threshold indices, and activation flags on the CTU instead of the slice level, was investigated.  This alternative configuration employs the same parametrization as the slice-level Pre-SAO (two 2-bit offset coefficients and one 1-bit threshold index for luma, two 2-bit offset coefficients shared by both chroma components in case of color video input and dual-tree operation) but signals the parameters individually for each coding tree block.  As with the slice-level Pre-SAO configuration described in Sec. 1, a slice-wise activation flag is provided, via slice_pre_sao_enabled_flag in slice_header( ), to allow disabling Pre-SAO on the picture level.

To reduce signalling cost, a CTU merge mode, governed by a 1-bit pre_sao_merge_flag element each for luma and chroma and the given Pre-SAO/deblocking direction (horizontal or vertical), is provided.  When this merge mode is enabled, each offset coefficient and (in case of luma) threshold index are derived from the respective parameters for the above, left, and above-left neighboring, if available, CTU using Martucci’s media adaptive prediction approach [3] of the form Clip3(min, max, above + left – above-left parameter). Furthermore, the pre_sao_enabled_flag for the given CTU, channel type (luma or chroma), and direction is derived from the merged offsets: pre_sao_enabled_flag = 1 if at least one offset is nonzero, or 0 otherwise.

The CTU-wise Pre-SAO parameters are CABAC coded, where all 1-bit elements (CTU-level threshold index, merge flag, and activation flag) feature their own individual context memory and the 2-bit offsets are coded with fixed, equal probability (EP).

Like the slice-level Pre-SAO proposal, the proposed CTU-level method was implemented on top of VTM 7.1 (with the additional merge request !1166 fixing decoder runtime issue #748) and evaluated for the SDR CTC [1].  A summary of the results is provided below, and detailed results can be found in the corresponding Excel file contained in the Zip file of this contribution.

Table 2. Results for CTU-level Pre-SAO:

|  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM 7.1 (Git rev. 98ff0062) |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,60% | -0,64% | -0,25% | 101% | 103% |
| Class A2 | -0,49% | -0,71% | -0,81% | 100% | 102% |
| Class B | -0,66% | -0,82% | -0,83% | 100% | 102% |
| Class C | -0,37% | -0,85% | -0,88% | 99% | 103% |
| Class E | -0,27% | -0,42% | -0,13% | 98% | 105% |
| Overall | -0,49% | -0,71% | -0,62% | 100% | 103% |
| Class D | -0,34% | -0,63% | -0,60% | 99% | 103% |
| Class F | -0,20% | -0,75% | -0,91% | 101% | 103% |
|  |  | Random Access |  |  |  |
|  |  | Over VTM 7.1 (Git rev. 98ff0062) |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,30% | -0,75% | -0,83% | 99% | 99% |
| Class C | -0,16% | -0,99% | -0,98% | 99% | 102% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0,19% | -0,82% | -0,90% | 100% | 102% |
| Class F | -0,11% | -1,05% | -1,15% | 100% | 99% |
|  |  | Low Delay B |  |  |  |
|  |  | Over VTM 7.1 (Git rev. 98ff0062) |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0,17% | -0,32% | -0,23% | 96% | 94% |
| Class E | 0,04% | 0,10% | 0,56% | 101% | 99% |
| Overall |  |  |  |  |  |
| Class D | -0,14% | -0,71% | 0,53% | 100% | 97% |
| Class F |  |  |  |  |  |
|  |  | Low Delay P |  |  |  |
|  |  | Over VTM 7.1 (Git rev. 98ff0062) |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0,12% | -0,12% | -0,20% | 102% | 98% |
| Class E | 0,09% | 0,10% | 0,87% | 101% | 95% |
| Overall |  |  |  |  |  |
| Class D | -0,11% | -0,01% | -0,27% | 99% | 96% |
| Class F |  |  |  |  |  |

With (8Y+Cb+Cr) / 10 luma-chroma averaging, the following overall results are obtained:

All Intra: –0.53%, Random Access: –0.xx%, Low Delay B: –0.xx%, Low Delay P: –0.xx%

# Conclusion

This contribution proposes a tool called Pre-Sample Adaptive Offset (Pre-SAO), operating jointly with the deblocking filter to further refine the luma and chroma channels.  Results show averaged luma-chroma gains of approximately 0.7%, 0.5% and 0.5% for All Intra, Random Access, and Low Delay B, respectively. An alternative configuration of the proposal, operating on the CTU level in terms of parameter signalling, reportedly results in averaged luma-chroma gains of approximately 0.5% (AI), 0.x% (RA), and 0.x% (LB).

# Appendix: Syntax and Semantics Specification

Please note that complete draft specification text will be provided in a future revision of this contribution.

## General Pre-SAO parameter set

| pre_sao( rx, ry ) { | Descriptor |
| --- | --- |
| hasMrgFlag = ( ( rx > 0 && rx != CtbToTileColBd[ rx ] ) ¦ ¦ |  |
| ( ry > 0 && ry != CtbToTileRowBd[ ry ] && !FirstCtbRowInSlice ) |  |
| for( cIdx = 0; cIdx < ( ChromaArrayType != 0 ? 2 : 1 ); cIdx++ ) { |  |
| for( dir = 0; dir < 2; dir++ ) { |  |
| if( hasMrgFlag && ( cIdx = = 0 ¦ ¦ qtbtt_dual_tree_intra_flag ) ) |  |
| pre_sao_merge_flag[ cIdx ][ dir ] | ae(v) |
| else |  |
| pre_sao_merge_flag[ cIdx ][ dir ] = 0 |  |
| if( pre_sao_merge_flag[ cIdx ][ dir ] = = 0 ) { |  |
| if( cIdx = = 0 ¦ ¦ qtbtt_dual_tree_intra_flag ) |  |
| pre_sao_enabled_flag[ cIdx ][ dir ] | ae(v) |
| else |  |
| pre_sao_enabled_flag[ cIdx ][ dir ] = 0 |  |
| if( pre_sao_enabled_flag[ cIdx ][ dir ] != 0 ) { |  |
| if( cIdx = = 0 ) |  |
| pre_sao_thresh_index[ cIdx ][ dir ] | ae(v) |
| else |  |
| pre_sao_thresh_index[ cIdx ][ dir ] = 1 |  |
| pre_sao_offset[ cIdx ][ dir ][ 0 ] | ae(v) |
| pre_sao_offset[ cIdx ][ dir ][ 1 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

## Syntax for slice-level Pre-SAO

| slice_header( ) { |  |
| --- | --- |
| … |  |
| if( slice_type != I ) { |  |
| if( cabac_init_present_flag ) |  |
| cabac_init_flag | u(1) |
| if( pic_temporal_mvp_enabled_flag ) { |  |
| if( slice_type = = B && !pps_collocated_from_l0_idc ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && NumRefIdxActive[ 0 ] > 1 ) ¦ ¦ ( !collocated_from_l0_flag && NumRefIdxActive[ 1 ] > 1 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| if( ( pps_weighted_pred_flag && slice_type = = P ) ¦ ¦ ( pps_weighted_bipred_flag && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| } |  |
| slice_qp_delta | se(v) |
| if( pps_slice_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| if( sps_joint_cbcr_enabled_flag ) |  |
| slice_joint_cbcr_qp_offset | se(v) |
| } |  |
| if( pps_cu_chroma_qp_offset_list_enabled_flag ) |  |
| cu_chroma_qp_offset_enabled_flag | u(1) |
| slice_pre_sao_enabled_flag | u(1) |
| if( slice_pre_sao_enabled_flag ) |  |
| pre_sao( 0, 0 ) |  |
| if( sps_sao_enabled_flag && !pic_sao_enabled_present_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| slice_sao_chroma_flag | u(1) |
| } |  |
| … |  |
| } |  |

## Syntax for alternative CTU-level Pre-SAO

| coding_tree_unit( ) { | Descriptor |
| --- | --- |
| xCtb = CtbAddrX << CtbLog2SizeY |  |
| yCtb = CtbAddrY << CtbLog2SizeY |  |
| if( slice_pre_sao_enabled_flag ) |  |
| pre_sao( CtbAddrX, CtbAddrY ) |  |
| if( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ) |  |
| sao( CtbAddrX, CtbAddrY ) |  |
| if( slice_alf_enabled_flag ){ |  |
| alf_ctb_flag[ 0 ][ CtbAddrX ][ CtbAddrY ] | ae(v) |
| if( alf_ctb_flag[ 0 ][ CtbAddrX ][ CtbAddrY ] ) { |  |
| if( slice_num_alf_aps_ids_luma > 0 ) |  |
| alf_use_aps_flag | ae(v) |
| if( alf_use_aps_flag ) { |  |
| if( slice_num_alf_aps_ids_luma > 1 ) |  |
| alf_luma_prev_filter_idx | ae(v) |
| } else |  |
| alf_luma_fixed_filter_idx | ae(v) |
| } |  |
| if( slice_alf_chroma_idc = = 1 ¦ ¦ slice_alf_chroma_idc = = 3 ) { |  |
| alf_ctb_flag[ 1 ][ CtbAddrX ][ CtbAddrY ] | ae(v) |
| if( alf_ctb_flag[ 1 ][ CtbAddrX ][ CtbAddrY ] && alf_chroma_num_alt_filters_minus1 > 0 ) |  |
| alf_ctb_filter_alt_idx[ 0 ][ CtbAddrX ][ CtbAddrY ] | ae(v) |
| } |  |
| if( slice_alf_chroma_idc = = 2 ¦ ¦ slice_alf_chroma_idc = = 3 ) { |  |
| alf_ctb_flag[ 2 ][ CtbAddrX ][ CtbAddrY ] | ae(v) |
| if( alf_ctb_flag[ 2 ][ CtbAddrX ][ CtbAddrX ] && alf_chroma_num_alt_filters_minus1 > 0 ) |  |
| alf_ctb_filter_alt_idx[ 1 ][ CtbAddrX ][ CtbAddrX ] | ae(v) |
| } |  |
| } |  |
| if( slice_type = = I && qtbtt_dual_tree_intra_flag ) |  |
| dual_tree_implicit_qt_split ( xCtb, yCtb, CtbSizeY, 0 ) |  |
| Else |  |
| coding_tree( xCtb, yCtb, CtbSizeY, CtbSizeY, 1, 1, 0, 0, 0, 0, 0, SINGLE_TREE, MODE_TYPE_ALL ) |  |
| } |  |

Note: slice_pre_sao_enabled_flag is transmitted in the slice_header( ), as for the slice-level Pre-SAO.

## Pre-Sample adaptive offset semantics

- cIdx: channel index (0 for luma, 1 for chroma)

- dir: Pre-SAO/deblocking direction (0 for vertical, 1 for horizontal)

pre_sao_enabled_flag[ cIdx ][ dir ] equal to 1 specifies that Pre-SAO processing is being applied for the specified channel index cIdx and direction dir (of the processed CTU).  pre_sao_enabled_flag[ cIdx ][ dir ] equal to 0 specifies that Pre-SAO processing is being disabled for the channel index cIdx and direction dir (of the given CTU). When pre_sao_enabled_flag[ cIdx ][ dir ] is not present, it is inferred to be equal to 0.

pre_sao_thresh_index[ cIdx ][ dir ] equal to 1 specifies that Pre-SAO processing is being applied with a classification threshold T = 2 for the specified channel index cIdx and direction dir (of the processed CTU). pre_sao_thresh_index[ cIdx ][ dir ] equal to 0 specifies that Pre-SAO processing is being applied with a classification threshold T = 1 for the specified channel index cIdx and direction dir (of the processed CTU). When pre_sao_thresh_index[ cIdx ][ dir ] is not present, it is inferred to be equal to 0, which means T = 1.

pre_sao_offset[ cIdx ][ dir ][ 0 ] represents d1 for the specified channel index cIdx and direction dir (of the processed CTU) during the application of the Pre-SAO processing and pre_sao_offset[ cIdx ][ dir ][ 1 ] represents d2 for the specified channel index cIdx and direction dir (of the processed CTU) during the application of the Pre-SAO processing.  When pre_sao_offset[ cIdx ][ dir ][ 0 ] is not present, it is inferred to be equal to 0, and when pre_sao_offset[ cIdx ][ dir ][ 1 ] is not present, it is inferred to be equal to 0.

pre_sao_merge_flag[ cIdx ][ dir ] equal to 1 specifies that the syntax elements pre_sao_offset[ cIdx ][ dir ] pre_sao_thresh_index[ cIdx ][ dir ] and pre_sao_enabled_flag[ cIdx ][ dir ] are derived from the corresponding syntax elements of the above, left, and/or above-left CTU. pre_sao_merge_flag[ cIdx ][ dir ] equal to 0 specifies that these syntax elements are not derived from the corresponding syntax elements of the neighboring CTU(s). When pre_sao_merge_flag[ cIdx ][ dir ] is not present, it is inferred to be equal to 0.

# References

[1]	JVET, “VTM version 7.1,” https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/commit/98ff0062.

[2]	F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Geneva, CH, Mar. 2019.

[3]	S. A. Martucci, “Reversible compression of HDTV images using median adaptive prediction and arithmetic coding,” in Proc. IEEE Symp. on Circuits and Systems, pp. 1310–1313, 1990.

[4]	Y.-K. Wang, B. Bross, V. Drugeon, J. Chen, “AHG2: Editorial input on VVC draft text,” JVET-Q0041, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Brussels, BE, Dec. 2019, online: http://phenix.it-sudparis.eu/jvet/doc_end_user/current_document.php?id=8866.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).