| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0458-v2 |
| --- | --- |

| Title: | CE4-Related: On-the-fly Weighting Index to Sample Blending Weight Conversion without Table Look-Up on Top of Combined Test in JVET-Q0061 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lien-Fei Chen, Xiang Li, Guichun Li, Shan Liu 2747 Park Boulevard, Palo Alto, CA 94306, U.S.A. | Email: | lienfeichen@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

In core experiment 4, CE4-1, CE4-2 and CE4-3 are all aiming to simplify the GEO design in the common based of CE4. In JVET-Q0061, two combinations, i.e., CE4-1+CE4-2.1+CE4-3.1 (test1) and CE4-1+CE4-2.1+CE4-3.2 (test2), are tested. However, two look-up tables are still necessary for sample blending weight calculation in geometric merge mode (GEO). The first table is used for cosine table look-up, and the seond one is designed to derive the final sample blending weight from weighting index. In this contribution, an on-the-fly weighting index to weight calculation method is proposed to eliminate the above second table such that blending weights of all samples in a blockcould be parallelly calculated on-the-fly, which is reported as much more friendly for both hardware and software implementation. The proposed method is implemented on top of CE4 common base. It is reported that, under the CTC, compared with VTM-7.0, the overall performance impact are as following:

Compared with Test 1

RA:  -0.01%/0.01%/-0.05% with 100% EncT and 100% DecT
LDB: 0.07%/0.01%/-0.24% with 100% EncT and 101% DecT
Compared with Test 2

RA:  -0.01%/-0.08%/-0.09% with 100% EncT and 99% DecT
LDB: 0.06%/-0.02%/-0.05% with 100% EncT and 99% DecT

Introduction

In CE4, the coding performance of geometric merge mode (GEO) is evaluated [1]-[3]. For the sample blending weight mask calculation, two look-up tables are used for cosine table look-up and blending weight index to weight conversion. The cosine table could be accessed once for all samples per coding block. However, multiple look-up tables for weighting index to weight conversion are necessary in parallel processing due to the fact that each sample has different input value of weighting index to weight table, which is not friendly for both hardware and software implementation. To address the issue, the weighting index to weight conversion is realized by using a simple equation such that expensive pixel level table lookup is avoided.

Proposal

On-the-fly Weighting Index to Weight Calculation

In order to reduce the implementation cost of look-up table and ease weighting derivation in parallel, the blending weight of samples could be derived with the following equations.

| wIdxL | =1≪idx2wShiftBit+2+partIdx ? wIdx : -wIdx | (1) |
| --- | --- | --- |

and

| weight | =Clip30, 8,wIdxL+1≪idx2wShiftBit-1≫idx2wShiftBit | (2) |
| --- | --- | --- |

Where idx2wShiftBit is the conversion factor of the weighting index to weight, and its typical value in Eq. (2) is 3.

Simulation Results

On-the-fly Weighting Index to Weight Calculation

Table 1 summarizes the CTC result of the proposed on-the-fly weighting index to weight conversion compared with VTM 7.0.

Table 1. Proposed CTC results compared with JVET-Q0061 Combined test 1

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7 |  |  |  |  | Over VTM-7 CE4 Q0061 Test 1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.23% | -0.24% | 101% | 100% | 0.00% | -0.05% | -0.04% | 100% | 100% |
| Class A2 | -0.17% | -0.14% | -0.13% | 101% | 101% | -0.01% | 0.02% | -0.04% | 100% | 101% |
| Class B | -0.14% | -0.10% | -0.18% | 101% | 102% | -0.02% | 0.02% | -0.07% | 99% | 100% |
| Class C | -0.53% | -0.86% | -0.89% | 100% | 98% | 0.01% | 0.05% | -0.05% | 99% | 98% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.25% | -0.34% | -0.37% | 101% | 100% | -0.01% | 0.01% | -0.05% | 100% | 100% |
| Class D | -0.33% | -0.56% | -0.49% | 101% | 98% | 0.08% | -0.03% | 0.10% | 99% | 100% |
| Class F | -0.23% | -0.29% | -0.25% | 99% | 99% | 0.04% | 0.04% | 0.03% | 99% | 99% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-7 |  |  |  |  | Over VTM-7 CE4 Q0061 Test 1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.26% | -0.58% | -0.45% | 103% | 100% | 0.00% | -0.10% | -0.17% | 100% | 100% |
| Class C | -0.60% | -1.15% | -0.76% | 101% | 101% | 0.12% | -0.02% | 0.11% | 100% | 100% |
| Class E | -0.87% | 0.13% | -0.71% | 100% | 101% | 0.10% | 0.24% | -0.82% | 100% | 101% |
| Overall | -0.52% | -0.59% | -0.62% | 101% | 100% | 0.07% | 0.01% | -0.24% | 100% | 101% |
| Class D | -0.45% | -0.83% | -0.66% | 100% | 100% | 0.21% | 0.11% | -0.34% | 99% | 101% |
| Class F | -0.44% | -1.01% | 0.03% | 100% | 97% | 0.05% | -0.18% | 0.06% | 99% | 98% |

Table 2. Proposed CTC results compared with JVET-Q0061 Combined test 2

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7 |  |  |  |  | Over VTM-7 CE4 Q0061 Test 2 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.17% | -0.23% | -0.24% | 102% | 100% | -0.02% | -0.02% | -0.14% | 100% | 100% |
| Class A2 | -0.19% | -0.15% | -0.06% | 102% | 100% | 0.00% | -0.05% | 0.01% | 100% | 100% |
| Class B | -0.17% | -0.18% | -0.22% | 102% | 99% | -0.02% | -0.08% | -0.12% | 99% | 98% |
| Class C | -0.62% | -0.96% | -0.87% | 101% | 99% | 0.01% | -0.16% | -0.09% | 99% | 98% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.29% | -0.39% | -0.37% | 102% | 100% | -0.01% | -0.08% | -0.09% | 100% | 99% |
| Class D | -0.38% | -0.66% | -0.53% | 102% | 98% | 0.06% | 0.16% | 0.11% | 99% | 98% |
| Class F | -0.27% | -0.33% | -0.30% | 100% | 99% | 0.05% | 0.05% | 0.02% | 100% | 98% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-7 |  |  |  |  | Over VTM-7 CE4 Q0061 Test 2 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.33% | -0.35% | -0.29% | 103% | 99% | 0.00% | 0.11% | 0.00% | 100% | 99% |
| Class C | -0.72% | -1.20% | -1.01% | 102% | 99% | 0.08% | -0.17% | -0.14% | 100% | 100% |
| Class E | -0.84% | 0.00% | -0.48% | 100% | 98% | 0.14% | -0.02% | -0.01% | 99% | 98% |
| Overall | -0.59% | -0.55% | -0.58% | 102% | 99% | 0.06% | -0.02% | -0.05% | 100% | 99% |
| Class D | -0.60% | -0.65% | -0.82% | 103% | 99% | 0.09% | 0.11% | -0.05% | 99% | 99% |
| Class F | -0.42% | -1.00% | -0.38% | 101% | 97% | 0.13% | -0.56% | -0.03% | 99% | 97% |

Conclusion

This contribution proposed a look-up table free weighting index to weight conversion. By using the proposed conversion equation, the blending weight of all samples in a coding block could be performed on-the-fly in parallel without the multiple table look-up access. It makes the blending mask calculation more efficient not only in software development but also in hardware implementation.

Reference

- H. Gao, S. Esenlik, E. Alshina, A. Kotra, B. Wang, M. Bläser, and J. Sauer, “Simplified GEO without multiplication and minimum blending mask storage (harmonization of JVET-P0107, JVET-P0264 and JVET-P0304)”, ISO/IEC JTC1/SC29/WG11 JVET-P0884, Oct. 2019.

- H. Gao, et. Al, “CE4 Common Base: Geometric inter prediction”, ISO/IEC JTC1/SC29/WG11, JVET-Q0079, Jan. 2020.

- H. Gao, et. al, “ CE4-related: Combined Test of CE4-1, CE4-2 and CE4-3,” ISO/IEC JTC1/SC29/WG11, JVET-Q0061, Jan. 2020.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).