| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0471-v2 |
| --- | --- |

| Title: | On chroma QT split |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guichun Li, XiangLi, Xiaozhong Xu, Shan Liu 2747 Park Boulevard, Palo Alto, CA 94306, U.S.A. | Tel: Email: | +1-650-798-3300 guichunli@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution firstly reports a VTM-7.0 bug with which luma block size is used when checking chroma QT split condition. Coding loss is reported for 4:2:2 sequences after fixing the bug. It is further proposed to use block height as a condition in QT split checking to compensate the loss. The proposed method doesn’t have impact on common test condition. It reportedly improves the coding performance when 4:2:2 color format is used and minimum chroma QT size is set to large values. On 4:2:2 test sequences, it was observed that when min chroma QT size is set to 32, the proposed method has BD-rate gain of -0.28% / -6.09% / -8.07% on AI, -0.60% / -4.21% / -5.07% on RA, and -0.03% / -2.42% / -3.29% on LB.

# Problem Statement

In VTM-7.0 software, there is a bug in chroma QT split checking, where the luma coding block size is checked against minimum chroma QT size, the equation used is (cbSize <= MinQtSizeC). To fix the bug and make the software align with the spec text of current VVC draft 7, the equation shall be changed to ( cbSize / SubWidthC ) <= MinQtSizeC. After fixing the bug, compared with VTM-7.0 anchor, coding loss is observed for 4:2:2 sequences when MinQtSizeC is relatively big, such as 32x32, as shown below.

Table 1 - Result of Min Chroma QT Size 32 with Bug Fix

|  |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Overall | 0.29% | 6.54% | 8.84% | 85% | 95% |
|  |  |  | RA |  |  |
|  | Y | U | V | EncT | DecT |
| Overall | 0.61% | 4.51% | 5.50% | 99% | 106% |
|  |  |  | LB |  |  |
|  | Y | U | V | EncT | DecT |
| Overall | 0.03% | 2.54% | 3.51% | 100% | 100% |

The variable allowSplitQt is derived as follows:

- If one or more of the following conditions are true, allowSplitQt is set equal to FALSE:

- treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA and cbSize is less than or equal to MinQtSizeY

- treeType is equal to DUAL_TREE_CHROMA and cbSize / SubWidthC is less than or equal to MinQtSizeC

- mttDepth is not equal to 0

- treeType is equal to DUAL_TREE_CHROMA and ( cbSize / SubWidthC ) is less than or equal to 4

- treeType is equal to DUAL_TREE_CHROMA and modeType is equal to MODE_TYPE_INTRA

- Otherwise, allowSplitQt is set equal to TRUE.

In current VVC design, the allowed quad split process is as described below:

Table 2 – SubWidthC and SubHeightC values derived from
chroma_format_idc and separate_colour_plane_flag

| chroma_format_idc | separate_colour_plane_flag | Chroma format | SubWidthC | SubHeightC |
| --- | --- | --- | --- | --- |
| 0 | 0 | Monochrome | 1 | 1 |
| 1 | 0 | 4:2:0 | 2 | 2 |
| 2 | 0 | 4:2:2 | 2 | 1 |
| 3 | 0 | 4:4:4 | 1 | 1 |
| 3 | 1 | 4:4:4 | 1 | 1 |

Among the conditions, when checking against minimum chroma QT size MinQtSizeC, ( cbSize / SubWidthC ) is used, where cbSize is the coding block size in luma samples, and SubWidthC is the chroma subsampling factor on horizontal direction as defined in Table 2.

When the chroma format is 4:2:2, the subsampling on horizontal direction and vertical direction are different. Using ( cbSize / SubWidthC ) <= MinQtSizeC as condition to disable QT splitting has higher chance of disabling chroma QT split, such as when luma coding block is equal to MinQtSizeC. This hurts coding performance on chroma components, especially when minimum QT size for Chroma is set to be a large value, as the results with bug fix shown above.

# Proposed Method

To compensate the loss by the bug fix, it is proposed to slightly change the condition for chroma QT split to allow more chances of quad split. In the allowed quad split checking process, ( cbSize / SubHeightC ) is used to check against MinQtSizeC.

## Proposed Spec Text Change

## Following the current spec text, the following change shall be made to reflect the proposed method.

### 6.4.1	Allowed quad split process

Inputs to this process are:

- a coding block size cbSize in luma samples,

- a multi-type tree depth mttDepth,

- a variable treeType specifying whether a single tree (SINGLE_TREE) or a dual tree is used to partition the coding tree node and, when a dual tree is used, whether the luma (DUAL_TREE_LUMA) or chroma components (DUAL_TREE_CHROMA) are currently processed,

- a variable modeType specifying whether intra (MODE_INTRA), IBC (MODE_IBC), and inter coding modes can be used (MODE_TYPE_ALL), or whether only intra and IBC coding modes can be used (MODE_TYPE_INTRA), or whether only inter coding modes can be used (MODE_TYPE_INTER) for coding units inside the coding tree node.

Output of this process is the variable allowSplitQt.

The variable allowSplitQt is derived as follows:

- If one or more of the following conditions are true, allowSplitQt is set equal to FALSE:

- treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA and cbSize is less than or equal to MinQtSizeY

- treeType is equal to DUAL_TREE_CHROMA and cbSize / SubWidthC SubHeightC is less than or equal to MinQtSizeC

- mttDepth is not equal to 0

- treeType is equal to DUAL_TREE_CHROMA and ( cbSize / SubWidthC ) is less than or equal to 4

- treeType is equal to DUAL_TREE_CHROMA and modeType is equal to MODE_TYPE_INTRA

- Otherwise, allowSplitQt is set equal to TRUE.

## Bug fix in current VVC draft

The variables MinQtLog2SizeY, MinQtLog2SizeC, MinQtSizeY, MinQtSizeC, MaxBtSizeY, MaxBtSizeC, MinBtSizeY, MaxTtSizeY, MaxTtSizeC, MinTtSizeY, MaxMttDepthY and MaxMttDepthC are derived as follows:

MinQtSizeY = 1  <<  MinQtLog2SizeY		(117)

MinQtSizeC = 1  <<  MinQtLog2SizeC		(118)

MinBtSizeY = 1  <<  MinCbLog2SizeY		(119)

MinTtSizeY = 1  <<  MinCbLog2SizeY		(120)

If slice_type equal to 2 (I),

MinQtLog2SizeY = MinCbLog2SizeY + pic_log2_diff_min_qt_min_cb_intra_slice_luma	(121)

MinQtLog2SizeC = MinCbLog2SizeC + pic_log2_diff_min_qt_min_cb_intra_slice_chroma	(122)

It was later found that the current VVC draft has an issue due to inconsistant unit used for some of the variables.

As shown in the above definitions as in current VVC draft, MinQtSizeC is derived from MinCbLog2SizeC. According to the semantic of pic_log2_diff_min_qt_min_cb_intra_slice_chroma, MinCbLog2SizeC shall be also in luma samples.

In the derivation process for allowSplitQt, current block size cbSize is in luma samples, following the convention of the spec text, MinQtSizeC should also be in luma samples. When comparing with cbSize, the division by chroma subsampling ratio SubWidthC is not needed. So the current text should be changed as following.

The variable allowSplitQt is derived as follows:

- If one or more of the following conditions are true, allowSplitQt is set equal to FALSE:

- treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA and cbSize is less than or equal to MinQtSizeY

- treeType is equal to DUAL_TREE_CHROMA and cbSize / SubWidthC is less than or equal to MinQtSizeC

- mttDepth is not equal to 0

- treeType is equal to DUAL_TREE_CHROMA and ( cbSize / SubWidthC ) is less than or equal to 4

- treeType is equal to DUAL_TREE_CHROMA and modeType is equal to MODE_TYPE_INTRA

- Otherwise, allowSplitQt is set equal to TRUE.

## Proposed Spec Text Change based on fixed text

The proposed to apply the proposed method on the fixed version of spec text as following.

### 6.4.1	Allowed quad split process

Inputs to this process are:

- a coding block size cbSize in luma samples,

- a multi-type tree depth mttDepth,

- a variable treeType specifying whether a single tree (SINGLE_TREE) or a dual tree is used to partition the coding tree node and, when a dual tree is used, whether the luma (DUAL_TREE_LUMA) or chroma components (DUAL_TREE_CHROMA) are currently processed,

- a variable modeType specifying whether intra (MODE_INTRA), IBC (MODE_IBC), and inter coding modes can be used (MODE_TYPE_ALL), or whether only intra and IBC coding modes can be used (MODE_TYPE_INTRA), or whether only inter coding modes can be used (MODE_TYPE_INTER) for coding units inside the coding tree node.

Output of this process is the variable allowSplitQt.

The variable allowSplitQt is derived as follows:

- If one or more of the following conditions are true, allowSplitQt is set equal to FALSE:

- treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA and cbSize is less than or equal to MinQtSizeY

- treeType is equal to DUAL_TREE_CHROMA and cbSize is less than or equal to ( MinQtSizeC * SubHeightC / SubWidthC )

- mttDepth is not equal to 0

- treeType is equal to DUAL_TREE_CHROMA and ( cbSize / SubWidthC ) is less than or equal to 4

- treeType is equal to DUAL_TREE_CHROMA and modeType is equal to MODE_TYPE_INTRA

- Otherwise, allowSplitQt is set equal to TRUE.

# Simulation Results

The proposed method was implemented on top of VTM-7.0. 4:2:2 sequences were used in simulations. Input color format is set to 4:2:2 and 2 Min chroma QT sizes are tested: 1) default min chroma QT size 4; 2) min chroma QT size of 32.

VTM-7.0 with the QT split condition bug fix as mentioned in section 1 was used as anchor.

As shown in Table 3, it was observed that when min chroma QT size is set to 32, the proposed method has BD-rate gain of -0.28% / -6.09% / -8.07% on AI, -0.60% / -4.21% / -5.07% on RA, and -0.03% / -2.42% / -3.29% on LB.

Table 3-Test results compared with VTM-7.0 with Bug Fix

|  |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| MinQTSizeC = 4 | 0.00% | 0.00% | 0.00% | 99% | 97% |
| MinQTSizeC = 32 | -0.28% | -6.09% | -8.07% | 116% | 105% |
|  |  |  | RA |  |  |
|  | Y | U | V | EncT | DecT |
| MinQTSizeC = 4 | 0.00% | 0.00% | 0.00% | 99% | 103% |
| MinQTSizeC = 32 | -0.60% | -4.21% | -5.07% | 100% | 103% |
|  |  |  | LB |  |  |
|  | Y | U | V | EncT | DecT |
| MinQTSizeC = 4 | 0.00% | 0.00% | 0.00% | 99% | 99% |
| MinQTSizeC = 32 | -0.03% | -2.42% | -3.29% | 100% | 98% |

# Conclusion

The proposed method reportedly has no performance impact on CTC. On 4:2:2 color format and when minimum chroma QT size is large, the proposed method has BD-rate gain on all configurations. It is proposed to adopt the proposed method to VVC.

# Reference

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-N1010 Marrakech, MA, Mar 2019.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-7.0

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 7)”, ISO/IEC JTC1/SC29/WG11 JVET-P2001, Oct. 2019.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).