| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0480-v2 |
| --- | --- |

| Title: | AHG9: A More Robust Syntax for Raster Rectangular Slices |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wade Wan Tim Hellman Brian Heng | Email: | firstname.lastname@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

The adoption of JVET-P1004 introduced a raster rectangular slice syntax to reduce the number of bits needed to describe common rectangular slice configurations.  Specifically, the syntax covers configurations in which all slices in a row have the same value for slice_height_in_tiles_minus1.  Despite this definition, the syntax presently requires slice_height_in_tiles_minus1 to be specified for every slice.  This contribution contends that the present syntax is both wasteful for raster rectangular slices as extra bits are encoded which are not necessary, and error-prone, as an encoder can erroneously set slice_height_in_tiles_minus1 to an illegal value.  It is proposed to add a condition to the syntax such that slice_height_in_tiles_minus1 is only specified for the first slice in a row when tile_idx_delta_present_flag is false.

Version 2: Added inference rule for slice_height_in_tile_minus1[i] and slice_width_in_tile_minus1[i] such that they are inferred to be equal to 0 when not present.  This was overlooked in version 1 resulting in corner case scenarios that were not properly handled and claimed this inference rule better avoids ambiguity in the text.

# Description

The rectangular slice structure is specified in the PPS in the current draft text [1].  Each rectangular slice has a width and height specified in tile units, and a delta value that indicates the start of the next slice.  This mechanism allows an arbitrary slice pattern to be created, but carries some overhead for common cases in which all slices in the same row have the same tile height.  For example, the diagram in Figure 1 shows an array of 2x2 tile rectangular slices.  An arrangement like that can assume a raster order for slice position and therefore the delta value can be skipped.

The adoption of JVET-P1004 [2] added a tile_idx_delta_present_flag to reduce the overhead in these cases.  When that flag is false, the tile_idx_delta syntax element isn’t included, as the position of each slice in the picture can be determined by proceeding in raster order.  However, a further optimization is possible as each slice in a row must, by definition, have the same tile height, yet the slice height is still being specified for every slice.  Not only does this increase the size of the header unnecessarily, but it also gives the encoder leeway to generate illegal streams by specifying a different height for slices in the same row.

The solution is straightforward:  when in raster rectangular slice mode (tile_idx_delta_present_flag equals 0), only specify the slice_height_in_tiles_minus1 syntax element at the start of each tile row.  For example, slice_height_in_tiles_minus1 would only be included for the left-most slices in Figure 1, and copied for the other 5 slices in each row.

Figure 1: Raster Rectangular Slice Example

Though the primary benefit of this change is elimination of one way to create illegal streams, it also provides a significant bit savings in common raster cases.  JVET-P1004 included a presentation with an analysis of the number of bits needed to encode two examples (example B is reproduced here as Figure 1).  Table 1 summarizes the bit savings resulting from the removal of the redundant tile height syntax elements from those two examples.

| Figure | Number of bits: JVET-P1004 | Number of bits: JVET-Q0480 |
| --- | --- | --- |
| A | 94 | 82 (-13%) |
| B | 148 | 91 (-25%) |

Table 1: Slice Description Bit Savings, Examples from JVET-P1004

# Proposal

Current draft text

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …. |  |
| tile_idx_delta_present_flag | u(1) |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) { |  |
| slice_width_in_tiles_minus1[ i ] | ue(v) |
| slice_height_in_tiles_minus1[ i ] | ue(v) |
| if( slice_width_in_tiles_minus1[ i ] = = 0 && slice_height_in_tiles_minus1[ i ] = = 0 ) { |  |
| num_slices_in_tile_minus1[ i ] | ue(v) |
| …. |  |

Proposed text changes

The text changes are provided as an attachment to this contribution.  The main text change is a single new condition on slice_height_in_tiles that is highlighted in yellow below.  Inclusion of slice_height_in_tiles_minus1 now only occurs for the first rectangular slice in a tile row when tile_idx_delta_present_flag is false.  The other two lines supply the default value for slice height for the rest of the slices in the row.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …. |  |
| tile_idx_delta_present_flag | u(1) |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) { |  |
| slice_width_in_tiles_minus1[ i ] | ue(v) |
| if (tile_idx_delta_present_flag ¦¦ (tileIdx % NumTileColumns == 0)) |  |
| slice_height_in_tiles_minus1[ i ] | ue(v) |
| else |  |
| slice_height_in_tiles_minus1[i] = slice_height_in_tiles_minus1[i-1] |  |
| if( slice_width_in_tiles_minus1[ i ] = = 0 && slice_height_in_tiles_minus1[ i ] = = 0 ) { |  |
| num_slices_in_tile_minus1[ i ] | ue(v) |
| …. |  |

# Conclusion

This proposal contends that a small change to the PPS rectangular slice syntax makes the syntax more robust and also significantly reduces the number of bits needed to describe raster rectangular slice structures.  It therefore recommends that this proposal be adopted into the draft text.

# References

[1] Y.-K. Wang, B. Bross, V. Drugeon, “AHG2: Editorial input on VVC draft text”, document JVET-Q0041, JVET, Brussels, Belgium, January 2020.

[2] T. Hellman, W. Wan, et al, “AHG17: Removal of bricks”, document JVET-P1004, JVET, Geneva, CH, October 2019.

# Patent rights declaration(s)

Broadcom Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).