| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0498 |
| --- | --- |

| Title: | Non-CE3: TB based Rice parameter selection for transform skip residual coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen, Tsung-Chuan Ma, Xiaoyu Xiu, Hong-Jheng Jhu, Xianglin Wang 8910 University Center Dr., Suite 400, San Diego, CA 92122, USA | Email: | yiwenchen@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In residual coding of a transform skip block, for each sample, the remaining absolute level “abs_remainder” is binarized using the fixed rice parameter equal to 1. However, it is observed that using different rice parameters for different conditions (e.g. different sequences, different QPs) improves the coding efficiency especially for low QP and lossless conditions. In this proposal, it is proposed to explicitly signal the rice parameter for each TB to indicate the rice parameter (either 0, 1, 2 or 3) for the binary codewords of abs_remainder.

The results of TB-level rice parameter compared to VTM7.0 on CTC sequences:

Standard QPs: 0.03% AI, 0.01% RA, -0.04% LB

Low QPs   : -0.04 % AI, -x.xx % RA, -0.04 % LB

Lossless    : -4.61% AI, -3.40% RA, -1.83% LB

The results of TB-level rice parameter compared to VTM7.0 on Class F sequences:

Standard QPs: 0.03% AI, 0.05% RA, -0.05% LB

Low QPs   : -1.14 % AI, -1.20 % RA, -0.49 % LB

Lossless    : -2.38% AI, -1.95% RA, -1.73% LB

The results of TB-level rice parameter compared to VTM7.0 on TGM sequences:

Standard QPs: -0.18% AI, 0.00% RA, -0.04% LB

Low QPs   : -2.43 % AI, -2.33 % RA, -1.42 % LB

Lossless    : -4.95% AI, -5.17% RA, -5.38% LB

# Introduction

In residual coding of a transform skip block, the remainder of the absolute levels are binarized using fixed rice parameter “1”. But it is observed that different blocks use different rice parameters obtains higher coding performance. In this proposal, a TB based rice parameter is proposed for the transform skip residual coding for higher coding efficiency especially for low QP and lossless conditions.

# Proposal

In the proposed method, one control flag is signaled in slice header to indicate the signaling of rice parameter for the transform skip blocks is enabled or disabled. When the control flag is signaled as enabled, one syntax element is further signaled for each transform skip TB to indicate the rice parameter of that TB. When the control flag is signaled as disabled (e.g. set equal to “0”), no further syntax element is signaled at lower level to indicate the rice parameter for the transform skip TB and a default rice parameter (e.g. 1) is used for all the transform skip TB.

In the proposed method, the available rice parameters include {0, 1, 2, 3} for each TB and the binary codeword (using truncated binary binarization) are given in Table 1.

Table 1 Proposed codewords for the rice parameters

| Rice Parameter | Binary codeword |
| --- | --- |
| 0 | 1 |
| 1 | 01 |
| 2 | 001 |
| 3 | 000 |

# Simulation Results

The proposed method is implemented based on the VTM-7.0 reference software [1] and tested using common test condition defined in [2]. Additional tests are also carried out by testing the proposed method under lossless and low QP conditions.

Table 2: Results of TB based Rice Parameters (0,1,2,3) for lossless condition

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM7.0 | TEST |  | VTM7.0 | TEST |  | VTM7.0 | TEST |  |
| Class A1 | 2.2 | 2.4 | -4.95% | 2.2 | 2.3 | -4.51% |  |  |  |
| Class A2 | 1.6 | 1.8 | -9.91% | 1.7 | 1.9 | -8.07% |  |  |  |
| Class B | 2.2 | 2.2 | -2.95% | 2.3 | 2.4 | -1.40% | 2.3 | 2.4 | -1.37% |
| Class C | 1.9 | 2.0 | -3.59% | 2.4 | 2.5 | -1.54% | 2.4 | 2.5 | -1.37% |
| Class D | 1.9 | 2.0 | -3.64% | 2.8 | 2.8 | -1.25% | 2.7 | 2.8 | -1.04% |
| Class E | 2.8 | 2.9 | -3.08% |  |  |  | 3.1 | 3.2 | -3.20% |
| Class F | 5.3 | 5.4 | -2.38% | 33.7 | 34.4 | -1.95% | 50.7 | 51.5 | -1.73% |
| TGM | 11.8 | 12.5 | -4.95% | 107.1 | 115.0 | -5.17% | 124.9 | 135.2 | -5.38% |
| Overall | 2.1 | 2.2 | -4.61% | 2.2 | 2.3 | -3.40% | 2.6 | 2.6 | -1.83% |
| Enc Time[%] | 116% |  |  | 139% |  |  | 135% |  |  |
| Dec Time[%] | 95% |  |  | 92% |  |  | 103% |  |  |

Table 3 Results of TB based Rice Parameters (0,1,2,3) for low QPs

|  | All intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.02% | -0.02% | 108% | 103% |
| Class A2 | 0.00% | 0.00% | 0.00% | 103% | 97% |
| Class B | -0.02% | 0.01% | 0.02% | 106% | 99% |
| Class C | -0.02% | -0.02% | 0.00% | 108% | 102% |
| Class E | -0.15% | -0.17% | -0.22% | 103% | 94% |
| Overall | -0.04% | -0.03% | -0.03% | 106% | 99% |
| Class D | 0.01% | -0.03% | 0.00% | 107% | 99% |
| Class F | -1.14% | -0.78% | -0.74% | 110% | 100% |
| Class TGM | -2.43% | -2.22% | -2.22% | 108% | 95% |
|  | Random access |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | 0.07% | 0.06% | 111% | 99% |
| Class C | -0.04% | 0.13% | 0.06% | 112% | 100% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.03% | 0.25% | 0.13% | 114% | 102% |
| Class F | -1.20% | -0.25% | -0.20% | 107% | 97% |
| Class TGM | -2.33% | -1.60% | -1.63% | 108% | 98% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | 0.09% | 0.13% | 113% | 98% |
| Class C | -0.02% | 0.13% | 0.13% | 119% | 103% |
| Class E | -0.09% | 0.10% | 0.13% | 105% | 92% |
| Overall | -0.04% | 0.11% | 0.13% | 113% | 98% |
| Class D | -0.03% | 0.20% | 0.07% | 111% | 101% |
| Class F | -0.49% | -0.24% | -0.40% | 109% | 99% |
| Class TGM | -1.42% | -1.06% | -1.07% | 112% | 101% |

Table 4 Results of TB based Rice Parameters (0,1,2,3) for standard QPs

|  | All intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | -0.12% | -0.11% | 101% | 98% |
| Class A2 | 0.00% | 0.07% | 0.02% | 105% | 100% |
| Class B | 0.02% | 0.05% | -0.06% | 104% | 98% |
| Class C | 0.04% | 0.04% | 0.03% | 98% | 94% |
| Class E | 0.03% | -0.08% | 0.02% | 106% | 96% |
| Overall | 0.03% | 0.00% | -0.02% | 103% | 97% |
| Class D | 0.05% | -0.11% | 0.01% | 105% | 99% |
| Class F | 0.03% | -0.20% | 0.01% | 100% | 88% |
| Class TGM | -0.18% | -0.26% | -0.25% | 101% | 93% |
|  | Random access |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.10% | -0.05% | 101% | 97% |
| Class A2 | -0.07% | -0.04% | 0.12% | 104% | 99% |
| Class B | 0.00% | 0.02% | 0.00% | 103% | 97% |
| Class C | 0.08% | -0.04% | -0.04% | 106% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.03% | 0.00% | 104% | 97% |
| Class D | 0.06% | -0.30% | -0.11% | 107% | 98% |
| Class F | 0.05% | -0.08% | -0.02% | 105% | 97% |
| Class TGM | 0.00% | -0.11% | -0.04% | 105% | 96% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.51% | 0.03% | 104% | 99% |
| Class C | 0.00% | -0.20% | 0.19% | 108% | 102% |
| Class E | -0.13% | 0.27% | -0.54% | 98% | 97% |
| Overall | -0.04% | -0.21% | -0.06% | 104% | 100% |
| Class D | 0.07% | 0.06% | -0.47% | 104% | 94% |
| Class F | -0.05% | -0.50% | 0.15% | 104% | 98% |
| Class TGM | -0.04% | -0.29% | -0.16% | 108% | 103% |

# Conclusions

- A TB based Rice parameter for transform skip is proposed in this contribution, which has no negligible complexity to the decoder but provides significant BD-rate saving for low QP and lossless conditions. It is recommended the proposed method be adopted into VVC.

# References

- VTM-7.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.0.

- A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy, “ JVET common test conditions and evaluation procedures for HDR/WCG video”, JVET-P2011, 16th meeting, Geneva, CH, 1–11 October 2019

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).