| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0507-v2 |
| --- | --- |

| Title: | CE4-related: Quality scalable GEO |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jingya Li Chong Soon Lim 202 Bedok South Ave 1 #02-11 Singapore, 469332 | Tel: Email: | +65 6552 5423 jingya.li@sg.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

It has been commented by the AHG16 on the JVET email reflector that the proposed GEO design under consideration is not quality scalable on the encoder side, a less capable encoder supporting a subset of GEO modes may incur loss instead of gain. This contribution proposes a quality scalable GEO design, in which the actual number of used GEO mode is signalled to make the coding of GEO mode index more efficient when a subset of GEO modes is in use. The test is done on CE4-1 software in which 64 GEO modes are supported, but the 64 GEO mode table is re-ordered based on statistics with the most probable modes having low indices in the table. To constrain the number of combination cases, only one level of quality scalability is enabled in the design, with a SPS flag equal to 0 and 1 meaning 16 and 64 modes in use, respectively. Compared to the VTM7.0 anchor, the summary results are as follows:

| Test # | Setting | RA (Y, U, V) | LDB (Y, U, V) |
| --- | --- | --- | --- |
| Test 1) | scalable number of modes (16 modes) | -0.09%, -0.09%, -0.08% | -0.28%, -0.20%, -0.20% |
| Test 2) | scalable number of modes (64 modes) | -0.28%, -0.36%, -0.30% | -0.70%, -0.62%, -0.62% |
| Test 3) | scalable number of modes (8 modes) | 0.06%, 0.18%, 0.16% | 0.06%, 0.14%, 0.26% |
| Test 4) | scalable number of modes (32 modes) | -0.19%, -0.25%, -0.19% | -0.53%, -0.47%, -0.35% |
| Test 5) | scalable number of modes (48 modes) | -0.25%, -0.29%, -0.27% | -0.64%, -0.68%, -0.73% |
| Test 6) | scalable number of modes (16 modes, but GEO mode table not re-ordered) | 0.04%, 0.06%, 0.14% | 0.07%, 0.31% ,0.47% |
| Test 7) | non-scalable number of modes (16 modes, GEO mode table not re-ordered) | 0.19% ,0.30%, 0.30% | 0.44%, 0.62% , 0.69% |

The results of Test 3 to Test 7) are provided as supplemental information.

In test 6), the CE4-1 software is used with only the first 16 GEO modes are enabled on the encoder side, the proposed signalling is used. Loss of 0.04% in RA and 0.07% in LDB are observed.

In test 7), the CE4-1 software is used with only the first 16 GEO modes are enabled on the encoder side, the proposed signalling is not used. The loss of 0.19% in RA and 0.44% in LDB verified the AHG16 finding that the proposed GEO design under consideration is not quality scalable on the encoder side.

When compared test 6) with test 7), the proposed signaling provides an average gain of roughly -0.15% in RA and -0.37% in LDB when the first 16 modes are enabled.

When compared test 1) with test 6), the re-ordered GEO table provides an average gain of roughly -0.13% in RA and -0.35% in LDB when the first 16 modes are enabled.

When compared test 1) and test 7), the re-ordered GEO table and the proposed signalling together provides an average gain of roughly -0.28% in RA and -0.72% in LDB when the first 16 modes are enabled.

# Introduction

This contribution proposes a quality scalable GEO design, in which the actual number of used GEO mode is signalled into the bitstream. Purpose of this proposal is to address a concern raised by the AHG16 on the JVET email reflector, which claimed that the proposed GEO design under consideration is not quality scalable on the encoder side, a less capable encoder supporting a subset of GEO modes may incur loss instead of gain. CE4-1 software is used for the investigation.

# Scalable GEO

The scalable idea for GEO was first proposed in JVET-P0174. This aspect focuses on making number of modes ‘scalable’ which is to allow encoder to choose to use either 16 valid geo modes or 64 valid geo modes.

If the GEO is enabled, a flag is signaled in SPS level to switch between 16 and 64 modes.

- If 16 mode is selected, only the first 16 modes of GEO mode table are valid.

- If 64 mode is selected, all 64 modes of GEO mode table are valid.

The GEO mode table is reordered according to statistics. and signaling total used number of GEO modes affects coding of merge_geo_parititon_idx with cmax set to 16 and 64 respectively.

Syntax change is illustrated in Table 1 as below:

Table 1: proposed SPS syntax change to support ‘scalable’ GEO

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … | u(4) |
| sps_geo_enabled_flag | u(1) |
| if(sps_geo_enabled_flag) |  |
| sps_geo_mode_set_id | u(1) |
| … |  |

sps_geo_mode_set_id specifies whether there are 16 or 64 used modes for GEO inter prediction. sps_geo_mode_set_id equals to 0 specifies that there are 16 used modes and sps_geo_mode_set_id equals to 1 specifies that there are 64 GEO modes in use. When sps_geo_mode_set_id is not present, it is inferred to be equal to 1.

| merge_geo_partition_idx[ ][ ] | TB | cMax = sps_geo_mode_set_id == 0 ? 16 : 64 |
| --- | --- | --- |

As supplemental information, scalable number GEO modes of 8, 32, 48 are also tested.

To verify the benefit of the proposed signaling, the following two tests are added:

- In test 6), the CE4-1 software is used with only the first 16 GEO modes are enabled on the encoder side, the proposed signalling is used.

- In test 7), the CE4-1 software is used with only the first 16 GEO modes are enabled on the encoder side, the CE4-1 signalling method is used.

# Experimental Results

The tests are carried out using the CE4-1 software [2], and with proposed changes. The common test conditions specified in [1] are followed.

| Test # | Setting | RA (Y, U, V) | LDB (Y, U, V) |
| --- | --- | --- | --- |
| Test 1) | scalable number of modes (16 modes) | -0.09%, -0.09%, -0.08% | -0.28%, -0.20%, -0.20% |
| Test 2) | scalable number of modes (64 modes) | -0.28%, -0.36%, -0.30% | -0.70%, -0.62%, -0.62% |
| Test 3) | scalable number of modes (8 modes) | 0.06%, 0.18%, 0.16% | 0.06%, 0.14%, 0.26% |
| Test 4) | scalable number of modes (32 modes) | -0.19%, -0.25%, -0.19% | -0.53%, -0.47%, -0.35% |
| Test 5) | scalable number of modes (48 modes) | -0.25%, -0.29%, -0.27% | -0.64%, -0.68%, -0.73% |
| Test 6) | scalable number of modes (16 modes, GEO mode table not re-ordered) | 0.04%, 0.06%, 0.14% | 0.07%, 0.31% ,0.47% |
| Test 7) | non-scalable number of modes (16 modes, GEO mode table not re-ordered) | 0.19% ,0.30%, 0.30% | 0.44%, 0.62% , 0.69% |

When compared test 6) with test 7), the proposed signaling provides an average gain of roughly -0.15% in RA and -0.37% in LDB when the first 16 modes are enabled.

When compared test 1) with test 6), the re-ordered GEO table provides an average gain of roughly -0.13% in RA and -0.35% in LDB when the first 16 modes are enabled.

When compared test 1) and test 7), the re-ordered GEO table and the proposed signalling together provides an average gain of roughly -0.28% in RA and -0.72% in LDB when the first 16 modes are enabled.

The summary results are provided in Table 2 to Table 8.

Table 2: Summary of using only first 16 modes with cmax of geo_parititoning_idx set to 16 (The GEO mode table is re-ordered, test 1)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.09% | 0.00% | 102% | 101% |
| Class A2 | -0.06% | 0.04% | 0.05% | 101% | 100% |
| Class B | -0.06% | 0.03% | -0.04% | 101% | 101% |
| Class C | -0.21% | -0.34% | -0.28% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | -0.09% | -0.08% | 101% | 100% |
| Class D | -0.16% | -0.32% | -0.25% | 101% | 100% |
| Class F | -0.22% | -0.21% | -0.19% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.16% | -0.28% | -0.17% | 101% | 100% |
| Class C | -0.25% | -0.42% | -0.09% | 101% | 99% |
| Class E | -0.52% | 0.22% | -0.38% | 99% | 99% |
| Overall | -0.28% | -0.20% | -0.20% | 100% | 99% |
| Class D | -0.33% | 0.12% | -0.53% | 100% | 100% |
| Class F | -0.34% | -0.49% | 0.01% | 100% | 100% |

Table 3: Summary of using only first 64 modes with cmax of geo_parititoning_idx set to 64 (The GEO mode table is re-ordered, test 2)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.20% | -0.18% | 103% | 101% |
| Class A2 | -0.19% | -0.14% | -0.02% | 103% | 100% |
| Class B | -0.14% | -0.11% | -0.09% | 102% | 100% |
| Class C | -0.61% | -0.97% | -0.86% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.28% | -0.36% | -0.30% | 102% | 100% |
| Class D | -0.41% | -0.64% | -0.52% | 102% | 100% |
| Class F | -0.33% | -0.34% | -0.35% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.32% | -0.44% | -0.26% | 103% | 100% |
| Class C | -0.81% | -1.15% | -0.96% | 103% | 99% |
| Class E | -1.18% | -0.21% | -0.77% | 101% | 99% |
| Overall | -0.70% | -0.62% | -0.62% | 102% | 99% |
| Class D | -0.68% | -0.56% | -0.76% | 102% | 99% |
| Class F | -0.53% | -0.75% | 0.05% | 102% | 99% |

Table 4: Summary of using only first 8 modes with cmax of geo_parititoning_idx set to 8 (The GEO mode table is re-ordered, test 3)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | 0.18% | 0.21% | 102% | 101% |
| Class A2 | 0.03% | 0.17% | 0.23% | 101% | 100% |
| Class B | 0.06% | 0.21% | 0.13% | 101% | 100% |
| Class C | 0.06% | 0.15% | 0.13% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | 0.18% | 0.16% | 101% | 100% |
| Class D | -0.03% | -0.01% | 0.00% | 101% | 100% |
| Class F | -0.11% | -0.03% | -0.08% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.09% | -0.09% | 0.26% | 101% | 100% |
| Class C | 0.18% | 0.09% | 0.40% | 101% | 99% |
| Class E | -0.14% | 0.57% | 0.08% | 99% | 99% |
| Overall | 0.06% | 0.14% | 0.26% | 100% | 99% |
| Class D | 0.00% | 0.32% | 0.48% | 100% | 99% |
| Class F | -0.03% | -0.47% | 0.43% | 100% | 99% |

Table 5: Summary of using only first 32 modes with cmax of geo_parititoning_idx set to 32 (The GEO mode table is re-ordered, test 4)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.19% | -0.07% | 102% | 101% |
| Class A2 | -0.13% | -0.12% | 0.02% | 101% | 100% |
| Class B | -0.12% | -0.08% | -0.11% | 101% | 100% |
| Class C | -0.41% | -0.60% | -0.53% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.19% | -0.25% | -0.19% | 101% | 100% |
| Class D | -0.31% | -0.46% | -0.52% | 101% | 100% |
| Class F | -0.29% | -0.32% | -0.27% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.29% | -0.34% | -0.17% | 101% | 100% |
| Class C | -0.59% | -0.80% | -0.48% | 101% | 99% |
| Class E | -0.84% | -0.24% | -0.49% | 99% | 99% |
| Overall | -0.53% | -0.47% | -0.35% | 100% | 99% |
| Class D | -0.54% | -0.46% | -0.64% | 100% | 99% |
| Class F | -0.41% | -1.17% | -0.22% | 100% | 99% |

Table 6: Summary of using only first 48 modes with cmax of geo_parititoning_idx set to 48 (The GEO mode table is re-ordered, test 5)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.15% | -0.18% | 108% | 106% |
| Class A2 | -0.17% | -0.07% | 0.02% | 103% | 100% |
| Class B | -0.15% | -0.11% | -0.10% | 102% | 97% |
| Class C | -0.55% | -0.79% | -0.74% | 105% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.25% | -0.29% | -0.27% | 104% | 100% |
| Class D | -0.41% | -0.53% | -0.60% | 115% | 125% |
| Class F (optional) | -0.33% | -0.36% | -0.29% | 109% | 119% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.37% | -0.54% | -0.43% | 108% | 111% |
| Class C | -0.76% | -1.14% | -0.81% | 109% | 107% |
| Class E | -0.93% | -0.30% | -1.11% | 103% | 107% |
| Overall | -0.64% | -0.68% | -0.73% | 107% | 109% |
| Class D | -0.64% | -0.42% | -0.44% | 99% | 98% |
| Class F (optional) | -0.51% | -1.04% | -0.09% | 109% | 103% |

Table 7: Summary of using only first 16 modes with cmax of geo_parititoning_idx set to 16 (The GEO mode table is not re-ordered, test 6)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.05% | 0.07% | 101% | 101% |
| Class A2 | 0.05% | 0.18% | 0.22% | 100% | 100% |
| Class B | 0.06% | 0.09% | 0.22% | 101% | 101% |
| Class C | -0.01% | 0.02% | 0.03% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.06% | 0.14% | 101% | 101% |
| Class D | -0.07% | 0.13% | 0.08% | 101% | 100% |
| Class F | 0.01% | 0.04% | 0.07% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.14% | 0.20% | 0.46% | 101% | 101% |
| Class C | 0.18% | 0.04% | 0.23% | 100% | 100% |
| Class E | -0.19% | 0.88% | 0.78% | 99% | 100% |
| Overall | 0.07% | 0.31% | 0.47% | 100% | 100% |
| Class D | 0.06% | 0.25% | 0.24% | 100% | 100% |
| Class F | 0.21% | -0.01% | 0.49% | 100% | 100% |

Table 8: Summary of using only first 16 modes with cmax of geo_parititoning_idx set to 64 (The GEO mode table is not re-ordered, test 7)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.13% | 0.20% | 0.24% | 101% | 100% |
| Class A2 | 0.19% | 0.35% | 0.41% | 101% | 100% |
| Class B | 0.18% | 0.31% | 0.22% | 101% | 100% |
| Class C | 0.23% | 0.33% | 0.36% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.19% | 0.30% | 0.30% | 101% | 100% |
| Class D | 0.15% | 0.02% | 0.21% | 101% | 100% |
| Class F | 0.13% | 0.20% | 0.26% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.40% | 0.37% | 0.69% | 101% | 100% |
| Class C | 0.55% | 0.52% | 0.79% | 100% | 100% |
| Class E | 0.35% | 1.18% | 0.56% | 99% | 100% |
| Overall | 0.44% | 0.62% | 0.69% | 100% | 100% |
| Class D | 0.48% | 0.96% | 0.37% | 100% | 100% |
| Class F | 0.29% | 0.55% | 0.76% | 100% | 100% |

# Discussions

Supporting 64 GEO modes on the encoder side is very challenging for a real time encoder implementation. This contribution proposes a solution to enable one-level quality scalability for the proposed GEO design under consideration.

# References

[1] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, the 13th JVET meeting, Jan. 2019, Marrakesh, MA

[2] H. Gao, S. Esenlik, E. Alshina, A. M. Kotra, B. Wang (Huawei), R.-L. Liao, J. Chen, Y. Ye, J. Luo (Alibaba), K. Reuzé, C.-C. Chen, H. Huang, W.-J. Chien, V. Seregin (Qualcomm), M. Blaser, J. Sauer (RWTH Aachen), “	CE4-1: Geometric Inter Prediction with 64 Modes”, 17th Meeting: Brussels, BE, 7–17 January 2020

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).