| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0587-v3 |
| --- | --- |

| Title: | Crosscheck of JVET-Q0493 (Non-CE2: Palette encoder improvements for lossless coding) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ru-Ling Liao Radiance Jin Hui Tower, Building No.6, The 4 District Wangjing Dong Yuan, Chaoyang District, Beijing, China | Tel: Email: | ruling.lrl@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

This contribution is a crosscheck report of JVET-Q0493. It is confirmed that the results match with the proponent’s results and the implementation match the proposal.

In the third version, the calculation equation of the overall performance impact is corrected.

Crosscheck results

Table 1. Simulation results relative to VTM-7.0 with dual tree on

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | VTM7 | test |  | VTM7 | test |  | VTM7 | test |  |
| TGM 1080p | 20.1 | 21.3 | -6.13% | 42.5 | 44.6 | -3.35% | 62.1 | 64.7 | -2.69% |
| TGM 720p | 11.0 | 12.1 | -7.19% | 13.9 | 15.3 | -4.28% | 25.2 | 26.5 | -2.14% |
| Animation | 5.8 | 6.9 | -7.66% | 1.7 | 2.0 | -6.27% | 1.6 | 2.0 | -7.14% |
| Mixed content | 7.2 | 7.3 | -1.89% | 6.2 | 6.2 | -0.29% | 6.9 | 6.9 | -0.18% |
| Camera-Captured content | 1.9 | 1.9 | 0.00% | 0.3 | 0.3 | 0.00% | 0.3 | 0.3 | 0.00% |
| Overall | 9.2 | 9.9 | -5.12% | 12.9 | 13.7 | -3.14% | 19.2 | 20.1 | -2.58% |
| Enc Time[%] | 99% |  |  | 99% |  |  | 100% |  |  |
| Dec Time[%] | 92% |  |  | 97% |  |  | 96% |  |  |

Table 2. Simulation results relative to VTM-7.0 with dual tree off

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | VTM7 | test |  | VTM7 | test |  | VTM7 | test |  |
| TGM 1080p | 26.8 | 27.4 | -2.46% | 54.1 | 55.0 | -1.39% | 75.9 | 76.9 | -1.11% |
| TGM 720p | 13.4 | 13.5 | -2.30% | 16.7 | 16.8 | -0.80% | 27.5 | 27.7 | -0.84% |
| Animation | 6.0 | 7.1 | -7.35% | 1.7 | 2.0 | -6.24% | 1.6 | 2.0 | -7.13% |
| Mixed content | 7.9 | 8.0 | -0.66% | 6.3 | 6.3 | -0.12% | 7.0 | 7.0 | -0.07% |
| Camera-Captured content | 1.8 | 1.8 | 0.00% | 0.3 | 0.3 | 0.00% | 0.3 | 0.3 | 0.00% |
| Overall | 11.2 | 11.6 | -2.69% | 15.8 | 16.1 | -1.74% | 22.5 | 22.8 | -1.84% |
| Enc Time[%] | 101% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 93% |  |  | 98% |  |  | 96% |  |  |