| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0593-v1 |
| --- | --- |

| Title: | AHG9: A summary of proposals on parameter sets cleanups |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang San Diego, CA, USA | Tel: Email: | yekui.wang@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution intends to provide a summary of all proposals on parameter sets cleanups submitted to this JVET meeting. It is suggested that this summary is used for the reviewing of these proposals, such that the discussions can be in a more structured and efficient manner.

# List of proposals on parameter sets cleanups

This category of proposals is listed in the agenda item 6.19.4, including 20 contributions. These contributions and summaryies of the related proposals are listed below.

JVET-Q0045 AHG9: On DPS identifier [V. Drugeon (Panasonic)]

It is suggested to remove the possibility to have DPS identifiers, since the DPS is not allowed to change within a whole bitstream and has no influence on the decoding process.

Option 1: If a DPS NAL unit is present within a bitstream or a DPS is available by other means, the SPS refers to the information signalled in that DPS.

Option 2: Replace the sps_decoding_parameter_set_id by an sps_decoding_parameter_set_flag to signal if the SPS refers to any DPS.

JVET-Q0117 AHG9: Cleanups on parameter sets [Y.-K. Wang, J. Xu (Bytedance)]

This contribution proposes some cleanup changes on DPS, SPS, PPS, and APS, listed below.

On DPS:

- Add the following constraint: It is required that all DPS NAL units with a particular value of dps_decoding_parameter_set_id in a bitstream shall have the same content.

- Add the following constraint: When present in a bitstream, a DPS NAL unit shall be present in the first AU of the bitstream, and may be present in any AU that has at least one coded slice NAL unit having nal_unit_type in the range of IDR_W_RADL to GDR_NUT, inclusive (such a NAL unit is an IRAP or a GDR VCL NAL unit), and shall not be present in any other AU.

- To make sure that each profile_tier_level( ) syntax structure included in the DPS does not contain sublayer level information, add one more input argument for the PTL syntax structure to control whether there is sublayer level information signalled at all. This argument is 0 (i.e., no sublayer level information) for DPS and 1 for VPS and SPS.

- The following constraint is specified: Each OLS in a CVS in the bitstream shall confirm to at least one of the PTL syntax structures in the DPS (as part of the DPS semantics). Accordingly, a change is also made to the semantics of the PTL syntax structure regarding the OlsInScope: "When a profile_tier_level( ) syntax structure is included in a DPS, the OlsInScope is one or more unidentified OLSs in one or more unidentified CVSs in the bitstream."

- The DPS ID (i.e., dps_decoding_parameter_set_id syntax element) is removed from the DPS syntax, and consequently the reference of the DPS ID in the SPS syntax (sps_decoding_parameter_set_id) is also removed. Effectively, the DPS then becomes a standalone NAL unit instead of being a parameter set. Therefore, the name of DPS can be changed to be "decoding parameters NAL unit". And the above items 1 and 2 apply to decoding parameters NAL units.

On SPS:

- It is proposed to move sps_seq_parameter_set_id to be the first syntax element in the SPS syntax now that it is u(4)-coded, to be aligned with the signalling position of the parameter IDs for all other parameter sets.

- Since sps_joint_cbcr_enabled_flag, which controls a chroma coding tool, is currently unnecessarily signalled when the color format is 4:0:0, it is proposed to be conditioned on "if(ChromaArrayType  !=  0 )".

On PPS: It is proposed to repeat ref_pic_resampling_enabled_flag and condition the picture width and height syntax elements as well as the scaling_window_flag based on the repeated PPS flag.

On APS: It is proposed to add the following rule regarding the APS ID value space: All APS NAL units with a particular value of aps_params_type, regardless of the nuh_layer_id values, share the same value space for adaptation_parameter_set_id. APS NAL units with different values of aps_params_type use separate values spaces for adaptation_parameter_set_id. Consequently, the NOTE saying "Each type of APSs uses a separate value space for adaptation_parameter_set_id" can be removed.

JVET-Q0280 AHG8/AHG9: On video parameter set ID [B. Choi, S. Wenger, S. Liu (Tencent)]

To make it clear that only one VPS is referred to by SPSs in a CVS, the following constraint is proposed: The value of sps_video_parameter_set_id shall be the same in all SPSs that are referred to by coded pictures in a CVS.

JVET-Q0355 AHG8/AHG9: Cleanup on multi-layer coding [X. Ma, H. Yang (Huawei)]

Item 3 of this contribution belongs to this category.

3. Fix the constraint of general_level_idc for multi-layer coding.

The intent: The maximum level indicated in the DPS shall not less than the level indicated in a lower level parameter set.

JVET-Q0357 AHG9: On clarification of DPS [X. Ma, H. Yang (Huawei)]

In the current VVC specification draft, there is a concept of decoding parameter set (DPS). This proposal is for the clarification of DPS, by adding constraint to the dps_decoding_parameter_set_id, or by improving the description of its property, depending on whether allowing a bitstream to contain more than one DPS NAL units.

Option 1. Add a constraint: All DPS NAL units with a particular value of dps_decoding_parameter_set_id in a bitstream shall have the same content.

Option 2: To allow at most one DPS NAL unit in a bitstream.

JVET-Q0147 AHG9: On JCCR signaling [K. Abe, T. Toma (Panasonic)]

This contribution proposes to skip signaling of sps_joint_cbcr_enabled_flag when ChromaArrayType is equal to 0.

JVET-Q0152 AHG9/AHG12: Miscellaneous HLS topics [M. Coban, V. Seregin, Y.-J. Chang, M. Karczewicz (Qualcomm)]

Item 2 of this contribution belongs to this category.

2) Move sps_seq_parameter_set_id syntax to the start of sequence parameter set signalling.

JVET-Q0155 AHG9: On separate colour plane coding [V. Seregin, B. Ray, A. K. Ramasubramonian, Y.-J. Chang, M. Coban, G. Van der Auwera, M. Karczewicz (Qualcomm)]

Item 2 of this contribution belongs to this category.

2. Since colour planes are coded as monochrome pictures without decoding dependencies between colour planes, it is proposed to modify signalling of enable flags for cross-components tools such as joint CbCr, adaptive color transform, palette and chroma BDPCM.

JVET-Q0265 Modifications to VVC Draft 7 [C. Auyeung, X. Li, X. Zhao, S. Liu (Tencent)]

Skip signalling of some chroma-specific syntax elements in the SPS based on the value of ChromaArrayType not equal to 0.

JVET-Q0329 AHG9: Syntax cleanup of chroma coding tools in 444 color format [R.-L. Liao, J. Chen, Y. Ye, J. Luo (Alibaba)]

In VVC draft 7, for a sequence with 4:4:4 color format, the three components can be separately processed as monochrome pictures by setting a SPS flag separate_colour_plane_flag to true. In this case, there is no chroma components for the sequence. However, the JCCR, chroma BDPCM, ACT flag are still signaled in SPS, which is redundant. Besides, the palette mode can be enabled for 4:4:4 color format with separate_colour_plane_flag equal to true but cannot be enabled for monochrome color format. In this contribution, it is proposed to signal the JCCR, chroma BDPCM, ACT and palette flag only when chroma components exist.

JVET-Q0520 AHG9: Cleanups on signaling for CC-ALF, BDPCM, ACT and Palette [Y. Wang, L. Zhang, K. Zhang, W. Zhu (Bytedance)]

In current VVC and the suggested specification for cross-component adaptive loop filter (CC-ALF), certain coding tools, including CC-ALF, chroma BDPCM, ACT, and Palette modes are only allowed under certain conditions. More specifically, CC-ALF should be enabled for non-4:0:0 colour format, chroma BDPCM, ACT and Palette mode are only enabled for 4:4:4 colour format without separate plane coding. However, the corresponding syntax elements and semantics are not well defined to reflect these restrictions. In this contribution, two aspects are proposed to cleanup the signalling for CC-ALF, and a third aspect is proposed to cleanup the signalling for BDPCM, ACT, and Palette.

Item 3 of this contribution belongs to this category.

3. BDPCM, ACT, and Palette mode for chroma are signalled under the condition of "ChromaArrayType is equal to 3".

JVET-Q0173 AHG9: On signalling the DPB parameters with delta values [C.-Y. Lai, C.-M. Tsai, T.-D. Chuang, O. Chubach, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

Signal the delta values of max_dec_pic_buffering_minus1[ ] and max_num_reorder_pics[ ] between two successive sublayers.

JVET-Q0176 AHG9: Overhead reduction for picture header [S.-T. Hsiang, Y.-W. Huang, S.-M. Lei (MediaTek)]

Item 2 of this contribution belongs to this category.

2. Skip coding mixed_nalu_types_in_pic_flag when the number of slices indicated by the PPS is equal to 1.

JVET-Q0210 [AHG9]: Miscellaneous HLS clean-ups [Hendry, S. Paluri (LGE)]

Items 8 and 9 of this contribution belong to this category.

8. Remove the use of uek(v).

9. Change the coding of bit_depth_minus8 and min_qp_prime_ts_minus4 from ue(v) to u(3) and u(6), respectively.

JVET-Q0285 AHG9: On adaptation parameter set ID [B. Choi, S. Wenger, S. Liu (Tencent)] [late]

Item 2 of this contribution belongs to this category.

Proposal 2: Replacement of u(5) with ue(v), to code adaptation_parameter_set_id, for future extensibility.

JVET-Q0374 AHG9: Cleanups on redundant signalling in HLS [D. Kim, G. Ko, J. Jung, J. Son, J. Kwak (WILUS)]

This contribution proposes cleanups on redundant signalling in the SPS.

1. The first proposal is to change the signalling condition of partition constraint parameters and CU QP delta related parameters. The maximum value of the above syntax element is decided based on the difference between MinCbSizeY(MinCbLog2SizeY) and CtbSizeY(CtbLog2SizeY). If the difference of MinCbSizeY and CtbSizeY is equal to 0, each of those syntax elements has only one value as 0. Therefore, it can be inferred to be equal to 0 without signalling.

2. The second proposal is to include the condition of either sps_palette_enabled_flag equal to 1 or sps_transform_skip_enabled_flag equal to 1 to signal min_qp_prime_ts_minus4 in the SPS. It is because this syntax element is only used for palette mode and transform skip mode.

JVET-Q0399 AHG8: On conformance window and scaling window [Y. Sanchez, R. Skupin, K. Sühring, T. Schierl (HHI)]

In the current VVC draft specification, the scaling ratio derivation for reference picture resampling and scalability support was changed from using the conformance window to using a scaling window that might be different to the conformance window.

In many cases both values are the same and therefore seems to be redundant information. This contribution proposes to add inference values of scaling window to be equal to the conformance window when the syntax elements are not present.

JVET-Q0416 AHG8/AHG9: On horizontal wrap-around motion compensation [J. Chen, Y. Ye, R.-L Liao, J. Luo (Alibaba)]

Item 1 of this contribution belongs to this category.

In VVC draft 7, sps_ref_wraparound_enabled_flag and sps_ref_wraparound_offset_minus1 are signaled in SPS for horizontal wrap-around motion compensation. There are two problems of these two syntax elements: 1) the minimum value sps_ref_wraparound_offset_minus1 signalled is a non-zero positive value that consumes more bits than it actually needs; 2) Two constraints are imposed for sps_ref_wraparound_enabled_flag and sps_ref_wraparound_offset_minus1, but the constraints are dependent on the picture width that is signalled in PPS and may change from one PPS to another PPS. This contribution is proposed to solve these two problems.

1. The first aspect of this contribution proposes to solve the first problem by subtracting ( CtbSizeY / MinCbSizeY ) + 2 from the wrap-around offset before signalling so that the minimum value of syntax element signaled in bitstream is 0. It can save unnecessary bits dedicated to the wrap-around offset signaling.

JVET-Q0420 AHG12: Signaling of chroma presence in PPS and APS [L. Li, X. Li, C. Auyeung, B. Choi, S. Wenger, S. Liu (Tencent)]

This contribution proposes chroma coding tool syntax element present flag in both PPS and APS to specify whether chroma component presents to simplify signaling while avoid parsing dependency between parameter sets.

- pps_chroma_coding_tool_syntax_elements_present_flag: specifies whether syntax elements for chroma coding tools are present or not in the PPS.

- aps_chroma_coding_tool_syntax_elements_present_flag: specifies whether syntax elements for chroma coding tools are present or not in the APS

JVET-Q0481 AHG9: Ordering of partition constraints syntax elements in the sequence parameter set and picture header [W. Wan, B. Heng (Broadcom)]

The SPS aspect of this contribution belongs to this category.

This contribution proposes to rearrange the ordering of the partition constraints syntax elements in the sequence parameter set and picture header. More specifically, it is noted that when there are similar syntax elements for intra and inter coded pictures, the typical convention has been to define the intra coded parameters before inter coded parameters. Currently, the mtt_hierarchy_depth syntax elements are defined in the reversed direction with inter coded parameters defined before intra coded parameters. It is claimed that this could be easy to overlook causing potential interop problems and there are no hardships to rearrange the syntax elements to be consistent with other syntax elements.

Additionally, it is proposed to group the partition constraint syntax elements by type (intra, inter, dual-tree chroma) to be consistent with how log2_diff_max_bt, log2_diff_max_tt, and the dual-tree chroma syntax elements are currently arranged.

The proposed SPS syntax change is as follows:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |
| sps_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| sps_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |
| if( sps_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |
| } |  |
| sps_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| sps_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| if( sps_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| sps_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| } |  |
| … |  |
| } |  |

# List of design questions

- Remove DPS ID from the DPS syntax, since the DPS is not allowed to change within a whole bitstream?

  - Yes. (JVET-Q0045, JVET-Q0117)

    - Either sepcify that the availabilty of the DPS indicatest that it is referred to the SPS, or change sps_decoding_parameter_set_id to sps_decoding_parameter_set_flag. (JVET-Q0045)

    - Specify DPS as a standalone NAL unit, named "decoding parameters NAL unit", and apply the following: (JVET-Q0117)

      - Require all decoding parameters NAL units in a bitstream to have the same content.

      - When present in a bitstream, a decoding parameters NAL unit shall be present in the first AU of the bitstream, and may be present in any AU that has at least one coded slice NAL unit having nal_unit_type in the range of IDR_W_RADL to GDR_NUT, inclusive (such a NAL unit is an IRAP or a GDR VCL NAL unit), and shall not be present in any other AU.

  - No, keep it.

    - It is required that all DPS NAL units with a particular value of dps_decoding_parameter_set_id in a bitstream shall have the same content. (JVET-Q0117, JVET-Q0357)

    - When present in a bitstream, a DPS NAL unit shall be present in the first AU of the bitstream, and may be present in any AU that has at least one coded slice NAL unit having nal_unit_type in the range of IDR_W_RADL to GDR_NUT, inclusive (such a NAL unit is an IRAP or a GDR VCL NAL unit), and shall not be present in any other AU. (JVET-Q0117)

    - To allow at most one DPS NAL unit in a bitstream. (JVET-Q0357)

- Disallow profile_tier_level( ) syntax structures included in the DPS to contain sublayer level information, by adding one more input argument for the PTL syntax structure to control whether there is sublayer level information signalled at all; this argument is 0 (i.e., no sublayer level information) for DPS and 1 for VPS and SPS. (JVET-Q0117)

- The following constraint is specified: Each OLS in a CVS in the bitstream shall confirm to at least one of the PTL syntax structures in the DPS (as part of the DPS semantics). Accordingly, a change is also made to the semantics of the PTL syntax structure regarding the OlsInScope: "When a profile_tier_level( ) syntax structure is included in a DPS, the OlsInScope is one or more unidentified OLSs in one or more unidentified CVSs in the bitstream." (JVET-Q0117)

- Move sps_seq_parameter_set_id to be the first syntax element in the SPS syntax now that it is u(4)-coded, to be aligned with the signalling position of the parameter IDs for all other parameter sets. (JVET-Q0117, JVET-Q0152)

- It is proposed to repeat ref_pic_resampling_enabled_flag and condition the picture width and height syntax elements as well as the scaling_window_flag based on the repeated PPS flag. (JVET-Q0117)

- It is proposed to add the following rule regarding the APS ID value space: All APS NAL units with a particular value of aps_params_type, regardless of the nuh_layer_id values, share the same value space for adaptation_parameter_set_id. APS NAL units with different values of aps_params_type use separate values spaces for adaptation_parameter_set_id. Consequently, the NOTE saying "Each type of APSs uses a separate value space for adaptation_parameter_set_id" can be removed. (JVET-Q0117)

- To make it clear that only one VPS is referred to by SPSs in a CVS, the following constraint is proposed: The value of sps_video_parameter_set_id shall be the same in all SPSs that are referred to by coded pictures in a CVS. (JVET-Q0280)

- The maximum level indicated in the DPS shall not less than the level indicated in a lower level parameter set. (JVET-Q0355)

- Condition sps_joint_cbcr_enabled_flag on "if(ChromaArrayType  !=  0 )". (JVET-Q0117, JVET-Q0147, JVET-Q0155, JVET-Q0265, JVET-Q0329)

- Condition sps_bdpcm_chroma_enabled_flag

  - on "if( sps_bdpcm_enabled_flag  &&  ChromaArrayType = = 3". (JVET-Q0155, JVET-Q0520)

  - on "if( sps_bdpcm_enabled_flag  &&  chroma_format_idc  = =  3  &&  ChromaArrayType  !=  0". (JVET-Q0265, JVET-Q0329)

- Condition sps_palette_enabled_flag on

  - "if( ChromaArrayType  = =  3 )". (JVET-Q0155, JVET-Q0520)

  - "if( chroma_format_idc  = =  3  &&  ChromaArrayType != 0 )". (JVET-Q0329)

- Condition sps_act_enabled_flag on

  - "if( ChromaArrayType  = =  3 )". (JVET-Q0155, JVET-Q0520)

  - "if( chroma_format_idc  = =  3  &&  ChromaArrayType != 0 )". (JVET-Q0265, JVET-Q0329)

- Signal the delta values of max_dec_pic_buffering_minus1[ ] and max_num_reorder_pics[ ] between two successive sublayers. (JVET-Q0173)

- Skip coding mixed_nalu_types_in_pic_flag when the number of slices indicated by the PPS is equal to 1. (JVET-Q0176)

- Remove the use of uek(v). (JVET-Q0201)

- Change the coding of bit_depth_minus8 and min_qp_prime_ts_minus4 from ue(v) to u(3) and u(6), respectively. (JVET-Q0201)

- Replacement of u(5) with ue(v), to code adaptation_parameter_set_id, for future extensibility. (JVET-Q0285)

- Change the signalling condition of partition constraint parameters and CU QP delta related parameters. The maximum value of the above syntax element is decided based on the difference between MinCbSizeY(MinCbLog2SizeY) and CtbSizeY(CtbLog2SizeY). If the difference of MinCbSizeY and CtbSizeY is equal to 0, each of those syntax elements has only one value as 0. Therefore, it can be inferred to be equal to 0 without signalling.

- Include the condition of either sps_palette_enabled_flag equal to 1 or sps_transform_skip_enabled_flag equal to 1 to signal min_qp_prime_ts_minus4 in the SPS. It is because this syntax element is only used for palette mode and transform skip mode.

- Remove "When ref_pic_resampling_enabled_flag is equal to 0, the value of scaling_window_flag shall be equal to 0." and inference the scaling window to be equal to the conformance window when scaling_window_flag is equal to 0. (JVET-Q0399)

- Subtract ( CtbSizeY / MinCbSizeY ) + 2 from the wrap-around offset before signalling so that the minimum value of the syntax element signalled in bitstream is 0. (JVET-Q0416)

- Add pps_chroma_coding_tool_syntax_elements_present_flag that specifies whether syntax elements for chroma coding tools are present or not in the PPS. (JVET-Q0420)

- Add aps_chroma_coding_tool_syntax_elements_present_flag that specifies whether syntax elements for chroma coding tools are present or not in the APS. (JVET-Q0420)

- Move sps_log2_diff_min_qt_min_cb_inter_slice and sps_max_mtt_hierarchy_depth_inter_slice to be just before sps_log2_diff_max_bt_min_qt_inter_slice in the SPS, thus the order of the partition constraints syntax elements in the SPS is rearranged to have the intra ones go before all the inter ones, and also the partition constraint syntax elements are grouped by type (intra, inter, dual-tree chroma) to be consistent with how log2_diff_max_bt, log2_diff_max_tt, and the dual-tree chroma syntax elements are currently arranged. (JVET-Q0481)