| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0764-v3 |
| --- | --- |

| Title: | AHG9: Text for combination of wrap around offset and RPR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi, Stephan Wenger, Shan Liu (Tencent) 2747 Park Blvd., Palo Alto, CA 94306 Brian Heng, Peisong Chen, Tim Hellman, Wade Wan, Minhua Zhou (Broadcom) 1320 Ridder Park Drive San Jose, California 95131 Miska M. Hannuksela (Nokia) Nokia Technologies, Tampere, Finland Yao-Jen Chang, Vadim Seregin, Muhammed Coban, Marta Karczewicz (Qualcomm) | Email: | bdchoi@tencent.com stewe@stewe.org shanl@tencent.com firstname.lastname@broadcom.com firstname.lastname@nokia.com yjchang@qti.qualomm.com |
| Source: | Tencent, Broadcom Inc., Nokia, Qualcomm |  |  |

_____________________________

# Abstract

Based on the HLS BoG meeting decision, the following changes for combination of wrap-around offset and reference picture resampling (RPR) to the current VVC spec. is proposed.

- Moving wrap-around offset syntax elements from SPS to PPS.

- Disabling the motion compensation with wrap-around offset for a reference picture, only when that picture requires scaling for RPR.

# Proposed changes to VVC

In order to enable the combination of wrap-around offset and reference picture resampling (RPR) in the VVC, the HLS BoG (led by Jill Boyce) tentatively agreed the following changes:

- Moving wrap-around offset syntax elements from SPS to PPS.

- Disabling the motion compensation with wrap-around offset for a reference picture, only when that picture requires scaling for RPR.

The motivation of the proposed changes is to fix a bug that the value of the wrap-around offset is based on the current picture width, but the offset is applied to the reference picture that may have a different picture size or/and different scaling window offsets than the current picture, when the RPR is used.

The BoG agreed to disable the motion compensation with the wrap-around offset, only when a reference picture requires scaling, so that the wrap-around motion compensation can work with the RPR when reference picture scaling is not required. Also, it is proposed to move the wrap-around offset syntax elements to PPS, to update the offset values when the picture size is updated in PPS.

The harmonized VVC specification text from JVET-Q0287, JVET-Q0335, JVET-Q0487 [1] and JVET-Q0134 is included as a separate document. JVET-Q0487 provides draft bug fix on reference picture scaling detection, which is critical to determine whether a reference picture requires scaling.

# References

- W. Wan, T. Hellman, B. Heng and P. Chen, “AHG8: RPR Scaling Window Issues”, JVET-Q0487, Brussels, BE, January 2020

# Patent rights declaration(s)

Tencent, Broadcom, Nokia and Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).