| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0782-v2 |
| --- | --- |

| Title: | CE5-related: High level syntax modifications for CCALF (combination of JVET-Q0253 and JVET-Q0520) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anand Meher Kotra, Semih Esenlik, Biao Wang, Han Gao, Elena Alshina Yang Wang, Li Zhang, Kai Zhang, Weijia Zhu Olena Chubach, Ching-Yeh Chen, Chen-Yen Lai, Yu-Wen Huang Nan Hu, Vadim Seregin, Marta Karczewicz Fangdong Chen, Liying Xu, Li Wang | Tel: Email: | anand.meher.kotra@huawei.com lizhang.idm@bytedance.com jenny.lai@mediatek.com nanh@qti.qualcomm.com chenfangdong@hikvision.com |
| Source: | Huawei Technologies Dusseldorf GmbH, Bytedance, MediaTek Inc., Qualcomm Inc, Hikvision. |  |  |

_____________________________

# Abstract

This contribution proposes to decouple the high level syntax control of Adaptive loop filter (ALF) and Cross component adaptive loop filter (CCALF) completely by introducing a new sequence level syntax element called “sps_ccalf_enabled_flag” and the corresponding new entries for CCALF in the picture header and slice header.  This decoupling of high level syntax control of ALF and CCALF reportedly provides encoders with better flexibility while encoding sequences with different signal characteristics. Version 2 of the contribution contains the feedback provided in Track B and the BoG recommendations from 11th January 2020

# Introduction

In the current design of CCALF [4], ALF and CCALF can be separately turned on or off for each slice. The syntax in the slice header is as shown below.

| if( sps_alf_enabled_flag && !pic_alf_enabled_present_flag ) { |  |
| --- | --- |
| slice_alf_enabled_flag | u(1) |
| if( slice_alf_enabled_flag ) { |  |
| slice_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < slice_num_alf_aps_ids_luma; i++ ) |  |
| slice_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| slice_alf_chroma_idc | u(2) |
| if( slice_alf_chroma_idc ) |  |
| slice_alf_aps_id_chroma | u(3) |
| } |  |
| if( ChromaArrayType != 0 ) |  |
| slice_cross_component_alf_cb_enabled_flag | u(1) |
| if( slice_cross_component_alf_cb_enabled_flag ) { |  |
| slice_cross_component_alf_cb_aps_id | u(3) |
| slice_cross_component_cb_filters_signalled_minus1 | ue(v) |
| } |  |
| if( ChromaArrayType != 0 ) |  |
| slice_cross_component_alf_cr_enabled_flag | u(1) |
| if( slice_cross_component_alf_cr_enabled_flag ) { |  |
| slice_cross_component_alf_cr_aps_id | u(3) |
| slice_cross_component_cr_filters_signalled_minus1 | ue(v) |
| } |  |

As depicted in table above, the syntax element slice_alf_enabled_flag only controls the ALF related syntax elements and not the CCALF syntax related elements. However the flag sps_alf_enabled_flag still controls the presence of CCALF related syntax elements in the picture header and slice header. The current proposal targets to completely decouple the signalling of ALF and CCALF related syntax elements and thereby allow encoders the flexibility to completely switch of CCALF at the sequence level.

# Proposed method

The proposal completely decouples the signaling of CCALF and ALF syntax elements by introducing a sequence level flag called “sps_ccalf_enabled_flag”.  Furthermore, the CCALF related syntax elements in the picture header and slice header are conditionally signalled based on the “sps_ccalf_enabled_flag”. This reportedly gives flexibility for encoders to simply switch off the CCALF at the sequence level and not signal any other CCALF related syntax elements in the picture header or slice header. The next section introduces the specification text changes.

Version 1 of the document was presented in the CE5 BoG which met on 11th Jan 2020 at 19:00. The following changes were requested to be made to the syntax:

- Remove pic_ccalf_enabled_present_flag

- Condition the signalling of CCALF related syntax elements by the pic_enabled_flag and slice_alf_enabled_flag in picture header and slice header respectively (i.e., proposed in JVET-Q0190)

- Consider the other HLS adoptions during the JVET-Q meeting related to signalling of ALF syntax elements (i.e., from JVET-Q0200 adoption in HLS track, moving present_flag from PH to PPS)

# Specification text changes

The specification text changes are provided on top of the CCALF specification text provided in JVET-Q0073[5], with newly added parts highlighted in green, and deleted text marked with red strikethrough.

7.3.2.3	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …… |  |
| sps_alf_enabled_flag | u(1) |
| if(sps_alf_enabled_flag && ChromaArrayType != 0 ) |  |
| sps_ccalf_enabled_flag | u(1) |
| ……. |  |

7.3.3.2	 General constraint information syntax

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| …. |  |
| no_alf_constraint_flag | u(1) |
| no_ccalf_constraint_flag | u(1) |

7.3.2.4	Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …. |  |
| alf_present_in_ph_flag | u(1) |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| ……. |  |

alf_present_in_ph_flag equal to 1 specifies the syntax elements for enabling ALF use may be present in the PHs referring to the PPS. alf_present_in_ph_flag equal to 0 specifies the syntax elements for enabling ALF use may be present in the slice headers referring to the PPS.

7.3.2.6	Picture header RBSP syntax

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| …….. |  |
| if( sps_alf_enabled_flag && alf_present_in_ph_flag) { |  |
| pic_alf_enabled_present_flag | u(1) |
| if( pic_alf_enabled_present_flag ) { |  |
| pic_alf_enabled_flag | u(1) |
| if( pic_alf_enabled_flag ) { |  |
| pic_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < pic_num_alf_aps_ids_luma; i++ ) |  |
| pic_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| pic_alf_chroma_idc | u(2) |
| if( pic_alf_chroma_idc ) |  |
| pic_alf_aps_id_chroma | u(3) |
| if( sps_ccalf_enabled_flag ) { |  |
| pic_cross_component_alf_cb_enabled_flag | u(1) |
| if( pic_cross_component_alf_cb_enabled_flag ) { |  |
| pic_cross_component_alf_cb_aps_id | u(3) |
| pic_cross_component_cb_filters_signalled_minus1 | ue(v) |
| } |  |
| pic_cross_component_alf_cr_enabled_flag | u(1) |
| if( pic_cross_component_alf_cr_enabled_flag ) { |  |
| pic_cross_component_alf_cr_aps_id | u(3) |
| pic_cross_component_cr_filters_signalled_minus1 | ue(v) |
| } |  |
| } /* end of sps_ccalf_enabled_flag */ |  |
| } /* end of pic_alf_enabled_flag */ |  |
| } /* end of (sps_alf_enabled_flag && alf_present_in_ph_flag) */ |  |
| …… |  |

7.3.7.1	General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| ……. |  |
| if( sps_alf_enabled_flag && !alf_present_in_ph_flag!pic_alf_enabled_present_flag ) { |  |
| slice_alf_enabled_flag | u(1) |
| if( slice_alf_enabled_flag ) { |  |
| slice_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < slice_num_alf_aps_ids_luma; i++ ) |  |
| slice_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| slice_alf_chroma_idc | u(2) |
| if( slice_alf_chroma_idc ) |  |
| slice_alf_aps_id_chroma | u(3) |
| if( sps_ccalf_enabled_flag) { |  |
| slice_cross_component_alf_cb_enabled_flag | u(1) |
| if( slice_cross_component_alf_cb_enabled_flag ) { |  |
| slice_cross_component_alf_cb_aps_id | u(3) |
| slice_cross_component_cb_filters_signalled_minus1 | ue(v) |
| } |  |
| slice_cross_component_alf_cr_enabled_flag | u(1) |
| if( slice_cross_component_alf_cr_enabled_flag ) { |  |
| slice_cross_component_alf_cr_aps_id | u(3) |
| slice_cross_component_cr_filters_signalled_minus1 | ue(v) |
| } |  |
| } /* end of sps_ccalf_enabled_flag */ |  |
| } /* end of slice_alf_enabled_flag */ |  |
| } /* end of (sps_alf_enabled_flag && !alf_present_in_ph_flag) */ |  |
| ….. |  |

The semantics of the newly introduced syntax elements are as follows:

sps_ccalf_enabled_flag equal to 0 specifies that the cross component adaptive loop filter is disabled. sps_ccalf_enabled_flag equal to 1 specifies that the cross component adaptive loop filter may be enabled.

no_ccalf_constraint_flag equal to 1 specifies that sps_ccalf_enabled_flag shall be equal to 0.  no_ccalf_constraint_flag equal to 0 does not impose such a constraint.

# Conclusion

ALF and CCALF high level syntax elements signalling are decoupled by introducing a new sequence level syntax element called “sps_ccalf_enabled_flag”. Furthermore the CCALF related syntax elements are conditionally signalled in the picture header and slice header based on the value of “sps_ccalf_enabled_flag”.  This reportedly provides encoders with better flexibility while encoding sequences with different signal characteristics.

It is suggested to adopt the proposed syntax changes for the CCALF tool.

# References

- Versatile Video Coding (Draft 7), Document JVET-P2001-vE, B. Bross, J. Chen, S. Liu,Y.K. Wang

- “JVET common test conditions and software reference configurations”, JVET-N1010

- Description of Core Experiment 5 (CE5): Cross Component Adaptive Loop Filtering, JVET-P2025

- CE5 Common Base: Cross Component Adaptive Loop Filter, JVET-Q0058

- CE5-2.1, CE5-3.1: Multiplication removal for cross component adaptive loop filter and  5x5 filter shape, JVET-Q0073, N.Hu, J. Dong, V. Seregin, M. Karczewicz

- BoG report on CE5 Related Contributions, JVET-Q0780, A. Segall, C.-Y. Chen

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Hikvision Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).