| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0794-v2 |
| --- | --- |

| Title: | BoG report of contributions on inter prediction and motion vector coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Haitao Yang Ru-Ling Liao | Email: | haitao.yang@huawei.com ruling.lrl@alibaba-inc.com |
| Source: | BoG coordinators |  |  |

_____________________________

Summary

The BoG is set up to review the 27 contributions on inter prediction an motion vector coding, as categorized into section 6.6 of JVET meeting notes.

Three sessions have been organized for the BoG meeting,

- Jan. 12, 1800 to 2200, on the category of AFFINE and MER.

- Jan. 14, 1000 to 1315, on the category of DMVR and BDOF, and Merge modifications.

- Jan. 14, 1445 to 1800, on the category of Weighted prediction, and Others.

JVET-Q0242 and JVET-Q0438 had been reviewed in track B and therefore were skipped in the BoG discussion.

Recommendations

Recommended adoptions

Normative change

JVET-Q0128 solution 2: Disable DMVR and BDOF when both luma WP and chroma WP are not applicable to a coding unit.

In the current design, DMVR is applicable only if luma WP is not applied. However, chroma WP would be disabled for a DMVR block even it is indicated to do so. It is claimed to be a bug by the proponent and confirmed by other experts. The problem is reported to be only in the spec. VTM SW does not have the bug.

JVET-Q0297 method 2: parallel merge estimation region (MER)

HEVC MER with two normative modifications,

- Same constraints applied to subblock merge list construction, spatial motion candidates are marked as unavailable during subblock merge list construction.

- HMVP is updated only for the CU locates at the bottom-right corner of a MER.

And one encoder only constraint,

- BT/TT splitting is constrained to guarantee that one CU is either entirely contained in a MER, or one CU contains integer number of MERs.

Bugfix

JVET-Q0368 method 2: shared context for mvp_l0_flag and mvp_l1_flag, and for ref_idx_l0 and ref_idx_l1.

In VTM-7.0, mvp_l0_flag and mvp_l1_flag are coded with a single context. While in WD7, these two syntaxes are coded with separate contexts. Same issue is detected for ref_idx_l0 and ref_idx_l1.

JVET-Q0370 method 1: shared context for coding the first bin of merge_idx, merge_geo_idx0, and merge_geo_idx1.

In the VTM-7.0, the first bin of merge_idx, the first bin of merge_triangle_idx0, and the first bin of merge_triangle_idx1 are coded with a single context model. While in WD7, these bins are coded with separate context models.

JVET-Q0483: add a clipping operation to the motion vector restored from the motion buffer.

A bug is reported in the current motion vector compression process. A restored motion vector may exceed the maximum range. To fix the bug, it is thus proposed to add a clipping operation in equation (611) in section 8.5.2.12 as

mvLXCol = Clip3(−131072, 131071, mvCol).

Editorial cleanup

JVET-Q0362 on DMVR specification cleanup

Suggest editors to consider the two editorial modifications.

- Revisits in track

JVET-Q0315 on TPM/GEO mode with motion vector difference

Motion vector refinement mechanism similar to MMVD is applied to the motion vectors of two partitions. It is implemented for TPM in the proposal with RD performance,

RA: -0.18%, 103%, 98%

LB: -0.29%, 105%, 100%

It is commented by the proponent that the proposed method can also be applied to GEO. But no results available by the time it was discussed.

Contributions by category

Affine motion compensation

JVET-Q0324 Simplification of MV derivation for affine chroma [S. H. Wang (PKU), X. Zheng (DJI), S. S. Wang, S. W. Ma (PKU)]

This contribution proposes to simplify the MV derivation process for chroma sub-block of an affine CU. (1) For 4:2:0 format, the motion vector of a chroma 4x4 sub-block is proposed to use only top-left luma sub-block’s MV; (2) For 4:2:2 format, the motion vector of a chroma 4x4 sub-block is proposed to use only the left luma sub-block’s MV.

RD performance is 0.04% for RA and 0.01% for LDB for 4:2:0 content.

On average no performance impact for 4:2:2 content, and the averaging operation of two motion vectors for each 4x4 chroma block is removed.

Because of the performance loss for the modification to 4:2:0 content, it is not considered for recommendation.

It is commented by two experts that the changes to 4:2:2 content dose not bring significant benefits.

JVET-Q0659 Crosscheck of JVET-Q0324 "Simplification of MV derivation for affine chroma" [G. Li (Tencent)] [late]

JVET-Q0525 Non-CE4: Support large rotation and flipping in affine and PROF [A. Aminlou, D. Naik (Nokia)] [late]

Current syntax for affine motion vector supports signalling large rotation (e.g. close to 90/180/270 degree) and flipping, but engine of affine motion compensation and PROF are not working optimally for these cases. In these cases, there is a big discontinuity in borders of predicted subblocks, and PROF increases this discontinuity furthermore. This contribution proposes two solutions to fix these problems. At the first step, large rotation and flipping are detected based on affine motion vectors, and eight processing modes are defined for the current block. Then, based on the processing mode, 1) during PROF, motion vector refinements are modified, and/or 2) after performing subblock-based motion compensation, samples of the motion compensated subblocks are rotated or flipped. Under common test condition, the overall bdrate for RA and LDB is 0.00% and 0.03%, respectively. In point cloud coding (PCC) use case, where YUV sequences are generated from point cloud data, -2.81% and -3.12% overall bdrate is achieved for RA and LDB, respectively. The method in this proposal does not have noticeable effect on encoding and decoding time.

This is a follow-up contribution of JVET-P0600. Notes taken during the BoG discussion in the previous meeting can be found in JVET-P0986.

Use case on point cloud compression is explained, where patches may be rotated and use for composing the picture for coding. The proposed methods is claimed to be able to handle large rotation more efficiently.

It is asked how many orientations in the packed coding picture in point cloud coding. It is said to be two, the original one the 90 degree rotation.

On average no coding gain is shown in JVET CTC test sequences.

On average -2.81% for RA and -3.12% for LDB is achieved using PCC content.

JVET-Q0662 Cross-check of JVET-Q0525 Non-CE4: Support large rotation and flipping in affine and PROF [D. Liu (Ericsson)

JVET-Q0431 Interaction between Affine and SBT [F. Le Léannec, K. Naser, T. Poirier, P. Bordes (InterDigital)]

This contribution proposed to deactivate the Sub Block Transform (SBT) in case of a CU coded in Affine Merge mode. More precisely, SBT is disallowed in case of sub-block merge mode when the merge candidate used is different from SbTMVP.

The proposed mutual exclusion reportedly leads to an average BD rate change of +0.01% in JVET CTC, in random access and 0.00% in low delay configurations. Encoding time is 98% in random access.

A second proposed method 2 deactivates SBT for all affine modes, leading +0.04% BD rate increase both in RA and LDB.

It is commented that an encoder can choose not checking SBT if the complexity a concern.

The encoder only modification of disabling SBT for affine merge mode is running, and no full results are available by the time of the presentation.

It is commented that the implementation does not cover a case where affine merge mode is the first candidate in the subblock merge list, and as a results SBT is occasionally selected for affine merge mode in method 1.

JVET-Q0666 Cross-check of JVET-Q0431 (Interaction between Affine and SBT) [A. Nalci (Qualcomm)] [late]

Parallel merge region

JVET-Q0185 AHG16: On merge estimation region for VVC [Y.-L. Hsiao, C.-C. Chen, C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek), H. Huang, W.-J. Chien, T. Hsieh, V. Seregin, C.-C. Chen, K. Reuzé, M. Karczewicz (Qualcomm)]

Merge estimation region (MER) is included in HEVC. MERs are non-overlapping square regions, and MER is used at the encoder side to estimate costs of merging candidates in parallel for different coding units (CUs) in one MER. This contribution proposes to add MER to VVC. The MER for VVC is achieved with minor normative changes with proper encoder-only constraints. Results are summarized as follows.

Proposed method with MER size equal to 8x8
VTM7.0-RA: {Y-BD-rate=1.29%, EncT=80%, DecT=98%}
VTM7.0-LB: {Y-BD-rate=1.67%, EncT=81%, DecT=97%}

Proposed method with MER size equal to 16x16
VTM7.0-RA: {Y-BD-rate=3.08%, EncT=62%, DecT=98%}
VTM7.0-LB: {Y-BD-rate=3.82%, EncT=63%, DecT=98%}

Proposed method with MER size equal to 32x32
VTM7.0-RA: {Y-BD-rate=3.23%, EncT=67%, DecT=96%}
VTM7.0-LB: {Y-BD-rate=3.82%, EncT=68%, DecT=97%}

Proposed method with MER size equal to 64x64
VTM7.0-RA: {Y-BD-rate=3.18%, EncT=101%, DecT=95%}
VTM7.0-LB: {Y-BD-rate=3.82%, EncT=99%, DecT=97%}

Additional results:

Encoder-only method of skipping inter merge modes when the current CU is smaller than 32x32 for luma
VTM7.0-RA: {Y-BD-rate=4.69%, EncT=93%, DecT=96%}
VTM7.0-LB: {Y-BD-rate=7.52%, EncT=88%, DecT=95%}

Encoder-only method of skipping inter merge modes when the current CU is smaller than 64x64 for luma
VTM7.0-RA: {Y-BD-rate=7.00%, EncT=96%, DecT=93%}
VTM7.0-LB: {Y-BD-rate=10.55%, EncT=90%, DecT=96%}

HEVC MER is proposed, with additional modifications shown below

- Same constraints applied to subblock merge list construction, spatial motion candidates are marked as unavailable.

Two encoder only constraints

- BT/TT splitting is constrained to make sure one CU is entirely contained in a MER, or one CU contains integer number of MERs.

- HMVP candidates, pairwise candidates and zero candidates are not selected.

It is asked what would happen if an encoder does not follow the proposed encoder-only non-normative constraints. It is claimed by the proponent that the decoder can work properly in this case.

It is claimed by several experts that this feature is important for an encoder for parallel processing.

It is noticed that the RD performance difference between the proposed method and an encoder only implementation is not significant for sequences of large resolution.

It is commented that the performance gap may depend on the level encoder optimization. E.g, the first CU inside a MER can do merge.

It is commented that the performance benefits may be larger if the same encoder constraint is applied. And the encoder only constraint is required for an encoder only parallel merge mechanism.

The parallel merge in HEVC was proposed in JCTVC-H0082.

With the proposed method, HMVP table is updated within a MER, but the HMVP candidates are not selected by the encoder.

It is commented that the cost for supporting the feature is two right shifts and two comparisons for each spatial merge candidate of each coding unit, which is not high.

It is agreed that the feature of MER is desirable.

Comparison of the complexity of four options

Option 1 (Q0185/Q0356 method 1): 2 shifts and 2 comparisons for each spatial merge candidate

Option 2 (Q0297 method 1): 2 shifts and 2 comparisons for each spatial merge candidate, and 2 comparisons for each CU

Option 3 (Q0297 method 2): 2 shifts and 2 comparisons for each spatial merge candidate, 2 additions and 2 comparisons for each CU

Option 4 (Q0356 method 2): 2 shifts and 2 comparisons for each spatial merge candidate, 2 additions and 2 comparisons for each CU

It is commented that the complexity introduced by HMVP update is not substantial.

It is commented that option 3 is slightly preferable by two experts.

Recommendation: adopt option 3.

JVET-Q0640 Crosscheck of JVET-P0185 (AHG16: On merge estimation region for VVC) [S. Esenlik (Huawei)] [late]

JVET-Q0297 AHG16: Merge estimation region [H. Huang, W.-J. Chien, T. Hsieh, V. Seregin, C.-C. Chen, K. Reuze, M. Karczewicz (Qualcomm)]

In addition to Q0185, additional normative changes are proposed. When a CU is smaller than the MER,

- Method 1: No update of HMVP table if CU is smaller than MER.

- Method 2: HMVP is updated only for the CU locates at the bottom-right corner of a MER.

Considering the case MER size is 32x32, the performance improvement compared against Q0185 is about 0.6% for method1 and about 0.8% for method 2 for RA configuration.

Spec change for both methods are confirmed.

See notes in Q0185.

JVET-Q0654 Cross-check of JVET-Q0297 (AHG16: Merge estimation region with constrain in HMVP update) [Y.-W. Chen (Kwai Inc.)] [late]

JVET-Q0356 AHG16: Parallel Merge Estimation for VVC [S. Esenlik, H. Gao, B. Wang, A. M. Kotra, E. Alshina (Huawei), Y.-L. Hsiao, C.-C. Chen, C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (Mediatek)]

The discussion regarding this proposal is chaired by R.-L. Liao.

Based on Q0185, normative change is made.

The difference between Q0297 and Q0356 is only happened when the non-normative block constraint is not applied.

It is commented that the complexity of these two proposals are the same as each other.

It is confirmed that the results in method 2 in Q0297 is bit exact with method 2 in Q0356. The logics are not the same.

Method 1 in Q0356 is the same as Q0185.

See notes in Q0185.

JVET-Q0583 Crosscheck of JVET-Q0356 (AHG16: Parallel Merge Estimation for VVC) [B. Bross (HHI)] [late]

DMVR and BDOF

Q0128, Q0362, Q0455, Q0522, Q0524

JVET-Q0128 On clarification of applicable conditions of DMVR and BDOF [T. Chujoh, T. Hashimoto, E. Sasaki, T. Ikai (Sharp)]

One of the applicable conditions of DMVR and BDOF is that luminance coefficients and offsets for weighted prediction do not exist for both L0 and L1 reference indexes. However, when any luminance coefficient does not exist and some chroma coefficients exist, the applicable conditions of DMVR and BDOF are claimed to be not clear. It is asserted that there are bugs of both text and software. In this contribution, add a condition that chroma coefficients and offsets for weighted prediction do not exist for both L0 and L1 reference indexes. The changes are to add only one line to the applicable condition of both DMVR and BDOF. There is no change of experimental results on CTC. There were negligible differences for fade sequences when WeightedPredB on VTM-7.0 is set to 1. If the chroma weighted prediction is not constrained, the bugs of text and software should be fixed.

In the current design, DMVR is applicable only if luma WP is not applied. However, chroma WP would be disabled for a DMVR block even it is indicated to do so. It is claimed to be a bug by the proponent and confirmed by other experts. The problem is reported to be only in the spec. VTM SW does not have the bug.

Solution 2 proposes to enable DMVR and BDOF only when both luma WP and chroma WP are not applicable to a coding unit.

No performance impact for solution 2.

Recommendation: adopt solution 2.

JVET-Q0362 Non-normative aspects for cleanup of DMVR specification and software [S. Esenlik, B. Wang, A. M. Kotra, H. Gao, E. Alshina (Huawei), J. Arumugam, S. Kotecha, S. Ramamurthy (Ittiam)]

The discussion regarding this proposal is chaired by R.-L. Liao.

Two non-normative improvements for the VVC draft 7 and one bugfix for VTM7.0 software have been proposed corresponding to the description and implementation of DMVR. Furthermore DMVR external memory access measurement results have been reported as information in response to the AhG request from previous meeting, which recommended fixing of a bug in the memory bandwidth measurement tool.

It is proposed to clean up the DMVR specification.

First aspect is purely editorial changes.

Second aspect is also purely editorial changes.

Third aspect is the same as JVET-Q0438 (fix the monochrome processing).

Recommendation: editors can consider 1st and 2nd changes (purely editorial)

JVET-Q0589 Crosscheck of JVET-Q0128 (On clarification of applicable conditions of DMVR and BDOF) [P. Bordes (InterDigital)] [late]

JVET-Q0455 On CU-level BDOF enable condition [S. Wan, Y. Xue (NPU), J.-Y. Huo, Y.-Z. Ma, F.-Z. Yang (Xidian Univ.), Y.-F. Yu, Y. Liu (OPPO)]

This contribution proposes a CU-level BDOF enable restriction based on the directions of forward and backward motion vectors. Besides the current BDOF enable condition in VVC, it is proposed to add an enable restriction as sign(mvL0[ xSbIdx ][ ySbIdx ][0]) * sign(mvL1[ xSbIdx ][ ySbIdx ][0]) not larger than zero or sign(mvL0[ xSbIdx ][ ySbIdx ][1]) * sign(mvL1[ xSbIdx ][ ySbIdx ][1]) not larger than zero. In this design, unnecessary BDOF can be skipped. Experimental results report that the BD-rate change in Y, Cb and Cr are 0.00%, -0.01% and 0.01%, respectively.

It is commented that the chance of applying BDOF would be decreased by the proposed method.

It is reported that the decrease of BDOF usage is less than 1%.

No performance impact by the proposed method.

JVET-Q0613 Crosscheck of JVET-O0455: On CU-level BDOF enable condition [J. Chen (Alibaba)] [late]

JVET-Q0522 Non-CE4: Early termination of DMVR and study of its pixel coverage [C.-C. Chen, H. Huang, W.-J. Chien, M. Karczewicz (Qualcomm)]

This contribution proposes to early-terminate DMVR at CU level when the respective lengths of the given bi-directional motion vectors (MVs) in a coding unit are both small than a given threshold value. Experimental results were conducted on top of VTM-7.0. The report shows that when the proposed method is applied to DMVR with the SAD-based early termination used in VTM, the average BD-rate change observed in Test C8 is less than ±0.01%, with 1.0% (when threshold value is 1) to 4.4% (when threshold value is 8) reduction of DMVR pixel coverage on average and less than 1% of decoding-time fluctuation. In addition, the MV-based method is tested alone with the SAD-based method disabled. The result in Test B8 shows that the MV-based method reduces the use of DMVR more in Class C/D/F than in other classes of CTC sequences, when compared with SAD-based method. In Test B8, the use of DMVR reduces from 32.4% to 23.9% for Class C, from 38.4% to 26.6% for Class D and from 13.6% to 9.7% for Class F. The average pixel coverage of DMVR for each test is summarized as follows:

- (VTM-7.0) 25.1%/35.0%/28.4%/32.4%/38.4%/13.6% for Class A1/A2/B/C/D/F (SAD on);

- (Test A)   34.9%/46.6%/38.3%/38.2%/43.5%/40.6% for Class A1/A2/B/C/D/F (SAD off);

- (Test B1)  32.5%/45.3%/36.2%/34.8%/39.2%/13.3% for Class A1/A2/B/C/D/F (SAD off + proposed with TH=1);

- (Test B8)  31.8%/42.8%/32.6%/23.9%/26.6%/9.7% for Class A1/A2/B/C/D/F (SAD off + proposed with TH=8).

- (Test C1)  24.6%/34.7%/27.8%/30.0%/35.0%/10.7% for Class A1/A2/B/C/D/F (SAD on + proposed with TH=1);

- (Test C8)  24.2%/33.7%/25.8%/20.7%/24.0%/8.0% for Class A1/A2/B/C/D/F (SAD on + proposed with TH=8).

JVET-Q0705 Crosscheck of JVET-Q0522 (Non-CE4: Early termination of DMVR and study of its pixel coverage) [H.-J. Jhu (Kwai Inc.)] [late]

JVET-Q0524 Non-CE4: DC balancing in DMVR [A. Aminlou, M. Homayouni (Nokia)] [late]

Decoder-side motion vector refinement (DMVR) refines the accuracy of motion vectors of merge mode by minimizing the sum of absolute difference (SAD) between the prediction blocks in reference picture lists L0 and L1. The refined MV might not be accurate in the case that two prediction blocks are not in the same level of brightness, for example in the case of fading scenes or lightening condition change in different frames. It is proposed to balance the DC component of two prediction blocks for SAD calculation. For this purpose, DC value of prediction blocks of L0 and L1 is calculated, and then delta DC value is subtracted from sample differences during SAD calculation. This modification stabilizes the functionality of DMVR in different situations. In the case of RA, the average BD-rate gain is reported to be 0.10% (0.17% gain for class A1 sequences). Additional non-CTC results shows that in the case of illumination change, DMVR not only provide gain, but also shows 0.07% loss (0.25% loss for Class A1), while the proposed method stabilize RD performance of DMVR and retains its original gain.

It is proposed to use mean-removed SAD instead of the regular SAD for the motion refinement of DMVR.

Two methods are proposed to calculate delta DC of the two predictions.

Method 2 proposed to calculate delta DC for the search points at four corners of the 5x5 search pattern and interpolate the delta DC for the other search points.

On average 0.09% coding gain for both methods. Encoder run-time is increased by 2%.

Generated fading sequences are used for simulation. Around 1% coding gain can be achieved.

It is commented that in case of illumination change, WP provide huge gain. However, WP and DMVR are mutually exclusive for a CU. WP is preferred in this case compared with DMVR and therefore the modification to DMVR to adapt to illumination may not be necessary.

It is commented that mean-removed SAD the same as the method 1 has been proposed before and was not considered because the trade-off of coding gain and implementation cost is not satisfactory.

It is commented that method 2 uses only four corner points for calculating the delta DC which is not friendly for HW implementation because of irregularity.

JVET-Q0675 Crosscheck of JVET-Q0524 (Non-CE4: DC balancing in DMVR) [K. Zhang (Bytedance)] [late]

Merge modifications

SbTMVP Q0313, Q0327, Q0389, Q0390

Miscellaneous Q0105, Q0306, Q0340, Q0350, Q0496, Q0337, Q0338, Q0601

JVET-Q0105 nonCE4: BCW SIF Derivation for Pairwise Candidate [A. Robert, T. Poirier, F. Le Léannec (InterDigital)]

The merge pairwise candidate derives SIF flag and BCW index with two distinct methods in the VTM-7.0. This contribution proposes to unify these derivations by setting both to default or by inheritance from the input candidates.

For the first proposed method, some luma BD-rate loss is observed in RA (0.01%) and some gain is obtained in LDB (-0.01%). But for the second proposed method, the BD-rate performances are the same as VTM-7.0 in RA and some gain is obtained in LDB (-0.02%). The encoding and decoding times are unchanged compared to VTM-7.0 with both methods.

Method 2 proposes one additional comparison of BCW index when constructing the pairwise candidate and decide the BCW index for the candidate.

BD-rate for method 2 is 0.00% for RA and -0.02 for LDB on average.

JVET-Q0558 Crosscheck of JVET-Q0105 "BCW SIF Derivation for Pairwise Candidate" [G. Li (Tencent)] [late]

JVET-Q0306 Non-CE4: Removal of MVD scaling process for 4x8/8x4 blocks in MMVD [N. Zhang, H. Liu, L. Zhang, K. Zhang, Y. Wang (Bytedance)]

This contribution proposes to remove the MVD scaling process in bi-predicted MMVD mode for 4×8/8×4 blocks. Specifically, if the base merge candidate for MMVD utilizes bi-prediction for a 4×8/8×4 block, the signaled MVD is directly used as the MVD of reference picture list 0. Simulation results reportedly show that the BD-rate changes are 0.00% for RA, and 0.03% for LB with almost no impact on encoding/decoding time compared to VTM-7.0.

It is commented that the claimed benefits of saving one MV scaling in the case MMVD is applied to a 4x8 or 8x4 block is not significant and does not worth consideration.

JVET-N0240 proposed similar method to the same job, saving one MV scaling in the case MMVD is applied to a 4x4 block since it is enforced to use uni-prediction for 4x4 blocks at the time it was discussed. It is commented that the change seems reasonable. However, it was discussed in BoG and in track and was not considered to be a necessary change.

JVET-Q0635 Crosscheck of JVET-Q0306 (Non-CE4: Removal of MVD scaling process for 4x8/8x4 blocks in MMVD) [N. Park (LGE)] [late]

JVET-Q0313 Non-CE4: Constraints on block size for ATMVP [Y. Sun, F. Chen, L. Wang (Hikvision)]

Current ATMVP applies on blocks which height or width not less than 8, and was put in the first place in sub-block merge list if it is available after checking the related spatial and temporal motion information. However, after the adoption of remove 4x4 sub-blocks for ATMVP mode. For an 8x8 block, the ATMVP is no longer a sub-block mode which has only one sub-block. This makes 8x8 ATMVP mode degenerates into a regular temporal merge mode. This contribution presents size constrains on ATMVP mode to remove 8x8 ATMVP candidate when construct sub-block merge list, only one line modification in the spec. After applying this modification, the spatial and temporal check is no longer needed for 8x8 block’s sub-block merge list construction. The test results reportedly show that apply this modification won’t influence the coding performance of ATMVP.

RA: 0.01% (Y), 0.01% (U), 0.03% (V), 100% (EnT), 100% (DeT)

LD: 0.00% (Y), -0.01% (U), -0.13% (V), 100% (EnT), 99% (DeT)

It is proposed to remove the SbTMVP candidate from the merge list 8x8 blocks.

It is commented the same method was proposed in previous meetings and was not considered necessary.

It is commented that treating 8x8 blocks differently when constructing merge list does not simplify the current design.

It is commented that in case a second TMVP candidate is generated, it is not the same as the current TMVP candidate most of the cases and may be desirable. Even if duplicated TMVP candidates are generated signalling it at the first place of the subblock merge list may provide a shortcut for TMVP and may potentially get some tiny benefits.

JVET-Q0532 Crosscheck of JVET-Q0313 (Non-CE4: Constraints on block size for ATMVP) [H. Chen (Huawei)] [late]

JVET-Q0315 Non-CE4: Triangular prediction mode with motion vector difference [K. Zhang, L. Zhang, H. Liu, Z. Deng, Y. Wang (Bytedance)]

The discussion regarding this proposal is chaired by R.-L. Liao.

In this contribution, Triangular prediction mod with Motion Vector Difference (TMVD) is proposed. With TMVD, the MV of a triangular partition in TPM is refined as a sum of a MV derived from a merge candidate and a signalled MVD. The MVD is signalled in a similar way as MMVD, wherein indices of a direction and a distance are coded. Simulation results reported as below:

RA: -0.18%, 103%, 98%

LB: -0.29%, 105%, 100%

It is proposed to add a MVD to the motion vector of triangular prediction mode.

It was asked whether it can be applied to GEO. No results are available now. It is estimated that the coding gain may be higher if the proposed method is applied to GEO.

Similar idea was proposed in JVET-M0736. It is reported that the coding gain of JVET-M0736 is similar to this proposal. However, the encoding time of JVET-M0736 increases significantly.

Recommendation: Add this test to CE if there is a CE.

JVET-Q0691 Crosscheck of JVET-Q0315: Non-CE4: Triangular prediction mode with motion vector difference [J. Ström (Ericsson)] [late]

JVET-Q0327 On IBC/ATMVP candidate list construction [X. W. Meng, S. H. Wang (PKU), X. Zheng (DJI), S. S. Wang, S. W. Ma (PKU)]

In this contribution, IBC and ATMVP candidate list construction process is modified to better harmonize with normal merge candidate list. As the first two merge candidates are swapped in the current VTM-7.0, we propose to swap the first two IBC merge candidate and IBC inter candidate. And change the derivation process of motion shift in ATMVP to only check the top block of the current block.

Test 1 proposes to swap the order of A1 and B1 candidates in IBC merge list.

This brings some coding gain for screen content, -0.11% for Class F and -0.05% for Class TGM, in LDB case.

Test 2 proposes to check B1 instead of A1 for offset vector for SbTMVP.

It is commented that changing the order of merge candidates, if not for significant coding gain or other real benefits, is not desirable.

The motion vector in B1 location is of higher probability to be available when processing the current block. And therefore the chance an encoder need to wait for A1 to be available is decreased, which is good for parallel processing at the encoder side.

It is commented that with the adoption of MER, encoder parallel processing has already been handled in case it is needed.

It is commented that whether A1 or B1 should be checked for offset vector had been discussed in Marrakech meeting and it is decided to use A1. It is commented that is because A1 is the first candidate in the merge list.

Q0389 and Q0498 method 1 proposed the same method as the test 2 in this contribution.

JVET-Q0612 Crosscheck of JVET-Q0327: On IBC/ATMVP candidate list construction [J. Chen (Alibaba)] [late]

JVET-Q0337 Non-CE4: On weight values of the chroma-component for TPM mode [N. Park, J. Nam, H. Jang, J. Lim, S. Kim (LGE)]

No need for presentation because TPM is not a part of the design.

JVET-Q0540 Crosscheck of JVET-Q0337 (Non-CE4: On weight values of the chroma-component for TPM mode) [T. Chujoh (Sharp)] [late]

JVET-Q0338 CE4-related: Harmonized conditions for CIIP and GEO [N. Park, J. Nam, H. Jang, J. Lim, S. Kim (LGE)]

In the previous meeting, GEO mode was proposed and common base code for CE was provided. In the common base, the several conditions to apply the GEO mode were introduced and additional block size restriction was proposed in CE4-2.1. These constraints contributes to the number of the parsing conditions for CIIP and GEO becomes larger. In this contribution, simplified parsing conditions are proposed by aligning with conditions to the regular merge mode, CIIP mode and GEO mode. The experimental results reportedly show 0.02%, 0.06% BD-rate changes with no encoding and decoding run-time changes compared to VTM7.0 with CE4-2.1 as anchor in RA, LB configurations. According to the adoption of the 64x8 and 8x64 block size reduction, additional restriction is tested.

It is commented that the proposed method disallows CIIP for 4xN and Nx4 CU. On average 0.02% and 0.06% loss is observed for RA and LDB.

It is commented that the complexity depends on the number of samples in one CU, regardless whether it is 8x8 or not. And therefore the proposed method cannot improve the throughput bottleneck in some of the implementations.

JVET-Q0340 Non-CE4: Cleanup of the MMVD offset derivation [N. Park, J. Nam, H. Jang, J. Lim, S. Kim (LGE)]

In this contribution, cleanup of the MMVD offset derivation process is proposed. When the current picture refers short-term reference pictures for both directions and distance between current picture and reference picture of each direction is different, mmvd offset is scaled. However, since the maximum value of the scaled MVD cannot exceed the range of the MVD, clipping operation is not necessary. The experimental results reportedly show no changes in coding performance and run-time compared to VTM7.0 as anchor in CTC.

This seems like an editorial issue. Since the derived MVD for MMVD will not exceed the range and therefore clipping operation will not be called.

It is commented that an implementer can skip the clipping operation anyway, no harm to keep the clipping.

Revisit after having opinions from editors.

JVET-Q0350 Non-CE: Enable CIIP for 4x8/8x4 block without SCIPU violation [H. Jang, J. Nam, N. Park, S. Kim, J. Lim (LGE)]

In this contribution, it is proposed to allow CIIP for 4x8/8x4 blocks, and the corresponding chroma block, which is smaller than SCIPU, uses only inter prediction block without chroma intra prediction in order to keep SCIPU restriction. The experimental results show 0.02%, 0.03% BD-rate saving in CTC RA and LDB configuration, respectively.

It is commented that small block intra prediction is not preferred since the average and the worst case throughput burden would be increased. Considering the coding gain, may not be a good trade off.

JVET-Q0600 Crosscheck of JVET-Q0350 (Non-CE: Enable CIIP for 4x8/8x4 block without SCIPU violation) [K. Panusopone (Nokia)] [late]

JVET-Q0389 Non-CE4: SbTMVP harmonization [D. Liu, C. Hollmann, R. Yu (Ericsson)]

See notes in Q0327.

JVET-Q0562 Crosscheck of JVET-Q0389 (Non-CE4: SbTMVP harmonization) [Y. Sun, F. Chen, L. Wang (Hikvision)] [late]

JVET-Q0390 Non-CE4: SbTMVP simplification [D. Liu, C. Hollmann, R. Yu (Ericsson)]

In VTM-7.0, the SbTMVP uses the motion information of a neighboring candidate block to derive the motion information of sub-CUs. Currently, the 8-tap interpolation filter is used in 8x8 subblocks of SbTMVP, whereas the 6-tap filter is used for 4x4 subblocks of affine CUs. This contribution proposes to use the 6-tap interpolation filter in SbTMVP. This aligns all the subblocks to use the 6-tap interpolation filter and reduces the memory bandwidth of SbTMVP by 25%.

Slight performance loss is observed, on average 0.02% for RA and 0.04% for LDB.

It is commented that the worst case memory bandwidth is not changed, though the average memory bandwidth may be saved.

JVET-Q0496 AHG16: On motion shift derivation of SbTMVP [Y.-W. Chen, X. Xiu, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai Inc.)]

The construction of subblock-based TMVP (SbTMVP) requires the derivation of motion shift from the left neighboring block (A1), which causes latency issue for hardware encoder. In fact, the left neighboring block (A1) is the worst choice among all the immediate neighboring blocks for that purpose.

In this contribution, it is proposed to obtain the motion shift from alternative positions to make the design hardware friendly. In one proposed method, the motion shift is derived from the above neighboring block (B1). In another proposed method, the motion shift is derived from a block outside a predefine regions such as a 64x64 VPDU. The first modification introduces 0.01% (RA) and -0.01%(LDB) BD-rate changes, and the second modification introduces 0.03% (RA) and 0.02%(LDB) BD-rate changes under CTC.

Method 2: In the current MER design, A1 position of a CU is not available for getting the offset vector for SbTMVP. With the proposed method, A1 position of the MER/VPDU containing the current CU is used for this purpose. It seems like an improvement of MER mechanism. No results for MER configuration.

JVET-Q0601 Non-CE4: Extension of JVET-P0325 [A. Robert, T. Poirier, F. Le Léannec (InterDigital)] [late]

It is clarified by the proponent that this proposal is similar to Q0327. See notes in Q0327.

Weighted prediction

JVET-Q0129 On editorial improvements for specification of explicit weighted prediction [T. Chujoh, T. Hashimoto, E. Sasaki, T. Ikai (Sharp)]

The discussion regarding this proposal is chaired by R.-L. Liao.

In this contribution, an editorial problem with specification of explicit weighted prediction is pointed out and three options to solve it are proposed. In the current WD7, if weighting factors in explicit weighted prediction are not present, the default weighting factors of explicit weighted prediction are used even though it is possible to be performed in default weighted prediction. In order to solve it, it is necessary to improve the decision condition to switch between default weighted prediction and explicit weighted prediction correctly. Three different options to solve its are shown. First solution is to change the decoding process of explicit weighted prediction. These solutions are only editorial changes. Second solution is to change the specification of variable weightedPredFlag which switches the decoding process of default weighted prediction and explicit weighted prediction. Third solution is to define a new decision function for switching explicit weighted prediction. The bitstream and the results of encoding and decoding are not changed.

Three methods proposed in this document are purely editorial.

It is commented that the changes make the specification more complicated.

No action.

JVET-Q0266 On TPM and GEO merge modes in presence of weighted prediction [A. Filippov, H. Chen, V. Rufitskiy, H. Yang, E. Alshina (Huawei)]

The discussion regarding this proposal is chaired by R.-L. Liao.

The proposal addresses the current VVC design issue related to how TPM merge mode is used in presence of slice-level weighted prediction. As requested at the previous JVET meeting, examples of subjective artifacts caused by this issue are provided. 3 mechanisms to solve the claimed problem are proposed. The 1st one disables TPM merge mode in such slices where weighted prediction is enabled. For the fade-to-black scenario, the simulation results for this method reportedly show the Y/U/V BD-rate changes of 0.16%/0.37%/0.33% and 0.40%/0.56%/0.53% in RA and LB cases, respectively. For the fade-to-white scenario, the simulation results for this method reportedly show the Y/U/V BD-rate changes of 0.18%/0.17%/0.13% and 0.31%/0.36%/0.63% in RA and LB cases, respectively. The 2nd one introduces a CU-level conformance constraint to disable the TPM coded block referring to the picture with weighted prediction factor. The 3rd mechanism enables deblocking filtering over the boundaries where blocking artifacts caused by this given design flaw can appear. For the fade-to-black scenario, the simulation results for this method reportedly show the Y/U/V BD-rate changes of 0.00%/0.02%/-0.03% and -0.03%/-0.25%/0.07% in RA and LB cases, respectively. For the fade-to-white scenario, the simulation results for this method reportedly show the Y/U/V BD-rate changes of 0.08%/ - 0.03%/0.01% and -0.16%/-0.25%/-0.22% in RA and LB cases, respectively. In these tests, VTM-7.1 was used as the anchor. Additionally, 1st and 3rd mechanisms were tested for GEO, using the common base software of Core Experiment 4 as an anchor. Applied to GEO, results for the 1st mechanism could be reportedly summarized as follows. For the fade-to-black scenario, the simulation results for this method reportedly show the Y/U/V BD-rate changes of 0.27%/0.36/0.48% and 0.96%/0.79%/1.33% in RA and LB cases, respectively. For the fade-to-white scenario, the simulation results for this method reportedly show the Y/U/V BD-rate changes of 0.16%/0.17%/0.32% and 0.72%/0.48%/0.46% in RA and LB cases, respectively. Applied to GEO, the results for 3rd mechanism could be reportedly summarized as follows. For the fade-to-black scenario, the simulation results for this method reportedly show the Y/U/V BD-rate changes of 0.04%/0.03/-0.03% and 0.03%/-0.18%/0.28% in RA and LB cases, respectively. For the fade-to-white scenario, the simulation results for this method reportedly show the Y/U/V BD-rate changes of 0.05%/0.02%/0.01% and 0.09%/-0.39%/-0.09% in RA and LB cases, respectively.

It is claimed that the current design of combination of WP and GEO may cause subjective artifacts.

Three methods are proposed:

#1. Disabling GEO on slice level when WP is enabled.

#2. Adding CU-level bitstream conformance requirement to disallow the combination of WP and GEO.

#3. Modifying deblocking process of boundary strength for GEO.

It is commented that whether the proponent can see the artifacts when the video playback. The proponents claim that there is artifacts.

It is commented by several experts that method 1 is straightforward and is preferred if the problem exists.

It is commented that encoder side can just not select GEO mode when WP is applied to the block.

No action.

JVET-Q0700 Cross-check of JVET-Q0266 on TPM and GEO merge modes in presence of weighted prediction [P. Onno (Canon)] [late]

JVET-Q0748 Crosscheck of JVET-Q0266 (On TPM and GEO merge modes in presence of weighted prediction) [P. Bordes (InterDigital)] [late]

JVET-Q0349 Combination of TPM and Weighted-Prediction [P. Bordes, T. Poirier, F. LeLeannec (InterDigital)]

The discussion regarding this proposal is chaired by R.-L. Liao.

In the current VTM-7.0 software, the weighted prediction (WP) is not enabled with TPM. In the last JVET meeting (Geneva, Oct-2019), it was agreed that combining WP and TPM was desirable feature. In this last meeting, it has been proposed to either cascade WP and TPM or to re-scale the blending weights with WP parameters prior the TPM blending stage. However, these methods have a latency issue since they add an additional stage and/or need an additional multiplier for the blending.

It is proposed to replace the combination of WP and TPM with either regular bi-prediction or regular weighted bi-prediction. It states that no additional process nor weighting stage is added to the inter-prediction pipeline compared to regular TPM or regular bi-prediction.

It is reported that the proposed method provides BD-rate gains of -0.12%(Y),-0.11%(U),-0.24%(V) / 0.09%(Y),-0.28%(U),0.14%(V) in RA / LDB configurations with fade-to-black sequences, and -0.28%(Y),-0.21%(U),-0.20%(V) / -0.17%(Y),-0.27%(U),-0.12%(V) in RA / LDB configurations with fade-to-white sequences.

It is commented that there is a case that the whole slice selects WP and only one block selects regular bi-prediction. In this case, the subjective artifacts still exist.

No action.

JVET-Q0538 Crosscheck of JVET-Q0349 (Combination of TPM and Weighted-Prediction) [T. Chujoh (Sharp)] [late]

JVET-Q0364 CE4.4-related: Combination of GEO and Weighted-Prediction [P. Bordes, T. Poirier, F. Le Léannec (InterDigital)]

The discussion regarding this proposal is chaired by R.-L. Liao.

In the current VTM-7.0 and CE4 software, the weighted prediction (WP) is not enabled with TPM and GEO. In the last JVET meeting (Geneva, Oct-2019), it has been proposed to either cascade WP and TPM or to re-scale the blending weights with WP parameters prior the TPM blending stage. However, these methods have a latency issue since they add an additional stage and/or need an additional multiplier for the blending.

In the CE4.4, it is studied turning off the blending filter for GEO in case of screen content.

In this proposal, when the weighted prediction (WP) is enabled for at least one GEO partition, it is proposed to apply the regular WP weights on the two GEO prediction partitions and to turn off the blending filter. It is stated that the proposed method avoids cascading two weighting stages (WP weighting and GEO blending filters).

It is reported that:

- compared to CE4.4 with WP enabled (CTC fade sequences), the proposed method provides BD-rate gains of 0.07%(Y),-0.08%(U),-0.01%(V) / 0.06%(Y),-0.48%(U),0.04%(V) in RA / LDB configurations with fade-to-black sequences, and -0.08%(Y),-0.07%(U),-0.26%(V) / -0.18%(Y),-0.13%(U),-0.47%(V) in RA / LDB configurations with fade-to-white sequences.

- compared to CE4.4 with WP enabled (TGM fade sequences), the proposed method provides BD-rate gains of -1.88%(Y),-1.48%(U),-1.37%(V) / -2.25%(Y),-1.79%(U),-1.62%(V) in RA / LDB configurations with fade-to-black sequences, and -1.96%(Y),-1.46%(U),-1.42%(V) / -2.38%(Y),-1.88%(U),-1.77%(V) in RA / LDB configurations with fade-to-white sequences.

The encoder and decoder complexity are unchanged compared to anchors.

This proposal propose to disable the blending process when weighted prediction is enabled.

It is commented that we already decide not to do this (turning off blending process) in the track meeting.

No action.

Others

Q0341, Q0368, Q0370, Q0483

JVET-Q0242 Cleanup for TPM, CIIP, and GEO in the colour format of 4:0:0 [L. Pham Van, G. Van der Auwera, A. K. Ramasubramonian, H. Huang, W.-J. Chien, M. Karczewicz (Qualcomm)]

Presented in track B.

JVET-Q0770 Cross-check of JVET-Q0242 (Cleanup of TPM, CIIP and GEO for 4:0:0 format) [Y.-W. Chen (Kwai Inc.)] [late]

JVET-Q0341 Non-CE4: Range of the motion vector [N. Park, J. Nam, H. Jang, J. Lim, S. Kim (LGE)]

In current VVC draft text, the motion vector range is defined as [-2^17, 2^17 – 1]. If a motion vector exceed the range, two different operations are applied according to coding mode. Specifically, wrapping operation is applied for INTER mode, and clipping operation is for MERGE mode. In this contribution, unified MVD range control method is proposed as follow:

Method #1: Use always the clipping operation

Method #2: Use always the wrapping operation

The experimental results reportedly show no changes in coding performance and run-time compared to VTM7.0.

It is commented that nothing is broken in the current VVC design.

It is commented that the current design is reasonable.

Applying modulo operation to the motion vectors of MMVD, DMVR and Affine merge does not bring real benefits.

JVET-Q0535 Crosscheck of JVET-Q0341 (Non-CE4: Range of the motion vector) [J. Li (Panasonic)] [late]

JVET-Q0354 Non-CE: Clean-up regarding syntaxes for prediction mode decision [H. Jang, J. Nam, N. Park, S. Kim, J. Lim (LGE)]

This contribution suggests to clean-up conditions regarding prediction modes syntaxes. The proposed clean-up condition do not change bitstream. In order to verify whether bitstream changing, the clean-up SW based on the suggested syntax table is provided in the contribution.

- Method 1: remove redundant conditions for pred_mode_plt_flag (e.g., cu_skip_flag == 0 && modeType != MODE_TYPE_INTER)

- Method 2: define 4x4 block or coding block in I-Slice as MODE_TYPE_INTRA and remove related redundant conditions.

The proposed modification is pure editorial.

The proposed clean-up methods simplify the conditions without any bitstream changes.

It is commented by an editor that this part of specification is kind of delicate, efforts are required to confirm the proposed cleanup does not introduce in bugs.

It is suggested to provide more evidence the proposed cleanup does not have side effect in all possible encoder configurations.

JVET-Q0651 Cross-check of JVET-Q0354 (Non-CE: Clean-up regarding syntaxes for prediction mode decision) [T.-C. Ma (Kwai Inc.)] [late]

JVET-Q0368 On context model for mvp_flag and ref_idx [H. Chen, H. Yang (Huawei)]

The discussion regarding this proposal is chaired by R.-L. Liao.

This contribution reports a mismatch between VTM-7.0 and VVC working draft 7 related to the coding of mvp_l0_flag, mvp_l1_flag, ref_idx_l0, and ref_idx_l1. In VTM-7.0, mvp_l0_flag and mvp_l1_flag are coded with a single context. While in WD7, these two syntaxes are coded with separate contexts. Same issue is detected for ref_idx_l0 and ref_idx_l1. This contribution investigated two methods for solving this issue: 1) software is aligned to spec and 2) spec is aligned to software.

Simulation results for method 1 reportedly show that the luma BD-rate 0.00% and 0.XX% in RA and LB cases, respectively, when compared to VTM-7.0.

Mismatch between specification and software is found in mvp_l0_flag and mvp_l1_flag, ref_idx_l0 and ref_idx_l1 flags.

Recommendation (BF): align spec to SW (shared ctx model).

JVET-Q0633 Crosscheck of JVET-Q0368 (On context model for mvp_flag and ref_idx) [X.-W. Li, J.-Y. Huo (Xidian Univ.), S. Wan (NPU)] [late]

JVET-Q0370 On context model for merge indices [H. Chen, H. Yang (Huawei)]

The discussion regarding this proposal is chaired by R.-L. Liao.

This contribution reports a mismatch between VTM-7.0 and VVC working draft 7 related to the coding of merge indices for regular merge mode and triangle merge mode. In the VTM-7.0, the first bin of merge_idx, the first bin of merge_triangle_idx0, and the first bin of merge_triangle_idx1 are coded with a single context model. While in WD7, these bins are coded with separate context models.

Firstly, the specification aligned with software is provided in this proposal. Following the spirit of sharing context for the merge indices of different merge modes using the regular merge candidate list, it is proposed to use the same context for the coding mmvd_cand_flag, the first bin of merge_idx, the first bin of merge_triangle_idx0, and the first bin of merge_triangle_idx1. Simulation results for this method reportedly show that the luma BD-rate 0.00 % and -0.01% in RA and LB cases, respectively, when compared to VTM-7.0.

In addition, the method for aligning the software to spec is tested. Simulation results for this method reportedly show that the luma BD-rate 0.00% and 0.01% in RA and LB cases, respectively, when compared to VTM-7.0.

Mismatch between specification and software is found in merge_idx, merge_triangle_idx0 and merge_triangle_idx1.

Recommendation (BF): align spec to SW (shared ctx model).

JVET-Q0534 Crosscheck of JVET-Q0370 (On context model for merge indices) [Y. Sun, F. Chen, L. Wang (Hikvision)] [late]

JVET-Q0438 Monochrome processing [A. Browne, K. Sharman, S. Keating (Sony)]

Presented in track B.

JVET-Q0483 On avoiding out of range motion vectors [F. Bossen, A. Segall (Sharp)]

It is asserted that in a corner case involving the use of the regular merge mode and the selection of a temporal merge candidate, a motion vector may exceed the signed 18-bit range. It is proposed to clip the motion vector in such case. This corner case occurs with displacements of more than 8000 samples.  Thus, it is not expected to occur under common test conditions and unlikely to occur in pictures with resolutions of 4K and below.

A bug is reported in the current motion vector compression process. A restored motion vector may exceed the maximum range.

To fix the bug, a clipping of motion vector restored from the motion buffer is proposed, regardless whether a scaling operation is applied to the restored motion vector or not.

Recommendation (BF): adopt.