| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q2009 |
| --- | --- |

| Title: | Preliminary plan for VVC verification testing |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Draft Verification Test Plan text |  |  |
| Author(s) or Contact(s): | Mathias Wien Vittorio Baroncini | Tel: Email: | wien@lfb.rwth-aachen.de baroncini@gmx.com |
| Source: | AHG |  |  |

_____________________________

# Introduction

This document contains the initial draft plan for the video verification test to be conducted to verify the coding performance of the VVC Main 10 profile. A formal subjective evaluation will be conducted comparing the VVC Main 10 profile to the HEVC Main 10 profile.

# Schedule

At the joint meeting of the MPEG requirements group and VCEG on Wed. January 15th 2020, the expected/desired completion of first formal test was suggested to be by October 2020.

Test Sequences Identified: T-X weeks

Encoding of sequences completed: T

Subjective evaluation starts: T+1 weeks

Subjective evaluation completed: T+2 weeks

Subjective evaluation results available: T +4 weeks.

# Test sites

[tbd]

# Test conditions

Test cases for SDR, HDR, and 360° video content are defined for the verification tests. The test conditions and configurations for these cases are defined below.

## SDR

The following test conditions will be used for the VVC verification test.

- Number of sequences and video resolutions:

  - X sequences for HD (1920x1080) and UHD (3840x2160) resolutions each

- Bitstreams

  - Generated with VTM X.Y for VVC bitstreams

  - Generated with HM 16.20 for HEVC bitstreams

  - In addition to a. and b., other VVC and/or HEVC bitstreams generated with encoders that are optimized for subjective quality may be tested if available.

- Encoding parameters

  - Fixed QP.

    - 4 bit rate points per sequences covering the whole MOS range as much as possible with QP values for the HM and the VTM. The QPs are to be selected such that the subjectively assessed quality is comparable between the two test models.

  - Bit depth of 10bits for all video resolutions

  - Coding structure depending on the nature of the sequence.

    - Random access, RA (Storage/Streaming)

      - Intra refresh at approximately 1 second intervals.

      - Picture reordering allowed.

    - Low delay, LD (Video conferencing)

      - No Intra refresh.

      - Without picture reordering.

  - Other settings as in the configuration files

    - VTM: 	cfg/encoder_randomaccess_vtm.cfg, and cfg/ encoder_lowdelay_vtm.cfg

    - HM: 	cfg/encoder_randomaccess_main10.cfg and encoder_lowdelay_main.cfg

## HDR

HDR testing will have to verify both HLG-based (1K nits) and PQ-based cases (4K nits).

[tbd]

## 360° Video

[tbd]

# Test sequences

[tbd]

Suitable test sequences for this testing purpose are to be determined. At the Brussels meeting, initial visual assessment experiments were conducted on the CTC Class A sequences (UHD) in order to test a potential method for rate point determination. The method is described in Annex A of this document.

# Encoding results

[tbd]

# Annex A – Informal visual testing method for quality matching

In order to prepare for the selection of test sequences and the determination of suitable rate points for the VVC verification tests, informal viewing sessions were conducted at the 17th JVET meeting in Brussels. The activity’s goal was to determine HM rate points exhibiting similar visual quality as the VTM under CTC. This type of comparison was favored over quality rating tests considering that participants of JVET might easily identify the shown coding scheme and therefore would not be able to provide an unbiased rating e.g. on an impairment scale.

The viewing compared the reconstruction quality of the VTM to the reconstruction quality of HM-16.20. The VTM bitstreams were encoded using the Random Access configuration as specified in the CTC. The Class A (UHD) sequences of the JVET CTC were evaluated in the test. The corresponding HM-16.20 bitstreams were taken from a set of streams encoded with a matching HM Random Access configuration available for QP=22:51. These streams were kindly provided by HHI.

A description of the tests and the results are provided in the following.

## Test sequences

The chosen test sequences are listed in Table 1. For all test sequences, the bitstreams with QP=37 were used in the tests in order to incorporate the test case with the lowest visual quality under CTC. An additional QP with less visible compression artifacts was selected based on a subjective pre-assessment for all sequences but Tango2. This sequence showed already comparably low visual impairment for QP=37 and therefore, a higher quality rate point at a lower QP value was therefore not considered.

In a second pre-assessment step, two HM bitstreams with a QP distance of 1 were selected for each sequence and each VTM QP which were considered to be close to VTM in terms of visual quality. The selected VTM QPs and the corresponding tested HM QP are listed in Table 1.

Table 1. Test sequences and tests QPs

| Class | Sequence name | Frame count | Frame rate | Bit depth | VVC QPs under Test | Compared HM QPs |
| --- | --- | --- | --- | --- | --- | --- |
| A1 | Tango2 | 294 | 60 | 10 | 37 | 35, 36 |
| A1 | FoodMarket4 | 300 | 60 | 10 | 32 37 | 29, 30 34, 35 |
| A1 | Campfire | 300 | 30 | 10 | 27 37 | 26, 27 35, 36 |
| A2 | CatRobot | 300 | 60 | 10 | 32 37 | 29, 30 34, 35 |
| A2 | DaylightRoad2 | 300 | 60 | 10 | 32 37 | 27, 28 34, 35 |
| A2 | ParkRunning3 | 300 | 50 | 10 | 27 37 | 25, 26 34, 35 |

## Test setup

The decoded YUV sequences were presented on an LG OLED55B9PLA display using a Linux PC (Ubuntu 16.04) with a pre-installed video player capable of playing UHD 60 Hz video at full frame rate and stable output. The viewing room was dark with an indirect light source placed behind the display. Three seats were placed at 1.6H distance to the display. A group of up to 9 test subjects participated in a test session with three subjects seated and the other subjects standing close to the three chairs in order to assess the videos at a similar distance. During the session, the test subjects were requested to frequently rotate viewing positions.

## Test protocol

For each sequence and each VTM QP, two test cells with the corresponding HM QPs each were created. In this test, the decoded HM sequence was denoted as Video A and was always shown first. The decoded VTM sequence was denoted Video B and was always shown second. A test cell was constructed as follows:

- “A” (on a mid-gray background, 0.5 s presentation)

- Video A

- “B” (on a mid-gray background, 0.5 s presentation)

- Video B

- “A” (on a mid-gray background, 0.5 s presentation)

- Video A

- “B” (on a mid-gray background, 0.5 s presentation)

- Video B

The order of the test cells was randomized and the QP relation of VTM and HM was not disclosed to the participants. The participants were advised not to reveal their opinions before completion of the visual assessment of each cell. Each test cell was shown multiple times until the participants indicated to have settled opinions. The participants were encouraged to watch each of the repetitions from different viewing positions. Then, the opinions were collected by the test coordinator and the subjects were requested to come to a consensus on the relation of video A and video B for the cell. If diverging opinions persisted, the cell was concluded to show similar quality for A and B. The result for each cell was recorded by the test coordinator.

## Assessment results

A test according to the described protocol with 22 randomized test cells as described in Table 1 was performed on January 14th, 2020 with eight volunteers from the JVET as test subjects. Decoded VTM-7.0 bitstreams were compared to decoded HM-16.20 bistreams. The duration of the test session was about 45 min. The participants confirmed that no break was required.

The recorded results are listed in Table 2 below. The group votes are recorded as ‘+’ if the VTM quality was considered better than HM, ‘o’ if both were considered to be similar, and ‘-‘ if the HM quality was considered to be better than VTM. It is noted that the distance was generally considered small, and most of the favoring expressions in the one or the other direction were only very subtle.

Table 2. VTM-7.0 test sequences and QPs

| Cell | VTM rate [kbps] | HM QP | Rate savings | Group vote |
| --- | --- | --- | --- | --- |
| Tango QP37 1 | 1980 | 36 | 31% | + |
| Tango QP37 2 | 1980 | 35 | 38% | + |
| Foodmarket QP32 1 | 4000 | 30 | 37% | + |
| Foodmarket QP32 2 | 4000 | 29 | 45% | o |
| Foodmarket QP37 1 | 2070 | 35 | 38% | + |
| Foodmarket QP37 2 | 2070 | 34 | 46% | + |
| Campfire QP27 1 | 16100 | 27 | 21% | + |
| Campfire QP27 2 | 16100 | 26 | 36% | + |
| Campfire QP37 1 | 3500 | 36 | 47% | + |
| Campfire QP37 2 | 3500 | 35 | 53% | - |
| CatRobot QP32 1 | 4200 | 30 | 42% | + |
| CatRobot QP32 2 | 4200 | 29 | 50% | o |
| CatRobot QP37 1 | 2200 | 35 | 44% | + |
| CatRobot QP37 2 | 2200 | 34 | 51% | + |
| DaylightRoad QP32 1 | 4150 | 28 | 59% | o |
| DaylightRoad QP32 2 | 4150 | 27 | 66% | - |
| DaylightRoad QP37 1 | 2060 | 35 | 44% | + |
| DaylightRoad QP37 2 | 2060 | 34 | 52% | o |
| ParkRunning QP27 1 | 44200 | 26 | 28% | o |
| ParkRunning QP27 2 | 44200 | 25 | 40% | o |
| ParkRunning QP37 1 | 8050 | 35 | 46% | + |
| ParkRunning QP37 2 | 850 | 34 | 55% | o |

## Assessment results for QP42

An additional test following the same protocol was performed comparing the visual quality of VTM at QP=42 to matching decoded HM bitstreams. The test was conducted on January 16th, 2020 with nine volunteers from the JVET as test subjects. In this test, the visual quality of available VTM-6.0 bitstreams was compared to the HM-16.20 bistreams.

The recorded results are listed in Table 3 below. The group votes are recorded in the same way as reported in Table 2.

Table 3. VTM-6.0 test sequences and QPs

| Cell | VTM rate [kbps] | HM QP | Rate savings | Group vote |
| --- | --- | --- | --- | --- |
| Tango QP42 1 | 1123 | 41 | 31% | + |
| Tango QP42 2 | 1123 | 40 | 39% | + |
| Foodmarket QP42 1 | 1108 | 39 | 46% | + |
| Foodmarket QP42 2 | 1108 | 38 | 53% | o |
| CatRobot QP42 1 | 1167 | 39 | 52% | o |
| CatRobot QP42 2 | 1167 | 38 | 57% | - |
| DaylightRoad QP42 1 | 891 | 41 | 37% | + |
| DaylightRoad QP42 2 | 891 | 40 | 45% | + |
| ParkRunning QP42 1 | 3487 | 41 | 34% | + |
| ParkRunning QP42 2 | 3487 | 40 | 46% | o |

## Discussion

The results of these preliminary tests could be seen as an indication at what bitrate relation between VTM and HM bitstreams a comparable visual quality might be achievable. For each VTM QP two HM bitstreams were assessed. If the HM bitstream with the lower QP was considered to be better than the VTM, the stated rate savings could be considered as an upper bound for the expectable VTM gains. In the opposite case, the rate savings might be a bit higher. It should be noted that these results were achieved by assessment of a small group of experts, and compression artifacts might be judged differently by naïve test subjects in a formal subjective evaluation. More comprehensive studies have to be carried out to determine the rate savings of VTM over HM.

The used test protocol was deemed to be helpful for determining comparable rate points. The test subjects reported the duration of the test sessions of up to 45min to be acceptable. This was attributed to the ability to change location and the discussion periods after viewing of each test cell. It was noted that the comparison at VTM QP=42 was more challenging than the higher quality sequences. At this low quality, both coding schemes exhibited clearly visible compression artifacts and balancing those to be comparable was found to be more difficult than in the cases of medium or even higher quality.