| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q2014-v3 |
| --- | --- |

| Title: | JVET common test conditions and software reference configurations for lossless, near lossless, and mixed lossy/lossless coding |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | T. Nguyen T.-C. Ma A. Nalci | Email: | tung.nguyen@hhi.fraunhofer.de tsung-chuanma@kwai.com analci@qti.qualcomm.com |
| Source: | JVET |  |  |

_____________________________

# Abstract

This document defines common test conditions (CTC) and reference software (VTM) configurations for lossless testing lossless applications for experiments conducted after the 17th and following JVET meetings and further, if applicable. The test scenarios cover applications using either 4:2:0 or 4:4:4 chroma formats, and YCbCr or RGB (shortly YUV and RGB) input signals for the 4:4:4 chroma format case. The test sets include camera captured and screen content.

# Introduction

The CTC described in this document are defined for conducting experiments in a controlled environment where the outcomes of the experiments are comparable. The two test scenarios for this CTC are as follows:

- Lossless 4:2:0 (camera captured and screen content)

- Lossless 4:4:4 (camera captured and screen content + RGB)

For all test scenarios, three temporal configurations are mandatory:

- All-Intra (AI)

- Random-Access (RA)

- Low-delay B (LB)

The default encoder options consisting of the temporal and coding tools configuration are according to the JVET CTC (JVET-N1010), and some encoding parameters are different depending on the test scenario specified in this document. They are overwritten by the additional configuration file provided with the VTM software package.

# Test sets

Test sequences are available from the following location:

- ftp://hevc@ftp.tnt.uni-hannover.de/testsequences/FrExt-candidate-sequences/ (properly accredited members of ISO/IEC MPEG and ITU-T VCEG may please contact the JCT-VC/JVET chairs for login information).

Note that test sequences from the EBU (i.e. with the string ‘EBU’ included in the test sequence name) are password protected.  The password can be obtained by registering at the following location:

- http://tech.ebu.ch/testsequences/uhd-1_public_form

YUV and RGB 4:4:4 test sequences are available from the following locations (properly accredited members of ISO/IEC MPEG and ITU-T VCEG please contact JCT-VC/JVET chairs for login information):

- (YCbCr only) ArenaOfValor 4:4:4 and GlassHalf 4:4:4:
ftp://ftp.ient.rwth-aachen.de.

- Other YCbCr and RGB 4:4:4 sequences:
ftp://hevc@ftp.tnt.uni-hannover.de/testsequences/FrExt-candidate-sequences/upload/screen_content/ScExt-TestSequences.

## 4:2:0 Test set

Not all sequences are mandatory for all temporal configurations, according to the JVET CTC. The following table lists the sequences and whether or not they are mandatory for a given temporal configuration, and whether or not they are included in the lossless test scenario.

| Class | Sequence name | Frames | Rate | Bit-depth | AI | RA | LB |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A1 | Tango2 | 294 | 60 | 10 | M | M | - |
| A1 | FoodMarket4 | 300 | 60 | 10 | M | M | - |
| A1 | Campfire | 300 | 30 | 10 | M | M | - |
| A2 | CatRobot | 300 | 60 | 10 | M | M | - |
| A2 | DaylightRoad2 | 300 | 60 | 10 | M | M | - |
| A2 | ParkRunning3 | 300 | 50 | 10 | M | M | - |
| B | MarketPlace | 600 | 60 | 10 | M | M | M |
| B | RitualDance | 600 | 60 | 10 | M | M | M |
| B | Cactus | 500 | 50 | 8 | M | M | M |
| B | BasketballDrive | 500 | 50 | 8 | M | M | M |
| B | BQTerrace | 600 | 60 | 8 | M | M | M |
| C | RaceHorses | 300 | 30 | 8 | M | M | M |
| C | BQMall | 600 | 60 | 8 | M | M | M |
| C | PartyScene | 500 | 50 | 8 | M | M | M |
| C | BasketballDrill | 500 | 50 | 8 | M | M | M |
| D | RaceHorses | 300 | 30 | 8 | M | M | M |
| D | BQSquare | 600 | 60 | 8 | M | M | M |
| D | BlowingBubbles | 500 | 50 | 8 | M | M | M |
| D | BasketballPass | 500 | 50 | 8 | M | M | M |
| E | FourPeople | 600 | 60 | 8 | M | - | M |
| E | Johnny | 600 | 60 | 8 | M | - | M |
| E | KristenAndSara | 600 | 60 | 8 | M | - | M |
| F | ArenaOfValor | 600 | 60 | 8 | M | M | M |
| F | BasketballDrillText | 500 | 50 | 8 | M | M | M |
| F | SlideEditing | 300 | 30 | 8 | M | M | M |
| F | SlideShow | 500 | 20 | 8 | M | M | M |
| TGM | FlyingGraphics | 300 | 60 | 8 | M | M | M |
| TGM | Desktop | 600 | 60 | 8 | M | M | M |
| TGM | Console | 600 | 60 | 8 | M | M | M |
| TGM | ChineseEditing | 600 | 60 | 8 | M | M | M |

## 4:4:4 Test sets

All 4:4:4 sequences of the test sets are mandatory for all temporal configurations. Note that the YUV and the RGB test sets consist of the same sequences except for the Animation class.

### RGB

| Class | Sequence name | Bit-depth | Rate | Frames |
| --- | --- | --- | --- | --- |
| TGM 1080p | sc_flyingGraphics_1920x1080_60_8bit.rgb | 8 | 60 | 0-299* |
| TGM 1080p | sc_desktop_1920x1080_60_8bit.rgb | 8 | 60 | 0-599 |
| TGM 1080p | sc_console_1920x1080_60_8bit.rgb | 8 | 60 | 0-599 |
| TGM 1080p | ChineseEditing_1920x1080_60_8bit.rgb | 8 | 60 | 0-599 |
| TGM 720p | sc_web_browsing_1280x720_30_8bit.rgb | 8 | 30 | 0-299 |
| TGM 720p | sc_map_1280x720_60_8bit.rgb | 8 | 60 | 0-599 |
| TGM 720p | sc_programming_1280x720_60_8bit.rgb | 8 | 60 | 0-599 |
| TGM 720p | sc_SlideShow_1280x720_20_8bit.rgb | 8 | 20 | 0-499 |
| A | ArenaOfValor_1920x1080_60_8bit_444.rgb | 8 | 60 | 0-599 |
| A | sc_robot_1280x720_30_8bit.rgb | 8 | 30 | 0-299 |
| A | Glasshalf_3840x2160_24p_8b_444.rgb | 8 | 24 | 0-239 |
| M | Basketball_Screen_2560x1440_60p_8b444.rgb | 8 | 60 | 322-621* |
| M | MissionControlClip2_2560x1440_60p_8444.rgb | 8 | 60 | 120-419* |
| M | MissionControlClip3_1920x1080_60p_8b444.rgb | 8 | 60 | 0-599 |
| CC | EBURainFruits_1920x1080_50_10bit_444.rgb | 10 | 50 | 0-249 |
| CC | Kimono1_1920x1080_24_10bit_444.rgb | 10 | 24 | 0-119 |

### YUV

| Class | Sequence name | Bit-depth | Rate | Frames |
| --- | --- | --- | --- | --- |
| TGM 1080p | sc_flyingGraphics_1920x1080_60_8bit.yuv | 8 | 60 | 0-299* |
| TGM 1080p | sc_desktop_1920x1080_60_8bit.yuv | 8 | 60 | 0-599 |
| TGM 1080p | sc_console_1920x1080_60_8bit.yuv | 8 | 60 | 0-599 |
| TGM 1080p | ChineseEditing_1920x1080_60_8bit.yuv | 8 | 60 | 0-599 |
| TGM 720p | sc_web_browsing_1280x720_30_8bit.yuv | 8 | 30 | 0-299 |
| TGM 720p | sc_map_1280x720_60_8bit.yuv | 8 | 60 | 0-599 |
| TGM 720p | sc_programming_1280x720_60_8bit.yuv | 8 | 60 | 0-599 |
| TGM 720p | sc_SlideShow_1280x720_20_8bit.yuv | 8 | 20 | 0-499 |
| A | ArenaOfValor_1920x1080_60_8bit_444.yuv | 8 | 60 | 0-599 |
| A | sc_robot_1280x720_30_8bit.yuv | 8 | 30 | 0-299 |
| A | Glasshalf_3840x2160_24p_10b_444.yuv | 10 | 24 | 0-239 |
| M | Basketball_Screen_2560x1440_60p_8b444.yuv | 8 | 60 | 322-621* |
| M | MissionControlClip2_2560x1440_60p_8444.yuv | 8 | 60 | 120-419* |
| M | MissionControlClip3_1920x1080_60p_8b444.yuv | 8 | 60 | 0-599 |
| CC | EBURainFruits_1920x1080_50_10bit_444.yuv | 10 | 50 | 0-249 |
| CC | Kimono1_1920x1080_24_10bit_444.yuv | 10 | 24 | 0-119 |

* Note that the frames used are different from the original number of frames in the sequence.

- TGM: Text and graphics with motion

- M: mixed content

- A: animation

- CC: camera-captured content

# Software configuration

VTM-8.0 encoder and later provides full-frame lossless operation mode by using a dedicated lossless configuration. The corresponding configuration file “lossless.cfg” is provided with the software package and should be used additionally to the CTC configuration files. The following applies for the lossless test scenarios:

- For all lossless and mixed lossy/lossless configurations, only 100 frames shall be encoded (e.g., by using the encoder option “--FramesToBeEncoded=100”).

- For all test sets, an additional coding tool is mandatory for all sequences: BDPCM (e.g., by using the encoder option “--BDPCM=1”).

- For the 4:2:0 test set, according to the JVET CTC (JVET-N1010), the following coding tools are mandatory for the class F and TGM sequences: IBC and HashME (e.g., by using the encoder options “--IBC=1” and “--HashME=1”).

- For the 4:4:4 test sets, the same applies for all sequences, and an additional coding tool is mandatory for all sequences: PLT (e.g., by using the encoder option “--PLT=1”).

- For the 444 RGB test set, the same applies for all sequences, and an additional coding tool is mandatory for the sequences: ACT (e.g., by using the encoder option “--ColorTransform=1”).

- All remaining encoder configurations are according to the JVET CTC.

The following text provides additional information:

For the RGB input sequences, the provided sequence configuration files use the encoder option “--InputColourSpaceConvert=RGBtoGBR” which changes the order of the color components for the encoding purpose. At the decoder side, the option “--OutputColourSpaceConvert=GBRtoRGB” is then necessary when the reconstructed sequence should have the same color component order as the original input sequence.

The VTM encoder parsing mechanism uses the latest specified value for a given option. Hence, when using the provided configuration files, it is recommended to obey the configuration files order as follows.

- General temporal configuration (e.g., encoder_intra_vtm.cfg)

- Input sequence configuration (e.g., BasketballDrive.cfg)

- Lossless mode (e.g., lossless.cfg)

- Class specific options (e.g., classF.cfg, lossless444.cfg, losslessrRGB.cfg)

- Number of frames, Quantization parameter

# Evaluation

The measurement relies on the compression ratio relative to the uncompressed raw data. A bit-rate saving or overhead is then calculated by comparing the compression ratio of the test relative to the VTM-8.0 anchor.