| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0060-v1 |
| --- | --- |

| Title: | AHG9/AHG12: On CLVSs with one slice per picture |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Zhipin Deng2, Li Zhang1 Kai Zhang1, Jizheng Xu1 18910 University Center Lane San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Tel: Email: | yekui.wang@bytedance.com zhipin.deng@bytedance.com lizhang.idm@bytedance.com zhangkai.video@bytedance.com xujizheng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following changes related to CLVSs in which each picture contains only one slice:

- In the SPS, add sps_one_slice_per_picture_flag, and, when sps_one_slice_per_picture_flag is equal to 1, skip the signalling of sps_num_subpics_minus1 and sps_independent_subpics_flag and infer the values instead. In addition, the following changes are also proposed:

  - Require the value of sps_one_slice_per_picture_flag to be equal to 1 when one_slice_per_pic_constraint_flag is equal to 1.

  - When sps_one_slice_per_picture_flag is equal to 1, the value of picture_header_in_slice_header_flag shall be equal to 1.

- In the PPS, add pps_one_slice_per_picture_flag, which is required to be equal to sps_one_slice_per_picture_flag, and when pps_one_slice_per_picture_flag is equal to 1, skip the signalling of pps_num_subpics_minus1, rect_slice_flag, single_slice_per_subpic_flag, num_slices_in_pic_minus1, loop_filter_across_slices_enabled_flag, rpl_info_in_ph_flag, dbf_info_in_ph_flag, sao_info_in_ph_flag, alf_info_in_ph_flag, wp_info_in_ph_flag, qp_delta_info_in_ph_flag, slice_address and num_tiles_in_slice_minus1 and infer the values instead.

The proposed spec text changes are included in the attachment enclosed in the zip file of this contribution.

# Introduction

In the latest VVC draft text, for any picture in a CLVS, if the PH syntax structure is included in the SH instead of in a PH NAL unit, then each picture in the CLVS has only one slice and the PH syntax structure for each picture in the CLVS is included in the SH instead of in a PH NAL unit. This design was included for a major use case where pictures are not split into multiple slices and each picture can be represented with just one NAL unit (instead of two NAL units, one for the PH and one for the slice).

This contribution proposes to further cleanup the design for this major use case, by adding an SPS flag that indicates whether in the CLVS each picture contains only one slice, repeating the flag in the PPS, and skipping the signalling of known parameters when the flag is equal to 1.

# Proposal

The following changes are proposed:

- In the SPS, add sps_one_slice_per_picture_flag, and, when sps_one_slice_per_picture_flag is equal to 1, skip the signalling of sps_num_subpics_minus1 and sps_independent_subpics_flag and infer the values instead. In addition, the following changes are also proposed:

  - Require the value of sps_one_slice_per_picture_flag to be equal to 1 when one_slice_per_pic_constraint_flag is equal to 1.

  - When sps_one_slice_per_picture_flag is equal to 1, the value of picture_header_in_slice_header_flag shall be equal to 1.

- In the PPS, add pps_one_slice_per_picture_flag, which is required to be equal to sps_one_slice_per_picture_flag, and when pps_one_slice_per_picture_flag is equal to 1, skip the signalling of pps_num_subpics_minus1, rect_slice_flag, single_slice_per_subpic_flag, num_slices_in_pic_minus1, loop_filter_across_slices_enabled_flag, rpl_info_in_ph_flag, dbf_info_in_ph_flag, sao_info_in_ph_flag, alf_info_in_ph_flag, wp_info_in_ph_flag, qp_delta_info_in_ph_flag, slice_address and num_tiles_in_slice_minus1 and infer the values instead.

The proposed spec text changes are included in the attachment enclosed in the zip file of this contribution.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).