| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0070-v1 |
| --- | --- |

| Title: | AHG9: On repetition and update of non-VCL data units |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Li Zhang1, Zhipin Deng2 18910 University Center Lane San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Tel: Email: | yekui.wang@bytedance.com lizhang.idm@bytedance.com zhipin.deng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contributions proposes the following aspects regarding repetition and update of non-VCL data units and sharing of APSs across layers:

- VPS, SPS, and DCI NAL units are disallowed to be present in an AU that has no nal_unit_type in the range of IDR_W_RADL to GDR_NUT, inclusive.

- A PU is disallowed to have more than one VPS, SPS, or PPS NAL unit with a particular VPS, SPS, or PPS ID, and is disallowed to have more than one DCI NAL unit.

- A slice unit (SU) is defined as a set of NAL units that are consecutive in decoding order and contain exactly one coded slice and all its associated non-VCL NAL units.

  - Within an SU it is disallowed to have more than one APS NAL unit with a particular APS ID and of particular APS type.

  - Within an SU it is disallowed to have more than one SEI payload with particular type and a particular content.

- Update to the content of an ALF APS NAL unit within a PU is allowed.

- Sharing of an APS NAL unit across layers is disallowed.

- The same tpes of APSs share the same value space for the APS ID, regardelss of whether the APSs are prefix or suffix APS NAL units.

# Introduction

We observed the following issues in the latest VVC draft text regarding repetition and update of non-VCL data units and sharing of APSs across layers:

- Currently, the repetition of majority of the SEI messages is limited to be at most 4 times within a PU, and the repetition of the decoding unit info SEI message is limited to be at most 4 times within a DU. Repititon of PH, AUD, EOS, and EOB NAL units is disallowed. Repetition of FD NAL units needs to be allowed as many times as needed (e.g., to achieve a constant bit rate). However, there is no limit on the repetition times of other non-VCL NAL units, i.e., VPS, SPS, PPS, APS, and DCI NAL units.

- There is no limit on the repetition of SEI message or some non-VCL NAL units between two VCL NAL units, while such repetition is meaningless.

- The following constraint exists:

All APS NAL units with a particular value of adaptation_parameter_set_id and a particular value of aps_params_type within a PU, regardless of whether they are prefix or suffix APS NAL units, shall have the same content.

The constraint disallows the update of the content of an APS NAL unit within a PU. However, when a picture is split into multiple slices, it makes sense to allow the encoder to individually figure out the ALF parameters to be for the slices. When the number of slice is large, e.g., in 360 o video applications, an update to an ALF APS NAL unit (i.e., change the values of the syntax elements of an APS NAL unit other than the NAL unit type, APS ID, and APS type fields) may be needed.

- An APS NAL unit can be shared across layers. However, for ALF, the filters are quite dependent on the reconstructed samples, while for different layers, even with the same video content, the QPs may be used differently, and therefore the inheritance of ALF filters from that generated for a picture of a different layer in most cases would not give any negligible coding gain benefit. Therefore, sharing of an APS NAL unit across layers should be disallowed to make the ALF APS operations simpler. Similarly for other types of APSs (i.e., LMCS APSs and scaling list APSs).

- It is specified that all APS NAL units with a particular value of aps_params_type, regardless of the nuh_layer_id values, share the same value space for adaptation_parameter_set_id. However, on the other hand, it is specified that all APS NAL units with a particular value of adaptation_parameter_set_id and a particular value of aps_params_type within a PU, regardless of whether they are prefix or suffix APS NAL units, shall have the same content. Therefore, it makes sense for APSs of the same type with different NAL unit types to also share the same value space for adaptation_parameter_set_id.

This contribution tries to address the above issues.

# Proposal

## On repetition of non-VCL data units

It is proposed to add the definition of slice unit as follows:

slice unit (SU): A set of NAL units that are consecutive in decoding order and contain exactly one coded slice and all its associated non-VCL NAL units.

The following constraints are proposed on repeition of non-VCL data units (note that when a particular type of data unit is constrainted to be not more than one in a scope, then update to the content of the data unit is disallowed):

On VPS

- When a VPS NAL unit with a particular value of vps_video_parameter_set_id is present in a CVS, the VPS NAL unit shall be present in the first AU of the CVS, may be present in any AU that has at least one VCL NAL unit having nal_unit_type in the range of IDR_W_RADL to GDR_NUT, inclusive, and shall not be present in any other AU.

- The number of VPS NAL units with a particular value of vps_video_parameter_set_id in a PU shall not be greater than 1.

On SPS

- Let the associated AU set of a CLVS be the set of AUs starting from the AU containing the first picture of the CLVS in decoding order to the AU containing the last picture of the CLVS in decoding order, inclusive.

- When an SPS NAL unit with a particular value of sps_seq_parameter_set_id is present in the associated AU set associatedAuSet of a CLVS that refers to the SPS, the SPS NAL unit shall be present in the first AU of associatedAuSet, and may be present in any AU of associatedAuSet that has at least one VCL NAL unit having nal_unit_type in the range of IDR_W_RADL to GDR_NUT, inclusive, and shall not be present in any other AU.

- The number of SPS NAL units with a particular value of sps_seq_parameter_set_id in a PU shall not be greater than 1.

On PPS

- The number of PPS NAL units with a particular value of pps_pic_parameter_set_id in a PU shall not be greater than 1.

On APS

- The number of APS NAL units with a particular value of adaptation_parameter_set_id and a particular value of aps_params_type in an SU shall not be greater than 1.

On DCI

- When a DCI NAL unit is present in a CVS, it shall be present in the first AU of the CVS, may be present in any AU that has at least one VCL NAL unit having nal_unit_type in the range of IDR_W_RADL to GDR_NUT, inclusive, and shall not be present in any other AU.

- The number of DCI NAL units in a PU shall not be greater than 1.

On SEI messages

- The number of identical sei_payload( ) syntax structures with any particular value of payloadType within an SU shall not be greater than 1.

## On update, cross-layer sharing, and ID value space for APSs

The following are proposed:

- Update to the content of an ALF APS NAL unit within a PU is allowed.

- Sharing of an APS NAL unit across layers is disallowed.

- The same tpes of APSs share the same value space for the APS ID, regardelss of whether the APSs are prefix or suffix APS NAL units.

The proposed text changes to the APS semantics are as follows, based on JVET-Q2001-vE, with deleted texts marked by strikethrough and added texts are marked by turquoise:

##### 7.4.3.5	Adaptation parameter set semantics

Each APS RBSP shall be available to the decoding process prior to it being referenced, included in at least one AU with TemporalId less than or equal to the TemporalId of the coded slice NAL unit that refers it or provided through external means.

All APS NAL units with a particular value of adaptation_parameter_set_id and a particular value of aps_params_type equal to LMCS_APS or SCALING_APS within a PU, regardless of whether they are prefix or suffix APS NAL units, shall have the same content.

adaptation_parameter_set_id provides an identifier for the APS for reference by other syntax elements.

When aps_params_type is equal to ALF_APS or SCALING_APS, the value of adaptation_parameter_set_id shall be in the range of 0 to 7, inclusive.

When aps_params_type is equal to LMCS_APS, the value of adaptation_parameter_set_id shall be in the range of 0 to 3, inclusive.

Let apsLayerId be the value of the nuh_layer_id of a particular APS NAL unit, and vclLayerId be the value of the nuh_layer_id of a particular VCL NAL unit. The particular VCL NAL unit shall not refer to the particular APS NAL unit unless apsLayerId is less than or equal to vclLayerId and the layer with nuh_layer_id equal to apsLayerId is included in at least one OLS that includes the layer with nuh_layer_id equal to vclLayerId.

aps_params_type specifies the type of APS parameters carried in the APS as specified in Table 6.

- Table 6 – APS parameters type codes and types of APS parameters

| aps_params_type | Name of aps_params_type | Type of APS parameters |
| --- | --- | --- |
| 0 | ALF_APS | ALF parameters |
| 1 | LMCS_APS | LMCS parameters |
| 2 | SCALING_APS | Scaling list parameters |
| 3..7 | Reserved | Reserved |

All APS NAL units with a particular value of aps_params_type, regardless of the nuh_layer_id values and whether they are prefix or suffix APS NAL units, share the same value space for adaptation_parameter_set_id. APS NAL units with different values of aps_params_type use separate values spaces for adaptation_parameter_set_id.

NOTE 1 – An APS NAL unit (with a particular value of nal_unit_type, a particular value of adaptation_parameter_set_id, and a particular value of aps_params_type) can be shared across pictures, and different slices within a picture can refer to different ALF APSs.

NOTE 2 – A suffix APS NAL unit associated with a particular VCL NAL unit (this VCL NAL unit precedes the suffix APS NAL unit in decoding order) is not for use by the particular VCL NAL unit, but for use by VCL NAL units following the suffix APS NAL unit in decoding order.

aps_extension_flag equal to 0 specifies that no aps_extension_data_flag syntax elements are present in the APS RBSP syntax structure. aps_extension_flag equal to 1 specifies that there are aps_extension_data_flag syntax elements present in the APS RBSP syntax structure.

aps_extension_data_flag may have any value. Its presence and value do not affect decoder conformance to profiles specified in this version of this Specification. Decoders conforming to this version of this Specification shall ignore all aps_extension_data_flag syntax elements.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).