| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0084-v2 |
| --- | --- |

| Title: | AHG14: On signaling for lossless coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz, Muhammed Coban, Alican Nalci, Hilmi E. Egilmez and Vadim Seregin 5775 Morehouse Dr., San Diego, CA, 92121, USA Tsung-Chuan Ma, Xiaoyu Xiu, Yi-Wen Chen, Hong-Jheng Jhu, Xianglin Wang 8910 University Center Dr., San Diego, CA 92122, USA | Tel: Email: | +1 (858) 845-6730 analci@qti.qualcomm.com tsung-chuanma@kwai.com |
| Source: | Qualcomm Inc., Kwai Inc. |  |  |

_____________________________

# Abstract

In 17th JVET meeting, a slice level flag “slice_ts_residual_coding_disabled_flag” was adopted into VVC as part of JVET-Q0089 for lossless coding. This flag can bypass transform skip residual coding (TSRC) regardless of the residual type and instead regular residual coding (RRC) is used for TS blocks. In this proposal an alternative TU level signaling is proposed such that switching between RRC and TSRC are performed at the TU level instead of at the slice level. This alternative provides further coding gains for screen content and 444 sequences and a more granular control over which blocks can use the residual switch approach.

- For lossless YUV 420 coding, the simulation results show overall bit-rate savings of -0.43% AI, -0.61% RA, and -0.72% LDB for Class F and -3.29% AI, -3.43% RA, and -4.50% LDB for Class TGM, with negligible overall bit-rate loss for natural content (0.02%).

- For lossless YUV 444 coding, the simulation results show overall bit-rate savings of -1.10% AI, -1.45% RA, and -1.59% LDB. For lossless RGB coding, the simulation results show overall bit-rate savings of -0.80% AI, -0.84% RA, and -0.90% LDB.

# Introduction

In VTM-8.0, lossless coding is possible via existing syntax with TS enabled both for luma and chroma [1], which uses regular residual coding (RRC) with a high-level TSRC bypass flag slice_ts_residual_coding_disabled_flag. The control with current design is at the slice level which restricts low-level control and granularity for switching between TSRC and RRC especially for mixed lossless and lossy coding. Here, a TU level signaling alternative is proposed to the existing slice level residual coding flag for lossless coding.

# Results

In the results presented in Table 1 (YUV 420), VTM-8.0 is used as the anchor with “lossless.cfg” config file on top of the CTC configurations. In Table 2 (YUV 444 and RGB), “lossless444.cfg” and “losslessRGB.cfg” config files are included additionally for YUV444 and RGB sequences, respectively. The experiments were run for 100 frames only for AI, RA and LDB configurations.

|  | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Compression Ratio |  | Bit-rate savings | Compression Ratio |  | Bit-rate savings | Compression Ratio |  | bit-rate savings |
|  | Anchor | Proposal |  | Anchor | Proposal |  | Anchor | Proposal |  |
| Class A1 | 2.4 | 2.4 | 0.01% | 2.4 | 2.4 | 0.01% |  |  |  |
| Class A2 | 1.8 | 1.8 | 0.01% | 1.9 | 1.9 | 0.01% |  |  |  |
| Class B | 2.3 | 2.3 | 0.01% | 2.5 | 2.5 | 0.01% | 2.4 | 2.4 | 0.01% |
| Class C | 2.1 | 2.1 | 0.02% | 2.6 | 2.6 | 0.02% | 2.6 | 2.6 | 0.02% |
| Class D | 2.1 | 2.1 | 0.04% | 2.9 | 2.9 | 0.06% | 2.9 | 2.9 | 0.05% |
| Class E | 3.1 | 3.1 | 0.02% |  |  |  | 3.4 | 3.4 | 0.03% |
| Class F | 5.8 | 5.8 | -0.43% | 35.4 | 35.9 | -0.61% | 51.8 | 52.9 | -0.72% |
| TGM | 12.4 | 12.9 | -3.29% | 109.0 | 113.3 | -3.43% | 122.8 | 129.9 | -4.50% |
| Overall | 2.3 | 2.3 | 0.02% | 2.4 | 2.4 | 0.01% | 2.7 | 2.7 | 0.02% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 99% |  |  |
| Dec Time[%] | 96% |  |  | 97% |  |  | 93% |  |  |

Table 1. Lossless Results for YUV 420 CTC sequences.

| RGB | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM-8.0 | Proposal |  | VTM-8.0 | Proposal |  | VTM-8.0 | Proposal |  |
| TGM 1080p | 10.9 | 11.0 | -1.74% | 21.6 | 22.1 | -2.55% | 31.4 | 31.9 | -2.29% |
| TGM 720p | 5.0 | 5.1 | -0.92% | 6.4 | 6.5 | -1.18% | 11.3 | 11.4 | -1.35% |
| A | 3.0 | 3.1 | -1.65% | 1.1 | 1.1 | -2.24% | 0.9 | 1.0 | -2.69% |
| M | 3.0 | 3.0 | -0.68% | 2.4 | 2.4 | -0.51% | 2.6 | 2.7 | -0.92% |
| CC | 0.9 | 0.9 | 0.01% | 0.1 | 0.1 | 0.00% | 0.1 | 0.1 | 0.00% |
| Overall | 5.2 | 5.3 | -1.10% | 7.7 | 7.8 | -1.45% | 11.3 | 11.5 | -1.59% |
| Enc Time[%] | 103% |  |  | 107% |  |  | 109% |  |  |
| Dec Time[%] | 97% |  |  | 97% |  |  | 101% |  |  |
| YUV | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM-8.0 | Proposal |  | VTM-8.0 | Proposal |  | VTM-8.0 | Proposal |  |
| TGM 1080p | 10.7 | 10.9 | -1.93% | 21.8 | 22.1 | -1.64% | 31.6 | 32.0 | -1.83% |
| TGM 720p | 6.4 | 6.4 | -0.79% | 8.1 | 8.1 | -0.95% | 14.5 | 14.6 | -1.00% |
| A | 3.5 | 3.6 | -0.16% | 1.0 | 1.0 | -1.13% | 1.0 | 1.0 | -0.77% |
| M | 3.9 | 3.9 | -0.47% | 3.3 | 3.3 | 0.10% | 3.6 | 3.6 | -0.24% |
| CC | 1.0 | 1.0 | 0.01% | 0.1 | 0.1 | 0.01% | 0.1 | 0.1 | 0.01% |
| Overall | 5.8 | 5.9 | -0.80% | 8.3 | 8.4 | -0.84% | 12.4 | 12.5 | -0.90% |
| Enc Time[%] | 104% |  |  | 109% |  |  | 111% |  |  |
| Dec Time[%] | 101% |  |  | 98% |  |  | 100% |  |  |

Table 2. Lossless Results for YUV 444 and RGB sequences.

# Recommendation

It is recommended to adopt the TU level signaling for residual coding into VVC.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 8)”, JVET-Q2001, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 17th meeting, Brussels, BE, 7-17 Jan. 2019.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).