| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0094 |
| --- | --- |

| Title: | AHG9: DPB output time offsets for temporal sublayers |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Virginie Drugeon Kiyofumi Abe | Tel: Email: | +49-173-6192228 virginie.drugeon@eu.panasonic.com abe.kiyo@jp.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

The author of the present document believes that there is an issue in the calculation of DPB output times for temporal scalable bitstreams. This issue may cause the DPB output time to be lower than the CPB removal time for some pictures when the highest decoded temporal sublayer is not the highest temporal sublayer present in the bitstream (i.e. when a temporal sub-bitstream is decoded). An easy fix consists in shifting the sequence of DPB output times by a fixed offset for the given highest decoded temporal sublayer. It is proposed to signal this fixed offset per temporal sublayer within the buffering period SEI message.

# Problem Statement

HRD parameters in temporal scalable VVC bitstreams are signalled in the picture timing SEI message by signalling different CPB removal delays for each value of highest decoded temporal sublayer. However, there is only one value of DPB output delay that is signalled for all values of highest decoded temporal sublayer. The reason is that dropping some pictures should not change the DPB output times of the remaining pictures. Unfortunately, since the CPB removal times are different for different values of the highest decoded temporal sublayer Htid, it means that for some pictures and some values of Htid, the DPB output time ends up being lower than the CPB removal time when using the current equations documented in Annexes C and D. As a result, in some cases, the current specification mandates some pictures to be output before being decoded.

The example below in Table 1 illustrates this issue. In the example of Table 1, the following equation is being used for the calculation of CPB removal time of picture n for n > 0:

CpbRemovalTime[ n ][ Htid ] = NominalRemovalTime[ n ][ Htid ] = 
								NominalRemovalTime[ firstPicInCurrBuffPeriod ][ Htid ] + 
								ClockTick * CpbRemovalDelayVal[ n ][ Htid ]

For simplicity, it is assumed that NominalRemovalTime[ firstPicInCurrBuffPeriod ][ Htid ] is equal to zero, since this value is simply shifting all the values in the example below.

The following equation is being used for the calculation of DPB output time for picture n:

DpbOutputTime[ n ][ Htid ] = CpbRemovalTime[ n ][ Htid ] + 
							ClockTick * ( picDpbOutputDelay[ n ] – picDpbOutputDelta[ n ][ Htid ] )
DpbOutputTime[ n ][ Htid ] = ClockTick * ( CpbRemovalDelayVal[ n ][ Htid ] +
							picDpbOutputDelay[ n ] ‒ picDpbOutputDelta[ n ][ Htid ] )

With picDpbOutputDelta[ n ][ Htid ] calculated as below:

picDpbOutputDelta[ n ][ Htid ] = CpbRemovalDelayVal[ n ][ Htid ] − 
								CpbRemovalDelayVal[ n ][ sps_max_sublayers_minus1 ]

The variables CpbRemovalDelayVal and picDpbOutputDelay are signalled for each picture in the picture timing SEI message associated to that picture, whereby CpbRemovalDelayVal is sent for each temporal sublayer, whereas there is only one value of picDpbOutputDelay for all temporal sublayers.

As an example, the DPB output time of the third picture in the bitstream in output order when the highest decoded temporal sublayer is equal to 3 is calculated as follows:

picDpbOutputDelta[ 2 ][ 3 ] = CpbRemovalDelayVal[ 2 ][ 3 ] − CpbRemovalDelayVal[ 2 ][ 4 ] 
picDpbOutputDelta[ 2 ][ 3 ] = 8 ‒ 4 = 4
DpbOutputTime[ 2 ][ 3 ] = ClockTick * ( CpbRemovalDelayVal[ 2 ][ 3 ] + 
							picDpbOutputDelay[ 2 ] ‒ picDpbOutputDelta[ 2 ][ 3 ] )
DpbOutputTime[ 2 ][ 3 ] = ClockTick * ( 8 + 2 ‒ 4 ) = ClockTick * 6

Therefore DpbOutputTime[ 2 ][ 3 ] < CpbRemovalTime[ 2 ][ 3 ].

Table 1: Example of values of CPB removal times and DPB output times

| Picture type | I | b | B | b | B | b | B | b | P | b | B | b | B | b | B | b | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TemporalId | 0 | 4 | 3 | 4 | 2 | 4 | 3 | 4 | 1 | 4 | 3 | 4 | 2 | 4 | 3 | 4 | 0 |
| CpbRemovalDelay Val[Htid=4] | 0 | 5 | 4 | 6 | 3 | 8 | 7 | 9 | 2 | 12 | 11 | 13 | 10 | 15 | 14 | 16 | 1 |
| picDpbOutput Delay | 4 | 0 | 2 | 1 | 5 | 1 | 3 | 2 | 10 | 1 | 3 | 2 | 6 | 2 | 4 | 3 | 19 |
| DpbOutputTime / ClockTick | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| CpbRemovalDelay Val[Htid=3] | 0 |  | 8 |  | 6 |  | 10 |  | 4 |  | 14 |  | 12 |  | 16 |  | 2 |
| DpbOutputTime / ClockTick | 4 |  | 6 |  | 8 |  | 10 |  | 12 |  | 14 |  | 16 |  | 18 |  | 20 |
| CpbRemovalDelay Val[Htid=2] | 0 |  |  |  | 12 |  |  |  | 8 |  |  |  | 16 |  |  |  | 4 |
| DpbOutputTime / ClockTick | 4 |  |  |  | 8 |  |  |  | 12 |  |  |  | 16 |  |  |  | 20 |
| CpbRemovalDelay Val[Htid=1] | 0 |  |  |  |  |  |  |  | 16 |  |  |  |  |  |  |  | 8 |
| DpbOutputTime / ClockTick | 4 |  |  |  |  |  |  |  | 12 |  |  |  |  |  |  |  | 20 |

The example in Table 1 clarifies that the DPB output time for different highest decoded temporal sublayers should not be exactly the same as the DPB output time calculated for decoding all the temporal sublayers present in the bitstream.

# DPB output time offset for each temporal sublayer

To avoid the DPB output time being lower than the CPB removal time for some pictures, it is only necessary to shift the sequence of DPB output times by a fixed offset. In the example of Table 1, if the sequence of DPB output times is shifted by an offset of 2 for Htid equal to 3, then the DPB output time is greater than or equal to the CPB removal time for all pictures. The sequence of DPB output times must be shifted by an offset of 4 for Htid equal to 2 and 1.

It is proposed to add an offset for each temporal sublayer to be subtracted in the equation of picDpbOutputDelta (which is mathematically equivalent to adding the offset to the equation of DpbOutputTime). The offsets are signalled within the buffering period SEI message, since the offset should be fixed for each temporal layer during a buffering period. The proposed modified syntax for the buffering period SEI message is shown in Table 2. As shown in the syntax of Table 2, no offset is sent for the highest temporal sublayer present in the bitstream, since the calculated DPB output times do not need any adjustment in this case.

Table 2: Proposed new syntax for the buffering period SEI message

| buffering_period( payloadSize ) { | Descriptor |
| --- | --- |
| bp_nal_hrd_params_present_flag | u(1) |
| bp_vcl_hrd_params_present_flag | u(1) |
| initial_cpb_removal_delay_length_minus1 | u(5) |
| cpb_removal_delay_length_minus1 | u(5) |
| dpb_output_delay_length_minus1 | u(5) |
| alt_cpb_params_present_flag | u(1) |
| bp_decoding_unit_hrd_params_present_flag | u(1) |
| if( bp_decoding_unit_hrd_params_present_flag ) { |  |
| du_cpb_removal_delay_increment_length_minus1 | u(5) |
| dpb_output_delay_du_length_minus1 | u(5) |
| du_cpb_params_in_pic_timing_sei_flag | u(1) |
| du_dpb_params_in_pic_timing_sei_flag | u(1) |
| } |  |
| concatenation_flag | u(1) |
| additional_concatenation_info_present_flag | u(1) |
| if( additional_concatenation_info_present_flag ) |  |
| max_initial_removal_delay_for_concatenation | u(v) |
| cpb_removal_delay_delta_minus1 | u(v) |
| cpb_removal_delay_deltas_present_flag | u(1) |
| if( cpb_removal_delay_deltas_present_flag ) { |  |
| num_cpb_removal_delay_deltas_minus1 | ue(v) |
| for( i = 0; i <= num_cpb_removal_delay_deltas_minus1; i++ ) |  |
| cpb_removal_delay_delta[ i ] | u(v) |
| } |  |
| bp_max_sublayers_minus1 | u(3) |
| bp_cpb_cnt_minus1 | ue(v) |
| sublayer_initial_cpb_removal_delay_present_flag | u(1) |
| for( i = ( sublayer_initial_cpb_removal_delay_present_flag ? 0 : bp_max_sublayers_minus1 ); i <= bp_max_sublayers_minus1; i++ ) { |  |
| if( bp_nal_hrd_params_present_flag ) |  |
| for( j = 0; j < bp_cpb_cnt_minus1 + 1; j++ ) { |  |
| nal_initial_cpb_removal_delay[ i ][ j ] | u(v) |
| nal_initial_cpb_removal_offset[ i ][ j ] | u(v) |
| if( bp_decoding_unit_hrd_params_present_flag) { |  |
| nal_initial_alt_cpb_removal_delay[ i ][ j ] | u(v) |
| nal_initial_alt_cpb_removal_offset[ i ][ j ] | u(v) |
| } |  |
| } |  |
| if( bp_vcl_hrd_params_present_flag ) |  |
| for( j = 0; j < bp_cpb_cnt_minus1 + 1; j++ ) { |  |
| vcl_initial_cpb_removal_delay[ i ][ j ] | u(v) |
| vcl_initial_cpb_removal_offset[ i ][ j ] | u(v) |
| if( bp_decoding_unit_hrd_params_present_flag ) { |  |
| vcl_initial_alt_cpb_removal_delay[ i ][ j ] | u(v) |
| vcl_initial_alt_cpb_removal_offset[ i ][ j ] | u(v) |
| } |  |
| } |  |
| } |  |
| sublayer_dpb_output_offsets_present_flag | u(1) |
| if( sublayer_dpb_output_offsets_present_flag ) { |  |
| for( i = 0; i < bp_max_sublayers_minus1; i++ ) { |  |
| dpb_output_tid_offset[ i ] | ue(v) |
| } |  |
| } |  |
| if( alt_cpb_params_present_flag ) |  |
| use_alt_cpb_params_flag | u(1) |
| } |  |

The proposed semantics for the additional syntax elements is the following:

sublayer_dpb_output_offsets_present_flag equal to 1 specifies that DPB output time offsets are present for temporal sublayer representation(s) in the range of 0 to bp_max_sublayers_minus1 − 1, inclusive. sublayer_dpb_output_offsets_present_flag equal to 0 specified that no such DPB output time offsets are present for temporal sublayer representation(s).

dpb_output_tid_offset[ i ] specifies the difference between the DPB output time of the i-th temporal sublayer and the DPB output time of the bp_max_sublayers_minus1-th temporal sublayer. When dpb_output_tid_offset[ i ] is not present, it is inferred to be equal to 0.

The equation for calculating picDpbOutputDelta is modified as follows:

The variable picDpbOutputDelta[ i ] is derived as follows:

–	If pt_sublayer_delays_present_flag[ i ] is equal to 0, picDpbOutputDelta[ i ] is set equal to 0.

–	Otherwise (pt_sublayer_delays_present_flag[ i ] is equal to 1), picDpbOutputDelta[ i ] is set equal to CpbRemovalDelayVal[ i ] − ( cpb_removal_delay_minus1[ sps_max_sublayers_minus1 ] + 1 ) − ( i == bp_max_sublayers_minus1 ? 0 : dpb_output_tid_offset[ i ] ) where the value of dpb_output_tid_offset[ i ] can be found in the associated BP SEI message.

The example from Table 1 is modified as below in Table 3, whereby dpb_output_tid_offset[ 3 ] is set equal to 2, dpb_output_tid_offset[ 2 ] is set equal to 4 and dpb_output_tid_offset[ 1 ] is set equal to 4.

In Table 3, the DPB output time of the third picture in the bitstream in output order when the highest decoded temporal sublayer is equal to 3 is now calculated as follows:

picDpbOutputDelta[ 2 ][ 3 ] = CpbRemovalDelayVal[ 2 ][ 3 ] − CpbRemovalDelayVal[ 2 ][ 4 ] −
								dpb_output_tid_offset[ 3 ]
picDpbOutputDelta[ 2 ][ 3 ] = 8 ‒ 4 ‒ 2 = 2
DpbOutputTime[ 2 ][ 3 ] = ClockTick * ( CpbRemovalDelayVal[ 2 ][ 3 ] + 
							picDpbOutputDelay[ 2 ] ‒ picDpbOutputDelta[ 2 ][ 3 ] )
DpbOutputTime[ 2 ][ 3 ] = ClockTick * ( 8 + 2 ‒ 2) = ClockTick * 8

Therefore DpbOutputTime[ 2 ][ 3 ] is now equal to CpbRemovalTime[ 2 ][ 3 ].

Table 3: Example of values of CPB removal times and DPB output times 
with the proposed DPB output time offsets

| Picture type | I | b | B | b | B | b | B | b | P | b | B | b | B | b | B | b | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TemporalId | 0 | 4 | 3 | 4 | 2 | 4 | 3 | 4 | 1 | 4 | 3 | 4 | 2 | 4 | 3 | 4 | 0 |
| CpbRemovalDelay Val[Htid=4] | 0 | 5 | 4 | 6 | 3 | 8 | 7 | 9 | 2 | 12 | 11 | 13 | 10 | 15 | 14 | 16 | 1 |
| picDpbOutput Delay | 4 | 0 | 2 | 1 | 5 | 1 | 3 | 2 | 10 | 1 | 3 | 2 | 6 | 2 | 4 | 3 | 19 |
| DpbOutputTime / ClockTick | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| CpbRemovalDelay Val[Htid=3] | 0 |  | 8 |  | 6 |  | 10 |  | 4 |  | 14 |  | 12 |  | 16 |  | 2 |
| DpbOutputTime / ClockTick | 6 |  | 8 |  | 10 |  | 12 |  | 14 |  | 16 |  | 18 |  | 20 |  | 22 |
| CpbRemovalDelay Val[Htid=2] | 0 |  |  |  | 12 |  |  |  | 8 |  |  |  | 16 |  |  |  | 4 |
| DpbOutputTime / ClockTick | 8 |  |  |  | 12 |  |  |  | 16 |  |  |  | 20 |  |  |  | 24 |
| CpbRemovalDelay Val[Htid=1] | 0 |  |  |  |  |  |  |  | 16 |  |  |  |  |  |  |  | 8 |
| DpbOutputTime / ClockTick | 8 |  |  |  |  |  |  |  | 16 |  |  |  |  |  |  |  | 24 |

# Conclusion

An issue has been identified in the calculation of DPB output times when the highest decoded temporal sublayer is not the highest temporal sublayer present in the bitstream. This issue may cause the DPB output time to be lower than the CPB removal time for some pictures. An easy fix consists in shifting the sequence of DPB output times by a fixed offset for the given highest decoded temporal sublayer. It is proposed to signal this fixed offset per temporal sublayer within the buffering period SEI message.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).