| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0114-v3 |
| --- | --- |

| Title: | AHG 9: On scaling window offsets |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson Sachin Deshpande Andrew Segall | Email: | samuelssonj@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This contribution includes a proposal for allowing negative scaling window offsets to be signalled in VVC. In the current draft of VVC, the scaling window offsets for both the current picture and the referenced picture are used to determine the scale factor and horizontal and vertical offsets used in the motion compensation process during inter prediction. It is currently possible that the vertical and horizontal offset becomes negative, for example if the current picture and the reference picture have the same resolution but the scaling window is smaller in the reference picture than in the current picture. It is proposed to allow signalling of negative scaling window offsets so that negative vertical and horizontal offsets can be derived even when the referenced picture did not include a scaling window. The second version of the proposal only updates the header to remove “Alpbach”. The third version of the proposal adds a little bit of further analysis and includes a software patch.

# Introduction

In the VVC draft there is a scaling window defined with four offsets indicating the size and position of the scaling window. The scaling window is used to calculate vertical and horizontal offsets as well as horizontal and vertical scale factor used when predicting from a reference picture to the current picture. Both the scaling window of the current picture and the scaling window of the reference picture are used for these calculations. In the current VVC draft, the scaling offsets are signaled as positive values, i.e. all scaling windows are smaller than the picture the scaling window applies to.

The scaling windows in VVC can for example be used when there is a change in encoded resolution between the reference picture and the current picture (sometimes referred to as RPR – reference picture resampling or ARC – Adaptive Resolution Change), but scaling windows can also be used when the resolution is the same, for example to represent a motion that has occurred between the reference picture and the current picture.

In the current VVC draft, it is possible to represent a zoom-in (scaling factor larger than one) by signaling a scaling window together with the reference picture, as shown in Figure 1.

Figure 1. Example of zoom-in from the reference picture 
to the current picture in the current VVC draft.

However, if no scaling window was signaled for the reference picture, it is not possible to represent a zoom-in (scaling factor larger than one) when encoding the current picture. Thus, it would not be possible to perform this operation when the encoding is done without lookahead or multi-pass processing.

# Proposal

It is asserted that it would be beneficial to allow scaling offsets to be negative. This would enable to use a scaling factor larger than one even when predicting from a reference picture that does not have a scaling window. Using a scaling factor larger than one would for example be useful for representing a zoom-in of the content where objects in the reference picture are smaller than what they are in the current picture and needs to be scaled up (using a scaling factor larger than one) in order to constitute a good reference for inter prediction. An example is shown in Figure 2. Another example use case for allowing negative scaling offset is to be able to represent a global motion (such as panning) for cases when the scene or content has moved from the reference picture to the current picture, as illustrated in Figure 3. This could be useful regardless of the scaling factor between the current picture and the reference picture, i.e. it could be a scaling factor > 1 (zoom-in) scaling factor = 1 (no zoom) or scaling factor < 1 (zoom out) combined with panning horizontally or vertically. In such a case, some of the offsets might be zero or positive while one or more of the offsets are negative.

Figure 2. Example of zoom-in from the reference picture to the current picture with negative offsets

Figure 3. Example of panning from the reference picture to 
the current picture with only one negative offset

## Proposed changes

The proposed syntax changes are as follows:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| scaling_window_explicit_signalling_flag | u(1) |
| if( scaling_window_explicit_signalling_flag ) { |  |
| scaling_win_left_offset | use(v) |
| scaling_win_right_offset | use(v) |
| scaling_win_top_offset | use(v) |
| scaling_win_bottom_offset | use(v) |
| } |  |
| … |  |
| } |  |

The only modification to the syntax table is to change the descriptor used for representing the offsets from ue(v) (unsigned integer 0-th order Exp-Golomb-coded syntax element) to se(v) (signed integer 0-th order Exp-Golomb-coded syntax element).

Additionally, it is proposed to change the allowed value range to ensure that there is a limit to where the scaling window is positioned. The suggested change is as follows:

The value of SubWidthC * ( Abs(scaling_win_left_offset) + Abs(scaling_win_right_offset) ) shall be less than pic_width_in_luma_samples, and the value of SubHeightC * ( Abs(scaling_win_top_offset) + Abs(scaling_win_bottom_offset) ) shall be less than pic_height_in_luma_samples.

# Further analysis

In discussions at the 18th JVET meeting it was mentioned that the approach proposed in this contribution is somewhat limited in the sense that it cannot use different scaling windows for different reference pictures of the same picture. However, it is asserted that the scaling factor that is applied between different pictures in practice will be determined on the movement of the scene and zoom of the camera. An example is illustrated in Figure 4. If the scale factor between Pic A and Pic B is 1.2x and the scale factor between Pic B and Pic C is another 1.2x, then it should be clear that the scale factor between Pic A and Pic C is 1.44 so there appears to be no reason to signal a different scale factor for this case.

Figure 3. Example of scale factor relation between multiple pictures

It is asserted that being able to signal a different scale factor would not have any practical benefit.

# Summary

It is proposed to allow negative scaling window offsets in VVC and to update the allowed value range accordingly.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).