| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0124 |
| --- | --- |

| Title: | AHG9: Clean-ups on picture header |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In this contribution, the following modifications on syntax and semantics related to picture header are proposed:

- Proposal-1: Signaling all_pic_coding_info_present_in_ph_flag in PPS as a gate flag to save some bits to specify whether the picture-level coding tool information is present in PH or SH

- Proposal-2: Correcting the identification of the first VCL NAL unit of a PU or AU

- Proposal-3: Modification on the semantics of gdr_or_irap_pic_flag to cope with the case that mixed_nalu_types_flag is equal to 1

# Proposal-1: Signaling all_pic_coding_info_present_in_ph_flag in PPS

In the latest version of VVC draft (JVET-Q2001-vE), there six flags indicate whether picture-level coding information is present in picture header or slice header in the PPS syntax structure. For example, rpl_info_in_ph_flag, dbf_info_in_ph_flag, sao_info_in_ph_flag and so on. In most cases, those values may have the same value, 0 or 1. It seems unlikely to have a different value for each xxx_info_in_ph flag.

Therefore, it is proposed to signal a gate flag all_pic_coding_info_present_in_ph_flag which indicates the presence of those flags in PPS, to save bits in PPS. If the value of all_pic_coding_info_present_in_ph_flag is equal to 1, those xxx_info_in_ph flags are not signaled and the values of those flags are inferred to be equal to 1, because signaling the picture-level coding information in picture header could happen more frequently than signaling the information in slice header for slice-level control.

Proposed Text:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| all_pic_coding_info_present_in_ph_flag | u(1) |
| if( all_pic_coding_info_present_in_ph_flag ) { |  |
| rpl_info_in_ph_flag | u(1) |
| if( deblocking_filter_override_enabled_flag ) |  |
| dbf_info_in_ph_flag | u(1) |
| sao_info_in_ph_flag | u(1) |
| alf_info_in_ph_flag | u(1) |
| if( ( pps_weighted_pred_flag ¦ ¦ pps_weighted_bipred_flag ) && rpl_info_in_ph_flag ) |  |
| wp_info_in_ph_flag | u(1) |
| qp_delta_info_in_ph_flag | u(1) |
| } |  |
| ... |  |
| } |  |

all_pic_coding_info_present_in_ph_flag equal to 1 specifies that rpl_info_in_ph_flag, dbf_info_in_ph_flag, sao_info_in_ph_flag, alf_info_in_ph_flag, wp_info_in_ph_flag and qp_delta_info_in_ph_flag are not present in PPS. all_pic_coding_info_present_in_ph_flag equal to 0 specifies that rpl_info_in_ph_flag, dbf_info_in_ph_flag, sao_info_in_ph_flag, alf_info_in_ph_flag, wp_info_in_ph_flag and qp_delta_info_in_ph_flag are present in PPS.

rpl_info_in_ph_flag equal to 1 specifies that reference picture list information is present in the PH syntax structure and not present in slice headers referring to the PPS that do not contain a PH syntax structure. rpl_info_in_ph_flag equal to 0 specifies that reference picture list information is not present in the PH syntax structure and may be present in slice headers referring to the PPS that do not contain a PH syntax structure. When not present, the value of rpl_info_in_ph_flag is inferred to be equal to 1.

dbf_info_in_ph_flag equal to 1 specifies that deblocking filter information is present in the PH syntax structure and not present in slice headers referring to the PPS that do not contain a PH syntax structure. dbf_info_in_ph_flag equal to 0 specifies that deblocking filter information is not present in the PH syntax structure and may be present in slice headers referring to the PPS that do not contain a PH syntax structure. When not present, the value of dbf_info_in_ph_flag is inferred to be equal to 0. When not present, the value of dbf_info_in_ph_flag is inferred to be equal to 1.

sao_info_in_ph_flag equal to 1 specifies that SAO filter information is present in the PH syntax structure and not present in slice headers referring to the PPS that do not contain a PH syntax structure. sao_info_in_ph_flag equal to 0 specifies that SAO filter information is not present in the PH syntax structure and may be present in slice headers referring to the PPS that do not contain a PH syntax structure. When not present, the value of sao_info_in_ph_flag is inferred to be equal to 1.

alf_info_in_ph_flag equal to 1 specifies that ALF information is present in the PH syntax structure and not present in slice headers referring to the PPS that do not contain a PH syntax structure. alf_info_in_ph_flag equal to 0 specifies that ALF information is not present in the PH syntax structure and may be present in slice headers referring to the PPS that do not contain a PH syntax structure. When not present, the value of alf_info_in_ph_flag is inferred to be equal to 1.

wp_info_in_ph_flag equal to 1 specifies that weighted prediction information may be present in the PH syntax structure and not present in slice headers referring to the PPS that do not contain a PH syntax structure. wp_info_in_ph_flag equal to 0 specifies that weighted prediction information is not present in the PH syntax structure and may be present in slice headers referring to the PPS that do not contain a PH syntax structure. When not present, the value of wp_info_in_ph_flag is inferred to be equal to 0. When not present, the value of wp_info_in_ph_flag is inferred to be equal to 1.

qp_delta_info_in_ph_flag equal to 1 specifies that QP delta information is present in the PH syntax structure and not present in slice headers referring to the PPS that do not contain a PH syntax structure. qp_delta_info_in_ph_flag equal to 0 specifies that QP delta information is not present in the PH syntax structure and may be present in slice headers referring to the PPS that do not contain a PH syntax structure. When not present, the value of qp_delta_info_in_ph_flag is inferred to be equal to 1.

# Proposal-2

In the latest VVC spec. draft, it is not clear how to identify the first VCL NAL unit of a PU or AU. The following modification on the descriptions of order of NAL units in 7.4.2.4. is proposed.

Proposed Text

A VCL NAL unit is the first VCL NAL unit of an AU (and consequently the PU containing the VCL NAL unit is the first PU of the AU) when the VCL NAL unit is the first VCL NAL unit that follows a PH NAL unit or has picture_header_in_slice_header_flag equal to 1 and one or more of the following conditions are true:

–	The value of nuh_layer_id of the VCL NAL unit is less than the nuh_layer_id of the previous picture in decoding order.

–	The value of ph_pic_order_cnt_lsb of the VCL NAL unit differs from the ph_pic_order_cnt_lsb of the previous picture in decoding order.

–	PicOrderCntVal derived for the VCL NAL unit differs from the PicOrderCntVal of the previous picture in decoding order.

# Proposal-3

The flag gdr_or_irap_pic_flag in picture header indicates whether or not the current picture is an IRAP or GDR picture. When the value of gdr_or_irap_pic_flag is equal to 1, the flag no_output_of_prior_pics_flag is present in picture header as well. When bitstreams of subpictures are merged, the value of no_output_of_prior_pics_flag of an IRAP subpicture needs to be kept for subpicture extraction. To address the issue, the following modification on the semantics of gdr_or_irap_pic_flag is proposed:

Proposed Text

gdr_or_irap_pic_flag equal to 1 specifies that the current picture is a GDR or IRAP picture or a picture with a VCL NAL unit equal to IDR_W_RADL, IDR_N_LP or CRA_NUT and mixed_nalu_types_in_pic_flag equal to 1. gdr_or_irap_pic_flag equal to 0 specifies that the current picture may or may not be a GDR or IRAP picture.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).