| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0130 |
| --- | --- |

| Title: | Cleanup of tC value derivation process for deblocking filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shunsuke Iwamura Shimpei Nemoto Atsuro Ichigaya | Tel: Email: | +81-3-5494-3353 iwamura.s-gc@nhk.or.jp |
| Source: | NHK |  |  |

_____________________________

- Abstract

This contribution proposes cleanup of tC value derivation process for deblocking filter. In the current draft specifications, tC table is defined for 10-bit video instead of 8-bit video as done in HEVC. In order to adapt various bit depth, tC is modified depending on the input bit depth. However, this bit-depth adaptation is not correctly described in the spec text, such that deblocking filter can be performed for block boundaries with lower QP when the input bit depth is equal to 9. To avoid this undesirable deblocking filter, this proposal introduces an offset depending on the input bit depth to tC derivation process. It is asserted that this modification only affects input bit depth of 9 and does not affect CTC results.

Introduction

In the current specification, supported bit depth of the input video signals is ranging from 8 to 16 as defined in SPS semantics as follows:

bit_depth_minus8 specifies the bit depth of the samples of the luma and chroma arrays, BitDepth, and the value of the luma and chroma quantization parameter range offset, QpBdOffset, as follows:

BitDepth      = 8 + bit_depth_minus8		(45)

QpBdOffset = 6 * bit_depth_minus8		(46)

bit_depth_minus8 shall be in the range of 0 to 8, inclusive.

To adapt these various input bit depth, tC value used in deblocking filter process is modified depending on the input bit depth by the following equation:

tC = BitDepth < 10 ? ( tC′ + 2 )  >>  ( 10 − BitDepth ) : tC′ * ( 1  <<  ( BitDepth − 10 ) )      (1297)

When the input bit depth is more than or equal to 10, tC’ is multiplied by 2 to the power of (BitDepth - 10) which seems correct. Meanwhile, in the case that the input bit depth is smaller than 10 (i.e. 8 or 9), tC’ is right shifted with constant offset of 2. This constant offset cause undesirable operation of deblocking filter when the input bit depth is equal to 9. Table 1 show the relationship between tC and tC’ when the bit depth is equal to 9. Original tC’ values for lower QP are defines as ‘0’ since blocking artifact may not appear in such low QP and applying deblocking filter may cause oversmoothing. On the other hand, tC values for low QP which are modified by equation (1297) according to the input bit depth are all set equal to ‘1’, which is unintended.

Table 1. Relationship between tC and tC’ when BitDepth is equal to 9

| Q | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tC′ | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| tC | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Q | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| tC′ | 0 | 3 | 4 | 4 | 4 | 4 | 5 | 5 | 5 | 5 | 7 | 7 | 8 | 9 | 10 | 10 | 11 |
| tC | 1 | 2 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 4 | 4 | 5 | 5 | 6 | 6 | 6 |
| Q | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 |
| tC′ | 13 | 14 | 15 | 17 | 19 | 21 | 24 | 25 | 29 | 33 | 36 | 41 | 45 | 51 | 57 | 64 | 71 |
| tC | 7 | 8 | 8 | 9 | 10 | 11 | 13 | 13 | 15 | 17 | 19 | 22 | 23 | 26 | 29 | 33 | 36 |
| Q | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 | 64 | 65 |  |  |
| tC′ | 80 | 89 | 100 | 112 | 125 | 141 | 157 | 177 | 198 | 222 | 250 | 280 | 314 | 352 | 395 |  |  |
| tC | 41 | 45 | 51 | 57 | 63 | 71 | 79 | 89 | 100 | 112 | 126 | 141 | 158 | 177 | 198 |  |  |

This unintended behavior in the bit-depth adaptation of tC value may cause undesirable deblocking filter for low QP block boundaries especially when slice_beta_offset_div2 and slice_tc_offset_div2 are signalled.

Proposed spec change

The proposed solution modifies the current bit-depth adaptation for tC value as follows:

ofs = 1 << ( 9 – BitDpeth )                                                      (XXX)

tC = BitDepth < 10 ? ( tC′ + 2ofs )  >>  ( 10 − BitDepth ) : tC′ * ( 1  <<  ( BitDepth − 10 ) )    (1297)

This small changes let the spec avoid undesirable behavior of the deblocking filter. Table 2 shows the relationship between tC’ and tC by the proposed modification when input bit depth is equal to 9. As shown in the Table 2, more precise bit-depth adaptation for tC value can be achieved by the proposed solution.

Table 2. Relationship between tC’ and tC by the proposed modification
(Yellow highlighted is modified by the spec change)

| Q | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tC′ | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| tC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Q | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| tC′ | 0 | 3 | 4 | 4 | 4 | 4 | 5 | 5 | 5 | 5 | 7 | 7 | 8 | 9 | 10 | 10 | 11 |
| tC | 0 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 4 | 4 | 4 | 5 | 5 | 5 | 5 |
| Q | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 |
| tC′ | 13 | 14 | 15 | 17 | 19 | 21 | 24 | 25 | 29 | 33 | 36 | 41 | 45 | 51 | 57 | 64 | 71 |
| tC | 7 | 7 | 7 | 9 | 10 | 11 | 12 | 13 | 15 | 17 | 18 | 22 | 23 | 26 | 29 | 32 | 36 |
| Q | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 | 64 | 65 |  |  |
| tC′ | 80 | 89 | 100 | 112 | 125 | 141 | 157 | 177 | 198 | 222 | 250 | 280 | 314 | 352 | 395 |  |  |
| tC | 40 | 45 | 50 | 56 | 63 | 71 | 79 | 89 | 99 | 111 | 125 | 140 | 157 | 176 | 198 |  |  |

Patent rights declaration(s)

NHK may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).