| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0138-v2 |
| --- | --- |

| Title: | AHG9: Some constraints of num_ref_entries |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Eiich Sasaki Tomohiro Ikai | Tel: Email: | +81-50-5213-6334 chujoh.takeshi@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

The syntax element num_ref_entries[ listIdx ][ rplsIdx ] specifies the number of entries in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure. Multiple variables related to reference picture lists are derived from this syntax element. In current VVC Draft 8, there are some problems depending on the value of num_ref_entries. If the value of num_ref_entries is set to 0, the reference picture lists can not be derived when slice_type is equal to P or B. According to the discussion at the previous meetings, in order to generate a mixed NAL picture, it seems to be necessary to set num_ref_entries to zero when nal_unit_type is equal to IDR_W_RADL or IDR_N_LP. In this contribution, some constraints of num_ref_entries have been proposed.

Problem Statement

In current Draft 8 [1], the following conditions are existed.

- num_ref_entries[listIdx][rplsIdx] specifies the number of entries in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure. The value of num_ref_entries[listIdx][rplsIdx] shall be in the range of 0 to MaxDpbSize + 13.

- When the current slice is a P slice, the value of NumRefIdxActive[0] shall be greater than 0.

- When the current slice is a B slice, both NumRefIdxActive[0] and NumRefIdxActive[1] shall be greater than 0.

- For each i equal to 0 or 1, num_ref_entries[i][RplsIdx[i]] shall not be less than NumRefIdxActive[i].

When slice_type is equal to P and listIdx is equal to 0, or when slice_type is equal to B, if num_ref_entries[listIdx][rplsIdx] is set to 0, the above conditions are conflicted.

For example, the variable NumRefIdxActive[ i ] is derived by equation (142) as follows:

for( i = 0; i < 2; i++ ) {
	if( slice_type  = =  B  | |  ( slice_type  = =  P  &&  i  = =  0 ) ) {
		if( num_ref_idx_active_override_flag )
			NumRefIdxActive[ i ] = num_ref_idx_active_minus1[ i ] + 1	(142)
		else {
			if( num_ref_entries[ i ][ RplsIdx[ i ] ]  >=  num_ref_idx_default_active_minus1[ i ] + 1 )
				NumRefIdxActive[ i ] = num_ref_idx_default_active_minus1[ i ] + 1
			else
				NumRefIdxActive[ i ] = num_ref_entries[ i ][ RplsIdx[ i ] ]
		}
	} else /* slice_type  = =  I  | |  ( slice_type  = =  P  &&  i  = =  1 ) */
		NumRefIdxActive[ i ] = 0
}

If slice_type is equal to B, num_ref_entries[ i ][ RplsIdx[ i ] ] is equal to 0 and num_ref_idx_default_active_minus1[ i ] is equal to 0, then NumRefIdxActive[ i ] is set to 0. However, when the current slice is a B slice, both NumRefIdxActive[ 0 ] and NumRefIdxActive[ 1 ] shall be greater than 0.

Another problem is that it is assumed that the value of num_ref_entries[ i ][ RplsIdx[ i ] ] shall be set to zero when nal_unit_type is equal to IDR_W_RADL or IDR_N_LP and sps_idr_rpl_present_flag is equal to 1, according to the BoG discussion at the Geneva meetings. However, there is not any such a constraint in current VVC Draft 8.

Proposal

The following constrains are added to the semantics of num_ref_idx_active_minus1[ i ].

- 7.4.8.1 General slice header semantics

num_ref_idx_active_minus1[ i ] is used for the derivation of the variable NumRefIdxActive[ i ] as specified by Equation 142. The value of num_ref_idx_active_minus1[ i ] shall be in the range of 0 to 14, inclusive.

The value of num_ref_idx_active_minus1[ i ] shall be less than num_ref_entries[ i ][ RplsIdx[ i ] ], inclusive.

For i equal to 0 or 1, when the current slice is a B slice, num_ref_idx_active_override_flag is equal to 1, and num_ref_idx_active_minus1[ i ] is not present, num_ref_idx_active_minus1[ i ] is inferred to be equal to 0.

When the current slice is a P slice, num_ref_idx_active_override_flag is equal to 1, and num_ref_idx_active_minus1[ 0 ] is not present, num_ref_idx_active_minus1[ 0 ] is inferred to be equal to 0.

The variable NumRefIdxActive[ i ] is derived as follows:

for( i = 0; i < 2; i++ ) {
	if( slice_type  = =  B  | |  ( slice_type  = =  P  &&  i  = =  0 ) ) {
		if( num_ref_idx_active_override_flag )
			NumRefIdxActive[ i ] = num_ref_idx_active_minus1[ i ] + 1	(142)
		else {
			if( num_ref_entries[ i ][ RplsIdx[ i ] ]  >=  num_ref_idx_default_active_minus1[ i ] + 1 )
				NumRefIdxActive[ i ] = num_ref_idx_default_active_minus1[ i ] + 1
			else
				NumRefIdxActive[ i ] = num_ref_entries[ i ][ RplsIdx[ i ] ]
		}
	} else /* slice_type  = =  I  | |  ( slice_type  = =  P  &&  i  = =  1 ) */
		NumRefIdxActive[ i ] = 0
}

The value of NumRefIdxActive[ i ] − 1 specifies the maximum reference index for reference picture list i that may be used to decode the slice. When the value of NumRefIdxActive[ i ] is equal to 0, no reference index for reference picture list i may be used to decode the slice.

When the current slice is a P slice, the value of NumRefIdxActive[ 0 ] shall be greater than 0.

When the current slice is a B slice, both NumRefIdxActive[ 0 ] and NumRefIdxActive[ 1 ] shall be greater than 0.

When slice_type is equal to P or B, num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] shall be equal to greater than 0.

When slice_type is equal to B, num_ref_entries[ 1 ][ RplsIdx[1 ] ] shall be equal to greater than 0.

When nal_unit_type is equal to IDR_W_RADL or IDR_N_LP, num_ref_entries[ i ][ RplsIdx[ i ] ] shall be equal to 0 for each i equal to 0 or 1.

Reference

[1]	B. Bross, J. Chen and S. Liu, Versatile Video Coding (Draft 8), Joint Video Experts Team Document, JVET-Q2001-vD, Brussels, March 2020.

[2]	S. Deshpande, BoG report on Coded Picture Regions, Joint Video Experts Team Document, JVET-N0754, Geneva, April 2019.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).