| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0145 |
| --- | --- |

| Title: | AHG 11/15: On the use of limited EGk signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | J. Gan, C. Rosewarne Building A, 5 Talavera Rd, Macquarie Park, 2113, NSW, Australia | Tel: Email: | +61 2 9805 2844 jonathan.gan@canon.com.au chris.rosewarne@canon.com.au |
| Source: | Canon Australia Pty Ltd |  |  |

_____________________________

# Abstract

Only four low level syntax elements remain (cu_qp_delta_abs, palette_predictor_run, num_signalled_palette_entries, and palette_escape_val) that use the general EGk binarisation. EGk is the only non-truncated binarisation used for low level signalling. This contribution proposes to replace the use of EGk with limited EGk for the four low level syntax elements. This change in binarisation has previously occurred for residual coding and mvd coding. It is asserted that the proposed changed binarisations remove unbounded codes from low-level syntax.

# Introduction

In working draft Q2001 [1], section 9.3.3 describes binarisations for all low level (slice data) syntax elements. Shared binarisation processes described are:

- Truncated Rice binarisation

- Truncated Binary binarisation

- k-th order Exp-Golomb binarisation

- Limited k-th order Exp-Golomb binarisation

- Fixed-length binarisation

It is asserted that of all the binarisations described in section 9.3.3, including custom binarisations for specific syntax elements, the only process that is not truncated is the k-th order Exp-Golomb binarisation.

In prior working drafts of VVC, syntax elements have been progressively converted from using the k-th order Exp-Golomb binarisation to the limited k-th order Exp-Golomb binarisation.

- abs_remainder and dec_abs_level were switched to limited EGk with the adoption of JVET-M0470 [2].

- abs_mvd_minus2 was switched to limited EGk with the adoption of JVET-P0090 [3].

In Q2001 there are now only four syntax elements remaining whose binarisations are defined based on k-th order Exp-Golomb:

- cu_qp_delta_abs

- palette_predictor_run

- num_signalled_palette_entries

- palette_escape_val

# Proposed method

It is proposed to replace the EGk binarisations currently used in the four syntax elements cu_qp_delta_abs, palette_predictor_run, num_signalled_palette_entries, and palette_escape_val with limited EGk binarisation as follows:

|  | Parameters |  |  |
| --- | --- | --- | --- |
| Syntax element | k | maxPreExtLen | log2TransformRange |
| cu_qp_delta_abs* | 0 | 5 | 5 |
| palette_predictor_run | 0 | 5 | 6 |
| num_signalled_palette_entries | 0 | 4 | 5 |
| palette_escape_val | 5 | 11 | 17 |

*cu_qp_delta_abs binarisation still consists of a TR prefix string and a suffix string. The suffix string is modified from EGk to limited EGk.

The general EGk process for low-level syntax (section 9.3.3.5) is removed.

It is asserted that the proposed changed binarisations remove unbounded codes from low-level syntax, which has the following advantages:

- Simplification of the standard

- Explicit max binarisation bit length

- Reduce illegal bitstream possibilities

# Experimental results

# Additional information

An editorial clean-up of limited EGk and fixes for its invocation from residual coding and MVD coding binarisation processes was submitted to the VVC bug tracker [4]. The suggested text associated with this bugfix have been attached to this contribution.

# References

- B. Bross, J. Chen, S. Liu, Y.K. Wang, “Versatile Video Coding (Draft 8)”, JVET-Q2001, Brussels, BE, Jan 2020.

- M. Coban, M. Karczewicz, “CE7-related: Golomb-Rice/exponential Golomb coding for abs_remainder and dec_abs_level syntax elements”, JVET-M0470, Marrakech, MA, Jan 2019.

- H. Huang, M. Coban, H. Wang, V. Seregin, W.-J. Chien, M. Karczewicz, “Non-CE4: 32 bits coding for abs_mvd_minus2”, JVET-P0090, Geneva, CH, Oct 2019.

- https://jvet.hhi.fraunhofer.de/trac/vvc/ticket/990

# Patent rights declaration(s)

Canon Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Australia Pty Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).