| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0166 |
| --- | --- |

| Title: | AHG15: Issue on chroma scaling matrix for 4:4:4 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiyofumi Abe Tadamasa Toma 3-1-1 Yagumo-naka-machi, Moriguchi, Osaka, Japan | Tel: Email: | +81-80-3083-1171 abe.kiyo@jp.panasonic.com toma.tadamasa@jp.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

- Abstract

This contribution points out the issue that VTM-8.0 does not work with enabling scaling matrix for 4:4:4 format and proposes two solutions. Solution1 proposes to fix VTM to strictly align to current VVC specification text, it needs to introduce size64 chroma scaling matrices reusing size32 matrices. Solution2 proposes alternative method of current VVC specification text by introducing size64 chroma scaling matrix using individual matrix id. It can simplify the specification and improve the tuning capability.

Introduction

VTM-8.0 dose not work with enabling scaling matrix for 4:4:4 format since size64 chroma scaling matrix is not implemented. While fixing this bug, we noticed there are two solutions to solve this issue. The proponent of this contribution believes that it needs to discuss which solution is more appropriate by considering the impact on the specification and the implementation, and the usability of scaling matrix.

Solution1:

Fix VTM to strictly align to current VVC specification text.

Solution2:

Alternative method that can simplify the specification and improve the tuning capability.

Solution1

In this solution, it is proposed to introduce size64 chroma scaling matrix into VTM following current VVC specification text. Table1 and Table2 show the scaling matrix index and syntax table of scaling list data described in VVC draft of JVET-Q2001-vE[1]. According to Table1, size64 chroma scaling matrices of 21, 22, 24, 25 have same index as size32 chroma scaling matrices, and 28 matrices are signaled in scaling_list_data( ) as shown in Table2. It means that size64 chroma scaling matrices are not signaled in the bitstream, and reuse size32 chroma scaling matrices instead. It was introduced by the editorial process of JVET-P2001-vA[2] following HEVC range extension specification. In HEVC range extension, this method was adopted by JCTVC-N0192[3] considering the influence on V1 specification while it was not ideal for the tuning capability.

Table1: Scaling matrix index of existing VVC text

| matrixSize |  | 2 | 4 | 8 | 16 | 32 | 64 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| INTRA | Y |  | 2 | 8 | 14 | 20 | 26 |
|  | Cb |  | 3 | 9 | 15 | 21 | 21 |
|  | Cr |  | 4 | 10 | 16 | 22 | 22 |
| INTER | Y |  | 5 | 11 | 17 | 23 | 27 |
|  | Cb | 0 | 6 | 12 | 18 | 24 | 24 |
|  | Cr | 1 | 7 | 13 | 19 | 25 | 25 |

Table2: Syntax table of existing VVC text

| scaling_list_data( ) { | Descriptor |
| --- | --- |
| scaling_matrix_for_lfnst_disabled_flag | u(1) |
| scaling_list_chroma_present_flag | u(1) |
| for( id = 0; id < 28; id ++ ) { |  |
| matrixSize = (id < 2 ) ? 2 : ( ( id < 8 ) ? 4 : 8 ) |  |
| if( scaling_list_chroma_present_flag ¦ ¦ ( id % 3 = = 2 ) ¦ ¦ ( id = = 27 ) ) { |  |
| scaling_list_copy_mode_flag[ id ] | u(1) |
| if( !scaling_list_copy_mode_flag[ id ] ) |  |
| scaling_list_pred_mode_flag[ id ] | u(1) |
| if( ( scaling_list_copy_mode_flag[ id ] ¦ ¦ scaling_list_pred_mode_flag [ id ] ) && id != 0 && id != 2 && id != 8 ) |  |
| scaling_list_pred_id_delta[ id ] | ue(v) |
| if( !scaling_list_copy_mode_flag[ id ] ) { |  |
| …. |  |
| } |  |

Solution2

In this solution, it is proposed to introduce size64 chroma scaling matrix using individual matrix index. Table3 and Table4 show the scaling matrix index and the syntax table of scaling list data of this solution. According to Table3, size64 chroma scaling matrices of 27, 28, 30, 31 have individual index, and 32 matrices are signaled in scaling_list_data( ) as shown in Table4. All matrix indexes are assigned simple ascending order, so it is not necessary to introduce matrix index table into the software. VTM can be fixed by simply extending the maximum matrix index value from 27 to 32. All necessary modifications to the VVC text are yellow parts of these tables. According to this solution, each size64 chroma scaling matrix can be signaled individually, therefore it has a possibility to improve tuning capability especially for considering professional use cases.

Table3: Scaling matrix index of solution2

| matrixSize |  | 2 | 4 | 8 | 16 | 32 | 64 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| INTRA | Y |  | 2 | 8 | 14 | 20 | 26 |
|  | Cb |  | 3 | 9 | 15 | 21 | 27 |
|  | Cr |  | 4 | 10 | 16 | 22 | 28 |
| INTER | Y |  | 5 | 11 | 17 | 23 | 29 |
|  | Cb | 0 | 6 | 12 | 18 | 24 | 30 |
|  | Cr | 1 | 7 | 13 | 19 | 25 | 31 |

Table4: Syntax table of solution2

| scaling_list_data( ) { | Descriptor |
| --- | --- |
| scaling_matrix_for_lfnst_disabled_flag | u(1) |
| scaling_list_chroma_present_flag | u(1) |
| for( id = 0; id < 32; id ++ ) { |  |
| matrixSize = (id < 2 ) ? 2 : ( ( id < 8 ) ? 4 : 8 ) |  |
| if( scaling_list_chroma_present_flag ¦ ¦ ( id % 3 = = 2 ) ¦ ¦ ( id = = 27 ) ) { |  |
| scaling_list_copy_mode_flag[ id ] | u(1) |
| if( !scaling_list_copy_mode_flag[ id ] ) |  |
| scaling_list_pred_mode_flag[ id ] | u(1) |
| if( ( scaling_list_copy_mode_flag[ id ] ¦ ¦ scaling_list_pred_mode_flag [ id ] ) && id != 0 && id != 2 && id != 8 ) |  |
| scaling_list_pred_id_delta[ id ] | ue(v) |
| if( !scaling_list_copy_mode_flag[ id ] ) { |  |
| …. |  |
| } |  |

Impact on overhead bits

For evaluating the impact on overhead bits, two tests are conducted for both solution1 and solution2. Test1 is enabling scaling list copy mode and prediction mode, it helps to know the typical example of minimum impact. Test2 is disabling scaling list copy mode and prediction mode, it helps to know the typical example of maximum impact. All tests use the scaling matrix described in JCTVC-F362 [4] (the scaling matrix is also used in the CE of JCTVC-H1104 (symmetry1) [5]).

Table5 and Table6 show the simulation result of Test1 using ArenaOfValor_1920x1080_60_8bit with YUV4:4:4 format and YUV4:2:0 format respectively. The difference of bitrate between solution1 and solution2 is only 0.0016kbps. Since size32 chroma matrix and size64 chroma matrix are identical in this test, the value of difference comes from the total number of bits of scaling_list_copy_mode_flag and scaling_list_pred_id_delta for size64 chroma matrix signaling.

Table7 shows the simulation result of Test2 using ArenaOfValor_1920x1080_60_8bit with YUV4:4:4 format. The difference of bitrate between solution1 and solution2 is 0.0992kbps. The value of difference indicates an example of worst overhead bits increasing of solution2 using different scaling matrices for each size64 chroma.

Table8 shows the total number of bits in one scaling list APS of each test. It shows the actual overhead bits increasing between solution1 and solution2.

Table5: Test1 (enabling copy and pred mode) ArenaOfValor_1920x1080_60_8bit (YUV4:4:4)

| QP | VTM-8.0 with scaling matrix (RA) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | kbps (solution1) | kbps (solution2) | diff |
| 22 | 43.41 | 47.02 | 47.57 | 15932.2008 | 15932.2024 | 0.0016 |
| 27 | 40.17 | 44.52 | 44.94 | 6837.7024 | 6837.7040 | 0.0016 |
| 32 | 37.41 | 42.34 | 42.64 | 3215.0232 | 3215.0248 | 0.0016 |
| 37 | 34.78 | 40.09 | 40.26 | 1547.3712 | 1547.3728 | 0.0016 |

Table6: Test1 (enabling copy and pred mode) ArenaOfValor_1920x1080_60_8bit (YUV4:2:0)

| QP | VTM-8.0 with scaling matrix (RA) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | kbps (solution1) | kbps (solution2) | diff |
| 22 | 43.40 | 45.57 | 46.06 | 12125.7984 | 12125.8000 | 0.0016 |
| 27 | 40.13 | 42.99 | 43.30 | 5232.7384 | 5232.7400 | 0.0016 |
| 32 | 37.35 | 40.76 | 40.93 | 2514.3552 | 2514.3568 | 0.0016 |
| 37 | 34.71 | 38.58 | 38.59 | 1247.6016 | 1247.6032 | 0.0016 |

Table7: Test2 (disabling copy and pred mode) ArenaOfValor_1920x1080_60_8bit (YUV4:4:4)

| QP | VTM-8.0 with scaling matrix (RA) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | kbps (solution1) | kbps (solution2) | diff |
| 22 | 43.41 | 47.02 | 47.57 | 15932.7688 | 15932.8680 | 0.0992 |
| 27 | 40.17 | 44.52 | 44.94 | 6838.2704 | 6838.3696 | 0.0992 |
| 32 | 37.41 | 42.34 | 42.64 | 3215.5912 | 3215.6904 | 0.0992 |
| 37 | 34.78 | 40.09 | 40.26 | 1547.9392 | 1548.0384 | 0.0992 |

Table8: Total number of bits in one scaling list APS

|  | solution1 (bits) | solution2 (bits) | Diff (bits) |
| --- | --- | --- | --- |
| test1 | 1072 | 1088 | 16 |
| test2 | 6752 | 7744 | 992 |

From the evaluation results in this section, the proponent of this contribution believes there are following observations.

For 4:4:4 format, flexibility of scaling matrix would be more important than additional overhead bits shown in this section, since the use cases are mainly professional and the bitrate is very high.

For non-4:4:4 format, the overhead bits increasing of solution2 can be ignored by using copy mode.

Conclusion

This contribution points out the issue that VTM-8.0 does not work with enabling scaling matrix for 4:4:4 format and proposes two solutions. Solution1 proposes to fix VTM to strictly align to current VVC specification text, it needs to introduce size64 chroma scaling matrices reusing size32 matrices. Solution2 proposes alternative method of current VVC specification text by introducing size64 chroma scaling matrix using individual matrix id. It can simplify the specification and improve the tuning capability. The proponent of this contribution believes that solution2 is more appropriate, since flexibility of scaling matrix would be more important than additional overhead bits of solution2 considering the use cases of scaling matrix with 4:4:4 format, and there are negligible overhead bits impact on non-4:4:4 format by using copy mode.

References

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 8)”, JVET-Q2001-vE, JVET 17th Meeting: Brussels, BE, 7–17 January 2020.

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 7)”, JVET-P2001-vA, JVET 16th Meeting: Geneva, CH, 1–11 Oct. 2019.

- K. Sharman, N. Saunders, J. Gamei, “AHG 5: 32x32 Scaling List Derivation for Chroma”, JCTVC-N0192, JCT-VC 14th Meeting: Vienna, AT, 25 July – 2 August 2013.

- T. Suzuki, A. Tabatabai, M. Zhou, V. Sze, “Proposal to support quantization matrix in HEVC”, JCTVC-F362, JCT-VC 6th Meeting: Torino, IT, 14–22 Jul. 2011.

- K. Sato, M. Zhou, R. Joshi, S. Liu, “Description of Core Experiment 4: Quantization Matrices”, JCTVC-H1104, JCT-VC 8th Meeting: San José, CA, USA, 1–10 Feb. 2012.

Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).