| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0167-v2 |
| --- | --- |

| Title: | Issue on LFNST index signaling condition |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiyofumi Abe Tadamasa Toma 3-1-1 Yagumo-naka-machi, Moriguchi, Osaka, Japan | Tel: Email: | +81-80-3083-1171 abe.kiyo@jp.panasonic.com toma.tadamasa@jp.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

- Abstract

This contribution points out the issue of LFNST index signaling. According to the existing VVC specification, chroma LFNST is disabled for the single tree partition mode, and LFNST latency can be reduced by removing the dependency of luma block on chroma block. However, the availability of LFNST index of luma block still depends on chroma transform_skip_flag, even though LFNST is never applied to chroma block in single tree partition mode. This contribution proposes to remove the checking of chroma transform_skip_flag in LFNST index signaling of luma block in single tree partition mode. It can improve the consistency of LFNST conditions and allow the encoder to make more appropriate LFNST selections for luma block. The simulation results reportedly show that the proposed method can simplify LFNST conditions with -0.01% BD-rate gain for RA on VTM-8.0, and -0.01% BD-rate gain for LDB on VTM-8.0 with enabling LFNST.

Introduction

At the Brussels meeting, JVET-Q0784[1] which proposed to introduce LFNST constraints has been adopted. The proposal had the following two constraints;

(1) Disable LFNST when either luma or chroma is transform skip (lfnstNotTsFlag is equal to 0).

lfnstNotTsFlag = ( treeType == DUAL_TREE_CHROMA | | transform_skip_flag[ x0 ][ y0 ][ 0 ] == 0 ) &&

( treeType == DUAL_TREE_LUMA | | ( transform_skip_flag[ x0 ][ y0 ][ 1 ] == 0 &&

transform_skip_flag[ x0 ][ y0 ][ 2 ] == 0 ) )

(2) Disable chroma LFNST for the single tree partition mode.

– If treeType is equal to SINGLE_TREE, the following applies:

ApplyLfnstFlag = ( lfnst_idx > 0 && cIdx  = =  0 ) ? 1 : 0

– Otherwise, the following applies:

ApplyLfnstFlag = ( lfnst_idx > 0 ) ? 1 : 0

According to this adoption, chroma LFNST is disabled for the single tree partition mode, and LFNST latency can be reduced by removing the dependency of luma block on chroma block. However, lfnstNotTsFlag derivation of luma block still depends on chroma transform_skip_flag, even though LFNST is never applied to chroma block in single tree partition mode. Table1 shows the result of lfnstNotTsFlag derivation of existing VVC specification. In this table, TS_flag[ 0 ] represents transform_skip_flag of luma block, TS_flag[ 1 ] and TS_flag[ 2 ] represent transform_skip_flag of each chroma component block respectively.

Table1: LFNST index signaling condition (existing)

|  | TS_flag[ 0 ] | TS_flag[ 1 ] ¦¦ TS_flag[ 2 ] | lfnstNotTsFlag |
| --- | --- | --- | --- |
| DUAL_TREE_LUMA | 0 | - | 1 (can apply LFNST to luma) |
|  | 1 | - | 0 |
| DUAL_TREE_CHROMA | - | 0 | 1 (can apply LFNST to chroma) |
|  | - | 1 | 0 |
| SINGLE_TREE | 0 | 0 | 1 (can apply LFNST to luma) |
|  | 0 | 1 | 0 (disable LFNST for luma) |
|  | 1 | 0 | 0 |
|  | 1 | 1 | 0 |

Proposal

It is proposed to remove the checking of chroma transform_skip_flag in single tree partition mode, and simplify the condition of lfnstNotTsFlag as follows. When the tree mode is equal to SINGLE_TREE or DUAL_TREE_LUMA, only transform_skip_flag of luma block is checked. On the other hand, when the tree mode is equal to DUAL_TREE_CHROMA, transform_skip_flag of both chroma component blocks are checked. As the result, luma block doesn’t need to check chroma transform_skip_flag in single tree partition mode, and the encoder can select LFNST for luma block more appropriately. This concept is similar to change (i) of JVET-Q0686[2], but not identical (the condition for DUAL_TREE_CHROMA is different). JVET-Q0686 is one of the contributions integrated into JVET-Q0784, but change (i) was not included into JVET-Q0784.

lfnstNotTsFlag = ( treeType != DUAL_TREE_CHROMA ? transform_skip_flag[ x0 ][ y0 ][ 0 ] == 0 :

( transform_skip_flag[ x0 ][ y0 ][ 1 ] == 0 && transform_skip_flag[ x0 ][ y0 ][ 2 ] == 0 ) )

Table2: LFNST index signaling condition (proposal)

|  | TS_flag[ 0 ] | TS_flag[ 1 ] ¦¦ TS_flag[ 2 ] | lfnstNotTsFlag |
| --- | --- | --- | --- |
| DUAL_TREE_LUMA | 0 | - | 1 (can apply LFNST to luma) |
|  | 1 | - | 0 |
| DUAL_TREE_CHROMA | - | 0 | 1 (can apply LFNST to chroma) |
|  | - | 1 | 0 |
| SINGLE_TREE | 0 | 0 | 1 (can apply LFNST to luma) |
|  | 0 | 1 | 1 (can apply LFNST to luma) |
|  | 1 | 0 | 0 |
|  | 1 | 1 | 0 |

Simulation results of solution1

Table3 shows the simulation results of proposed method. The simulation is conducted using VTM-8.0 with the test condition described in JVET-N1010[3]. The result of AI condition is not changed from the anchor since AI doesn’t use single tree partition mode. And also, the result of LDB condition is not changed from the anchor since LFNST is disabled in LDB by the configuration setting of CTC.

Table3: Simulation results of proposed method

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 100% |
| TGM | 0.00% | 0.00% | 0.00% | 100% | 98% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.00% | -0.03% | 100% | 100% |
| Class A2 | -0.01% | -0.06% | 0.03% | 100% | 100% |
| Class B | 0.01% | -0.02% | -0.02% | 100% | 100% |
| Class C | -0.01% | 0.03% | 0.01% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.01% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | -0.04% | 100% | 95% |
| Class F | 0.00% | 0.03% | 0.02% | 100% | 99% |
| TGM | 0.00% | 0.04% | 0.03% | 101% | 99% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 98% |
| TGM | 0.00% | 0.00% | 0.00% | 101% | 96% |

To evaluate the effectiveness of the proposed method on LDB, we conducted the additional simulation of LDB with enabling LFNST. Table4 shows the simulation results of this additional test. The anchor is VTM-8.0 enabling LFNST and the test is proposed method with enabling LFNST.

Table4: Simulation results of proposed method with enabling LFNST for LDB

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 enabling LFNST |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.05% | 0.02% | 100% | 100% |
| Class C | -0.02% | -0.02% | -0.06% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Overall | -0.01% | 0.02% | -0.01% | 100% | 99% |
| Class D | 0.00% | -0.09% | 0.07% | 101% | 100% |
| Class F | -0.02% | 0.11% | -0.23% | 100% | 99% |
| TGM | -0.04% | 0.00% | -0.04% | 102% | 102% |

Specification text changes

The suggested specification changes of this proposal are described on top of VVC draft of JVET-Q2001-vE[4]. The newly added parts are highlighted in yellow and the text to be removed is marked as red.

7.3.10.5　Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| …. |  |
| LfnstDcOnly = 1 |  |
| LfnstZeroOutSigCoeffFlag = 1 |  |
| MtsDcOnly = 1 |  |
| MtsZeroOutSigCoeffFlag = 1 |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType, chType ) |  |
| lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : ( ( IntraSubPartitionsSplitType = = ISP_VER_SPLIT ) ? cbWidth / NumIntraSubPartitions : cbWidth ) |  |
| lfnstHeight = ( treeType = = DUAL_TREE_CHROMA ) ? cbHeight / SubHeightC : ( ( IntraSubPartitionsSplitType = = ISP_HOR_SPLIT) ? cbHeight / NumIntraSubPartitions : cbHeight ) |  |
| lfnstNotTsFlag = ( treeType = = DUAL_TREE_CHROMA ¦ ¦ transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 ) && ( treeType = = DUAL_TREE_LUMA ¦ ¦ ( transform_skip_flag[ x0 ][ y0 ][ 1 ] = = 0 && transform_skip_flag[ x0 ][ y0 ][ 2 ] = = 0 ) ) lfnstNotTsFlag = ( treeType != DUAL_TREE_CHROMA ? transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 : ( transform_skip_flag[ x0 ][ y0 ][ 1 ] = = 0 && transform_skip_flag[ x0 ][ y0 ][ 2 ] = = 0 ) ) |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && lfnstNotTsFlag = = 1 && ( treeType = = DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |
| if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx | ae(v) |
| } |  |
| …. |  |
| } |  |

Conclusion

This contribution points out the issue of LFNST index signaling. According to the existing VVC specification, chroma LFNST is disabled for the single tree partition mode, and LFNST latency can be reduced by removing the dependency of luma block on chroma block. However, the availability of LFNST index of luma block still depends on chroma transform_skip_flag, even though LFNST is never applied to chroma block in single tree partition mode. This contribution proposes to remove the checking of chroma transform_skip_flag in LFNST index signaling of luma block in single tree partition mode. It can improve the consistency of LFNST conditions and allow the encoder to make more appropriate LFNST selections for luma block. The simulation results reportedly show that the proposed method can simplify LFNST conditions with -0.01% BD-rate gain for RA on VTM-8.0, and -0.01% BD-rate gain for LDB on VTM-8.0 with enabling LFNST. We propose to adopt it into VVC.

References

- H.E. Egilmez, A. Nalci, A.K. Ramasubramonian, M. Coban, V.Seregin, M. Karczewicz, T. Hashimoto, T. Chujoh, E. Sasaki, T. Ikai, “Combination of JVET-Q0106, JVET-0686 and JVET-Q0133 for LFNST Signaling, Latency Reduction and Scaling Process”, JVET-Q0784, JVET 17th Meeting: Brussels, BE, 7–17 January 2020.

- H.E. Egilmez, A. Nalci, M. Coban, V.Seregin, M. Karczewicz, “Chroma LFNST Simplification and Signaling”, JVET-Q0686, JVET 17th Meeting: Brussels, BE, 7–17 January 2020.

- F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, JVET 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 8)”, JVET-Q2001-vE, JVET 17th Meeting: Brussels, BE, 7–17 January 2020.

Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).