| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0177 |
| --- | --- |

| Title: | AhG 9: APS Cleanup |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam Naser Fabrice Le Léannec Tangi Poirier Philippe de Lagrange | Tel: Email: | karam.naser@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

In the last JVET meeting, a partial cleanup for APS was adopted in JVET-Q0505, where chroma related scaling matrices are not signaled in APS in monochrome mode. It was motivated by the principle “not signaling something which would never be used” according to the meeting notes. It is asserted, however, that this cleaning is not complete for APS. This is because the other APS parameters (ALF and LMCS) still signal redundant chroma information in the case of monochrome mode. This contribution proposes removing this redundant signaling by conditioning it on the chroma type.

# Introduction

APS contains the parameters of ALF (also CC-ALF), LMCS and scaling matrices. When APS is signaled, the syntax aps_params_type is coded to identify the type of parameters (ALF, LMCS or scaling matrices). The parameters of ALF are coded using alf_data() function:

| alf_data( ) { | Descriptor |
| --- | --- |
| alf_luma_filter_signal_flag | u(1) |
| alf_chroma_filter_signal_flag | u(1) |
| alf_cc_cb_filter_signal_flag | u(1) |
| alf_cc_cr_filter_signal_flag | u(1) |
| if( alf_luma_filter_signal_flag ) { |  |
| … |  |
| } |  |
| if( alf_chroma_filter_signal_flag ) { |  |
| … |  |
| } |  |
| if( alf_cc_cb_filter_signal_flag ) { |  |
| … |  |
| } |  |
| if( alf_cc_cr_filter_signal_flag ) { |  |
| … |  |
| } |  |
| } |  |

Clearly, the three flags highlighted in gray in the above syntax table are coded even for monochrome, although they are never used in such a case.

The LMCS parameters are coded in the following way:

| lmcs_data( ) { | Descriptor |
| --- | --- |
| lmcs_min_bin_idx | ue(v) |
| lmcs_delta_max_bin_idx | ue(v) |
| lmcs_delta_cw_prec_minus1 | ue(v) |
| for( i = lmcs_min_bin_idx; i <= LmcsMaxBinIdx; i++ ) { |  |
| lmcs_delta_abs_cw[ i ] | u(v) |
| if( lmcs_delta_abs_cw[ i ] > 0 ) |  |
| lmcs_delta_sign_cw_flag[ i ] | u(1) |
| } |  |
| lmcs_delta_abs_crs | u(3) |
| if( lmcs_delta_abs_crs > 0 ) |  |
| lmcs_delta_sign_crs_flag | u(1) |
| } |  |

For monochrome, lmcs_delta_abs_crs shall be set to zero. However, this is a 3-bit syntax element that must be set to zero.

Alternatively, the scaling matrices data are coded with checking the chroma type:

| scaling_list_data( ) { | Descriptor |
| --- | --- |
| scaling_matrix_for_lfnst_disabled_flag | u(1) |
| scaling_list_chroma_present_flag | u(1) |
| for( id = 0; id < 28; id ++ ) |  |
| matrixSize = (id < 2 ) ? 2 : ( ( id < 8 ) ? 4 : 8 ) |  |
| if( scaling_list_chroma_present_flag ¦ ¦ ( id % 3 = = 2 ) ¦ ¦ ( id = = 27 ) ) { |  |
| scaling_list_copy_mode_flag[ id ] | u(1) |
| if( !scaling_list_copy_mode_flag[ id ] ) |  |
| scaling_list_pred_mode_flag[ id ] | u(1) |
| if( ( scaling_list_copy_mode_flag[ id ] ¦ ¦ scaling_list_pred_mode_flag[ id ] ) && id != 0 && id != 2 && id != 8 ) |  |
| scaling_list_pred_id_delta[ id ] | ue(v) |
| if( !scaling_list_copy_mode_flag[ id ] ) { |  |
| nextCoef = 0 |  |
| if( id > 13 ) { |  |
| scaling_list_dc_coef[ id − 14 ] | se(v) |
| nextCoef += scaling_list_dc_coef[ id − 14 ] |  |
| } |  |
| for( i = 0; i < matrixSize * matrixSize; i++ ) { |  |
| x = DiagScanOrder[ 3 ][ 3 ][ i ][ 0 ] |  |
| y = DiagScanOrder[ 3 ][ 3 ][ i ][ 1 ] |  |
| if( !( id > 25 && x >= 4 && y >= 4 ) ) { |  |
| scaling_list_delta_coef[ id ][ i ] | se(v) |
| nextCoef += scaling_list_delta_coef[ id ][ i ] |  |
| } |  |
| ScalingList[ id ][ i ] = nextCoef |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

The flag “scaling_list_chroma_present_flag” is coded as zero for monochrome. This is different from ALF and LMCS where no such control exists.

This contribution solves the problem of redundant coding of parameters related to chroma for monochrome. Specifically, it harmonizes ALF and LMCS with scaling matrices parameters coding where a check for chroma type is used before coding.

# Proposal 1

Similar to scaling matrices, a flag for the availability of chroma components is coded in ALF and LMCS. It is done in the following way:

| alf_data( ) { | Descriptor |
| --- | --- |
| alf_luma_filter_signal_flag | u(1) |
| alf_chroma_present_flag | u(1) |
| if (alf_chroma_present_flag){ |  |
| alf_chroma_filter_signal_flag | u(1) |
| alf_cc_cb_filter_signal_flag | u(1) |
| alf_cc_cr_filter_signal_flag | u(1) |
| } |  |
| if( alf_luma_filter_signal_flag ) { |  |
| … |  |
| } |  |
| if( alf_chroma_filter_signal_flag ) { |  |
| … |  |
| } |  |
| if( alf_cc_cb_filter_signal_flag ) { |  |
| … |  |
| } |  |
| if( alf_cc_cr_filter_signal_flag ) { |  |
| … |  |
| } |  |
| } |  |

The semantic of this flag is:

alf_chroma_present_flag equal to 1 specifies that chroma alf data are present in alf_data( ). alf_chroma_present_flag equal to 0 specifies that chroma alf data are not present in alf_data( ). It is a requirement of bitstream conformance that alf_chroma_present_flag shall be equal to 0 when ChromaArrayType is equal to 0 and shall be equal to 1 when ChromaArrayType is not equal to 0.

Further, the following flags shall be inferred to zero if not available:

alf_chroma_filter_signal_flag equal to 1 specifies that a chroma filter is signalled. alf_chroma_filter_signal_flag equal to 0 specifies that a chroma filter is not signalled. When ChromaArrayType is equal to 0, alf_chroma_filter_signal_flag shall be equal to 0. When not present, alf_chroma_filter_signal_flag value is inferred to be equal to 0.

alf_cc_cb_filter_signal_flag equal to 1 specifies that cross-component filters for the Cb colour component are signalled. alf_cc_cb_filter_signal_flag equal to 0 specifies that cross-component filters for Cb colour component are not signalled. When ChromaArrayType is equal to 0, alf_cc_cb_filter_signal_flag shall be equal to 0. When not present, alf_cc_cb_filter_signal_flag value is inferred to be equal to 0.

alf_cc_cr_filter_signal_flag equal to 1 specifies that cross-component filters for the Cr colour component are signalled. alf_cc_cr_filter_signal_flag equal to 0 specifies that cross-component filters for the Cr colour component are not signalled. When ChromaArrayType is equal to 0, alf_cc_cr_filter_signal_flag shall be equal to 0. When not present, alf_cc_cr_filter_signal_flag value is inferred to be equal to 0.

The same is done for LMCS:

| lmcs_data( ) { | Descriptor |
| --- | --- |
| lmcs_min_bin_idx | ue(v) |
| lmcs_delta_max_bin_idx | ue(v) |
| lmcs_delta_cw_prec_minus1 | ue(v) |
| for( i = lmcs_min_bin_idx; i <= LmcsMaxBinIdx; i++ ) { |  |
| lmcs_delta_abs_cw[ i ] | u(v) |
| if( lmcs_delta_abs_cw[ i ] > 0 ) |  |
| lmcs_delta_sign_cw_flag[ i ] | u(1) |
| } |  |
| lmcs_chroma_present_flag | u(1) |
| if(lmcs_chroma_present_flag) |  |
| lmcs_delta_abs_crs | u(3) |
| if( lmcs_delta_abs_crs > 0 ) |  |
| lmcs_delta_sign_crs_flag | u(1) |
| } |  |

The semantic of this flag is:

lmcs_chroma_present_flag equal to 1 specifies that chroma lmcs data are present in lmcs_data( ). lmcs_chroma_present_flag equal to 0 specifies that chroma lmcs data are not present in lmcs_data( ). It is a requirement of bitstream conformance that lmcs_chroma_present_flag shall be equal to 0 when ChromaArrayType is equal to 0, and shall be equal to 1 when ChromaArrayType is not equal to 0.

# Proposal 2

Instead of having chroma_present_flag in ALF, LMCS and Scaling matrices function, it is proposed here to have one flag in APS. This is a cleaner design and easier for the encoder setting. Specifically, the following modifications are done:

| adaptation_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| adaptation_parameter_set_id | u(5) |
| aps_params_type | u(3) |
| aps_chroma_present_flag | u(1) |
| if( aps_params_type = = ALF_APS ) |  |
| alf_data( aps_chroma_present_flag ) |  |
| else if( aps_params_type = = LMCS_APS ) |  |
| lmcs_data(aps_chroma_present_flag) |  |
| else if( aps_params_type = = SCALING_APS ) |  |
| scaling_list_data(aps_chroma_present_flag) |  |
| aps_extension_flag | u(1) |
| if( aps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| aps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

The semantic of this flag is:

aps_chroma_present_flag equal to 1 specifies that chroma data related to ALF, LMCS and Scaling Matrices are present in APS. lmcs_chroma_present_flag equal to 0 specifies that chroma data related to ALF, LMCS and Scaling Matrices are not present in APS. It is a requirement of bitstream conformance that lmcs_chroma_present_flag shall be equal to 0 when ChromaArrayType is equal to 0, and shall be equal to 1 when ChromaArrayType is not equal to 0.

Then the individual function is modified as follows:

| alf_data(aps_chroma_present_flag ) { | Descriptor |
| --- | --- |
| alf_luma_filter_signal_flag | u(1) |
| if (aps_chroma_present_flag){ |  |
| alf_chroma_filter_signal_flag | u(1) |
| alf_cc_cb_filter_signal_flag | u(1) |
| alf_cc_cr_filter_signal_flag | u(1) |
| } |  |
| if( alf_luma_filter_signal_flag ) { |  |
| … |  |
| } |  |
| if( alf_chroma_filter_signal_flag ) { |  |
| … |  |
| } |  |
| if( alf_cc_cb_filter_signal_flag ) { |  |
| … |  |
| } |  |
| if( alf_cc_cr_filter_signal_flag ) { |  |
| … |  |
| } |  |
| } |  |

| lmcs_data(aps_chroma_present_flag ) { | Descriptor |
| --- | --- |
| lmcs_min_bin_idx | ue(v) |
| lmcs_delta_max_bin_idx | ue(v) |
| lmcs_delta_cw_prec_minus1 | ue(v) |
| for( i = lmcs_min_bin_idx; i <= LmcsMaxBinIdx; i++ ) { |  |
| lmcs_delta_abs_cw[ i ] | u(v) |
| if( lmcs_delta_abs_cw[ i ] > 0 ) |  |
| lmcs_delta_sign_cw_flag[ i ] | u(1) |
| } |  |
| if(aps_chroma_present_flag) |  |
| lmcs_delta_abs_crs | u(3) |
| if( lmcs_delta_abs_crs > 0 ) |  |
| lmcs_delta_sign_crs_flag | u(1) |
| } |  |

| scaling_list_data(aps_chroma_present_flag ) { | Descriptor |
| --- | --- |
| scaling_matrix_for_lfnst_disabled_flag | u(1) |
| scaling_list_chroma_present_flag | u(1) |
| for( id = 0; id < 28; id ++ ) |  |
| matrixSize = (id < 2 ) ? 2 : ( ( id < 8 ) ? 4 : 8 ) |  |
| if(aps_chroma_present_flag scaling_list_chroma_present_flag ¦ ¦ ( id % 3 = = 2 ) ¦ ¦ ( id = = 27 ) ) { |  |
| scaling_list_copy_mode_flag[ id ] | u(1) |
| if( !scaling_list_copy_mode_flag[ id ] ) |  |
| scaling_list_pred_mode_flag[ id ] | u(1) |
| if( ( scaling_list_copy_mode_flag[ id ] ¦ ¦ scaling_list_pred_mode_flag[ id ] ) && id != 0 && id != 2 && id != 8 ) |  |
| scaling_list_pred_id_delta[ id ] | ue(v) |
| if( !scaling_list_copy_mode_flag[ id ] ) { |  |
| nextCoef = 0 |  |
| if( id > 13 ) { |  |
| scaling_list_dc_coef[ id − 14 ] | se(v) |
| nextCoef += scaling_list_dc_coef[ id − 14 ] |  |
| } |  |
| for( i = 0; i < matrixSize * matrixSize; i++ ) { |  |
| x = DiagScanOrder[ 3 ][ 3 ][ i ][ 0 ] |  |
| y = DiagScanOrder[ 3 ][ 3 ][ i ][ 1 ] |  |
| if( !( id > 25 && x >= 4 && y >= 4 ) ) { |  |
| scaling_list_delta_coef[ id ][ i ] | se(v) |
| nextCoef += scaling_list_delta_coef[ id ][ i ] |  |
| } |  |
| ScalingList[ id ][ i ] = nextCoef |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).