| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0186 |
| --- | --- |

| Title: | AHG12: On misc updates for picture partitioning signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Seethal Paluri Seung Hwan Kim 10225 Willow Creek Rd, San Diego, CA 92131, USA | Email | dr.hendry@lge.com seethal.paluri@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics. |  |  |

_____________________________

# Abstract

This contribution proposed the following changes:

- Move the signalling of no_pic_partition_flag to be earlier than the signalling of pps_num_subpics_minus1. When the value of no_pic_partition_flag is equal to 1, pps_num_subpics_minus1 is not present and inferred to be equal to 0.

- Constrain the value of single_slice_per_subpic_flag to be equal to 0 when no_pic_partition_flag is equal to 0, number of tiles in picture is equal to 1, and the number of subpicture is equal to 1.

- Constrain the subpic_treated_as_pic_flag[ i ] equal to 1 to align the VVC specification text with the picture size, subpicture layout, and the value of loop_filter_across_subpic_enabled_flag[ i ] when the bitstream has multi-layers and the pictures within each layer has more than one subpicture, thus making bitstreams independently extractable.

# Proposal 1: Signal no_pic_partition_flag earlier in the PPS

It is observed that if the signalling of no_pic_partition_flag is earlier than the signalling of pps_num_subpics_minus1, the value of no_pic_partition_flag can be used to condition the presence of pps_num_subpics_minus1.

The spec text changes for this is as follows:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| no_pic_partition_flag | u(1) |
| subpic_id_mapping_in_pps_flag | u(1) |
| if( subpic_id_mapping_in_pps_flag ) { |  |
| if( !no_pic_partition_flag ) |  |
| pps_num_subpics_minus1 | ue(v) |
| pps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= pps_num_subpic_minus1; i++ ) |  |
| pps_subpic_id[ i ] | u(v) |
| } |  |
| no_pic_partition_flag | u(1) |
| if( !no_pic_partition_flag ) { |  |
| ... |  |
| //Tile and slice signalling here// |  |
| ... |  |
| } |  |
| ... |  |
| } |  |

# Proposal 2: Constraint on single_slice_per_subpic_flag

The picture partitioning related signalling currently exists in both SPS and PPS and currently they may contradict each other. When no_pic_partition_flag is equal to 0, number of tiles in picture is equal to 1, it is currently possible to set the value of single_slice_per_subpic_flag to be equal to 1 even when there is only 1 subpicture. Such setting would create a problem since in such situation it is asserted that the number of rectangular slices is greater than 1 and the layout of the slices need to be signalled.

It is suggested that the value of single_slice_per_subpic_flag is constrained to be equal to 0 when no_pic_partition_flag is equal to 0, number of tiles in picture is equal to 1, and the number of subpicture is equal to 1.

The spec text changes for this is as follows:

single_slice_per_subpic_flag equal to 1 specifies that each subpicture consists of one and only one rectangular slice. single_slice_per_subpic_flag equal to 0 specifies that each subpicture may consist of one or more rectangular slices. When not present, the value of single_slice_per_subpic_flag is inferred to be equal to 0.

It is a constraint of bitstream conformance that when the value of no_pic_partition_flag is equal to 0, NumTilesInPic is equal to 1, and the number of subpicture is equal to 1, the value of single_slice_per_subpic_flag shall be equal to 0.

# Proposal 3: Semantic changes on subpic_treated_as_pic_flag

Currently, the VVC specification text aligns the picture size, subpicture layout, and the value of loop_filter_across_subpic_enabled_flag[ i ] when the bitstream has multi-layers and the pictures within each layer has more than one subpicture. However, the value of subpic_treated_as_pic_flag[ i ] is not constrained to be aligned. We believe this may simply be an overlook.

We suggest that the alignment constraint to be update to include the value of subpic_treated_as_pic_flag[ j ] as well.

The spec text changes for this is as follows:

subpic_treated_as_pic_flag[ i ] equal to 1 specifies that the i-th subpicture of each coded picture in the CLVS is treated as a picture in the decoding process excluding in-loop filtering operations. subpic_treated_as_pic_flag[ i ] equal to 0 specifies that the i-th subpicture of each coded picture in the CLVS is not treated as a picture in the decoding process excluding in-loop filtering operations. When not present, the value of subpic_treated_as_pic_flag[ i ] is inferred to be equal to sps_independent_subpics_flag.

When subpic_treated_as_pic_flag[ i ] is equal to 1, it is a requirement of bitstream conformance that all of the following conditions are true for each output layer and its reference layers in an OLS that includes the layer containing the i-th subpicture as an output layer:

–	All pictures in the output layer and its reference layers shall have the same value of pic_width_in_luma_samples and the same value of pic_height_in_luma_samples.

–	All the SPSs referred to by the output layer and its reference layers shall have the same value of sps_num_subpics_minus1 and shall have the same values of subpic_ctu_top_left_x[ j ], subpic_ctu_top_left_y[ j ], subpic_width_minus1[ j ], subpic_height_minus1[ j ], and loop_filter_across_subpic_enabled_flag[ j ], and subpic_treated_as_pic_flag[ j ], respectively, for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive.

–	All pictures in each access unit in the output layer and its reference layers shall have the same value of SubpicIdVal[ j ] for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).