| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0192 |
| --- | --- |

| Title: | AHG9: On signalling recovery point picture |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry 10225 Willow Creek Rd, San Diego, CA 92131, USA | Email: | dr.hendry@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

It is asserted that the signalling of recovery_poc_cnt is not optimal and can be made more efficient based on the following observations:

- The value of recovery_poc_cnt shall be positive and should be greater than 0 because the recovery point picture associated with a GDR picture cannot be the GDR picture itself. However, the value range of recovery_poc_cnt is specified to range from 0 to MaxPicOrderCntLsb – 1.

- In practice, the POC difference between any pair of recovery point picture and its associated GDR picture may be uniform

Based on the observations, this contribution proposed two options as follows:

Option 1: When GDR is enabled (i.e., gdr_enabled_flag is equal to 1), a non-zero value is signalled in SPS to be used as an offset to be added into the equation for deriving the POC of the recovery point picture.

Option 2: Change the syntax element recovery_poc_cnt to become recovery_poc_cnt_minus1.

It is remarked that option 1 is preferred as this would give more bit saving in common use case scenario of the application of GDR.

# Introduction

In the current VVC working draft, in the picture header associated with a GDR picture, recovery_poc_cnt is signalled which is used for deriving the POC value of the recovery point picture associated with the GDR picture. For convenience, the syntax and semantics of recovery_poc_cnt are shown below.

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| gdr_or_irap_pic_flag | u(1) |
| if( gdr_or_irap_pic_flag ) |  |
| gdr_pic_flag | u(1) |
| ... |  |
| if( gdr_or_irap_pic_flag ) |  |
| no_output_of_prior_pics_flag | u(1) |
| if( gdr_pic_flag ) |  |
| recovery_poc_cnt | ue(v) |
| ... |  |
| } |  |

...

recovery_poc_cnt specifies the recovery point of decoded pictures in output order. If the current picture is a GDR picture that is associated with the PH, and there is a picture picA that follows the current GDR picture in decoding order in the CLVS that has PicOrderCntVal equal to the PicOrderCntVal of the current GDR picture plus the value of recovery_poc_cnt, the picture picA is referred to as the recovery point picture. Otherwise, the first picture in output order that has PicOrderCntVal greater than the PicOrderCntVal of the current picture plus the value of recovery_poc_cnt is referred to as the recovery point picture. The recovery point picture shall not precede the current GDR picture in decoding order. The value of recovery_poc_cnt shall be in the range of 0 to MaxPicOrderCntLsb − 1, inclusive.

When the current picture is a GDR picture, the variable RpPicOrderCntVal is derived as follows:

RpPicOrderCntVal = PicOrderCntVal + recovery_poc_cnt	(81)

NOTE 2 – When gdr_enabled_flag is equal to 1 and PicOrderCntVal of the current picture is greater than or equal to RpPicOrderCntVal of the associated GDR picture, the current and subsequent decoded pictures in output order are exact match to the corresponding pictures produced by starting the decoding process from the previous IRAP picture, when present, preceding the associated GDR picture in decoding order.

...

From the above signalling, we observe the following:

- The designated recovery point picture associated with a GDR picture cannot be the GDR picture itself, otherwise, the GDR picture should be labelled as IRAP picture.

- Since the value of recovery_poc_cnt shall be positive and greater than 0 (i.e., since it cannot be the GDR picture itself) but yet the value range of recovery_poc_cnt is from 0 to MaxPicOrderCntLsb – 1

- In practice, the POC difference between any pair of recovery point picture and its associated GDR picture may be uniform.

Based on the above observations, it is asserted that the signalling of recovery_poc_cnt can be improved. We propose two options for improving the signalling of recovery_poc_cnt. Between the two options, we prefer option 1 since this would give more bit saving in common use case scenario of the application of GDR.

# Proposal and spec text changes

## Option 1

It is proposed that when GDR is enabled (i.e., gdr_enabled_flag is equal to 1), a non-zero value is signalled to be used as an offset to be added into the equation for deriving the POC of the recovery point picture.

The spec text changes for implementing this option is shown below.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| gdr_enabled_flag | u(1) |
| if( gdr_enabled_flag ) |  |
| recovery_poc_cnt_offset_minus1 | ue(v) |
| ... |  |
| } |  |

...

recovery_poc_cnt_offset_minus1 plus 1 specifies the offset used for deriving the POC of the recovery point picture. The value of recovery_poc_cnt_offset_minus1 shall be in the range of 0 to MaxPicOrderCntLsb − 2, inclusive.

...

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| gdr_or_irap_pic_flag | u(1) |
| if( gdr_or_irap_pic_flag ) |  |
| gdr_pic_flag | u(1) |
| ... |  |
| if( gdr_or_irap_pic_flag ) |  |
| no_output_of_prior_pics_flag | u(1) |
| if( gdr_pic_flag ) |  |
| recovery_poc_cnt | ue(v) |
| ... |  |
| } |  |

...

recovery_poc_cnt specifies the recovery point of decoded pictures in output order. If the current picture is a GDR picture that is associated with the PH, and there is a picture picA that follows the current GDR picture in decoding order in the CLVS that has PicOrderCntVal equal to the PicOrderCntVal of the current GDR picture plus the value of recovery_poc_cnt, the picture picA is referred to as the recovery point picture. Otherwise, the first picture in output order that has PicOrderCntVal greater than the PicOrderCntVal of the current picture plus the value of recovery_poc_cnt is referred to as the recovery point picture. The recovery point picture shall not precede the current GDR picture in decoding order. The value of recovery_poc_cnt shall be in the range of 0 to MaxPicOrderCntLsb − 1, inclusive.

When the current picture is a GDR picture, the variable RpPicOrderCntVal is derived as follows:

RpPicOrderCntVal = PicOrderCntVal + recovery_poc_cnt + recovery_poc_cnt_offset_minus1 + 1	(81)

NOTE 2 – When gdr_enabled_flag is equal to 1 and PicOrderCntVal of the current picture is greater than or equal to RpPicOrderCntVal of the associated GDR picture, the current and subsequent decoded pictures in output order are exact match to the corresponding pictures produced by starting the decoding process from the previous IRAP picture, when present, preceding the associated GDR picture in decoding order.

...

## Option 2

It is proposed to change the syntax element recovery_poc_cnt to become recovery_poc_cnt_minus1.

The spec text changes for implementing this option is shown below.

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| gdr_or_irap_pic_flag | u(1) |
| if( gdr_or_irap_pic_flag ) |  |
| gdr_pic_flag | u(1) |
| ... |  |
| if( gdr_or_irap_pic_flag ) |  |
| no_output_of_prior_pics_flag | u(1) |
| if( gdr_pic_flag ) |  |
| recovery_poc_cnt_minus1 | ue(v) |
| ... |  |
| } |  |

...

recovery_poc_cnt_minus1 plus 1 specifies the recovery point of decoded pictures in output order. If the current picture is a GDR picture that is associated with the PH, and there is a picture picA that follows the current GDR picture in decoding order in the CLVS that has PicOrderCntVal equal to the PicOrderCntVal of the current GDR picture plus the value of recovery_poc_cnt_minus1 + 1, the picture picA is referred to as the recovery point picture. Otherwise, the first picture in output order that has PicOrderCntVal greater than the PicOrderCntVal of the current picture plus the value of recovery_poc_cnt_minus1 + 1 is referred to as the recovery point picture. The recovery point picture shall not precede the current GDR picture in decoding order. The value of recovery_poc_cnt_minus1 shall be in the range of 0 to MaxPicOrderCntLsb − 2, inclusive.

When the current picture is a GDR picture, the variable RpPicOrderCntVal is derived as follows:

RpPicOrderCntVal = PicOrderCntVal + recovery_poc_cnt_minus1 + 1	(81)

NOTE 2 – When gdr_enabled_flag is equal to 1 and PicOrderCntVal of the current picture is greater than or equal to RpPicOrderCntVal of the associated GDR picture, the current and subsequent decoded pictures in output order are exact match to the corresponding pictures produced by starting the decoding process from the previous IRAP picture, when present, preceding the associated GDR picture in decoding order.

...

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).