| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, AT, 15–24 April 2020 | Document: JVET-R0210 |
| --- | --- |

| Title: | AHG9: Cleanup of Picture Header Syntax Structure in Slice Header |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Semih Esenlik, Biao Wang, Anand Meher Kotra, Elena Alshina | Tel: Email: | semih.esenlik@huawei.com |
| Source: | Huawei Technologies Co., LTD |  |  |

_____________________________

# Abstract

In the 17th JVET meeting in Brussels it was decided to adopt the possibility of including the picture header syntax structure in the slice header. The present contribution proposes refinements to the slice header, especially for the case when PH is included in the slice header. Namely it is proposed to condition the inclusion of following three syntax elements in slice header on the presence of picture header syntax structure in slice header:

- num_tiles_in_slice_minus1

- slice_address

- slice_lmcs_enabled_flag.

# Introduction

In the 17th JVET meeting in Brussels it was adopted to include PH to slice header conditioned on the value of picture_header_in_slice_header_flag. It is requirement of the bitstream if there is more than 1 slice per picture, the picture_header_in_slice_header_flag shall be equal to 0. In other words the picture header structure can be included in the slice header only if there is a only one slice per picture.

# Problem Statement

It is asserted that when the picture header is included in the slice header, redundancies are introduced. When the picture_header_in_slice_header_flag indicates that picture header is included in the slice header, it can also be inferred that there is only one slice in the said picture. The following syntax elements do not need to be included in the slice header if there is only one slice per picture:

- slice_address

- num_tiles_in_slice_minus1

- slice_lmcs_enabled_flag

It is asserted that one slice per picture would be the most prominent use case of VVC standard. Therefore the present contribution proposes to eliminate the above redundancies in the slice header, since the redundancies are related to this use case.

# Proposal

According to the proposal the slice header syntax is modified as follows:

| slice_header( ) { | Descriptor |
| --- | --- |
| picture_header_in_slice_header_flag | u(1) |
| if( picture_header_in_slice_header_flag ) |  |
| picture_header_structure( ) |  |
| if( subpic_info_present_flag ) |  |
| slice_subpic_id | u(v) |
| if( ( rect_slice_flag && NumSlicesInSubpic[ CurrSubpicIdx ] > 1 ) ¦ ¦ (!picture_header_in_slice_header_flag && !rect_slice_flag && NumTilesInPic > 1 ) ) |  |
| slice_address | u(v) |
| for( i = 0; i < NumExtraShBits; i++ ) |  |
| sh_extra_bit[ i ] | u(1) |
| if( !rect_slice_flag && NumTilesInPic > 1 && !picture_header_in_slice_header_flag && NumTilesInPic - slice_address > 1) |  |
| num_tiles_in_slice_minus1 | ue(v) |
| … … |  |
| if( ph_lmcs_enabled_flag && !picture_header_in_slice_header_flag) |  |
| slice_lmcs_enabled_flag | u(1) |
| … … |  |

num_tiles_in_slice_minus1 plus 1, when present, specifies the number of tiles in the slice. The value of num_tiles_in_slice_minus1 shall be in the range of 0 to NumTilesInPic − 1, inclusive.

When not present its value is inferred as follows:

- If the picture_header_in_slice_header_flag is equal to 1, the value of num_tiles_in_slice_minus1 is infered to be equal to NumTilesInPic – 1.

- Otherwise if NumTilesInPic – slice_address is equal to 1, the value of num_tiles_in_slice_minus1 is infered to be equal to 0.

slice_lmcs_enabled_flag equal to 1 specifies that luma mapping with chroma scaling is enabled for the current slice. slice_lmcs_enabled_flag equal to 0 specifies that luma mapping with chroma scaling is not enabled for the current slice. When slice_lmcs_enabled_flag is not present, it is inferred to be equal to 0. ph_lmcs_enabled_flag.

ph_lmcs_enabled_flag equal to 1 specifies that luma mapping with chroma scaling might be is enabled for all slices associated with the PH. ph_lmcs_enabled_flag equal to 0 specifies that luma mapping with chroma scaling may be is disabled for one or more or all slices associated with the PH. When not present, the value of ph_lmcs_enabled_flag is inferred to be equal to 0.

## Explanation of the proposed modifications

When the value of picture_header_in_slice_header_flag is equal to 1, the picture header is included in the slice header, and hence it is the requirement of the bitstream that there must be only 1 slice in the picture. Therefore it does not make sense to include slice_address and num_tiles_in_slice_minus1 syntax elements in the slice header. The contribution proposes to include picture_header_in_slice_header_flag as presence condition.

Furthermore in the current VVC, the num_tiles_in_slice_minus1 syntax element is conditioned on the “NumTilesInPic > 1” condition. This condition is modified as “NumTilesInPic - slice_address > 1”, so that if the last slice in the picture includes only 1 tile, the num_tiles_in_slice_minus1 syntax element is not included in the slice header, which is redundant.

Moreover the slice_lmcs_enabled_flag syntax element also becomes redundant when picture header syntax structure is included in the slice header, since an equivalaent flag (ph_lmcs_enabled_flag) is present in the picture header syntax structure. In this contribution it is proposed to condition the presence of slice_lmcs_enabled_flag on picture_header_in_slice_header_flag.

## Analysis of bitrate savings

When there is one slice per picture, the amount of bit savings in the slice header depends on the number of tiles in picture. Following table shows the savings per picture in with different number of tiles:

| Number of tiles in pic | Savings in bits in slice header (slice_adress + num_tiles_in_slice_minus1 + slice_lmcs_enabled_flag |
| --- | --- |
| 1 | 0 + 0 + 1 = 1 |
| 2 | 1 + 3 + 1 = 5 |
| 4 | 2 + 5 + 1 = 8 |
| 6 | 3 + 5 + 1 = 9 |
| 8 | 3 + 7 + 1 = 11 |
| 10 | 4 + 7 + 1 = 12 |
| 12 | 4 + 7 + 1 = 12 |
| 16 | 4 + 9 + 1 = 14 |
| … |  |

# Conclusion

In the last JVET meeting in Brussels a new mechanism is included in the VVC specification that enables including the PH syntax structure in SH. Several redundancies are identified in the present contribution relating to the case when PH is included in the slice header. Since it is expected that inclusion of PH in SH would be the most prominent configuration of VVC standard, it is proposed to eliminate the identified redundancies as proposed in the contribution.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).