| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0232 |
| --- | --- |

| Title: | AHG9: APS, LMCS, deblocking and PPS constraints |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu, Vadim Seregin, Muhammed Coban, Marta Karczewicz 5775 Morehouse Dr, San Diego, CA, 92121, USA | Tel: Email: | +1(858)651-3483 nanh@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to add a constraint for APS type based on the enabled tools in SPS. If a tool, which uses an APS, is disabled then the APS with the corresponding APS type should not be present in a bitstream. Secondly, the contribution proposes to add a constraint for CCALF filters in an ALF APS based on CCALF SPS enable flag. If CCALF is disabled in SPS, an ALF_APS cannot contain any CCALF filters. Thirdly, the contribution proposes to add constraints for chroma deblocking and chroma LMCS scaling parameter for monochrome content, similar constraints are already in place for other chroma related tools and syntax elements. At last, as another option, to decouple constraints in both PPS and APS from SPS, ChromaArrayType and BitDepth constraints are removed from both PPS and APS semantics.

# Introduction

In VVC draft 8, some APS types can still be present in a bitstream even if the corresponding tools are disabled in SPS. This contribution proposes to add a constraint for APS type. If a tool, which using an APS, is disabled, an APS with the corresponding APS type should not be present in a bitstream.

Secondly, if CCALF is disabled in SPS, CCALF filters can still be present in an ALF_APS. This contribution proposes to add a constraint, such that if CCALF is disabled in SPS, CCALF filters shall not be present in an ALF_APS.

Thirdly, some chroma syntax elements are constrained or not signalled if chroma_format_idc indicates monochrome content. For example, chroma offset tools flag, chroma ALF related syntax, chroma SAO and etc. However, chroma deblocking filter parameters (tc offsets and beta offsets) are signalled for all chroma formats. These parameters are redundant for monochrome pictures. In addition, non-zero LMCS chroma scaling parameter lmcs_delta_abs_crs can be signalled for monochrome contents. In this contribution, constraints for chroma deblocking filter parameters and chroma LMCS scaling parameters are added for monochrome contents.

At last, as an alternative solution, to decouple constraints in both PPS and APS from SPS, BitDepth and ChromaArrayType constraints, which are signalled in SPS, are removed from both PPS and APS semantics.

# APS types

LMCS, scaling matrices and ALF use APS to carry tool’s parameters. If a tool is disabled at SPS level, then APS of the corresponding APS type should not be present in the bitstream. It is proposed to add such constraints.

aps_params_type specifies the type of APS parameters carried in the APS as specified in Table 6.

Table 6 – APS parameters type codes and types of APS parameters

| aps_params_type | Name of aps_params_type | Type of APS parameters |
| --- | --- | --- |
| 0 | ALF_APS | ALF parameters |
| 1 | LMCS_APS | LMCS parameters |
| 2 | SCALING_APS | Scaling list parameters |
| 3..7 | Reserved | Reserved |

When sps_alf_enabled_flag is equal to 0, aps_params_type shall not be equal to ALF_APS

When sps_lmcs_enabled_flag is equal to 0, aps_params_type shall not be equal to LMCS_APS

When sps_scaling_list_enabled_flag is equal to 0, aps_params_type shall not be equal to SCALING_APS

Such constraints can be expressed as a bitstream conformance constraints.

# CCALF filters in ALF_APS

## Constraints added in ALF_APS semantics

In ALF_APS, it is proposed to add a constraint to cross component adaptive loop filters based on sps_ccalf_enabled_flag. When sps_ccalf_enabled_flag is equal to 0, an ALF_APS cannot contain any CCALF filter.

alf_cc_cb_filter_signal_flag equal to 1 specifies that cross-component filters for the Cb colour component are signalled. alf_cc_cb_filter_signal_flag equal to 0 specifies that cross-component filters for Cb colour component are not signalled. When ChromaArrayType sps_ccalf_enabled_flag is equal to 0, alf_cc_cb_filter_signal_flag shall be equal to 0.

alf_cc_cr_filter_signal_flag equal to 1 specifies that cross-component filters for the Cr colour component are signalled. alf_cc_cr_filter_signal_flag equal to 0 specifies that cross-component filters for the Cr colour component are not signalled. When ChromaArrayType sps_ccalf_enabled_flag is equal to 0, alf_cc_cr_filter_signal_flag shall be equal to 0.

## Constraints added in PH and SH semantics

PH:

ph_alf_aps_id_luma[ i ] specifies the adaptation_parameter_set_id of the i-th ALF APS that the luma component of the slices associated with the PH refers to.

The value of alf_luma_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to ph_alf_aps_id_luma[ i ] shall be equal to 1.

The TemporalId of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to ph_alf_aps_id_luma[ i ] shall be less than or equal to the TemporalId of the picture associated with the PH.

When sps_ccalf_enabled_flag is equal to 0, the values of alf_cc_cb_filter_signal_flag and alf_cc_cr_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to ph_alf_aps_id_luma[ i ] shall be equal to 0.

ph_alf_aps_id_chroma specifies the adaptation_parameter_set_id of the ALF APS that the chroma component of the slices associated with the PH refers to.

The value of alf_chroma_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to ph_alf_aps_id_chroma shall be equal to 1.

The TemporalId of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to ph_alf_aps_id_chroma shall be less than or equal to the TemporalId of the picture associated with the PH.

When sps_ccalf_enabled_flag is equal to 0, the values of alf_cc_cb_filter_signal_flag and alf_cc_cr_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to ph_alf_aps_id_chroma shall be equal to 0.

SH:

slice_alf_aps_id_luma[ i ] specifies the adaptation_parameter_set_id of the i-th ALF APS that the luma component of the slice refers to. The TemporalId of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_alf_aps_id_luma[ i ] shall be less than or equal to the TemporalId of the coded slice NAL unit. When slice_alf_enabled_flag is equal to 1 and slice_alf_aps_id_luma[ i ] is not present, the value of slice_alf_aps_id_luma[ i ] is inferred to be equal to the value of ph_alf_aps_id_luma[ i ].

The value of alf_luma_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_alf_aps_id_luma[ i ] shall be equal to 1.

When sps_ccalf_enabled_flag is equal to 0, the values of alf_cc_cb_filter_signal_flag and alf_cc_cr_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_alf_aps_id_luma[ i ] shall be equal to 0.

slice_alf_aps_id_chroma specifies the adaptation_parameter_set_id of the ALF APS that the chroma component of the slice refers to. The TemporalId of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_alf_aps_id_chroma shall be less than or equal to the TemporalId of the coded slice NAL unit. When slice_alf_enabled_flag is equal to 1 and slice_alf_aps_id_chroma is not present, the value of slice_alf_aps_id_chroma is inferred to be equal to the value of ph_alf_aps_id_chroma.

The value of alf_chroma_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_alf_aps_id_chroma shall be equal to 1.

When sps_ccalf_enabled_flag is equal to 0, the values of alf_cc_cb_filter_signal_flag and alf_cc_cr_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_alf_aps_id_chroma shall be equal to 0.

# Chroma deblocking

It is proposed to add a constraint for deblocking tc and beta offsets for Cb and Cr when chroma_format_idc indicated monochrome input. It can be done by semantic constraints to require the value of those syntax elements to be equal to 0 or a flag can be introduced to condition such offsets signalling. However, in VVC there is a flag to indicate chroma offset tools, so it is proposed to use this flag to conditionally signal the tc and beta offsets of chroma components.

PPS:

| if( !pps_deblocking_filter_disabled_flag ) { |  |
| --- | --- |
| pps_beta_offset_div2 | se(v) |
| pps_tc_offset_div2 | se(v) |
| if( pps_chroma_tool_offsets_present_flag ) { |  |
| pps_cb_beta_offset_div2 | se(v) |
| pps_cb_tc_offset_div2 | se(v) |
| pps_cr_beta_offset_div2 | se(v) |
| pps_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |

PH:

| if( !ph_deblocking_filter_disabled_flag ) { |  |
| --- | --- |
| ph_beta_offset_div2 | se(v) |
| ph_tc_offset_div2 | se(v) |
| if( ChromaArrayType != 0 ¦ ¦ pps_chroma_tool_offsets_present_flag ) { |  |
| ph_cb_beta_offset_div2 | se(v) |
| ph_cb_tc_offset_div2 | se(v) |
| ph_cr_beta_offset_div2 | se(v) |
| ph_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |

SH:

| if( !slice_deblocking_filter_disabled_flag ) { |  |
| --- | --- |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| if( ChromaArrayType != 0 ¦ ¦ pps_chroma_tool_offsets_present_flag ) { |  |
| slice_cb_beta_offset_div2 | se(v) |
| slice_cb_tc_offset_div2 | se(v) |
| slice_cr_beta_offset_div2 | se(v) |
| slice_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |

The semantics of pps_chroma_tool_offsets_present_flag is updated to reflect the usage in PH and SH as follows.

pps_chroma_tool_offsets_present_flag equal to 1 specifies that chroma tool offsets related syntax elements are present in the PPS RBSP syntax structure and the related syntax elements are present in PH and SH referring to the PPS. pps_chroma_tool_offsets_present_flag equal to 0 specifies that chroma tool offsets related syntax elements are not present in in the PPS RBSP, PH, and SH syntax structures. When ChromaArrayType is equal to 0, the value of pps_chroma_tool_offsets_present_flag shall be equal to 0.

# LMCS for monochrome content

Non-zero LMCS chroma scaling parameter lmcs_delta_abs_crs can be signalled for monochrome input, it is proposed to constraint the value to be equal to 0 for monochrome. Similar constraints are already done in the semantics, such alf_chroma_filter_signal_flag in ALF_APS already.

lmcs_delta_abs_crs specifies the absolute codeword value of the variable lmcsDeltaCrs. The value of lmcs_delta_abs_crs shall be in the range of 0 and 7, inclusive. When not present, lmcs_delta_abs_crs is inferred to be equal to 0. When ChromaArrayType is equal to 0, lmcs_delta_abs_crs shall be equal to 0.

Alternatively, a flag is signaled to indicate whether lmcs_delta_abs_crs is present in an LMCS_APS.

| lmcs_data( ) { | Descriptor |
| --- | --- |
| lmcs_min_bin_idx | ue(v) |
| lmcs_delta_max_bin_idx | ue(v) |
| lmcs_delta_cw_prec_minus1 | ue(v) |
| for( i = lmcs_min_bin_idx; i <= LmcsMaxBinIdx; i++ ) { |  |
| lmcs_delta_abs_cw[ i ] | u(v) |
| if( lmcs_delta_abs_cw[ i ] > 0 ) |  |
| lmcs_delta_sign_cw_flag[ i ] | u(1) |
| } |  |
| lmcs_cs_present_flag | u(1) |
| if( lmcs_cs_present_flag ) { |  |
| lmcs_delta_abs_crs | u(3) |
| if( lmcs_delta_abs_crs > 0 ) |  |
| lmcs_delta_sign_crs_flag | u(1) |
| } |  |
| } |  |

lmcs_cs_present_flag is equal to 1 specifies that lmcs_delta_abs_crs is present in LMCS_APS. lmcs_cs_present_flag is equal to 0 specifies that lmcs_delta_abs_crs is not present in LMCS_APS. When ChromaArrayType is equal to 0, lmcs_cs_present_flag shall be equal to 0.

lmcs_delta_abs_crs specifies the absolute codeword value of the variable lmcsDeltaCrs. The value of lmcs_delta_abs_crs shall be in the range of 0 and 7, inclusive. When not present, lmcs_delta_abs_crs is inferred to be equal to 0.

# Removing ChromaArrayType in PPS and APS

PPS:

pps_chroma_tool_offsets_present_flag equal to 1 specifies that chroma tool offsets related syntax elements are present in the PPS RBSP syntax structure. pps_chroma_tool_offsets_present_flag equal to 0 specifies that chroma tool offsets related syntax elements are not present in in the PPS RBSP syntax structure. When ChromaArrayType is equal to 0, the value of pps_chroma_tool_offsets_present_flag shall be equal to 0.

pps_cb_qp_offset and pps_cr_qp_offset specify the offsets to the luma quantization parameter Qp′Y used for deriving Qp′Cb and Qp′Cr, respectively. The values of pps_cb_qp_offset and pps_cr_qp_offset shall be in the range of −12 to +12, inclusive. When ChromaArrayType is equal to 0, pps_cb_qp_offset and pps_cr_qp_offset are not used in the decoding process and decoders shall ignore their value. When not present, the values of pps_cb_qp_offset and pps_cr_qp_offset are inferred to be equalt to 0.

pps_joint_cbcr_qp_offset_present_flag equal to 1 specifies that pps_joint_cbcr_qp_offset_value and joint_cbcr_qp_offset_list[ i ] are present in the PPS RBSP syntax structure. pps_joint_cbcr_qp_offset_present_flag equal to 0 specifies that pps_joint_cbcr_qp_offset_value and joint_cbcr_qp_offset_list[  i ] are not present in the PPS RBSP syntax structure. When ChromaArrayType is equal to 0 or sps_joint_cbcr_enabled_flag is equal to 0, the value of pps_joint_cbcr_qp_offset_present_flag shall be equal to 0. When not present, the value of pps_joint_cbcr_qp_offset_present_flag is inferred to be equal to 0.

pps_joint_cbcr_qp_offset_value specifies the offset to the luma quantization parameter Qp′Y used for deriving Qp′CbCr. The value of pps_joint_cbcr_qp_offset_value shall be in the range of −12 to +12, inclusive. When ChromaArrayType is equal to 0 or sps_joint_cbcr_enabled_flag is equal to 0, pps_joint_cbcr_qp_offset_value is not used in the decoding process and decoders shall ignore its value. When pps_joint_cbcr_qp_offset_present_flag is equal to 0, pps_joint_cbcr_qp_offset_value is not present and is inferred to be equal to 0.

APS:

alf_chroma_filter_signal_flag equal to 1 specifies that a chroma filter is signalled. alf_chroma_filter_signal_flag equal to 0 specifies that a chroma filter is not signalled. When ChromaArrayType is equal to 0, alf_chroma_filter_signal_flag shall be equal to 0.

alf_cc_cb_filter_signal_flag equal to 1 specifies that cross-component filters for the Cb colour component are signalled. alf_cc_cb_filter_signal_flag equal to 0 specifies that cross-component filters for Cb colour component are not signalled. When ChromaArrayType is equal to 0, alf_cc_cb_filter_signal_flag shall be equal to 0.

alf_cc_cr_filter_signal_flag equal to 1 specifies that cross-component filters for the Cr colour component are signalled. alf_cc_cr_filter_signal_flag equal to 0 specifies that cross-component filters for the Cr colour component are not signalled. When ChromaArrayType is equal to 0, alf_cc_cr_filter_signal_flag shall be equal to 0.

scaling_list_chroma_present_flag equal to 1 specifies that chroma scaling lists are present in scaling_list_data( ). scaling_list_chroma_present_flag equal to 0 specifies that chroma scaling lists are not present in scaling_list_data( ). It is a requirement of bitstream conformance that scaling_list_chroma_present_flag shall be equal to 0 when ChromaArrayType is equal to 0, and shall be equal to 1 when ChromaArrayType is not equal to 0.

After removing the constraint of ChromaArrayType in PPS, some chroma-related syntax elements can still be signalled in PH and SH. To fix it,

PH:

| if( cu_qp_delta_enabled_flag && ChromaArrayType != 0 ) |  |
| --- | --- |
| ph_cu_qp_delta_subdiv_intra_slice | ue(v) |
| if( pps_cu_chroma_qp_offset_list_enabled_flag && ChromaArrayType != 0 ) |  |
| ph_cu_chroma_qp_offset_subdiv_intra_slice | ue(v) |

| if( cu_qp_delta_enabled_flag && ChromaArrayType != 0 ) |  |
| --- | --- |
| ph_cu_qp_delta_subdiv_inter_slice | ue(v) |
| if( pps_cu_chroma_qp_offset_list_enabled_flag && ChromaArrayType != 0 ) |  |
| ph_cu_chroma_qp_offset_subdiv_inter_slice | ue(v) |

SH:

| if( pps_slice_chroma_qp_offsets_present_flag && ChromaArrayType != 0 ) { |  |
| --- | --- |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| if( sps_joint_cbcr_enabled_flag ) |  |
| slice_joint_cbcr_qp_offset | se(v) |
| } |  |
| if( pps_cu_chroma_qp_offset_list_enabled_flag && ChromaArrayType != 0 ) |  |
| cu_chroma_qp_offset_enabled_flag | u(1) |

# Removing BitDepth constraint from LMCS_APS semantics

LMCS_APS:

lmcs_delta_cw_prec_minus1 plus 1 specifies the number of bits used for the representation of the syntax lmcs_delta_abs_cw[ i ]. The value of lmcs_delta_cw_prec_minus1 shall be in the range of 0 to BitDepth − 214, inclusive.

PH:

ph_lmcs_aps_id specifies the adaptation_parameter_set_id of the LMCS APS that the slices associated with the PH refers to. The TemporalId of the APS NAL unit having aps_params_type equal to LMCS_APS and adaptation_parameter_set_id equal to ph_lmcs_aps_id shall be less than or equal to the TemporalId of the picture associated with PH.

The value of lmcs_delta_cw_prec_minus1 of the APS NAL unit having aps_params_type equal to LMCS_APS and adaptation_parameter_set_id equal to ph_lmcs_aps_id shall be in the range of 0 to BitDepth – 2, inclusive.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).