| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0236-v1 |
| --- | --- |

| Title: | Latency reduction in transformation process with TU-level signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hilmi E. Egilmez, Alican Nalci, Vadim Seregin, Wei-Jung Chien, Ted Hsieh, Marta Karczewicz 5775 Morehouse Dr., San Diego, CA, 92121, USA | Email: | hegilmez@qti.qualcomm.com analci@qti.qualcomm.com vseregin@qti.qualcomm.com wchien@qti.qualcomm.com thsieh@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In the current version of VVC (draft 8), the transform-related signaling is performed after residual coding at CU-level, and it depends on the positions of coded coefficients. This dependency inherently introduces additional latency, particularly critical for the two-stage LFNST process, since the parsing of LFNST index and the (inverse) transformation process can only start after coefficients from all color components are decoded. This contribution document proposes to signal the transform syntax elements earlier at the TU-level in order to alleviate the latency of the LFNST process. The experimental results under common test conditions (CTC) show that the proposed change has negligible impact in terms of average BD-rates.

# Introduction

In the VVC specification working draft (JVET-Q2001-vE), both MTS and LFNST indices (mts_idx and lfnst_idx) are signalled after residual coding of all color components at coding unit (CU) level as follows:

#### Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| chType = treeType = = DUAL_TREE_CHROMA ? 1 : 0 |  |
| if( slice_type != I ¦ ¦ sps_ibc_enabled_flag ) { |  |
| if( treeType != DUAL_TREE_CHROMA && ( ( !( cbWidth = = 4 && cbHeight = = 4 ) && modeType != MODE_TYPE_INTRA ) ¦ ¦ ( sps_ibc_enabled_flag && cbWidth <= 64 && cbHeight <= 64 ) ) ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 && slice_type != I && !( cbWidth = = 4 && cbHeight = = 4 ) && modeType = = MODE_TYPE_ALL ) |  |
| pred_mode_flag | ae(v) |

…

| if( sps_act_enabled_flag && CuPredMode[ chType ][ x0 ][ y0 ] != MODE_INTRA && treeType = = SINGLE_TREE ) |  |
| --- | --- |
| cu_act_enabled_flag | ae(v) |
| LfnstDcOnly = 1 |  |
| LfnstZeroOutSigCoeffFlag = 1 |  |
| MtsDcOnly = 1 |  |
| MtsZeroOutSigCoeffFlag = 1 |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType, chType ) |  |
| lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : ( ( IntraSubPartitionsSplitType = = ISP_VER_SPLIT ) ? cbWidth / NumIntraSubPartitions : cbWidth ) |  |
| lfnstHeight = ( treeType = = DUAL_TREE_CHROMA ) ? cbHeight / SubHeightC : ( ( IntraSubPartitionsSplitType = = ISP_HOR_SPLIT) ? cbHeight / NumIntraSubPartitions : cbHeight ) |  |
| lfnstNotTsFlag = ( treeType = = DUAL_TREE_CHROMA ¦ ¦ transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 ) && ( treeType = = DUAL_TREE_LUMA ¦ ¦ ( transform_skip_flag[ x0 ][ y0 ][ 1 ] = = 0 && transform_skip_flag[ x0 ][ y0 ][ 2 ] = = 0 ) ) |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && lfnstNotTsFlag = = 1 && ( treeType = = DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |
| if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx | ae(v) |
| } |  |
| if( treeType != DUAL_TREE_CHROMA && lfnst_idx = = 0 && transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 && Max( cbWidth, cbHeight ) <= 32 && IntraSubPartitionsSplitType = = ISP_NO_SPLIT && cu_sbt_flag = = 0 && MtsZeroOutSigCoeffFlag = = 1 && MtsDcOnly = = 0 ) { |  |
| if( ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag ) ) ) |  |
| mts_idx | ae(v) |
| } |  |
| } |  |
| } |  |

Note that the current transform signaling design requires all coefficients from all components to be decoded first, and then the lfnst_idx and mts_idx syntax elements are parsed. This can introduce excessive latency  in certain common decoder pipelines (especially in LFNST) as the inverse transformation process cannot start before decoding coefficients from all components. This contribution addresses this problem by signaling necessary transform syntax elements earlier at TU-level. The following section describes the proposed changes specifically.

# Proposed Method

In order to reduce the latency in transformation process, this document simply to aims to move signaling of the transform syntax elements (i.e., lfnst_idx and mts_idx) so that they could be parsed rigthafter coefficients of necessary color components are decoded in a TU. To achieve this, the following changes are proposed for coding luma and chroma TBs:

- Luma TB coding: In coding luma blocks, the lfnst_idx and mts_idx syntax elements are signaled rightafter all necessary luma coefficients are parsed. Accordingly, if ISP is used, the lfnst_idx is signaled rightafter luma coefficients from all TUs are parsed (i.e., lfnst_index is signaled with the last TU obtained via ISP).

- Chroma TB coding: In coding chroma blocks, lfnst_idx is needed only for separate-tree (dual-tree) partitioning mode (in all other cases both MTS and LFNST are disabled and not signalled). Therefore, the lfnst_idx signaling is done rightafter coefficients of both Cr and Cb components are parsed.

Note that JVET-R0235 proposes to remove LFNST for chroma in separate-tree partitioning mode to further reduce complexity and harmonize with the existing VVC design, where LFNST for chroma is disabled in single-tree partitioning mode. If LFNST for chroma is completely disabled, the item 2) discussed above becomes unnecessary and would be trivially removed.

Specifically, the proposed changes can be reflected on top of the VVC specification working draft (JVET-Q2001-vE) as follows, where (i) changes highlighted in green show the changes related to moving syntax elements from CU-level to TU-level, and (ii) changes highlighted in yellow indicate the additional changes needed (e.g., ones related to handle the case where ISP is used):

#### Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| chType = treeType = = DUAL_TREE_CHROMA ? 1 : 0 |  |
| if( slice_type != I ¦ ¦ sps_ibc_enabled_flag ) { |  |
| if( treeType != DUAL_TREE_CHROMA && ( ( !( cbWidth = = 4 && cbHeight = = 4 ) && modeType != MODE_TYPE_INTRA ) ¦ ¦ ( sps_ibc_enabled_flag && cbWidth <= 64 && cbHeight <= 64 ) ) ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 && slice_type != I && !( cbWidth = = 4 && cbHeight = = 4 ) && modeType = = MODE_TYPE_ALL ) |  |
| pred_mode_flag | ae(v) |
| … |  |
| if( sps_act_enabled_flag && CuPredMode[ chType ][ x0 ][ y0 ] != MODE_INTRA && treeType = = SINGLE_TREE ) |  |
| cu_act_enabled_flag | ae(v) |
| LfnstDcOnly = 1 |  |
| LfnstZeroOutSigCoeffFlag = 1 |  |
| MtsDcOnly = 1 |  |
| MtsZeroOutSigCoeffFlag = 1 |  |
| ISPTuFlag = 1 |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType, chType ) |  |
| lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : ( ( IntraSubPartitionsSplitType = = ISP_VER_SPLIT ) ? cbWidth / NumIntraSubPartitions : cbWidth ) |  |
| lfnstHeight = ( treeType = = DUAL_TREE_CHROMA ) ? cbHeight / SubHeightC : ( ( IntraSubPartitionsSplitType = = ISP_HOR_SPLIT) ? cbHeight / NumIntraSubPartitions : cbHeight ) |  |
| lfnstNotTsFlag = ( treeType = = DUAL_TREE_CHROMA ¦ ¦ transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 ) && ( treeType = = DUAL_TREE_LUMA ¦ ¦ ( transform_skip_flag[ x0 ][ y0 ][ 1 ] = = 0 && transform_skip_flag[ x0 ][ y0 ][ 2 ] = = 0 ) ) |  |

| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && lfnstNotTsFlag = = 1 && ( treeType = = DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |
| --- | --- |
| if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx | ae(v) |
| } |  |
| if( treeType != DUAL_TREE_CHROMA && lfnst_idx = = 0 && transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 && Max( cbWidth, cbHeight ) <= 32 && IntraSubPartitionsSplitType = = ISP_NO_SPLIT && cu_sbt_flag = = 0 && MtsZeroOutSigCoeffFlag = = 1 && MtsDcOnly = = 0 ) { |  |
| if( ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag ) ) ) |  |
| mts_idx | ae(v) |
| } |  |
| } |  |
| } |  |

#### Transform unit syntax

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex, chType ) { | Descriptor |
| --- | --- |
| if( IntraSubPartitionsSplitType != ISP_NO_SPLIT && treeType = = SINGLE_TREE && subTuIndex = = NumIntraSubPartitions − 1 ) { |  |
| xC = CbPosX[ chType ][ x0 ][ y0 ] |  |
| yC = CbPosY[ chType ][ x0 ][ y0 ] |  |
| wC = CbWidth[ chType ][ x0 ][ y0 ] / SubWidthC |  |
| hC = CbHeight[ chType ][ x0 ][ y0 ] / SubHeightC |  |
| } else { |  |
| xC = x0 |  |
| yC = y0 |  |
| wC = tbWidth / SubWidthC |  |
| hC = tbHeight / SubHeightC |  |
| } |  |
| … |  |
| if( ( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) && ChromaArrayType != 0 && ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && ( ( subTuIndex = = 0 && cu_sbt_pos_flag ) ¦ ¦ ( subTuIndex = = 1 && !cu_sbt_pos_flag ) ) ) ) ¦ ¦ ( IntraSubPartitionsSplitType != ISP_NO_SPLIT && ( subTuIndex = = NumIntraSubPartitions − 1 ) ) ) { |  |
| tu_cb_coded_flag[ xC ][ yC ] | ae(v) |
| tu_cr_coded_flag[ xC ][ yC ] | ae(v) |
| } |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !( cu_sbt_flag && ( ( subTuIndex = = 0 && cu_sbt_pos_flag ) ¦ ¦ ( subTuIndex = = 1 && !cu_sbt_pos_flag ) ) ) && ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA ¦ ¦ ( chromaAvailable && ( tu_cb_coded_flag[ xC ][ yC ] ¦ ¦ tu_cr_coded_flag[ xC ][ yC ] ) ) ¦ ¦ CbWidth[ chType ][ x0 ][ y0 ] > MaxTbSizeY ¦ ¦ CbHeight[ chType ][ x0 ][ y0 ] > MaxTbSizeY ) ) ¦ ¦ ( IntraSubPartitionsSplitType != ISP_NO_SPLIT && ( subTuIndex < NumIntraSubPartitions − 1 ¦ ¦ !InferTuCbfLuma ) ) ) |  |
| tu_y_coded_flag[ x0 ][ y0 ] | ae(v) |
| if(IntraSubPartitionsSplitType != ISP_NO_SPLIT ) |  |
| InferTuCbfLuma = InferTuCbfLuma && !tu_y_coded_flag[ x0 ][ y0 ] |  |
| } |  |
| if (IntraSubPartitionsSplitType != ISP_NO_SPLIT && subTuIndex = = NumIntraSubPartitions − 1) |  |
| ISPTuFlag = 0 |  |
| … |  |
| if( sps_joint_cbcr_enabled_flag && ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && ( tu_cb_coded_flag[ xC ][ yC ] ¦ ¦ tu_cr_coded_flag[ xC ][ yC ] ) ) ¦ ¦ ( tu_cb_coded_flag[ xC ][ yC ] && tu_cr_coded_flag[ xC ][ yC ] ) ) && chromaAvailable ) |  |
| tu_joint_cbcr_residual_flag[ xC ][ yC ] | ae(v) |
| if( tu_y_coded_flag[ x0 ][ y0 ] && treeType != DUAL_TREE_CHROMA ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 0 ] && tbWidth <= MaxTsSize && tbHeight <= MaxTsSize && ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT ) && !cu_sbt_flag ) |  |
| transform_skip_flag[ x0 ][ y0 ][ 0 ] | ae(v) |
| if( !transform_skip_flag[ x0 ][ y0 ][ 0 ] ¦ ¦ slice_ts_residual_coding_disabled_flag ) |  |
| residual_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| else |  |
| residual_ts_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| } |  |
| lfnstWidth = tbWidth ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : ( ( IntraSubPartitionsSplitType = = ISP_VER_SPLIT ) ? cbWidth / NumIntraSubPartitions : cbWidth ) |  |
| lfnstHeight = tbHeight ( treeType = = DUAL_TREE_CHROMA ) ? cbHeight / SubHeightC : ( ( IntraSubPartitionsSplitType = = ISP_HOR_SPLIT) ? cbHeight / NumIntraSubPartitions : cbHeight ) |  |
| if(treeType != DUAL_TREE_CHROMA ) { |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 lfnstNotTsFlag = = 1 && ( treeType = = DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && Max(CbWidth[ chType ][ x0 ][ y0 ]cbWidth, CbHeight[ chType ][ x0 ][ y0 ]cbHeight ) <= MaxTbSizeY) && (IntraSubPartitionsSplitType == ISP_NO_SPLIT ¦ ¦ ISPTuFlag = = 0) { |  |
| if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx | ae(v) |
| } |  |
| if( treeType != DUAL_TREE_CHROMA && lfnst_idx = = 0 && transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 && Max(CbWidth[ chType ][ x0 ][ y0 ]cbWidth, CbHeight[ chType ][ x0 ][ y0 ]cbHeight) <= 32 && IntraSubPartitionsSplitType = = ISP_NO_SPLIT && cu_sbt_flag = = 0 && MtsZeroOutSigCoeffFlag = = 1 && MtsDcOnly = = 0 ) { |  |
| if( ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag ) ) ) |  |
| mts_idx | ae(v) |
| } |  |
| } |  |
| if( tu_cb_coded_flag[ xC ][ yC ] && treeType != DUAL_TREE_LUMA ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 1 ] && wC <= MaxTsSize && hC <= MaxTsSize && !cu_sbt_flag ) |  |
| transform_skip_flag[ xC ][ yC ][ 1 ] | ae(v) |
| if( !transform_skip_flag[ xC ][ yC ][ 1 ] ¦ ¦ slice_ts_residual_coding_disabled_flag ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| else |  |
| residual_ts_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| } |  |
| if( tu_cr_coded_flag[ xC ][ yC ] && treeType != DUAL_TREE_LUMA && !( tu_cb_coded_flag[ xC ][ yC ] && tu_joint_cbcr_residual_flag[ xC ][ yC ] ) ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 2 ] && wC <= MaxTsSize && hC <= MaxTsSize && !cu_sbt_flag ) |  |
| transform_skip_flag[ xC ][ yC ][ 2 ] | ae(v) |
| if( !transform_skip_flag[ xC ][ yC ][ 2 ] ¦ ¦ slice_ts_residual_coding_disabled_flag ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| else |  |
| residual_ts_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| } |  |
| if(treeType = = DUAL_TREE_CHROMA ) { |  |
| lfnstNotTsFlag = ( treeType = = DUAL_TREE_CHROMA ¦ ¦ transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 ) && ( treeType = = DUAL_TREE_LUMA ¦ ¦ ( transform_skip_flag[ x0 ][ y0 ][ 1 ] = = 0 && transform_skip_flag[ x0 ][ y0 ][ 2 ] = = 0 ) ) |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && lfnstNotTsFlag = = 1 && ( treeType = = DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && Max(CbWidth[ chType ][ x0 ][ y0 ]cbWidth, CbHeight[ chType ][ x0 ][ y0 ]cbHeight ) <= MaxTbSizeY) { |  |
| if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx | ae(v) |
| } |  |
| } |  |
| } |  |

# Experimental Results

The proposed method is implemented on top of VTM-8.0 and evaluated under common test conditions (CTC). Note that low-delay B results only show the effect of MTS (which produces identical results as anchor), since LFNST is disabled in CTC for low-delay B. Table 1 presents the results of proposed changes discussed in the previous section.

Table 1.  Results of the proposed method over VTM-8.0 under CTC

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | 0.00% | 103% | 102% |
| Class A2 | 0.00% | -0.03% | -0.01% | 100% | 99% |
| Class B | 0.00% | -0.01% | 0.06% | 102% | 104% |
| Class C | -0.01% | 0.00% | -0.04% | 99% | 94% |
| Class E | -0.01% | 0.01% | -0.04% | 100% | 98% |
| Overall | 0.00% | -0.01% | 0.00% | 101% | 100% |
| Class D | -0.01% | -0.01% | -0.10% | 102% | 102% |
| Class F | -0.02% | 0.09% | -0.04% | 101% | 101% |
|  | Random Access |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.02% | 0.00% | 102% | 101% |
| Class A2 | -0.01% | 0.05% | 0.11% | 98% | 96% |
| Class B | 0.00% | -0.05% | 0.05% | 102% | 101% |
| Class C | -0.01% | 0.01% | -0.24% | 99% | 96% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.00% | -0.02% | 100% | 99% |
| Class D | 0.03% | 0.16% | -0.26% | 102% | 103% |
| Class F | 0.03% | 0.03% | -0.09% | 102% | 101% |

|  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 97% | 96% |
| Class E | 0.00% | 0.00% | 0.00% | 98% | 95% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 97% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 103% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 99% |

# Conclusions

This document proposes to move MTS and LFNST index syntax elements to TU-level, and signaling is done rightafter necessary coefficient information from luma/chroma components is parsed. The proposed method reduces the parsing latency, which is especially critical for the two-stage LFSNT process. The proposed changes have negligible impact in terms of BD-rates. It is recommended to adopt the proposed method in VVC.

The file “JVET-Q2001-vE_JVET-R0236-spec.docx” attached to this submission includes the proposed changes over the VVC specification working draft (JVET-Q2001-vE).

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 14th Meeting, Geneva, CH, 19–27 Mar. 2019.

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 8),” JVET-Q2001, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 17th Meeting, Brussels, BE, 7–17 Jan. 2020.

- H.E. Egilmez, A. Nalci, M. Coban, V. Seregin, M. Karczewicz, “Chroma LFNST Simplification and Signaling,” JVET-Q0686, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 17th Meeting, Brussels, BE, 7–17 Jan. 2020.

- H.E. Egilmez, A. Nalci, A.K. Ramasubramonian, M. Coban, V. Seregin, M. Karczewicz, T. Hashimoto, T. Chujoh, E. Sasaki, T. Ikai, “Combination of JVET-Q0106, JVET-0686 and JVET-Q0133 for LFNST Signaling, Latency Reduction and Scaling Process,” JVET-Q0784, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 17th Meeting, Brussels, BE, 7–17 Jan. 2020.

- H.E. Egilmez, A. Nalci, V. Seregin, W.-J. Chien, T. Hsieh, M. Karczewicz, “Removal of LFNST for chroma chroma components,” JVET-R0235, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 18th Meeting, 15–24 April 2020.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).