| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0262-v2 |
| --- | --- |

| Title: | AHG9: On PPS syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He Vadim Seregin Muhammed Coban Marta Karczewicz | Email: | YongHe@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes following PPS syntax element changes for such typical no resolution change case.

- Propose the value of pps_conformance_window_flag to be equal to 0 when the PPS picture width and height are the same as the SPS maximum picture width and height

- Propose to infer the values of pps_conf_win_left_offset, pps_conf_win_right_offset, pps_conf_win_top_offset, and pps_conf_win_bottom_offset when the value of pps_conformance_window_flag is 0

- Propose pps_res_change_allowed_flag to condition the presence of PPS conformance window and scaling window syntax elements.

# Proposal

## PPS conformance window

In VVC draft 8, the constraint on conformance window is specified as follows:

When pic_width_in_luma_samples is equal to pic_width_max_in_luma_samples and pic_height_in_luma_samples is equal to pic_height_max_in_luma_samples, it is a requirement of bitstream conformance that pps_conf_win_left_offset, pps_conf_win_right_offset, pps_conf_win_top_offset, and pps_conf_win_bottom_offset, are equal to sps_conf_win_left_offset, sps_conf_win_right_offset, sps_conf_win_top_offset, and sps_conf_win_bottom_offset, respectively.

The semantic of conformance window flag and offset are as follows:

pps_conformance_window_flag equal to 1 indicates that the conformance cropping window offset parameters follow next in the PPS. pps_conformance_window_flag equal to 0 indicates that the conformance cropping window offset parameters are not present in the PPS. pps_conf_win_left_offset, pps_conf_win_right_offset, pps_conf_win_top_offset, and pps_conf_win_bottom_offset specify the samples of the pictures in the CLVS that are output from the decoding process, in terms of a rectangular region specified in picture coordinates for output. When pps_conformance_window_flag is equal to 0, the values of pps_conf_win_left_offset, pps_conf_win_right_offset, pps_conf_win_top_offset, and pps_conf_win_bottom_offset are inferred to be equal to 0.

When there is no resolution changes and conformance window offset is not 0, the same conformance window offset values are signaled in both SPS and PPS redundantly.

It is proposed to modify the semantic of pps_conformance_window_flag and conformance window offset syntax elements as follows:

pps_conformance_window_flag equal to 1 indicates that the conformance cropping window offset parameters follow next in the PPS. pps_conformance_window_flag equal to 0 indicates that the conformance cropping window offset parameters are not present in the PPS. pps_conf_win_left_offset, pps_conf_win_right_offset, pps_conf_win_top_offset, and pps_conf_win_bottom_offset specify the samples of the pictures in the CLVS that are output from the decoding process, in terms of a rectangular region specified in picture coordinates for output. When pps_conformance_window_flag is equal to 0, the values of pps_conf_win_left_offset, pps_conf_win_right_offset, pps_conf_win_top_offset, and pps_conf_win_bottom_offset are inferred to be equal to 0 sps_conf_win_left_offset, sps_conf_win_right_offset, sps_conf_win_top_offset, and sps_conf_win_bottom_offset, respectively.

The bitstream conformance constraint can be simplified as below:

When pic_width_in_luma_samples is equal to pic_width_max_in_luma_samples and pic_height_in_luma_samples is equal to pic_height_max_in_luma_samples, it is a requirement of bitstream conformance that pps_conformance_window_flag is equal to 0 pps_conf_win_left_offset, pps_conf_win_right_offset, pps_conf_win_top_offset, and pps_conf_win_bottom_offset, are equal to sps_conf_win_left_offset, sps_conf_win_right_offset, sps_conf_win_top_offset, and sps_conf_win_bottom_offset, respectively.

## pps_res_change_allowed_flag

In VVC draft 8, It is bitstream conformance requirement that the PPS picture resolution shall be equal to the SPS max resolution, PPS conformance window offset shall be equal to SPS conformance window offset, and scaling_window_explicit_signalling_flag shall be equal to 0 when no resolution change in CLVS as follows.

- When res_change_in_clvs_allowed_flag equal to 0, the value of pic_width_in_luma_samples shall be equal to pic_width_max_in_luma_samples.

- When res_change_in_clvs_allowed_flag equal to 0, the value of pic_height_in_luma_samples shall be equal to pic_height_max_in_luma_samples.

- When pic_width_in_luma_samples is equal to pic_width_max_in_luma_samples and pic_height_in_luma_samples is equal to pic_height_max_in_luma_samples, it is a requirement of bitstream conformance that pps_conf_win_left_offset, pps_conf_win_right_offset, pps_conf_win_top_offset, and pps_conf_win_bottom_offset, are equal to sps_conf_win_left_offset, sps_conf_win_right_offset, sps_conf_win_top_offset, and sps_conf_win_bottom_offset, respectively.

- When res_change_in_clvs_allowed_flag is equal to 0, the value of scaling_window_explicit_signalling_flag shall be equal to 0.

It is proposed to add a resolution change allowed flag to indicate a typical no resolution change coding case, and the conformance window and scaling window are not explicitly signaled in PPS for such typical case.

The proposed PPS syntax is shown in Table 1.

Table 1 Proposed PPS syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | u(4) |
| mixed_nalu_types_in_pic_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| pps_res_change_allowed_flag | u(1) |
| if( pps_res_change_allowed_flag ) { |  |
| pic_height_in_luma_samples | ue(v) |
| pps_conformance_window_flag | u(1) |
| if( pps_conformance_window_flag ) { |  |
| pps_conf_win_left_offset | ue(v) |
| pps_conf_win_right_offset | ue(v) |
| pps_conf_win_top_offset | ue(v) |
| pps_conf_win_bottom_offset | ue(v) |
| } |  |
| if( scaling_window_explicit_signalling_flag ) { |  |
| scaling_win_left_offset |  |
| scaling_win_right_offset |  |
| scaling_win_top_offset |  |
| scaling_win_bottom_offset |  |
| } |  |
| } |  |
| … |  |
| } |  |

pps_res_change_allowed_flag equal to 1 specifies that the picture spatial resolution may change on the picture referring to the PPS. pps_res_change_allowed_flag equal to 0 specifies that the picture spatial resolution does not change on the picture referring to the PPS

When the value of res_change_in_clvs_allowed_flag is equal to 0, the value of pps_res_change_allowed_flag shall be equal to 0.

pps_conformance_window_flag equal to 1 indicates that the conformance cropping window offset parameters follow next in the PPS. pps_conformance_window_flag equal to 0 indicates that the conformance cropping window offset parameters are not present in the PPS. When not present, the value of pps_conformance_window_flag is inferred to be equal to 0.

pps_conf_win_left_offset, pps_conf_win_right_offset, pps_conf_win_top_offset, and pps_conf_win_bottom_offset specify the samples of the pictures in the CLVS that are output from the decoding process, in terms of a rectangular region specified in picture coordinates for output. When pps_conformance_window_flag is equal to 0, the values of pps_conf_win_left_offset, pps_conf_win_right_offset, pps_conf_win_top_offset, and pps_conf_win_bottom_offset are inferred to be equal to 0 sps_conf_win_left_offset, sps_conf_win_right_offset, sps_conf_win_top_offset, and sps_conf_win_bottom_offset, respectively.

scaling_window_explicit_signalling_flag equal to 1 specifies that the scaling window offset parameters are present in the PPS. scaling_window_explicit_signalling_flag equal to 0 specifies that the scaling window offset parameters are not present in the PPS. When not present, the value of scaling_window_explicit_signalling_flag  is inferred to be equal to 0. When res_change_in_clvs_allowed_flag is equal to 0, the value of scaling_window_explicit_signalling_flag shall be equal to 0.

Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).