| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0268-v3 |
| --- | --- |

| Title: | Implicit binary split at picture boundary |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guichun Li, Xiang Li, Shan Liu 2747 Park Boulevard, Palo Alto, CA 94306, U.S.A. | Tel: Email: | +1-650-798-3300 guichunli@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In VVC draft 8, implicit binary split is used only inside a CTU but not at CTU level, which looks an inconsistent design and leads to worse performance. This contribution proposes a simple change to always allow implicit binary split when the current coding block is across picture boundary and block size is smaller than or equal to minimum allowed QT size. When intra dual tree is used, the dual tree implicit QT split remains unchanged. The method was implemented on top of VTM-8.0. And average BD-rate results with min QT size of 128 luma samples of the proposed method compared with VTM-8.0 are as listed below:

- Intra dual tree on: AI -2.63%, RA -4.81%, LB -6.01%;

- Intra dual tree off: AI -2.89%, RA -4.94%, LB -6.05%.

# Problem Statement

In current VVC draft, an implicit QT split method was previously adopted as in JVET-Q0330. It is applied when a CU is across the picture boundary and under one of the following conditions

- A CTU block exceeds picture boundary and both the CTU size and min QT size are set to 128;

- A block exceeds picture boundary and its size is not larger than min QT size, and min QT size is larger than the min CU size and the max BT/TT depth is equal to 0.

The split_qt_flag is inferred to be equal to 1 under any of above conditions.

While the method solves the problem of CU splitting at picture boundaries, only allow QT split under such conditions may not be the optimal solution in terms of coding efficiency. Moreover, it introduces inconsistent design with implicit BT split.

# Proposed Method

It is proposed to improve design consistency by allowing implicit binary split up to CTU level when the current coding block is across picture boundary, similar to the current implicit QT splitting design. In the proposed method, implicit binary tree split is done by conditionally setting the initial value of the multi-type tree depth offset according to whether the current coding block (including CTU level) is across the picture boundary. The multi-type tree depth offset is added to the current maximum multi-type tree depth (e.g. MaxMttDepthY in case of single tree or dual-tree luma case, or MaxMttDepthC in case of dual tree chroma case), and the result will be used as the new maximum multi-type tree depth to determine whether the binary split is allowed. When intra dual tree is used, the dual tree implicit QT split remains unchanged.

## Proposed Spec Text Change

The spec text changes are marked with yellow highlights for additional text, and red strike lines on deleted text.

The coding tree unit syntax table may be changed as followings:

| coding_tree_unit( ) { | Descriptor |
| --- | --- |
| xCtb = CtbAddrX << CtbLog2SizeY |  |
| yCtb = CtbAddrY << CtbLog2SizeY |  |
| if( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ) |  |
| sao( CtbAddrX, CtbAddrY ) |  |
| if( slice_alf_enabled_flag ){ |  |
| alf_ctb_flag[ 0 ][ CtbAddrX ][ CtbAddrY ] | ae(v) |
| if( alf_ctb_flag[ 0 ][ CtbAddrX ][ CtbAddrY ] ) { |  |
| if( slice_num_alf_aps_ids_luma > 0 ) |  |
| alf_use_aps_flag | ae(v) |
| if( alf_use_aps_flag ) { |  |
| if( slice_num_alf_aps_ids_luma > 1 ) |  |
| alf_luma_prev_filter_idx | ae(v) |
| } else |  |
| alf_luma_fixed_filter_idx | ae(v) |
| } |  |
| if( slice_alf_chroma_idc = = 1 ¦ ¦ slice_alf_chroma_idc = = 3 ) { |  |
| alf_ctb_flag[ 1 ][ CtbAddrX ][ CtbAddrY ] | ae(v) |
| if( alf_ctb_flag[ 1 ][ CtbAddrX ][ CtbAddrY ] && alf_chroma_num_alt_filters_minus1 > 0 ) |  |
| alf_ctb_filter_alt_idx[ 0 ][ CtbAddrX ][ CtbAddrY ] | ae(v) |
| } |  |
| if( slice_alf_chroma_idc = = 2 ¦ ¦ slice_alf_chroma_idc = = 3 ) { |  |
| alf_ctb_flag[ 2 ][ CtbAddrX ][ CtbAddrY ] | ae(v) |
| if( alf_ctb_flag[ 2 ][ CtbAddrX ][ CtbAddrY ] && alf_chroma_num_alt_filters_minus1 > 0 ) |  |
| alf_ctb_filter_alt_idx[ 1 ][ CtbAddrX ][ CtbAddrY ] | ae(v) |
| } |  |
| } |  |
| if( slice_cc_alf_cb_enabled_flag ) |  |
| alf_ctb_cc_cb_idc[ CtbAddrX ][ CtbAddrY ] | ae(v) |
| if( slice_cc_alf_cr_enabled_flag ) |  |
| alf_ctb_cc_cr_idc[ CtbAddrX ][ CtbAddrY ] | ae(v) |
| x1 = xCtb + CtbSizeY |  |
| y1 = yCtb + CtbSizeY |  |
| mttDepthOffset = (x1 > pic_width_in_luma_samples ¦¦ y1 > pic_height_in_luma_samples ) ? 1 : 0 |  |
| if( slice_type = = I && qtbtt_dual_tree_intra_flag ) |  |
| dual_tree_implicit_qt_split( xCtb, yCtb, CtbSizeY, 0, mttDepthOffset ) |  |
| else { |  |
| coding_tree( xCtb, yCtb, CtbSizeY, CtbSizeY, 1, 1, 0, 0, 0, mttDepthOffset, 0, SINGLE_TREE, MODE_TYPE_ALL ) |  |
| } |  |
| } |  |

The dual tree implicit QT split syntax table may be changed as followings:

| dual_tree_implicit_qt_split( x0, y0, cbSize, cqtDepth, depthOffset ) { | Descriptor |
| --- | --- |
| cbSubdiv = 2 * cqtDepth |  |
| if( cbSize > 64 ) { |  |
| if( cu_qp_delta_enabled_flag && cbSubdiv <= CuQpDeltaSubdiv ) { |  |
| IsCuQpDeltaCoded = 0 |  |
| CuQpDeltaVal = 0 |  |
| CuQgTopLeftX = x0 |  |
| CuQgTopLeftY = y0 |  |
| } |  |
| if( cu_chroma_qp_offset_enabled_flag && cbSubdiv <= CuChromaQpOffsetSubdiv ) { |  |
| IsCuChromaQpOffsetCoded = 0 |  |
| CuQpOffsetCb = 0 |  |
| CuQpOffsetCb = 0 |  |
| CuQpOffsetCbCr = 0 |  |
| } |  |
| dual_tree_implicit_qt_split( x0, y0, cbSize / 2, cqtDepth + 1, depthOffset ) |  |
| if( x1 < pic_width_in_luma_samples ) |  |
| dual_tree_implicit_qt_split( x1, y0, cbSize / 2, cqtDepth + 1, depthOffset ) |  |
| if( y1 < pic_height_in_luma_samples ) |  |
| dual_tree_implicit_qt_split( x0, y1, cbSize / 2, cqtDepth + 1, depthOffset ) |  |
| if( x1 < pic_width_in_luma_samples && y1 < pic_height_in_luma_samples ) |  |
| dual_tree_implicit_qt_split( x1, y1, cbSize / 2, cqtDepth + 1, depthOffset ) |  |
| } else { |  |
| coding_tree( x0, y0, cbSize, cbSize, 1, 0, cbSubdiv, cqtDepth, 0, 0depthOffset, 0, DUAL_TREE_LUMA, MODE_TYPE_ALL ) |  |
| coding_tree( x0, y0, cbSize, cbSize, 0, 1, cbSubdiv, cqtDepth, 0, 0depthOffset, 0, DUAL_TREE_CHROMA, MODE_TYPE_ALL ) |  |
| } |  |
| } |  |

# Simulation Results

The proposed method was implemented on top of VTM-8.0. The VTM-8.0 with min QT size of 128 is used as anchor.

As shown in Table 3, it was observed that when min QT size is set to 128, the proposed method has average BD-rate gain of -2.63% / -4.81% / -6.01% on AI / RA / LB respectively.

Table 3-Test results compared with VTM-8.0

|  |  |  | All Intra Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-8.0 MinQT 128 |  |  |  |
|  | Y | U | V | EncT | DecT | YUV |
| Class A1 | -0.74% | 0.71% | 0.46% | 108% | 98% | -0.28% |
| Class A2 | -0.21% | -0.08% | -0.04% | 106% | 97% | -0.17% |
| Class B | -0.22% | -0.31% | -0.26% | 107% | 96% | -0.24% |
| Class C | -9.98% | 1.62% | 1.41% | 139% | 101% | -6.92% |
| Class E | -1.14% | -1.11% | -1.04% | 112% | 98% | -1.12% |
| Overall | -2.63% | 0.19% | 0.14% | 114% | 98% | -1.87% |
| Class D | -16.54% | 3.16% | 2.41% | 130% | 100% | -12.90% |
| Class F | -18.06% | 0.05% | 0.12% | 121% | 99% | -7.95% |
|  |  |  | Random access Main10 |  |  |  |
|  |  |  | Over VTM-8.0 MinQT 128 |  |  |  |
|  | Y | U | V | EncT | DecT | YUV |
| Class A1 | -1.04% | -0.58% | -0.79% | 109% | 101% | -0.89% |
| Class A2 | -0.16% | -0.06% | -0.09% | 104% | 100% | -0.14% |
| Class B | -0.68% | -0.65% | -0.72% | 104% | 95% | -0.68% |
| Class C | -16.30% | -3.69% | -3.66% | 123% | 99% | -12.36% |
| Class E |  |  |  |  |  |  |
| Overall | -4.81% | -1.33% | -1.39% | 110% | 98% | -3.73% |
| Class D | -25.14% | -4.11% | -3.78% | 122% | 98% | -19.98% |
| Class F | -20.38% | -4.15% | -4.04% | 106% | 95% | -10.97% |
|  |  |  | Low delay B Main10 |  |  |  |
|  |  |  | Over VTM-8.0 MinQT 128 |  |  |  |
|  | Y | U | V | EncT | DecT | YUV |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.62% | -0.91% | -0.97% | 106% | 100% | -0.70% |
| Class C | -15.51% | -3.11% | -3.10% | 122% | 96% | -12.07% |
| Class E | -2.34% | -1.28% | -1.84% | 103% | 95% | -2.15% |
| Overall | -6.01% | -1.73% | -1.90% | 110% | 97% | -4.85% |
| Class D | -24.23% | -3.91% | -4.71% | 118% | 97% | -20.11% |
| Class F | -13.74% | -5.13% | -4.74% | 105% | 96% | -10.39% |

Additional test with min QT size of 128 and intra dual tree off is tested. VTM-8.0 with min QT size of 128 and dual tree off is used as anchor. Results are as shown in table-4, the proposed method has average BD-rate gain of -2.89% / -4.94% / -6.05% on AI / RA / LB respectively. Note that there is an encoder bug is to be fixed in this case as VTM-8 encoder skips all intra mode checking when CU is larger than 64x64. In special case, intra mode is checked for such CUs if no further split is allowed.

Table 4 – Intra Dual Tree off test results compared with VTM-8.0

|  |  |  | All Intra Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-8.0 |  |  |  |
|  | Y | U | V | EncT | DecT | YUV |
| Class A1 | -0.91% | -0.16% | -0.17% | 108% | 98% | -0.59% |
| Class A2 | -0.18% | 0.00% | 0.00% | 107% | 98% | -0.11% |
| Class B | -0.23% | -0.50% | -0.50% | 108% | 94% | -0.32% |
| Class C | -10.98% | 1.47% | 1.30% | 132% | 104% | -5.84% |
| Class E | -1.24% | -1.01% | -1.08% | 114% | 101% | -1.16% |
| Overall | -2.89% | -0.01% | -0.06% | 114% | 99% | -1.70% |
| Class D | -17.27% | 2.19% | 1.92% | 130% | 105% | -11.99% |
| Class F | -6.89% | -0.49% | -0.43% | 120% | 101% | -5.68% |
|  |  |  | Random access Main10 |  |  |  |
|  |  |  | Over VTM-8.0 |  |  |  |
|  | Y | U | V | EncT | DecT | YUV |
| Class A1 | -1.10% | -0.77% | -0.94% | 106% | 99% | -0.98% |
| Class A2 | -0.19% | -0.12% | -0.16% | 103% | 99% | -0.18% |
| Class B | -0.73% | -0.84% | -0.85% | 103% | 95% | -0.76% |
| Class C | -16.64% | -3.68% | -3.92% | 124% | 101% | -12.25% |
| Class E |  |  |  |  |  |  |
| Overall | -4.94% | -1.44% | -1.55% | 109% | 98% | -3.75% |
| Class D | -25.74% | -4.31% | -3.91% | 124% | 101% | -20.05% |
| Class F | -18.04% | -4.37% | -4.28% | 107% | 99% | -9.63% |
|  |  |  | Low delay B Main10 |  |  |  |
|  |  |  | Over VTM-8.0 |  |  |  |
|  | Y | U | V | EncT | DecT | YUV |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.60% | -0.96% | -0.89% | 103% | 97% | -0.68% |
| Class C | -15.55% | -3.23% | -3.05% | 123% | 99% | -12.02% |
| Class E | -2.47% | -1.82% | -1.08% | 104% | 96% | -2.21% |
| Overall | -6.05% | -1.93% | -1.66% | 110% | 97% | -4.84% |
| Class D | -24.26% | -3.87% | -3.84% | 122% | 98% | -19.99% |
| Class F | -11.14% | -4.82% | -4.91% | 106% | 95% | -8.84% |

# Conclusion

The proposed method provides a consistent design for implicit binary split. It reportedly has BD-rate gain on all configurations when implicit BT split is extended up to the CTU level. It is asserted that this design is consistent with current implicit QT splitting design. It is proposed to adopt the proposed method to VVC.

# Reference

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-N1010 Marrakech, MA, Mar 2019.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-8.0

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 8)”, ISO/IEC JTC1/SC29/WG11 JVET-Q2001, Jan. 2020.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).