| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0281-v1 |
| --- | --- |

| Title: | AHG16: Cleanup MIP flag signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Heo Hyeongmun Jang Jangwon Choi Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | {jin78.heo, hm.jang, jangwon84.choi, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

The CABAC context model for matrix-based intra prediction (MIP) flag signaling depends on the mode information and availability of the left and above blocks. Method 1 proposes not to consider the upper block, when the current CU is at the upper boundary of each CTU. Method 2 proposes to consider only the left block. Method 3 proposes not to consider all neighboring blocks. The proposed methods remove the required memory usage for line buffer for MIP flag. Moreover, the Method 1 can also achieve a unification on limiting the use of information of the upper block at the CTU boundary in intra coding. The experimental results show that the proposed method 1 provides the coding performance changes of 0.00% and -0.01% BD-rate in AI and RA configuration, respectively. Method 2 provides the coding performance changes of 0.00% and XXX% BD-rate in AI and RA configuration, respectively. Method 3 provides the coding performance changes of 0.01% and XXX% BD-rate in AI and RA configuration, respectively.  Encoding and decoding run-times are not changed.

Proposed method

- Method 1

The changes in the VVC working draft [2] is highlight below.

- 9.3.4.2.2  Derivation process of ctxInc using left and above syntax elements

Input to this process is the luma location ( x0, y0 ) specifying the top-left luma sample of the current luma block relative to the top-left sample of the current picture, the colour component cIdx, the current coding quadtree depth cqtDepth, the dual tree channel type chType, the width and the height of the current coding block in luma samples cbWidth and cbHeight, and the variables allowSplitBtVer, allowSplitBtHor, allowSplitTtVer, allowSplitTtHor, and allowSplitQt as derived in the coding tree semantics in clause 7.4.11.4.

Output of this process is ctxInc.

The location ( xNbL, yNbL ) is set equal to ( x0 − 1, y0 ) and the derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the location ( xCurr, yCurr ) set equal to ( x0, y0 ), the neighbouring location ( xNbY, yNbY ) set equal to ( xNbL, yNbL ), checkPredModeY set equal to FALSE, and cIdx as inputs, and the output is assigned to availableL.

The location ( xNbA, yNbA ) is set equal to ( x0, y0 − 1 ) and the derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the location ( xCurr, yCurr ) set equal to ( x0, y0 ), the neighbouring location ( xNbY, yNbY ) set equal to ( xNbA, yNbA ), checkPredModeY set equal to FALSE, and cIdx as inputs, and the output is assigned to availableA.

The location ( xCtb, yCtb ) is set equal to ( x0 >> CtbLog2SizeY , y0 >> CtbLog2SizeY ), the location ( xCtbA, yCtbA ) is set equal to ( xNbA >> CtbLog2SizeY , yNbA >> CtbLog2SizeY ), and the location ( xCtbL, yCtbL ) is set equal to ( xNbL >> CtbLog2SizeY , yNbL >> CtbLog2SizeY ).

The assignment of ctxInc is specified as follows with condL and condA specified in Table 132:

- For the syntax elements alf_ctb_flag[ xCtb ][ yCtb ][ cIdx ], alf_ctb_cc_cb_idc[ xCtb ][ yCtb ], alf_ctb_cc_cr_idc[ xCtb ][ yCtb ], split_qt_flag, split_cu_flag, cu_skip_flag[ x0 ][ y0 ], pred_mode_ibc_flag[ x0 ][ y0 ], intra_mip_flag[ x0 ][ y0 ], inter_affine_flag[ x0 ][ y0 ] and merge_subblock_flag[ x0 ][ y0 ]:

ctxInc = ( condL  &&  availableL ) + ( condA  &&  availableA ) + ctxSetIdx * 3	(1561)

- For the syntax elements pred_mode_flag[ x0 ][ y0 ] and mode_constraint_flag:

ctxInc = ( condL  &&  availableL ) | | ( condA  &&  availableA )	(1562)

- For the syntax element intra_mip_flag [ x0 ][ y0 ]:

ctxInc = ( condL  &&  availableL ) + ( ( y0 − 1 is less than ( ( y0  >>  CtbLog2SizeY )  <<  CtbLog2SizeY ) ) ? 0 : ( condA  &&  availableA ) )	(1563)

- Method 2

The changes in the VVC working draft [2] is highlight below.

Table 131 – Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| intra_mip_flag[ ][ ] | (Abs( Log2(cbWidth) − Log2(cbHeight) ) > 1) ? 3 : (0,1) ( 0,1,2 (clause 9.3.4.2.2) ) [Ed. (YK): Something is wrong here.] | na | na | na | na | na |

Table 132 – Specification of ctxInc using left and above syntax elements

| Syntax element | condL | condA | ctxSetIdx |
| --- | --- | --- | --- |
| intra_mip_flag[ x0 ][ y0 ] | intra_mip_flag[ xNbL ][ yNbL ] | 0 intra_mip_flag[ xNbA ][ yNbA ] | 0 |

- Method 3

The changes in the VVC working draft [2] is highlight below.

Table 131 – Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| intra_mip_flag[ ][ ] | (Abs( Log2(cbWidth) − Log2(cbHeight) ) > 1) ? 3 : 0 ( 0,1,2 (clause 9.3.4.2.2) ) [Ed. (YK): Something is wrong here.] | na | na | na | na | na |

Table 132 – Specification of ctxInc using left and above syntax elements

| Syntax element | condL | condA | ctxSetIdx |
| --- | --- | --- | --- |
| intra_mip_flag[ x0 ][ y0 ] | 0 intra_mip_flag[ xNbL ][ yNbL ] | 0 intra_mip_flag[ xNbA ][ yNbA ] | 0 |

- 9.3.4.2.2  Derivation process of ctxInc using left and above syntax elements

Input to this process is the luma location ( x0, y0 ) specifying the top-left luma sample of the current luma block relative to the top-left sample of the current picture, the colour component cIdx, the current coding quadtree depth cqtDepth, the dual tree channel type chType, the width and the height of the current coding block in luma samples cbWidth and cbHeight, and the variables allowSplitBtVer, allowSplitBtHor, allowSplitTtVer, allowSplitTtHor, and allowSplitQt as derived in the coding tree semantics in clause 7.4.11.4.

Output of this process is ctxInc.

The location ( xNbL, yNbL ) is set equal to ( x0 − 1, y0 ) and the derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the location ( xCurr, yCurr ) set equal to ( x0, y0 ), the neighbouring location ( xNbY, yNbY ) set equal to ( xNbL, yNbL ), checkPredModeY set equal to FALSE, and cIdx as inputs, and the output is assigned to availableL.

The location ( xNbA, yNbA ) is set equal to ( x0, y0 − 1 ) and the derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the location ( xCurr, yCurr ) set equal to ( x0, y0 ), the neighbouring location ( xNbY, yNbY ) set equal to ( xNbA, yNbA ), checkPredModeY set equal to FALSE, and cIdx as inputs, and the output is assigned to availableA.

The location ( xCtb, yCtb ) is set equal to ( x0 >> CtbLog2SizeY , y0 >> CtbLog2SizeY ), the location ( xCtbA, yCtbA ) is set equal to ( xNbA >> CtbLog2SizeY , yNbA >> CtbLog2SizeY ), and the location ( xCtbL, yCtbL ) is set equal to ( xNbL >> CtbLog2SizeY , yNbL >> CtbLog2SizeY ).

The assignment of ctxInc is specified as follows with condL and condA specified in Table 132:

- For the syntax elements alf_ctb_flag[ xCtb ][ yCtb ][ cIdx ], alf_ctb_cc_cb_idc[ xCtb ][ yCtb ], alf_ctb_cc_cr_idc[ xCtb ][ yCtb ], split_qt_flag, split_cu_flag, cu_skip_flag[ x0 ][ y0 ], pred_mode_ibc_flag[ x0 ][ y0 ], intra_mip_flag[ x0 ][ y0 ], inter_affine_flag[ x0 ][ y0 ] and merge_subblock_flag[ x0 ][ y0 ]:

ctxInc = ( condL  &&  availableL ) + ( condA  &&  availableA ) + ctxSetIdx * 3	(1561)

- For the syntax elements pred_mode_flag[ x0 ][ y0 ] and mode_constraint_flag:

ctxInc = ( condL  &&  availableL ) | | ( condA  &&  availableA )	(1562)

Experimental result

The proposed method has been implemented using VTM-8.0 and under the common test conditions [1].

- Method 1

Table 1. Coding performance of the proposed method (AI)

Table 2. Coding performance of the proposed method (RA)

- Method 2

Table 3. Coding performance of the proposed method (AI)

Table 4. Coding performance of the proposed method (RA)

- Method 3

Table 5. Coding performance of the proposed method (AI)

Table 6. Coding performance of the proposed method (RA)

Conclusions

This contribution proposes the cleanup MIP flag signaling. The proposed methods provide the minor coding performance changes without increasing the complexity of encoding and decoding. It is recommended for the proposed change to be adopted as a part of VVC.

Reference

- F. Bossen, el al., “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, 14th Meeting: Geneva, CH, 19–27 March 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).