| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0292 |
| --- | --- |

| Title: | Fixes for 4-tap interpolation filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson, R. Ruoyang, Z. Zhang, J. Ström | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

It is asserted that the current 4-tap interpolation filters which are used both for intra angular prediction and chroma motion compensation have significant phase misalignments compared to the ideal phases of the interpolation filters. For ten of the interpolation filters the phase misalignments are as large as the expected phase differences between two interpolation filters of adjacent fractional positions. This contribution proposes 4-tap interpolation filters that fixes the phase misalignments of the current 4-tap interpolation filters.

The fixes were reported to be tested under VTM-8.0 CTC. The impact on coding efficiency is reported for Luma/Cb/Cr as follows:

-0.05%/-0.02%-0.06% for AI, -0.03%/-0.17%/-0.06% for RA and -0.07%/-0.30%/-0.17% for LDB.

The impact of only fixing the 4-tap interpolation filters for intra is reported to be:

-0.05%/-0.02%-0.06% for AI, -0.03%/-0.05%/-0.00% for RA and -0.05%/0.00%/-0.04% for LDB.

# Introduction

The current 4-tap interpolation filters in VVC have significant issues with phase alignment which can be seen in the below plots of phase delay and magnitude of frequency response of each interpolation filter. Only a symmetrical filter can have a phase that varies linearly with frequency, e.g., have a constant phase delay of 0.5 for all frequencies, whereas asymmetrical filters at best can have a constant phase delay response at lower frequencies. The green straight lines in Figure 1 (a) show the ideal phase delay responses, and the red curves show the phase delay responses for the current filters. Likewise, the green line in Figure 1 (b) shows the ideal magnitude responses and the red curves show the magnitude responses of the current filters. As can be seen in the figure, the phase delay responses of almost all filters deviate significantly from the ideal phase delay responses at low frequencies. It is asserted that such deviations at low frequencies are undesirable and should be fixed. It is also noticeable that the phase delay response for low frequencies are identical for interpolation filters for fractional positions 12/32, 13/32 and 14/32, which is clearly incorrect.

Figure 1: Plots of phase delay responses (a) and magnitude of frequency responses (b) of the 4-tap interpolation filters fC used in VTM-8.0.

# Proposal

The following modifications of the VVC specification is suggested, high-lighted in yellow:

Table 25 – Specification of interpolation filter coefficients fC and fG

| Fractional sample position p | fC interpolation filter coefficients |  |  |  | fG interpolation filter coefficients |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | fC[ p ][ 0 ] | fC[ p ][ 1 ] | fC[ p ][ 2 ] | fC[ p ][ 3 ] | fG[ p ][ 0 ] | fG[ p ][ 1 ] | fG[ p ][ 2 ] | fG[ p ][ 3 ] |
| 0 | 0 | 64 | 0 | 0 | 16 | 32 | 16 | 0 |
| 1 | 0 | 61 | 4 | -1 | 16 | 32 | 16 | 0 |
| 2 | -1 | 61 | 5 | -1 | 15 | 31 | 17 | 1 |
| 3 | -1 | 59 | 7 | -1 | 15 | 31 | 17 | 1 |
| 4 | −2 | 58 | 10 | −2 | 14 | 30 | 18 | 2 |
| 5 | -2 | 56 | 12 | -2 | 14 | 30 | 18 | 2 |
| 6 | -3 | 55 | 15 | -3 | 13 | 29 | 19 | 3 |
| 7 | -3 | 53 | 17 | -3 | 13 | 29 | 19 | 3 |
| 8 | -3 | 51 | 19 | -3 | 12 | 28 | 20 | 4 |
| 9 | -4 | 50 | 22 | -4 | 12 | 28 | 20 | 4 |
| 10 | -4 | 48 | 24 | -4 | 11 | 27 | 21 | 5 |
| 11 | -4 | 46 | 26 | -4 | 11 | 27 | 21 | 5 |
| 12 | -4 | 44 | 28 | -4 | 10 | 26 | 22 | 6 |
| 13 | -4 | 42 | 30 | -4 | 10 | 26 | 22 | 6 |
| 14 | -4 | 40 | 32 | -4 | 9 | 25 | 23 | 7 |
| 15 | -4 | 38 | 34 | -4 | 9 | 25 | 23 | 7 |
| 16 | -5 | 37 | 37 | -5 | 8 | 24 | 24 | 8 |
| 17 | -4 | 34 | 38 | -4 | 8 | 24 | 24 | 8 |
| 18 | -4 | 32 | 40 | -4 | 7 | 23 | 25 | 9 |
| 19 | -4 | 30 | 42 | -4 | 7 | 23 | 25 | 9 |
| 20 | -4 | 28 | 44 | -4 | 6 | 22 | 26 | 10 |
| 21 | -4 | 26 | 46 | -4 | 6 | 22 | 26 | 10 |
| 22 | -4 | 24 | 48 | -4 | 5 | 21 | 27 | 11 |
| 23 | -4 | 22 | 50 | -4 | 5 | 21 | 27 | 11 |
| 24 | -3 | 19 | 51 | -3 | 4 | 20 | 28 | 12 |
| 25 | -3 | 17 | 53 | -3 | 4 | 20 | 28 | 12 |
| 26 | -3 | 15 | 55 | -3 | 3 | 19 | 29 | 13 |
| 27 | -2 | 12 | 56 | -2 | 3 | 19 | 29 | 13 |
| 28 | −2 | 10 | 58 | −2 | 2 | 18 | 30 | 14 |
| 29 | -1 | 7 | 59 | -1 | 2 | 18 | 30 | 14 |
| 30 | -1 | 5 | 61 | -1 | 1 | 17 | 31 | 15 |
| 31 | -1 | 4 | 61 | 0 | 1 | 17 | 31 | 15 |

Table 33 – Specification of the chroma interpolation filter coefficients fC[ p ] for each 1/32 fractional sample position p

| Fractional sample position p | interpolation filter coefficients |  |  |  |
| --- | --- | --- | --- | --- |
|  | fC[ p ][ 0 ] | fC[ p ][ 1 ] | fC[ p ][ 2 ] | fC[ p ][ 3 ] |
| 1 | 0 | 61 | 4 | -1 |
| 2 | -1 | 61 | 5 | -1 |
| 3 | -1 | 59 | 7 | -1 |
| 4 | -2 | 58 | 10 | -2 |
| 5 | -2 | 56 | 12 | -2 |
| 6 | -3 | 55 | 15 | -3 |
| 7 | -3 | 53 | 17 | -3 |
| 8 | -3 | 51 | 19 | -3 |
| 9 | -4 | 50 | 22 | -4 |
| 10 | -4 | 48 | 24 | -4 |
| 11 | -4 | 46 | 26 | -4 |
| 12 | -4 | 44 | 28 | -4 |
| 13 | -4 | 42 | 30 | -4 |
| 14 | -4 | 40 | 32 | -4 |
| 15 | -4 | 38 | 34 | -4 |
| 16 | -5 | 37 | 37 | -5 |
| 17 | -4 | 34 | 38 | -4 |
| 18 | -4 | 32 | 40 | -4 |
| 19 | -4 | 30 | 42 | -4 |
| 20 | -4 | 28 | 44 | -4 |
| 21 | -4 | 26 | 46 | -4 |
| 22 | -4 | 24 | 48 | -4 |
| 23 | -4 | 22 | 50 | -4 |
| 24 | -3 | 19 | 51 | -3 |
| 25 | -3 | 17 | 53 | -3 |
| 26 | -3 | 15 | 55 | -3 |
| 27 | -2 | 12 | 56 | -2 |
| 28 | -2 | 10 | 58 | -2 |
| 29 | -1 | 7 | 59 | -1 |
| 30 | -1 | 5 | 61 | -1 |
| 31 | -1 | 4 | 61 | 0 |

Plots of phase delay and magnitude of frequency response of the proposed 4-tap interpolation filters are shown in Figure 2. Comparing to the phase delay responses in Figure 1, the suggested interpolation filters deviate substantially less from the ideal phase delay responses at low frequencies.

Figure 3: Plots of phase delay responses (a) and magnitude of frequency responses (b) of proposed 4-tap interpolation filters fC.

# Results

The objective performance for the proposal is shown below in comparison to VTM-8.0 [2] using CTC [3]. The objective results show a small benefit in coding efficiency. Encoding time is measured on simulations run on a cluster with machines with different capability and is not reliable. Decoding time is measured by running test and anchor on same machine without yuv output. Decoding time is similar to that of the anchor.

## Modification of 4-tap interpolation filters

|  | All intra Over VTM-8.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,01% | 0,02% | -0,03% | 99% | 102% |
| Class A2 | -0,03% | 0,00% | -0,03% | 98% | 101% |
| Class B | -0,04% | 0,03% | 0,03% | 99% | 100% |
| Class C | -0,03% | -0,04% | -0,03% | 94% | 101% |
| Class E | -0,14% | -0,13% | -0,30% | 90% | 99% |
| Overall | -0,05% | -0,02% | -0,06% | 96% | 101% |
| Class D | -0,05% | -0,06% | -0,04% | 95% | 99% |
| Class F | 0,00% | -0,03% | -0,13% | 96% | 101% |

|  | Random access Over VTM-8.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,01% | -0,17% | -0,09% | 100% | 100% |
| Class A2 | -0,03% | -0,04% | 0,01% | 101% | 101% |
| Class B | -0,03% | -0,24% | -0,07% | 96% | 99% |
| Class C | -0,04% | -0,17% | -0,07% | 86% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0,03% | -0,17% | -0,06% | 95% | 100% |
| Class D | -0,02% | 0,38% | 0,19% | 85% | 102% |
| Class F | 0,03% | -0,08% | -0,08% | 91% | 101% |

|  | Low-delay B Over VTM-8.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0,00% | 0,02% | -0,19% | 102% | 98% |
| Class C | -0,12% | -0,67% | -0,23% | 96% | 101% |
| Class E | -0,12% | -0,36% | -0,05% | 97% | 100% |
| Overall | -0,07% | -0,30% | -0,17% | 99% | 100% |
| Class D | 0,00% | -0,41% | -0,91% | 88% | 99% |
| Class F | 0,08% | 0,70% | 0,04% | 96% | 101% |

## Only modification of the 4-tap interpolation filters for intra

|  | All intra Over VTM-8.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,01% | 0,02% | -0,03% | 99% | 102% |
| Class A2 | -0,03% | 0,00% | -0,03% | 98% | 101% |
| Class B | -0,04% | 0,03% | 0,03% | 99% | 100% |
| Class C | -0,03% | -0,04% | -0,03% | 94% | 101% |
| Class E | -0,14% | -0,13% | -0,30% | 90% | 99% |
| Overall | -0,05% | -0,02% | -0,06% | 96% | 101% |
| Class D | -0,05% | -0,06% | -0,04% | 95% | 99% |
| Class F | 0,00% | -0,03% | -0,13% | 96% | 101% |

|  | Random access Over VTM-8.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,00% | -0,04% | -0,06% | 96% | 98% |
| Class A2 | -0,02% | -0,12% | 0,17% | 97% | 100% |
| Class B | -0,04% | 0,04% | 0,03% | 90% | 101% |
| Class C | -0,05% | -0,07% | -0,14% | 83% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0,03% | -0,04% | 0,00% | 91% | 100% |
| Class D | 0,02% | 0,23% | 0,02% | 83% | 99% |
| Class F | 0,02% | -0,08% | -0,05% | 85% | 102% |

|  | Low-delay B Over VTM-8.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,01% | 0,20% | 0,10% | 101% | 99% |
| Class C | -0,07% | -0,18% | -0,15% | 95% | 101% |
| Class E | -0,09% | -0,08% | -0,12% | 93% | 96% |
| Overall | -0,05% | 0,00% | -0,04% | 97% | 99% |
| Class D | 0,01% | -0,62% | 0,23% | 84% | 100% |
| Class F | 0,00% | 0,30% | 0,42% | 91% | 103% |

# Conclusions

# It has been shown that the phase delay responses of the current 4-tap interpolation filters have significant deviations from the ideal phase delay responses and that some of the filters also have the same phase delay at low frequency which is clearly incorrect.

# We propose to adopt the fixes of filter coefficients for the 4-tap interpolation filters used for intra angular prediction and chroma motion compensation to VVC specification and software such that VVC has 4-tap interpolation filters with proper interpolation properties.

# References

- B. Bross, J. Chen, and S. Liu, "Versatile Video Coding (Draft 8)", Document of Joint Video Experts Team, JVET-Q2001.

- VTM-8.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-8.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations”, JVET-N1010

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).