| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0315 |
| --- | --- |

| Title: | AHG9: On mixed nal unit type signalling and PPS cleanup |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammed Coban Vadim Seregin Yong He Yao-Jen Chang Marta Karczewicz | Tel: Email: | +1 858 658 3973 mcoban@qti.qualcomm.com vseregin@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In this contribution the following are proposed:

- Move signalling of mixed nal unit types in a picture to PH.

- Condition the presence of mixed nal unit type in PH at SPS level for multiple subpicture case.

# Proposal

In current VVC draft [1], presence of a mixed nal unit types in a picture are signalled in picture parameter set. In cases where mixed nal unit types are used in a bitstream, pictures with and without mixed nal units would exist, requiring generation of another set of picture parameter sets. In order to avoid having to carry, potentially, double the number of PPS of the non mixed nal unit case bitstream, the existence of mixed nal units are signalled in the picture header. The presence of mixed_nalu_unit_types_in_pic_flag in PH can be indicated in SPS for multiple subpicture use case.

## Proposed text changes

The changed texts are based on the latest VVC text in JVET-Q2001-vE/v15. Parts that have been added or modified are highlighted in turquoise, and deleted parts are highlighted in yellow strikethrough in red fonts.

=========================VVC draft 8 changes BEGIN==============================

#### 7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| …. |  |
| sps_log2_ctu_size_minus5 | u(2) |
| subpic_info_present_flag | u(1) |
| if( subpic_info_present_flag ) { |  |
| sps_num_subpics_minus1 | ue(v) |
| if ( sps_num_subpics_minus1 > 0 ) |  |
| sps_mixed_nalu_types_in_pic_present_flag | u(1) |
| sps_independent_subpics_flag | u(1) |
| for( i = 0; sps_num_subpics_minus1 > 0 && i <= sps_num_subpics_minus1; i++ ) { |  |
| if( i > 0 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| if( i > 0 && pic_height_max_in_luma_samples > CtbSizeY ) { |  |
| subpic_ctu_top_left_y[ i ] | u(v) |
| …. |  |

#### 7.3.2.4 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | u(4) |
| mixed_nalu_types_in_pic_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| ….. |  |
| no_pic_partition_flag | u(1) |
| if( !no_pic_partition_flag ) { |  |
| pps_log2_ctu_size_minus5 | u(2) |
| num_exp_tile_columns_minus1 | ue(v) |
| num_exp_tile_rows_minus1 | ue(v) |
| …. |  |
| } |  |
| … |  |

#### 7.3.2.7 Picture header structure syntax

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| gdr_or_irap_pic_flag | u(1) |
| if( gdr_or_irap_pic_flag ) |  |
| gdr_pic_flag | u(1) |
| if ( sps_mixed_nalu_types_in_pic_present_flag && !gdr_or_irap_pic_flag ) |  |
| mixed_nalu_types_in_pic_flag | u(1) |
| ph_inter_slice_allowed_flag | u(1) |
| if( ph_inter_slice_allowed_flag ) |  |
| ph_intra_slice_allowed_flag | u(1) |
| non_reference_picture_flag | u(1) |
| ph_pic_parameter_set_id | ue(v) |
| …. |  |

#### 7.4.3.3 Sequence parameter set RBSP semantics

…….

sps_num_subpics_minus1 plus 1 specifies the number of subpictures in each picture in the CLVS. The value of sps_num_subpics_minus1 shall be in the range of 0 to Ceil( pic_width_max_in_luma_samples ÷ CtbSizeY ) * Ceil( pic_height_max_in_luma_samples ÷ CtbSizeY ) − 1, inclusive. When not present, the value of sps_num_subpics_minus1 is inferred to be equal to 0.

sps_mixed_nalu_types_in_pic_present_flag equal to 1 specifies that mixed_nau_types_in_pic_flag information is present in picture header for the CLVS. sps_mixed_nalu_types_in_pic_present_flag equal to 0 specifies that mixed_nau_types_in_pic_flag information is not present for the CLVS in picture header.

…

#### 7.4.3.4 Picture parameter set RBSP semantics

……

pps_seq_parameter_set_id specifies the value of sps_seq_parameter_set_id for the SPS. The value of pps_seq_parameter_set_id shall be in the range of 0 to 15, inclusive. The value of pps_seq_parameter_set_id shall be the same in all PPSs that are referred to by coded pictures in a CLVS.

mixed_nalu_types_in_pic_flag equal to 1 specifies that each picture referring to the PPS has more than one VCL NAL unit and the VCL NAL units do not have the same value of nal_unit_type. mixed_nalu_types_in_pic_flag equal to 0 specifies that each picture referring to the PPS has one or more VCL NAL units and the VCL NAL units of each picture refering to the PPS have the same value of nal_unit_type.

When no_mixed_nalu_types_in_pic_constraint_flag is equal to 1, the value of mixed_nalu_types_in_pic_flag shall be equal to 0.

For each slice with a nal_unit_type value nalUnitTypeA in the range of IDR_W_RADL to CRA_NUT, inclusive, in a picture picA that also contains one or more slices with another value of nal_unit_type (i.e., the value of mixed_nalu_types_in_pic_flag for the picture picA is equal to 1), the following applies:

–	The slice shall belong to a subpicture subpicA for which the value of the corresponding subpic_treated_as_pic_flag[ i ] is equal to 1.

–	The slice shall not belong to a subpicture of picA containing VCL NAL units with nal_unit_type not equal to nalUnitTypeA.

–	If nalUnitTypeA is equal to CRA, for all the following PUs following the current picture in the CLVS in decoding order and in output order, neither RefPicList[ 0 ] nor RefPicList[ 1 ] of a slice in subpicA in those PUs shall include any picture preceding picA in decoding order in an active entry.

–	Otherwise (i.e., nalUnitTypeA is equal to IDR_W_RADL or IDR_N_LP), for all the PUs in the CLVS following the current picture in decoding order, neither RefPicList[ 0 ] nor RefPicList[ 1 ] of a slice in subpicA in those PUs shall include any picture preceding picA in decoding order in an active entry.

NOTE 1– mixed_nalu_types_in_pic_flag equal to 1 indicates that pictures referring to the PPS contain slices with different NAL unit types, e.g., coded pictures originating from a subpicture bitstream merging operation for which encoders have to ensure matching bitstream structure and further alignment of parameters of the original bitstreams. One example of such alignments is as follows: When the value of sps_idr_rpl_present_flag is equal to 0 and mixed_nalu_types_in_pic_flag is equal to 1, a picture referring to the PPS cannot have slices with nal_unit_type equal to IDR_W_RADL or IDR_N_LP.

……

no_pic_partition_flag equal to 1 specifies that no picture partitioning is applied to each picture referring to the PPS. no_pic_partition_flag equal to 0 specifies each picture referring to the PPS may be partitioned into more than one tile or slice.

It is a requirement of bitstream conformance that the value of no_pic_partition_flag shall be the same for all PPSs that are referred to by coded pictures within a CLVS.

It is a requirement of bitstream conformance that the value of no_pic_partition_flag shall not be equal to 1 when the value of sps_num_subpics_minus1 + 1 is greater than 1.

#### 7.4.3.7 Picture header structure semantics

…

gdr_pic_flag equal to 1 specifies the picture associated with the PH is a GDR picture. gdr_pic_flag equal to 0 specifies that the picture associated with the PH is not a GDR picture. When not present, the value of gdr_pic_flag is inferred to be equal to 0. When gdr_enabled_flag is equal to 0, the value of gdr_pic_flag shall be equal to 0. [Ed. (YK): Should the semantics of gdr_pic_flag also be specified "one way"? That'd be consistent with the other flags herein, but then there is some issue for using it to condition the presence of recovery_poc_cnt.]

NOTE 1 – When gdr_or_irap_pic_flag is equal to 1 and gdr_pic_flag is equal to 0, the picture associated with the PH is an IRAP picture.

mixed_nalu_types_in_pic_flag equal to 1 specifies that picture associated with the PH has more than one VCL NAL unit and the VCL NAL units do not have the same value of nal_unit_type. mixed_nalu_types_in_pic_flag equal to 0 specifies that picture associated with the PH has one or more VCL NAL units and the VCL NAL units of each picture refering to the PPS have the same value of nal_unit_type.

When no_mixed_nalu_types_in_pic_constraint_flag is equal to 1, the value of mixed_nalu_types_in_pic_flag shall be equal to 0.

For each slice with a nal_unit_type value nalUnitTypeA in the range of IDR_W_RADL to CRA_NUT, inclusive, in a picture picA that also contains one or more slices with another value of nal_unit_type (i.e., the value of mixed_nalu_types_in_pic_flag for the picture picA is equal to 1), the following applies:

–	The slice shall belong to a subpicture subpicA for which the value of the corresponding subpic_treated_as_pic_flag[ i ] is equal to 1.

–	The slice shall not belong to a subpicture of picA containing VCL NAL units with nal_unit_type not equal to nalUnitTypeA.

–	If nalUnitTypeA is equal to CRA, for all the following PUs following the current picture in the CLVS in decoding order and in output order, neither RefPicList[ 0 ] nor RefPicList[ 1 ] of a slice in subpicA in those PUs shall include any picture preceding picA in decoding order in an active entry.

–	Otherwise (i.e., nalUnitTypeA is equal to IDR_W_RADL or IDR_N_LP), for all the PUs in the CLVS following the current picture in decoding order, neither RefPicList[ 0 ] nor RefPicList[ 1 ] of a slice in subpicA in those PUs shall include any picture preceding picA in decoding order in an active entry.

NOTE 1– mixed_nalu_types_in_pic_flag equal to 1 indicates that the picture associated with the PH contain slices with different NAL unit types, e.g., coded pictures originating from a subpicture bitstream merging operation for which encoders have to ensure matching bitstream structure and further alignment of parameters of the original bitstreams. One example of such alignments is as follows: When the value of sps_idr_rpl_present_flag is equal to 0 and mixed_nalu_types_in_pic_flag is equal to 1, a picture referring to the PPS cannot have slices with nal_unit_type equal to IDR_W_RADL or IDR_N_LP.

….

=========================VVC draft 8 changes END==============================

# References

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 7)”, JVET-Q2001, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 17th Meeting: Brussels, BE, 1–11 Jan. 2020.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).