| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0317 |
| --- | --- |

| Title: | AHG9: On slice transform skip residual coding method signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammed Coban Vadim Seregin Alican Nalci Marta Karczewicz | Tel: Email: | +1 858 658 3973 mcoban@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In this contribution a PPS flag indicating presence of slice_ts_residual_coding_disabled_flag in slice header is proposed.

# Proposal

In current VVC draft [1], slice_ts_residual_coding_disabled_flag is always signalled in slice header. It is proposed to indicate the presence of this flag in slice header in the PPS similar to cabac_init_presence_flag in PPS to indicate the presence of slice level cabac_init_flag. Additionally since slice_ts_residual_coding_disabled_flag is TS specific, if TS is disabled at SPS level, it should not be signalled.

## Proposed text changes

The changed texts are based on the latest VVC text in JVET-Q2001-vE/v15. Parts that have been added or modified are highlighted in turquoise, and deleted parts are highlighted in yellow strikethrough in red fonts.

=========================VVC draft 8 changes BEGIN==============================

#### 7.3.2.4 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| …. |  |
| cabac_init_present_flag | u(1) |
| ts_residual_coding_override_present_flag | u(1) |
| … |  |

#### 7.3.7.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| picture_header_in_slice_header_flag | u(1) |
| …. |  |
| if( slice_deblocking_filter_override_flag ) { |  |
| slice_deblocking_filter_disabled_flag | u(1) |
| …. |  |
| } |  |
| if( sps_transform_skip_enabled_flag && ts_residual_coding_override_present_flag ) |  |
| slice_ts_residual_coding_disabled_flag | u(1) |
| if( ph_lmcs_enabled_flag ) |  |
| slice_lmcs_enabled_flag | u(1) |

#### 7.4.3.4 Picture parameter set RBSP semantics

……

cabac_init_present_flag equal to 1 specifies that cabac_init_flag is present in slice headers referring to the PPS. cabac_init_present_flag equal to 0 specifies that cabac_init_flag is not present in slice headers referring to the PPS.

ts_residual_coding_override_present_flag equal to 1 specifies that ts_residual_coding_disabled_flag is present in slice headers referring to the PPS. ts_residual_coding_override_present_flag equal to 0 specifies that ts_residual_coding_disabled_flag is not present in slice headers referring to the PPS.

…

=========================VVC draft 8 changes END==============================

# References

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 7)”, JVET-Q2001, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 17th Meeting: Brussels, BE, 1–11 Jan. 2020.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).