| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0334-v2 |
| --- | --- |

| Title: | AHG11: Disabling chroma CU palette mode under local dual tree |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hong-Jheng Jhu, Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Xianglin Wang | Email: | jhuhong-jheng@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In VTM8.0 [1], under local dual tree case, the palette prediction update process is performed after each palette mode coded CU. In other words, a palette mode chroma CU cannot be decoded until all palette mode luam CUs under the same local-dual tree are decoded. This may become be a complexity bottleneck and cause latency in the pipeline.

In this proposal, it is proposed to disable palette mode for chroma CU under local dual tree cases. The results on YUV4:2:0 sequences compared to VTM8.0 are:

Dual tree ON:

Class F: 0.00% AI, 0.00% RA, -0.01% LB

Class SCC: 0.00% AI, 0.03% RA, -0.02% LB

Dual tree OFF:

Class F: 0.00% AI, 0.03% RA, 0.02% LB

Class SCC: -0.06% AI, -0.04% RA, -0.09% LB

# Introduction

In VTM8.0 [1], under local dual tree case, the palette prediction update process is performed after each palette mode coded CU. More specifically, the palette prediction is updated first while coding each luma CU under the local-dual tree, followed by each chroma CU under the same local-dual tree. In other words, a palette mode chroma CU cannot be decoded until all palette mode luam CUs under the same local-dual tree are decoded. Given that the palette prediction update process is non-trivial, performing such a process sequentially on multiple small size CUs under local dual tree case can be a complexity bottleneck and cause latency in the pipeline.

# Proposal

It is proposed to disable palette mode for Chroma CU under local dual tree cases. As a result, under local-dual tree case the palette prediction update process is only performed for luma CUs.

# Simulation Results

The proposed method was implemented based on the VTM-8.0 reference software [1]. Following table shows the simulation results of the palette mode for 4:2:0 color format

Table 1. Simulation results of the palette mode

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class SCC | 0.00% | 0.00% | 0.00% | 100% | 101% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.00% | -0.01% | 0.12% | 100% | 100% |
| Class SCC | 0.03% | 0.04% | 0.02% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.01% | -0.13% | 0.33% | 99% | 100% |
| Class SCC | -0.02% | -0.02% | -0.05% | 99% | 100% |

Table 2. Simulation results of the palette mode

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.00% | 0.00% | -0.06% | 100% | 100% |
| Class SCC | -0.06% | 0.46% | 0.47% | 100% | 101% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.03% | 0.13% | 0.03% | 100% | 100% |
| Class SCC | -0.04% | 0.19% | 0.15% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.02% | 0.00% | 0.40% | 100% | 100% |
| Class SCC | -0.09% | -0.03% | 0.01% | 100% | 100% |

# Working Draft Text

The changes in the VVC working draft [2] are highlighted below.

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_palette_enabled_flag && cbWidth <= 64 && cbHeight <= 64 && cu_skip_flag[ x0 ][ y0 ] = = 0 && modeType != MODE_TYPE_INTER && (cbWidth* cbHeight > (treeType != DUAL_TREE_CHROMA? 16:16* SubWidthC* SubHeightC)) && ( (modeType ! = MODE_TYPE_INTRA) ¦ ¦ (treeType ! = DUAL_TREE_CHROMA) ) ) |  |
| pred_mode_plt_flag | ae(v) |
| } |  |
| … |  |
| } |  |

8.4.5.3 Decoding process for palette mode

……

When localDualTree is equal to 1, the following applies:

- If treeType is equal to DUAL_TREE_LUMA, the following applies for i = 0..num_signalled_palette_entries[ startComp ] − 1:

CurrentPaletteEntries[ 1 ][ NumPredictedPaletteEntries + i ] = 1 << ( BitDepth − 1 )	(450)

CurrentPaletteEntries[ 2 ][ NumPredictedPaletteEntries + i ] = 1 << ( BitDepth − 1 )	(451)

- Otherwise (if treeType is equal to DUAL_TREE_CHROMA), the following applies for i = 0..num_signalled_palette_entries[ startComp ] − 1:

CurrentPaletteEntries[ 0 ][ NumPredictedPaletteEntries + i ] = 1 << ( BitDepth − 1 )	(452)

- The variables CurrentPaletteSize[ 0 ], startComp, numComps and maxNumPalettePredictorSize are derived as follows:

CurrentPaletteSize[ 0 ] = CurrentPaletteSize[ startComp ]	(453)

startComp = 0		(454)

numComps = 3		(455)

maxNumPalettePredictorSize = 63		(456)

When one of the following conditions is true:

- cIdx is equal to 0 and numComps is equal to 1;

- cIdx is equal to 0 and localDualTree is equal to 1;

- cIdx is equal to 2 and localDualTree is not equal to 1;

the value PredictorPaletteSize[ startComp ] and the array PredictorPaletteEntries are derived or modified as follows:

…

# Conclusions

In this proposal, a method is proposed to disable palette mode for a chroma CU under local dual tree case to mitigate the latency due to the sequential palette prediciton update operations. The results show that the proposed changes result in negligible BD-rate changes compared to VTM8. It is recommended to adopt the proposed changes into VVC.

# Reference

VTM-8.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-8.0.

B. Bross, J. Chen, S. Liu and Y.-K. Wang, “Versatile Video Coding (Draft 8)”, JVET-Q2001-vE, the 17th JVET meeting, Brussels, BE, 7–17 January 2020.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).