| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0336-v1 |
| --- | --- |

| Title: | Adaptive colour transform cleanups |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jizheng Xu, Li Zhang, Weijia Zhu, Kai Zhang 8910 University Center Ln, San Diego, CA 92122, USA | Tel: Email: | +1-619-436-7216 xujizheng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following cleanups for adaptive colour transform.

- Enlarge the clipping range for the Co and Cg components by 1 more bit plane.

- Remove the clipping operation for the Y component.

# Introduction

A clipping operation is applied to both Co and Cg components before they are converted back to RGB domain. In the current text, the clipping range is [ −1<<BitDepth, (1<<BitDepth) − 1]. However, because YCoCg-R transform is used, the Co and Cg components will need 1 more bit. Besides, the JCCR mode will need another 1 more bit. Thus, the current clipping does not cover the whole possible range.

# Proposed methods

It is proposed to increase the clipping range by 1 more bit to handle the situation caused by the JCCR mode and YCoCg-R. It is asserted that the proposed text matches the current reference software implementation. The other aspect is that for luma component, there is no need to perform the clipping since there is no dynamic range change. The proposed method does not influence the CTC performance.

# Text changes

In the following, based on JVET-Q2001-vE document, we marked the newly added text in turquoise and the deleted text in yellow strikethrough in red fonts.

#### 8.7.4.6 Residual modification process for blocks using colour space conversion

Inputs to this process are:

–	a variable nTbW specifying the block width,

–	a variable nTbH specifying the block height,

–	an (nTbW)x(nTbH) array of luma residual samples rY with elements rY[ x ][ y ],

–	an (nTbW)x(nTbH) array of chroma residual samples rCb with elements rCb[ x ][ y ],

–	an (nTbW)x(nTbH) array of chroma residual samples rCr with elements rCr[ x ][ y ].

Outputs of this process are:

–	a modified (nTbW)x(nTbH) array rY of luma residual samples,

–	a modified (nTbW)x(nTbH) array rCb of chroma residual samples,

–	a modified (nTbW)x(nTbH) array rCr of chroma residual samples.

The (nTbW)x(nTbH) arrays of residual samples rY, rCb and rCr are modified as follows:

rY[ x ][ y ] = Clip3( −( 1 << BitDepth ), ( 1 << BitDepth ) − 1, rY[ x ][ y ] )	(1220)

rCb[ x ][ y ] = Clip3( −( 1 << ( BitDepth + 1 )), ( 1 << ( BitDepth + 1 )) − 1, rCb[ x ][ y ] )	(1221)

rCr[ x ][ y ] = Clip3( −( 1 << ( BitDepth + 1 )), ( 1 << ( BitDepth + 1 )) − 1, rCr[ x ][ y ] )	(1222)

tmp = rY[ x ][ y ] − ( rCb[ x ][ y ] >> 1 )	(1223)

rY[ x ][ y ] = tmp + rCb[ x ][ y ]	(1224)

rCb[ x ][ y ] = tmp − ( rCr[ x ][ y ] >> 1 )	(1225)

rCr[ x ][ y ] += rCb[ x ][ y ]	(1226)

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).