| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0340-v2 |
| --- | --- |

| Title: | Agenda and report of the category 2 AHG pre-meeting of the 18th JVET meeting |  |  |
| --- | --- | --- | --- |
| Status: | Report document from the coordinators of the category 2 AHG pre-meeting |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jens-Rainer Ohm Institute of Communication Engineering RWTH Aachen Melatener Straße 23 D-52074 Aachen Benjamin Bross Fraunhofer HHI Andrew Segall Sharp Labs of America, Inc. Yan Ye Alibaba Group | Tel: Email: Email: Email: Email: | +49 241 80 27671 ohm@ient.rwth-aachen.de benjamin.bross@hhi.fraunhofer.de asegall@sharplabs.com yan.ye@alibaba-inc.com |
| Source: | Coordinators of the category 2 AHG pre-meeting |  |  |

_____________________________

# Summary

The category 2 AHG pre-meeting of the 18th JVET meeting, for addressing coding tool topics, mostly in the scopes of AHGs 2, 3, 6, 11, 14, 15 and 16, held the following teleconference meeting sessions on April 9 (dates and time slots are all in UTC time):

- 1300-1500 1st "afternoon" session [break after 2 hours]

- 1515-1715 2nd "afternoon" session

- ["dinner" break – nearly 4 hours – enough for some side work or a nap]

- 2100-2300 1st "night" session [break after 2 hours]

- 2315-0115 2nd "night" session

- [end of day – nearly 12 hours to the next session]

and the following teleconference meeting sessions on each day of April 13 and 14 (dates and time slots are all in UTC time):

- 0500-0700 1st "morning" session [break after 2 hours]

- 0715-0915 2nd "morning" session

- ["lunch" break – nearly 4 hours – enough for some side work or a nap]

- 1300-1500 1st "afternoon" session [break after 2 hours]

- 1515-1715 2nd "afternoon" session

- [end of day – nearly 12 hours to the next session]

Approximately XXX people attended the AHG pre-meeting, and approximately XXX input documents, including X AHG reports were discussed.

Remarks from opening session 2.1 on Thu 9 April 2020, 1300 UTC (chaired by JRO)

- AHG reports will contain recommendations developed during AHG pre-meetings, decisions will only be made during the JVET meeting

- Goal is to cluster documents with related topics, and identify need for action

- At this point of standard development, stability of the design should have highest priority

- Late documents are all those registered more than 12 hours after the announced “AHG deadline” of April 3 – those could only be discussed if time allows

The following recommendations were made to the JVET:

- JVET-Rxxxx:

- ...

# Attendance

The list of participants in the AHG pre-meeting meeting can be found in Annex A of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/‌SC 29/‌WG 11 (including experts who had been personally invited as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the responsible coordinators.

# AHG reports (7)

These reports were discussed Tuesday 7 January 2020 during 1010-1215 (chaired by GJS) and Wednesday 8 January during 0940–1300 and 1430–XX (chaired by GJS and JRO), except as otherwise noted.

JVET-Q0002 JVET AHG report: Draft text and test model algorithm description editing (AHG2) [B. Bross, J. Chen, J. Boyce, S. Kim, S. Liu, Y.-K. Wang, Y. Ye]

This AHG report was discussed Wednesday 8 January 2020 at 1000 (chaired by GJS & JRO).

JVET-Q0003 JVET AHG report: Test model software development (AHG3) [F. Bossen, X. Li, K. Sühring]

This AHG report was discussed Wednesday 8 January 2020 at 1020 (chaired by GJS & JRO).

JVET-Q0006 JVET AHG report: 360° video coding tools, software and test conditions (AHG6) [J. Boyce, Y. He, K. Choi, J.-L. Lin, Y. Ye]

This AHG report was discussed Tuesday 7 January 2020 at 1010 (chaired by GJS).

JVET-Q0007 JVET AHG report: Coding of HDR/WCG material (AHG7) [A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy]

This AHG report was discussed Tuesday 7 January 2020 at 1020 (chaired by GJS).

JVET-Q0011 JVET AHG report: Screen content coding (AHG11) [S. Liu, J. Boyce, A. Filippov, Y.-C. Sun, J. Xu, H. Yang]

This AHG report was discussed Tuesday 7 January 2020 at 1110 (chaired by GJS).

JVET-Q0014 JVET AHG report: Lossless and near-lossless coding (AHG14) [T. Nguyen, T.-C. Ma, M. Ikeda, H. Jang, X. Zhao]

This AHG report was discussed Wednesday 8 January 2020 at 1445 (chaired by GJS & JRO).

JVET-Q0016 JVET AHG report: Implementation studies (AHG16) [M. Zhou, J. An, E. Chai, K. Choi, S. Sethuraman, T. Hsieh, X. Xiu]

This AHG report was discussed Wednesday 8 January 2020 at 1600 (chaired by GJS & JRO).

# Coding tool proposals (109)

## AHG2/AHG3/AHG16: General coding tools (68)

### Inter prediction and MV coding (9)

JVET-R0137 On mvd_l1_zero_flag and NoBackwadPredFlag [T. Chujoh, E. Sasaki, T. Ikai (Sharp)]

Only first aspect (problem 1) on NoBackwardPredFlag and ColPic

JVET-R0212 On modes in geometric partitioning [C. Hollmann, D. Liu, R. Yu, J. Ström (Ericsson)]

JVET-R0213 Modifications of motion storage in geometric partition mode [R. Yu, D. Liu, C. Hollmann, J. Ström (Ericsson)]

JVET-R0223 AHG16: On DMVR and wraparound motion compensation [J. Luo, J. Chen, Y. Ye (Alibaba)]

JVET-R0282 GEO with MMVD [K. Panusopone, S. Hong, L. Wang (Nokia)]

JVET-R0292 Fixes for 4-tap interpolation filtering [K. Andersson, R. Yu, Z. Zhang, J. Ström (Ericsson)]

Only one aspect related to inter:

- 4-tap interpolation filter for chroma motion compensation

- 4-tap interpolation filter for intra angular prediction

JVET-R0293 Fixes for 6-tap interpolation filtering for affine motion compensation [K. Andersson, R. Yu, Z. Zhang, J. Ström (Ericsson)]

JVET-R0311 [AHG2] Fix cu_skip_flag signaling for IBC [H. Jang, J. Nam, N. Park, S. Kim, J. Lim (LGE)]

JVET-R0357 Geometric prediction mode with motion vector difference [K. Zhang, L. Zhang, Z. Deng, H. Liu, Y. Wang (Bytedance)] [late]

### Intra prediction and mode coding (6)

JVET-R0280 AHG16: Cleanup of intra reference sample filter selection [J. Heo, H. Jang, J. Choi, J. Nam, M. Koo, J. Lim, S. Kim (LGE)]

JVET-R0281 AHG16: Cleanup MIP flag signaling [J. Heo, H. Jang, J. Choi, J. Lim, S. Kim (LGE)]

JVET-R0288 AHG16: Reference samples for ISP [F. Bossen (Sharp)]

JVET-R0314 AHG2/AHG16: Fixes on CCLM [Y. Wang, K. Zhang, L. Zhang, H. Liu (Bytedance)]

JVET-R0350 MIP for all channels in the case of 4:4:4 and single tree [J. Pfaff, B. Stallenberger, P. Merkle, M. Schäfer, P. Helle, T. Hinz, H. Schwarz, D. Marpe, T. Wiegand (HHI)] [late]

JVET-R0356 CCLM-related bugfixes for the VVC specification draft [A. Filippov, V. Rufitskiy, E. Alshina (Huawei)] [late]

### Loop filtering (21)

#### Deblocking filter (7)

#### Reviewed in session 2.2 Thu 9 April 1520-1705 UTC (chaired by JRO)

JVET-R0130 Cleanup of tC value derivation process for deblocking filter [S. Iwamura, S. Nemoto, A. Ichigaya (NHK)] [late]

TBP

JVET-R0134 AHG2: Mismatch related to deblocking of subblock motion edges [B. Heng, M. Zhou, W. Wan (Broadcom)]

This contribution asserts that there is a mismatch between the VVC draft text and VTM sotware related to deblocking of coding subblock boundaries. Within a subblock motion CU, the length of the deblocking filter used depends on the distance the nearest transform edge. However, when this neighboring transform edge aligns with a virtual boundary, the behavior of the text and software differ.

Specifically, the VTM software treats the neighboring transform edge as a transform edge, regardless of whether it aligns with a virtual boundary or not. While the VVC draft text ignores the neighboring transform edge altogether if it aligns with a virtual boundary. This difference will cause the text and software to use different filter lengths for subblock motion edges, and therefore they will produce different results.

This contribution proposes to modify the text to match the software behavior to resolve this mismatch. Proposed text changes are provided.

Presented Thu 9 April 1733 (chaired by JRO).

There is a ticket #857 which also identifies this issue (as well as other issues). This was partially resolved by submitting a software patch. The new contribution points out that after that we have still a mismatch between text and software. Text appears to be appropriate.

Recommendation (BF text): The proposed text changes should be adopted.

This also would resolve ticket #857 as far as the text is concerned.

JVET-R0168 Issue on bS derivation of deblocking filter [K. Abe, T. Toma (Panasonic)]

This contribution points out the mismatch between VVC text and VTM on bS derivation process of deblocking filter. In VVC text, bS is set equal to 1 for the boundary between IBC and inter block on both luma edge and chroma edge. On the other hand, in VTM, bS is set equal to 1 for the boundary between IBC and inter block only on luma edge. The proponent of this contribution thinks there are two solutions, solution1: fix VTM to align to VVC text, solution2: fix VVC text to align to VTM. This contribution shows the difference of coding performance and text changes for both solutions.

No ticket yet.

It is mentioned by the proponents that solution 1 would have small impact on coding results, therefore they would better suggest aligning the text with software.

Several experts expressed support for solution 2, as also in the past it had been agreed that MV differences should not be checked for chroma deblocking.

Recommendation (BF text): The proposed text changes should be adopted. There may however be some interaction with a related issue in R0228, where palette mode is also considered.

JVET-R0228 AHG11: Bugfix to deblocking filter boundary strength setting for palette [R.-L. Liao, Y. Ye, M. G. Sarwer (Alibaba)]

In VVC draft 8, the deblocking filter boundary strength is set according to the coding modes of two neighboring blocks along a deblocking edge. However, it is reported that, when one of neighboring blocks is coded in palette and the other is coded in IBC or inter mode, the boundary filtering strength is not clearly defined in the spec. It is also reported that in VTM-8.0, this ambiguity partially exists as well, and can cause the software mismatch in encoder and decoder in some cases. In this contribution, it is proposed to clearly define the boundary filtering strength for the aforementioned case by setting it to one of 1 or 2. It is reported that, as compared to VTM-8.0 with palette off, the overall coding performance impact for {Y, U, V} in 4:4:4 color format is:

- VTM8:{-6.25%,-6.42%,-6.26%}for AI,{-5.04%,-6.81%,-6.86%}for RA,{-3.18%,-4.39%,-4.60%}for LB

- bS = 1:{-6.25%,-6.42%,-6.26%}for AI,{-5.03%,-6.82%,-6.87%}for RA,{-3.20%,-4.34%,-4.16%}for LB

- bS = 2:{-6.25%,-6.42%,-6.26%}for AI,{-5.02%,-6.76%,-6.81%}for RA,{-3.14%,-4.26%,-4.41%}for LB

For class F in 4:2:0 color format, the overall coding performance impact for {Y, U, V} is:

- VTM8:{-1.24%,-0.43%,-0.47%}for AI,{-1.29%,-0.47%,-0.56%}for RA,{-0.56%,-0.24%,-1.24%}for LB

- bS = 1:{-1.24%,-0.43%,-0.47%}for AI,{-1.29%,-0.48%,-0.51%}for RA,{-0.61%,-0.14%,-0.76%}for LB

- bS = 2:{-1.24%,-0.43%,-0.47%}for AI,{-1.32%,-0.53%,-0.52%}for RA,{-0.60%,-0.38%,-0.98%}for LB

For class SCC in 4:2:0 color format, the overall coding performance impact for {Y, U, V} is:

- VTM8:{-6.50%,-5.18%,-4.94%}for AI,{-3.78%,-2.45%,-2.42%}for RA,{-1.22%,-0.59%,-0.66%}for LB

- bS = 1:{-6.50%,-5.18%,-4.94%}for AI,{-3.76%,-2.42%,-2.43%}for RA,{-1.23%,-0.57%,-0.60%}for LB

- bS = 2:{-6.50%,-5.18%,-4.94%}for AI,{-3.76%,-2.36%,-2.39%}for RA,{-1.28%,-0.68%,-0.69%}for LB

In terms of subjective quality, no significant visual difference was observed in all three cases based on the informal subjective viewing conducted.

It I agreed during the discussion that the spec text does not clearly define the bS in case where palette is used at the other side of the block. For the case of not using local dual tree, the VTM mode of operation is clearly defined. For this case, it is agreed that the text should be aligned with the software (setting Bs=1 when the other side is a palette block, and operated in single tree mode). For local dual tree, more investigation is necessary to understand if the SW is covering all possible cases (e.g. when luma is palette and chroma in IBC, etc.)

Recommendation (BF text): The proposed text change on more clearly specifying bS in case of palette should be adopted. There may however be some interaction with a related issue in R0168

Revisit: Further offline study needed for the case of local dual tree.

JVET-R0289 [AHG16] On deblocking filter process [N. Park, J. Nam, H. Jang, J. Lim, S. Kim (LGE)]

In Ticket #899, it was claimed that there is a mismatch between VVC draft 8 and VTM8.0 on deblocking filter process when CU size is greater than maximum transform block size and cu_coded_flag is equal to 0.

In VVC draft 8, implicit TU tiling is applied even though cu_coded_flag is equal to 0 for inter prediction mode. These internal transform block boundaries within the CU is considered as transform block boundary in deblocking filter process. However, in the VTM8.0, the above case is treated as a TU which has the same size as CU therefore there is no internal transform block boundary. Although the mismatch has already resolved by the ticket #899, this cause misalignment in terms of filter length on the each block boundary with same property.

In this proposal, to fix abovementioned behavior two solutions are suggested.

- Method1: Implicit TU tiling is restricted when cu_coded_flag is equal to 0.

- Method2: Implicit TU tiling is retained but edges inside a CU are not treated as transform edge when cu_coded_flag is equal to 0.

Since Ticket #899 is reverted by Method 1 and deblocking filter process is fixed to be performed same as VTM8.0 by Method 2, the experimental results for both solutions are same as results of VTM8.0.

In revision 1, it is added the simulation results based on Ticket #899 bugfix.

After the fix of ticket #899, there is no mismatch between text and software.

The suggested solution 1 would revert the solution of the ticket, and require additional checks.

Also the solution 2 would be requiring additional logic.

There is no problem with the current design, no need for action.

JVET-R0300 Additional fix for long luma deblocking decisions [K. Andersson, J. Enhorn (Ericsson)]

The contribution proposes to fix the long deblocking decision such that all lines of respective 4 samples boundary segment are checked to avoid over filtering of lines 1 and 2 due to decision based only on line 0 and line 3. It is asserted that the proposal ensures that the deblocking filtering is robust and that the fix does not increase worst case complexity for deblocking decisions.

The BD rate impact for luma for CTC SDR:

AI: 0.00%, RA: -0.01%, LDB: -0.05%, LDP: -0.01%.

The BD rate impact for CTC HDR:

AI:  DE:   0.01% PSNRL: 0.00% wPSNRY: 0.00%, PSNRY: 0.00%

RA: DE: -0.04% PSNRL: -0.03% wPSNRY: -0.02%, PSNRY: 0.00%

Similar encoding and decoding time as the anchor.

The number of operations increases, but is still less than worst case for deblocking.

The target is rather a corner case, it is reported that the effect was visible in the sequence slide editing. Other experts mentioned that deblocking for screen content is a very special case, where an encoder might want to align parameters.

Not obvious that there is need for action on this issue.

JVET-R0279 AHG9: On decoupling luma deblocking parameters [K. Misra, F. Bossen, J. Samuelsson, S. Deshpande, A. Segall (Sharp Labs of America)]

In the current VVC draft, the deblocking activation and clipping threshold values (beta and tc) are common for the luma long filters and luma strong filters. This contribution proposes that the threshold values for each type of filtering be separate. This is achieved by signaling separate beta offset and tc offset for luma long filters and luma strong filters. The proposed modification enables the decoupling of activation and clipping control thresholds for the two types of filtering. The proposed change is asserted to be a desirable improvement over the signalling in the current VVC draft.

Question: Is there evidence that this is needed? Can it be expected that the values would be so different?

It also requires some additional logic at low level, switching the offset values between two different options.

No action.

#### Adaptive loop filter (7)

#### Reviewed in session 2.2 Thu 9 April 1705-1720 and session 2.3 2100- UTC (chaired by JRO)

JVET-R0133 AHG16: On Clipping values for Non-linear ALF [T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki (Sony)] [late]

In VVC WD, clipping values for Non-linear ALF filtering process are defined in a lookup table, as a form of “2N” depending on BitDepth and clipIdx, where N is an integer value. When BitDepth is equal to 16 and clipIdx is equal to 0, clipping values become 216; Thus, the lookup table needs up to 17 bits per element.

This contribution proposes to replace the clipping values in a form of “2N” to a form of “2N - 1”; Any clipping values are kept within 16 bits and can be derived by simple logical operation without the lookup table. It is asserted that the proposed method has negligible bdrate changes of (0.00%,0.00%, 0.00%) for AI, (0.00%, -0.05%, 0.01%) for RA and (0.01%, -0.06%, -0.17%) for LDB under CTC.

In v2, results of IBDI equal to 8/12 are attached.

It is observed that:

- For IBDI=8, average bdrate changes are (0.02%, 0.10%, 0.11%) for AI, (0.03%, -0.19%, -0.21%) for RA and (0.02%, -0.40%, -0.44%) for LB.

- For IBDI=12, average bdrate changes are (0.00%, -0.01%, 0.01%) for AI, (0.00%, -0.03%, 0.07%) for RA and (0.05%, 0.03%, 0.04%) for LB.

Presented Thu April 9 1705 UTC (chaired by JRO)

The advantage would only apply to case of profiles beyond 15 bit, where the cost of lookup table storage seems almost irrelevant

There was a contribution in Gothenburg (JVET-O0188) which proposed the same approach. It was not adopted by that time

Several experts expressed that this change is not needed, as in the only case that would require 17 bit implementation clipping would have no effect and could be skipped.

No action.

JVET-R0208 AHG16: Rounding correction for ALF virtual boundary processing [A. M. Kotra, S. Esenlik, B. Wang, H. Gao, E. Alshina (Huawei)]

In VTM-8.0 [1], to avoid extreme padding for the sample rows which are immediately adjacent to the adaptive loop filter (ALF) virtual boundary, the correction value applied during the filtering is quantized by a larger value 1024 (210) instead of 128(27). However the rounding value used during the filtering is still 64. The current proposal proposes a fix by changing the rounding value to 512 when the quantization value used in the ALF filtering is 1024.

The objective results, over VTM8.0 Anchor for CTC configuration are as follows:

Config.	Y	U	V	EncT	DecT

AI	0.00%	0.01%	0.01%	100%	99%

RA	0.01%	-0.01%	0.00%	100%	100%

LDB	0.01%	0.02%	-0.07%	100%	102%

LDP	0.01%	-0.25%	-0.14%	100%	103%

By modifying filters at virtual boundaries per adoption of Q0150 solution 2, the rounding operation in case of shift 10 is no longer doing the nearest integer rounding. This appears as an inconsistency rather than a bug. This issue had been detected during software integration. Proponents of Q0150 also support this change. It is agreed that the change is minor and there is no harm that it would introduce any problems.

R0231 method 1 and R0291 target the same problem, basically the same solution but different specification text. There may be more elegant ways of expressing the change than suggested in R0208, which introduces another column in the table 45/46, e.g. by an equation.

Recommendation(cleanup/text+software): The rounding operation in case of the modified filter at virtual boundary should be aligned. Editors should decide the best way of expressing it in text.

JVET-R0231 AHG2: Rounding offsets for adaptive loop filter [N. Hu, V. Seregin, M. Karczewicz (Qualcomm)]

Virtual boundary (VB) processing is adopted to VVC to avoid line buffer increment for adaptive loop filter (ALF) and cross component adaptive loop filter (CC-ALF). Symmetrical sample padding is applied when VB processing is applied to ALF and CC-ALF. In some cases, extreme padding for the closest row on each side of a VB may introduce visual artifacts. In VVC, to account for that in ALF when ALF is applied to samples on the rows adjacent to a VB, filter strength is reduced by increasing the right shift for ALF filtering. However, the rounding offset of the right shift is not changed and is kept the same for all values of the right shift. In addition, the filter strength for samples on the rows adjacent to a virtual boundary is not changed in CC-ALF. In this contribution, at first, the rounding offset is changed for different right shift values for ALF. In another aspect, the filter strength is reduced when applying CC-ALF to the samples on the rows adjacent to a VB. Compared to VTM-8.0, the average BD-rate for the proposed methods is as follows:

| Results over VTM-8.0 | All Intra |  |  | Random Access |  |  | Low Delay B |  |  | Low Delay P |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Method 1 | 0.00% | 0.01% | 0.01% | 0.01% | -0.01% | 0.00% | 0.01% | 0.02% | -0.07% | 0.01% | -0.25% | -0.14% |
| Method 2 | 0.00% | 0.05% | 0.05% | 0.00% | -0.01% | 0.00% | -0.05% | 0.00% | -0.09% | -0.01% | -0.31% | 0.20% |
| Method 3 | 0.00% | 0.07% | 0.07% | 0.01% | 0.01% | 0.04% | -0.04% | 0.09% | 0.05% | 0.01% | -0.32% | -0.17% |

Method 1 is identical to R0208.

Method 2 applies the modified boundary processing of Q0150 to CCALF

Method 3 combines methods 1 and 2.

It is asked if there is evidence that a similar problem of artifacts is present in CCALF which was shown in ALF in context of the Q0150 adoption. Currently, there is no such evidence.

The main argument is for design consistency. ALF and CCALF could share the table 45, but otherwise equations would be different. In implementations, different logic would likely be used.

Benefit not obvious – no action on Method 2/3.

JVET-R0291 AHG16: On ALF attenuation near virtual boundaries [F. Bossen (Sharp)] [late]

Not presented yet, as this is a late document. It was verbally reported that all issues raised in this document are also covered by other contributions (R0208, R0231, R0233, R0312).

JVET-R0299 Additional fix for ALF virtual boundary processing [K. Andersson, J. Ström, Z. Zhang, J. Enhorn (Ericsson)]

At the last previous meeting, a low complexity fix for ALF virtual horizontal CTU boundary was adopted from JVET-Q0150. An alternative approach proposed in the same contribution was rejected since the increase of two luma line buffers and two chroma line buffers for each component was undesirable. This contribution proposes a combination of the two approaches in JVET-Q0150 as follows: Filtering of a row just above the virtual horizontal CTU boundary is performed as currently using the low complexity technique, i.e., not using samples below the virtual horizontal CTU boundary. When filtering a row just below the virtual horizontal CTU boundary on the other hand, this contribution proposes to change the filtering process so as to let it use also one row just above the virtual horizontal CTU boundary. This combined approach is asserted to further reduce visual artifacts from virtual horizontal CTU boundary processing. Proposal 1 of this contribution only changes the ALF filtering. It is claimed that the memory cost for proposal 1 is 60% of one 10-bit line buffers for luma samples and 60% of one 10-bit line buffer for each chroma channel for chroma samples. Proposal 2 of this contribution combines proposal 1 with an approach that avoids filtering across the virtual boundary also for SAO when filtering samples just below the virtual boundary, by employing padding. Samples just above the virtual boundary are SAO-filtered as currently. It is claimed that proposal 2 comes at no memory cost in terms of line buffers over the current draft of VVC.

The claimed benefit of the proposal is suppression of coding artifacts from virtual horizontal CTU boundary processing. The BD rate impact for luma for CTC is as follows:

Proposal 1: AI: -0.01%, RA: -0.03%, LDB: -0.09%

Proposal 2: AI: -0.01%, RA: -0.xx%. LDB: -0.xx%

Similar encoding and decoding times as the anchor are reported.

It is claimed that the method improves over the method from Q0150 adopted by last meeting, but requires approx. 0.6 additional line buffer (by using buffer jointly with SAO).

It is however pointed out that SAO does not need to store sample values, so it would be more like 1 line buffer.

The proposal would require a substantial amount of changes, and the additional subjective benefit over the Q0150 method may not be too large.

No action.

JVET-R0312 AHG2/AHG16: A fix on chroma ALF virtual boundary position [Y. Wang, L. Zhang, H. Liu, K. Zhang, Z. Deng (Bytedance)]

In current VVC, luma and chroma ALF virtual boundaries are always set to four and two lines above the bottom luma CTB and chroma CTB boundaries, respectively. Such a design works well for 4:2:0 colour format since the height of a chroma CTB is half of that of a luma CTB. However, for 4:2:2 and 4:4:4 colour formats in which heights of a luma CTB and a chroma CTB are equal, it is asserted that the design could result in misaligned ALF virtual boundaries for luma and chroma samples. This contribution proposes to align ALF virtual boundaries of luma and chroma components for 4:2:2 and 4:4:4 colour formats. The simulation results for 4:4:4 colour format screen sequences, 4:4:4 and 4:2:2 natural sequences following the common test conditions are summarized as follows:

Dual tree on:

AI: {0.00%, -0.01%, 0.00%}; RA: {-0.02%, -0.02%, -0.03%}; LDB: {0.02%, -0.01%, 0.08%}

Dual tree off:

AI: {0.00%, 0.00%, 0.00%}; RA: {0.02%, 0.02%, -0.02%}; LDB: {0.01%, 0.07%, 0.05%}

Natural sequences:

YUV 4:4:4, AI: {0.00%, -0.02%, -0.04%}; RA: {0.01, -0.07%, 0.03%}; LDB: {}

YUV 4:2:2, AI: {0.00%, 0.00%, -0.02%}; RA: {0.01, -0.11%, -0.08%}; LDB: {0.00%, -0.15%, 0.01%}

It is proposed that for the cases of 4:4:4 and 4:2:0 the virtual boundary height should be aligned for luma and chroma.

It is pointed out that in terms of quality this may not be needed, as luma and chroma have different characteristics.

The motivation is about improving pipelining.

From the discussion, it is not fully clear if this would have consequences on the interaction with deblocking and SAO in the pipeline. There are different opinions on that.

Contribution 233 method is identical. See further discussion there.

JVET-R0322 ALF virtual boundary issue [X.W. Meng (PKU), X. Zheng (DJI), S.S. Wang, S.W. Ma (PKU)] [late]

#### CCALF (5)

JVET-R0128 AHG16: On CCALF clipping [M. G. Sarwer, Y. Ye, J. Luo (Alibaba)]

JVET-R0230 AHG2: Syntax clean-up for cross component adaptive loop filter [N. Hu, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-R0233 AHG16: Line buffer problem of CC-ALF for 4:2:2 and 4:4:4 sequences [N. Hu, V. Seregin, M. Karczewicz (Qualcomm)]

In VVC draft 8, cross component adaptive loop filter (CC-ALF) is adopted to refine chroma components by using luma samples. To get an offset for a chroma sample, CC-ALF is applied to luma samples (after applying luma sample adaptive offset) where the centre of the filter is the co-located luma sample of the current chroma sample.

To reduce the line buffer requirement for adaptive loop filter (ALF) and CC-ALF, virtual boundary (VB) processing is applied to both ALF and CC-ALF. The position of a VB is 4 lines of luma samples and 2 lines of chroma samples above a horizontal coding tree unit (CTU) boundary. When ALF or CC-ALF is applied, a current to-be-filtered sample above (below resp.) a VB can not use samples below (above resp.) the VB.

However, when CC-ALF is applied to 4:2:2 and 4:4:4 video sequences, in some cases, when current to-be-filtered chroma sample is above a VB, its co-located luma sample is below the VB, which conflicts the design of VB.

In this contribution, three methods are proposed to solve the problem. Compared to VTM-8.0, the average BD-rate for the proposed methods is as follows:

Results over VTM-8.0	All Intra	Random Access	Low Delay B	Low Delay P

Y	U	V	Y	U	V	Y	U	V	Y	U	V

Method 1	0.00%	0.00%	0.01%	0.01%	-0.03%	0.02%	0.01%	-0.06%	0.01%	-0.02%	-0.02%	-0.04%

Method 2	0.00%	0.02%	0.06%	0.02%	0.04%	0.11%	0.00%	0.03%	0.08%	-0.06%	0.00%	0.04%

Method 3	0.00%	-0.01%	-0.02%	0.00%	-0.06%	-0.03%	0.01%	-0.07%	-0.05%	-0.02%	-0.04%	-0.13%

Discussed in session 2.3 Thu 9 Apr 2225-2300UTC (chaired by JRO)

Method 3 is conceptually identical with R0312. The problem is that in case of 4:4:4 and 4:2:2 the processing of chroma in CCALF can only be started when the VB processing of luma at co-located positions has been finished. As a consequence, two additional line buffers are required for each chroma component (above the 2 lines of chroma VB).

The problem only arises due to CCALF, but as the VB definition of ALF and CCALF is identical, cannot be separated. Method 3 is not saving any line buffers, but just redefines the height of the chroma VB.

It is not obvious (different opinions) that method 3 has a clear benefit. It claims to be more consistent between luma and chroma VB processing for 444 and 422, but on the other hand is less consistent with 420 chroma in those cases.

Methods 1 and 2 are saving the additional chroma line buffers in 444 and 422. Method 1 proposes to use luma samples from line above which are not co-located. Method 2 skips CCALF for the two rows where the additional line buffers would be necessary. Both methods would require some additional logic. It is not known whether they might impose subjective artifacts. Likely, the second method seems preferable in both aspects.

It is mentioned that for 444 (which requires more memory anyway) the four additional line buffers are not too critical. Clarify this aspect – revisit. If the line buffers are not asserted to be critical, the current design should be good enough. Otherwise, method 2 should be considered.

JVET-R0259 AHG7: On CCALF filtering of chroma sample location type-2 content [M. G. Sarwer, Y. Ye, J. Luo (Alibaba)]

JVET-R0313 AHG2/AHG16: Cleanups of chroma ALF and CC-ALF on/off control [Y. Wang, L. Zhang, H. Liu, K. Zhang (Bytedance)]

#### Luma mapping with chroma scaling (2)

JVET-R0290 AHG16: LMCS constraint cleanup [F. Bossen (Sharp)]

JVET-R0330 AHG16: On clipping average luma value for chroma residual scaling factor derivation [X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai)]

### Transforms and transform signalling (1)

JVET-R0345 Unified primary transform kernel for ISP mode [J.-Y. Huo, W.-H. Qiao, H.-X. Wang, Y.-Z. Ma, F.-Z. Yang (Xidian Univ.), S. Wan (NPU), Y.-F. Yu, Y. Liu (OPPO)] [late]

### LFNST (15)

JVET-R0056 LFNST complexity reduction [T.-D. Chuang, M.-S. Chiang, Z.-Y. Lin, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-R0057 LFNST redundant syntax removal [T.-D. Chuang, M.-S. Chiang, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-R0167 Issue on LFNST index signaling condition [K. Abe, T. Toma (Panasonic)]

JVET-R0174 LFNST index signaling [C. Rosewarne, J. Gan (Canon)]

JVET-R0176 On chroma LFNST [C. Rosewarne, J. Gan (Canon)]

JVET-R0234 Removal of redundant LFNST index signalling [H. E. Egilmez, A. Nalci, V. Seregin, W.-J. Chien, M. Karczewicz (Qualcomm)]

JVET-R0235 Removal of LFNST for chroma components [H. E. Egilmez, A. Nalci, V. Seregin, W.-J. Chien, T. Hsieh, M. Karczewicz (Qualcomm)]

JVET-R0236 Latency reduction in transformation process with TU-level signalling [H. E. Egilmez, A. Nalci, V. Seregin, W.-J. Chien, T. Hsieh, M. Karczewicz (Qualcomm)]

JVET-R0303 Modified LFNST signalling for single tree blocks [Y. Kidani, K. Unno, K. Kawamura (KDDI)]

JVET-R0304 Restriction on LFNST signalling for local dual tree chroma coding blocks [Y. Kidani, K. Unno, K. Kawamura (KDDI)]

JVET-R0318 Alternative methods of LFNST index signaling [M. Koo, M. Salehifar, J. Lim, S. Kim (LGE)]

JVET-R0319 The interaction between LFNST and BDPCM [M. Koo, M. Salehifar, J. Lim, S. Kim (LGE)]

JVET-R0331 Cleanup of LFNST signalling in single tree [X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai)]

JVET-R0352 On LFNST in shared tree [J. Jung, D. Kim, G. Ko, J.-H. Son, J. S. Kwak (WILUS)] [late]

JVET-R0358 A combined solution for latency reduction in transformation process with TU-level signalling and removal of chroma LFNST [H.E. Egilmez, A. Nalci, V. Seregin, W.-J. Chien, T. Hsieh, M. Karczewicz (Qualcomm), T.-D. Chuang, M.-S. Chiang, Z.-Y. Lin, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek), X. Xiu, T.-C. Ma, Y.-W. Chen, H.-J. Jhu, X. Wang (Kwai), C. Rosewarne, J. Gan (Canon)] [late]

### Partitioning (5)

JVET-R0131 AHG2: On Chroma QT split in 4:2:2 format coding [H. Huang, W.-J Chien, V. Seregin, M. Karczewicz (Qualcomm), T.-D. Chuang, C.-M. Tsai, S.-T. Hsiang, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-R0268 Implicit binary split at picture boundary [G. Li, X. Li, S. Liu (Tencent)]

JVET-R0269 Fix on minimum QT size value range [G. Li, X. Li, S. Liu (Tencent)]

JVET-R0316 AhG16: Normative constraints on BT and TT split under MER [Y. Wang, K. Zhang, L. Zhang, H. Liu, Z. Deng (Bytedance)]

JVET-R0347 AHG2: On minimum QT size, maximum BT size and maximum TT size [H. Huang, J. Chen, W.-J. Chien, M. Karczewicz (Qualcomm)] [late]

### ACT related (4)

JVET-R0305 CU level transform size restriction for adaptive color transform [L.-F. Chen, X. Li, S. Liu (Tencent)]

JVET-R0329 Mismatch on clipping input residuals to IACT [X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai), J. Zhao, S.-H. Kim (LGE)]

JVET-R0336 Adaptive colour transform clean-ups [J. Xu, L. Zhang, W. Zhu, K. Zhang (Bytedance)]

JVET-R0355 On clipping input residuals to IACT [X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai), J. Zhao, S.-H. Kim (LGE), J. Xu, L. Zhang, W. Zhu, K. Zhang (Bytedance)] [late]

### Other (1)

JVET-R0348 AHG16: On clipping operation for residual samples [T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki (Sony)] [late]

### Test conditions (1)

JVET-R0321 AHG3: Chroma QP table bug-fix and CTC update for RGB coding in VTM-8.0 [J. Xu, L. Zhang, W. Zhu (Bytedance), X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai)]

### Encoder optimization (6)

JVET-R0110 AHG14: Mixed lossy/lossless coding of VTM reference software [M. G. Sarwer, Y. Ye, J. Luo (Alibaba)]

JVET-R0140 AHG14: Max BT/TT size restriction for lossless coding encoder configuration [T. Zhou, E. Sasaki, T. Ikai (Sharp)]

JVET-R0143 AHG14: Configuration parameter to enable TSRC for lossless coding [C. Hollmann, M. Damghanian, L. Litwic, M. von Strauss (Ericsson)]

JVET-R0164 AHG10: Mean-scaled SATD for VTM encoder [J. Lainema, A. Hallapuro (Nokia)]

JVET-R0327 CCALF encoder optimization X.W. Meng (PKU), X. Zheng (DJI), S.S. Wang, S.W. Ma (PKU)] [late]

JVET-R0328 ALF encoder optimization [X.W. Meng (PKU), X. Zheng (DJI), S.S. Wang, S.W. Ma (PKU)] [late]

## AHG6: 360° video coding tools, software and test conditions (1)

JVET-R0151 AHG6/AHG12: Uncoded subpictures and potential applications [J. Sauer (RWTH Aachen Univ.)]

## AHG11: Screen content coding (9)

JVET-R0145 AHG 11/15: On the use of limited EGk signalling [J. Gan, C. Rosewarne (Canon)]

Only four low level syntax elements remain (cu_qp_delta_abs, palette_predictor_run, num_signalled_palette_entries, and palette_escape_val) that use the general EGk binarisation. EGk is the only non-truncated binarisation used for low level signalling. This contribution proposes to replace the use of EGk with limited EGk for the four low level syntax elements. This change in binarisation has previously occurred for residual coding and mvd coding. It is asserted that the proposed changed binarisations remove unbounded codes from low-level syntax.

Presented Thu 9 1317 UTC (chaired by JRO)

There is no problem solved by this proposal, EGk has been used practically for a long time

It is pointed out that the EGk has advantages for syntax elements that change the valid range, such as delta QP

No benefit in terms of simplification

Other experts expressed concerns about adoption

No action.

JVET-R0146 AHG11: Context coded bin limits for palette coding [J. Gan, C. Rosewarne (Canon)]

To limit the required throughput of the CABAC engine, the number of context coded bins is limited to 1.75 bins per sample. This limit is explicitly calculated and enforced in the residual coding process. In palette coding, the number of context coded bins is not explicitly limited. With the adoption of JVET-Q0504, an “evil” bitstream can be constructed that would reach 1.984 context coded bins/sample. This contribution proposes a number of options for solving this bug. The preferred option explicitly calculates the context coded bins limit for palette mode CUs.

Presented Thu 9 1333 UTC (chaired by JRO)

More investigation is necessary to understand if the worst case is really exceeded, as

- The comparison is only made against the max number of transform coefficients per color component. As palette either codes all color comp jointly, this might only exceed the max number for monochrome case (which only would apply if there was a dedicated monochrome profile)

- In residual coding, additional context coded bins occur for motion, mode etc.

Revisit – more detailed analysis needed to understand if the problem exists, when the worst case for all context coded bins in residual coding is considered.

JVET-R0229 AHG11: Fixed number of reuse flags for palette mode [R.-L. Liao, Y. Ye, M. G. Sarwer (Alibaba)]

In the VVC draft 8, number of reuse flags to be decoded is related to the size of the palette predictor. In the case that two neighboring blocks are both coded in palette mode, the syntax of the second block cannot be parsed until the new palette predictor size is obtained. In this contribution, the number of reuse flags is set to a fixed value, that is, 63 and 31 for single tree slice and dual tree slice, respectively. Therefore, the syntax of second block can be parsed with no dependency on how large the previous block’s palette predictor is. Fixed number of reuse flags can be achieved by two methods: 1) initializing the palette predictor size to the fixed value; 2) add a requirement of bitstream conformance to palette_predictor_run signaling. It is reported that, as compared to VTM-8.0 with palette on, the overall coding performance impact for {Y, U, V} in 4:4:4 color format is:

- #1:{0.01%,0.01%,0.01%}for AI,{0.03%,0.05%,0.01%}for RA,{x.xx%,x.xx%,x.xx%}for LB

- #2:{0.01%,0.00%,0.02%}for AI,{0.03%,0.04%,-0.01%}for RA,{x.xx%,x.xx%,x.xx%}for LB

For class F in 4:2:0 color format, the overall coding performance impact for {Y, U, V} is:

- #1:{-0.01%,-0.07%,0.03%}for AI,{0.01%, 0.08%,0.07%}for RA,{x.xx%,x.xx%,x.xx%}for LB

- #2:{-0.02%,-0.04%,0.05%}for AI,{-0.02%,0.10%,0.08%}for RA,{x.xx%,x.xx%,x.xx%}for LB

For class TGM in 4:2:0 color format, the overall coding performance impact for {Y, U, V} is:

- #1:{0.00%,0.01%,0.00%}for AI,{0.05%,-0.06%,-0.04%}for RA,{0.01%,-0.09%,0.11%}for LB

#2:{0.01%,0.00%,-0.01%}for AI,{0.05%,-0.04%,-0.04%}for RA,{0.07%,-0.06%,0.07%}for LB

Presented Thu 9 1350 UTC (chaired by JRO)

It is not obvious that there is a severe issue. It is mentioned that method 1 could even be more complicated.

It is pointed out that JVET-R0310 is same as method 1. However, in that other contribution it is claimed that a parsing dependency exists. Several experts pointed out that there is no parsing dependency as it is typically defined.

Also the bit stream restriction does not seem to be necessary.

No action.

JVET-R0240 AHG11: On maximum palette size and palette predictor size [Y.-H. Chao, T. Hsieh, W.-J. Chien, V. Seregin, M. Karczewicz (Qualcomm)]

In this contribution, it is proposed to reduce maximum palette and palette predictor size. Three methods are proposed in this contribution:

- Reduce the maximum palette and palette predictor size to 16 and 32 in single tree slice and to 8 and 16 in dual tree slice

- Reduce the maximum palette and palette predictor size to 16 and 32 in single tree slice and to 8 and 16 in dual tree slice for YUV420 format, while the size for other color formats stays the same as in VVC draft 8.

- Allow the signaling of maximum palette size and maximum palette predictor size in SPS, same syntax as in HEVC SCC extension. The allowed maximum palette and palette predictor size are restricted to 32 and 64.

The results of method 1 on YUV4:4:4 screen content sequences versus non-420 CTC anchor (PLT=1) are shown as:

1.48% AI, 1.05% RA and x.xx% LDB

The results of method 1 and 2 on YUV4:2:0 class F and TGM versus VTM8 with PLT=1 are shown as:

Class F: 0.16% AI, 0.30% RA and 0.09% LDB

Class TGM: 0.71% AI, 0.59% RA, and 0.19% LDB

The results of method 3 with maximum palette and palette predictor size signaled to be 16 and 32 for YUV4:2:0 format versus VTM8 with PLT=1 are shown as:

Class F: 0.16% AI, 0.30% RA and 0.09% LDB

Class TGM: 0.72% AI, 0.59% RA, and 0.19% LDB

Presented Thu 9 1407 UTC (chaired by JRO)

Presentation deck not uploaded.

Method 3 does not reduce decoder worst case complexity (and was also proposed previously, but not adopted). Also method 2 is not reducing worst case complexity, as 4:2:0 is not the worst case. Method 1 is reducing the complexity in terms of CABAC throughput, and the memory storage, but also has loss of up to 1.5% (AI)

Memory is not a significant issue here, according other experts’ opinion

It is not obvious that CABAC throughput of palette is the worst case.

No evidence that further reduction of the palette size is needed.

JVET-R0309 [AHG16] Clean-up on palette predictor update for local dual tree [H. Jang, J. Nam, S. Yoo, N. Park, S. Kim, J. Lim (LGE)]

This contribution proposes to clean-up regarding predictor palette update for local dual tree structure. Three alternative clean-up methods are suggested as described below:

- Clean-up 1. There is mismatch between SPEC and SW. Here, propose to fix SPEC to align with SW regarding local dual tree predictor palette update process.

- Clean-up 2. Update predictor palette entry only for luma component but not for chroma component in local dual tree structure.

- Clean-up 3. Remove palette predictor update process of local dual tree structure.

The suggested clean-up method 2 shows {0.00%, 0.00%, -0.06%} and method 3 shows{0.00%, 0.07%, 0.10%} coding performance respectively for average of ClassF/SCC under CTC with palette on. It is asserted that proposed clean-up methods save H/W power consumption with closed to zero coding loss and also cut worst case decoding latency by disabling predictor palette update for local dual tree.

Presented Thu 9 1427 UTC (chaired by JRO)

For “clean-up 1”, spec should be aligned with software. This is also included in JVET-R0333. See further notes there.

Items #2 and #3 try to simplify the interaction of palette with local dual tree structures. It is agreed that there is no problem in the spec such as unspecified decoder behaviour. Also in local dual tree the total predictor table size is identical with single tree. No need for action on these items.

JVET-R0310 [AHG16] Clean-up by removing parsing dependency for palette [H. Jang, J. Nam, S. Yoo, N. Park, S. Kim, J. Lim (LGE)]

See notes under JVET-R0229.

JVET-R0320 AHG11: Maximum QP for escape value in palette coding [J. Xu, L. Zhang, W. Zhu, K. Zhang (Bytedance)]

JVET-R0333 AHG11: Mismatches related to palette prediction [H.-J. Jhu, X. Xiu, Y.-W. Chen, T.-C. Ma, X. Wang (Kwai Inc.)] [late]

This contribution reports some mismatches between VVC draft 8 and test model software VTM-8.0 on updating the palette prediction in local dual tree cases. First, in the VVC draft 8, under local-dual tree case the update process of palette prediction is performed only for chroma CU while in the VTM-8.0 it is performed for both luma and chroma CUs. Secondly, in the VVC draft 8, the palette prediction for a luma CU under a local-dual tree does not use the chroma values of those entries in the palette predictor, and vice versa. But in the VTM-8.0, luma and chroma values of each entry in the palette predictor are used together in palette prediction.

In this contribution, one method is proposed to solve the first mismatch and two methods are proposed to solve the second mismatch in different ways. In the first change, it is proposed to change the specification text to align with the VTM-8.0 software for palette update in the first mismatch. The proposed change has no impact on coding performance. In the second change, it is proposed to change the specification text to align with the VTM-8.0 software for forming a palette in the second mismatch. The proposed change has no impact on coding performance. In the third change, in forming a palette, the same default values used for signaled palette entries are also used for predicted palette entries to fix the second mismatch. The proposed change reports negligible BD-rate changes compared to VTM8.0.

Presented Thu 9 1450 UTC (chaired by JRO)

Presentation deck not included.

It is obvious that the text spec is incomplete. The third change which modifies both text and software does not seem to be justified, as it is not simpler and has some small coding loss.

Recommendation (text BF): first and second change (as called in v2 of word or v3 zip) should be adopted.

Session 2.1 ended Thu April 9 1505 UTC

JVET-R0334 AHG11: Simplification of palette mode for local dual tree cases [H.-J. Jhu, X. Xiu, Y.-W. Chen, T.-C. Ma, X. Wang (Kwai Inc.)] [late]

## AHG14: Lossless and near-lossless coding (23)

Reviewed in session 2.4 Thursday 9 April 2320 - Friday 10 April 0115 (chaired by BB)

### Transform skip-related (22)

JVET-R0045 AHG15: cleanup for signaling of minimum QP of transform skip [J. Li, K. Abe (Panasonic)]

This contribution proposes to replace syntax “min_qp_prime_ts_minus4” (in range of [0-48]) with “internal_minus_input_bit_depth” (in range of[0-8]) to remove redundancy of signalling and make the design more compact.

It was commented that the name of the new syntax element could be changed and the semantics could be improved.

There was a question whether an encoder would choose one of the values of min_qp_prime_ts_minus4 not suited for lossless coding. Participants were generally in favor of this cleanup.

Recommendation (cleanup): adopt (with editorial improvements of the specification draft text).

JVET-R0049 AHG9: HLS on disabling TSRC [S.-T. Hsiang, C.-W. Hsu, Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

In VVC Draft 8, extra bit costs are consumed for signalling the new syntax element slice_ts_residual_coding_disabled_flag for each of the coded slices. This contribution proposes three methods for high-level syntax modifications related to disabling transform skip residual coding, as follows:

- In Method 1, a new syntax element sps_ts_residual_coding_disabled_slice_present_flag is added to the sequence parameter set (SPS) to specify whether slice_ts_residual_coding_disabled_flag is present in the slice header.

- In Method 2, two new syntax elements sps_ts_residual_coding_disabled_slice_present_flag and sps_ts_residual_coding_disabled_slice_default_flag are added to the SPS. When slice_ts_residual_coding_disabled_flag is not present, it is inferred to be equal to the value of sps_ts_residual_coding_disabled_slice_default_flag.

- In Method 3, signalling slice_ts_residual_coding_disabled_flag in the slice header is conditioned on sps_transform_skip_enabled_flag equal to 1 without adding any new syntax element.

Method 1 same is in JVET-R0097.

Method 3 same as in JVET-R0097, JVET-R0068 (item 8) plus inference to 1 for slice_ts_residual_coding_disabled_flag, JVET-R0142, JVET-R0317 without the PPS flag, JVET-R0153 aspect 1, JVET-R0182 with inverse semantics.

The motivation behind method 1 is to not signal slice_ts_residual_coding_disabled_flag for lossy coding cases, which are considered to be the main application.

It was commented that at the last meeting, slice_ts_residual_coding_disabled_flag was adopted as a slice level flag instead of PPS or SPS flag. This would not prevent syntax as proposed here to gate its presence.

A participant questions whether the additional control syntax to save the signalling of slice_ts_residual_coding_disabled_flag is really needed. Functionality is not affected by any of the proposed methods.

It was agreed that it is desirable to not always send slice_ts_residual_coding_disabled_flag since it is expected to only be used for lossless coding scenarios.

It was further agreed that it makes sense to condition slice_ts_residual_coding_disabled_flag on sps_transform_skip_enabled_flag (Method 3).

Method 3:

SPS: sps_transform_skip_enabled_flag

SH: if(sps_transform_skip_enabled_flag )

slice_ts_residual_coding_disabled_flag

Method 1:

SPS: sps_transform_skip_enabled_flag

SPS: if(sps_transform_skip_enabled_flag )

sps_ts_residual_coding_disabled_slice_present_flag

SH: if(sps_ts_residual_coding_disabled_slice_present_flag)

slice_ts_residual_coding_disabled_flag

Recommendation (cleanup): method 3 (revisit to consider method 1).

JVET-R0068 AHG8/AHG9/AHG12: Miscellaneous HLS topics [Y.-K. Wang, L. Zhang, Z. Deng, J. Xu, K. Zhang, K. Fan (Bytedance)]

Item 8 of this contribution belongs to this category.

See notes under JVET-R0049.

JVET-R0083 AHG14: Residual coding constraints for transform skip blocks [A. Nalci, H.E. Egilmez, M. Coban, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-R0084 AHG14: On signaling for lossless coding [M. Karczewicz, M. Coban, A. Nalci, H.E. Egilmez, V. Seregin (Qualcomm), T.-C. Ma, X. Xiu, Y.-W. Chen, H.-J. Jhu, X. Wang (Kwai Inc.)]

JVET-R0097 AHG9: Transform and transform-skip related HLS clean-up [M. G. Sarwer, Y. Ye, J. Luo, J. Chen (Alibaba)]

See notes under JVET-R0049 for aspect 2.

JVET-R0116 AHG11/AHG14: On sign data hiding of transform skip block [M. G. Sarwer, Y. Ye, J. Luo (Alibaba)]

JVET-R0139 AHG9: High-level control flag for lossless coding [T. Zhou, E. Sasaki, T. Ikai (Sharp)]

JVET-R0141 Disabling Dependent Quantization and Sign Data Hiding in Transform Skip blocks [T. Hashimoto, E. Sasaki, T. Aono, T. Ikai (Sharp)]

JVET-R0142 AHG9: Slice header signalling clean up [T. Hashimoto, T. Aono, T. Ikai (Sharp)]

See notes under JVET-R0049.

JVET-R0144 AHG14: On lossless operation with RRC [J. Gan, C. Rosewarne (Canon)]

JVET-R0153 AHG9/AHG16: On slice_ts_residual_coding_disabled_flag [J. Choi, S. Yoo, J. Heo, J. Choi, J. Lim, S. Kim (LGE)]

See notes under JVET-R0049 for aspect 1.

JVET-R0154 AHG9/16: On sign data hiding for BDPCM blocks [S. Yoo, J. Choi, J. Lim, S. Kim (LGE)]

JVET-R0155 AHG9/14: On lossless coding granularity [S. Yoo, J. Choi, J. Lim, S. Kim (LGE)]

JVET-R0182 AHG9: Removed Redundant Slice Level TSRC Flag [K. Naser, F. Le Leannec, T. Poirier, M. Kerdranvat (InterDigital)]

See notes under JVET-R0049.

JVET-R0219 Alternative block size conditions for BDPCM [K. Unno, K. Kawamura, S. Naito (KDDI)]

JVET-R0258 AHG9: Reduce redundant signalling in picture header [J. Enhorn, M. Pettersson, R. Sjöberg, M. Damghanian, Z. Zhang (Ericsson)]

The ph_dep_quant_enabled_flag aspect of item 1 of this contribution belongs to this category.

JVET-R0271 AHG9: High-level constraints of dependent quantization and sign data hiding [A. Nalci, M. Coban, M. Karczewicz (Qualcomm)]

JVET-R0317 AHG9: On slice transform skip residual coding method signalling [M. Coban, V. Seregin, Y. He, A. Nalci, M. Karczewicz (Qualcomm)]

See notes under JVET-R0049 except the PPS aspect.

JVET-R0325 AHG14: Disabling dependent quantization and sign bit hiding for transform skip mode [T.-C. Ma, X. Xiu, Y.-W. Chen, H.-J. Jhu, X. Wang (Kwai Inc.)]

JVET-R0353 AHG14: On Interaction between ACT and BDPCM [T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki (Sony)] [late]

JVET-R0354 AHG14: BDPCM for Inter/IBC-predicted residuals [T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki (Sony)] [late]

### Other (1)

JVET-R0169 AHG14: Report of CABAC skip mode results on VTM-8.0 [K. Abe, T. Toma, V. Drugeon (Panasonic)]

## AHG15: Quantization control (4)

JVET-R0055 AHG15: On referencing a non-existent scaling list [C.-Y. Lai, O. Chubach, C.-Y. Chen, T.-D. Chuang, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-R0127 AHG15: On scaling list prediction [A. K. Ramasubramonian, B. Ray, G. Van der Auwera, M. Karczewicz (Qualcomm)]

JVET-R0166 AHG15: Issue on chroma scaling matrix for 4:4:4 [K. Abe, T. Toma (Panasonic)]

JVET-R0326 AHG15: On Chroma Quantization Matrix Signaling [H. Zhang, X. Li, G. Li, L. Li, S. Liu (Tencent)]

## AHG16: Implementation studies (4)

JVET-R0224 AHG16: Realization of RPR based real-time VVC decode and playback on ARM based mobile devices [J. Shingala, A. Natesan, A. Chelawat (Ittiam)] [late]

JVET-R0359 AHG 17: Illustration of the film grain characteristics SEI message for VVC [Sean McCarthy, Fangjun Pu, Taoran Lu, Peng Yin, Walt Husak, Tao Chen] [late]

JVET-R0351 High bit depth coding [A. Browne, S. Keating, K. Sharman (Sony)] [late]

JVET-R0364 Information on cinematic aspect ratios in the context of JVET-Q0065 [Sean McCarthy, Walt Husak, Peng Yin, Taoran Lu, Fangjun Pu, Tao Chen] [late]

# Annex A:
List of meeting participants

The participants of the seventeenth meeting of the JVET, according to an attendance sheet circulated during the meeting sessions (approximately XXX people in total), were as follows: