| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0340-v4 |
| --- | --- |

| Title: | Agenda and report of the category 2 AHG pre-meeting of the 18th JVET meeting |  |  |
| --- | --- | --- | --- |
| Status: | Report document from the coordinators of the category 2 AHG pre-meeting |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jens-Rainer Ohm Institute of Communication Engineering RWTH Aachen Melatener Straße 23 D-52074 Aachen Benjamin Bross Fraunhofer HHI Andrew Segall Sharp Labs of America, Inc. Yan Ye Alibaba Group | Tel: Email: Email: Email: Email: | +49 241 80 27671 ohm@ient.rwth-aachen.de benjamin.bross@hhi.fraunhofer.de asegall@sharplabs.com yan.ye@alibaba-inc.com |
| Source: | Coordinators of the category 2 AHG pre-meeting |  |  |

_____________________________

# Summary

The category 2 AHG pre-meeting of the 18th JVET meeting, for addressing coding tool topics, mostly in the scopes of AHGs 2, 3, 6, 11, 14, 15 and 16, held the following teleconference meeting sessions on April 9 (dates and time slots are all in UTC time):

- 1300-1500 1st "afternoon" session [break after 2 hours]

- 1515-1715 2nd "afternoon" session

- ["dinner" break – nearly 4 hours – enough for some side work or a nap]

- 2100-2300 1st "night" session [break after 2 hours]

- 2315-0115 2nd "night" session

- [end of day – nearly 12 hours to the next session]

and the following teleconference meeting sessions on April 14 (dates and time slots are all in UTC time):

- 0500-0700 1st "morning" session [break after 2 hours]

- 0715-0915 2nd "morning" session

- ["lunch" break – nearly 4 hours – enough for some side work or a nap]

- 1300-1500 1st "afternoon" session [break after 2 hours]

- 1515-1715 2nd "afternoon" session

- [end of day – nearly 12 hours to the next session]

Approximately 227 people attended the AHG pre-meeting, and approximately 45 input documents were discussed.

Remarks from opening session 2.1 on Thu 9 April 2020, 1300 UTC (chaired by JRO)

- AHG reports will contain recommendations developed during AHG pre-meetings, decisions will only be made during the JVET meeting

- Goal is to cluster documents with related topics, and identify need for action

- At this point of standard development, stability of the design should have highest priority

- Late documents are all those registered more than 12 hours after the announced “AHG deadline” of April 3 – those could only be discussed if time allows

The following recommendations were made to the JVET:

- R0223 (Ed.) Editorial improvement of DMVR process left to editor.

- R0134 (BF text) Mismatch related to deblocking of subblock motion edges

- R0168 (BF text) Issue on bS derivation of deblocking filter (check interaction with R0228)

- R0228 (BF text) Bugfix to deblocking filter boundary strength setting for palette

- R0208 (cleanup/text+software) Rounding correction for ALF virtual boundary processing

- R0330 (Ed. + SW cleanup) Remove clipping average luma value for chroma residual scaling

- R0309 (BF text) Mismatches related to palette prediction (1st+2nd, R0333 cleanup 1)

- R0045 (cleanup) Cleanup for signaling of minimum QP of transform skip

- R0049 (cleanup) Condition slice_ts_residual_coding_disabled_flag on sps_transform_skip_enabled_flag (method 3, also R0097)

- R0166 (BF SW) Issue on chroma scaling matrix for 4:4:4 (method 1)

- R0326 (BF) Infer scaling_list_copy_mode_flag[id] to1 when not present (method 2, also R0055)

Revisits:

- R0137 Definition of NoBackwardPredFlag and ColPic (revisit with HLS group)

- R0292/R0293 Fixes for inter/intra/affine interpolation filters

- R0233 Line buffer problem of CC-ALF for 4:2:2 and 4:4:4 sequences (method 2)

- R0305 CU level transform size restriction for adaptive color transform (pending results)

- R0146 Context coded bin limits for palette coding (pending on more detailed analysis)

- R0049 Add additional SPS flag to gate slice_ts_residual_coding_disabled_flag (method 1, also R0097, R0068 item 8, R0142, R0317, R0153 aspect 1, R0182)

- R0083, R0116 Interaction of DQ, SDH and TSRC (also R0141, R0144, R0154, R0325)

# Attendance

The list of participants in the AHG pre-meeting meeting can be found in Annex A of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/‌SC 29/‌WG 11 (including experts who had been personally invited as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the responsible coordinators.

# AHG reports (7)

These reports are related but have not been reviewed in the pre-meeting:

JVET-R0002 JVET AHG report: Draft text and test model algorithm description editing (AHG2) [B. Bross, J. Chen, J. Boyce, S. Kim, S. Liu, Y.-K. Wang, Y. Ye]

JVET-R0003 JVET AHG report: Test model software development (AHG3) [F. Bossen, X. Li, K. Sühring]

JVET-R0006 JVET AHG report: 360° video coding tools, software and test conditions (AHG6) [J. Boyce, J. He, K. Choi, J.-L. Lin, Y. Ye]

JVET-R0011 JVET AHG report: Screen content coding (AHG11) [S. Liu, J. Boyce, A. Filippov, Y.-C. Sun, J. Xu]

JVET-R0014 JVET AHG report: Lossless and near-lossless coding (AHG14) [T. Nguyen, T.-C. Ma, M. Ikeda, H. Jang, X. Zhao]

JVET-R0015 JVET AHG report: Quantization control (AHG15) [R. Chernyak, E. François, C. Helmrich, S. McCarthy, A. Segall]

JVET-R0016 JVET AHG report: Implementation studies (AHG16) [M. Zhou, J. An, E. Chai, K. Choi, S. Sethuraman, T. Hsieh, X. Xiu]

# Coding tool proposals (109)

## AHG2/AHG3/AHG16: General coding tools (68)

### Inter prediction and MV coding (15)

Discussed in session 1.8 Tuesday 14 April 1530-1720 (chaired by JRO)

JVET-R0137 On mvd_l1_zero_flag and NoBackwardPredFlag [T. Chujoh, E. Sasaki, T. Ikai (Sharp)]

Only first aspect (problem 1) on NoBackwardPredFlag and ColPic

In this contribution, some solutions for two problems of current VVC Draft 8 have been proposed. One problem is that there is no specification of the variables ColPic and NoBackwardPredFlag and the other problem is that mvd_l1_zero_flag is specified in only picture header even if reference picture list structure can be changed on slice header. Two solutions for the first problem have been shown Option 1 is that the variable ColPic as the almost same as that of HEVC is defined and a new variable IdenticalDirectionalFlag which is replaced to previous NoBackwadPredFlag is specified by using the decoding process for symmetric motion vector difference reference indices. Option 2 is that the variables ColPic and NoBackwadPredFlag as the almost same as that of HEVC are defined. Also, two solutions for the second problem have been shown. Option 1 is that the change of enabling condition of symmetric motion vector difference and mvd_l1_zero_flag by the variable IdenticalDirectionalFlag and option 2 is that mvd_l1_zero_flag is specified in the picture header or in the slice header by the syntax element rpl_info_in_ph_flag exclusively. Neither proposal changes the results of the CTC.

The proposal is filling an existing hole regarding the definition of collocated picture and NoBackwardPredFlag.

The definition proposed for collocated picture seems appropriate (just transferring the HEVC method which is also matching with software). In terms of the NoBackwardPredFlag, the proposed option 1.1 seems to deviate from the SW implementation, option 1.2 also but with less change.

It is recommended to fill the gap in the spec by transferring the corresponding text from HEVC as much as possible, while matching with the decoding process as implemented in SW. The difference compared to HEVC is e.g. related to processing of long term pictures. To be further discussed with HLS experts what the issues are – revisit.

JVET-R0212 On modes in geometric partitioning [C. Hollmann, D. Liu, R. Yu, J. Ström (Ericsson)]

In this contribution three methods to reduce the number of modes for geometric partitioning are presented. These methods are claimed to reduce the number of modes from 64 to 50, 38 and 32, respectively. It is further claimed that these methods have a minor impact on the compression efficiency. It is also asserted that the number of combinations that are required to be tested during verification testing is reduced by up to 50%.

- Method 1 (50 modes): 0.03% RA, 0.00% LDB, 700 combinations to test (-22%)

- Method 2 (38 modes): 0.05% RA, 0.01% LDB, 532 combinations to test (-40%)

- Method 3 (32 modes): 0.07% RA, 0.06% LDB, 448 combinations to test (-50%)

The main intent is reducing the number of combinations for conformance testing.

The issue of testing a large number of combinations in geo was already discussed in the last meeting, and the adopted solution was agreed to be a good compromise.

No action.

JVET-R0385 Crosscheck of JVET-R0212 (On modes in geometric partitioning) [K. Zhang (Bytedance)] [late]

JVET-R0213 Modifications of motion storage in geometric partition mode [R. Yu, D. Liu, C. Hollmann, J. Ström (Ericsson)]

In the current VVC, for a geometric partition mode (GPM) coded block, the block is split into two partitions with a splitting line defined by an angle and a distance. Each partition is associated with a uni-motion. The prediction sample for each partition is generated using the uni-motion. For the prediction samples near the splitting line, a blending operation is carried out to reduce discontinuity.

The motion storage process for the GPM coded block stores three types of motion. The three types of motion are the two uni-motions associated with each partition and a third motion which is a combination of the two uni-motions. The third motion is also referred to as type2 motion. Each 4x4 subblock within the GPM coded block stores one of the three types of motion. The type2 motion is stored within 4x4 subblocks that are within the blending area. It is asserted that since the blending area is narrow in general the storage of the type2 motion will be unnecessary.

This contribution proposes to remove the storage of type2 motion for GPM coded blocks. In other words, only the two uni-motions are stored. It is claimed that with the modification, one absolute operation and one comparison operation for each 4x4 subblock can be saved if the motion storage map is computed on the fly. It is also claimed that the determination process for the type2 motion can also be removed. It is further claimed that the specification text for the motion storage for GPM can be significantly cleaned up. The modification was implemented in the VTM-8.0 and the BD-rate impact is reported to be -0.01% for RA and 0.01% for LDB. It is proposed to adopt the modification considering that the BD-rate impact is negligible.

There is no real problem to be solved, several experts pointed out that similar methods had been proposed earlier in the context of TPM. Some concern was also expressed with regard to possible impact on subjective quality with regard to blending.

No action.

JVET-R0389 Crosscheck of JVET-R0213 (Modifications of motion storage in geometric partition mode) [Z. Deng (Bytedance)] [late]

JVET-R0223 AHG16: On DMVR and wraparound motion compensation [J. Luo, J. Chen, Y. Ye (Alibaba)]

In VVC draft 8, when wrap around motion compensation is enabled, the bilinear interpolation in DMVR motion search process also uses wrap-around clipping operation. In this contribution, it is proposed to apply regular clipping operation during DMVR motion search, and apply wrap-around clipping operation only for regular interpolation during the final motion compensation process. It is asserted that this simplifies the motion search process in DMVR. Experiment results reportedly show that the BD rate difference is -0.01%, -0.01%, -0.02% for end-to-end WS-PSNR for Y, U and V respectively. Informal subjective viewing was conducted and no visible difference was observed.

Results with 360 video PERP.

It was commented that the implementation seems to become more complicated, as an additional reference area fetch would become necessary for DMVR.

No action on the proposal

There is another aspect in the proposed text that the DMVR text could be simplified, e.g. in terms of that the combination with RPR would never be used. Recommendation (Ed.): Editorial improvement left to editor.

JVET-R0425 Crosscheck of JVET-R0223 (AHG16: On DMVR and wraparound motion compensation) [Y.-H. Lee, J.-L. Lin (MediaTek)] [late]

JVET-R0282 GEO with MMVD [K. Panusopone, S. Hong, L. Wang (Nokia)]

This contribution proposes to harmonize GEO with MMVD such that MVD can be applied to derive MV of a GEO partition. The proposed method first determines base MV using current GEO MV calculation, then computes MVD for each base MV following method like MVD calculation for MMVD. Simulation results show the proposed method has BD-rate of approximately -0.25%, -0.36%, -0.49% for RA, and -0.51%, -0.66%, -0.97% for LB, respectively, compared to VTM-8.0 anchor.

Encoding time increase is roughly 30% for RA, and over 30% for LB. This is not an attractive tradeoff.

No action.

JVET-R0407 Crosscheck of JVET-R0282: GEO with MMVD [K. Reuzé (??)] [late]

JVET-R0292 Fixes for 4-tap interpolation filtering [K. Andersson, R. Yu, Z. Zhang, J. Ström (Ericsson)]

Only one aspect related to inter:

- 4-tap interpolation filter for chroma motion compensation

- 4-tap interpolation filter for intra angular prediction

It is asserted that the current 4-tap interpolation filters which are used both for intra angular prediction and chroma motion compensation have significant phase misalignments compared to the ideal phases of the interpolation filters. For ten of the interpolation filters the phase misalignments are as large as the expected phase differences between two interpolation filters of adjacent fractional positions. This contribution proposes 4-tap interpolation filters that fixes the phase misalignments of the current 4-tap interpolation filters.

The fixes were reported to be tested under VTM-8.0 CTC. The impact on coding efficiency is reported for Luma/Cb/Cr as follows:

-0.05%/-0.03%-0.06% for AI, -0.02%/-0.05%/-0.15% for RA and -0.11%/-0.24%/-0.30% for LDB.

The impact of only fixing the 4-tap interpolation filters for intra is reported to be:

-0.05%/-0.03%-0.06% for AI, -0.04%/-0.04%/-0.02% for RA and -0.06%/0.01%/-0.09% for LDB.

Other experts commented that the problem of phase mismatch is known to them and expressed that an alignment might be desirable.

It is asked if the energy gain of the filters is close to unity over the different filters from the set?

It was also asked if there could be visual impact? For coded video, proponents did not observe differences, but they did not check the prediction.

It is confirmed that the 16 bit precision is retained.

It is pointed out that the variation of magnitude responses among the different filters of the set may be of concern (this also relates to the energy gain question). Is it better or worse in that compared to the current filters?

Revisit (along with the related contribution R0293)

JVET-R0293 Fixes for 6-tap interpolation filtering for affine motion compensation [K. Andersson, R. Yu, Z. Zhang, J. Ström (Ericsson)]

JVET-R0311 [AHG2] Fix cu_skip_flag signaling for IBC [H. Jang, J. Nam, N. Park, S. Kim, J. Lim (LGE)]

JVET-R0357 Geometric prediction mode with motion vector difference [K. Zhang, L. Zhang, Z. Deng, H. Liu, Y. Wang (Bytedance)] [AHG late]

JVET-R0429 Cross-check of JVET-R0357: Geometric prediction mode with motion vector differences [C. Hollmann (Ericsson)] [late]

JVET-R0366 Simplified disLut for GPM [Y.-Z. Ma, Q.-H. Ran, R.-P. Qiu, H.-X. Wang, J.-Y. Huo, F.-Z. Yang (Xidian Univ), S. Wan (NPU), Y.-F. Yu, Y. Liu (OPPO)] [AHG late]

JVET-R0447 Crosscheck of JVET-R0366 (Simplified disLut for GPM) [Y.-W. Chen (Kwai Inc.)] [late]

JVET-R0367 Adjustment of shiftHor calculation in GPM [Y.-Z. Ma, Q.-H. Ran, R.-P. Qiu, M.-L. Zhang, J.-Y. Huo, F.-Z. Yang (Xidian Univ), S. Wan (NPU), Y.-F. Yu, Y. Liu (OPPO)] [AHG late]

JVET-R0448 Crosscheck of JVET-R0367 (Adjustment of shiftHor calculation in GPM) [Y.-W. Chen (Kwai Inc.)] [late]

JVET-R0368 GPM merge list construction modification [Y.-Z. Ma, Q.-H. Ran, R.-P. Qiu, J.-Y. Huo, F.-Z. Yang (Xidian Univ), S. Wan (NPU), Y.-F. Yu, Y. Liu (OPPO)] [AHG late]

JVET-R0422 Crosscheck of JVET-R0368 (GPM merge list construction modification) [H. Chen, H. Yang (Huawei)] [late]

JVET-R0369 Combination of JVET-R0367 and JVET-R0368 for GPM [Y.-Z. Ma, Q.-H. Ran, R.-P. Qiu, J.-Y. Huo, F.-Z. Yang (Xidian Univ), S. Wan (NPU), Y.-F. Yu, Y. Liu (OPPO)] [AHG late]

JVET-R0423 Crosscheck of JVET-R0369 (Combination of JVET-R0367 and JVET-R0368 for GPM) [H. Chen, H. Yang (Huawei)] [late]

JVET-R0371 AHG2/9: On max num of subblock merge candidates [H. Huang, J. Chen, W.-J. Chien, M. Karczewicz (Qualcomm)] [AHG late]

JVET-R0373 AHG9: On Maximum Number of Subblock Merge Candidates [Y.-C. Yang, C.-Y. Teng (Foxconn)] [AHG late]

### Intra prediction and mode coding (9)

JVET-R0280 AHG16: Cleanup of intra reference sample filter selection [J. Heo, H. Jang, J. Choi, J. Nam, M. Koo, J. Lim, S. Kim (LGE)]

JVET-R0432 Crosscheck of JVET-R0280 (AHG16: Cleanup of intra reference sample filter selection) [F. Bossen (Sharp)] [late]

JVET-R0281 AHG16: Cleanup MIP flag signaling [J. Heo, H. Jang, J. Choi, J. Lim, S. Kim (LGE)]

JVET-R0435 Crosscheck of JVET-R0281 (AHG16: Cleanup MIP flag signalling) [J. Pfaff (HHI)] [late]

JVET-R0288 AHG16: Reference samples for ISP [F. Bossen (Sharp)]

JVET-R0399 Crosscheck of JVET-R0288 (AHG16: Reference samples for ISP) [S. De-Luxán-Hernández (HHI)] [late]

JVET-R0314 AHG2/AHG16: Fixes on CCLM [Y. Wang, K. Zhang, L. Zhang, H. Liu (Bytedance)]

JVET-R0350 MIP for all channels in the case of 4:4:4 and single tree [J. Pfaff, B. Stallenberger, P. Merkle, M. Schäfer, P. Helle, T. Hinz, H. Schwarz, D. Marpe, T. Wiegand (HHI)] [AHG late]

JVET-R0356 CCLM-related bugfixes for the VVC specification draft [A. Filippov, V. Rufitskiy, E. Alshina (Huawei)] [AHG late]

JVET-R0375 AHG2/AHG16: CCLM bug fix in luma reference down-sampling [L. Pham Van, G. Van Der Auwera, J. Chen, V. Seregin, M. Karczewicz (Qualcomm)] [AHG late]

JVET-R0434 Crosscheck of JVET-R0375 (AHG2/AHG16: CCLM bug fix in luma reference down-sampling) [J. Pfaff (HHI)] [late]

JVET-R0391 Simplification on CCLM [L. Li, X. Li, S. Liu (Tencent)] [AHG late]

JVET-R0449 Crosscheck of JVET-R0391 (Simplification on CCLM) [Y.-W. Chen (Kwai Inc.)] [late]

JVET-R0452 CCLM: common text for spec bugfixes [L. Li, X. Li, S. Liu (Tencent), A. Filippov, V. Rufitskiy, E. Alshina (Huawei)] [late]

### Loop filtering (24)

#### Deblocking filter (10)

Reviewed in session 2.2 Thu 9 April 1520-1705 UTC (chaired by JRO)

JVET-R0130 Cleanup of tC value derivation process for deblocking filter [S. Iwamura, S. Nemoto, A. Ichigaya (NHK)] [AHG late]

TBP

JVET-R0134 AHG2: Mismatch related to deblocking of subblock motion edges [B. Heng, M. Zhou, W. Wan (Broadcom)]

This contribution asserts that there is a mismatch between the VVC draft text and VTM sotware related to deblocking of coding subblock boundaries. Within a subblock motion CU, the length of the deblocking filter used depends on the distance the nearest transform edge. However, when this neighboring transform edge aligns with a virtual boundary, the behavior of the text and software differ.

Specifically, the VTM software treats the neighboring transform edge as a transform edge, regardless of whether it aligns with a virtual boundary or not. While the VVC draft text ignores the neighboring transform edge altogether if it aligns with a virtual boundary. This difference will cause the text and software to use different filter lengths for subblock motion edges, and therefore they will produce different results.

This contribution proposes to modify the text to match the software behavior to resolve this mismatch. Proposed text changes are provided.

Presented Thu 9 April 1733 (chaired by JRO).

There is a ticket #857 which also identifies this issue (as well as other issues). This was partially resolved by submitting a software patch. The new contribution points out that after that we have still a mismatch between text and software. Text appears to be appropriate.

Recommendation (BF text): The proposed text changes should be adopted.

This also would resolve ticket #857 as far as the text is concerned.

JVET-R0168 Issue on bS derivation of deblocking filter [K. Abe, T. Toma (Panasonic)]

This contribution points out the mismatch between VVC text and VTM on bS derivation process of deblocking filter. In VVC text, bS is set equal to 1 for the boundary between IBC and inter block on both luma edge and chroma edge. On the other hand, in VTM, bS is set equal to 1 for the boundary between IBC and inter block only on luma edge. The proponent of this contribution thinks there are two solutions, solution1: fix VTM to align to VVC text, solution2: fix VVC text to align to VTM. This contribution shows the difference of coding performance and text changes for both solutions.

No ticket yet.

It is mentioned by the proponents that solution 1 would have small impact on coding results, therefore they would better suggest aligning the text with software.

Several experts expressed support for solution 2, as also in the past it had been agreed that MV differences should not be checked for chroma deblocking.

Recommendation (BF text): The proposed text changes should be adopted. There may however be some interaction with a related issue in R0228, where palette mode is also considered.

JVET-R0372 Crosscheck of JVET-R0168 (Issue on bS derivation of deblocking filter) [T. Hashimoto, T. Ikai (Sharp)]

JVET-R0228 AHG11: Bugfix to deblocking filter boundary strength setting for palette [R.-L. Liao, Y. Ye, M. G. Sarwer (Alibaba)]

In VVC draft 8, the deblocking filter boundary strength is set according to the coding modes of two neighboring blocks along a deblocking edge. However, it is reported that, when one of neighboring blocks is coded in palette and the other is coded in IBC or inter mode, the boundary filtering strength is not clearly defined in the spec. It is also reported that in VTM-8.0, this ambiguity partially exists as well, and can cause the software mismatch in encoder and decoder in some cases. In this contribution, it is proposed to clearly define the boundary filtering strength for the aforementioned case by setting it to one of 1 or 2. It is reported that, as compared to VTM-8.0 with palette off, the overall coding performance impact for {Y, U, V} in 4:4:4 color format is:

- VTM8:{-6.25%,-6.42%,-6.26%}for AI,{-5.04%,-6.81%,-6.86%}for RA,{-3.18%,-4.39%,-4.60%}for LB

- bS = 1:{-6.25%,-6.42%,-6.26%}for AI,{-5.03%,-6.82%,-6.87%}for RA,{-3.20%,-4.34%,-4.16%}for LB

- bS = 2:{-6.25%,-6.42%,-6.26%}for AI,{-5.02%,-6.76%,-6.81%}for RA,{-3.14%,-4.26%,-4.41%}for LB

For class F in 4:2:0 color format, the overall coding performance impact for {Y, U, V} is:

- VTM8:{-1.24%,-0.43%,-0.47%}for AI,{-1.29%,-0.47%,-0.56%}for RA,{-0.56%,-0.24%,-1.24%}for LB

- bS = 1:{-1.24%,-0.43%,-0.47%}for AI,{-1.29%,-0.48%,-0.51%}for RA,{-0.61%,-0.14%,-0.76%}for LB

- bS = 2:{-1.24%,-0.43%,-0.47%}for AI,{-1.32%,-0.53%,-0.52%}for RA,{-0.60%,-0.38%,-0.98%}for LB

For class SCC in 4:2:0 color format, the overall coding performance impact for {Y, U, V} is:

- VTM8:{-6.50%,-5.18%,-4.94%}for AI,{-3.78%,-2.45%,-2.42%}for RA,{-1.22%,-0.59%,-0.66%}for LB

- bS = 1:{-6.50%,-5.18%,-4.94%}for AI,{-3.76%,-2.42%,-2.43%}for RA,{-1.23%,-0.57%,-0.60%}for LB

- bS = 2:{-6.50%,-5.18%,-4.94%}for AI,{-3.76%,-2.36%,-2.39%}for RA,{-1.28%,-0.68%,-0.69%}for LB

In terms of subjective quality, no significant visual difference was observed in all three cases based on the informal subjective viewing conducted.

It I agreed during the discussion that the spec text does not clearly define the bS in case where palette is used at the other side of the block. For the case of not using local dual tree, the VTM mode of operation is clearly defined. For this case, it is agreed that the text should be aligned with the software (setting Bs=1 when the other side is a palette block, and operated in single tree mode). For local dual tree, more investigation is necessary to understand if the SW is covering all possible cases (e.g. when luma is palette and chroma in IBC, etc.)

Recommendation (BF text): The proposed text change on more clearly specifying bS in case of palette should be adopted. There may however be some interaction with a related issue in R0168

Revisit: Further offline study needed for the case of local dual tree.

JVET-R0440 Crosscheck of JVET-R0228 (AHG11: Bugfix to deblocking filter boundary strength setting for palette) [Y.-H. Chao (Qualcomm)] [late]

JVET-R0289 [AHG16] On deblocking filter process [N. Park, J. Nam, H. Jang, J. Lim, S. Kim (LGE)]

In Ticket #899, it was claimed that there is a mismatch between VVC draft 8 and VTM8.0 on deblocking filter process when CU size is greater than maximum transform block size and cu_coded_flag is equal to 0.

In VVC draft 8, implicit TU tiling is applied even though cu_coded_flag is equal to 0 for inter prediction mode. These internal transform block boundaries within the CU is considered as transform block boundary in deblocking filter process. However, in the VTM8.0, the above case is treated as a TU which has the same size as CU therefore there is no internal transform block boundary. Although the mismatch has already resolved by the ticket #899, this cause misalignment in terms of filter length on the each block boundary with same property.

In this proposal, to fix abovementioned behavior two solutions are suggested.

- Method1: Implicit TU tiling is restricted when cu_coded_flag is equal to 0.

- Method2: Implicit TU tiling is retained but edges inside a CU are not treated as transform edge when cu_coded_flag is equal to 0.

Since Ticket #899 is reverted by Method 1 and deblocking filter process is fixed to be performed same as VTM8.0 by Method 2, the experimental results for both solutions are same as results of VTM8.0.

In revision 1, it is added the simulation results based on Ticket #899 bugfix.

After the fix of ticket #899, there is no mismatch between text and software.

The suggested solution 1 would revert the solution of the ticket, and require additional checks.

Also the solution 2 would be requiring additional logic.

There is no problem with the current design, no need for action.

JVET-R0395 Crosscheck of JVET-R0289 ([AHG16] On deblocking filter process) [R.-L. Liao (Alibaba)] [late]

JVET-R0300 Additional fix for long luma deblocking decisions [K. Andersson, J. Enhorn (Ericsson)]

The contribution proposes to fix the long deblocking decision such that all lines of respective 4 samples boundary segment are checked to avoid over filtering of lines 1 and 2 due to decision based only on line 0 and line 3. It is asserted that the proposal ensures that the deblocking filtering is robust and that the fix does not increase worst case complexity for deblocking decisions.

The BD rate impact for luma for CTC SDR:

AI: 0.00%, RA: -0.01%, LDB: -0.05%, LDP: -0.01%.

The BD rate impact for CTC HDR:

AI:  DE:   0.01% PSNRL: 0.00% wPSNRY: 0.00%, PSNRY: 0.00%

RA: DE: -0.04% PSNRL: -0.03% wPSNRY: -0.02%, PSNRY: 0.00%

Similar encoding and decoding time as the anchor.

The number of operations increases, but is still less than worst case for deblocking.

The target is rather a corner case, it is reported that the effect was visible in the sequence slide editing. Other experts mentioned that deblocking for screen content is a very special case, where an encoder might want to align parameters.

Not obvious that there is need for action on this issue.

JVET-R0279 AHG9: On decoupling luma deblocking parameters [K. Misra, F. Bossen, J. Samuelsson, S. Deshpande, A. Segall (Sharp Labs of America)]

In the current VVC draft, the deblocking activation and clipping threshold values (beta and tc) are common for the luma long filters and luma strong filters. This contribution proposes that the threshold values for each type of filtering be separate. This is achieved by signaling separate beta offset and tc offset for luma long filters and luma strong filters. The proposed modification enables the decoupling of activation and clipping control thresholds for the two types of filtering. The proposed change is asserted to be a desirable improvement over the signalling in the current VVC draft.

Question: Is there evidence that this is needed? Can it be expected that the values would be so different?

It also requires some additional logic at low level, switching the offset values between two different options.

No action.

JVET-R0388 AHG9: Cleanups on deblocking signalling [Z. Deng, Y.-K. Wang, L. Zhang, K. Zhang, J. Xu (Bytedance)] [AHG late]

JVET-R0403 On the boundary strength derivation of IBC coded blocks [B. Ray, G. Van der Auwera, M.Karczewicz (Qualcomm)] [late] [AHG late]

JVET-R0454 Cross-check of JVET-R0403 (On the boundary strength derivation of IBC coded blocks) [K. Andersson (Ericsson)]

JVET-R0437 Combination of JVET-R0168 and JVET-R0228 on deblocking filter boundary strength setting [R.-L. Liao, Y. Ye, M. G. Sarwer (Alibaba), K. Abe, T. Toma (Panasonic)] [late]

#### Adaptive loop filter (7)

Reviewed in session 2.2 Thu 9 April 1705-1720 and session 2.3 2100- UTC (chaired by JRO)

JVET-R0133 AHG16: On Clipping values for Non-linear ALF [T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki (Sony)] [AHG late]

In VVC WD, clipping values for Non-linear ALF filtering process are defined in a lookup table, as a form of “2N” depending on BitDepth and clipIdx, where N is an integer value. When BitDepth is equal to 16 and clipIdx is equal to 0, clipping values become 216; Thus, the lookup table needs up to 17 bits per element.

This contribution proposes to replace the clipping values in a form of “2N” to a form of “2N - 1”; Any clipping values are kept within 16 bits and can be derived by simple logical operation without the lookup table. It is asserted that the proposed method has negligible bdrate changes of (0.00%,0.00%, 0.00%) for AI, (0.00%, -0.05%, 0.01%) for RA and (0.01%, -0.06%, -0.17%) for LDB under CTC.

In v2, results of IBDI equal to 8/12 are attached.

It is observed that:

- For IBDI=8, average bdrate changes are (0.02%, 0.10%, 0.11%) for AI, (0.03%, -0.19%, -0.21%) for RA and (0.02%, -0.40%, -0.44%) for LB.

- For IBDI=12, average bdrate changes are (0.00%, -0.01%, 0.01%) for AI, (0.00%, -0.03%, 0.07%) for RA and (0.05%, 0.03%, 0.04%) for LB.

Presented Thu April 9 1705 UTC (chaired by JRO)

The advantage would only apply to case of profiles beyond 15 bit, where the cost of lookup table storage seems almost irrelevant

There was a contribution in Gothenburg (JVET-O0188) which proposed the same approach. It was not adopted by that time

Several experts expressed that this change is not needed, as in the only case that would require 17 bit implementation clipping would have no effect and could be skipped.

No action.

JVET-R0208 AHG16: Rounding correction for ALF virtual boundary processing [A. M. Kotra, S. Esenlik, B. Wang, H. Gao, E. Alshina (Huawei)]

In VTM-8.0 [1], to avoid extreme padding for the sample rows which are immediately adjacent to the adaptive loop filter (ALF) virtual boundary, the correction value applied during the filtering is quantized by a larger value 1024 (210) instead of 128(27). However the rounding value used during the filtering is still 64. The current proposal proposes a fix by changing the rounding value to 512 when the quantization value used in the ALF filtering is 1024.

The objective results, over VTM8.0 Anchor for CTC configuration are as follows:

Config.	Y	U	V	EncT	DecT

AI	0.00%	0.01%	0.01%	100%	99%

RA	0.01%	-0.01%	0.00%	100%	100%

LDB	0.01%	0.02%	-0.07%	100%	102%

LDP	0.01%	-0.25%	-0.14%	100%	103%

By modifying filters at virtual boundaries per adoption of Q0150 solution 2, the rounding operation in case of shift 10 is no longer doing the nearest integer rounding. This appears as an inconsistency rather than a bug. This issue had been detected during software integration. Proponents of Q0150 also support this change. It is agreed that the change is minor and there is no harm that it would introduce any problems.

R0231 method 1 and R0291 target the same problem, basically the same solution but different specification text. There may be more elegant ways of expressing the change than suggested in R0208, which introduces another column in the table 45/46, e.g. by an equation.

Recommendation(cleanup/text+software): The rounding operation in case of the modified filter at virtual boundary should be aligned. Editors should decide the best way of expressing it in text.

JVET-R0231 AHG2: Rounding offsets for adaptive loop filter [N. Hu, V. Seregin, M. Karczewicz (Qualcomm)]

Virtual boundary (VB) processing is adopted to VVC to avoid line buffer increment for adaptive loop filter (ALF) and cross component adaptive loop filter (CC-ALF). Symmetrical sample padding is applied when VB processing is applied to ALF and CC-ALF. In some cases, extreme padding for the closest row on each side of a VB may introduce visual artifacts. In VVC, to account for that in ALF when ALF is applied to samples on the rows adjacent to a VB, filter strength is reduced by increasing the right shift for ALF filtering. However, the rounding offset of the right shift is not changed and is kept the same for all values of the right shift. In addition, the filter strength for samples on the rows adjacent to a virtual boundary is not changed in CC-ALF. In this contribution, at first, the rounding offset is changed for different right shift values for ALF. In another aspect, the filter strength is reduced when applying CC-ALF to the samples on the rows adjacent to a VB. Compared to VTM-8.0, the average BD-rate for the proposed methods is as follows:

| Results over VTM-8.0 | All Intra |  |  | Random Access |  |  | Low Delay B |  |  | Low Delay P |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Method 1 | 0.00% | 0.01% | 0.01% | 0.01% | -0.01% | 0.00% | 0.01% | 0.02% | -0.07% | 0.01% | -0.25% | -0.14% |
| Method 2 | 0.00% | 0.05% | 0.05% | 0.00% | -0.01% | 0.00% | -0.05% | 0.00% | -0.09% | -0.01% | -0.31% | 0.20% |
| Method 3 | 0.00% | 0.07% | 0.07% | 0.01% | 0.01% | 0.04% | -0.04% | 0.09% | 0.05% | 0.01% | -0.32% | -0.17% |

Method 1 is identical to R0208.

Method 2 applies the modified boundary processing of Q0150 to CCALF

Method 3 combines methods 1 and 2.

It is asked if there is evidence that a similar problem of artifacts is present in CCALF which was shown in ALF in context of the Q0150 adoption. Currently, there is no such evidence.

The main argument is for design consistency. ALF and CCALF could share the table 45, but otherwise equations would be different. In implementations, different logic would likely be used.

Benefit not obvious – no action on Method 2/3.

JVET-R0291 AHG16: On ALF attenuation near virtual boundaries [F. Bossen (Sharp)] [AHG late]

Not presented yet, as this is a late document. It was verbally reported that all issues raised in this document are also covered by other contributions (R0208, R0231, R0233, R0312).

JVET-R0444 Crosscheck of JVET-R0291 (AHG16: On ALF attenuation near virtual boundaries) [N. Hu (Qualcomm)] [late]

JVET-R0299 Additional fix for ALF virtual boundary processing [K. Andersson, J. Ström, Z. Zhang, J. Enhorn (Ericsson)]

At the last previous meeting, a low complexity fix for ALF virtual horizontal CTU boundary was adopted from JVET-Q0150. An alternative approach proposed in the same contribution was rejected since the increase of two luma line buffers and two chroma line buffers for each component was undesirable. This contribution proposes a combination of the two approaches in JVET-Q0150 as follows: Filtering of a row just above the virtual horizontal CTU boundary is performed as currently using the low complexity technique, i.e., not using samples below the virtual horizontal CTU boundary. When filtering a row just below the virtual horizontal CTU boundary on the other hand, this contribution proposes to change the filtering process so as to let it use also one row just above the virtual horizontal CTU boundary. This combined approach is asserted to further reduce visual artifacts from virtual horizontal CTU boundary processing. Proposal 1 of this contribution only changes the ALF filtering. It is claimed that the memory cost for proposal 1 is 60% of one 10-bit line buffers for luma samples and 60% of one 10-bit line buffer for each chroma channel for chroma samples. Proposal 2 of this contribution combines proposal 1 with an approach that avoids filtering across the virtual boundary also for SAO when filtering samples just below the virtual boundary, by employing padding. Samples just above the virtual boundary are SAO-filtered as currently. It is claimed that proposal 2 comes at no memory cost in terms of line buffers over the current draft of VVC.

The claimed benefit of the proposal is suppression of coding artifacts from virtual horizontal CTU boundary processing. The BD rate impact for luma for CTC is as follows:

Proposal 1: AI: -0.01%, RA: -0.03%, LDB: -0.09%

Proposal 2: AI: -0.01%, RA: -0.xx%. LDB: -0.xx%

Similar encoding and decoding times as the anchor are reported.

It is claimed that the method improves over the method from Q0150 adopted by last meeting, but requires approx. 0.6 additional line buffer (by using buffer jointly with SAO).

It is however pointed out that SAO does not need to store sample values, so it would be more like 1 line buffer.

The proposal would require a substantial amount of changes, and the additional subjective benefit over the Q0150 method may not be too large.

No action.

JVET-R0312 AHG2/AHG16: A fix on chroma ALF virtual boundary position [Y. Wang, L. Zhang, H. Liu, K. Zhang, Z. Deng (Bytedance)]

In current VVC, luma and chroma ALF virtual boundaries are always set to four and two lines above the bottom luma CTB and chroma CTB boundaries, respectively. Such a design works well for 4:2:0 colour format since the height of a chroma CTB is half of that of a luma CTB. However, for 4:2:2 and 4:4:4 colour formats in which heights of a luma CTB and a chroma CTB are equal, it is asserted that the design could result in misaligned ALF virtual boundaries for luma and chroma samples. This contribution proposes to align ALF virtual boundaries of luma and chroma components for 4:2:2 and 4:4:4 colour formats. The simulation results for 4:4:4 colour format screen sequences, 4:4:4 and 4:2:2 natural sequences following the common test conditions are summarized as follows:

Dual tree on:

AI: {0.00%, -0.01%, 0.00%}; RA: {-0.02%, -0.02%, -0.03%}; LDB: {0.02%, -0.01%, 0.08%}

Dual tree off:

AI: {0.00%, 0.00%, 0.00%}; RA: {0.02%, 0.02%, -0.02%}; LDB: {0.01%, 0.07%, 0.05%}

Natural sequences:

YUV 4:4:4, AI: {0.00%, -0.02%, -0.04%}; RA: {0.01, -0.07%, 0.03%}; LDB: {}

YUV 4:2:2, AI: {0.00%, 0.00%, -0.02%}; RA: {0.01, -0.11%, -0.08%}; LDB: {0.00%, -0.15%, 0.01%}

It is proposed that for the cases of 4:4:4 and 4:2:0 the virtual boundary height should be aligned for luma and chroma.

It is pointed out that in terms of quality this may not be needed, as luma and chroma have different characteristics.

The motivation is about improving pipelining.

From the discussion, it is not fully clear if this would have consequences on the interaction with deblocking and SAO in the pipeline. There are different opinions on that.

Contribution R0233 method is identical. See further discussion there.

JVET-R0363 Crosscheck of JVET-R0312 (AHG2/AHG16: A fix on chroma ALF virtual boundary position) [C.-M. Tsai (MediaTek)] [late]

JVET-R0322 ALF virtual boundary issue [X.W. Meng (PKU), X. Zheng (DJI), S.S. Wang, S.W. Ma (PKU)] [AHG late]

#### CCALF (5)

Contributions presented in session 2.6 Tuesday 14 April 0715-0815 except otherwise noted.

JVET-R0128 AHG16: On CCALF clipping [M. G. Sarwer, Y. Ye, J. Luo (Alibaba)]

In VVC CCALF process, an 8-tap filter is applied to luma sample to generate a residual correction for the chroma samples. At first, an offset value is generated from the luma samples and then the offset value is clipped. Then, the clipped offset value is added to the chroma sample to generate filtered output. Another clipping operation is performed to generate final filtered sample.  This contribution proposes to remove the first clipping operation (i.e. clipping the offset value before sum) from the CCALF process. Following results are reported as compared to VTM-8.0.

	AI :  0.00% (Y), 0.00% (Cb), 0.00% (Cr)

	RA : 0.00% (Y), 0.00% (Cb), 0.00% (Cr)

	LB : 0.00% (Y), 0.00% (Cb), 0.00% (Cr)

In v2, the results of HDR sequences are added.

It is reported that the first clipping is never triggered in CTC.

One reason for introducing this clipping was saving memory for intermediate storage of luma data for later use in CCALF. There is also a conformance stream designed to check if the decoder implements the clipping.

No action.

JVET-R0443 Crosscheck of JVET-R0128 (AHG16: On CCALF clipping) [N. Hu (Qualcomm)] [late]

JVET-R0230 AHG2: Syntax clean-up for cross component adaptive loop filter [N. Hu, V. Seregin, M. Karczewicz (Qualcomm)]

In VVC draft 8, cross component adaptive loop filter (CC-ALF) is adopted to refine chroma components by using luma samples. Filter coefficients of CC-ALF are signalled in adaptation parameter sets (APSs). In an APS, a Cb (Cr resp.) CC-ALF filter set with up to 4 filters could be signalled and a filter from this Cb (Cr resp.) filter set could be applied to a Cb (Cr resp.) coding tree block. On the other hand, in an APS, a chroma adaptive loop filter (ALF) filter set with up to 8 filters could be signalled and a filter from this chroma filter set could be applied to a chroma coding tree block. In this contribution, CC-ALF filters are unified for the two chroma components. Cb and Cr components share the same CC-ALF filter set in an APS, which is the same method used in a regular chroma ALF. Under common test conditions, compared with VTM-8.0, the average BD rate of the proposed method is

AI: 0.03%(Y), -0.17%(U), -0.20%(V)

RA: 0.01%(Y), -0.10%(U), 0.06%(V)

LDB: -0.04%(Y), -0.16%(U), -0.04%(V)

LDP: -0.06%(Y), -0.09%(U), 0.21%(V)

The intent of the proposal is unifying CCALF with ALF, using the same APS ID for Cb and Cr at slice.

There is nothing wrong with the current spec, and the proposal might give up some flexibility.

No action.

JVET-R0233 AHG16: Line buffer problem of CC-ALF for 4:2:2 and 4:4:4 sequences [N. Hu, V. Seregin, M. Karczewicz (Qualcomm)]

In VVC draft 8, cross component adaptive loop filter (CC-ALF) is adopted to refine chroma components by using luma samples. To get an offset for a chroma sample, CC-ALF is applied to luma samples (after applying luma sample adaptive offset) where the centre of the filter is the co-located luma sample of the current chroma sample.

To reduce the line buffer requirement for adaptive loop filter (ALF) and CC-ALF, virtual boundary (VB) processing is applied to both ALF and CC-ALF. The position of a VB is 4 lines of luma samples and 2 lines of chroma samples above a horizontal coding tree unit (CTU) boundary. When ALF or CC-ALF is applied, a current to-be-filtered sample above (below resp.) a VB can not use samples below (above resp.) the VB.

However, when CC-ALF is applied to 4:2:2 and 4:4:4 video sequences, in some cases, when current to-be-filtered chroma sample is above a VB, its co-located luma sample is below the VB, which conflicts the design of VB.

In this contribution, three methods are proposed to solve the problem. Compared to VTM-8.0, the average BD-rate for the proposed methods is as follows:

Results over VTM-8.0	All Intra	Random Access	Low Delay B	Low Delay P

Y	U	V	Y	U	V	Y	U	V	Y	U	V

Method 1	0.00%	0.00%	0.01%	0.01%	-0.03%	0.02%	0.01%	-0.06%	0.01%	-0.02%	-0.02%	-0.04%

Method 2	0.00%	0.02%	0.06%	0.02%	0.04%	0.11%	0.00%	0.03%	0.08%	-0.06%	0.00%	0.04%

Method 3	0.00%	-0.01%	-0.02%	0.00%	-0.06%	-0.03%	0.01%	-0.07%	-0.05%	-0.02%	-0.04%	-0.13%

Discussed in session 2.3 Thu 9 Apr 2225-2300UTC (chaired by JRO)

Method 3 is conceptually identical with R0312. The problem is that in case of 4:4:4 and 4:2:2 the processing of chroma in CCALF can only be started when the VB processing of luma at co-located positions has been finished. As a consequence, two additional line buffers are required for each chroma component (above the 2 lines of chroma VB).

The problem only arises due to CCALF, but as the VB definition of ALF and CCALF is identical, cannot be separated. Method 3 is not saving any line buffers, but just redefines the height of the chroma VB.

It is not obvious (different opinions) that method 3 has a clear benefit. It claims to be more consistent between luma and chroma VB processing for 444 and 422, but on the other hand is less consistent with 420 chroma in those cases.

Methods 1 and 2 are saving the additional chroma line buffers in 444 and 422. Method 1 proposes to use luma samples from line above which are not co-located. Method 2 skips CCALF for the two rows where the additional line buffers would be necessary. Both methods would require some additional logic. It is not known whether they might impose subjective artifacts. Likely, the second method seems preferable in both aspects.

It is mentioned that for 444 (which requires more memory anyway) the four additional line buffers are not too critical. Clarify this aspect – revisit. If the line buffers are not asserted to be critical, the current design should be good enough. Otherwise, method 2 should be considered.

JVET-R0387 Crosscheck of JVET-R0233 (AHG16: Line buffer problem of CC-ALF for 4:2:2 and 4:4:4 sequences) [Y. Wang (Bytedance)] [late]

JVET-R0259 AHG7: On CCALF filtering of chroma sample location type-2 content [M. G. Sarwer, Y. Ye, J. Luo (Alibaba)]

It is asserted that the CCALF filter shape is not optimal for chroma sample location type-2 content. Accordingly, this contribution proposes three cross shaped CCALF filters. Following results are reported.

For chroma sample location type-2 content,

	9 tap 5x5 cross shaped filter:

o	AI :  0.01% (Y), -0.77% (Cb), -0.81% (Cr)

o	RA:  0.07% (Y), -1.99% (Cb), -1.95% (Cr)

o	LB:  0.19% (Y), -5.85% (Cb), -9.09% (Cr)

	13 tap 7x7 cross shaped filter:

o	AI :  0.05% (Y), -1.40% (Cb), -1.34% (Cr)

o	RA:  0.04% (Y), -2.85% (Cb), -3.22% (Cr)

o	LB:   0.10% (Y), -7.28% (Cb), -11.34% (Cr)

	8 tap 5x4 cross shaped filter:

o	AI :  0.00% (Y), -0.35% (Cb), -0.39% (Cr)

o	RA:  0.05% (Y), -1.43% (Cb), -1.52% (Cr)

o	LB:   0.17% (Y), -4.10% (Cb), -7.06% (Cr)

When the proposed cross-shape filters are applied on chroma sample location type-0 content, it is reported that some coding gain can also be achieved:

	9 tap 5x5 cross shaped filter:

o	AI :  0.02% (Y), -0.26% (Cb), -0.68% (Cr)

o	RA:  0.01% (Y), -0.39 % (Cb), -0.48 % (Cr)

o	LB:  -0.04 % (Y), -1.06 % (Cb), -1.19 % (Cr)

	13 tap 7x7 cross shaped filter:

o	AI :  0.04% (Y), -0.82% (Cb), -1.16% (Cr)

o	RA:  0.00% (Y), -1.30% (Cb), -1.28% (Cr)

o	LB:   -0.02% (Y), -3.25% (Cb), -2.80% (Cr)

	8 tap 5x4 cross shaped filter:

o	AI :  0.01% (Y), -0.09% (Cb), -0.55% (Cr)

o	RA:  -0.01% (Y), -0.19% (Cb), -0.31% (Cr)

o	LB:   -0.01% (Y), -0.55% (Cb), -0.91% (Cr)

It is commented that the current filter shape could allow asymmetric tuning of coefficients regarding type 2 content.

Filters beyond 8-tap would be more complex than current design. Buffer requirements would not be increased. In hardware, also the 8-tap filter could be less regular.

Gains are mainly observed in HDR (where HDR H1 is the only type 2 sampling). Compared to the gain of CCALF in those sequences, the additional gain is approximately one tenth of that (or even less for the 8-tap filter).

Gains are largest in LB, which may not be the primary use case of CCALF.

It was asked for visual quality. It is reported by proponents that they inspected visual quality and did not find problems, nor differences compared to current CCALF.

It is commented that this is a quite substantial low level modification with the main intent of compression improvement, which only applies for certain type of content.

No action.

JVET-R0446 Crosscheck of JVET-R0259 (AHG7: On CCALF filtering of chroma sample location type-2 content) [F. Pu (Dolby)] [late]

JVET-R0313 AHG2/AHG16: Cleanups of chroma ALF and CC-ALF on/off control [Y. Wang, L. Zhang, H. Liu, K. Zhang (Bytedance)]

In current VVC, chroma ALF and CC-ALF are disabled implicitly when luma ALF is disabled at SPS/PH/SH as it is unlikely that chroma ALF/CC-ALF would be used when luma ALF is disabled. Such a design could benefit power consumption. However, luma ALF, chroma ALF and CC-ALF are controlled independently at CTU level. Therefore, for a slice, it is still possible that chroma ALF/CC-ALF is enabled in some CTUs, even luma ALF is disabled for all CTUs, which conflicts with the original intention. In this contribution, it is proposed to disable chroma ALF/CC-ALF implicitly when luma ALF is disabled for a CTU to keep the design consistent for all video processing units. Simulation results reportedly show that BD-rate changes are {0.00%, 0.02%, 0.04%}, {-0.01%, 0.18%, 0.14%}, and {-0.09%, 0.56%, 0.04%} with AI, RA, and LDB configurations under CTC, respectively.

It is commented that the reason of coupling the enabling at high level is rather an encoder choice, and it is not necessary to transfer that to the low level. The consistency argument is not necessarily applicable here.

No justification for a low level change. There is nothing conceptually broken. No action.

JVET-R0445 Crosscheck of JVET-R0313 (AHG2/AHG16: Cleanups of chroma ALF and CC-ALF on/off control) [N. Hu (Qualcomm)] [late]

#### Luma mapping with chroma scaling (3)

discussed in session 2.6 Tuesday 14 April 0825-0845

JVET-R0290 AHG16: LMCS constraint cleanup [F. Bossen (Sharp)]

It is asserted that the expression of constraints on LMCS parameters is needlessly convoluted. An alternative definition of constraints is proposed. While the proposed constraints are not strictly equivalent to the ones in VVC draft 8, no impact on coding efficiency is observed under common test conditions. It is asserted that the proposed constraint is much more straightforward.

The proposal would simplify the expression of the encoder restriction, but give up some flexibility of LMCS. No need to change a decoder implementation, though perhaps a decoder could be simplified by knowing the range is more restricted. There are however divergent opinions on this.

No urgent need of doing this change, nothing is broken, and it gives up some flexibility.

No action on this proposal

It is noted that conformance bitstreams should be made available which exercise the entire range of the current spec.

JVET-R0330 AHG16: On clipping average luma value for chroma residual scaling factor derivation [X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai)]

In VVC draft 8, the average of neighboring reconstructed luma samples above and left to one 64×64 region is used to calculate the chroma residual scaling factor for the coding units (CUs) inside the region. In the chroma sample reconstruction process 8.7.5.3 in VVC draft 8, one clipping operation is applied to clip the luma average to the full range of the internal bit-depth when deriving the chroma residual scaling factor. Additionally, the same clipping operation is also applied when generating chroma samples even if the chroma residual samples are zeros, i.e., chroma CBF is zero. It is asserted that those two clipping operations are redundant. For a cleaner design, this contribution proposes to remove those unnecessary clipping operations from the current VVC specification. Simulation results reportedly show that the proposed modification provides bit-exact BD-rate performance.

Agreed that the redundant clipping is not needed.

Recommendation (Ed. + SW cleanup): Remove the clipping from text, up to editor. Cleanup of software should be done for alignment with the text.

JVET-R0402 Crosscheck of JVET-R0330 AHG16: On clipping average luma value for chroma residual scaling factor derivation [J. Chen (Alibaba)] [late]

JVET-R0393 AHG9: On LMCS for GDR [L. Wang, S. Hong, K. Panusopone, M. M. Hannuksela (Nokia)] [AHG late]

### Transforms and transform signalling (1)

JVET-R0345 Unified primary transform kernel for ISP mode [J.-Y. Huo, W.-H. Qiao, H.-X. Wang, Y.-Z. Ma, F.-Z. Yang (Xidian Univ.), S. Wan (NPU), Y.-F. Yu, Y. Liu (OPPO)] [AHG late]

JVET-R0457 Crosscheck of JVET-R0345 (Unified primary transform kernel for ISP mode) [X. Zhao (Tencent)] [late]

### LFNST (15)

JVET-R0056 LFNST complexity reduction [T.-D. Chuang, M.-S. Chiang, Z.-Y. Lin, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-R0426 Crosscheck of JVET-R0056 (LFNST complexity reduction) [T.-C. Ma (Kwai Inc.)] [late]

JVET-R0057 LFNST redundant syntax removal [T.-D. Chuang, M.-S. Chiang, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-R0427 Crosscheck of JVET-R0057 (LFNST redundant syntax removal) [T.-C. Ma (Kwai Inc.)] [late]

JVET-R0167 Issue on LFNST index signaling condition [K. Abe, T. Toma (Panasonic)]

JVET-R0424 Crosscheck of JVET-R0167 (Issue on LFNST index signalling condition) [T. Tsukuba (Sony)] [late]

JVET-R0174 LFNST index signaling [C. Rosewarne, J. Gan (Canon)]

JVET-R0176 On chroma LFNST [C. Rosewarne, J. Gan (Canon)]

JVET-R0400 Crosscheck of JVET-R0176 (On chroma LFNST) [S. De-Luxán-Hernández (HHI)] [late]

JVET-R0234 Removal of redundant LFNST index signalling [H. E. Egilmez, A. Nalci, V. Seregin, W.-J. Chien, M. Karczewicz (Qualcomm)]

JVET-R0360 Crosscheck of JVET-R0234 (Removal of redundant LFNST index signalling) [Z.-Y. Lin (MediaTek)] [late]

JVET-R0235 Removal of LFNST for chroma components [H. E. Egilmez, A. Nalci, V. Seregin, W.-J. Chien, T. Hsieh, M. Karczewicz (Qualcomm)]

JVET-R0361 Crosscheck of JVET-R0235 (Removal of LFNST for chroma components) [Z.-Y. Lin (MediaTek)] [late]

JVET-R0236 Latency reduction in transformation process with TU-level signalling [H. E. Egilmez, A. Nalci, V. Seregin, W.-J. Chien, T. Hsieh, M. Karczewicz (Qualcomm)]

JVET-R0362 Crosscheck of JVET-R0236 (Latency reduction in transformation process with TU-level signalling) [Z.-Y. Lin (MediaTek)] [late]

JVET-R0303 Modified LFNST signalling for single tree blocks [Y. Kidani, K. Unno, K. Kawamura (KDDI)]

JVET-R0416 Crosscheck of JVET-R0303 (Modified LFNST signalling for single tree blocks) [C. Rosewarne, J. Gan (Canon)] [late]

JVET-R0304 Restriction on LFNST signalling for local dual tree chroma coding blocks [Y. Kidani, K. Unno, K. Kawamura (KDDI)]

JVET-R0318 Alternative methods of LFNST index signaling [M. Koo, M. Salehifar, J. Lim, S. Kim (LGE)]

JVET-R0430 Crosscheck of JVET-R0318 (Alternative methods of LFNST index signalling) [C. Rosewarne, J. Gan (Canon)] [late]

JVET-R0319 The interaction between LFNST and BDPCM [M. Koo, M. Salehifar, J. Lim, S. Kim (LGE)]

JVET-R0421 Crosscheck of JVET-R0319 (The interaction between LFNST and BDPCM) [J. Jung (WILUS)] [late]

JVET-R0331 Cleanup of LFNST signalling in single tree [X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai)]

JVET-R0352 On LFNST in shared tree [J. Jung, D. Kim, G. Ko, J.-H. Son, J. S. Kwak (WILUS)] [AHG late]

JVET-R0436 Crosscheck of JVET-R0352 (On LFNST in shared tree) [M. Koo (LGE)] [late]

JVET-R0358 A combined solution for latency reduction in transformation process with TU-level signalling and removal of chroma LFNST [H.E. Egilmez, A. Nalci, V. Seregin, W.-J. Chien, T. Hsieh, M. Karczewicz (Qualcomm), T.-D. Chuang, M.-S. Chiang, Z.-Y. Lin, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek), X. Xiu, T.-C. Ma, Y.-W. Chen, H.-J. Jhu, X. Wang (Kwai), C. Rosewarne, J. Gan (Canon)] [AHG late]

JVET-R0401 Crosscheck of JVET-R0358 (A combined solution for latency reduction in transformation process with TU-level signalling and removal of chroma LFNST) [S. De-Luxán-Hernández (HHI)] [late]

### Partitioning (5)

JVET-R0131 AHG2: On Chroma QT split in 4:2:2 format coding [H. Huang, W.-J Chien, V. Seregin, M. Karczewicz (Qualcomm), T.-D. Chuang, C.-M. Tsai, S.-T. Hsiang, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-R0394 Crosscheck of JVET-R0131 (AHG2: On Chroma QT split in 4:2:2 format coding) [R.-L. Liao (Alibaba)] [late]

JVET-R0268 Implicit binary split at picture boundary [G. Li, X. Li, S. Liu (Tencent)]

JVET-R0269 Fix on minimum QT size value range [G. Li, X. Li, S. Liu (Tencent)]

JVET-R0316 AhG16: Normative constraints on BT and TT split under MER [Y. Wang, K. Zhang, L. Zhang, H. Liu, Z. Deng (Bytedance)]

JVET-R0431 Crosscheck of JVET-R0316 (AhG16: Normative constraints on BT and TT split under MER) [H. Huang (Qualcomm)] [late]

JVET-R0347 AHG2: On minimum QT size, maximum BT size and maximum TT size [H. Huang, J. Chen, W.-J. Chien, M. Karczewicz (Qualcomm)] [AHG late]

### ACT related (6)

JVET-R0305 CU level transform size restriction for adaptive color transform [L.-F. Chen, X. Li, S. Liu (Tencent)]

In order to avoid large temporary buffer, it was decided to set max transform size to 32-point at SPS level when adaptive color transform (ACT) is enabled. However, disallowing 64-point transform leads to 1+% performance drop for RGB camera content. To compensate the coding loss, a simple fix is proposed in this contribution that max transform size is constrained to 32-point for CUs in ACT while for other CUs 64-point transform may still be used. The proposed method is implemented on top of VTM-8.0. Compared with VTM-8.0, the coding loss for RGB camera content is recovered. By using our method, the performance impact for RGB camera content is shown as following.

- AI: -1.5%/2.6%/0.1%

- RA: -1.5%/2.7%/-0.4%

- LB: -1.6%/3.4%/0.8%

Presented in AHG session 2.5 0630-0700 (chaired by JRO)

It is pointed out that a number of restrictions were introduced in the last meeting with regard to combining ACT with other tools.

It is also pointed out that the VTM until recently had various bugs in 444 and 422 modes, in particular regarding the QP setting for chroma. This may make some of the results less interpretable. For example, luma and chroma are showing different tendency here. See contribution R0321.

420 coding would not be affected since ACT is not used there.

The restriction is necessary due to buffering constraints.

Q0378 is also related to this topic.

Revisit – wait for new results with bug-fixed software

JVET-R0329 Mismatch on clipping input residuals to IACT [X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai), J. Zhao, S.-H. Kim (LGE)]

JVET-R0336 Adaptive colour transform clean-ups [J. Xu, L. Zhang, W. Zhu, K. Zhang (Bytedance)]

JVET-R0355 On clipping input residuals to IACT [X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai), J. Zhao, S.-H. Kim (LGE), J. Xu, L. Zhang, W. Zhu, K. Zhang (Bytedance)] [AHG late]

JVET-R0420 Crosscheck of JVET-R0355 (On clipping input residuals to IACT) [T. Tsukuba (Sony)] [late]

JVET-R0378 TU split for ACT [K. Kondo, M. Ikeda, T. Suzuki (Sony)] [AHG late]

JVET-R0380 Scaling list for adaptive colour transform [S. Iwamura, S. Nemoto, A. Ichigaya (NHK), K. Naser, P. de Lagrange, F. Le Leannec, P. Bordes (InterDigital)] [AHG late]

### Other (1)

JVET-R0348 AHG16: On clipping operation for residual samples [T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki (Sony)] [AHG late]

### Test conditions (1)

JVET-R0321 AHG3: Chroma QP table bug-fix and CTC update for RGB coding in VTM-8.0 [J. Xu, L. Zhang, W. Zhu (Bytedance), X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai)]

JVET-R0442 Crosscheck of JVET-R0321 (AHG3: Chroma QP table bug-fix and CTC update for RGB coding in VTM-8.0) [Y.-H. Chao (Qualcomm)] [late]

### Encoder optimization (6)

JVET-R0110 AHG14: Mixed lossy/lossless coding of VTM reference software [M. G. Sarwer, Y. Ye, J. Luo (Alibaba)]

JVET-R0428 Crosscheck of JVET-R0110 (AHG14: Mixed lossy/lossless coding of VTM reference software) [T.-C. Ma (Kwai Inc.)] [late]

JVET-R0140 AHG14: Max BT/TT size restriction for lossless coding encoder configuration [T. Zhou, E. Sasaki, T. Ikai (Sharp)]

JVET-R0143 AHG14: Configuration parameter to enable TSRC for lossless coding [C. Hollmann, M. Damghanian, L. Litwic, M. von Strauss (Ericsson)]

JVET-R0164 AHG10: Mean-scaled SATD for VTM encoder [J. Lainema, A. Hallapuro (Nokia)]

JVET-R0453 Crosscheck of R0164 (AHG10: Mean-scaled SATD for VTM encoder) [J. Enhorn, R. Sjöberg (Ericsson)] [late]

JVET-R0327 CCALF encoder optimization X.W. Meng (PKU), X. Zheng (DJI), S.S. Wang, S.W. Ma (PKU)] [AHG late]

JVET-R0328 ALF encoder optimization [X.W. Meng (PKU), X. Zheng (DJI), S.S. Wang, S.W. Ma (PKU)] [AHG late]

## AHG6: 360° video coding tools, software and test conditions (1)

JVET-R0151 AHG6/AHG12: Uncoded subpictures and potential applications [J. Sauer (RWTH Aachen Univ.)]

`Moved to JVET-R0339

## AHG11: Screen content coding (10)

JVET-R0145 AHG 11/15: On the use of limited EGk signalling [J. Gan, C. Rosewarne (Canon)]

Only four low level syntax elements remain (cu_qp_delta_abs, palette_predictor_run, num_signalled_palette_entries, and palette_escape_val) that use the general EGk binarisation. EGk is the only non-truncated binarisation used for low level signalling. This contribution proposes to replace the use of EGk with limited EGk for the four low level syntax elements. This change in binarisation has previously occurred for residual coding and mvd coding. It is asserted that the proposed changed binarisations remove unbounded codes from low-level syntax.

Presented Thu 9 1317 UTC (chaired by JRO)

There is no problem solved by this proposal, EGk has been used practically for a long time

It is pointed out that the EGk has advantages for syntax elements that change the valid range, such as delta QP

No benefit in terms of simplification

Other experts expressed concerns about adoption

No action.

It was later remarked by the proponents that an asserted benefit would be simplification of conformance testing.

JVET-R0397 Crosscheck of JVET-R0145 (AHG 11/15: On the use of limited EGk signalling) [H.-J. Jhu (Kwai Inc.)] [late]

JVET-R0146 AHG11: Context coded bin limits for palette coding [J. Gan, C. Rosewarne (Canon)]

To limit the required throughput of the CABAC engine, the number of context coded bins is limited to 1.75 bins per sample. This limit is explicitly calculated and enforced in the residual coding process. In palette coding, the number of context coded bins is not explicitly limited. With the adoption of JVET-Q0504, an “evil” bitstream can be constructed that would reach 1.984 context coded bins/sample. This contribution proposes a number of options for solving this bug. The preferred option explicitly calculates the context coded bins limit for palette mode CUs.

Presented Thu 9 1333 UTC (chaired by JRO)

More investigation is necessary to understand if the worst case is really exceeded, as

- The comparison is only made against the max number of transform coefficients per color component. As palette either codes all color comp jointly, this might only exceed the max number for monochrome case (which only would apply if there was a dedicated monochrome profile)

- In residual coding, additional context coded bins occur for motion, mode etc.

Revisit – more detailed analysis needed to understand if the problem exists, when the worst case for all context coded bins in residual coding is considered.

JVET-R0229 AHG11: Fixed number of reuse flags for palette mode [R.-L. Liao, Y. Ye, M. G. Sarwer (Alibaba)]

In the VVC draft 8, number of reuse flags to be decoded is related to the size of the palette predictor. In the case that two neighboring blocks are both coded in palette mode, the syntax of the second block cannot be parsed until the new palette predictor size is obtained. In this contribution, the number of reuse flags is set to a fixed value, that is, 63 and 31 for single tree slice and dual tree slice, respectively. Therefore, the syntax of second block can be parsed with no dependency on how large the previous block’s palette predictor is. Fixed number of reuse flags can be achieved by two methods: 1) initializing the palette predictor size to the fixed value; 2) add a requirement of bitstream conformance to palette_predictor_run signaling. It is reported that, as compared to VTM-8.0 with palette on, the overall coding performance impact for {Y, U, V} in 4:4:4 color format is:

- #1:{0.01%,0.01%,0.01%}for AI,{0.03%,0.05%,0.01%}for RA,{x.xx%,x.xx%,x.xx%}for LB

- #2:{0.01%,0.00%,0.02%}for AI,{0.03%,0.04%,-0.01%}for RA,{x.xx%,x.xx%,x.xx%}for LB

For class F in 4:2:0 color format, the overall coding performance impact for {Y, U, V} is:

- #1:{-0.01%,-0.07%,0.03%}for AI,{0.01%, 0.08%,0.07%}for RA,{x.xx%,x.xx%,x.xx%}for LB

- #2:{-0.02%,-0.04%,0.05%}for AI,{-0.02%,0.10%,0.08%}for RA,{x.xx%,x.xx%,x.xx%}for LB

For class TGM in 4:2:0 color format, the overall coding performance impact for {Y, U, V} is:

- #1:{0.00%,0.01%,0.00%}for AI,{0.05%,-0.06%,-0.04%}for RA,{0.01%,-0.09%,0.11%}for LB

#2:{0.01%,0.00%,-0.01%}for AI,{0.05%,-0.04%,-0.04%}for RA,{0.07%,-0.06%,0.07%}for LB

Presented Thu 9 1350 UTC (chaired by JRO)

It is not obvious that there is a severe issue. It is mentioned that method 1 could even be more complicated.

It is pointed out that JVET-R0310 is same as method 1. However, in that other contribution it is claimed that a parsing dependency exists. Several experts pointed out that there is no parsing dependency as it is typically defined.

Also the bit stream restriction does not seem to be necessary.

No action.

JVET-R0441 Crosscheck of JVET-R0229 (AHG11: Fixed number of reuse flags for palette mode) [Y.-H. Chao (Qualcomm)] [late]

JVET-R0240 AHG11: On maximum palette size and palette predictor size [Y.-H. Chao, T. Hsieh, W.-J. Chien, V. Seregin, M. Karczewicz (Qualcomm)]

In this contribution, it is proposed to reduce maximum palette and palette predictor size. Three methods are proposed in this contribution:

- Reduce the maximum palette and palette predictor size to 16 and 32 in single tree slice and to 8 and 16 in dual tree slice

- Reduce the maximum palette and palette predictor size to 16 and 32 in single tree slice and to 8 and 16 in dual tree slice for YUV420 format, while the size for other color formats stays the same as in VVC draft 8.

- Allow the signaling of maximum palette size and maximum palette predictor size in SPS, same syntax as in HEVC SCC extension. The allowed maximum palette and palette predictor size are restricted to 32 and 64.

The results of method 1 on YUV4:4:4 screen content sequences versus non-420 CTC anchor (PLT=1) are shown as:

1.48% AI, 1.05% RA and x.xx% LDB

The results of method 1 and 2 on YUV4:2:0 class F and TGM versus VTM8 with PLT=1 are shown as:

Class F: 0.16% AI, 0.30% RA and 0.09% LDB

Class TGM: 0.71% AI, 0.59% RA, and 0.19% LDB

The results of method 3 with maximum palette and palette predictor size signaled to be 16 and 32 for YUV4:2:0 format versus VTM8 with PLT=1 are shown as:

Class F: 0.16% AI, 0.30% RA and 0.09% LDB

Class TGM: 0.72% AI, 0.59% RA, and 0.19% LDB

Presented Thu 9 1407 UTC (chaired by JRO)

Presentation deck not uploaded.

Method 3 does not reduce decoder worst case complexity (and was also proposed previously, but not adopted). Also method 2 is not reducing worst case complexity, as 4:2:0 is not the worst case. Method 1 is reducing the complexity in terms of CABAC throughput, and the memory storage, but also has loss of up to 1.5% (AI)

Memory is not a significant issue here, according other experts’ opinion

It is not obvious that CABAC throughput of palette is the worst case.

No evidence that further reduction of the palette size is needed.

JVET-R0381 Crosscheck of JVET-R0240 (AHG11: On maximum palette size and palette predictor size) [H.-J. Jhu (Kwai Inc.)] [late]

JVET-R0309 [AHG16] Clean-up on palette predictor update for local dual tree [H. Jang, J. Nam, S. Yoo, N. Park, S. Kim, J. Lim (LGE)]

This contribution proposes to clean-up regarding predictor palette update for local dual tree structure. Three alternative clean-up methods are suggested as described below:

- Clean-up 1. There is mismatch between SPEC and SW. Here, propose to fix SPEC to align with SW regarding local dual tree predictor palette update process.

- Clean-up 2. Update predictor palette entry only for luma component but not for chroma component in local dual tree structure.

- Clean-up 3. Remove palette predictor update process of local dual tree structure.

The suggested clean-up method 2 shows {0.00%, 0.00%, -0.06%} and method 3 shows{0.00%, 0.07%, 0.10%} coding performance respectively for average of ClassF/SCC under CTC with palette on. It is asserted that proposed clean-up methods save H/W power consumption with closed to zero coding loss and also cut worst case decoding latency by disabling predictor palette update for local dual tree.

Presented Thu 9 1427 UTC (chaired by JRO)

For “clean-up 1”, spec should be aligned with software. This is also included in JVET-R0333. See further notes there.

Items #2 and #3 try to simplify the interaction of palette with local dual tree structures. It is agreed that there is no problem in the spec such as unspecified decoder behaviour. Also in local dual tree the total predictor table size is identical with single tree. No need for action on these items.

JVET-R0310 [AHG16] Clean-up by removing parsing dependency for palette [H. Jang, J. Nam, S. Yoo, N. Park, S. Kim, J. Lim (LGE)]

See notes under JVET-R0229.

JVET-R0320 AHG11: Maximum QP for escape value in palette coding [J. Xu, L. Zhang, W. Zhu, K. Zhang (Bytedance)]

Currently, EG(5) is used to code escape values in palette mode. It is observed that when QP is larger than or equal to 23, the bit length of escape values reaches its minimal, i.e. 6. Thus, it is proposed to limit the maximum QP to be 23 to reduce quantization distortion.

Presented in Session 2.5 Tue 14 April 0500 (chaired by JRO)

This does not appear to be a cleanup (another min check necessary). Small compression gain is observed.

Even small loss in LDB configuration (mainly coming from one sequence).

The relative small gain may be due to the fact that palette is less used in higher QP, and that also escape may be less selected.

No action.

JVET-R0396 Crosscheck JVET-R0320 (AHG11: Maximum QP for escape value in palette coding) [R.-L. Liao (Alibaba)] [late]

JVET-R0333 AHG11: Mismatches related to palette prediction [H.-J. Jhu, X. Xiu, Y.-W. Chen, T.-C. Ma, X. Wang (Kwai Inc.)] [AHG late]

This contribution reports some mismatches between VVC draft 8 and test model software VTM-8.0 on updating the palette prediction in local dual tree cases. First, in the VVC draft 8, under local-dual tree case the update process of palette prediction is performed only for chroma CU while in the VTM-8.0 it is performed for both luma and chroma CUs. Secondly, in the VVC draft 8, the palette prediction for a luma CU under a local-dual tree does not use the chroma values of those entries in the palette predictor, and vice versa. But in the VTM-8.0, luma and chroma values of each entry in the palette predictor are used together in palette prediction.

In this contribution, one method is proposed to solve the first mismatch and two methods are proposed to solve the second mismatch in different ways. In the first change, it is proposed to change the specification text to align with the VTM-8.0 software for palette update in the first mismatch. The proposed change has no impact on coding performance. In the second change, it is proposed to change the specification text to align with the VTM-8.0 software for forming a palette in the second mismatch. The proposed change has no impact on coding performance. In the third change, in forming a palette, the same default values used for signaled palette entries are also used for predicted palette entries to fix the second mismatch. The proposed change reports negligible BD-rate changes compared to VTM8.0.

Presented Thu 9 1450 UTC (chaired by JRO)

Presentation deck not included.

It is obvious that the text spec is incomplete. The third change which modifies both text and software does not seem to be justified, as it is not simpler and has some small coding loss.

Recommendation (text BF): first and second change (as called in v2 of word or v3 zip) should be adopted.

Session 2.1 ended Thu April 9 1505 UTC

JVET-R0438 Crosscheck of JVET-R0333 (AHG11: Mismatches related to palette prediction) [Y.-H. Chao (Qualcomm)] [late]

JVET-R0334 AHG11: Simplification of palette mode for local dual tree cases [H.-J. Jhu, X. Xiu, Y.-W. Chen, T.-C. Ma, X. Wang (Kwai Inc.)] [AHG late]

JVET-R0439 Crosscheck of JVET-R0334 (AHG11: Disabling chroma CU palette mode under local dual tree) [Y.-H. Chao (Qualcomm)] [late]

JVET-R0412 AHG11: On maximum palette size and palette predictor size under local dual tree [Y.-H. Chao, W.-J. Chien, M. Karczewicz (Qualcomm)] [late]

## AHG14: Lossless and near-lossless coding (23)

Reviewed in session 2.4 Thursday 9 April 2320 - Friday 10 April 0115 (chaired by BB), continued Tuesday 14 April in session 2.6 0850-0920 and session 2.7 1300-1515 (chaired by JRO and BB).

### Transform skip-related (22)

JVET-R0045 AHG15: cleanup for signaling of minimum QP of transform skip [J. Li, K. Abe (Panasonic)]

This contribution proposes to replace syntax “min_qp_prime_ts_minus4” (in range of [0-48]) with “internal_minus_input_bit_depth” (in range of[0-8]) to remove redundancy of signalling and make the design more compact.

It was commented that the name of the new syntax element could be changed and the semantics could be improved.

There was a question whether an encoder would choose one of the values of min_qp_prime_ts_minus4 not suited for lossless coding. Participants were generally in favor of this cleanup.

Recommendation (cleanup): adopt (with editorial improvements of the specification draft text).

JVET-R0049 AHG9: HLS on disabling TSRC [S.-T. Hsiang, C.-W. Hsu, Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

In VVC Draft 8, extra bit costs are consumed for signalling the new syntax element slice_ts_residual_coding_disabled_flag for each of the coded slices. This contribution proposes three methods for high-level syntax modifications related to disabling transform skip residual coding, as follows:

- In Method 1, a new syntax element sps_ts_residual_coding_disabled_slice_present_flag is added to the sequence parameter set (SPS) to specify whether slice_ts_residual_coding_disabled_flag is present in the slice header.

- In Method 2, two new syntax elements sps_ts_residual_coding_disabled_slice_present_flag and sps_ts_residual_coding_disabled_slice_default_flag are added to the SPS. When slice_ts_residual_coding_disabled_flag is not present, it is inferred to be equal to the value of sps_ts_residual_coding_disabled_slice_default_flag.

- In Method 3, signalling slice_ts_residual_coding_disabled_flag in the slice header is conditioned on sps_transform_skip_enabled_flag equal to 1 without adding any new syntax element.

Method 1 same is in JVET-R0097.

Method 3 same as in JVET-R0097, JVET-R0068 (item 8) plus inference to 1 for slice_ts_residual_coding_disabled_flag, JVET-R0142, JVET-R0317 without the PPS flag, JVET-R0153 aspect 1, JVET-R0182 with inverse semantics.

The motivation behind method 1 is to not signal slice_ts_residual_coding_disabled_flag for lossy coding cases, which are considered to be the main application.

It was commented that at the last meeting, slice_ts_residual_coding_disabled_flag was adopted as a slice level flag instead of PPS or SPS flag. This would not prevent syntax as proposed here to gate its presence.

A participant questions whether the additional control syntax to save the signalling of slice_ts_residual_coding_disabled_flag is really needed. Functionality is not affected by any of the proposed methods.

It was agreed that it is desirable to not always send slice_ts_residual_coding_disabled_flag since it is expected to only be used for lossless coding scenarios.

It was further agreed that it makes sense to condition slice_ts_residual_coding_disabled_flag on sps_transform_skip_enabled_flag (Method 3).

Method 3:

SPS: sps_transform_skip_enabled_flag

SH: if(sps_transform_skip_enabled_flag )

slice_ts_residual_coding_disabled_flag

Method 1:

SPS: sps_transform_skip_enabled_flag

SPS: if(sps_transform_skip_enabled_flag )

sps_ts_residual_coding_disabled_slice_present_flag

SH: if(sps_ts_residual_coding_disabled_slice_present_flag)

slice_ts_residual_coding_disabled_flag

Recommendation (cleanup): method 3 (revisit to consider method 1).

JVET-R0068 AHG8/AHG9/AHG12: Miscellaneous HLS topics [Y.-K. Wang, L. Zhang, Z. Deng, J. Xu, K. Zhang, K. Fan (Bytedance)]

Item 8 of this contribution belongs to this category.

See notes under JVET-R0049.

JVET-R0083 AHG14: Residual coding constraints for transform skip blocks [A. Nalci, H.E. Egilmez, M. Coban, V. Seregin, M. Karczewicz (Qualcomm)]

In 17th JVET meeting, the slice level flag “slice_ts_residual_coding_disabled_flag” was adopted into VVC as part of JVET-Q0089. This flag can bypass transform skip residual coding (TSRC) and enables the use of regular residual coding (RRC) for TS blocks. In VVC Draft 8, when slice_ts_residual_coding_disabled_flag=1 both dependent quantization (DQ) and sign data hiding (SDH) can be used for TS blocks. Additionally, as reported in [Spec Ticket #859], though VTM-8.0 software disables the dequantization part of DQ for TS blocks it keeps the DQ related state-based context derivation.

In variant (#1a), it is proposed to disable both DQ and SDH for TS blocks. In addition to this variant, (variant #1b) further encoder fixes for RDOQ are provided for both BDPCM and non-BDPCM TS blocks when slice_ts_residual_coding_disabled_flag=1 as discussed in [VVC Ticket #981].

- For lossless coding on YUV420 sequences, the simulation results show overall bit-rate savings of -0.16% AI, -0.17% RA, and -0.28% LDB with Class F: -0.47% AI, -0.56% RA, and -0.65% LDB and Class TGM: -0.47% AI, -0.57% RA, and -0.60% LDB.

- For lossless coding on YUV444 and RGB sequences, the simulation results show overall bit-rate savings about -0.32% AI, -0.43% RA, and -0.46% LDB.

- For lossy coding without encoder fixes, the simulation results show overall BD-rate savings (luma) of -0.03% AI, -0.03% RA, and -0.03% LDB with Class F: -0.80% AI, -0.70% RA, and -1.02% LDB and Class TGM: -0.86% AI, -0.86% RA, and -1.32% LDB.

- For lossy coding after encoder fixes, the simulation results show overall BD-rate savings (luma) of -0.05% AI, -0.05% RA, and -0.10% LDB with Class F: -0.99% AI, -0.83% RA, and -1.21% LDB and Class TGM: -1.23% AI, -1.08% RA, and -1.80% LDB.

In variant #2, only DQ is disabled for TS blocks and in variant #3 only SDH is disabled for TS blocks. The results for variant #2 is the same as variant #1, no results were provided for variant #3 since SDH is disabled under CTC and encoder crash occurs when SDH is enabled in current VVC software as reported in Ticket #981.

In variant #4, it is proposed to align the current spec text to the VVC software, in which dequantization part of DQ is disabled for TS blocks as in current VVC software however DQ related state transitions and contexts are kept.

Discussed in session 2.6 Tuesday April 14 0850-0920 (chaired by JRO and BB)

Question: Is anything wrong with the current spec? The ticket #981 refers to software. There is a spec related ticket #859.

To achieve lossless coding, an encoder has to disable both DQ and SDH (and other things) at high level. This also applies for mixed lossy/lossless coding, which might then be performing worse than in case of local disabling these tools. Currently, disabling is possible at picture level.

Currently, the SW modifies DQ reconstruction for TS with RRC blocks locally, whereas the spec does not have such an element. Otherwise, it would not be possible to get lossless reconstruction. It is noted that when introducing the switch between RRC and TSRC, the modification of DQ reconstruction was never mentioned. The context derivation of DQ is retained.

In the results for test 1 above, the anchor is not the CTC config for lossless, but a version that disables the context derivation of DQ as well as the reconstruction part. Results indicate that the SW mismatch (using context derivation of DQ) is not providing benefit.

There is no problem with TSRC, it is able to achieve lossless coding and there is no mismatch between spec and software.

An encoder could, with current spec, take the following options for mixed lossy/lossless

- Disable DQ, and enable RRC with TS, which would penalize the lossy coded parts

- Enable DQ, and disable RRC with TS, which would penalize the lossless coded parts (mainly for natural content, as per previous findings)

For lossless-only coding, there is no problem at all. For natural content, an encoder would just take the first choice if it is natural content.

No results are available for mixed lossy/lossless (as we don’t have CTC for this). Also, the current VTM encoder would need to modified, and realistic conditions (in terms of applications) are missing.

Aligning the text with software would introduce a block-level change that is not in the spec currently.

Another option would be a high-level restriction disallowing usage of DQ if TS/RRC is enabled (or the other way round). Also BDPCM should be considered in this context, as it can be enabled when TS is enabled.

It is mentioned that such a high-level restriction would be the cleanest approach with least danger of introducing even additional problems. See also further notes under R0116.

JVET-R0084 AHG14: On signaling for lossless coding [M. Karczewicz, M. Coban, A. Nalci, H.E. Egilmez, V. Seregin (Qualcomm), T.-C. Ma, X. Xiu, Y.-W. Chen, H.-J. Jhu, X. Wang (Kwai Inc.)]

No need to present according to proponents

JVET-R0417 Crosscheck of JVET-R0084 (AHG14: On signalling for lossless coding) [T. Tsukuba (Sony)] [late]

JVET-R0097 AHG9: Transform and transform-skip related HLS clean-up [M. G. Sarwer, Y. Ye, J. Luo, J. Chen (Alibaba)]

See notes under JVET-R0049 for aspect 2.

JVET-R0116 AHG11/AHG14: On sign data hiding of transform skip block [M. G. Sarwer, Y. Ye, J. Luo (Alibaba)]

It is asserted that, in VTM-8.0, if slice_ts_residual_coding_disabled_flag == 1, the encoder RDOQ process does not select proper residual coding method for transform skip (TS) and BDPCM blocks. In this contribution, the encoder RDOQ process is modified so that it can select correct residual coding method during RDOQ process.

It is further reported that, in VTM-8.0 software, an encoder-decoder mismatch is also observed if both slice_ts_residual_coding_disabled_flag == 1 and pic_sign_data_hiding_enabled_flag == 1. This contribution also proposes two methods to resolve the encoder-decoder mismatch issue. In the first method, sign data hiding (SDH) is disabled for BDPCM blocks only, whereas in the second method, SDH is disabled for both TS and BDPCM blocks. The encoder RDOQ bug fix (mentioned above) are included in both of the methods.

During the discussion, it is questioned whether the encoder/decoder mismatch is not just an issue of VTM implementation. The spec is asserted to clearly define how SDH works from a decoder perspective, regardless if it is regular transform, TS or TS/BDPCM. The encoder may have a problem of determining the hidden sign value, though. A normative change may not be needed to resolve the encoder/decoder mismatch.

The normative changes that are suggested: disable SDH at block level when BDPCM is on (method 2.1), or when TS is on (regardless if with or without BDPCM (method 2.2)

The problem only occurs with TS/RRC

The proposal also includes a cleanup, removing the condition on DQ/SDH at block level, as both are mutually exclusive at high level.

Relative to this cleanup, another condition would be introduced at block level.

R0141 also proposes method 2.2, which is also equivalent to R0083 variant 3. Document R0154 also proposes both methods 2.1 and 2.2, and also a high level solution additionally. R0144 method 2 is identical to 2.2. R0325 aspect 2 also proposes method 2.2.

It is the general opinion that the combination of TS/BDPCM with SDH/DQ and RRC is not beneficial and difficult to handle by an encoder. A high level disabling of this combination would also resolve the problem.

A solution resolving the problems with SDH/DQ should be consistent.

Options:

- Fix the software bugs, and keep the text unchanged

- High level disabling of the combination TS/RRC with either DQ or SDH (which are mutually exclusive)

- Block-level inhibiting that said combinations.

Several experts expressed the opinion that the second option (high level disabling the combination) would be the safest solution for the problems raised, with least danger of introducing additional problems, making the software bug fixes most simple (the fix of DQ context derivation would still be necessary).

Revisit

JVET-R0418 Crosscheck of JVET-R0116 (AHG11/AHG14: On sign data hiding of transform skip block) [T. Tsukuba (Sony)] [late]

JVET-R0139 AHG9: High-level control flag for lossless coding [T. Zhou, E. Sasaki, T. Ikai (Sharp)]

JVET-R0141 Disabling Dependent Quantization and Sign Data Hiding in Transform Skip blocks [T. Hashimoto, E. Sasaki, T. Aono, T. Ikai (Sharp)]

See notes under JVET-R0116.

JVET-R0398 Crosscheck of JVET-R0141 (Disabling Dependent Quantization and Sign Data Hiding in Transform Skip blocks) [K. Abe (Panasonic)] [late]

JVET-R0142 AHG9: Slice header signalling clean up [T. Hashimoto, T. Aono, T. Ikai (Sharp)]

See notes under JVET-R0049.

JVET-R0144 AHG14: On lossless operation with RRC [J. Gan, C. Rosewarne (Canon)]

See notes under JVET-R0116.

JVET-R0450 Crosscheck of JVET-R0144 (AHG14: On lossless operation with RRC) [A. Nalci (Qualcomm)] [late]

JVET-R0153 AHG9/AHG16: On slice_ts_residual_coding_disabled_flag [J. Choi, S. Yoo, J. Heo, J. Choi, J. Lim, S. Kim (LGE)]

See notes under JVET-R0049 for aspect 1.

JVET-R0154 AHG9/16: On sign data hiding for BDPCM blocks [S. Yoo, J. Choi, J. Lim, S. Kim (LGE)]

See notes under JVET-R0116.

JVET-R0155 AHG9/14: On lossless coding granularity [S. Yoo, J. Choi, J. Lim, S. Kim (LGE)]

JVET-R0182 AHG9: Removed Redundant Slice Level TSRC Flag [K. Naser, F. Le Leannec, T. Poirier, M. Kerdranvat (InterDigital)]

See notes under JVET-R0049.

JVET-R0219 Alternative block size conditions for BDPCM [K. Unno, K. Kawamura, S. Naito (KDDI)]

JVET-R0419 Crosscheck of JVET-R0219 (Alternative block size conditions for BDPCM) [T. Tsukuba (Sony)] [late]

JVET-R0258 AHG9: Reduce redundant signalling in picture header [J. Enhorn, M. Pettersson, R. Sjöberg, M. Damghanian, Z. Zhang (Ericsson)]

The ph_dep_quant_enabled_flag aspect of item 1 of this contribution belongs to this category.

JVET-R0271 AHG9: High-level constraints of dependent quantization and sign data hiding [A. Nalci, M. Coban, M. Karczewicz (Qualcomm)]

JVET-R0317 AHG9: On slice transform skip residual coding method signalling [M. Coban, V. Seregin, Y. He, A. Nalci, M. Karczewicz (Qualcomm)]

See notes under JVET-R0049 except the PPS aspect.

JVET-R0325 AHG14: Disabling dependent quantization and sign bit hiding for transform skip mode [T.-C. Ma, X. Xiu, Y.-W. Chen, H.-J. Jhu, X. Wang (Kwai Inc.)]

See notes under JVET-R0116.

JVET-R0353 AHG14: On Interaction between ACT and BDPCM [T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki (Sony)] [AHG late]

JVET-R0354 AHG14: BDPCM for Inter/IBC-predicted residuals [T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki (Sony)] [AHG late]

JVET-R0451 Crosscheck of JVET-R0354 (AHG14: BDPCM for Inter/IBC-predicted residuals) [A. Nalci (Qualcomm)] [late]

### Other (1)

JVET-R0169 AHG14: Report of CABAC skip mode results on VTM-8.0 [K. Abe, T. Toma, V. Drugeon (Panasonic)]

## AHG15: Quantization control (4)

Discussed in session 2.5 Tue 14 April 0520-0630

JVET-R0055 AHG15: On referencing a non-existent scaling list [C.-Y. Lai, O. Chubach, C.-Y. Chen, T.-D. Chuang, Y.-W. Huang, S.-M. Lei (MediaTek)]

In this contribution, three methods to fix the issue of referencing a non-existent scaling list are presented. It is proposed to infer a non-existent scaling list to be the pre-defined matrix with all elements equal to 16 according to scaling_list_copy_mode_flag and scaling_list_pred_id_delta.

Problem occurs in 400 where chroma lists are not signalled

Method 1: define the non-existing scaling list as default

Method 2: always signal a scaling list for chroma (i.e. revert the decision of last meeting)

Method 3: bitstream constraint

Refers to ticket #926. In the discussion following that ticket. Methods 1 and 3 had been discussed there.

JVET-R0127 AHG15: On scaling list prediction [A. K. Ramasubramonian, B. Ray, G. Van der Auwera, M. Karczewicz (Qualcomm)]

This document proposes changes to the derivation of scaling list reference ID when the chroma scaling lists are not signalled. Scaling lists may be explicitly signalled, copied or predicted from other scaling lists – the scaling list used for copying or predicting is referred to as the reference scaling list. The current specification allows luma scaling lists to be copied or predicted from lists that correspond to chroma scaling lists even when the chroma lists are absent; it is asserted that this behaviour is undesirable. The document proposes to modify the reference scaling list ID derivation by skipping the absent chroma lists. It is asserted that the proposed method fixes the issue of referring absent lists; due to the small bit savings, the proposed method is argued to be preferable compared to explicit constraints or inferring default lists.

Also related to ticket #926. Another method is proposed that disallows prediction from chroma scaling list when it is not present. This would also require a syntax change, reducing the range of code words to only allowing reference to luma values.

JVET-R0166 AHG15: Issue on chroma scaling matrix for 4:4:4 [K. Abe, T. Toma (Panasonic)]

This contribution points out the issue that VTM-8.0 does not work with enabling scaling matrix for 4:4:4 format and proposes two solutions. Solution1 proposes to fix VTM to strictly align to current VVC specification text, it needs to introduce size64 chroma scaling matrices reusing size32 matrices. Solution2 proposes alternative method of current VVC specification text by introducing size64 chroma scaling matrix using individual matrix id. It can simplify the specification and improve the tuning capability.

The proposed solution 2 would also require signalling the size 64 chroma matrices for the 420 case.

It is not fully clear that the separate scaling matrices for chroma block size 64 are really needed.

No support for solution 2.

Recommendation (SW BF): Align the VTM with text regarding the upscaling of chroma matrices for 444 64 size (adopt method1 of R0166).

JVET-R0326 AHG15: On Chroma Quantization Matrix Signaling [H. Zhang, X. Li, G. Li, L. Li, S. Liu (Tencent)]

This contribution proposes two fixes for the issue reported in ticket #926 that prediction may be from chroma quantization matrix which is not signaled.

Method #1 Values in non-signaled chroma QM is set to 16.

Method #2 Value of scaling_list_copy_mode_flag[id] is inferred to be equal to 1 when it is not present

Also related to ticket #926.

Both methods are using default value (similar to method 1 of R0055), but defining it differently. Method 2 of R0326 is identical to method 1 of R0055

Method 1 of R0326 was already proposed in the original Q0505, but not included in the last meeting’s decision. It is pointed out by the editor that the text description is not optimum.

It is agreed that method 3 of R0055 (encoder constraint) is not desirable.

It is agreed that method 2 of R0055 (reverting the decision of last meeting) is not desirable.

Compression efficiency is not an argument for scaling matrices.

Both method 1 of R0055 (same as method 2 of R0326), and the method of R0127 would solve the problem.

R0055 method 1 would have less impact on implementation logic changes, and had already been discussed in the reflector.

R0127 inhibits unnecessary codewords which seems to be more clean from the spec perspective.

Recommendation (BF): R0055M1/R0326M2 should be adopted.

## AHG16: Implementation studies (4)

JVET-R0224 AHG16: Realization of RPR based real-time VVC decode and playback on ARM based mobile devices [J. Shingala, A. Natesan, A. Chelawat (Ittiam)] [AHG late]

JVET-R0351 High bit depth coding [A. Browne, S. Keating, K. Sharman (Sony)] [AHG late]

JVET-R0364 Information on cinematic aspect ratios in the context of JVET-Q0065 [Sean McCarthy, Walt Husak, Peng Yin, Taoran Lu, Fangjun Pu, Tao Chen] [AHG late]

JVET-R0390 [AHG16] VVC multi-thread decoder and performance analysis [S. Gudumasu, T. Poirier, F. Urban, F. Hiron, P. de Lagrange (InterDigital)] [AHG late]

# Annex A:
List of meeting participants

The participants of the Cat 2 AHG meetings preceeding the eighteenth meeting of the JVET, according to Zoom reports of the meeting sessions (approximately 227 people in total), were as follows:

- Mohsen Abdoli (Ateme)

- Kiyofumi Abe (Panasonic)

- Elena Alshina (Huawei)

- Alireza Aminlou (Nokia)

- Kenneth Andersson (Ericsson)

- Ichiro Ando (Nikon)

- Jeeva Raj Arumugam (Ittiam)

- Gun Bang (ETRI)

- Philippe Bordes (InterDigital)

- Frank Bossen (Sharp)

- Jill Boyce (Intel)

- Benjamin Bross (HHI)

- Adrian Browne (Sony)

- Hung CaoVan (Renesas)

- Eric Chai (Ubilinx)

- Yao-Jen Chang (Qualcomm)

- Yung-Hsuan Chao (Qualcomm)

- Ching-Yeh Chen (MediaTek)

- Chun-Chi Chen (Qualcomm)

- Huanbang Chen (Huawei)

- Jianle Chen (Qualcomm)

- Jie Chen (Alibaba)

- Lien-Fei Chen (Tencent)

- Lulin Chen (MediaTek)

- Peisong Chen (Broadcom)

- Ya Chen (InterDigital)

- Yi-Wen Chen (Kwai)

- Wei-Jung Chien (Qualcomm)

- Yi-Jen Chiu (Intel)

- Byeongdoo Choi (Tencent)

- Jangwon Choi (LGE)

- Jungah Choi (LGE)

- Tzu-Der Chuang (MediaTek)

- Olena Chubach (MediaTek Inc.)

- Takeshi Chujoh (Sharp)

- Muhammed Coban (Qualcomm)

- Mitra Damghanian (Ericsson)

- Philippe de Lagrange (Interdigital)

- Santiago de Luxán (HHI)

- Zhipin Deng (Bytedance)

- Sachin Deshpande (Sharp)

- Virginie Drugeon (Panasonic)

- Fanyi Duanmu (Apple)

- Hilmi Egilmez (Qualcomm)

- Semih Esenlik (Huawei)

- Alexey Filippov (Huawei)

- Chad Fogg (MovieLabs)

- Remy Foray (Allegro DVT)

- Edouard Francois (InterDigital)

- Per Fröjdh (Ericsson)

- Masanori Fukada (Canon)

- Jonathan Gan (Canon)

- Han Gao (Huawei)

- Dan Grois (Comcast)

- Srinivas Gudumasu (InterDigital)

- Ryoji Hashimoto (Renesas)

- Yong HE (Qualcomm)

- Christian Helmrich (HHI)

- Hendry (LGE)

- Jin Heo (LGE)

- Christopher Hollmann (Ericsson)

- Seungwook Hong (Nokia)

- Maiki Hosokawa (Renesas)

- Shih-Ta Hsiang (MediaTek)

- Ted Hsieh (Qualcomm)

- Chih-Wei Hsu (MediaTek)

- Nan Hu (Qualcomm)

- Han Huang (Qualcomm)

- Yu-Wen Huang (MediaTek)

- Junyan Huo (Xidian Univ.)

- Walt Husak (Dolby)

- Atsuro Ichigaya (NHK)

- Tomohiro Ikai (Sharp)

- Masaru Ikeda (Sony)

- Sergey Ikonin (Huawei)

- Shunsuke Iwamura (NHK)

- Hyeongmun Jang (LGE)

- Byeungwoo Jeon (SKKU)

- Hong-Jheng Jhu (Kwai)

- Jaehong Jung (WILUS)

- Marta Karczewicz (Qualcomm)

- Kei Kawamura (KDDI)

- Steve Keating (Sony)

- Louie Kerofsky (Qualcomm)

- DaeYeon Kim (Chips&Media)

- Dong-Cheol Kim (WILUS)

- Hyun-Gyu Kim (Chips&Media)

- Jaeil Kim (SK Telecom)

- Seung-Hwan Kim (LGE)

- Kenji Kondo (Sony)

- Moonmo Koo (LGE)

- Anand Meher Kotra (Huawei)

- Matthias Kränzler (FAU Erlangen)

- Gosala Kulupana (BBC)

- Ankit Kumar (Chosun Uni.)

- Chen-Yen Lai (MediaTek)

- Jani Lainema (Nokia)

- Guillaume Laroche (Canon)

- Fabrice Le Léannec (InterDigital)

- Bae-Keun Lee (Xris)

- Brian Lee (Dolby)

- Jongseok Lee (Digitalinsights)

- Ju Ock Lee (Chips&Media)

- Ya-Hsuan Lee (MediaTek)

- Shawmin Lei (MediaTek)

- Guichun Li (Tencent)

- Jingya Li (Panasonic)

- Ling Li (Tencent)

- Ming Li (OPPO)

- Tsung-Hua Li (Foxconn)

- Xiang Li (Tencent)

- Ru-Ling Liao (Alibaba)

- Jaehyun Lim (LGE)

- Woong Lim (ETRI)

- Ching-Chieh Lin (ITRI)

- Du Liu (Ericsson)

- Shan Liu (Tencent)

- Jiancong Luo (Alibaba)

- Ajay Luthra (Picsel Labs)

- Tsung-chuan Ma (Kwai)

- Yanzhuo Ma (Xidian Univ.)

- Sean McCarthy (Dolby)

- Dominik Mehlem (RWTH)

- Xuewei Meng (Peking University)

- Koohyar Minoo (IRNB-Expert)

- Kiran Misra (Sharp Labs of America)

- Iole Moccagatta (Intel)

- Kaito Mori (Renesas)

- Alican Nalci (Qualcomm)

- Junghak Nam (LGE)

- karam naser (InterDigital)

- Shimpei Nemoto (NHK)

- Tung Nguyen (HHI)

- Yu-Chieh Nien (Foxconn)

- Andrey Norkin (Netflix)

- Jens Ohm (RWTH)

- Patrice Onno (Canon)

- Nael Ouedraogo (Canon)

- Seethal Paluri (LGE)

- Krit Panusopone (Nokia)

- Min Woo Park (Samsung)

- Minsoo Park (Samsung)

- Naeri Park (LGE)

- Jonathan Pfaff (HHI)

- Yinji Piao (Samsung)

- Tangi Poirier (InterDigital)

- Adarsh K. Ramasubramonian (Qualcomm)

- Krishna Rapaka (Apple)

- Bappaditya Ray (Qualcomm)

- Kevin Reuzé (Qualcomm)

- Justin Ridge (Nokia)

- Antoine Robert (Interdigital)

- Chernyak Roman (Huawei)

- Chris Rosewarne (Canon)

- Vasily Rufitskiy (Huawei)

- Dmytro Rusanovskyy (Qualcomm)

- Mehdi Salehifar (LGE)

- Mohammed Sarwer (Alibaba)

- Eiichi Sasaki (Sharp)

- Johannes Sauer (RWTH)

- Heiko Schwarz (HHI)

- Andrew Segall (Sharp)

- Vadim Seregin (Qualcomm)

- Masato Shima (Canon)

- Jay Shingala (Ittiam)

- Robert Skupin (HHI)

- Kashyap Kammachi Sreedhar (Nokia)

- Jacob Ström (Ericsson)

- Karsten Suehring (HHI)

- Gary Sullivan (Microsoft)

- Teruhiko Suzuki (Sony)

- Yasser Syed (Comcast)

- Chih-Yu Teng (Foxconn)

- Han Boon Teo (Panasonic)

- Dong Tianyu (Hanyang Univ.)

- Tadamasa Toma (Panasonic)

- Alexandros Tourapis (Apple)

- Chia-Ming Tsai (MediaTek)

- Yiting Tsai (ITRI)

- Takeshi Tsukuba (Sony)

- Kyohei Unno (KDDI)

- Luong Pham Van (Qualcomm)

- Geert Van der Auwera (Qualcomm)

- Mikael von Strauss (Ericsson)

- Shuai Wan (NPU Univ.)

- Wade Wan (Broadcom)

- Biao Wang (Huawei)

- Dong Wang (OPPO)

- Fan Wang (OPPO)

- Hongtao Wang (Qualcomm)

- Limin Wang (Nokia)

- Mingze Wang (NPU Univ.)

- Sheng-Po Wang (ITRI)

- Simon Wang (Intel)

- Xianglin Wang (Kwai)

- Yang Wang (Bytedance)

- Ye-Kui Wang (Bytedance)

- Zhecheng Wang (NPU Univ.)

- Stephan Wenger (Tencent)

- Samuel Wong (Intel)

- Ping Wu (ZTE)

- Zhihuang Xie (OPPO)

- Xiaoyu Xiu (Kwai)

- Jizheng Xu (Bytedance)

- Lidong Xu (Intel)

- Xiaozhong Xu (Tencent)

- Yoichi Yagasaki (Sony)

- Haitao Yang (Huawei)

- Yu-Chiao Yang (Foxconn)

- Yan Ye (Alibaba)

- Peng Yin (Dolby)

- Sunmi Yoo (LGE)

- Haoping Yu

- Ruoyang Yu (Ericsson)

- Hui Yuan (Shandong Univ.)

- Weimin Zeng (Ubilinx)

- Hongbin Zhang (Tencent)

- Kai Zhang (Bytedance)

- Li Zhang (Bytedance)

- Wenhao Zhang (Hulu)

- Zhi Zhang (Ericsson)

- Wei Zhang(Xidian Univ.)

- Jane Zhao

- Xin Zhao (Tencent)

- Xiaozhen Zheng (DJI)

- Minhua Zhou (Broadcom)

- Tianyang Zhou (Sharp)