| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0347 |
| --- | --- |

| Title: | AHG2: On minimum QT size, maximum BT size and maximum TT size |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Huang, Jianle Chen, Wei-Jung Chien, Marta Karczewicz | Email: | {hanhuang, cjianle, wchien, martak}@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In this contribution, it’s reported that the value 128 for the minimum QT size, maximum BT size and maximum TT size are redundant under dual-tree condition. It’s also reported that the value 128 for maximum TT size is redundant for single tree condition as well. To address the first issue, it’s proposed to change the upper limit of minimum QT size, maximum BT size and maximum TT size under dual-tree condition to 64. To address the second issue as well as the first, it’s proposed to set the upper limit of minimum QT size and maximum TT size to 64 in both dual-tree and single tree, and set the upper limit of maximum BT size to 64 in dual-tree.

# Problem Statement

In the current VVC, when dual-tree block partitioning structure is applied, implicit QT split is inferred for the coding unit that has size larger than 64. Therefore, setting the minimum QT size equals to 128 is redundant. It also leads to redundant value of 128 for maximum BT size and maximum TT size. This issue is previously reported in VVC ticket #925.

Similar issue is also found for the maximum TT size in both dual-tree and single tree block partitioning structure. In the current VVC, TT split is disallowed for a coding unit that has size larger than 64 (due to the VPDU constrain). However, the maximum TT size can be set equals to 128.

# Solutions

## Solution for the ticket #925

To avoid the invalid value for minimum QT size, maximum BT size and maximum TT size under dual-tree condition, it’s proposed to change the upper limit of minimum QT size, maximum BT size and maximum TT size under dual-tree condition to 64. The relative semantics changes in the text are described in the following.

sps_log2_diff_min_qt_min_cb_intra_slice_luma specifies the default difference between the base 2 logarithm of the minimum size in luma samples of a luma leaf block resulting from quadtree splitting of a CTU and the base 2 logarithm of the minimum coding block size in luma samples for luma CUs in slices with slice_type equal to 2 (I) referring to the SPS. When partition_constraints_override_enabled_flag is equal to 1, the default difference can be overridden by ph_log2_diff_min_qt_min_cb_luma present in PHs referring to the SPS. When the qtbtt_dual_tree_intra_flag is 1, the value of sps_log2_diff_min_qt_min_cb_intra_slice_luma shall be in the range of 0 to min(6, CtbLog2SizeY) − MinCbLog2SizeY. Otherwise, Tthe value of sps_log2_diff_min_qt_min_cb_intra_slice_luma shall be in the range of 0 to CtbLog2SizeY − MinCbLog2SizeY, inclusive. The base 2 logarithm of the minimum size in luma samples of a luma leaf block resulting from quadtree splitting of a CTU is derived as follows:

MinQtLog2SizeIntraY = sps_log2_diff_min_qt_min_cb_intra_slice_luma + MinCbLog2SizeY

sps_log2_diff_max_bt_min_qt_intra_slice_luma specifies the default difference between the base 2 logarithm of the maximum size (width or height) in luma samples of a luma coding block that can be split using a binary split and the minimum size (width or height) in luma samples of a luma leaf block resulting from quadtree splitting of a CTU in slices with slice_type equal to 2 (I) referring to the SPS. When partition_constraints_override_enabled_flag is equal to 1, the default difference can be overridden by ph_log2_diff_max_bt_min_qt_luma present in PHs referring to the SPS. When the qtbtt_dual_tree_intra_flag is 1, the value of sps_log2_diff_max_bt_min_qt_intra_slice_luma shall be in the range of 0 to min(6, CtbLog2SizeY) − MinQtLog2SizeIntraY. Otherwise, Tthe value of sps_log2_diff_max_bt_min_qt_intra_slice_luma shall be in the range of 0 to CtbLog2SizeY − MinQtLog2SizeIntraY, inclusive. When sps_log2_diff_max_bt_min_qt_intra_slice_luma is not present, the value of sps_log2_diff_max_bt_min_qt_intra_slice_luma is inferred to be equal to 0.

sps_log2_diff_max_tt_min_qt_intra_slice_luma specifies the default difference between the base 2 logarithm of the maximum size (width or height) in luma samples of a luma coding block that can be split using a ternary split and the minimum size (width or height) in luma samples of a luma leaf block resulting from quadtree splitting of a CTU in slices with slice_type equal to 2 (I) referring to the SPS. When partition_constraints_override_enabled_flag is equal to 1, the default difference can be overridden by ph_log2_diff_max_tt_min_qt_luma present in PHs referring to the SPS. When the qtbtt_dual_tree_intra_flag is 1, the value of sps_log2_diff_max_tt_min_qt_intra_slice_luma shall be in the range of 0 to min(6, CtbLog2SizeY) − MinQtLog2SizeIntraY. Otherwise, Tthe value of sps_log2_diff_max_tt_min_qt_intra_slice_luma shall be in the range of 0 to CtbLog2SizeY − MinQtLog2SizeIntraY, inclusive. When sps_log2_diff_max_tt_min_qt_intra_slice_luma is not present, the value of sps_log2_diff_max_tt_min_qt_intra_slice_luma is inferred to be equal to 0.

sps_log2_diff_min_qt_min_cb_intra_slice_chroma specifies the default difference between the base 2 logarithm of the minimum size in luma samples of a chroma leaf block resulting from quadtree splitting of a chroma CTU with treeType equal to DUAL_TREE_CHROMA and the base 2 logarithm of the minimum coding block size in luma samples for chroma CUs with treeType equal to DUAL_TREE_CHROMA in slices with slice_type equal to 2 (I) referring to the SPS. When partition_constraints_override_enabled_flag is equal to 1, the default difference can be overridden by ph_log2_diff_min_qt_min_cb_chroma present in PHs referring to the SPS. The value of sps_log2_diff_min_qt_min_cb_intra_slice_chroma shall be in the range of 0 to min(6, CtbLog2SizeY) − MinCbLog2SizeY, inclusive. When not present, the value of sps_log2_diff_min_qt_min_cb_intra_slice_chroma is inferred to be equal to 0. The base 2 logarithm of the minimum size in luma samples of a chroma leaf block resulting from quadtree splitting of a CTU with treeType equal to DUAL_TREE_CHROMA is derived as follows:

MinQtLog2SizeIntraC = sps_log2_diff_min_qt_min_cb_intra_slice_chroma + MinCbLog2SizeY

sps_log2_diff_max_bt_min_qt_intra_slice_chroma specifies the default difference between the base 2 logarithm of the maximum size (width or height) in luma samples of a chroma coding block that can be split using a binary split and the minimum size (width or height) in luma samples of a chroma leaf block resulting from quadtree splitting of a chroma CTU with treeType equal to DUAL_TREE_CHROMA in slices with slice_type equal to 2 (I) referring to the SPS. When partition_constraints_override_enabled_flag is equal to 1, the default difference can be overridden by ph_log2_diff_max_bt_min_qt_chroma present in PHs referring to the SPS. The value of sps_log2_diff_max_bt_min_qt_intra_slice_chroma shall be in the range of 0 to min(6, CtbLog2SizeY) − MinQtLog2SizeIntraC, inclusive. When sps_log2_diff_max_bt_min_qt_intra_slice_chroma is not present, the value of sps_log2_diff_max_bt_min_qt_intra_slice_chroma is inferred to be equal to 0.

sps_log2_diff_max_tt_min_qt_intra_slice_chroma specifies the default difference between the base 2 logarithm of the maximum size (width or height) in luma samples of a chroma coding block that can be split using a ternary split and the minimum size (width or height) in luma samples of a chroma leaf block resulting from quadtree splitting of a chroma CTU with treeType equal to DUAL_TREE_CHROMA in slices with slice_type equal to 2 (I) referring to the SPS. When partition_constraints_override_enabled_flag is equal to 1, the default difference can be overridden by ph_log2_diff_max_tt_min_qt_chroma present in PHs referring to the SPS. The value of sps_log2_diff_max_tt_min_qt_intra_slice_chroma shall be in the range of 0 to min(6, CtbLog2SizeY) − MinQtLog2SizeIntraC, inclusive. When sps_log2_diff_max_tt_min_qt_intra_slice_chroma is not present, the value of sps_log2_diff_max_tt_min_qt_intra_slice_chroma is inferred to be equal to 0.

ph_log2_diff_min_qt_min_cb_intra_slice_luma specifies the difference between the base 2 logarithm of the minimum size in luma samples of a luma leaf block resulting from quadtree splitting of a CTU and the base 2 logarithm of the minimum coding block size in luma samples for luma CUs in the slices with slice_type equal to 2 (I) associated with the PH. When the qtbtt_dual_tree_intra_flag is 1, the value of ph_log2_diff_min_qt_min_cb_intra_slice_luma shall be in the range of 0 to min(6, CtbLog2SizeY) − MinCbLog2SizeY. Otherwise, Tthe value of ph_log2_diff_min_qt_min_cb_intra_slice_luma shall be in the range of 0 to CtbLog2SizeY − MinCbLog2SizeY, inclusive. When not present, the value of ph_log2_diff_min_qt_min_cb_luma is inferred to be equal to sps_log2_diff_min_qt_min_cb_intra_slice_luma.

ph_log2_diff_max_bt_min_qt_intra_slice_luma specifies the difference between the base 2 logarithm of the maximum size (width or height) in luma samples of a luma coding block that can be split using a binary split and the minimum size (width or height) in luma samples of a luma leaf block resulting from quadtree splitting of a CTU in slices with slice_type equal to 2 (I) associated with the PH. When the qtbtt_dual_tree_intra_flag is 1, the value of ph_log2_diff_max_bt_min_qt_intra_slice_luma shall be in the range of 0 to min(6, CtbLog2SizeY) − MinQtLog2SizeIntraY. Otherwise, Tthe value of ph_log2_diff_max_bt_min_qt_intra_slice_luma shall be in the range of 0 to CtbLog2SizeY − MinQtLog2SizeIntraY, inclusive. When not present, the value of ph_log2_diff_max_bt_min_qt_intra_slice_luma is inferred to be equal to sps_log2_diff_max_bt_min_qt_intra_slice_luma.

ph_log2_diff_max_tt_min_qt_intra_slice_luma specifies the difference between the base 2 logarithm of the maximum size (width or height) in luma samples of a luma coding block that can be split using a ternary split and the minimum size (width or height) in luma samples of a luma leaf block resulting from quadtree splitting of a CTU in slices with slice_type equal to 2 (I) associated with the PH. When the qtbtt_dual_tree_intra_flag is 1, the value of ph_log2_diff_max_tt_min_qt_intra_slice_luma shall be in the range of 0 to min(6, CtbLog2SizeY) − MinQtLog2SizeIntraY. Otherwise, Tthe value of ph_log2_diff_max_tt_min_qt_intra_slice_luma shall be in the range of 0 to CtbLog2SizeY − MinQtLog2SizeIntraY, inclusive. When not present, the value of ph_log2_diff_max_tt_min_qt_intra_slice_luma is inferred to be equal to sps_log2_diff_max_tt_min_qt_intra_slice_luma.

ph_log2_diff_min_qt_min_cb_intra_slice_chroma specifies the difference between the base 2 logarithm of the minimum size in luma samples of a chroma leaf block resulting from quadtree splitting of a chroma CTU with treeType equal to DUAL_TREE_CHROMA and the base 2 logarithm of the minimum coding block size in luma samples for chroma CUs with treeType equal to DUAL_TREE_CHROMA in slices with slice_type equal to 2 (I) associated with the PH. The value of ph_log2_diff_min_qt_min_cb_intra_slice_chroma shall be in the range of 0 to min(6, CtbLog2SizeY) − MinCbLog2SizeY, inclusive. When not present, the value of ph_log2_diff_min_qt_min_cb_intra_slice_chroma is inferred to be equal to sps_log2_diff_min_qt_min_cb_intra_slice_chroma.

ph_log2_diff_max_bt_min_qt_intra_slice_chroma specifies the difference between the base 2 logarithm of the maximum size (width or height) in luma samples of a chroma coding block that can be split using a binary split and the minimum size (width or height) in luma samples of a chroma leaf block resulting from quadtree splitting of a chroma CTU with treeType equal to DUAL_TREE_CHROMA in slices with slice_type equal to 2 (I) associated with the PH. The value of ph_log2_diff_max_bt_min_qt_intra_slice_chroma shall be in the range of 0 to min(6,CtbLog2SizeY) − MinQtLog2SizeIntraC, inclusive. When not present, the value of ph_log2_diff_max_bt_min_qt_intra_slice_chroma is inferred to be equal to sps_log2_diff_max_bt_min_qt_intra_slice_chroma.

ph_log2_diff_max_tt_min_qt_intra_slice_chroma specifies the difference between the base 2 logarithm of the maximum size (width or height) in luma samples of a chroma coding block that can be split using a ternary split and the minimum size (width or height) in luma samples of a chroma leaf block resulting from quadtree splitting of a chroma CTU with treeType equal to DUAL_TREE_CHROMA in slices with slice_type equal to 2 (I) associated with the PH. The value of ph_log2_diff_max_tt_min_qt_intra_slice_chroma shall be in the range of 0 to min(6,CtbLog2SizeY) − MinQtLog2SizeIntraC, inclusive. When not present, the value of ph_log2_diff_max_tt_min_qt_intra_slice_chroma is inferred to be equal to sps_log2_diff_max_tt_min_qt_intra_slice_chroma

## Solution for both issues

To address both the reported issues, it’s proposed to set the upper limit of minimum QT size and maximum TT size to 64 in both dual-tree and single tree, and set the upper limit of maximum BT size to 64 in dual-tree. The relative semantics changes in the text are as following:

sps_log2_diff_min_qt_min_cb_intra_slice_luma specifies the default difference between the base 2 logarithm of the minimum size in luma samples of a luma leaf block resulting from quadtree splitting of a CTU and the base 2 logarithm of the minimum coding block size in luma samples for luma CUs in slices with slice_type equal to 2 (I) referring to the SPS. When partition_constraints_override_enabled_flag is equal to 1, the default difference can be overridden by ph_log2_diff_min_qt_min_cb_luma present in PHs referring to the SPS. The value of sps_log2_diff_min_qt_min_cb_intra_slice_luma shall be in the range of 0 to min(6, CtbLog2SizeY) − MinCbLog2SizeY, inclusive. The base 2 logarithm of the minimum size in luma samples of a luma leaf block resulting from quadtree splitting of a CTU is derived as follows:

MinQtLog2SizeIntraY = sps_log2_diff_min_qt_min_cb_intra_slice_luma + MinCbLog2SizeY

sps_log2_diff_max_bt_min_qt_intra_slice_luma specifies the default difference between the base 2 logarithm of the maximum size (width or height) in luma samples of a luma coding block that can be split using a binary split and the minimum size (width or height) in luma samples of a luma leaf block resulting from quadtree splitting of a CTU in slices with slice_type equal to 2 (I) referring to the SPS. When partition_constraints_override_enabled_flag is equal to 1, the default difference can be overridden by ph_log2_diff_max_bt_min_qt_luma present in PHs referring to the SPS. When the qtbtt_dual_tree_intra_flag is 1, the value of sps_log2_diff_max_bt_min_qt_intra_slice_luma shall be in the range of 0 to min(6, CtbLog2SizeY) − MinQtLog2SizeIntraY. Otherwise, Tthe value of sps_log2_diff_max_bt_min_qt_intra_slice_luma shall be in the range of 0 to CtbLog2SizeY − MinQtLog2SizeIntraY, inclusive. When sps_log2_diff_max_bt_min_qt_intra_slice_luma is not present, the value of sps_log2_diff_max_bt_min_qt_intra_slice_luma is inferred to be equal to 0.

sps_log2_diff_max_tt_min_qt_intra_slice_luma specifies the default difference between the base 2 logarithm of the maximum size (width or height) in luma samples of a luma coding block that can be split using a ternary split and the minimum size (width or height) in luma samples of a luma leaf block resulting from quadtree splitting of a CTU in slices with slice_type equal to 2 (I) referring to the SPS. When partition_constraints_override_enabled_flag is equal to 1, the default difference can be overridden by ph_log2_diff_max_tt_min_qt_luma present in PHs referring to the SPS. The value of sps_log2_diff_max_tt_min_qt_intra_slice_luma shall be in the range of 0 to min(6, CtbLog2SizeY) − MinQtLog2SizeIntraY, inclusive. When sps_log2_diff_max_tt_min_qt_intra_slice_luma is not present, the value of sps_log2_diff_max_tt_min_qt_intra_slice_luma is inferred to be equal to 0.

sps_log2_diff_max_bt_min_qt_intra_slice_chroma specifies the default difference between the base 2 logarithm of the maximum size (width or height) in luma samples of a chroma coding block that can be split using a binary split and the minimum size (width or height) in luma samples of a chroma leaf block resulting from quadtree splitting of a chroma CTU with treeType equal to DUAL_TREE_CHROMA in slices with slice_type equal to 2 (I) referring to the SPS. When partition_constraints_override_enabled_flag is equal to 1, the default difference can be overridden by ph_log2_diff_max_bt_min_qt_chroma present in PHs referring to the SPS. The value of sps_log2_diff_max_bt_min_qt_intra_slice_chroma shall be in the range of 0 to min(6, CtbLog2SizeY) − MinQtLog2SizeIntraC, inclusive. When sps_log2_diff_max_bt_min_qt_intra_slice_chroma is not present, the value of sps_log2_diff_max_bt_min_qt_intra_slice_chroma is inferred to be equal to 0.

sps_log2_diff_min_qt_min_cb_inter_slice specifies the default difference between the base 2 logarithm of the minimum size in luma samples of a luma leaf block resulting from quadtree splitting of a CTU and the base 2 logarithm of the minimum luma coding block size in luma samples for luma CUs in slices with slice_type equal to 0 (B) or 1 (P) referring to the SPS. When partition_constraints_override_enabled_flag is equal to 1, the default difference can be overridden by ph_log2_diff_min_qt_min_cb_luma present in PHs referring to the SPS. The value of sps_log2_diff_min_qt_min_cb_inter_slice shall be in the range of 0 to min(6, CtbLog2SizeY) − MinCbLog2SizeY, inclusive. The base 2 logarithm of the minimum size in luma samples of a luma leaf block resulting from quadtree splitting of a CTU is derived as follows:

MinQtLog2SizeInterY = sps_log2_diff_min_qt_min_cb_inter_slice + MinCbLog2SizeY

sps_log2_diff_max_tt_min_qt_inter_slice specifies the default difference between the base 2 logarithm of the maximum size (width or height) in luma samples of a luma coding block that can be split using a ternary split and the minimum size (width or height) in luma samples of a luma leaf block resulting from quadtree splitting of a CTU in slices with slice_type equal to 0 (B) or 1 (P) referring to the SPS. When partition_constraints_override_enabled_flag is equal to 1, the default difference can be overridden by ph_log2_diff_max_tt_min_qt_luma present in PHs referring to the SPS. The value of sps_log2_diff_max_tt_min_qt_inter_slice shall be in the range of 0 to min(6, CtbLog2SizeY) − MinQtLog2SizeInterY, inclusive. When sps_log2_diff_max_tt_min_qt_inter_slice is not present, the value of sps_log2_diff_max_tt_min_qt_inter_slice is inferred to be equal to 0.

sps_log2_diff_min_qt_min_cb_intra_slice_chroma specifies the default difference between the base 2 logarithm of the minimum size in luma samples of a chroma leaf block resulting from quadtree splitting of a chroma CTU with treeType equal to DUAL_TREE_CHROMA and the base 2 logarithm of the minimum coding block size in luma samples for chroma CUs with treeType equal to DUAL_TREE_CHROMA in slices with slice_type equal to 2 (I) referring to the SPS. When partition_constraints_override_enabled_flag is equal to 1, the default difference can be overridden by ph_log2_diff_min_qt_min_cb_chroma present in PHs referring to the SPS. The value of sps_log2_diff_min_qt_min_cb_intra_slice_chroma shall be in the range of 0 to min(6, CtbLog2SizeY) − MinCbLog2SizeY, inclusive. When not present, the value of sps_log2_diff_min_qt_min_cb_intra_slice_chroma is inferred to be equal to 0. The base 2 logarithm of the minimum size in luma samples of a chroma leaf block resulting from quadtree splitting of a CTU with treeType equal to DUAL_TREE_CHROMA is derived as follows:

MinQtLog2SizeIntraC = sps_log2_diff_min_qt_min_cb_intra_slice_chroma + MinCbLog2SizeY

sps_log2_diff_max_tt_min_qt_intra_slice_chroma specifies the default difference between the base 2 logarithm of the maximum size (width or height) in luma samples of a chroma coding block that can be split using a ternary split and the minimum size (width or height) in luma samples of a chroma leaf block resulting from quadtree splitting of a chroma CTU with treeType equal to DUAL_TREE_CHROMA in slices with slice_type equal to 2 (I) referring to the SPS. When partition_constraints_override_enabled_flag is equal to 1, the default difference can be overridden by ph_log2_diff_max_tt_min_qt_chroma present in PHs referring to the SPS. The value of sps_log2_diff_max_tt_min_qt_intra_slice_chroma shall be in the range of 0 to min(64, CtbLog2SizeY) − MinQtLog2SizeIntraC, inclusive. When sps_log2_diff_max_tt_min_qt_intra_slice_chroma is not present, the value of sps_log2_diff_max_tt_min_qt_intra_slice_chroma is inferred to be equal to 0.

ph_log2_diff_min_qt_min_cb_intra_slice_luma specifies the difference between the base 2 logarithm of the minimum size in luma samples of a luma leaf block resulting from quadtree splitting of a CTU and the base 2 logarithm of the minimum coding block size in luma samples for luma CUs in the slices with slice_type equal to 2 (I) associated with the PH. The value of ph_log2_diff_min_qt_min_cb_intra_slice_luma shall be in the range of 0 to min(6,CtbLog2SizeY) − MinCbLog2SizeY, inclusive. When not present, the value of ph_log2_diff_min_qt_min_cb_luma is inferred to be equal to sps_log2_diff_min_qt_min_cb_intra_slice_luma.

ph_log2_diff_max_bt_min_qt_intra_slice_luma specifies the difference between the base 2 logarithm of the maximum size (width or height) in luma samples of a luma coding block that can be split using a binary split and the minimum size (width or height) in luma samples of a luma leaf block resulting from quadtree splitting of a CTU in slices with slice_type equal to 2 (I) associated with the PH. When the qtbtt_dual_tree_intra_flag is 1, the value of ph_log2_diff_max_bt_min_qt_intra_slice_luma shall be in the range of 0 to min(6, CtbLog2SizeY) − MinQtLog2SizeIntraY. Otherwise, Tthe value of ph_log2_diff_max_bt_min_qt_intra_slice_luma shall be in the range of 0 to CtbLog2SizeY − MinQtLog2SizeIntraY, inclusive. When not present, the value of ph_log2_diff_max_bt_min_qt_intra_slice_luma is inferred to be equal to sps_log2_diff_max_bt_min_qt_intra_slice_luma.

ph_log2_diff_max_tt_min_qt_intra_slice_luma specifies the difference between the base 2 logarithm of the maximum size (width or height) in luma samples of a luma coding block that can be split using a ternary split and the minimum size (width or height) in luma samples of a luma leaf block resulting from quadtree splitting of a CTU in slices with slice_type equal to 2 (I) associated with the PH. The value of ph_log2_diff_max_tt_min_qt_intra_slice_luma shall be in the range of 0 to min(6,CtbLog2SizeY) − MinQtLog2SizeIntraY, inclusive. When not present, the value of ph_log2_diff_max_tt_min_qt_intra_slice_luma is inferred to be equal to sps_log2_diff_max_tt_min_qt_intra_slice_luma.

ph_log2_diff_min_qt_min_cb_intra_slice_chroma specifies the difference between the base 2 logarithm of the minimum size in luma samples of a chroma leaf block resulting from quadtree splitting of a chroma CTU with treeType equal to DUAL_TREE_CHROMA and the base 2 logarithm of the minimum coding block size in luma samples for chroma CUs with treeType equal to DUAL_TREE_CHROMA in slices with slice_type equal to 2 (I) associated with the PH. The value of ph_log2_diff_min_qt_min_cb_intra_slice_chroma shall be in the range of 0 to min(6,CtbLog2SizeY) − MinCbLog2SizeY, inclusive. When not present, the value of ph_log2_diff_min_qt_min_cb_intra_slice_chroma is inferred to be equal to sps_log2_diff_min_qt_min_cb_intra_slice_chroma.

ph_log2_diff_max_bt_min_qt_intra_slice_chroma specifies the difference between the base 2 logarithm of the maximum size (width or height) in luma samples of a chroma coding block that can be split using a binary split and the minimum size (width or height) in luma samples of a chroma leaf block resulting from quadtree splitting of a chroma CTU with treeType equal to DUAL_TREE_CHROMA in slices with slice_type equal to 2 (I) associated with the PH. The value of ph_log2_diff_max_bt_min_qt_intra_slice_chroma shall be in the range of 0 to min(6,CtbLog2SizeY) − MinQtLog2SizeIntraC, inclusive. When not present, the value of ph_log2_diff_max_bt_min_qt_intra_slice_chroma is inferred to be equal to sps_log2_diff_max_bt_min_qt_intra_slice_chroma.

ph_log2_diff_max_tt_min_qt_intra_slice_chroma specifies the difference between the base 2 logarithm of the maximum size (width or height) in luma samples of a chroma coding block that can be split using a ternary split and the minimum size (width or height) in luma samples of a chroma leaf block resulting from quadtree splitting of a chroma CTU with treeType equal to DUAL_TREE_CHROMA in slices with slice_type equal to 2 (I) associated with the PH. The value of ph_log2_diff_max_tt_min_qt_intra_slice_chroma shall be in the range of 0 to min(6,CtbLog2SizeY) − MinQtLog2SizeIntraC, inclusive. When not present, the value of ph_log2_diff_max_tt_min_qt_intra_slice_chroma is inferred to be equal to sps_log2_diff_max_tt_min_qt_intra_slice_chroma

ph_log2_diff_min_qt_min_cb_inter_slice specifies the difference between the base 2 logarithm of the minimum size in luma samples of a luma leaf block resulting from quadtree splitting of a CTU and the base 2 logarithm of the minimum luma coding block size in luma samples for luma CUs in the slices with slice_type equal to 0 (B) or 1 (P) associated with the PH. The value of ph_log2_diff_min_qt_min_cb_inter_slice shall be in the range of 0 to min(6,CtbLog2SizeY) − MinCbLog2SizeY, inclusive. When not present, the value of ph_log2_diff_min_qt_min_cb_luma is inferred to be equal to sps_log2_diff_min_qt_min_cb_inter_slice.

ph_log2_diff_max_tt_min_qt_inter_slice specifies the difference between the base 2 logarithm of the maximum size (width or height) in luma samples of a luma coding block that can be split using a ternary split and the minimum size (width or height) in luma samples of a luma leaf block resulting from quadtree splitting of a CTU in slices with slice_type equal to 0 (B) or 1 (P) associated with the PH. The value of ph_log2_diff_max_tt_min_qt_inter_slice shall be in the range of 0 to min(6,CtbLog2SizeY) − MinQtLog2SizeInterY, inclusive. When not present, the value of ph_log2_diff_max_tt_min_qt_inter_slice is inferred to be equal to sps_log2_diff_max_tt_min_qt_inter_slice.

# Supplimental test results

In this section, some supplemental tests are provided to show that setting minimum QT size to 128 causes too much loss and therefore not necessary to be provided as an option. The merge request !1500 is applied in VTM-8.0 to fix the ticket #1003.

In test 1, the following partitioning configurations are used, and the result is shown in Table 1.

MinQTChromaISlice             : 64    # in chroma samples

MinQTLumaISlice               : 128

MinQTNonISlice                : 128

MaxBTLumaISlice               : 128

MaxBTChromaISlice             : 128

MaxBTNonISlice              : 128

MaxTTLumaISlice             : 128

MaxTTChromaISlice           : 128

MaxTTNonISlice              : 128

As a comparison, in test 2, the following partitioning configurations are used, and the result is shown in Table 2.

MinQTChromaISlice             : 32  # in chroma samples

MinQTLumaISlice               : 64

MinQTNonISlice                : 64

MaxBTLumaISlice               : 128

MaxBTChromaISlice             : 128

MaxBTNonISlice                : 128

MaxTTLumaISlice               : 128

MaxTTChromaISlice             : 128

MaxTTNonISlice                : 128

In test 3, the MTT depths are set equal to 0, i.e. QT split only. The performance compares to test 1 is shown in Table 3. It demonstrates that by setting MTT depth to 0 we can achieve much better performance and faster compares to setting minimum QT size to 128.

Table 1 minQT size equals to 128

|  |  |  | All Intra Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-8.0 |  |  |  |
|  | Y | U | V | EncT | DecT | YUV |
| Class A1 | 6.04% | 30.20% | 32.89% | 80% | 78% | 11.85% |
| Class A2 | 3.93% | 14.91% | 10.05% | 55% | 82% | 5.51% |
| Class B | 5.92% | 14.12% | 16.64% | 56% | 73% | 7.73% |
| Class C | 13.23% | 23.83% | 25.82% | 45% | 68% | 15.45% |
| Class E | 8.98% | 7.52% | 8.38% | 54% | 78% | 8.75% |
| Overall | 7.74% | 17.99% | 18.91% | 56% | 75% | 9.93% |
| Class D | 12.34% | 21.20% | 22.26% | 44% | 66% | 14.04% |
| Class F | 25.63% | 34.10% | 39.98% | 35% | 70% | 27.98% |
|  |  |  | Random access Main10 |  |  |  |
|  |  |  | Over VTM-8.0 |  |  |  |
|  | Y | U | V | EncT | DecT | YUV |
| Class A1 | 14.84% | 21.05% | 24.63% | 26% | 83% | 17.15% |
| Class A2 | 23.72% | 32.21% | 27.40% | 25% | 88% | 24.79% |
| Class B | 29.81% | 31.52% | 32.73% | 26% | 82% | 30.29% |
| Class C | 62.52% | 54.00% | 61.77% | 33% | 85% | 61.70% |
| Class E |  |  |  |  |  |  |
| Overall | 34.32% | 35.56% | 37.79% | 27% | 84% | 34.94% |
| Class D | 53.11% | 50.54% | 52.56% | 44% | 89% | 52.79% |
| Class F | 56.60% | 54.26% | 59.36% | 31% | 92% | 56.65% |
|  |  |  | Low delay B Main10 |  |  |  |
|  |  |  | Over VTM-8.0 |  |  |  |
|  | Y | U | V | EncT | DecT | YUV |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | 29.54% | 24.06% | 24.41% | 24% | 74% | 28.76% |
| Class C | 70.26% | 45.04% | 50.74% | 34% | 84% | 66.21% |
| Class E | 30.19% | 22.93% | 24.56% | 37% | 107% | 28.80% |
| Overall | 43.27% | 30.77% | 33.23% | 30% | 85% | 41.25% |
| Class D | 59.61% | 42.48% | 47.68% | 49% | 90% | 57.12% |
| Class F | 97.28% | 68.84% | 73.67% | 33% | 98% | 91.97% |

Table 2 minQT size equals to 64

|  |  |  | All Intra Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-8.0 |  |  |  |
|  | Y | U | V |  | EncT | DecT |
| Class A1 | 6.05% | 30.20% | 32.89% | 11.85% | 80% | 78% |
| Class A2 | 3.93% | 14.91% | 10.05% | 5.51% | 55% | 81% |
| Class B | 5.92% | 14.13% | 16.64% | 7.73% | 56% | 73% |
| Class C | 13.23% | 23.83% | 25.83% | 15.46% | 46% | 68% |
| Class E | 8.99% | 7.54% | 8.39% | 12.36% | 54% | 76% |
| Overall | 7.75% | 17.99% | 18.92% | 9.94% | 56% | 74% |
| Class D | 12.36% | 21.23% | 22.28% | 14.06% | 44% | 67% |
| Class F | 25.63% | 34.10% | 39.99% | 27.99% | 35% | 69% |
|  |  |  | Random Access Main10 |  |  |  |
|  |  |  | Over VTM-8.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 2.70% | 6.97% | 7.33% | 3.99% | 63% | 86% |
| Class A2 | 3.84% | 10.41% | 7.46% | 4.88% | 60% | 91% |
| Class B | 5.05% | 10.50% | 10.64% | 6.14% | 62% | 88% |
| Class C | 12.33% | 16.32% | 18.44% | 13.35% | 60% | 87% |
| Class E |  |  |  |  |  |  |
| Overall | 6.28% | 11.33% | 11.42% | 7.38% | 61% | 88% |
| Class D | 13.32% | 20.95% | 22.27% | 14.86% | 63% | 95% |
| Class F | 21.68% | 24.39% | 27.32% | 22.57% | 52% | 95% |
|  |  |  | Low Delay B Main10 |  |  |  |
|  |  |  | Over VTM-8.0 |  |  |  |
|  | Y | U | V |  | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | 3.05% | 4.36% | 3.94% | 3.30% | 61% | 84% |
| Class C | 8.00% | 9.98% | 11.33% | 8.55% | 62% | 87% |
| Class E | 3.05% | 3.50% | 5.03% | 8.11% | 67% | 103% |
| Overall | 4.70% | 6.02% | 6.67% | 5.05% | 63% | 90% |
| Class D | 9.54% | 11.91% | 13.72% | 8.11% | 67% | 93% |
| Class F | 13.50% | 15.08% | 16.45% | 14.00% | 61% | 99% |

Table 3 QT only and minQtSize 8 v.s. minQtSize128

|  |  |  | All Intra Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 3 over Test 1 |  |  |  |
|  | Y | U | V | EncT | DecT | YUV |
| Class A1 | -1.52% | -14.26% | -15.69% | 14% | 103% | -5.23% |
| Class A2 | 1.20% | -0.61% | 1.97% | 14% | 104% | 1.16% |
| Class B | 0.64% | -0.18% | -0.71% | 13% | 113% | 0.25% |
| Class C | -3.21% | -8.63% | -8.88% | 13% | 128% | -4.38% |
| Class E | 1.38% | 5.51% | 4.83% | 16% | 122% | 2.18% |
| Overall | -0.36% | -3.53% | -3.65% | 14% | 114% | -1.22% |
| Class D | -3.13% | -6.62% | -7.32% | 15% | 123% | -3.87% |
| Class F | -6.65% | -10.48% | -13.20% | 21% | 122% | -7.81% |
|  |  |  | Random access Main10 |  |  |  |
|  |  |  | Test 3 over Test 1 |  |  |  |
|  | Y | U | V | EncT | DecT | YUV |
| Class A1 | -4.48% | -7.03% | -7.48% | 46% | 106% | -5.39% |
| Class A2 | -8.41% | -8.66% | -6.90% | 51% | 108% | -8.10% |
| Class B | -13.60% | -8.62% | -8.60% | 50% | 114% | -12.67% |
| Class C | -29.49% | -23.62% | -26.11% | 38% | 115% | -28.70% |
| Class E |  |  |  |  |  |  |
| Overall | -14.97% | -12.31% | -12.71% | 46% | 111% | -14.57% |
| Class D | -25.80% | -19.57% | -20.25% | 37% | 111% | -24.74% |
| Class F | -22.61% | -19.97% | -21.87% | 53% | 111% | -22.27% |
|  |  |  | Low delay B Main10 |  |  |  |
|  |  |  | Test 3 over Test 1 |  |  |  |
|  | Y | U | V | EncT | DecT | YUV |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -14.32% | -4.15% | -3.77% | 55% | 122% | -12.66% |
| Class C | -33.70% | -19.45% | -21.19% | 41% | 124% | -31.63% |
| Class E | -13.73% | -4.88% | -5.01% | 68% | 116% | -11.91% |
| Overall | -20.63% | -9.43% | -9.89% | 53% | 121% | -18.80% |
| Class D | -29.84% | -15.85% | -17.55% | 37% | 112% | -27.82% |
| Class F | -35.06% | -22.73% | -24.09% | 66% | 113% | -32.97% |

# Conclusion

In this contribution, redundant value of 128 for the minimum QT size, maximum BT size under dual-tree condition and redundant value of 128 for the maximum TT size are reported. It’s proposed to fix these issues to avoid confusion of the VVC text.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).