| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0356-v1 |
| --- | --- |

| Title: | Suggested bugfixes for CCLM filtering in the VVC specification draft |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexey Filippov Vasily Rufitskiy Elena Alshina | Tel: Email: | alexey.filippov@huawei.com vasily.rufitskiy@huawei.com. elena.alshina@huawei.com |
| Source: | Huawei Technologies Co., LTD |  |  |

_____________________________

# Abstract

This contribution proposes to fix several issues in the VVC specification draft to align it with the VTM-8.0 reference software. These bugs relate to CCLM intra prediction mode and have been already reported on the VVC bug tracker as tickets #1001 and #1002.

# Introduction

As described in [1], cross-component linear model prediction is a tool of VVC that predicts chrominance samples using reconstructed luminance samples and linear model parameters that are estimated using luminance and chrominance neighboring samples on the left and above sides of a predicted block. Although (nTbW + nTbH) reference samples are used for mode INTRA_LT_CCLM, modes INTRA_T_CCLM and INTRA_L_CCLM require up to 2*nTbW and 2*nTbH reference samples, respectively. However, there are several mismatches between the VVC specification draft and the VTM-8.0 software.

# Specification text changes

## Aspect 1: Wrong clipping ranges in formulas (355) and (356) defining the number of available neighbouring chroma samples numSampT and numSampL for CCLM (ticket #1001)

The VVC spec draft [1] specifies the number of available neighbouring chroma samples numSampT and numSampL for CCLM as follows:

numSampT = ( availT  &&  predModeIntra  = =  INTRA_T_CCLM ) ?
			( nTbW +Min( numTopRight, nTbH ) ) : 0	(355)

numSampL = ( availL  &&  predModeIntra  = =  INTRA_L_CCLM ) ?
			( nTbH +Min( numLeftBelow, nTbW ) ) : 0	(356)

To match with VTM-8.0, these formulas should be modified as follows [2] (modifications are highlighted by yellow):

numSampT = ( availT  &&  predModeIntra  = =  INTRA_T_CCLM ) ?
			( nTbW +Min( numTopRight, nTbW ) ) : 0	(355)

numSampL = ( availL  &&  predModeIntra  = =  INTRA_L_CCLM ) ?
			( nTbH +Min( numLeftBelow, nTbH ) ) : 0	(356)

## Aspect 2: Wrong upper bounds of numTopRight and numLeftBelow (ticket #1002)

The VVC spec draft [1] specifies the numbers of available top-right numTopRight and left-below numLeftBelow neighbouring chroma samples as follows:

- The number of available top-right neighbouring chroma samples numTopRight is derived as follows:

- The variable numTopRight is set equal to 0 and availTR is set equal to TRUE.

- When predModeIntra is equal to INTRA_T_CCLM, the following applies for x = nTbW..2 * nTbW − 1 until availTR is equal to FALSE or x is equal to 2 * nTbW − 1:

- The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xTbY , yTbY ) the neighbouring luma location ( xTbY + x, yTbY − 1 ), checkPredModeY set equal to FALSE, and cIdx as inputs, and the output is assigned to availTR

- When availTR is equal to TRUE, numTopRight is incremented by one.

- The number of available left-below neighbouring chroma samples numLeftBelow is derived as follows:

- The variable numLeftBelow is set equal to 0 and availLB is set equal to TRUE.

- When predModeIntra is equal to INTRA_L_CCLM, the following applies for y = nTbH..2 * nTbH − 1 until availLB is equal to FALSE or y is equal to 2 * nTbH − 1:

- The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xTbY , yTbY ), the neighbouring luma location ( xTbY − 1, yTbY + y ), checkPredModeY set equal to FALSE, and cIdx as inputs, and the output is assigned to availLB

- When availLB is equal to TRUE, numLeftBelow is incremented by one.

Note that the iteration ranges of (nTbW..2 * nTbW − 1) and (nTbH..2 * nTbH − 1) are equal to nTbW and nTbH, respectively. However, the top-right most and left-below most samples can't be reached due to the following conditions:

- until availTR is equal to FALSE or x is equal to 2 * nTbW – 1

and

- until availLB is equal to FALSE or y is equal to 2 * nTbH − 1,
respectively. This results in that numTopRight and numLeftBelow cannot take values of nTbW and nTbH, respectively.

To align the VVC spec draft with VTM-8.0, the following modification highlighted by yellow is suggested:

- The number of available top-right neighbouring chroma samples numTopRight is derived as follows:

- The variable numTopRight is set equal to 0 and availTR is set equal to TRUE.

- When predModeIntra is equal to INTRA_T_CCLM, the following applies for x = nTbW..2 * nTbW − 1 until availTR is equal to FALSE or x is equal to 2 * nTbW − 1:

- The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xTbY , yTbY ) the neighbouring luma location ( xTbY + x, yTbY − 1 ), checkPredModeY set equal to FALSE, and cIdx as inputs, and the output is assigned to availTR

- When availTR is equal to TRUE, numTopRight is incremented by one.

- The number of available left-below neighbouring chroma samples numLeftBelow is derived as follows:

- The variable numLeftBelow is set equal to 0 and availLB is set equal to TRUE.

- When predModeIntra is equal to INTRA_L_CCLM, the following applies for y = nTbH..2 * nTbH − 1 until availLB is equal to FALSE or y is equal to 2 * nTbH − 1:

- The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xTbY , yTbY ), the neighbouring luma location ( xTbY − 1, yTbY + y ), checkPredModeY set equal to FALSE, and cIdx as inputs, and the output is assigned to availLB

- When availLB is equal to TRUE, numLeftBelow is incremented by one.

Thus, the suggested changes into clause 8.4.5.2.13 “Specification of INTRA_LT_CCLM, INTRA_L_CCLM and INTRA_T_CCLM intra prediction mode” are summarized below:

##### 8.4.5.2.13	Specification of INTRA_LT_CCLM, INTRA_L_CCLM and INTRA_T_CCLM intra prediction mode

Inputs to this process are:

- the intra prediction mode predModeIntra,

- a sample location ( xTbC, yTbC ) of the top-left sample of the current transform block relative to the top-left sample of the current picture,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable cIdx specifying the colour component of the current block,

- chroma neighbouring samples p[ x ][ y ], with x = −1, y = 0..2 * nTbH − 1 and x = 0.. 2 * nTbW − 1, y = − 1.

Output of this process are predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1.

The current luma location ( xTbY, yTbY ) is derived as follows:

( xTbY, yTbY )  =  ( xTbC << ( SubWidthC − 1 ), yTbC << ( SubHeightC − 1 ) )	(351)

The variables availL, availT and availTL are derived as follows:

- The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xTbY, yTbY ), the neighbouring luma location ( xTbY − 1, yTbY ), checkPredModeY set equal to FALSE, and cIdx as inputs, and the output is assigned to availL.

- The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xTbY, yTbY ), the neighbouring luma location ( xTbY, yTbY − 1 ), checkPredModeY set equal to FALSE, and cIdx as inputs, and the output is assigned to availT.

- The variable availTL is derived as follows:

availTL  =  availL  &&  availT		(352)

- The number of available top-right neighbouring chroma samples numTopRight is derived as follows:

- The variable numTopRight is set equal to 0 and availTR is set equal to TRUE.

- When predModeIntra is equal to INTRA_T_CCLM, the following applies for x = nTbW..2 * nTbW − 1 until availTR is equal to FALSE or x is equal to 2 * nTbW − 1:

- The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xTbY , yTbY ) the neighbouring luma location ( xTbY + x, yTbY − 1 ), checkPredModeY set equal to FALSE, and cIdx as inputs, and the output is assigned to availTR

- When availTR is equal to TRUE, numTopRight is incremented by one.

- The number of available left-below neighbouring chroma samples numLeftBelow is derived as follows:

- The variable numLeftBelow is set equal to 0 and availLB is set equal to TRUE.

- When predModeIntra is equal to INTRA_L_CCLM, the following applies for y = nTbH..2 * nTbH − 1 until availLB is equal to FALSE or y is equal to 2 * nTbH − 1:

- The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xTbY , yTbY ), the neighbouring luma location ( xTbY − 1, yTbY + y ), checkPredModeY set equal to FALSE, and cIdx as inputs, and the output is assigned to availLB

- When availLB is equal to TRUE, numLeftBelow is incremented by one.

The number of available neighbouring chroma samples on the top and top-right numSampT and the number of available neighbouring chroma samples on the left and left-below numSampL are derived as follows:

- If predModeIntra is equal to INTRA_LT_CCLM, the following applies:

numSampT = availT ? nTbW : 0		(353)

numSampL = availL ? nTbH : 0		(354)

- Otherwise, the following applies:

numSampT = ( availT  &&  predModeIntra  = =  INTRA_T_CCLM ) ?
			( nTbW +Min( numTopRight, nTbHW ) ) : 0	(355)

numSampL = ( availL  &&  predModeIntra  = =  INTRA_L_CCLM ) ? 
			 ( nTbH +Min( numLeftBelow, nTbWH ) ) : 0	(356)

# Conclusion

It is proposed to adopt the suggested modifications to the VVC specification draft 9 to align it with the VTM-8.0 reference software.

# References

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 8)”, JVET Q2001-vE, 17th JVET meeting, Brussels, January 2020.

- VVC bug tracker ticket #1001, “Wrong clipping ranges in formulas (355) and (356) defining the number of available neighbouring chroma samples numSampT and numSampL for CCLM”,. https://jvet.hhi.fraunhofer.de/trac/vvc/ticket/1001

- VVC bug tracker ticket #1002, “Wrong upper bounds of numTopRight and numLeftBelow in 8.4.5.2.13. Specification of INTRA_LT_CCLM, INTRA_L_CCLM and INTRA_T_CCLM intra prediction mode”, https://jvet.hhi.fraunhofer.de/trac/vvc/ticket/1002

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).