| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0364-v1 |
| --- | --- |

| Title: | Information on cinematic aspect ratios in the context of JVET-Q0065 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Sean McCarthy, Walt Husak, Peng Yin, Taoran Lu, Fangjun Pu,, Tao Chen 432 Lakeside Drive, Sunnyvale, CA, 940850 | Tel: Email: | Sean.McCarthy@dolby.com |
| Source: | Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

JVET-Q0065 proposed level constraints on maximum tile size such as to reduce line buffer memory requirements. JVET-Q0065 was adopted at the 17th JVET meeting, Brussels, BE; but further study was suggested regarding some cinematic aspect ratios. This contribution provides information on HD, UHD, and cinematic aspect ratios to resolve that further study. This contribution proposes that the level restrictions adopted in JVET-Q0065 are appropriate and sufficient for standards specifying aspect ratios for HD, UHD, and digital cinema use cases.

# Aspect ratios for HD, UHD, and digital cinema

It was noted at the 17th JVET meeting that the level restrictions on maximum tile width adopted in JVET-Q0065 [1] may force the use of tiles for pictures with aspect ratios larger than 2:1. It was also noted that some cinematic formats such as CinemaScope have 2.39:1 aspect ratios.

CinemaScope is a historically important anamorphic cinema format. It was used in culturally significant films such as “Lady and the Tramp”, “Bridge on the River Kwai”, “The Robe”, and others.

SMPTE ST 428-1:2019 [2] defines D-Cinema Distribution Master (DCDM) image characteristics, including 2K and 4K operational levels in terms of maximum pixel width and height. The aspect ratio of all operational levels is 2:1. SMPTE ST 428-1:2019 also provides an informative annex to give examples of how other aspect ratios, including 2:39:1, may be accommodated within the operational levels. For the Cinemascope case, the annex gives the example of letterboxing within the DCDM 2:1 aspect ratio.

ITU-R BT.709 [3] and ITU-R BT.2020 [4] both specify an aspect ratio of 16:9 for HD and UHD television, respectively.

# Conclusion

The level restrictions adopted in JVET-Q0065 are appropriate and sufficient for standards specifying aspect ratios for HD, UHD, and digital cinema use cases.

# References

- M. Ikeda, Y. Yagasaki, T. Suzuki, “Level restrictions on maximum tile width for line buffer reduction”, JVET-Q0065, Brussels, BE, Jan 2020

- ST 428-1:2019, “SMPTE Standard - D-Cinema Distribution Master — Image Characteristics,”, Mar. 2019

- ITU-R BT.709-6, “Parameter values for the HDTV standards for production and international programme exchange”, Jun 2015

- ITU-R BT.2020-2, “Parameter values for ultra-high definition television systems for production and international programme exchange”, Oct 2015

# Patent rights declaration(s)

Dolby Laboratories, Inc. does not have any current or pending patent rights relating to the technology described in this contribution.