| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0378-v4 |
| --- | --- |

| Title: | TU split for ACT |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenji Kondo, Masaru Ikeda, Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81 50 3750 2782 kenji.kondo@sony.com Masaru.Ikeda@sony.com Teruhiko.S@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

In this contribution, it is proposed to allow a large transform such 64-points when an adaptive color transform (ACT) is disabled at the coding block. In the last meeting, to restrict the large transform was adopted. It can help to reduce buffer size and latency between inverse-transform and inverse-colour transform. However, it was restrictive because the 64-points transform can not be used even if ACT was disabled. This contribution reports the test results when the 64-points transform is allowed. Based on the common test condition for RGB 4:4:4 as described in JVET-Q2013, simulation results reportedly show that the average {G, B, R} BD-rate {0.01%,-0.26%,-0.25%} for screen contents and {-0.57%, -0.25%, -0.43%} for natural contents in AI configurations.

Introduction

In the last meeting, it was reported that the ACT framework increases buffer size and latency in a contribution [1]. Also, the one of solution was proposed in the [1] that was to restrict to use 64-points transform. The solution can reduce above buffer size and latency, but also it restricts the 64-points transform when the ACT is not used in the coding block. It brings additional coding efficiency loss.

This contribution proposes the other solution that is to use transform unit (TU) split only when cu_act_enabled_flag is equal to 1. With the proposed solution, the 64-points transform can be allowed when cu_act_enabled_flag is equal to 0.

The split method is used as same as the transform split method when the transform size is larger than MaxTbSize.

To enable the 64-points transform, this contribution proposes to change the syntax as shown in following section. The decoding process change is shown in the other document file.

Syntax change for sequence parameter set

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_max_luma_transform_size_64_flag | u(1) |
| … |  |
| if( ChromaArrayType = = 3 && ! sps_max_luma_transform_size_64_flag ) |  |
| sps_act_enabled_flag | u(1) |
| … |  |
| } |  |

Syntax change for transform unit

| transform_tree( x0, y0, tbWidth, tbHeight , treeType, chType ) { | Descriptor |
| --- | --- |
| … |  |
| if( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !cu_sbt_flag ) { |  |
| if( tbWidth > MaxTbSizeY ¦ ¦ tbHeight > MaxTbSizeY ¦ ¦ (cu_act_enabled_flag && ( tbWidth > 32 ¦ ¦ tbHeight > 32 ) ) ) { |  |
| verSplitFirst = ( ( tbWidth > MaxTbSizeY ¦ ¦ (cu_act_enabled_flag && tbWidth > 32 ) ) && tbWidth > tbHeight ) ? 1 : 0 |  |
| … |  |
| } else if( cu_sbt_flag ) { |  |
| if(cu_act_enabled_flag && ( tbWidth > 32 ¦ ¦ tbHeight > 32 ) ) { |  |
| verSplitFirst = ( (cu_act_enabled_flag && tbWidth > 32 ) && tbWidth > tbHeight ) ? 1 : 0 |  |
| trafoWidth = verSplitFirst ? ( tbWidth / 2 ) : tbWidth |  |
| trafoHeight = !verSplitFirst ? ( tbHeight / 2 ) : tbHeight |  |
| transform_tree( x0, y0, trafoWidth, trafoHeight, treeType, chType ) |  |
| if( verSplitFirst ) |  |
| transform_tree( x0 + trafoWidth, y0, trafoWidth, trafoHeight, treeType, chType ) |  |
| else |  |
| transform_tree( x0, y0 + trafoHeight, trafoWidth, trafoHeight, treeType, chType ) |  |
| … |  |
| } |  |

Experiment

To test the proposal as introduce above, the method was implemented in VTM-8.0. And, the coding efficiency was checked. In this test, the test common condition for RGB 4:4:4 was used as described in [2].

Test results

The test results are shown in Table 1 and Table 2. The encoding runtime is not reliable because the machine platforms were not same. The decoding runtime is reliable. It was measured using a same simulation machine.

(The test results had been updated in version 2, because source code bugs were found.)

Table 1: BD-rate for screen contents

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RGB 4:4:4 |  |  |  |  |  |  |  |  |
|  | G |  | B |  | R |  | EncT |  | DecT |
| TGM 1080p | 0.06% |  | -0.18% |  | -0.17% |  | 96% |  | 101% |
| TGM 720p | -0.01% |  | -0.26% |  | -0.23% |  | 95% |  | 99% |
| Animation | 0.02% |  | -0.30% |  | -0.28% |  | 98% |  | 100% |
| Mixed content | -0.01% |  | -0.34% |  | -0.38% |  | 96% |  | 100% |
| Overall | 0.01% |  | -0.26% |  | -0.25% |  | 96% |  | 100% |
|  | Random access Main10 |  |  |  |  |  |  |  |  |
|  | RGB 4:4:4 |  |  |  |  |  |  |  |  |
|  | G |  | B |  | R |  | EncT |  | DecT |
| TGM 1080p | -0.02% |  | -0.24% |  | -0.21% |  | 102% |  | 100% |
| TGM 720p | 0.11% |  | -0.31% |  | -0.27% |  | 98% |  | 100% |
| Animation | 0.17% |  | -0.57% |  | -0.47% |  | 103% |  | 100% |
| Mixed content | 0.10% |  | -0.29% |  | -0.41% |  | 99% |  | 100% |
| Overall | 0.08% |  | -0.34% |  | -0.33% |  | 101% |  | 100% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |
|  | RGB 4:4:4 |  |  |  |  |  |  |  |  |
|  | G | B |  | R |  | EncT |  | DecT |  |
| TGM 1080p | -0.20% | -0.40% |  | -0.36% |  | 106% |  | 99% |  |
| TGM 720p | -0.44% | -0.24% |  | -0.31% |  | 105% |  | 101% |  |
| Animation | 0.23% | -0.72% |  | -0.61% |  | 105% |  | 100% |  |
| Mixed content | 0.16% | -0.72% |  | -0.52% |  | 104% |  | 99% |  |
| Overall | -0.10% | -0.49% |  | -0.43% |  | 105% |  | 100% |  |

Table 2: BD-rate for natural contents

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | VTM8.0 |  |  |  |  |  |  |  |  |
|  | G | B |  | R |  | EncT |  | DecT |  |
| RGB 4:4:4 | -0.57% | -0.25% |  | -0.43% |  | 99% |  | 99% |  |
|  | Random access Main10 |  |  |  |  |  |  |  |  |
|  | VTM8.0 |  |  |  |  |  |  |  |  |
|  | G | B |  | R |  | EncT |  | DecT |  |
| RGB 4:4:4 | 0.20% | -0.27% |  | -0.24% |  | 104% |  | 100% |  |

Additional test

In a contribution JVET-R0321[3] , a bug related chroma QP table is reported. In the [3], It proposed to change simulation configuration as following.

UseIdentityTableForNon420Chroma sets to 0

SameCQPTablesForAllChroma           : 1

QpInValCb                           : 22 62

QpOutValCb                          : 22 62

With above configurations, the proposal of this contribution was additionally tested. For anchor data, it was brought from the [3]. (Where, for only Traffic, the anchor data was incorrect. We confirmed it with a proponent of [3]. So, we tested the data only for the Traffic sequence.) The summary results are shown in Table 3 and Table 4.

Table 3: BD-rate for AI based on JVET-R0321

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over JVET-R0321 |  |  |  |  |
|  | G | B | R | EncT | DecT |
| TGM1080p | -0.05% | -0.20% | -0.20% | #NUM! | #NUM! |
| TGM720p | -0.02% | -0.19% | -0.13% | #NUM! | #NUM! |
| Animation | -0.02% | -0.23% | -0.27% | #NUM! | #NUM! |
| Mixed Content | -0.13% | -0.38% | -0.40% | #NUM! | #NUM! |
| Natural | -0.60% | -0.27% | -0.39% | #NUM! | #NUM! |
| Overall | -0.25% | -0.25% | -0.30% | #NUM! | #NUM! |

For screen content, the average is {-0.05%,	-0.25%, -0.25%} for G, B and R component.

Table 4: BD-rate for RA based on JVET-R0321

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over JVET-R0321 |  |  |  |  |
|  | R | G | B | EncT | DecT |
| TGM1080p | -0.07% | -0.26% | -0.20% | #NUM! | #NUM! |
| TGM720p | -0.03% | -0.37% | -0.25% | #NUM! | #NUM! |
| Animation | 0.00% | -0.60% | -0.51% | #NUM! | #NUM! |
| Mixed Content | -0.06% | -0.40% | -0.45% | #NUM! | #NUM! |
| Natural | 0.15% | -0.33% | -0.24% | #NUM! | #NUM! |
| Overall | 0.03% | -0.37% | -0.30% | #NUM! | #NUM! |

For screen content, the average is { -0.04%, -0.41%, -0.35%} for G, B and R component.

Additional test for YUV 4:4:4

YUV 4:4:4 contents are also tested. The results are shown in the Table 5. In this test, Dual-tree was disabled to enable adaptive color transform.

Table 5: Test results for YUV 4:4:4

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| TGM 1080p | 0.0% | 0.0% | 0.0% |
| TGM 720p | -0.2% | -0.2% | -0.2% |
| Animation | 0.0% | 0.0% | -0.2% |
| Mixed content | -0.1% | -0.1% | -0.1% |
| Camera-Captured content | -1.0% | 0.2% | -0.5% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| TGM 1080p | 0.1% | 0.0% | 0.0% |
| TGM 720p | -0.3% | -0.2% | -0.3% |
| Animation | -0.2% | -0.2% | -0.3% |
| Mixed content | -0.3% | -0.2% | -0.2% |
| Camera-Captured content | -0.6% | 0.0% | -0.7% |
| Enc Time[%] | 96% |  |  |
| Dec Time[%] | 100% |  |  |

Conclusion

This contribution proposed to allow to use 64-points transform when ACT is not used. On the other hand, if ACT is used at the coding block, the transform size is restricted to 32-points transform. It can help to reduce buffer size and latency. The simulation results showed that the average {G, B, R} BD-rate {0.01%,-0.26%,-0.25%} for screen contents and {-0.57%, -0.25%, -0.43%} for natural contents in AI configurations. Additionally, with encoding configurations proposed in JVET-Q0321, the proposal of this contribution was tested. The results showed that {-0.05%, -0.25%, -0.25%} for screen contents and {-0.60%, -0.27%, -0.39%} for natural contents in AI configurations.

Reference

- Kenji Kondo, Masaru Ikeda, Teruhiko Suzuki, “A memory issue on adaptive color transform”, Document of JVET-Q0162, JVET of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020

- Yung-Hsuan Chao, Yu-Chen Sun, Jizheng Xu, Xiaozhong Xu, “JVET common test conditions and software reference configurations for non-4:2:0 colour formats”, Document of JVET-Q2013, JVET of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020

- Ji-Zheng Xu, Li Zhang, Weijia Zhu, Xiaoyu Xiu, Yi-Wen Chen2, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang, “AHG3: Chroma QP table bug-fix and CTC update for RGB coding in VTM-8.0”, JVET-R0321-v1, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, April 2020

Patent rights declaration(s)

Sony corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).