| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, 6–17 January 2020 | Document: JVET-R0407-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-R0282: GEO with MMVD |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck |  |  |
| Author(s) or Contact(s): | Kevin Reuzé | Tel: Email: | kreuz@qti.qualcomm.com |
| Source: | Qualcomm Inc |  |  |

_____________________________

# Abstract

This is a crosscheck for the contribution number R0282 from Nokia, proposing to combine the Geometric Partition Mode (GEO or GPM) with Merge with Motion Vector Difference (MMVD). The crosschecker verified that the software implementation matches the description and the simulation BD-rate results reportedly match those provided by the proponent.

# Introduction

One method is described in JVET-R0282.

The described method offers a way to combine GEO with MMVD. 2 additional syntax elements indicate whether each GEO partition uses MMVD, and another 2 additional syntax elements are used for each partition that uses MMVD, to signal the distance and sign. The same distance and sign look-up tables as in the original MMVD are used.

# Results

Results 	are proposed over the VTM8.0 anchor. Runtimes are not reliable.

Option 1:

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.16% | -0.24% | -0.36% | 136% | 102% |
| Class C | -0.42% | -0.53% | -0.83% | 131% | 97% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.41% | -0.57% | -0.81% | 138% | 105% |
| Class F | -0.07% | -0.18% | -0.24% | 121% | 103% |
|  |  |  | Low delay B |  |  |
|  |  |  | Over VTM-8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.77% | -1.32% | -1.25% | 138% | 92% |
| Class E | -0.47% | -0.28% | -0.85% | 133% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.68% | -1.55% | -1.32% | 146% | 105% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).