| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0451-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-R0354: (AHG14: BDPCM for Inter/IBC-predicted residuals) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Alican Nalci | Tel: Email: | +1 (858) 845-6730 analci@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document reports the crosscheck summary for the methods and results in JVET-Q0354. The crosschecked contribution proposes to enable BDPCM for inter-predicted residuals and intra block copy (IBC) residuals. Currently, BDPCM is only enabled for intra residuals.. Results are provided by the proponent for the 444 and RGB compression cases for both lossy and lossless contribution.

# Simulation Results

The source code for JVET-R0354 was provided by the proponent. The provided software is based on VTM-8.0 and results were obtained for YUV444 (lossy and lossless coding) and RGB lossless configurations. A partial crosscheck is performed for 444 and RGB lossless configurations. The crosschecker confirms that both the simulation results match the proponents and the spec text changes are accurate and reflect the proposed changes.

| RGB | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM-8.0 | Tested |  | VTM-8.0 | Tested |  | VTM-8.0 | Tested |  |
| TGM 1080p | 10.9 | 10.9 | -0.59% | 173.2 | 174.9 | -1.15% | 251.2 | 254.2 | -1.47% |
| TGM 720p | 5.0 | 5.1 | -0.27% | 51.5 | 51.9 | -1.62% | 90.1 | 90.9 | -1.92% |
| A | 3.0 | 3.1 | -2.29% | 8.6 | 8.9 | -1.87% | 7.1 | 7.7 | -3.45% |
| M | 3.0 | 3.0 | -0.37% | 19.1 | 19.5 | -1.74% | 21.0 | 21.5 | -2.12% |
| CC | 0.9 | 0.9 | -0.04% | 0.9 | 0.9 | -0.21% | 0.9 | 0.9 | -0.32% |
| Overall | 5.2 | 5.3 | -0.72% | 61.5 | 62.1 | -1.40% | 90.7 | 91.8 | -1.93% |
| Enc Time[%] | 143% |  |  | 150% |  |  | 159% |  |  |
| Dec Time[%] | 132% |  |  | 111% |  |  | 115% |  |  |
| YUV | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM-8.0 | Tested |  | VTM-8.0 | Tested |  | VTM-8.0 | Tested |  |
| TGM 1080p | 10.7 | 10.7 | -0.62% | 174.7 | 176.5 | -1.28% | 252.9 | 256.1 | -1.60% |
| TGM 720p | 6.4 | 6.4 | -0.35% | 64.5 | 65.0 | -1.50% | 115.8 | 116.6 | -1.75% |
| A | 3.5 | 3.6 | -0.96% | 7.9 | 8.0 | -0.94% | 7.9 | 7.9 | -0.68% |
| M | 3.9 | 3.9 | -0.52% | 26.3 | 26.7 | -1.28% | 28.9 | 29.4 | -1.64% |
| CC | 1.0 | 1.0 | -0.06% | 1.1 | 1.1 | -0.22% | 1.1 | 1.1 | -0.35% |
| Overall | 5.8 | 5.8 | -0.53% | 66.3 | 67.0 | -1.14% | 99.2 | 100.3 | -1.32% |
| Enc Time[%] | 146% |  |  | 153% |  |  | 165% |  |  |
| Dec Time[%] | 138% |  |  | 112% |  |  | 113% |  |  |