| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0483 |
| --- | --- |

| Title: | AHG9: Combination of JVET-R0049 and JVET-R0271 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohammed Golam Sarwer Yan Ye, Jiancong Luo, Jie Chen 525 Almanor Ave, Sunnyvale, CA Alican Nalci, Muhammed Coban, Hilmi E. Egilmez and Marta Karczewicz 5775 Morehouse Dr., San Diego, CA, 92121, USA Shih-Ta Hsiang, Chih-Wei Hsu, Zhi-Yi Lin, Tzu-Der Chuang, Olena Chubach, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan Tsung-Chuan Ma, Xiaoyu Xiu, Yi-Wen Chen, Hong-Jheng Jhu, Xianglin Wang 8910 University Center Dr., Suite 400, San Diego, CA, 92122, USA Sunmi Yoo, Jungah Choi, Jaehyun Lim, Seunghwan Kim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Zhipin Deng1, Ye-Kui Wang2, Li Zhang2 Kai Zhang 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA Karam Naser Fabrice Le Léannec Tangi Poirier Michel Kerdranvat Tomonori Hashimoto, Eiichi Sasaki, Tomoko Aono, Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba, Japan Jonathan Gan, Building A, 5 Talavera Rd, Macquarie Park, 2113, NSW, Australia | Tel: Email: | + 1 408-858-6780 m.sarwer@alibaba-inc.com |
| Source: | Alibaba Group, Qualcomm Inc., MediaTek Inc, Kwai Inc., LGE, Bytedance Inc., InterDigital, Sharp, Canon Australia |  |  |

_____________________________

# Abstract

This contribution combines method 1 and method 3 of JVET-R0049 with variation 2 and variation 3 of JVET-R0271.

# Proposed methods

## Combination 1: method 1 of R0049 + variation 2 of R0271

### Syntax Table

Table 1: SPS syntax table of the combination 1

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| ….. |  |
| sps_transform_skip_enabled_flag | u(1) |
| if( sps_transform_skip_enabled_flag ) { |  |
| log2_transform_skip_max_size_minus2 | ue(v) |
| sps_bdpcm_enabled_flag | u(1) |
| sps_ts_residual_coding_disabled_slice_present_flag | u(1) |
| } |  |
| …. |  |
| } |  |

Table 2: Slice header syntax table of the combination 1

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if (sps_ts_residual_coding_disabled_slice_present_flag ) |  |
| slice_ts_residual_coding_disabled_flag | u(1) |
| if( sps_dep_quant_enabled_flag && !slice_ts_residual_coding_disabled_flag ) |  |
| slice_dep_quant_enabled_flag | u(1) |
| if( sps_sign_data_hiding_enabled_flag && !slice_dep_quant_enabled_flag && !slice_ts_residual_coding_disabled_flag ) |  |
| slice_sign_data_hiding_enabled_flag | u(1) |
| if( ph_lmcs_enabled_flag ) |  |
| slice_lmcs_enabled_flag | u(1) |
| if( ph_explicit_scaling_list_enabled_flag ) |  |
| slice_explicit_scaling_list_used_flag | u(1) |
| … |  |
| } |  |

### Truth Table:

Encoder settings of the combination 1: method 1 of R0049 + variation 2 of R0271 for different use cases.

- Full sequence lossy

o   sps_ts_residual_coding_disabled_slice_present_flag = 0

o   slice_ts_residual_coding_disabled_flag = 0 (inferred)

o   slice_dep_quant_enabled_flag = 0/1 (can be either 0 or 1)

o   slice_sign_data_hiding_enabled_flag = 1/0 (can be either 0 or 1)

- Full sequence lossless

o   sps_ts_residual_coding_disabled_slice_present_flag = 1

o   slice_ts_residual_coding_disabled_flag = 1

o   slice_dep_quant_enabled_flag = 0 (inferred)

o   slice_sign_data_hiding_enabled_flag = 0 (inferred)

- Mixed lossy/lossless at slice level

o   sps_ts_residual_coding_disabled_slice_present_flag = 1

o   slice_ts_residual_coding_disabled_flag = 0/1 based on lossy/lossless slice

o   slice_dep_quant_enabled_flag = 0/1 (inferred to 0 if slice_ts_residual_coding_disabled_flag == 1)

o   slice_sign_data_hiding_enabled_flag = 0/1 (inferred to 0 if slice_ts_residual_coding_disabled_flag == 1)

- Only RRC is used for all lossy/lossless cases (assuming low complexity encoder wants to implement only 1 residual coding method)

o   sps_ts_residual_coding_disabled_slice_present_flag = 1

o   slice_ts_residual_coding_disabled_flag = 1

o   slice_dep_quant_enabled_flag = 0 (inferred)

o   slice_sign_data_hiding_enabled_flag = 0 (inferred)

| Use case | sps_TS_enabled | sps_tsrc_disable_present | sh_tsrc_disable_flag | ( sh_SDH ¦¦ sh_DQ ) | Number of SH bits |
| --- | --- | --- | --- | --- | --- |
| TS disabled | 0 | x (0 inferred) | x (0 inferred) | 0/1 | 1 |
| Lossy in CLVS | 1 | 0 | x (0 inferred) | 0/1 | 1 |
| lossless in CLVS** | 1 | 1 | 1 | x (0 inferred) | 1 |
| Mixed LL lossy Slices | 1 | 1 | 0 | 0/1 | 2 |
| Mixed LL lossless Slices | 1 | 1 | 1 | x (0 inferred) | 1 |
| RRC only | 1 | 1 | 1 | x (0 inferred) | 1 |

** is lossless CTC

## Combination 2: method 1 of R0049 + variation 3 of R0271

### Syntax Table:

Table 3: SPS syntax table of the combination 2

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| ….. |  |
| sps_transform_skip_enabled_flag | u(1) |
| if( sps_transform_skip_enabled_flag ) { |  |
| log2_transform_skip_max_size_minus2 | ue(v) |
| sps_bdpcm_enabled_flag | u(1) |
| sps_ts_residual_coding_disabled_slice_present_flag | u(1) |
| } |  |
| …. |  |
| } |  |

Table 4: Slice header syntax table of the combination 2

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_dep_quant_enabled_flag ) |  |
| slice_dep_quant_enabled_flag | u(1) |
| if( sps_sign_data_hiding_enabled_flag && !slice_dep_quant_enabled_flag ) |  |
| slice_sign_data_hiding_enabled_flag | u(1) |
| if( sps_ts_residual_coding_disabled_slice_present_flag && !slice_dep_quant_enabled_flag && ! slice_sign_data_hiding_enabled_flag ) |  |
| slice_ts_residual_coding_disabled_flag | u(1) |
| if( ph_lmcs_enabled_flag ) |  |
| slice_lmcs_enabled_flag | u(1) |
| if( ph_explicit_scaling_list_enabled_flag ) |  |
| slice_explicit_scaling_list_used_flag | u(1) |
| if( NumEntryPoints > 0 ) { |  |
| … |  |
| } |  |

### Truth Table:

Encoder settings of the combination 2: method 1 of R0049 + variation 3 of R0271 for different use cases.

- Full sequence lossy

  - sps_ts_residual_coding_disabled_slice_present_flag = 0

  - slice_dep_quant_enabled_flag = 1/0 (can be either 0 or 1)

  - slice_sign_data_hiding_enabled_flag = 0/1 (can be either 0 or 1)

  - slice_ts_residual_coding_disabled_flag = 0 (inferred)

- Full sequence lossless

o   sps_ts_residual_coding_disabled_slice_present_flag = 1

o   slice_dep_quant_enabled_flag = 0 (inferred if sps_dep_quant_enabled_flag )

- slice_sign_data_hiding_enabled_flag=0 (inferred if sps_sign_data_hiding_enabled_flag)

- slice_ts_residual_coding_disabled_flag = 1

- Mixed lossy/lossless at slice level

- sps_ts_residual_coding_disabled_slice_present_flag = 1

- slice_dep_quant_enabled_flag = 0/1 (0 for lossless, 1 for lossy)

- slice_sign_data_hiding_enabled_flag = 0/1 (0 for lossless, 1 for lossy)

- slice_ts_residual_coding_disabled_flag = 1/0 ( 1 for lossless, 0 for lossy)

- Only RRC is used for all lossy/lossless cases (assuming low complexity encoder wants to implement only 1 residual coding method)

o   sps_ts_residual_coding_disabled_slice_present_flag = 1

o   slice_dep_quant_enabled_flag = 0 (inferred if sps_dep_quant_enabled_flag )

- slice_sign_data_hiding_enabled_flag = 0 (inferred if sps_sign_data_hiding_enabled_flag)

- slice_ts_residual_coding_disabled_flag = 1

| Use case | sps_TS_enabled | sps_tsrc_disable_present | ( sh_SDH ¦¦ sh_DQ ) | sh_tsrc_disable_flag | Number of SH bits |
| --- | --- | --- | --- | --- | --- |
| TS disabled | 0 | x (0 inferred) | 0/1 | x (0 inferred) | 1 |
| Lossy in CLVS | 1 | 0 | 0/1 | x (0 inferred) | 1 |
| lossless in CLVS** | 1 | 1 | x (0 inferred) | 1 | 1 |
| Mixed LL lossy Slices | 1 | 1 | 1 | x (0 inferred) | 1 |
| Mixed LL lossless Slices | 1 | 1 | 0 | 1 | 2 |
| RRC only | 1 | 1 | x (0 inferred) | 1 | 1 |

** is lossless CTC

## Combination 3: method 3 of R0049 + variation 2 of R0271

### Syntax Table:

Table 5: Slice header syntax table of the combination 3

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if (sps_transform_skip_enabled_flag ) |  |
| slice_ts_residual_coding_disabled_flag | u(1) |
| if( sps_dep_quant_enabled_flag && !slice_ts_residual_coding_disabled_flag ) |  |
| slice_dep_quant_enabled_flag | u(1) |
| if( sps_sign_data_hiding_enabled_flag && !slice_dep_quant_enabled_flag && !slice_ts_residual_coding_disabled_flag ) |  |
| slice_sign_data_hiding_enabled_flag | u(1) |
| if( ph_lmcs_enabled_flag ) |  |
| slice_lmcs_enabled_flag | u(1) |
| if( ph_explicit_scaling_list_enabled_flag ) |  |
| slice_explicit_scaling_list_used_flag | u(1) |
| … |  |
| } |  |

### Truth Table:

Encoder settings of the combination 3: method 3 of R0049 + variation 2 of R0271 for different use cases.

- Full sequence lossy

o   slice_ts_residual_coding_disabled_flag = 0

o   slice_dep_quant_enabled_flag = 0/1

o   slice_sign_data_hiding_enabled_flag = 0/1

- Full sequence lossless

o   slice_ts_residual_coding_disabled_flag = 1

o   slice_dep_quant_enabled_flag = 0 (inferred)

o   slice_sign_data_hiding_enabled_flag = 0 (inferred)

- Mixed lossy/lossless at slice level

o   slice_ts_residual_coding_disabled_flag = 0/1 based on lossy/lossless slice

o   slice_dep_quant_enabled_flag = 0/1 (inferred to 0 if slice_ts_residual_coding_disabled_flag == 1)

o   slice_sign_data_hiding_enabled_flag = 0/1 (inferred to 0 if slice_ts_residual_coding_disabled_flag == 1)

- Only RRC is used for all lossy/lossless cases (assuming low complexity encoder wants to implement only 1 residual coding method)

o   slice_ts_residual_coding_disabled_flag = 1

o   slice_dep_quant_enabled_flag = 0 (inferred)

- slice_sign_data_hiding_enabled_flag = 0 (inferred)

| Use case | sps_TS_enabled | sh_tsrc_disable_flag | ( sh_SDH ¦¦ sh_DQ ) | Number of SH bits |
| --- | --- | --- | --- | --- |
| TS disabled | 0 | x (0 inferred)) | 0/1 | 1 |
| Lossy in CLVS | 1 | 0 | 0/1 | 2 |
| lossless in CLVS** | 1 | 1 | x (0 inferred)) | 1 |
| Mixed LL lossy Slices | 1 | 0 | 0/1 | 2 |
| Mixed LL lossless Slices | 1 | 1 | x (0 inferred)) | 1 |
| RRC only | 1 | 1 | x (0 inferred)) | 1 |

** is lossless CTC

## Combination 4: method 3 of R0049 + variation 3 of R0271

### Syntax Table:

Table 6: Slice header syntax table of the combination 4

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_dep_quant_enabled_flag ) |  |
| slice_dep_quant_enabled_flag | u(1) |
| if( sps_sign_data_hiding_enabled_flag && !slice_dep_quant_enabled_flag ) |  |
| slice_sign_data_hiding_enabled_flag | u(1) |
| if(sps_transform_skip_enabled_flag && !slice_dep_quant_enabled_flag && ! slice_sign_data_hiding_enabled_flag ) |  |
| slice_ts_residual_coding_disabled_flag | u(1) |
| if( ph_lmcs_enabled_flag ) |  |
| slice_lmcs_enabled_flag | u(1) |
| if( ph_explicit_scaling_list_enabled_flag ) |  |
| slice_explicit_scaling_list_used_flag | u(1) |
| if( NumEntryPoints > 0 ) { |  |
| … |  |
| } |  |

### Truth Table:

Encoder settings of the combination 2: method 1 of R0049 + variation 3 of R0271 for different use cases.

- Full sequence lossy

  - slice_dep_quant_enabled_flag = 1/0

  - slice_sign_data_hiding_enabled_flag = 0/1

  - slice_ts_residual_coding_disabled_flag = 0

Full sequence lossless

o   slice_dep_quant_enabled_flag = 0 (inferred if sps_dep_quant_enabled_flag )

- slice_sign_data_hiding_enabled_flag=0 (inferred if sps_sign_data_hiding_enabled_flag)

- slice_ts_residual_coding_disabled_flag = 1

- Mixed lossy/lossless at slice level

- slice_dep_quant_enabled_flag = 0/1 (0 for lossless, 1 for lossy)

- slice_sign_data_hiding_enabled_flag = 0/1 (0 for lossless, 1 for lossy)

- slice_ts_residual_coding_disabled_flag = 1/0 ( 1 for lossless, 0 for lossy)

- Only RRC is used for all lossy/lossless cases (assuming low complexity encoder wants to implement only 1 residual coding method)

o   slice_dep_quant_enabled_flag = 0 (inferred if sps_sign_data_hiding_enabled_flag)

- slice_sign_data_hiding_enabled_flag = 0 (inferred if sps_sign_data_hiding_enabled_flag)

- slice_ts_residual_coding_disabled_flag = 1

|  | sps_TS_enabled | ( sh_SDH ¦¦ sh_DQ ) | sh_tsrc_disable_flag | Number of SH bits |
| --- | --- | --- | --- | --- |
| TS disabled | 0 | 0/1 | x (0 inferred) | 1 |
| Lossy in CLVS | 1 | 0/1 | 0/x (0 inferred) | 1* |
| lossless in CLVS** | 1 | x (0 inferred) | 1 | 1 |
| Mixed LL lossy Slices | 1 | 1 | x (0 inferred) | 1 |
| Mixed LL lossless Slices | 1 | 0 | 1 | 2 |
| RRC only | 1 | x (0 inferred) | 1 | 1 |

** is lossless CTC

*some experts disagreed.

# Discussion

|  | Number of bits in SH |  |  |  |
| --- | --- | --- | --- | --- |
|  | Combination 1: Method 1 + variation 2 | Combination 2: Method 1 + variation 3 | Combination 3: Method 3 + variation 2 | Combination 4: Method 3 + variation 3 |
| TS disabled | 1 | 1 | 1 | 1 |
| Lossy in CLVS | 1 | 1 | 2 | 1* |
| lossless in CLVS | 1 | 1 | 1 | 1 |
| Mixed LL lossy Slices | 2 | 1 | 2 | 1 |
| Mixed LL lossless Slices | 1 | 2 | 1 | 2 |
| RRC only | 1 | 1 | 1 | 1 |

*some experts disagreed.

Combination 1 and combination 2 require additional sps flag.

For SCC combinations 1 and 2 have 1 SH saving for lossless.

# References

A. Nalci, M. Coban, M. Karczewicz, “AHG9/AHG14: High-level constraints of dependent quantization and sign data hiding”, JVET-R0271

S.-T. Hsiang, C.-W. Hsu, Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei. “ AHG9: HLS on disabling TSRC”, JVET-R0049

# Patent rights declaration(s)

Alibaba Group, Qualcomm Inc., MediaTek Inc, Kwai Inc., LGE, Bytedance Inc., InterDigital, Sharp, Canon Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).