| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R2001-v1 |
| --- | --- |

| Title: | Supplemental enhancement information for coded video bitstreams (Draft 4) |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Draft text |  |  |
| Author(s) or Contact(s): | Jill Boyce Virginie Drugeon Gary Sullivan Ye-Kui Wang | Email: | jill.boyce@intel.com virginie.drugeon@eu.panasonic.com garysull@microsoft.com yekui.wang@bytedance.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This document is Draft 4 of a standard that specifies the syntax and semantics of video usability information (VUI) parameters and supplemental enhancement information (SEI) messages for use with coded video bitstreams. The VUI parameters and SEI messages defined in this draft standard may be conveyed within coded video bitstreams in a manner specified in a video coding specification or may be conveyed by other means as determined by the specifications for systems that make use of such coded video bitstreams. This draft standard is particularly intended for use with coded video bitstreams as specified by Rec. ITU-T H.VVC | ISO/IEC 23090-3, although it is drafted in a manner intended to be sufficiently generic that it may also be used with other types of coded video bitstreams.

Editors' Notes:

Draft 4 changes:

- Allow the value 6 for chroma sample location type indicators when present.

- Add a byte to the decoded picture hash SEI message with a single_component_flag followed by 7 reserved zero bits. The width and height of each component are defined externally.

- When used with a RWP SEI message, the generalized cubemap message shall have the same functionality as the "traditional" cubemap message (define by syntax element constraints).

- Change the coding of display_elemental_periods_minus1 from ue(v) to u(4).

- JVET-R0090: Move the two flags about GeneralProgressiveSourceFlag and GeneralInterlacedSourceFlag into the VUI specification.

Draft 3:

- Incorporated JVET-Q0042: Move overscan info just after aspect ratio, infer the chroma sample location type as unspecified when not present, and use the general source scan type flags to determine the presence of the chroma location type identifier(s).

- Incorporated JVET-Q0343: Updates to generalized cubemap projection SEI message

- Migration of filler payload SEI message from VVC spec

- Incorporated JVET-Q0818: Migration of field frame SEI message from VVC spec

- Added ue(v) and se(v), since these are used in the syntax of some SEI messages

- Removed the definition and use of picture order count, since it is not really needed

- Renamed the referenced parameter sets SEI message to the parameter sets inclusion indication SEI message for improved clarity (since this SEI message provides only an indication rather than a copy of the parameter sets).

- Renamed the picture component array variable of the decoded picture hash SEI message to follow the variable naming convention (starting with an uppercase letter and including at least one more uppercase letter to avoid appearing to be an ordinary word).

- Fixed bugs #764 and #927.

Draft 2:

- Reorganized the document structure such that the syntax and semantics for each particular SEI message go together.

- Acted upon USNB comment 1 to move VUI to this Specification except for field_seq_flag.

- Incorporated JVET-P0359: Add referenced parameter sets SEI message.

- Incorporated JVET-P0337: Add the following 9 SEI messages from AVC and HEVC:

- User data registered by Recommendation ITU-T.35 SEI message

- User data unregistered SEI message

- Film grain characteristics SEI message

- Frame packing arrangement SEI message

- Mastering display colour volume SEI message

- Content light level information SEI message

- Alternative transfer characteristic information SEI message

- Ambient viewing environment SEI message

- Content colour volume SEI message

- Integrated JVET-P0450: Sample aspect ratio information SEI message.

- Incorporated JVET-P0462: Add the following 4 SEI messages from AVC and HEVC:

- Equirectangular projection SEI message

- Sphere rotation SEI message

- Region-wise packing SEI message

- Omnidirectional viewport SEI message

- Integrated JVET-P0597: Generalized cubemap projection SEI message.

- Editorial

- Added missing normative references and bibliography references, among some other fixes.

- Replaced the figures in the semantics of the frame packing arrangement SEI message.

Draft 1:

- Migration from JVET-N1001-v10 Annex F: Decoded picture hash SEI message and dependent random access point indication SEI message

CONTENTS

Page

Abstract	i

1	Scope	1

2	Normative references	1

2.1	Identical Recommendations | International Standards	1

2.2	Paired Recommendations | International Standards equivalent in technical content	1

2.3	Additional references	1

3	Terms and definitions	2

4	Abbreviations	5

5	Conventions	6

5.1	General	6

5.2	Arithmetic operators	6

5.3	Logical operators	6

5.4	Relational operators	7

5.5	Bit-wise operators	7

5.6	Assignment operators	7

5.7	Range notation	7

5.8	Mathematical functions	8

5.9	Order of operation precedence	9

5.10	Variables, syntax elements and tables	10

5.11	Text description of logical operations	11

5.12	Processes	12

6	Syntax and semantics	13

6.1	Method of specifying syntax in tabular form	13

6.2	Specification of syntax functions and descriptors	14

7	Video usability information parameters	15

7.1	VUI parameters syntax	15

7.2	VUI parameters semantics	15

8	SEI message payload specifications	20

8.1	General	20

8.2	Filler payload SEI message	21

8.2.1	Filler payload SEI message syntax	21

8.2.2	Filler payload SEI message semantics	21

8.3	User data registered by Recommendation ITU-T T.35 SEI message	22

8.3.1	User data registered by Recommendation ITU-T T.35 SEI message syntax	22

8.3.2	User data registered by Recommendation ITU-T T.35 SEI message semantics	22

8.4	User data unregistered SEI message	22

8.4.1	User data unregistered SEI message syntax	22

8.4.2	User data unregistered SEI message semantics	22

8.5	Film grain characteristics SEI message	23

8.5.1	Film grain characteristics SEI message syntax	23

8.5.2	Film grain characteristics SEI message semantics	23

8.6	Frame packing arrangement SEI message	29

8.6.1	Frame packing arrangement SEI message syntax	29

8.6.2	Frame packing arrangement SEI message semantics	29

8.7	Parameter sets inclusion indication SEI message	36

8.7.1	Parameter sets inclusion indication SEI message syntax	36

8.7.2	Parameter sets inclusion indication SEI message semantics	36

8.8	Decoded picture hash SEI message	37

8.8.1	Decoded picture hash SEI message syntax	37

8.8.2	Decoded picture hash SEI message semantics	37

8.9	Mastering display colour volume SEI message	39

8.9.1	Mastering display colour volume SEI message syntax	39

8.9.2	Mastering display colour volume SEI message semantics	39

8.10	Content light level information SEI message	40

8.10.1	Content light level information SEI message syntax	40

8.10.2	Content light level information SEI message semantics	40

8.11	Dependent random access point indication SEI message	41

8.11.1	Dependent random access point indication SEI message syntax	41

8.11.2	Dependent random access point indication SEI message semantics	41

8.12	Alternative transfer characteristics information SEI message	41

8.12.1	Alternative transfer characteristics information SEI message syntax	41

8.12.2	Alternative transfer characteristics SEI message semantics	42

8.13	Ambient viewing environment SEI message	42

8.13.1	Ambient viewing environment SEI message syntax	42

8.13.2	Ambient viewing environment SEI message semantics	42

8.14	Content colour volume SEI message	43

8.14.1	Content colour volume SEI message syntax	43

8.14.2	Content colour volume SEI message semantics	43

8.15	Omnidirectional video specific SEI messages	45

8.15.1	Sample location remapping process	45

8.15.2	Equirectangular projection SEI message	53

8.15.3	Generalized cubemap projection SEI message	55

8.15.4	Sphere rotation SEI message	59

8.15.5	Region-wise packing SEI message	60

8.15.6	Omnidirectional viewport SEI message	66

8.16	Frame-field information SEI message	68

8.16.1	Frame-field information SEI message syntax	68

8.16.2	Frame-field information SEI message semantics	68

8.17	Sample aspect ratio information SEI message	71

8.17.1	Sample aspect ratio information SEI message syntax	71

8.17.2	Sample aspect ratio information SEI message semantics	71

8.18	Reserved SEI message	72

8.18.1	Reserved SEI message syntax	72

8.18.2	Reserved SEI message semantics	72

9	Parsing process for k-th order Exp-Golomb codes	72

9.1	General	72

9.2	Mapping process for signed Exp-Golomb codes	73

INTERNATIONAL  STANDARD

Draft ISO/IEC 23002-7

Draft Rec. ITU-T H.SEI

ITU-T  RECOMMENDATION

# Supplemental enhancement information messages for coded video bitstreams

# Scope

This Recommendation | International Standard specifies the syntax and semantics of video usability information (VUI) parameters and supplemental enhancement information (SEI) messages. The VUI parameters and SEI messages defined in this standard may be conveyed within coded video bitstreams in a manner specified in a video coding specification or may be conveyed by other means determined by the specifications for systems that make use of such coded video bitstreams. This document is particularly intended for use with coded video bitstreams as specified by Rec. ITU-T H.VVC | ISO/IEC 23090-3, although it is drafted in a manner intended to be sufficiently generic that it may also be used with other types of coded video bitstreams.

This document is written in a manner such that it is intended to be referenced by other technical specifications. Such other technical specifications are to be written in a manner to specify certain necessary elements to enable the use of the specified VUI parameters and SEI messages.

VUI parameters and SEI messages can assist in processes related to decoding, display or other purposes. However, unless otherwise specified in a referencing specification, the interpretation and use of the VUI parameters and SEI messages specified in this document is not a required functionality of a video decoder or receiving video system. Although semantics are specified for the VUI parameters and SEI messages, decoders and receiving video systems may simply ignore the content of the and VUI parameters and SEI messages or may use them in some manner not specified in this document.

# Normative references

The following Recommendations and International Standards contain provisions which, through reference in this text, constitute provisions of this Recommendation | International Standard. At the time of publication, the editions indicated were valid. All Recommendations and Standards are subject to revision, and parties to agreements based on this Recommendation | International Standard are encouraged to investigate the possibility of applying the most recent edition of the Recommendations and Standards listed below. Members of IEC and ISO maintain registers of currently valid International Standards. The Telecommunication Standardization Bureau of the ITU maintains a list of currently valid ITU-T Recommendations.

## Identical Recommendations | International Standards

–	None

## Paired Recommendations | International Standards equivalent in technical content

–	Recommendation ITU-T H.273 | ISO/IEC 23091-2 Coding-independent code points for video signal type identification.

## Additional references

–	ISO 11664-1: in force, Colorimetry – Part 1: CIE standard colorimetric observers.

–	ISO/IEC 11578: in force, Information technology – Open Systems Interconnection – Remote Procedure Call (RPC).

–	Recommendation ITU-T T.35 (in force), Procedure for the allocation of ITU-T defined codes for non standard facilities.

–	IETF RFC 1321 (in force), The MD5 Message-Digest Algorithm.

# Terms and definitions

For the purposes of this Recommendation | International Standard, the following definitions apply.

  - access unit (AU): A set of PUs that belong to different layers and contain coded pictures associated with the same output time.

  - adaptation parameter set (APS): A syntax structure containing syntax elements that apply to zero or more slices as determined by zero or more syntax elements found in slice headers.

  - associated IRAP picture (of a particular picture): The previous IRAP picture in decoding order (when present) in the same layer as the particular picture.

  - azimuth circle: A circle on a sphere connecting all points with the same azimuth value.

NOTE – An azimuth circle is always a great circle like a longitude line on the earth.

  - byte-aligned: A position in a coded video bitstream or syntax structure is byte-aligned when the position is an integer multiple of 8 bits from the position of the first bit in the coded video bitstream or syntax structure, and a bit or byte or syntax element is said to be byte-aligned when the position at which it appears in a coded video bitstream or syntax structure is byte-aligned.

  - byte stream: An encapsulation of a NAL unit stream containing start code prefixes and NAL units.

  - chroma: An adjective, represented by the symbols Cb and Cr, specifying that a sample array or single sample is representing one of the two colour difference signals related to the primary colours.

NOTE – The term chroma is used rather than the term chrominance in order to avoid the implication of the use of linear light transfer characteristics that is often associated with the term chrominance.

  - coded layer video sequence (CLVS): A sequence of PUs of the same layer that consists, in decoding order, of a CLVSS PU, followed by zero or more PUs that are not CLVSS PUs, including all subsequent PUs up to but not including any subsequent PU that is a CLVSS PU.

  - coded layer video sequence start (CLVSS) PU: A PU in which the coded picture is a CLVSS picture.

  - coded layer video sequence start (CLVSS) picture: A coded picture that starts a new CLVS as specified in a video coding specification.

NOTE – In Rec. ITU-T H.VVC | ISO/IEC 23090-3, a CLVSS picture is an IRAP picture with NoIncorrectPicOutputFlag equal to 1 or a GDR picture with NoIncorrectPicOutputFlag equal to 1. In Rec. ITU-T H.265 | ISO/IEC 23008-2, a CLVSS picture is an IRAP picture with NoRaslOutputFlag equal to 1.

  - coded picture: A coded representation of a picture containing all CTUs of the picture.

  - coded video bitstream: A sequence of bits that forms the representation of a sequence of AUs forming one or more coded video sequences (CVSs).

  - coded video sequence (CVS): A sequence of AUs that consists, in decoding order, of an IRAP AU, followed by zero or more AUs that are not IRAP AUs, including all subsequent AUs up to but not including any subsequent AU that is an IRAP AU. [Ed. (YK): Note that there is a discrepancy here of the CVS definition compared to what's in HEVC and VVC, where an IRAP AU may or may not start a new CVS, and in VVC a CVS may also start with a GDR AU. It goes more subtle when thinking about muti-layer contexts, particularly when thinking about like in SHVC and VVC, the definitions of IRAP AUs are different. On the other hand, it seems that we are OK without having a CVS definition in this document that is perfect for each codec spec, as so far only CLVS is really needed in the semantics of the VUI parameters and SEI messages included.]

  - component: An array or single sample from one of the three arrays (luma and two chroma) that compose a picture in 4:2:0, 4:2:2, or 4:4:4 colour format or the array or a single sample of the array that compose a picture in monochrome format.

  - constituent picture: A part of a spatially frame-packed stereoscopic picture that corresponds to one view, or a picture itself when frame packing is not in use or the temporal interleaving frame packing arrangement is in use.

  - cropped decoded picture: The result of cropping a decoded picture based on the conformance cropping window for the corresponding coded picture.

  - decoded picture: A decoded picture is derived by decoding a coded picture.

  - decoder: An embodiment of a decoding process.

  - decoding order: The order in which syntax elements are conveyed in the coded video bitstream and are processed by a decoding process.

  - decoding process: The process that reads a coded video bitstream and derives decoded pictures from it.

  - elevation circle: A circle on a sphere connecting all points with the same elevation value.

NOTE – An elevation circle is similar to a lattitude line on the earth. Except when the elevation value is zero, an elevation circle is not a great circle like a longitude circle on the earth.

  - emulation prevention byte: A byte equal to 0x03 that is present within a NAL unit when the syntax elements of the coded video bitstream form certain patterns of byte values in a manner that ensures that no sequence of consecutive byte-aligned bytes in the NAL unit can contain a start code prefix.

  - encoder: An embodiment of an encoding process.

  - encoding process: A process that produces a coded video bitstream.

  - field: An assembly of alternative rows of samples of a frame.

  - flag: A variable or single-bit syntax element that can take one of the two possible values: 0 and 1.

  - frame: The composition of a top field and a bottom field, where sample rows 0, 2, 4, ... originate from the top field and sample rows 1, 3, 5, ... originate from the bottom field.

  - global coordinate axes: The coordinate axes associated with omnidirectional video that are associated with an externally referenceable position and orientation.

NOTE – The global coordinate axes may correspond to the position and orientation of a device or rig used for omnidirectional audio/video acquisition as well as the position of an observer's head in the three-dimensional space of the omnidirectional video rendering environment.

  - great circle: The intersection of a sphere and a plane that passes through the centre point of the sphere.

NOTE – A great circle is also known as an orthodrome or Riemannian circle.

  - informative: A term used to refer to content provided in this Specification that does not establish any mandatory requirements for conformance to this Specification and thus is not considered an integral part of this Specification.

  - intra random access point (IRAP) AU: An AU in which each coded picture is an IRAP picture.

  - intra random access point (IRAP) picture: A coded picture starting from which all pictures in the same layer in both decoding order and output order can be decoded without first decoding any picture in the same layer earlier in decoding order in the coded video bitstream.

  - layer: A set of VCL NAL units that all have a particular value of LayerIdVal and the associated non-VCL NAL units, wherein LayerIdVal is a variable for which the value is specified by a video coding specification.

NOTE – In the contexts of Rec. ITU-T H.VVC | ISO/IEC 23090-3 and Rec. ITU-T H.265 | ISO/IEC 23008-2, LayerIdVal is the value of the nuh_layer_id field in the NAL unit header.

  - leading picture (of an IRAP picture): A picture that is in the same layer as the associated IRAP picture and precedes the associated IRAP picture in output order.

  - local coordinate axes: The coordinate axes having a specified rotation relationship relative to the global coordinate axes.

  - luma: An adjective, represented by the symbol or subscript Y or L, specifying that a sample array or single sample is representing the monochrome signal related to the primary colours.

NOTE – The term luma is used rather than the term luminance in order to avoid the implication of the use of linear light transfer characteristics that is often associated with the term luminance. The symbol L is sometimes used instead of the symbol Y to avoid confusion with the symbol y as used for vertical location.

  - may: A term that is used to refer to behaviour that is allowed, but not necessarily required.

NOTE – In some places where the optional nature of the described behaviour is intended to be emphasized, the phrase "may or may not" is used to provide emphasis.

  - must: A term that is used in expressing an observation about a requirement or an implication of a requirement that is specified elsewhere in this Specification (used exclusively in an informative context).

  - network abstraction layer (NAL) unit: A syntax structure containing an indication of the type of data to follow and bytes containing that data in the form of an RBSP interspersed as necessary with emulation prevention bytes.

  - network abstraction layer (NAL) unit stream: A sequence of NAL units.

  - non-VCL NAL unit: A NAL unit that is not a VCL NAL unit.

  - note: A term that is used to prefix informative remarks (used exclusively in an informative context).

  - omnidirectional video: A video content in a format that enables rendering according to the user's viewing orientation, e.g., if viewed using a head-mounted device, or according to a user's desired viewport, reflecting a potentially rotated viewing position.

  - output order: The order in which the decoded pictures are output from the decoded picture buffer (for the decoded pictures that are to be output from the decoded picture buffer).

  - output time: A time when a decoded picture is to be output from the DPB (for the decoded pictures that are to be output from the DPB).

  - packed region: A region in a region-wise packed picture that is mapped to a projected region according to a region-wise packing.

  - parameter: A syntax element of an adaptation parameter set (APS), picture parameter set (PPS), or video usability information (VUI) syntax structure.

  - partitioning: The division of a set into subsets such that each element of the set is in exactly one of the subsets.

  - picture: An array of luma samples in monochrome format or an array of luma samples and two corresponding arrays of chroma samples in 4:2:0, 4:2:2, and 4:4:4 colour format.

NOTE – A picture may be either a frame or a field. However, in one CLVS, either all pictures are frames or all pictures are fields.

  - picture parameter set (PPS): A syntax structure containing syntax elements that apply to zero or more entire coded pictures as determined by a syntax element that is the same for all slices of a picture and found in the picture header or slice headers of each picture.

  - picture unit (PU): A set of NAL units that contain all VCL NAL units of a coded picture and their associated non-VCL NAL units.

  - profile: A specified subset of the syntax of this Specification.

  - projected picture: A picture that uses a projection format for omnidirectional video.

  - projected region: A region in a projected picture that is mapped to a packed region according to a region-wise packing.

  - projection: A specified correspondence between the colour samples of a projected picture and azimuth and elevation positions on a sphere.

  - random access: The act of starting the decoding process for a coded video bitstream at a point other than the beginning of the stream.

  - random access decodable leading (RADL) picture: A leading picture that can be correctly decoded when the decoding process starts from the associated IRAP picture.

  - random access skipped leading (RASL) picture: A leading picture that cannot be correctly decoded when the decoding process starts from the associated IRAP picture.

  - raw byte sequence payload (RBSP): A syntax structure containing an integer number of bytes that is encapsulated in a NAL unit and that is either empty or has the form of a string of data bits containing syntax elements followed by an RBSP stop bit and zero or more subsequent bits equal to 0.

  - raw byte sequence payload (RBSP) stop bit: A bit equal to 1 present within a raw byte sequence payload (RBSP) after a string of data bits, for which the location of the end within an RBSP can be identified by searching from the end of the RBSP for the RBSP stop bit, which is the last non-zero bit in the RBSP.

  - reference picture: A picture that is a short-term reference picture.

NOTE – A reference picture contains samples that may be used for inter prediction in the decoding process of subsequent pictures in decoding order.

  - reference picture list: A list of reference pictures that is used for inter prediction of a P or B slice.

NOTE – For the decoding process of a P slice, there is one reference picture list – reference picture list 0. For the decoding process of a B slice, there are two reference picture lists – reference picture list 0 and reference picture list 1.

  - region-wise packed picture: A decoded picture that contains one or more packed regions.

NOTE – A region-wise packed picture may contain a region-wise packing of a projected picture.

  - region-wise packing: A transformation, resizing, and relocation of packed regions of a region-wise packed picture to remap the packed regions to projected regions of a projected picture.

  - reserved: A term that may be used to specify that some values of a particular syntax element are for future use by ITU-T | ISO/IEC and shall not be used in syntax structures conforming to this version of this Specification, but may be used in syntax structures conforming to future extensions of this Specification by ITU-T | ISO/IEC.

  - shall: A term used to express mandatory requirements for conformance to this Specification.

NOTE – When used to express a mandatory constraint on the values of syntax elements, it is the responsibility of the encoder to ensure that the constraint is fulfilled.

  - short-term reference picture: A picture that is marked as "used for short-term reference".

  - should: A term used to refer to behaviour of an implementation that is encouraged to be followed under anticipated ordinary circumstances, but is not a mandatory requirement for conformance to this Specification.

  - source: A term used to describe the video material or some of its attributes before encoding.

  - sphere coordinates: The azimuth and elevation angles identifying a location of a point on a sphere.

  - sphere region: A region on a sphere, specified either by four great circles or by two azimuth circles and two elevation circles, or such a region on a rotated sphere after applying yaw, pitch, and roll rotations.

  - start code prefix: A unique sequence of three bytes equal to 0x000001 embedded in the byte stream as a prefix to each NAL unit.

NOTE – The location of a start code prefix can be used by a decoder to identify the beginning of a new NAL unit and the end of a previous NAL unit. Emulation of start code prefixes is prevented within NAL units by the inclusion of emulation prevention bytes.

  - step-wise temporal sublayer access (STSA) picture: A coded picture that enables up-switching, at coded picture, to the sublayer containing the coded picture, from the immediately lower sublayer.

NOTE – An STSA picture does not use pictures with the same TemporalId as the STSA picture for inter prediction reference. Pictures following an STSA picture in decoding order with the same TemporalId as the STSA picture do not use pictures prior to the STSA picture in decoding order with the same TemporalId as the STSA picture for inter prediction reference. STSA pictures must have TemporalId greater than 0.

  - string of data bits (SODB): A sequence of some number of bits representing syntax elements present within a raw byte sequence payload prior to the raw byte sequence payload stop bit, where the left-most bit is considered to be the first and most significant bit, and the right-most bit is considered to be the last and least significant bit.

  - sublayer: A temporal scalable layer of a temporal scalable bitstream, consisting of VCL NAL units with a particular value of TemporalId and the associated non-VCL NAL units, wherein TemporalId is a variable for which the value is specified by a video coding specification.

  - syntax element: An element of data represented in a syntax structure.

  - syntax structure: Zero or more syntax elements that are present together in a specified order.

  - temporal sublayer identifier: A number greater than or equal to 0, denoted herein by TemporalId, such that pictures of all temporal sublayers have a specified temporal output order relative to each other and pictures with a lower temporal identifier can be decoded without reference to pictures with a higher temporal identifier.

  - tilt angle: The angle indicating the amount of tilt of a sphere region, measured as the amount of rotation of a sphere region along the axis originating from the sphere origin passing through the centre point of the sphere region, where the angle value increases clockwise when looking from the origin towards the positive end of the axis.

  - trailing picture: A coded picture that follows an IRAP picture in both decoding order and output order.

  - unspecified: A term that may be used to specify some values of a particular syntax element to indicate that the values have no specified meaning in this Specification and will not have a specified meaning in the future as an integral part of future versions of this Specification.

  - video coding layer (VCL) NAL unit: A collective term for coded slice NAL units and the subset of NAL units that have reserved values of nal_unit_type that are classified as VCL NAL units in this Specification.

  - video usability information (VUI) parameters: A syntax structure that identifies properties of interpretation of decoded pictures for display purposes, particularly including colour representation information.

  - viewport: A region of omnidirectional video content suitable for display and viewing by the user.

# Abbreviations

AU	Access Unit

APS	Adaptation Parameter Set

CRC	Cyclic Redundancy Check

CLVS	Coded Layer Video Sequence

CLVSS	Coded Layer Video Sequence Start

CVS	Coded Video Sequence

DRAP	Dependent Random Access Point

FIR	Finite Impulse Response

IDR	Instantaneous Decoding Refresh

IRAP	Intra Random Access Point

NAL	Network Abstraction Layer

PPS	Picture Parameter Set

PU	Picture Unit

RADL	Random Access Decodable Leading (Picture)

RASL	Random Access Skipped Leading (Picture)

RPS	Reference Picture Set

SEI	Supplemental Enhancement Information

STSA	Step-wise Temporal Sublayer Access

VCL	Video Coding Layer

VPS	Video Parameter Set

VUI	Video Usability Information

# Conventions

## General

NOTE – The mathematical operators used in this Specification are similar to those used in the C programming language. However, the results of integer division and arithmetic shift operations are defined more precisely, and additional operations are defined, such as exponentiation and real-valued division. Numbering and counting conventions generally begin from 0, e.g., "the first" is equivalent to the 0-th, "the second" is equivalent to the 1-th, etc.

## Arithmetic operators

The following arithmetic operators are defined as follows:

| + | Addition |
| --- | --- |
| − | Subtraction (as a two-argument operator) or negation (as a unary prefix operator) |
| * | Multiplication, including matrix multiplication |
| xy | Exponentiation. Specifies x to the power of y. In other contexts, such notation is used for superscripting not intended for interpretation as exponentiation. |
| / | Integer division with truncation of the result toward zero. For example, 7 / 4 and −7 / −4 are truncated to 1 and −7 / 4 and 7 / −4 are truncated to −1. |
| ÷ | Used to denote division in mathematical equations where no truncation or rounding is intended. |
| xy | Used to denote division in mathematical equations where no truncation or rounding is intended. |
| i = xyf( i ) | The summation of f( i ) with i taking all integer values from x up to and including y. |
| x % y | Modulus. Remainder of x divided by y, defined only for integers x and y with x >= 0 and y > 0. |

## Logical operators

The following logical operators are defined as follows:

x  &&  y	Boolean logical "and" of x and y

x  | |  y	Boolean logical "or" of x and y

!	Boolean logical "not"

x ? y : z	If x is TRUE or not equal to 0, evaluates to the value of y; otherwise, evaluates to the value of z.

## Relational operators

The following relational operators are defined as follows:

>	Greater than

>=	Greater than or equal to

<	Less than

<=	Less than or equal to

= =	Equal to

!=	Not equal to

When a relational operator is applied to a syntax element or variable that has been assigned the value "na" (not applicable), the value "na" is treated as a distinct value for the syntax element or variable. The value "na" is considered not to be equal to any other value.

## Bit-wise operators

The following bit-wise operators are defined as follows:

&	Bit-wise "and". When operating on integer arguments, operates on a two's complement representation of the integer value. When operating on a binary argument that contains fewer bits than another argument, the shorter argument is extended by adding more significant bits equal to 0.

|	Bit-wise "or". When operating on integer arguments, operates on a two's complement representation of the integer value. When operating on a binary argument that contains fewer bits than another argument, the shorter argument is extended by adding more significant bits equal to 0.

^	Bit-wise "exclusive or". When operating on integer arguments, operates on a two's complement representation of the integer value. When operating on a binary argument that contains fewer bits than another argument, the shorter argument is extended by adding more significant bits equal to 0.

x  >>  y	Arithmetic right shift of a two's complement integer representation of x by y binary digits. This function is defined only for non-negative integer values of y. Bits shifted into the most significant bits (MSBs) as a result of the right shift have a value equal to the MSB of x prior to the shift operation.

x  <<  y	Arithmetic left shift of a two's complement integer representation of x by y binary digits. This function is defined only for non-negative integer values of y. Bits shifted into the least significant bits (LSBs) as a result of the left shift have a value equal to 0.

## Assignment operators

The following arithmetic operators are defined as follows:

=	Assignment operator

+ +	Increment, i.e., x+ + is equivalent to x = x + 1; when used in an array index, evaluates to the value of the variable prior to the increment operation.

− −	Decrement, i.e., x− − is equivalent to x = x − 1; when used in an array index, evaluates to the value of the variable prior to the decrement operation.

+=	Increment by amount specified, i.e., x  +=  3 is equivalent to x = x + 3, and x  +=  (−3) is equivalent to x = x + (−3).

−=	Decrement by amount specified, i.e., x  −=  3 is equivalent to x = x − 3, and x  −=  (−3) is equivalent to x = x − (−3).

## Range notation

The following notation is used to specify a range of values:

x = y..z	x takes on integer values starting from y to z, inclusive, with x, y, and z being integer numbers and z being greater than y.

## Mathematical functions

The following mathematical functions are defined:

Abs( x ) =  x;x  >=  0-x;x < 0		(1)

Asin( x )	the trigonometric inverse sine function, operating on an argument x that is
in the range of −1.0 to 1.0, inclusive, with an output value in the range of 
−π÷2 to π÷2, inclusive, in units of radians		(2)

Atan( x )	the trigonometric inverse tangent function, operating on an argument x, with
an output value in the range of −π÷2 to π÷2, inclusive, in units of radians	(3)

Atan2( y, x ) =  Atan yx ;x > 0Atan yx +πAtan yx -π;;x < 0  &&  y  >=  0x < 0  &&  y  <  0+π2-π2;;x = =  0  &&  y  >=  0otherwise	(4)

Ceil( x )	the smallest integer greater than or equal to x.	(5)

Clip1Y( x ) = Clip3( 0, ( 1  <<  BitDepthY ) − 1, x )		(6)

Clip1C( x ) = Clip3( 0, ( 1  <<  BitDepthC ) − 1, x )		(7)

Clip3( x, y, z ) =  x;z < xy;z > yz;otherwise		(8)

Cos( x )	the trigonometric cosine function operating on an argument x in units of radians.	(9)

Floor( x )	the largest integer less than or equal to x.		(10)

GetCurrMsb( a, b, c, d ) =  c+d;b-a  >=  d / 2  c-d;a-b  >  d / 2c;otherwise	(11)

Ln( x )	the natural logarithm of x (the base-e logarithm, where e is the natural logarithm base constant 2.718 281 828...).		(12)

Log2( x )	the base-2 logarithm of x.		(13)

Log10( x ) the base-10 logarithm of x.		(14)

Min( x, y ) =   x;x <= yy;x > y		(15)

Max( x, y ) =   x;x >= yy;x < y		(16)

Round( x ) = Sign( x ) * Floor( Abs( x ) + 0.5 )		(17)

Sign( x ) = 1;x > 00;x  = =  0-1;x  <  0		(18)

Sin( x )	the trigonometric sine function operating on an argument x in units of radians	(19)

Sqrt( x )	the square root of x		(20)

Swap( x, y ) equivalent to setting a temporary variable t equal to x, then x = y and y = t	(21)

Tan( x )	the trigonometric tangent function operating on an argument x in units of radians	(22)

## Order of operation precedence

When order of precedence in an expression is not indicated explicitly by use of parentheses, the following rules apply:

–	Operations of a higher precedence are evaluated before any operation of a lower precedence.

–	Operations of the same precedence are evaluated sequentially from left to right.

Table 1 specifies the precedence of operations from highest to lowest; a higher position in the table indicates a higher precedence.

NOTE – For those operators that are also used in the C programming language, the order of precedence used in this Specification is the same as used in the C programming language.

Table 1 – Operation precedence from highest (at top of table) to lowest (at bottom of table)

operations (with operands x, y, and z)

"x++", "x− −"

"!x", "−x" (as a unary prefix operator)

xy

"x * y", "x / y", "x ÷ y", "xy", "x % y"

"x + y", "x − y" (as a two-argument operator), ""

"x << y", "x >> y"

"x < y", "x <= y", "x > y", "x >= y"

"x = = y", "x != y"

"x & y"

"x ¦ y"

"x && y"

"x ¦ ¦ y"

"x ? y : z"

"x..y"

"x = y", "x += y", "x −= y"

## Variables, syntax elements and tables

Syntax elements in the syntax tables are represented in bold type. Each syntax element is described by its name (all lower case letters with underscore characters), and one descriptor for its method of coded representation. The decoding process behaves according to the value of the syntax element and to the values of previously decoded syntax elements. When a value of a syntax element is used in the syntax tables or the text, it appears in regular (i.e., not bold) type.

In some cases the syntax tables may use the values of other variables derived from syntax elements values. Such variables appear in the syntax tables, or text, named by a mixture of lower case and upper case letter and without any underscore characters. Variables starting with an upper case letter are derived for the decoding of the current syntax structure and all depending syntax structures. Variables starting with an upper case letter may be used in the decoding process for later syntax structures without mentioning the originating syntax structure of the variable. Variables starting with a lower case letter are only used within the clause in which they are derived.

In some cases, "mnemonic" names for syntax element values or variable values are used interchangeably with their numerical values. Sometimes "mnemonic" names are used without any associated numerical values. The association of values and names is specified in the text. The names are constructed from one or more groups of letters separated by an underscore character. Each group starts with an upper case letter and may contain more upper case letters.

NOTE – The syntax is described in a manner that closely follows the C-language syntactic constructs.

Functions that specify properties of the current position in the SEI message payload data are referred to as syntax functions. These functions are specified in clause 6.2 and assume the existence of a pointer with an indication of the position of the next bit to be read by the decoding process from the payload data. Syntax functions are described by their names, which are constructed as syntax element names and end with left and right round parentheses including zero or more variable names (for definition) or values (for usage), separated by commas (if more than one variable).

Functions that are not syntax functions (including mathematical functions specified in clause 5.8) are described by their names, which start with an upper case letter, contain a mixture of lower and upper case letters without any underscore character, and end with left and right parentheses including zero or more variable names (for definition) or values (for usage) separated by commas (if more than one variable).

A one-dimensional array is referred to as a list. A two-dimensional array is referred to as a matrix. Arrays can either be syntax elements or variables. Subscripts or square parentheses are used for the indexing of arrays. In reference to a visual depiction of a matrix, the first subscript is used as a row (vertical) index and the second subscript is used as a column (horizontal) index. The indexing order is reversed when using square parentheses rather than subscripts for indexing. Thus, an element of a matrix s at horizontal position x and vertical position y may be denoted either as s[ x ][ y ] or as syx. A single column of a matrix may be referred to as a list and denoted by omission of the row index. Thus, the column of a matrix s at horizontal position x may be referred to as the list s[ x ].

A specification of values of the entries in rows and columns of an array may be denoted by { {...} {...} }, where each inner pair of brackets specifies the values of the elements within a row in increasing column order and the rows are ordered in increasing row order. Thus, setting a matrix s equal to { { 1  6 } { 4 9 }} specifies that s[ 0 ][ 0 ] is set equal to 1, s[ 1 ][ 0 ] is set equal to 6, s[ 0 ][ 1 ] is set equal to 4, and s[ 1 ][ 1 ] is set equal to 9.

Binary notation is indicated by enclosing the string of bit values by single quote marks. For example, '01000001' represents an eight-bit string having only its second and its last bits (counted from the most to the least significant bit) equal to 1.

Hexadecimal notation, indicated by prefixing the hexadecimal number by "0x", may be used instead of binary notation when the number of bits is an integer multiple of 4. For example, 0x41 represents an eight-bit string having only its second and its last bits (counted from the most to the least significant bit) equal to 1.

Numerical values not enclosed in single quotes and not prefixed by "0x" are decimal values.

A value equal to 0 represents a FALSE condition in a test statement. The value TRUE is represented by any value different from zero.

## Text description of logical operations

In the text, a statement of logical operations as would be described mathematically in the following form:

if( condition 0 )
  statement 0
else if( condition 1 )
  statement 1
...
else /* informative remark on remaining condition */
  statement n

may be described in the following manner:

... as follows / ... the following applies:

–	If condition 0, statement 0

–	Otherwise, if condition 1, statement 1

–	...

–	Otherwise (informative remark on remaining condition), statement n

Each "If ... Otherwise, if ... Otherwise, ..." statement in the text is introduced with "... as follows" or "... the following applies" immediately followed by "If ... ". The last condition of the "If ... Otherwise, if ... Otherwise, ..." is always an "Otherwise, ...". Interleaved "If ... Otherwise, if ... Otherwise, ..." statements can be identified by matching "... as follows" or "... the following applies" with the ending "Otherwise, ...".

In the text, a statement of logical operations as would be described mathematically in the following form:

if( condition 0a  &&  condition 0b )
  statement 0
else if( condition 1a  | |  condition 1b )
  statement 1
...
else
  statement n

may be described in the following manner:

... as follows / ... the following applies:

–	If all of the following conditions are true, statement 0:

–	condition 0a

–	condition 0b

–	Otherwise, if one or more of the following conditions are true, statement 1:

–	condition 1a

–	condition 1b

–	...

–	Otherwise, statement n

In the text, a statement of logical operations as would be described mathematically in the following form:

if( condition 0 )
  statement 0
if( condition 1 )
  statement 1

may be described in the following manner:

When condition 0, statement 0

When condition 1, statement 1

## Processes

Processes are used to describe the decoding of syntax elements. A process has a separate specification and invoking. All syntax elements and upper case variables that pertain to the current syntax structure and depending syntax structures are available in the process specification and invoking. A process specification may also have a lower case variable explicitly specified as input. Each process specification has explicitly specified an output. The output is a variable that can either be an upper case variable or a lower case variable.

When invoking a process, the assignment of variables is specified as follows:

–	If the variables at the invoking and the process specification do not have the same name, the variables are explicitly assigned to lower case input or output variables of the process specification.

–	Otherwise (the variables at the invoking and the process specification have the same name), assignment is implied.

# Syntax and semantics

## Method of specifying syntax in tabular form

Technical specifications that reference this document shall specify some means to carry the payload syntax of each specified SEI message, to identify which SEI message is conveyed, and to identify the length in bits of the SEI message payload data. The length of the SEI message payload data is referred to herein by the variable name payloadSize.

For example, each SEI message may be carried as a string of data bits (SODB) that is prefixed with an SEI message payload type indication derived as a payloadType variable within a NAL unit that may contain emulation prevention bytes as specified in Rec. ITU-T H.VVC | ISO/IEC 23090-3.

The syntax tables in this document specify a superset of the syntax of all allowed SEI message payloads. Additional constraints on the syntax may be specified, either directly or indirectly, in other clauses.

The following table lists examples of the syntax specification format. When syntax_element appears, it specifies that a syntax element is parsed from the SEI message payload data and the data pointer is advanced to the next position beyond the syntax element in the syntax parsing process.

|  | Descriptor |
| --- | --- |
| /* A statement can be a syntax element with an associated descriptor or can be an expression used to specify conditions for the existence, type and quantity of syntax elements, as in the following two examples */ |  |
| syntax_element | ue(v) |
| conditioning statement |  |
| /* A group of statements enclosed in curly brackets is a compound statement and is treated functionally as a single statement. */ |  |
| { |  |
| statement |  |
| statement |  |
| ... |  |
| } |  |
| /* A "while" structure specifies a test of whether a condition is true, and if true, specifies evaluation of a statement (or compound statement) repeatedly until the condition is no longer true */ |  |
| while( condition ) |  |
| statement |  |
| /* A "do ... while" structure specifies evaluation of a statement once, followed by a test of whether a condition is true, and if true, specifies repeated evaluation of the statement until the condition is no longer true */ |  |
| do |  |
| statement |  |
| while( condition ) |  |
| /* An "if ... else" structure specifies a test of whether a condition is true and, if the condition is true, specifies evaluation of a primary statement, otherwise, specifies evaluation of an alternative statement. The "else" part of the structure and the associated alternative statement is omitted if no alternative statement evaluation is needed */ |  |
| if( condition ) |  |
| primary statement |  |
| else |  |
| alternative statement |  |
| /* A "for" structure specifies evaluation of an initial statement, followed by a test of a condition, and if the condition is true, specifies repeated evaluation of a primary statement followed by a subsequent statement until the condition is no longer true. */ |  |
| for( initial statement; condition; subsequent statement ) |  |
| primary statement |  |

## Specification of syntax functions and descriptors

The functions presented here are used in the syntactical description. These functions are expressed in terms of the value of an SEI message payload data pointer that indicates the position of the next bit to be read by the decoding process from the payload data.

byte_aligned( ) is specified as follows:

–	If the current position in the payload data is on a byte boundary, i.e., the next bit in the payload data is the first bit in a byte, the return value of byte_aligned( ) is equal to TRUE.

–	Otherwise, the return value of byte_aligned( ) is equal to FALSE.

more_data_in_payload( ) is specified as follows:

–	If byte_aligned( ) is equal to TRUE and the current position in the sei_payload( ) syntax structure is 8 * payloadSize bits from the beginning of the sei_payload( ) syntax structure, the return value of more_data_in_payload( ) is equal to FALSE.

–	Otherwise, the return value of more_data_in_payload( ) is equal to TRUE.

more_rbsp_data( ) is specified as follows:

–	If there is no more data in the raw byte sequence payload (RBSP), the return value of more_rbsp_data( ) is equal to FALSE.

–	Otherwise, the RBSP data are searched for the last (least significant, right-most) bit equal to 1 that is present in the RBSP. Given the position of this bit, which is the first bit (rbsp_stop_one_bit) of the rbsp_trailing_bits( ) syntax structure, the following applies:

–	If there is more data in an RBSP before the rbsp_trailing_bits( ) syntax structure, the return value of more_rbsp_data( ) is equal to TRUE.

–	Otherwise, the return value of more_rbsp_data( ) is equal to FALSE.

The method for enabling determination of whether there is more data in the RBSP is specified by the application.

more_rbsp_trailing_data( ) is specified as follows:

–	If there is more data in an RBSP, the return value of more_rbsp_trailing_data( ) is equal to TRUE.

–	Otherwise, the return value of more_rbsp_trailing_data( ) is equal to FALSE.

read_bits( n ) reads the next n bits from the payload data and advances the data pointer by n bit positions. When n is equal to 0, read_bits( n ) is specified to return a value equal to 0 and to not advance the data pointer.

The following descriptors specify the parsing process of each syntax element:

–	b(8): byte having any pattern of bit string (8 bits). The parsing process for this descriptor is specified by the return value of the function read_bits( 8 ).

–	f(n): fixed-pattern bit string using n bits written (from left to right) with the left bit first. The parsing process for this descriptor is specified by the return value of the function read_bits( n ).

–	i(n): signed integer using n bits. When n is "v" in the syntax table, the number of bits varies in a manner dependent on the value of other syntax elements. The parsing process for this descriptor is specified by the return value of the function read_bits( n ) interpreted as a two's complement integer representation with most significant bit written first.

–	se(v): signed integer 0-th order Exp-Golomb-coded syntax element with the left bit first. The parsing process for this descriptor is specified in clause 9 with the order k equal to 0.

–	u(n): unsigned integer using n bits. When n is "v" in the syntax table, the number of bits varies in a manner dependent on the value of other syntax elements. The parsing process for this descriptor is specified by the return value of the function read_bits( n ) interpreted as a binary representation of an unsigned integer with most significant bit written first.

–	ue(v): unsigned integer 0-th order Exp-Golomb-coded syntax element with the left bit first. The parsing process for this descriptor is specified in clause 9 with the order k equal to 0.

# Video usability information parameters

## VUI parameters syntax

| vui_parameters( ) { | Descriptor |
| --- | --- |
| vui_general_progressive_source_flag | u(1) |
| vui_general_interlaced_source_flag | u(1) |
| vui_aspect_ratio_info_present_flag | u(1) |
| if( vui_aspect_ratio_info_present_flag ) { |  |
| vui_aspect_ratio_constant_flag | u(1) |
| vui_aspect_ratio_idc | u(8) |
| if( vui_aspect_ratio_idc = = 255 ) { |  |
| vui_sar_width | u(16) |
| vui_sar_height | u(16) |
| } |  |
| } |  |
| vui_overscan_info_present_flag | u(1) |
| if( vui_overscan_info_present_flag ) |  |
| vui_overscan_appropriate_flag | u(1) |
| vui_colour_description_present_flag | u(1) |
| if( vui_colour_description_present_flag ) { |  |
| vui_colour_primaries | u(8) |
| vui_transfer_characteristics | u(8) |
| vui_matrix_coeffs | u(8) |
| vui_full_range_flag | u(1) |
| } |  |
| vui_chroma_loc_info_present_flag | u(1) |
| if( vui_chroma_loc_info_present_flag ) { |  |
| if( vui_general_progressive_source_flag && !vui_general_interlaced_source_flag ) |  |
| vui_chroma_sample_loc_type_frame | ue(v) |
| else { |  |
| vui_chroma_sample_loc_type_top_field | ue(v) |
| vui_chroma_sample_loc_type_bottom_field | ue(v) |
| } |  |
| } |  |
| } |  |

## VUI parameters semantics

VUI parameters apply to one or more CLVSs.

Use of the VUI parameters requires the definition of the following variables:

–	A chroma format indicator, denoted herein by ChromaFormatIdc, such that the value 0 indicates that the picture has only a luma component and other values indicate that the picture has three colour components that consist of a luma component and two associated chroma components, such that the width and height of each chroma component are are the width and height of the luma component divided by SubWidthC and SubHeightC, respectively, where SubWidthC and SubHeightC are determined from ChromaFormatIdc as specified by Table 2.

–	A bit depth for the samples of the luma component, denoted herein by BitDepthY, and when ChromaFormatIdc is not equal to 0, a bit depth for the samples of the two associated chroma components, denoted herein by BitDepthC.

Table 2 – SubWidthC and SubHeightC values derived from
ChromaFormatIdc

| ChromaFormatIdc | Chroma format | SubWidthC | SubHeightC |
| --- | --- | --- | --- |
| 0 | Monochrome | 1 | 1 |
| 1 | 4:2:0 | 2 | 2 |
| 2 | 4:2:2 | 2 | 1 |
| 3 | 4:4:4 | 1 | 1 |

vui_general_progressive_source_flag and vui_general_interlaced_source_flag are interpreted as follows:

–	If vui_general_progressive_source_flag is equal to 1 and vui_general_interlaced_source_flag is equal to 0, the source scan type of the pictures should be interpreted as progressive only.

–	Otherwise, if vui_general_progressive_source_flag is equal to 0 and vui_general_interlaced_source_flag is equal to 1, the source scan type of the pictures should be interpreted as interlaced only.

–	Otherwise, if vui_general_progressive_source_flag is equal to 0 and vui_general_interlaced_source_flag is equal to 0, the source scan type of the pictures should be interpreted as unknown or unspecified.

–	Otherwise (vui_general_progressive_source_flag is equal to 1 and vui_general_interlaced_source_flag is equal to 1), the source scan type of each picture is indicated at the picture level using the syntax element source_scan_type in a frame-field information SEI message.

vui_aspect_ratio_info_present_flag equal to 1 specifies that vui_aspect_ratio_idc is present. vui_aspect_ratio_info_present_flag equal to 0 specifies that vui_aspect_ratio_idc is not present.

vui_aspect_ratio_constant_flag equal to 1 specifies that the values of vui_aspect_ratio_idc, SarWidth, and SarHeight are constant in the CLVS and there is no sample aspect ratio information SEI message present in the CLVS. vui_aspect_ratio_constant_flag equal to 0 specifies that the values of vui_aspect_ratio_idc, SarWidth, and SarHeight may not be constant in the CLVS and that sample aspect ratio information SEI messages may be present in the CLVS indicating a different sample aspect ratio applicable to the pictures associated to the sample aspect ratio information SEI message.

vui_aspect_ratio_idc, when not equal to 255, indicates the sample aspect ratio of the luma samples. Its semantics are as specified for SampleAspectRatio parameter in Rec. ITU-T H.273 | ISO/IEC 23091-2. When the vui_aspect_ratio_idc syntax element is not present, the value of vui_aspect_ratio_idc is inferred to be equal to 0. Values of vui_aspect_ratio_idc that are specified as reserved for future use in Rec. ITU-T H.273 | ISO/IEC 23091-2 shall not be present in bitstreams conforming to this version of this Specification. Decoders shall interpret values of vui_aspect_ratio_idc that are reserved for future use in Rec. ITU-T H.273 | ISO/IEC 23091-2 as equivalent to the value 0. [Ed. (YK): Some clarification is needed herein depending on the value of vui_aspect_ratio_constant_flag. Also for the semantics of vui_sar_width and vui_sar_height.]

vui_sar_width, when present, indicates the horizontal size of the sample aspect ratio (in arbitrary units).

vui_sar_height, when present, indicates the vertical size of the sample aspect ratio (in the same arbitrary units as vui_sar_width).

When present, vui_sar_width and vui_sar_height shall be relatively prime or equal to 0. When vui_aspect_ratio_idc is equal to 0 or vui_sar_width is equal to 0 or vui_sar_height is equal to 0, the sample aspect ratio is unspecified in this Specification.

[Ed. (GJS) There seems to be an issue with the range of values allowed for SarWidth and SarHeight in Rec. ITU-T H.273 | ISO/IEC 23091-2. If that is corrected in the other document, the above semantics can be slightly simplified. Unlike for other things such as ColourPrimaries, the CICP spec does not exactly say what wordlength or range of values would be appropriate for SarWidth and SarHeight, but appears to imply that they cannot be bigger than 255. This is incorrect, since these two parameters are typically sent as 16-bit numbers without value restrictions. Thus, the CICP spec is not referenced for those two.]

vui_overscan_info_present_flag equal to 1 specifies that the vui_overscan_appropriate_flag is present. When vui_overscan_info_present_flag is equal to 0 or is not present, the preferred display method for the video signal is unspecified.

vui_overscan_appropriate_flag equal to 1 indicates that the cropped decoded pictures output are suitable for display using overscan. vui_overscan_appropriate_flag equal to 0 indicates that the cropped decoded pictures output contain visually important information in the entire region out to the edges of the conformance cropping window of the picture, such that the cropped decoded pictures output should not be displayed using overscan. Instead, they should be displayed using either an exact match between the display area and the conformance cropping window, or using underscan. As used in this paragraph, the term "overscan" refers to display processes in which some parts near the borders of the cropped decoded pictures are not visible in the display area. The term "underscan" describes display processes in which the entire cropped decoded pictures are visible in the display area, but they do not cover the entire display area. For display processes that neither use overscan nor underscan, the display area exactly matches the area of the cropped decoded pictures.

NOTE – For example, vui_overscan_appropriate_flag equal to 1 might be used for entertainment television programming or for a live view of people in a videoconference, and vui_overscan_appropriate_flag equal to 0 might be used for computer screen capture or security camera content.

vui_colour_description_present_flag equal to 1 specifies that vui_colour_primaries, vui_transfer_characteristics, and vui_matrix_coeffs are present. vui_colour_description_present_flag equal to 0 specifies that vui_colour_primaries, vui_transfer_characteristics, and vui_matrix_coeffs are not present.

vui_colour_primaries indicates the chromaticity coordinates of the source colour primaries. Its semantics are as specified for the ColourPrimaries parameter in Rec. ITU-T H.273 | ISO/IEC 23091-2. When the vui_colour_primaries syntax element is not present, the value of vui_colour_primaries is inferred to be equal to 2 (the chromaticity is unspecified or is determined by the application). Values of vui_colour_primaries that are identified as reserved for future use in Rec. ITU-T H.273 | ISO/IEC 23091-2 shall not be present in bitstreams conforming to this version of this Specification. Decoders shall interpret reserved values of vui_colour_primaries as equivalent to the value 2.

vui_transfer_characteristics indicates the transfer characteristics function of the colour representation. Its semantics are as specified for the TransferCharacteristics parameter in Rec. ITU-T H.273 | ISO/IEC 23091-2. When the vui_transfer_characteristics syntax element is not present, the value of vui_transfer_characteristics is inferred to be equal to 2 (the transfer characteristics are unspecified or are determined by the application). Values of vui_transfer_characteristics that are identified as reserved for future use in Rec. ITU-T H.273 | ISO/IEC 23091-2 shall not be present in bitstreams conforming to this version of this Specification. Decoders shall interpret reserved values of vui_transfer_characteristics as equivalent to the value 2.

vui_matrix_coeffs describes the formulae used in deriving luma and chroma signals from the green, blue, and red, or Y, Z, and X primaries. Its semantics are as specified for MatrixCoefficients in Rec. ITU-T H.273 | ISO/IEC 23091-2.

vui_matrix_coeffs shall not be equal to 0 unless both of the following conditions are true:

–	BitDepthC is equal to BitDepthY.

–	ChromaFormatIdc is equal to 3 (the 4:4:4 chroma format).

The specification of the use of vui_matrix_coeffs equal to 0 under all other conditions is reserved for future use by ITU-T | ISO/IEC.

vui_matrix_coeffs shall not be equal to 8 unless one of the following conditions is true:

–	BitDepthC is equal to BitDepthY,

–	BitDepthC is equal to BitDepthY + 1 and ChromaFormatIdc is equal to 3 (the 4:4:4 chroma format).

The specification of the use of vui_matrix_coeffs equal to 8 under all other conditions is reserved for future use by ITU-T | ISO/IEC.

When the vui_matrix_coeffs syntax element is not present, the value of vui_matrix_coeffs is inferred to be equal to 2 (unspecified).

vui_full_range_flag indicates the scaling and offset values applied in association with the matrix_coefficients. Its semantics are as specified for the VideoFullRangeFlag parameter in Rec. ITU-T H.273 | ISO/IEC 23091-2. When not present, the value of vui_full_range_flag is inferred to be equal to 0.

vui_chroma_loc_info_present_flag equal to 1 specifies that either vui_chroma_sample_loc_type_frame or both vui_chroma_sample_loc_type_top_field and vui_chroma_sample_loc_type_bottom_field are present. vui_chroma_loc_info_present_flag equal to 0 specifies that vui_chroma_sample_loc_type_frame, vui_chroma_sample_loc_type_top_field, and vui_chroma_sample_loc_type_bottom_field are not present.

When ChromaFormatIdc is not equal to 1, vui_chroma_loc_info_present_flag should be equal to 0.

vui_chroma_sample_loc_type_frame, vui_chroma_sample_loc_type_top_field, and vui_chroma_sample_loc_type_bottom_field, when present, specify the location of chroma samples as follows:

–	If GeneralProgressiveSourceFlag is equal to 1, GeneralInterlacedSourceFlag is equal to 0, and ChromaFormatIdc is equal to 1 (4:2:0 chroma format), vui_chroma_sample_loc_type_frame specifies the location of chroma samples for both fields of each frame of the CLVS as shown in Figure 1.

–	Otherwise, if ChromaFormatIdc is equal to 1 (4:2:0 chroma format), vui_chroma_sample_loc_type_top_field and vui_chroma_sample_loc_type_bottom_field specify the location of chroma samples for each top field and bottom field of the CLVS, respectively, as shown in Figure 1.

–	Otherwise (ChromaFormatIdc is not equal to 1), the values of the syntax elements chroma_sample_loc_type, vui_chroma_sample_loc_type_top_field and vui_chroma_sample_loc_type_bottom_field shall be ignored.

Figure 1 – Location of chroma samples for top and bottom fields for ChromaFormatIdc equal to 1 (4:2:0 chroma format) as a function of vui_chroma_sample_loc_type_top_field and vui_chroma_sample_loc_type_bottom_field in the range of 0 to 5, inclusive

When ChromaFormatIdc is equal to 2 (4:2:2 chroma format), the chroma samples are co-sited with the corresponding luma samples and the nominal locations in a picture are as shown in Figure 2.

Figure 2 – Nominal vertical and horizontal locations of 4:2:2 luma and chroma samples in a picture

When ChromaFormatIdc is equal to 3 (4:4:4 chroma format), all array samples are co-sited for all cases of pictures and the nominal locations in a picture are as shown in Figure 3.

Figure 3 – Nominal vertical and horizontal locations of 4:4:4 luma and chroma samples in a picture

When ChromaFormatIdc is equal to 0, there is no chroma sample array.

When present, the values of vui_chroma_sample_loc_type_frame, vui_chroma_sample_loc_type_top_field and vui_chroma_sample_loc_type_bottom_field shall be in the range of 0 to 6, inclusive.

When ChromaFormatIdc is equal to 1 and vui_chroma_loc_info_present_flag is equal to 0, vui_chroma_sample_loc_type_frame is not present and is inferred to be equal to 6, which indicates that the the location of the chroma samples is unspecified. When vui_chroma_sample_loc_type_top_field and vui_chroma_sample_loc_type_bottom_field are not present, the values of vui_chroma_sample_loc_type_top_field and vui_chroma_sample_loc_type_bottom_field are inferred to be equal to vui_chroma_sample_loc_type_frame.

Figure 4 illustrates the indicated relative position of the top-left chroma sample when ChromaFormatIdc is equal to 1 (4:2:0 chroma format), and vui_chroma_sample_loc_type_top_field or vui_chroma_sample_loc_type_bottom_field is equal to the value of a variable ChromaLocType. The region represented by the top-left 4:2:0 chroma sample (depicted as a large red square with a large red dot at its centre) is shown relative to the region represented by the top-left luma sample (depicted as a small black square with a small black dot at its centre). The regions represented by neighbouring luma samples are depicted as small grey squares with small grey dots at their centres.

Figure 4 – Location of the top-left chroma sample when ChromaFormatIdc is equal to 1
(4:2:0 chroma format) as a function of ChromaLocType

The relative spatial positioning of the chroma samples, as illustrated in Figure 5, can be expressed by defining two variables HorizontalOffsetC and VerticalOffsetC as a function of ChromaFormatIdc and the variable ChromaLocType as given by Table 3, where HorizontalOffsetC is the horizontal (x) position of the centre of the top-left chroma sample relative to the centre of the top-left luma sample in units of luma samples and VerticalOffsetC is the vertical (y) position of the centre of the top-left chroma sample relative to the centre of the top-left luma sample in units of luma samples.

In a typical FIR filter design, when ChromaFormatIdc is equal to 1 (4:2:0 chroma format) or 2 (4:2:2 chroma format), HorizontalOffsetC and VerticalOffsetC would serve as the phase offsets for the horizontal and vertical filter operations, respectively, for separable downsampling from 4:4:4 chroma format to the chroma format indicated by ChromaFormatIdc.

Figure 5 – Location of the top-left chroma sample when ChromaFormatIdc is equal to 1
(4:2:0 chroma format) when ChromaLocType is equal to 1

Table 3 – Definition of HorizontalOffsetC and VerticalOffsetC
as a function of ChromaFormatIdc and ChromaLocType

| ChromaFormatIdc | ChromaLocType | HorizontalOffsetC | VerticalOffsetC |
| --- | --- | --- | --- |
| 1 (4:2:0) | 0 | 0 | 0.5 |
| 1 (4:2:0) | 1 | 0.5 | 0.5 |
| 1 (4:2:0) | 2 | 0 | 0 |
| 1 (4:2:0) | 3 | 0.5 | 0 |
| 1 (4:2:0) | 4 | 0 | 1 |
| 1 (4:2:0) | 5 | 0.5 | 1 |
| 2 (4:2:2) | – | 0 | 0 |
| 3 (4:4:4) | – | 0 | 0 |

When ChromaFormatIdc is equal to 1 (4:2:0 chroma format) and the decoded video content is intended for interpretation according to Rec. ITU-R BT.2020 or Rec. ITU-R BT.2100, vui_chroma_loc_info_present_flag should be equal to 1, and vui_chroma_sample_loc_type_frame, vui_chroma_sample_loc_type_top_field, and vui_chroma_sample_loc_type_bottom_field (as applicable) should be equal to 2.

# SEI message payload specifications

## General

This clause specifies the syntax and semantics for SEI message payloads.

The syntax of the container of the SEI messages as well as the method of identifying which SEI message is outside the scope of this document. The syntax specified in this subclause is for the payload data of each associated SEI message. For SEI messages for which the specified syntax structure is empty, such as the dependent random access point SEI message, there is no payload data and the mere indication that the SEI message is present (e.g., as indicated by a payload type indicator with no payload data) is sufficient to convey the associated information (e.g., by indicating that a set of specified constraints are fulfilled).

The semantics and persistence scope for each SEI message are specified in the semantics specification for each particular SEI message.

NOTE – Persistence information for SEI messages is informatively summarized in Table 4.

Table 4 – Persistence scope of SEI messages (informative)

| SEI message | Persistence scope |
| --- | --- |
| Filler payload | The AU containing the SEI message |
| User data registered by Rec. ITU-T T.35 | Unspecified |
| User data unregistered | Unspecified |
| Film grain characteristics | Specified by the syntax of the SEI message |
| Frame packing arrangement | Specified by the syntax of the SEI message |
| Referenced parameter sets | The CLVS containing the SEI message |
| Decoded picture hash | The PU containing the SEI message |
| Mastering display colour volume | The CLVS containing the SEI message |
| Content light level information | The CLVS containing the SEI message |
| DRAP indication | The AU containing the SEI message |
| Alternative transfer characteristics | The CLVS containing the SEI message |
| Ambient viewing environment | The CLVS containing the SEI message |
| Content colour volume | Specified by the syntax of the SEI message |
| Equirectangular projection | Specified by the syntax of the SEI message |
| Generalized cubemap projection | Specified by the syntax of the SEI message |
| Sphere rotation | Specified by the syntax of the SEI message |
| Region-wise packing | Specified by the syntax of the SEI message |
| Omnidirectional viewport | Specified by the syntax of the SEI message |
| Frame-field information | The AU containing the SEI message |
| Sample aspect ratio information | Specified by the syntax of the SEI message |

In the semantics of a particular SEI message, the phrase "the current layer" in the semantics refer to the layer that the particular SEI message is associated with, the phrase "the current picture" refer to the picture that the particular SEI message is associated with, and the phrase "the current CLVS" or "the CLVS" refers to the CLVS containing the current picture. The association of an SEI message to a layer or a picture is specified in a video coding specification that specifies a coded video bitstream with which the SEI messages may be used.

## Filler payload SEI message

### Filler payload SEI message syntax

| filler_payload( payloadSize ) { | Descriptor |
| --- | --- |
| for( k = 0; k < payloadSize; k++ ) |  |
| ff_byte /* equal to 0xFF */ | f(8) |
| } |  |

### Filler payload SEI message semantics

This SEI message contains a series of payloadSize bytes of value 0xFF, which can be discarded.

ff_byte shall be a byte having the value 0xFF.

## User data registered by Recommendation ITU-T T.35 SEI message

### User data registered by Recommendation ITU-T T.35 SEI message syntax

| user_data_registered_itu_t_t35( payloadSize ) { | Descriptor |
| --- | --- |
| itu_t_t35_country_code | b(8) |
| if( itu_t_t35_country_code != 0xFF ) |  |
| i = 1 |  |
| else { |  |
| itu_t_t35_country_code_extension_byte | b(8) |
| i = 2 |  |
| } |  |
| do { |  |
| itu_t_t35_payload_byte | b(8) |
| i++ |  |
| } while( i < payloadSize ) |  |
| } |  |

### User data registered by Recommendation ITU-T T.35 SEI message semantics

This SEI message contains user data registered as specified in Recommendation ITU-T T.35, the contents of which are not specified in this Specification.

itu_t_t35_country_code shall be a byte having a value specified as a country code by Annex A of Recommendation ITU-T T.35.

itu_t_t35_country_code_extension_byte shall be a byte having a value specified as a country code by Annex B of Recommendation ITU-T T.35.

itu_t_t35_payload_byte shall be a byte containing data registered as specified in Recommendation ITU-T T.35.

The ITU-T T.35 terminal provider code and terminal provider oriented code shall be contained in the first one or more bytes of the itu_t_t35_payload_byte, in the format specified by the Administration that issued the terminal provider code. Any remaining itu_t_t35_payload_byte data shall be data having syntax and semantics as specified by the entity identified by the ITU-T T.35 country code and terminal provider code.

## User data unregistered SEI message

### User data unregistered SEI message syntax

| user_data_unregistered( payloadSize ) { | Descriptor |
| --- | --- |
| uuid_iso_iec_11578 | u(128) |
| for( i = 16; i < payloadSize; i++ ) |  |
| user_data_payload_byte | b(8) |
| } |  |

### User data unregistered SEI message semantics

This SEI message contains unregistered user data identified by a universal unique identifier (UUID), the contents of which are not specified in this Specification.

uuid_iso_iec_11578 shall have a value specified as a UUID according to the procedures of Annex A of ISO/IEC 11578.

user_data_payload_byte shall be a byte containing data having syntax and semantics as specified by the UUID generator.

## Film grain characteristics SEI message

### Film grain characteristics SEI message syntax

| film_grain_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| film_grain_characteristics_cancel_flag | u(1) |
| if( !film_grain_characteristics_cancel_flag ) { |  |
| film_grain_model_id | u(2) |
| separate_colour_description_present_flag | u(1) |
| if( separate_colour_description_present_flag ) { |  |
| film_grain_bit_depth_luma_minus8 | u(3) |
| film_grain_bit_depth_chroma_minus8 | u(3) |
| film_grain_full_range_flag | u(1) |
| film_grain_colour_primaries | u(8) |
| film_grain_transfer_characteristics | u(8) |
| film_grain_matrix_coeffs | u(8) |
| } |  |
| blending_mode_id | u(2) |
| log2_scale_factor | u(4) |
| for( c = 0; c < 3; c++ ) |  |
| comp_model_present_flag[ c ] | u(1) |
| for( c = 0; c < 3; c++ ) |  |
| if( comp_model_present_flag[ c ] ) { |  |
| num_intensity_intervals_minus1[ c ] | u(8) |
| num_model_values_minus1[ c ] | u(3) |
| for( i = 0; i <= num_intensity_intervals_minus1[ c ]; i++ ) { |  |
| intensity_interval_lower_bound[ c ][ i ] | u(8) |
| intensity_interval_upper_bound[ c ][ i ] | u(8) |
| for( j = 0; j <= num_model_values_minus1[ c ]; j++ ) |  |
| comp_model_value[ c ][ i ][ j ] | se(v) |
| } |  |
| } |  |
| film_grain_characteristics_persistence_flag | u(1) |
| } |  |
| } |  |

### Film grain characteristics SEI message semantics

This SEI message provides the decoder with a parameterized model for film grain synthesis.

NOTE 1 – For example, an encoder may use the film grain characteristics SEI message to characterize film grain that was present in the original source video material and was removed by pre-processing filtering techniques. Synthesis of simulated film grain on the decoded images for the display process is optional and does not affect the decoding process specified in this Specification. When synthesis of simulated film grain on the decoded images for the display process is performed, there is no requirement that the method by which the synthesis is performed be the same as the parameterized model for the film grain as provided in the film grain characteristics SEI message.

NOTE 2 – The display process is not specified in this Specification.

NOTE 3 – SMPTE RDD 5 specifies a film grain simulator based on the information provided in the film grain characteristics SEI message.

Use of this SEI message requires the definition of the following variables:

–	A picture width and picture height in units of luma samples, denoted herein by PicWidthInLumaSamples and PicHeightInLumaSamples, respectively.

–	A chroma format indicator, denoted herein by ChromaFormatIdc, as described in clause 7.2.

–	A bit depth for the samples of the luma component, denoted herein by BitDepthY, and when ChromaFormatIdc is not equal to 0, a bit depth for the samples of the two associated chroma components, denoted herein by BitDepthC.

film_grain_characteristics_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous film grain characteristics SEI message in output order that applies to the current layer. film_grain_characteristics_cancel_flag equal to 0 indicates that film grain modelling information follows.

film_grain_model_id identifies the film grain simulation model as specified in Table 5. The value of film_grain_model_id shall be in the range of 0 to 1, inclusive. The values of 2 and 3 for film_grain_model_id are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore film grain characteristic SEI messages with film_grain_model_id equal to 2 or 3.

Table 5 – film_grain_model_id values

| Value | Description |
| --- | --- |
| 0 | Frequency filtering |
| 1 | Auto-regression |

separate_colour_description_present_flag equal to 1 indicates that a distinct colour space description for the film grain characteristics specified in the SEI message is present in the film grain characteristics SEI message syntax. separate_colour_description_present_flag equal to 0 indicates that the colour description for the film grain characteristics specified in the SEI message is the same as for the CLVS as specified in clause 7.2.

NOTE 4 – When separate_colour_description_present_flag is equal to 1, the colour space specified for the film grain characteristics specified in the SEI message may differ from the colour space specified for the CLVS as specified in clause 7.2

film_grain_bit_depth_luma_minus8 plus 8 specifies the bit depth used for the luma component of the film grain characteristics specified in the SEI message. When film_grain_bit_depth_luma_minus8 is not present in the film grain characteristics SEI message, the value of film_grain_bit_depth_luma_minus8 is inferred to be equal to bit_depth_luma_minus8.

The value of filmGrainBitDepth[ 0 ] is derived as follows:

filmGrainBitDepth[ 0 ] = film_grain_bit_depth_luma_minus8 + 8	(23)

film_grain_bit_depth_chroma_minus8 plus 8 specifies the bit depth used for the Cb and Cr components of the film grain characteristics specified in the SEI message. When film_grain_bit_depth_chroma_minus8 is not present in the film grain characteristics SEI message, the value of film_grain_bit_depth_chroma_minus8 is inferred to be equal to bit_depth_chroma_minus8.

The value of filmGrainBitDepth[ c ] for c = 1 and 2 is derived as follows:

filmGrainBitDepth[ c ] = film_grain_bit_depth_chroma_minus8 + 8, with c = 1, 2	(24)

film_grain_full_range_flag has the same semantics as specified in clause 7.2 for the vui_full_range_flag syntax element, except as follows:

–	film_grain_full_range_flag specifies the colour space of the film grain characteristics specified in the SEI message, rather than the colour space used for the CLVS.

–	When film_grain_full_range_flag is not present in the film grain characteristics SEI message, the value of film_grain_full_range_flag is inferred to be equal to vui_full_range_flag.

film_grain_colour_primaries has the same semantics as specified in clause 7.2 for the vui_colour_primaries syntax element, except as follows:

–	film_grain_colour_primaries specifies the colour space of the film grain characteristics specified in the SEI message, rather than the colour space used for the CLVS.

–	When film_grain_colour_primaries is not present in the film grain characteristics SEI message, the value of film_grain_colour_primaries is inferred to be equal to vui_colour_primaries.

film_grain_transfer_characteristics has the same semantics as specified in clause 7.2 for the vui_transfer_characteristics syntax element, except as follows:

–	film_grain_transfer_characteristics specifies the colour space of the film grain characteristics specified in the SEI message, rather than the colour space used for the CLVS.

–	When film_grain_transfer_characteristics is not present in the film grain characteristics SEI message, the value of film_grain_transfer_characteristics is inferred to be equal to vui_transfer_characteristics.

film_grain_matrix_coeffs has the same semantics as specified in clause 7.2 for the vui_matrix_coeffs syntax element, except as follows:

–	film_grain_matrix_coeffs specifies the colour space of the film grain characteristics specified in the SEI message, rather than the colour space used for the CLVS.

–	When film_grain_matrix_coeffs is not present in the film grain characteristics SEI message, the value of film_grain_matrix_coeffs is inferred to be equal to vui_matrix_coeffs.

–	The values allowed for film_grain_matrix_coeffs are not constrained by the value of ChromaFormatIdc.

The ChromaFormatIdc of the film grain characteristics specified in the film grain characteristics SEI message is inferred to be equal to 3 (4:4:4).

NOTE 5 – Because the use of a specific method is not required for performing film grain generation function used by the display process, a decoder may, if desired, down-convert the model information for chroma in order to simulate film grain for other chroma formats (4:2:0 or 4:2:2) rather than up-converting the decoded video (using a method not specified in this Specification) before performing film grain generation.

blending_mode_id identifies the blending mode used to blend the simulated film grain with the decoded images as specified in Table 6. blending_mode_id shall be in the range of 0 to 1, inclusive. The values of 2 and 3 for blending_mode_id are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore film grain characteristic SEI messages with blending_mode_id equal to 2 or 3.

Table 6 – blending_mode_id values

| Value | Description |
| --- | --- |
| 0 | Additive |
| 1 | Multiplicative |

Depending on the value of blending_mode_id, the blending mode is specified as follows:

–	If blending_mode_id is equal to 0, the blending mode is additive as specified by:

Igrain[ x, y, c ] = Clip3( 0, ( 1  <<  filmGrainBitDepth[ c ] ) − 1, Idecoded[ x, y, c ] + G[ x, y, c ] )	(25)

–	Otherwise (blending_mode_id is equal to 1), the blending mode is multiplicative as specified by:

Igrain[ x, y, c ] = Clip3( 0, ( 1  <<  filmGrainBitDepth[ c ] ) − 1, Idecoded[ x, y, c ] +	(26)
			Round( ( Idecoded[ x, y, c ] * G[ x, y, c ] ) ÷ ( ( 1  <<  bitDepth[ c ] ) − 1 ) ) )

where Idecoded[ x, y, c ] represents the sample value at coordinates x, y of the colour component c of the decoded image Idecoded, G[ x, y, c ] is the simulated film grain value at the same position and colour component, filmGrainBitDepth[ c ] is the number of bits used for each sample in a fixed-length unsigned binary representation of the array Igrain[ x, y, c ], and bitDepth[ c ] is specified by:

bitDepth[ c ] = 		(27)

log2_scale_factor specifies a scale factor used in the film grain characterization equations.

comp_model_present_flag[ c ] equal to 0 indicates that film grain is not modelled on the c-th colour component, where c equal to 0 refers to the luma component, c equal to 1 refers to the Cb component, and c equal to 2 refers to the Cr component. comp_model_present_flag[ c ] equal to 1 indicates that syntax elements specifying modelling of film grain on colour component c are present in the SEI message.

num_intensity_intervals_minus1[ c ] plus 1 specifies the number of intensity intervals for which a specific set of model values has been estimated.

NOTE 6 – The intensity intervals may overlap in order to simulate multi-generational film grain.

num_model_values_minus1[ c ] plus 1 specifies the number of model values present for each intensity interval in which the film grain has been modelled. The value of num_model_values_minus1[ c ] shall be in the range of 0 to 5, inclusive.

intensity_interval_lower_bound[ c ][ i ] specifies the lower bound of the interval i of intensity levels for which the set of model values applies.

intensity_interval_upper_bound[ c ][ i ] specifies the upper bound of the interval i of intensity levels for which the set of model values applies.

Depending on the value of film_grain_model_id, the selection of the sets of model values is specified as follows:

–	If film_grain_model_id is equal to 0, the average value of each block b of 8x8 samples in Idecoded, referred as bavg, is used to select the sets of model values with index s[ j ] that apply to all the samples in the block:

for( i = 0, j = 0; i  <=  num_intensity_intervals_minus1[ c ]; i++ )
	if( bavg  >=  intensity_interval_lower_bound[ c ][ i ]  &&
			bavg  <=  intensity_interval_upper_bound[ c ][ i ] ) {
		s[ j ] = i		(28)
		j++
	}

–	Otherwise (film_grain_model_id is equal to 1), the sets of model values used to generate the film grain are selected for each sample value in Idecoded as follows:

for( i = 0, j = 0; i  <=  num_intensity_intervals_minus1[ c ]; i++ )
	if( Idecoded[ x, y, c ]  >=  intensity_interval_lower_bound[ c ][ i ]  &&
			Idecoded[ x, y, c ]  <=  intensity_interval_upper_bound[ c ][ i ] ) {	(29)
		s[ j ] = i
		j++
	}

Samples that do not fall into any of the defined intervals are not modified by the grain generation function. Samples that fall into more than one interval will originate multi-generation grain. Multi-generation grain results from adding the grain computed independently for each intensity interval.

comp_model_value[ c ][ i ][ j ] represents each one of the model values present for the colour component c and the intensity interval i. The set of model values has different meaning depending on the value of film_grain_model_id.

The value of comp_model_value[ c ][ i ][ j ] is constrained as follows, and may be additionally constrained as specified elsewhere in this clause:

–	If film_grain_model_id is equal to 0, comp_model_value[ c ][ i ][ j ] shall be in the range of 0 to 2filmGrainBitDepth[ c ] − 1, inclusive.

–	Otherwise (film_grain_model_id is equal to 1), comp_model_value[ c ][ i ][ j ] shall be in the range of −2( filmGrainBitDepth[ c ] − 1 ) to 2( filmGrainBitDepth[ c ] − 1 ) − 1, inclusive.

Depending on the value of film_grain_model_id, the synthesis of the film grain is modelled as follows:

–	If film_grain_model_id is equal to 0, a frequency filtering model enables simulating the original film grain for c = 0..2, x = 0..PicWidthInLumaSamples and y = 0..PicHeightInLumaSamples as specified by:

G[ x, y, c ] = ( comp_model_value[ c ][ s ][ 0 ] * Q[ c ][ x, y ] + comp_model_value[ c ][ s ][ 5 ] *
			G[ x, y, c − 1 ] )  >>  log2_scale_factor	(30)

where Q[ c ] is a two-dimensional random process generated by filtering 16x16 blocks gaussRv with random-value elements gaussRvij generated with a normalized Gaussian distribution (independent and identically distributed Gaussian random variable samples with zero mean and unity variance) and where the value of an element G[ x, y, c − 1 ] used in the right-hand side of the equation is inferred to be equal to 0 when c − 1 is less than 0.

NOTE 7 – A normalized Gaussian random value can be generated from two independent, uniformly distributed random values over the interval from 0 to 1 (and not equal to 0), denoted as uRv0 and uRv1, using the Box-Muller transformation specified by:

(31)

where  is Archimedes' constant 3.141 592 653 589 793....

The band-pass filtering of blocks gaussRv may be performed in the discrete cosine transform (DCT) domain as follows:

for( y = 0; y < 16; y++ )
	for( x = 0; x < 16; x++ )
		if( ( x < comp_model_value[ c ][ s ][ 3 ]  &&  y < comp_model_value[ c ][ s ][ 4 ] )  | |	(32)
				x > comp_model_value[ c ][ s ][ 1 ]  | |  y > comp_model_value[ c ][ s ][ 2 ] )
			gaussRv[ x, y ] = 0
filteredRv = IDCT16x16( gaussRv )

where IDCT16x16( z ) refers to a unitary inverse discrete cosine transformation (IDCT) operating on a 16x16 matrix argument z as specified by:

IDCT16x16( z ) = r * z * rT		(33)

where the superscript T indicates a matrix transposition and r is the 16x16 matrix with elements rij specified by:

(34)

where  is Archimedes' constant 3.141 592 653 589 793....

Q[ c ] is formed by the frequency-filtered blocks filteredRv.

NOTE 8 – Coded model values are based on blocks of 16x16, but a decoder implementation may use other block sizes. For example, decoders implementing the IDCT on 8x8 blocks should down-convert by a factor of two the set of coded model values comp_model_value[ c ][ s ][ i ] for i equal to 1..4.

NOTE 9 – To reduce the degree of visible blocks that can result from mosaicking the frequency-filtered blocks filteredRv, decoders may apply a low-pass filter to the boundaries between frequency-filtered blocks.

–	Otherwise (film_grain_model_id is equal to 1), an auto-regression model enables simulating the original film grain for c = 0..2, x = 0..PicWidthInLumaSamples and y = 0..PicHeightInLumaSamples as specified by:

G[ x, y, c ] = ( comp_model_value[ c ][ s ][ 0 ] * n[ x, y, c ] +
comp_model_value[ c ][ s ][ 1 ] * ( G[ x − 1, y, c ] + ( ( comp_model_value[ c ][ s ][ 4 ] * G[ x, y − 1, c ] )  >>
		log2_scale_factor ) ) +
comp_model_value[ c ][ s ][ 3 ] * ( ( ( comp_model_value[ c ][ s ][ 4 ] * G[ x − 1, y − 1, c ] )  >>	(35)
		log2_scale_factor ) + G[ x + 1, y − 1, c ] ) +
comp_model_value[ c ][ s ][ 5 ] * ( G[ x − 2, y, c ] +
		( ( comp_model_value[ c ][ s ][ 4 ] * comp_model_value[ c ][ s ][ 4 ] * G[ x, y − 2, c ] )  >>
			( 2 * log2_scale_factor ) ) ) +
	comp_model_value[ c ][ s ][ 2 ] * G[ x, y, c − 1 ] )  >>  log2_scale_factor

where n[ x, y, c ] is a random value with normalized Gaussian distribution (independent and identically distributed Gaussian random variable samples with zero mean and unity variance for each value of x, y and c) and where the value of an element G[ x, y, c ] used in the right-hand side of the equation is inferred to be equal to 0 when any of the following conditions are true:

–	x is less than 0,

–	y is less than 0,

–	c is less than 0.

comp_model_value[ c ][ i ][ 0 ] provides the first model value for the model as specified by film_grain_model_id. comp_model_value[ c ][ i ][ 0 ] corresponds to the standard deviation of the Gaussian noise term in the generation functions specified in Equations 30 through 35.

comp_model_value[ c ][ i ][ 1 ] provides the second model value for the model as specified by film_grain_model_id. When film_grain_model_id is equal to 0, comp_model_value[ c ][ i ][ 1 ] shall be greater than or equal to 0 and less than 16.

When not present in the film grain characteristics SEI message, comp_model_value[ c ][ i ][ 1 ] is inferred as follows:

–	If film_grain_model_id is equal to 0, comp_model_value[ c ][ i ][ 1 ] is inferred to be equal to 8.

–	Otherwise (film_grain_model_id is equal to 1), comp_model_value[ c ][ i ][ 1 ] is inferred to be equal to 0.

comp_model_value[ c ][ i ][ 1 ] is interpreted as follows:

–	If film_grain_model_id is equal to 0, comp_model_value[ c ][ i ][ 1 ] indicates the horizontal high cut frequency to be used to filter the DCT of a block of 16x16 random values.

–	Otherwise (film_grain_model_id is equal to 1), comp_model_value[ c ][ i ][ 1 ] indicates the first order spatial correlation for neighbouring samples ( x − 1, y ) and ( x, y − 1 ).

comp_model_value[ c ][ i ][ 2 ] provides the third model value for the model as specified by film_grain_model_id. When film_grain_model_id is equal to 0, comp_model_value[ c ][ i ][ 2 ] shall be greater than or equal to 0 and less than 16.

When not present in the film grain characteristics SEI message, comp_model_value[ c ][ i ][ 2 ] is inferred as follows:

–	If film_grain_model_id is equal to 0, comp_model_value[ c ][ i ][ 2 ] is inferred to be equal to comp_model_value[ c ][ i ][ 1 ]

–	Otherwise (film_grain_model_id is equal to 1), comp_model_value[ c ][ i ][ 2 ] is inferred to be equal to 0.

comp_model_value[ c ][ i ][ 2 ] is interpreted as follows:

–	If film_grain_model_id is equal to 0, comp_model_value[ c ][ i ][ 2 ] indicates the vertical high cut frequency to be used to filter the DCT of a block of 16x16 random values.

–	Otherwise (film_grain_model_id is equal to 1), comp_model_value[ c ][ i ][ 2 ] indicates the colour correlation between consecutive colour components.

comp_model_value[ c ][ i ][ 3 ] provides the fourth model value for the model as specified by film_grain_model_id. When film_grain_model_id is equal to 0, comp_model_value[ c ][ i ][ 3 ] shall be greater than or equal to 0 and less than or equal to comp_model_value[ c ][ i ][ 1 ].

When not present in the film grain characteristics SEI message, comp_model_value[ c ][ i ][ 3 ] is inferred to be equal to 0.

comp_model_value[ c ][ i ][ 3 ] is interpreted as follows:

–	If film_grain_model_id is equal to 0, comp_model_value[ c ][ i ][ 3 ] indicates the horizontal low cut frequency to be used to filter the DCT of a block of 16x16 random values.

–	Otherwise (film_grain_model_id is equal to 1), comp_model_value[ c ][ i ][ 3 ] indicates the first order spatial correlation for neighbouring samples ( x − 1, y − 1 ) and ( x + 1, y − 1 ).

comp_model_value[ c ][ i ][ 4 ] provides the fifth model value for the model as specified by film_grain_model_id. When film_grain_model_id is equal to 0, comp_model_value[ c ][ i ][ 4 ] shall be greater than or equal to 0 and less than or equal to comp_model_value[ c ][ i ][ 2 ].

When not present in the film grain characteristics SEI message, comp_model_value[ c ][ i ][ 4 ] is inferred to be equal to film_grain_model_id.

comp_model_value[ c ][ i ][ 4 ] is interpreted as follows:

–	If film_grain_model_id is equal to 0, comp_model_value[ c ][ i ][ 4 ] indicates the vertical low cut frequency to be used to filter the DCT of a block of 16x16 random values.

–	Otherwise (film_grain_model_id is equal to 1), comp_model_value[ c ][ i ][ 4 ] indicates the aspect ratio of the modelled grain.

comp_model_value[ c ][ i ][ 5 ] provides the sixth model value for the model as specified by film_grain_model_id.

When not present in the film grain characteristics SEI message, comp_model_value[ c ][ i ][ 5 ] is inferred to be equal to 0.

comp_model_value[ c ][ i ][ 5 ] is interpreted as follows:

–	If film_grain_model_id is equal to 0, comp_model_value[ c ][ i ][ 5 ] indicates the colour correlation between consecutive colour components.

–	Otherwise (film_grain_model_id is equal to 1), comp_model_value[ c ][ i ][ 5 ] indicates the second order spatial correlation for neighbouring samples ( x, y − 2 ) and ( x − 2, y ).

film_grain_characteristics_persistence_flag specifies the persistence of the film grain characteristics SEI message for the current layer.

film_grain_characteristics_persistence_flag equal to 0 specifies that the film grain characteristics SEI message applies to the current decoded picture only.

film_grain_characteristics_persistence_flag equal to 1 specifies that the film grain characteristics SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with a film grain characteristics SEI message is output that follows the current picture in output order.

## Frame packing arrangement SEI message

### Frame packing arrangement SEI message syntax

| frame_packing_arrangement( payloadSize ) { | Descriptor |
| --- | --- |
| frame_packing_arrangement_id | ue(v) |
| frame_packing_arrangement_cancel_flag | u(1) |
| if( !frame_packing_arrangement_cancel_flag ) { |  |
| frame_packing_arrangement_type | u(7) |
| quincunx_sampling_flag | u(1) |
| content_interpretation_type | u(6) |
| spatial_flipping_flag | u(1) |
| frame0_flipped_flag | u(1) |
| field_views_flag | u(1) |
| current_frame_is_frame0_flag | u(1) |
| frame0_self_contained_flag | u(1) |
| frame1_self_contained_flag | u(1) |
| if( !quincunx_sampling_flag && frame_packing_arrangement_type != 5 ) { |  |
| frame0_grid_position_x | u(4) |
| frame0_grid_position_y | u(4) |
| frame1_grid_position_x | u(4) |
| frame1_grid_position_y | u(4) |
| } |  |
| frame_packing_arrangement_reserved_byte | u(8) |
| frame_packing_arrangement_persistence_flag | u(1) |
| } |  |
| upsampled_aspect_ratio_flag | u(1) |
| } |  |

### Frame packing arrangement SEI message semantics

This SEI message informs the decoder that the cropped decoded picture contains samples of multiple distinct spatially packed constituent frames that are packed into one frame, or that the output cropped decoded pictures in output order form a temporal interleaving of alternating first and second constituent frames, using an indicated frame packing arrangement scheme. This information can be used by the decoder to appropriately rearrange the samples and process the samples of the constituent frames appropriately for display or other purposes (which are outside the scope of this Specification).

This SEI message may be associated with pictures that are either frames or fields (as determined outside the scope of this Specification). The frame packing arrangement of the samples is specified in terms of the sampling structure of a frame in order to define a frame packing arrangement structure that is invariant with respect to whether a picture is a single field of such a packed frame or is a complete packed frame.

frame_packing_arrangement_id contains an identifying number that may be used to identify the usage of the frame packing arrangement SEI message. The value of frame_packing_arrangement_id shall be in the range of 0 to 232 − 2, inclusive.

Values of frame_packing_arrangement_id from 0 to 255, inclusive, and from 512 to 231 − 1, inclusive, may be used as determined by the application. Values of frame_packing_arrangement_id from 256 to 511, inclusive, and from 231 to 232 − 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders encountering a value of frame_packing_arrangement_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore it.

frame_packing_arrangement_cancel_flag equal to 1 indicates that the frame packing arrangement SEI message cancels the persistence of any previous frame packing arrangement SEI message in output order that applies to the current layer. frame_packing_arrangement_cancel_flag equal to 0 indicates that frame packing arrangement information follows.

frame_packing_arrangement_type identifies the indicated interpretation of the sample arrays of the output cropped decoded picture as specified in Table 7.

When frame_packing_arrangement_type is equal to 3 or 4, each component plane of the output cropped decoded picture contains all samples (when field_pic_flag is equal to 0) or the samples corresponding to the top or bottom field (when field_pic_flag is equal to 1) of the samples of a frame packing arrangement structure.

Table 7 – Definition of frame_packing_arrangement_type

| Value | Interpretation |
| --- | --- |
| 3 | The frame packing arrangement structure contains a side-by-side packing arrangement of corresponding planes of two constituent frames as illustrated in Figure 6, Figure 7 and Figure 10. |
| 4 | The frame packing arrangement structure contains a top-bottom packing arrangement of corresponding planes of two constituent frames as illustrated in Figure 8 and Figure 9. |
| 5 | The component planes of the output cropped decoded pictures in output order form a temporal interleaving of alternating first and second constituent frames as illustrated in Figure 11. |

NOTE 1 – Figure 6 to Figure 10 provide typical examples of rearrangement and upconversion processing for various packing arrangement schemes. Actual characteristics of the constituent frames are signalled in detail by the subsequent syntax elements of the frame packing arrangement SEI message. In Figure 6 to Figure 10, an upconversion processing is performed on each constituent frame to produce frames having the same resolution as that of the decoded frame. An example of the upsampling method to be applied to a quincunx sampled frame as shown in Figure 10 is to fill in missing positions with an average of the available spatially neighbouring samples (the average of the values of the available samples above, below, to the left and to the right of each sample to be generated). The actual upconversion process to be performed, if any, is outside the scope of this Specification.

NOTE 2 – When the output time of the samples of constituent frame 0 differs from the output time of the samples of constituent frame 1 (i.e., when field_views_flag is equal to 1 or frame_packing_arrangement_type is equal to 5) and the display system in use presents two views simultaneously, the display time for constituent frame 0 should be delayed to coincide with the display time for constituent frame 1. (The display process is not specified in this Specification.)

NOTE 3 – When field_views_flag is equal to 1 or frame_packing_arrangement_type is equal to 5, the value 0 for fixed_pic_rate_within_cvs_flag is not expected to be prevalent in industry use of this SEI message.

NOTE 4 – frame_packing_arrangement_type equal to 5 describes a temporal interleaving process of different views.

All other values of frame_packing_arrangement_type are reserved for future use by ITU-T | ISO/IEC. It is a requirement of bitstream conformance that bitstreams conforming to this version of this Specification shall not contain such other values of frame_packing_arrangement_type. Decoders shall ignore frame packing arrangement SEI messages that contain reserved values of frame_packing_arrangement_type.

quincunx_sampling_flag equal to 1 indicates that each colour component plane of each constituent frame is quincunx sampled as illustrated in Figure 10 and quincunx_sampling_flag equal to 0 indicates that the colour component planes of each constituent frame are not quincunx sampled.

When frame_packing_arrangement_type is equal to 5, it is a requirement of bitstream conformance that quincunx_sampling_flag shall be equal to 0.

NOTE 5 – For any chroma format (4:2:0, 4:2:2 or 4:4:4), the luma plane and each chroma plane is quincunx sampled as illustrated in Figure 10 when quincunx_sampling_flag is equal to 1.

content_interpretation_type indicates the intended interpretation of the constituent frames as specified inTable 8. Values of content_interpretation_type that do not appear in Table 8 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore frame packing arrangement SEI messages that contain reserved values of content_interpretation_type.

For each specified frame packing arrangement scheme, there are two constituent frames that are referred to as frame 0 and frame 1.

Table 8 – Definition of content_interpretation_type

| Value | Interpretation |
| --- | --- |
| 0 | Unspecified relationship between the frame packed constituent frames |
| 1 | Indicates that the two constituent frames form the left and right views of a stereo view scene, with frame 0 being associated with the left view and frame 1 being associated with the right view |
| 2 | Indicates that the two constituent frames form the right and left views of a stereo view scene, with frame 0 being associated with the right view and frame 1 being associated with the left view |

NOTE 6 – The value 2 for content_interpretation_type is not expected to be prevalent in industry use of this SEI message. However, the value was specified herein for purposes of completeness.

spatial_flipping_flag equal to 1, when frame_packing_arrangement_type is equal to 3 or 4, indicates that one of the two constituent frames is spatially flipped relative to its intended orientation for display or other such purposes.

When frame_packing_arrangement_type is equal to 3 or 4 and spatial_flipping_flag is equal to 1, the type of spatial flipping that is indicated is as follows:

–	If frame_packing_arrangement_type is equal to 3, the indicated spatial flipping is horizontal flipping.

–	Otherwise (frame_packing_arrangement_type is equal to 4), the indicated spatial flipping is vertical flipping.

When frame_packing_arrangement_type is not equal to 3 or 4, it is a requirement of bitstream conformance that spatial_flipping_flag shall be equal to 0. When frame_packing_arrangement_type is not equal to 3 or 4, the value 1 for spatial_flipping_flag is reserved for future use by ITU-T | ISO/IEC. When frame_packing_arrangement_type is not equal to 3 or 4, decoders shall ignore the value 1 for spatial_flipping_flag.

frame0_flipped_flag, when spatial_flipping_flag is equal to 1, indicates which one of the two constituent frames is flipped.

When spatial_flipping_flag is equal to 1, frame0_flipped_flag equal to 0 indicates that frame 0 is not spatially flipped and frame 1 is spatially flipped and frame0_flipped_flag equal to 1 indicates that frame 0 is spatially flipped and frame 1 is not spatially flipped.

When spatial_flipping_flag is equal to 0, it is a requirement of bitstream conformance that frame0_flipped_flag shall be equal to 0. When spatial_flipping_flag is equal to 0, the value 1 for spatial_flipping_flag is reserved for future use by ITU-T | ISO/IEC. When spatial_flipping_flag is equal to 0, decoders shall ignore the value of frame0_flipped_flag.

field_views_flag equal to 1 indicates that all pictures in the current CLVS are coded as fields, all fields of a particular parity are considered a first constituent frame and all fields of the opposite parity are considered a second constituent frame. It is a requirement of bitstream conformance that the field_views_flag shall be equal to 0, the value 1 for field_views_flag is reserved for future use by ITU-T | ISO/IEC and decoders shall ignore the value of field_views_flag.

current_frame_is_frame0_flag equal to 1, when frame_packing_arrangement is equal to 5, indicates that the current decoded frame is constituent frame 0 and the next decoded frame in output order is constituent frame 1 and the display time of the constituent frame 0 should be delayed to coincide with the display time of constituent frame 1. current_frame_is_frame0_flag equal to 0, when frame_packing_arrangement is equal to 5, indicates that the current decoded frame is constituent frame 1 and the previous decoded frame in output order is constituent frame 0 and the display time of the constituent frame 1 should not be delayed for purposes of stereo-view pairing.

When frame_packing_arrangement_type is not equal to 5, the constituent frame associated with the upper-left sample of the decoded frame is considered to be constituent frame 0 and the other constituent frame is considered to be constituent frame 1. When frame_packing_arrangement_type is not equal to 5, it is a requirement of bitstream conformance that current_frame_is_frame0_flag shall be equal to 0. When frame_packing_arrangement_type is not equal to 5, the value 1 for current_frame_is_frame0_flag is reserved for future use by ITU-T | ISO/IEC. When frame_packing_arrangement_type is not equal to 5, decoders shall ignore the value of current_frame_is_frame0_flag.

frame0_self_contained_flag equal to 1 indicates that no inter prediction operations within the decoding process for the samples of constituent frame 0 of the CLVS refer to samples of any constituent frame 1. frame0_self_contained_flag equal to 0 indicates that some inter prediction operations within the decoding process for the samples of constituent frame 0 of the CLVS may or may not refer to samples of some constituent frame 1. Within a CLVS, the value of frame0_self_contained_flag in all frame packing arrangement SEI messages shall be the same.

frame1_self_contained_flag equal to 1 indicates that no inter prediction operations within the decoding process for the samples of constituent frame 1 of the CLVS refer to samples of any constituent frame 0. frame1_self_contained_flag equal to 0 indicates that some inter prediction operations within the decoding process for the samples of constituent frame 1 of the CLVS may or may not refer to samples of some constituent frame 0. Within a CLVS, the value of frame1_self_contained_flag in all frame packing arrangement SEI messages shall be the same.

When quincunx_sampling_flag is equal to 0 and frame_packing_arrangement_type is not equal to 5, two ( x, y ) coordinate pairs are specified to determine the indicated luma sampling grid alignment for constituent frame 0 and constituent frame 1, relative to the upper left corner of the rectangular area represented by the samples of the corresponding constituent frame.

NOTE 7 – The location of chroma samples relative to luma samples can be indicated by the vui_chroma_sample_loc_type_frame, vui_chroma_sample_loc_type_top_field, and vui_chroma_sample_loc_type_bottom_field syntax elements in the VUI parameters, when present.

frame0_grid_position_x (when present) specifies the x component of the ( x, y ) coordinate pair for constituent frame 0.

frame0_grid_position_y (when present) specifies the y component of the ( x, y ) coordinate pair for constituent frame 0.

frame1_grid_position_x (when present) specifies the x component of the ( x, y ) coordinate pair for constituent frame 1.

frame1_grid_position_y (when present) specifies the y component of the ( x, y ) coordinate pair for constituent frame 1.

When quincunx_sampling_flag is equal to 0 and frame_packing_arrangement_type is not equal to 5 the ( x, y ) coordinate pair for each constituent frame is interpreted as follows:

–	If the ( x, y ) coordinate pair for a constituent frame is equal to ( 0, 0 ), this indicates a default sampling grid alignment specified as follows:

–	If frame_packing_arrangement_type is equal to 3, the indicated position is the same as for the ( x, y ) coordinate pair value ( 4, 8 ), as illustrated in Figure 6.

–	Otherwise (frame_packing_arrangement_type is equal to 4), the indicated position is the same as for the ( x, y ) coordinate pair value ( 8, 4 ), as illustrated in Figure 8.

–	Otherwise, if the ( x, y ) coordinate pair for a constituent frame is equal to ( 15, 15 ), this indicates that the sampling grid alignment is unknown or unspecified or specified by other means not specified in this Specification.

–	Otherwise, the x and y elements of the ( x, y ) coordinate pair specify the indicated horizontal and vertical sampling grid alignment positioning to the right of and below the upper left corner of the rectangular area represented by the corresponding constituent frame, respectively, in units of one sixteenth of the luma sample grid spacing between the samples of the columns and rows of the constituent frame that are present in the decoded frame (prior to any upsampling for display or other purposes).

NOTE 8 – The spatial location reference information frame0_grid_position_x, frame0_grid_position_y, frame1_grid_position_x, and frame1_grid_position_y is not provided when quincunx_sampling_flag is equal to 1 because the spatial alignment in this case is assumed to be such that constituent frame 0 and constituent frame 1 cover corresponding spatial areas with interleaved quincunx sampling patterns as illustrated in Figure 10.

frame_packing_arrangement_reserved_byte is reserved for future use by ITU-T | ISO/IEC. It is a requirement of bitstream conformance that the value of frame_packing_arrangement_reserved_byte shall be equal to 0. All other values of frame_packing_arrangement_reserved_byte are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of frame_packing_arrangement_reserved_byte.

frame_packing_arrangement_persistence_flag specifies the persistence of the frame packing arrangement SEI message for the current layer.

frame_packing_arrangement_persistence_flag equal to 0 specifies that the frame packing arrangement SEI message applies to the current decoded frame only.

frame_packing_arrangement_persistence_flag equal to 1 specifies that the frame packing arrangement SEI message applies to the current decoded picture and persists all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with a frame packing arrangement SEI message is output that follows the current picture in output order.

upsampled_aspect_ratio_flag equal to 1 indicates that the sample aspect ratio (SAR) indicated by the VUI parameters or the sample aspect ratio information SEI message identifies the SAR of the samples after the application of an upconversion process to produce a higher resolution frame from each constituent frame as illustrated in Figure 6 to Figure 10. upsampled_aspect_ratio_flag equal to 0 indicates that the SAR indicated by the VUI parameters or the sample aspect ratio information SEI message identifies the SAR of the samples before the application of any such upconversion process.

NOTE 9 – The default display window parameters in the VUI parameters or the sample aspect ratio information SEI message can be used by an encoder to indicate to a decoder that does not interpret the frame packing arrangement SEI message that the default display window is an area within only one of the two constituent frames.

NOTE 10 – The SAR indicated in the VUI parameters or the sample aspect ratio information SEI message should indicate the preferred display picture shape for the packed decoded frame output by a decoder that does not interpret the frame packing arrangement SEI message. When upsampled_aspect_ratio_flag is equal to 1, the SAR produced in each up-converted colour plane is indicated to be the same as the SAR indicated in the VUI parameters or the sample aspect ratio information SEI message in the examples shown in Figure 6 to Figure 10. When upsampled_aspect_ratio_flag is equal to 0, the SAR produced in each colour plane prior to upconversion is indicated to be the same as the SAR indicated in the VUI parameters or the sample aspect ratio information SEI message in the examples shown in Figure 6 to Figure 10.

Figure 6 – Rearrangement and upconversion of side-by-side packing arrangement with
frame_packing_arrangement_type equal to 3, quincunx_sampling_flag equal to 0
and ( x, y ) equal to ( 0, 0 ) or ( 4, 8 ) for both constituent frames

Figure 7 – Rearrangement and upconversion of side-by-side packing arrangement with frame_packing_arrangement_type equal to 3, quincunx_sampling_flag equal to 0,
( x, y ) equal to ( 12, 8 ) for constituent frame 0 and ( x, y ) equal to ( 0, 0 ) or ( 4, 8 ) for constituent frame 1

Figure 8 – Rearrangement and upconversion of top-bottom packing arrangement with
frame_packing_arrangement_type equal to 4, quincunx_sampling_flag equal to 0
and ( x, y ) equal to ( 0, 0 ) or ( 8, 4 ) for both constituent frames

Figure 9 – Rearrangement and upconversion of top-bottom packing arrangement with frame_packing_arrangement_type equal to 4, quincunx_sampling_flag equal to 0,
( x, y ) equal to ( 8, 12 ) for constituent frame 0 and ( x, y ) equal to ( 0, 0 ) or ( 8, 4 ) for constituent frame 1

Figure 10 – Rearrangement and upconversion of side-by-side packing arrangement with quincunx sampling
(frame_packing_arrangement_type equal to 3 with quincunx_sampling_flag equal to 1)

Figure 11 – Rearrangement of a temporal interleaving frame arrangement
(frame_packing_arrangement_type equal to 5)

## Parameter sets inclusion indication SEI message

### Parameter sets inclusion indication SEI message syntax

| parameter_sets_inclusion_indication( payloadSize ) { | Descriptor |
| --- | --- |
| self_contained_clvs_flag | u(1) |
| } |  |

### Parameter sets inclusion indication SEI message semantics

This message provides an indication of whether the CLVS contains all the required NAL units for decoding the CLVS that is associated with the SEI message and whether sublayer up-switching within the CLVS works without a need of fetching parameter sets from PUs earlier in decoding order than the PU containing the picture at which the sublayer up-switching occurs. When the CLVS does not contain all the required NAL units, the NAL units that are not present in the CLVS may be provided externally.

self_contained_clvs_flag equal to 1 indicates that the following restrictions apply:

–	Each parameter set that is (directly or indirectly) referenced by any VCL NAL unit of the CLVS that is not a VCL NAL unit of a RASL picture (when present) associated with the first AU of the CLVS is present within the CLVS at a position that precedes, in decoding order, any NAL unit that (directly or indirectly) references the parameter set.

–	For any STSA picture stsaPicA with TemporalId equal to tIdA in the CLVS, the following applies:

–	stsaPicA does not refer to a PPS or an APS that precedes the first NAL unit of the PU containing stsaPicA in decoding order and has TemporalId equal to the TemporalId of stsaPicA.

–	For any picture picB with TemporalId equal to tIdA and following stsaPicA in decoding order, picB does not refer to a PPS or an APS that has TemporalId equal to tIdA that precedes the first NAL unit of the PU containing stsaPicA in decoding order.

self_contained_clvs_flag equal to 0 indicates that this property may or may not apply.

## Decoded picture hash SEI message

### Decoded picture hash SEI message syntax

| decoded_picture_hash( payloadSize ) { | Descriptor |
| --- | --- |
| dph_sei_hash_type | u(8) |
| dph_sei_single_component_flag | u(1) |
| dph_sei_reserved_zero_7bits | u(7) |
| for( cIdx = 0; cIdx < ( dph_sei_single_component_flag ? 1 : 3 ); cIdx++ ) |  |
| if( dph_sei_hash_type = = 0 ) |  |
| for( i = 0; i < 16; i++) |  |
| dph_sei_picture_md5[ cIdx ][ i ] | b(8) |
| else if(dph_sei_hash_type = = 1 ) |  |
| dph_sei_picture_crc[ cIdx ] | u(16) |
| else if(dph_sei_hash_type = = 2 ) |  |
| dph_sei_picture_checksum[ cIdx ] | u(32) |
| } |  |

### Decoded picture hash SEI message semantics

This message provides a hash for each colour component of the current decoded picture.

Use of this SEI message requires the definition of the following variables:

–	A picture width and picture height in units of luma samples, denoted herein by PicWidthInLumaSamples and PicHeightInLumaSamples, respectively.

–	A chroma format indicator, denoted herein by ChromaFormatIdc, as described in clause 7.2.

–	A bit depth for the samples of the luma component, denoted herein by BitDepthY, and when ChromaFormatIdc is not equal to 0, a bit depth for the samples of the two associated chroma components, denoted herein by BitDepthC.

–	For each colour component cIdx, an array ComponentSamples[ cIdx ][ i ] in raster scan order of decoded sample values in two's complement representation. [Ed. (GJS): Is this really supposed to be a two's complement representation, or is it an unsigned integer representation? Note that this is the same in HEVC. This should be further studied.]

Prior to computing the hash, the decoded picture data are arranged into one or three strings of bytes called pictureData[ cIdx ] of lengths dataLen[ cIdx ] as follows:

for( cIdx = 0; cIdx < dph_sei_single_component_flag ? 1 : 3; cIdx++ ) {
	if( cIdx  = =  0 ) {
		compWidth[ cIdx ] = PicWidthInLumaSamples
		compHeight[ cIdx ] = PicHeightInLumaSamples
		compDepth[ cIdx ] = BitDepthY
	} else {
		compWidth[ cIdx ] = PicWidthInLumaSamples / SubWidthC
		compHeight[ cIdx ] = PicHeightInLumaSamples / SubHeightC
		compDepth[ cIdx ] = BitDepthC		(36)
	}
	iLen = 0
	for( i = 0; i < compWidth[ cIdx ] * compHeight[ cIdx ]; i++ ) {
		pictureData[ cIdx ][ iLen++ ] = ComponentSamples[ cIdx ][ i ] & 0xFF
		if( compDepth[ cIdx ] > 8 )
			pictureData[ cIdx ][ iLen++ ] = ComponentSamples[ cIdx ][ i ]  >>  8
	}
	dataLen[ cIdx ] = iLen
}

where ComponentSamples[ cIdx ][ i ] is an array in raster scan of decoded sample values in two's complement representation. [Ed. (GJS): Is this really supposed to be a two's complement representation, or is it an unsigned integer representation?]

dph_sei_hash_type indicates the method used to calculate the checksum as specified in Table 9 . Values of dph_sei_hash_type that are not listed in in Table 9  are reserved for future use by ITU-T | ISO/IEC and shall not be present in payload data conforming to this version of this Specification. Decoders shall ignore decoded picture hash SEI messages that contain reserved values of dph_sei_hash_type.

Table 9 –  Interpretation of dph_sei_hash_type

| dph_sei_hash_type | Method |
| --- | --- |
| 0 | MD5 (IETF RFC 1321) |
| 1 | CRC |
| 2 | Checksum |

dph_sei_single_component_flag equal to 1 specifies that the picture associated with the decoded picture hash SEI message contains a single colour component. dph_sei_single_component_flag equal to 0 specifies that the picture associated with the decoded picture hash SEI message contains three colour components. The value of dph_sei_single_component_flag shall be equal to ( ChromaFormatIdc  = =  0 ).

dph_sei_reserved_zero_7bits shall be equal to 0. Values greater than 0 for dph_sei_single_component_flag are reserved for future use by ITU-T | ISO/IEC and shall not be present in payload data conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore the value of dph_sei_reserved_zero_7bits.

dph_sei_picture_md5[ cIdx ][ i ] is the 16-byte MD5 hash of the cIdx-th colour component of the decoded picture. The value of dph_sei_picture_md5[ cIdx ][ i ] shall be equal to the value of digestVal[ cIdx ] obtained as follows, using the MD5 functions defined in IETF RFC 1321:

MD5Init( context )
MD5Update( context, pictureData[ cIdx ], dataLen[ cIdx ] )	(37)
MD5Final( digestVal[ cIdx ], context )

dph_sei_picture_crc[ cIdx ] is the cyclic redundancy check (CRC) of the colour component cIdx of the decoded picture. The value of dph_sei_picture_crc[ cIdx ] shall be equal to the value of crcVal[ cIdx ] obtained as follows:

crc = 0xFFFF
pictureData[ cIdx ][  dataLen[ cIdx ] ] = 0
pictureData[ cIdx ][  dataLen[ cIdx ] + 1 ] = 0
for( bitIdx = 0; bitIdx < ( dataLen[ cIdx ]  + 2 ) * 8; bitIdx++ ) {	(38)
	dataByte = pictureData[ cIdx ][ bitIdx  >>  3 ]
	crcMsb = ( crc  >>  15 ) & 1
	bitVal = ( dataByte  >>  ( 7 − ( bitIdx & 7 ) ) ) & 1
	crc = ( ( ( crc  <<  1 ) + bitVal ) & 0xFFFF ) ^ ( crcMsb * 0x1021 )
}
crcVal[ cIdx ] = crc

NOTE – The same CRC specification is found in Rec. ITU-T H.271.

dph_sei_picture_checksum[ cIdx ] is the checksum of the colour component cIdx of the decoded picture. The value of dph_sei_picture_checksum[ cIdx ] shall be equal to the value of checksumVal[ cIdx ] obtained as follows:

sum = 0
for( y = 0; y < compHeight[ cIdx ]; y++ )
	for( x = 0; x < compWidth[ cIdx ]; x++ ) {
		xorMask = ( x & 0xFF ) ^ ( y & 0xFF ) ^ ( x  >>  8 ) ^ ( y  >>  8 )
		sum = ( sum + ( ( ComponentSamples[ cIdx ][ y * compWidth[ cIdx ] + x ] & 0xFF ) ^	(39)
				xorMask ) ) & 0xFFFFFFFF
		if( compDepth[ cIdx ] > 8 )
			sum = ( sum + ( ( ComponentSamples[ cIdx ][ y * compWidth[ cIdx ] + x ]  >>  8 ) ^
					xorMask ) ) & 0xFFFFFFFF
	}
checksumVal[ cIdx ] = sum

## Mastering display colour volume SEI message

### Mastering display colour volume SEI message syntax

| mastering_display_colour_volume( payloadSize ) { | Descriptor |
| --- | --- |
| for( c = 0; c < 3; c++ ) { |  |
| display_primaries_x[ c ] | u(16) |
| display_primaries_y[ c ] | u(16) |
| } |  |
| white_point_x | u(16) |
| white_point_y | u(16) |
| max_display_mastering_luminance | u(32) |
| min_display_mastering_luminance | u(32) |
| } |  |

### Mastering display colour volume SEI message semantics

This SEI message identifies the colour volume (the colour primaries, white point, and luminance range) of a display considered to be the mastering display for the associated video content – e.g., the colour volume of a display that was used for viewing while authoring the video content. The described mastering display is a three-colour additive display system that has been configured to use the indicated mastering colour volume.

This SEI message does not identify the measurement methodologies and procedures used for determining the indicated values or provide any description of the mastering environment. It also does not provide information on colour transformations that would be appropriate to preserve creative intent on displays with colour volumes different from that of the described mastering display.

The information conveyed in this SEI message is intended to be adequate for purposes corresponding to the use of SMPTE ST 2086.

When a mastering display colour volume SEI message is present for any picture of a CLVS of a particular layer, a mastering display colour volume SEI message shall be present for the first picture of the CLVS. The mastering display colour volume SEI message persists for the current layer in decoding order from the current picture until the end of the CLVS. All mastering display colour volume SEI messages that apply to the same CLVS shall have the same content.

display_primaries_x[ c ], when in the range of 5 to 37 000, inclusive, specifies the normalized x chromaticity coordinate of the colour primary component c of the mastering display, according to the CIE 1931 definition of x as specified in ISO 11664-1 (see also ISO 11664-3 and CIE 15), in increments of 0.00002. When display_primaries_x[ c ] is not in the range of 5 to 37 000, inclusive, the normalized x chromaticity coordinate of the colour primary component c of the mastering display is unknown or unspecified or specified by other means not specified in this Specification.

display_primaries_y[ c ], when in the range of 5 to 42 000, inclusive, specifies the normalized y chromaticity coordinate of the colour primary component c of the mastering display, according to the CIE 1931 definition of y as specified in ISO 11664-1 (see also ISO 11664-3 and CIE 15), in increments of 0.00002. When display_primaries_y[ c ] is not in the range of 5 to 42 000, inclusive, the normalized y chromaticity coordinate of the colour primary component c of the mastering display is unknown or unspecified or specified by other means not specified in this Specification.

For describing mastering displays that use red, green, and blue colour primaries, it is suggested that index value c equal to 0 should correspond to the green primary, c equal to 1 should correspond to the blue primary, and c equal to 2 should correspond to the red colour primary specified in the VUI parameters.

white_point_x, when in the range of 5 to 37 000, inclusive, specifies the normalized x chromaticity coordinate of the white point of the mastering display, according to the CIE 1931 definition of x as specified in ISO 11664-1 (see also ISO 11664-3 and CIE 15), in normalized increments of 0.00002. When white_point_x is not in the range of 5 to 37 000, inclusive, the normalized x chromaticity coordinate of the white point of the mastering display is indicated to be unknown or unspecified or specified by other means not specified in this Specification.

white_point_y, when in the range of 5 to 42 000, inclusive, specifies the normalized y chromaticity coordinate of the white point of the mastering display, according to the CIE 1931 definition of y as specified in ISO 11664-1 (see also ISO 11664-3 and CIE 15), in normalized increments of 0.00002. When white_point_y is not in the range of 5 to 42 000, inclusive, the normalized y chromaticity coordinate of the white point of the mastering display is indicated to be unknown or unspecified or specified by other means not specified in this Specification.

NOTE 1 – SMPTE ST 2086 specifies that the normalized x and y chromaticity coordinate values for the mastering display colour primaries and white point are to be represented with four decimal places. This would correspond with using values of the syntax elements display_primaries_x[ c ], display_primaries_y[ c ], white_point_x, and white_point_y, as defined in this Specification, that are multiples of 5.

NOTE 2 – An example of the use of values outside the range for which semantics are specified in this Specification is that ANSI/CTA 861-G uses normalized (x, y) chromaticity coordinate values of (0,0) for the white point to indicate that the white point chromaticity is unknown.

max_display_mastering_luminance, when in the range of 50 000 to 100 000 000, specifies the nominal maximum display luminance of the mastering display in units of 0.0001 candelas per square metre. When max_display_mastering_luminance is not in the range of 50 000 to 100 000 000, the nominal maximum display luminance of the mastering display is indicated to be unknown or unspecified or specified by other means not specified in this Specification.

NOTE 3 – SMPTE ST 2086 specifies that the nominal maximum display luminance of the mastering display is to be specified as a multiple of 1 candela per square meter. This would correspond with using values of the syntax element max_display_mastering_luminance, as defined in this Specification, that are a multiple of 10 000.

NOTE 4 – An example of the use of values outside the range for which semantics are specified in this Specification is that ANSI/CTA 861-G uses the value 0 for the nominal maximum display luminance of the mastering display to indicate that the nominal maximum display luminance of the mastering display is unknown.

min_display_mastering_luminance, when in the range of 1 to 50 000, specifies the nominal minimum display luminance of the mastering display in units of 0.0001 candelas per square metre. When min_display_mastering_luminance is not in the range of 1 to 50 000, the nominal maximum display luminance of the mastering display is unknown or unspecified or specified by other means not specified in this Specification. When max_display_mastering_luminance is equal to 50 000, min_display_mastering_luminance shall not be equal to 50 000.

NOTE 5 – SMPTE ST 2086 specifies that the nominal minimum display luminance of the mastering display is to be specified as a multiple of 0.0001 candelas per square metre, which corresponds to the semantics specified in this Specification.

NOTE 6 – An example of the use of values outside the range for which semantics are specified in this Specification is that ANSI/CTA 861-G uses the value 0 for the nominal minimum display luminance of the mastering display to indicate that the nominal minimum display luminance of the mastering display is unknown.

NOTE 7 – Another example of the potential use of values outside the range for which semantics are specified in this Specification is that SMPTE ST 2086 indicates that values outside the specified range could be used to indicate that the black level and contrast of the mastering display have been adjusted using picture line-up generation equipment (PLUGE).

At the minimum luminance, the mastering display is considered to have the same nominal chromaticity as the white point.

## Content light level information SEI message

### Content light level information SEI message syntax

| content_light_level_info( payloadSize ) { | Descriptor |
| --- | --- |
| max_content_light_level | u(16) |
| max_pic_average_light_level | u(16) |
| } |  |

### Content light level information SEI message semantics

This SEI message identifies upper bounds for the nominal target brightness light level of the pictures of the CLVS.

The information conveyed in this SEI message is intended to be adequate for purposes corresponding to the use of the Consumer Electronics Association 861.3 specification.

The semantics of the content light level information SEI message are defined in relation to the values of samples in a 4:4:4 representation of red, green, and blue colour primary intensities in the linear light domain for the pictures of the CLVS, in units of candelas per square metre. However, this SEI message does not, by itself, identify a conversion process for converting the sample values of a decoded picture to the samples in a 4:4:4 representation of red, green, and blue colour primary intensities in the linear light domain for the picture.

NOTE 1 – Other syntax elements, such as vui_colour_primaries, vui_transfer_characteristics, and vui_matrix_coeffs, when present, may assist in the identification of such a conversion process.

Given the red, green, and blue colour primary intensities in the linear light domain for the location of a luma sample in a corresponding 4:4:4 representation, denoted as ER, EG, and EB, the maximum component intensity is defined as EMax = Max( ER, Max( EG, EB ) ). The light level corresponding to the stimulus is then defined as the CIE 1931 luminance corresponding to equal amplitudes of EMax for all three colour primary intensities for red, green, and blue (with appropriate scaling to reflect the nominal luminance level associated with peak white – e.g., ordinarily scaling to associate peak white with 10 000 candelas per square metre when vui_transfer_characteristics is equal to 16).

NOTE 2 – Since the maximum value EMax is used in this definition at each sample location, rather than a direct conversion from ER, EG, and EB to the corresponding CIE 1931 luminance, the CIE 1931 luminance at a location may in some cases be less than the indicated light level. This situation would occur, for example, when ER and EG are very small and EB is large, in which case the indicated light level would be much larger than the true CIE 1931 luminance associated with the ( ER, EG, EB ) triplet.

All content light level information SEI messages that apply to the same CLVS shall have the same content.

max_content_light_level, when not equal to 0, indicates an upper bound on the maximum light level among all individual samples in a 4:4:4 representation of red, green, and blue colour primary intensities (in the linear light domain) for the pictures of the CLVS, in units of candelas per square metre. When equal to 0, no such upper bound is indicated by max_content_light_level.

max_pic_average_light_level, when not equal to 0, indicates an upper bound on the maximum average light level among the samples in a 4:4:4 representation of red, green, and blue colour primary intensities (in the linear light domain) for any individual picture of the CLVS, in units of candelas per square metre. When equal to 0, no such upper bound is indicated by max_pic_average_light_level.

NOTE 3 – When the visually relevant region does not correspond to the entire cropped decoded picture, such as for "letterbox" encoding of video content with a wide picture aspect ratio within a taller cropped decoded picture, the indicated average should be performed only within the visually relevant region.

## Dependent random access point indication SEI message

### Dependent random access point indication SEI message syntax

| dependent_rap_indication( payloadSize ) { | Descriptor |
| --- | --- |
| } |  |

### Dependent random access point indication SEI message semantics

The picture associated with a dependent random access point (DRAP) indication SEI message is referred to as a DRAP picture.

The presence of the DRAP indication SEI message indicates that the constraints on picture order and picture referencing specified in this subclause apply. These constraints can enable a decoder to properly decode the DRAP picture and the pictures that follow it in both decoding order and output order without needing to decode any other pictures except the associated IRAP picture of the DRAP picture.

The constraints indicated by the presence of the DRAP indication SEI message, which shall all apply, are as follows:

–	The DRAP picture is a trailing picture.

–	The DRAP picture has a temporal sublayer identifier equal to 0.

–	The DRAP picture does not include any pictures in the active entries of its reference picture lists except the associated IRAP picture of the DRAP picture.

–	Any picture that follows the DRAP picture in both decoding order and output order does not include, in the active entries of its reference picture lists, any picture that precedes the DRAP picture in decoding order or output order, with the exception of the associated IRAP picture of the DRAP picture.

## Alternative transfer characteristics information SEI message

### Alternative transfer characteristics information SEI message syntax

| alternative_transfer_characteristics ( payloadSize ) { | Descriptor |
| --- | --- |
| preferred_transfer_characteristics | u(8) |
| } |  |

### Alternative transfer characteristics SEI message semantics

The alternative transfer characteristics SEI message provides a preferred alternative value for the transfer_characteristics syntax element that is indicated by the colour description syntax of the VUI parameters. This SEI message is intended to be used in cases when some value of vui_transfer_characteristics is preferred for interpretation of the pictures of the CLVS although some other value of vui_transfer_characteristics may also be acceptable for interpretation of the pictures of the CLVS and that other value is provided in the colour description syntax of the VUI parameters for interpretation by decoders that do not support interpretation of the preferred value (e.g., because the preferred value had not yet been defined in a previous version of this Specification).

When an alternative transfer characteristics SEI message is present for any picture of a CLVS of a particular layer and the first picture of the CLVS is an IRAP picture, an alternative transfer characteristics SEI message shall be present for that IRAP picture. The alternative transfer characteristics SEI message persists for the current layer in decoding order from the current picture until the end of the CLVS. All alternative transfer characteristics SEI messages that apply to the same CLVS shall have the same content.

preferred_transfer_characteristics specifies a preferred alternative value for the vui_transfer_characteristics syntax element of the colour description syntax of the VUI parameters. The semantics for preferred_transfer_characteristics are otherwise the same as for the vui_transfer_characteristics syntax element specified in the VUI parameters. When preferred_transfer_characteristics is not equal to the value of vui_transfer_characteristics indicated in the VUI parameters, decoders should ignore the value of vui_transfer_characteristics indicated in the VUI parameters and instead use the value indicated by preferred_transfer_characteristics.

## Ambient viewing environment SEI message

### Ambient viewing environment SEI message syntax

| ambient_viewing_environment( payloadSize ) { | Descriptor |
| --- | --- |
| ambient_illuminance | u(32) |
| ambient_light_x | u(16) |
| ambient_light_y | u(16) |
| } |  |

### Ambient viewing environment SEI message semantics

The ambient viewing environment SEI message identifies the characteristics of the nominal ambient viewing environment for the display of the associated video content. The syntax elements of the ambient viewing environment SEI message may assist the receiving system in adapting the received video content for local display in viewing environments that may be similar or may substantially differ from those assumed or intended when mastering the video content.

This SEI message does not provide information on colour transformations that would be appropriate to preserve creative intent on displays with colour volumes different from that of the described mastering display.

When an ambient viewing environment SEI message is present for any picture of a CLVS of a particular layer and the first picture of the CLVS is an IRAP picture, an ambient viewing environment SEI message shall be present for that IRAP picture. The ambient viewing environment SEI message persists for the current layer in decoding order from the current picture until the end of the CLVS. All ambient viewing environment SEI messages that apply to the same CLVS shall have the same content.

ambient_illuminance specifies the environmental illuminance of the ambient viewing environment in units of 0.0001 lux. ambient_illuminance shall not be equal to 0.

ambient_light_x and ambient_light_y specify the normalized x and y chromaticity coordinates, respectively, of the environmental ambient light in the nominal viewing environment, according to the CIE 1931 definition of x and y as specified in ISO 11664-1 (see also ISO 11664-3 and CIE 15), in normalized increments of 0.00002. The values of ambient_light_x and ambient_light_y shall be in the range of 0 to 50 000.

NOTE – For example, the conditions identified in Rec. ITU-R BT.2035 can be expressed using ambient_illuminance equal to 100 000 with background chromaticity indicating D65 (ambient_light_x equal to 15 635, ambient_light_y equal to 16 450), or optionally in some regions, background chromaticity indicating D93 (ambient_light_x equal to 14 155, ambient_light_y equal to 14 855).

## Content colour volume SEI message

### Content colour volume SEI message syntax

| content_colour_volume( payloadSize ) { | Descriptor |
| --- | --- |
| ccv_cancel_flag | u(1) |
| if( !ccv_cancel_flag ) { |  |
| ccv_persistence_flag | u(1) |
| ccv_primaries_present_flag | u(1) |
| ccv_min_luminance_value_present_flag | u(1) |
| ccv_max_luminance_value_present_flag | u(1) |
| ccv_avg_luminance_value_present_flag | u(1) |
| ccv_reserved_zero_2bits | u(2) |
| if( ccv_primaries_present_flag ) |  |
| for( c = 0; c < 3; c++ ) { |  |
| ccv_primaries_x[ c ] | i(32) |
| ccv_primaries_y[ c ] | i(32) |
| } |  |
| if( ccv_min_luminance_value_present_flag ) |  |
| ccv_min_luminance_value | u(32) |
| if( ccv_max_luminance_value_present_flag ) |  |
| ccv_max_luminance_value | u(32) |
| if( ccv_avg_luminance_value_present_flag ) |  |
| ccv_avg_luminance_value | u(32) |
| } |  |
| } |  |

### Content colour volume SEI message semantics

The content colour volume SEI message describes the colour volume characteristics of the associated pictures. These colour volume characteristics are expressed in terms of a nominal range, although deviations from this range may occur.

The variable transferCharacteristics is specified as follows:

–	If an alternative transfer characteristics SEI message is present for the CLVS, transferCharacteristics is set equal to preferred_transfer_characteristics;

–	Otherwise, (an alternative transfer characteristics SEI message is not present for the CLVS), transferCharacteristics is set equal to vui_transfer_characteristics.

The content colour volume SEI message shall not be present, and decoders shall ignore it, when any of the following conditions is true:

–	Any of the values of transferCharacteristics, vui_colour_primaries, and vui_matrix_coeffs has a value defined as unspecified.

–	The value of vui_transfer_characteristics is equal to 2, 4, or 5.

–	The value of vui_colour_primaries is equal to 2.

The following applies when converting the signal from a non-linear to a linear representation:

–	If the value of transferCharacteristics is equal to 1, 6, 7, 14, or 15, the Rec. ITU-R BT.1886 reference electro-optical transfer function should be used to convert the signal to its linear representation, where the value of screen luminance for white is set equal to 100 cd/m2, the value of screen luminance for black is set equal to 0 cd/m2, and the value of the exponent of the power function is set equal to 2.4.

–	Otherwise, if the value of transferCharacteristics is equal to 18, the hybrid log-gamma reference electro-optical transfer function specified in Rec. ITU-R BT.2100 should be used to convert the signal to its linear representation, where the value of nominal peak luminance of the display is set equal to 1000 cd/m2, the value of the display luminance for black is set equal to 0 cd/m2, and the value of system gamma is set equal to 1.2.

–	Otherwise (the value of transferCharacteristics is not equal to 1, 6, 7, 14, 15, or 18) when the content colour volume SEI message is present, the exact inverse of the transfer function specified in specified in the VUI parameters should be used to convert the non-linear signal to a linear representation.

ccv_cancel_flag equal to 1 indicates that the content colour volume SEI message cancels the persistence of any previous content colour volume SEI message in output order that applies to the current layer. ccv_cancel_flag equal to 0 indicates that content colour volume information follows.

ccv_persistence_flag specifies the persistence of the content colour volume SEI message for the current layer.

ccv_persistence_flag equal to 0 specifies that the content colour volume applies to the current decoded picture only.

ccv_persistence_flag equal to 1 specifies that the content colour volume SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with a content colour volume SEI message is output that follows the current picture in output order.

ccv_primaries_present_flag equal to 1 specifies that the syntax elements ccv_primaries_x[ c ] and ccv_primaries_y[ c ] are present. ccv_primaries_present_flag equal to 0 specifies that the syntax elements ccv_primaries_x[ c ] and ccv_primaries_y[ c ] are not present.

ccv_min_luminance_value_present_flag equal to 1 specifies that the syntax element ccv_min_luminance_value is present. ccv_min_luminance_value_present_flag equal to 0 specifies that the syntax element ccv_min_luminance_value is not present.

ccv_max_luminance_value_present_flag equal to 1 specifies that the syntax element ccv_max_luminance_value is present. ccv_max_luminance_value_present_flag equal to 0 specifies that the syntax element ccv_max_luminance_value is not present.

ccv_avg_luminance_value_present_flag equal to 1 specifies that the syntax element ccv_avg_luminance_value is present. ccv_avg_luminance_value_present_flag equal to 0 specifies that the syntax element ccv_avg_luminance_value is not present.

It is a requirement of bitstream conformance that the values of ccv_primaries_present_flag, ccv_min_luminance_value_present_flag, ccv_max_luminance_value_present_flag, and ccv_avg_luminance_value_present_flag shall not all be equal to 0.

ccv_reserved_zero_2bits[ i ] shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for reserved_zero_2bits[ i ] are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of reserved_zero_2bits[ i ].

ccv_primaries_x[ c ] and ccv_primaries_y[ c ] specify the normalized x and y chromaticity coordinates, respectively, of the colour primary component c of the nominal content colour volume, according to the CIE 1931 definition of x and y as specified in ISO 11664-1 (see also ISO 11664-3 and CIE 15), in normalized increments of 0.00002. For describing colour volumes that use red, green, and blue colour primaries, it is suggested that index value c equal to 0 should correspond to the green primary, c equal to 1 should correspond to the blue primary, and c equal to 2 should correspond to the red colour primary specified in the VUI parameters.

The values of ccv_primaries_x[ c ] and ccv_primaries_y[ c ] shall be in the range of −5 000 000 to 5 000 000, inclusive.

When ccv_primaries_x[ c ] and ccv_primaries_y[ c ] are not present, they are inferred to be equal to the normalized x and y chromaticity coordinates, respectively, specified by vui_colour_primaries.

ccv_min_luminance_value specifies the normalized minimum luminance value, according to CIE 1931, that is expected to be present in the content, where values are normalized to Lo or Lc as specified in the VUI parameters according to the indicated transfer characteristics of the signal. The values of ccv_min_luminance_value are in normalized increments of 0.0000001.

ccv_max_luminance_value specifies the maximum luminance value, according to CIE 1931, that is expected to be present in the content, where values are normalized to Lo or Lc as specified in the VUI parameters according to the transfer characteristics of the signal. The values of ccv_max_luminance_value are in normalized increments of 0.0000001.

ccv_avg_luminance_value specifies the average luminance value, according to CIE 1931, that is expected to be present in the content, where values are normalized to Lo or Lc as specified in the VUI parameters according to the transfer characteristics of the signal. The values of ccv_avg_luminance_value are in normalized increments of 0.0000001.

NOTE – The resulting domain from this conversion process may or may not represent light in a source or display domain – it is merely a gamut representation domain rather than necessarily being a representation of actual light in either the scene or display domain. Therefore, the values corresponding to ccv_min_luminance_value, ccv_max_luminance_value, and ccv_avg_luminance_value might not necessarily correspond to a true luminance value.

The value of ccv_min_luminance_value, when present, shall be less than or equal to ccv_avg_luminance_value, when present. The value of ccv_avg_luminance_value, when present, shall be less than or equal to ccv_max_luminance_value, when present. The value of ccv_min_luminance_value, when present, shall be less than or equal to ccv_max_luminance_value, when present.

When the visually relevant region does not correspond to the entire cropped decoded picture, such as for "letterbox" encoding of video content with a wide picture aspect ratio within a taller cropped decoded picture, the indicated ccv_min_luminance_value, ccv_max_luminance_value, and ccv_avg_luminance_value should correspond only to values within the visually relevant region.

## Omnidirectional video specific SEI messages

### Sample location remapping process

##### General

Use of this process requires the definition of the following variable:

–	A chroma format indicator, denoted herein by ChromaFormatIdc, as described in clause 7.2..

To remap colour sample locations of a region-wise packed picture to a unit sphere, the following ordered steps are applied:

- A region-wise packed picture is obtained as the cropped decoded picture by decoding a coded picture. For purposes of interpretation of chroma samples, the input to the indicated remapping process is the set of decoded sample values after applying an (unspecified) upsampling conversion process to the 4:4:4 colour sampling format as necessary when ChromaFormatIdc is equal to 1 (4:2:0 chroma format) or 2 (4:2:2 chroma format). This (unspecified) upsampling process should account for the relative positioning relationship between the luma and chroma samples as indicated by vui_chroma_sample_loc_type_frame, vui_chroma_sample_loc_type_top_field, and vui_chroma_sample_loc_type_bottom_field syntax elements in the VUI parameters, when present.

- If region-wise packing is indicated, the sample locations of the region-wise packed picture are converted to sample locations of the respective projected picture as specified in clause 8.15.1.1.4. Otherwise, the projected picture is identical to the region-wise packed picture.

- If frame packing is indicated, the sample locations of the projected picture are converted to sample locations of the respective constituent picture of the projected picture, as specified in clause 8.15.1.1.5. Otherwise, the constituent picture of the projected picture is identical to the projected picture.

- The sample locations of a constituent picture of the projected picture are converted to sphere coordinates relative to the local coordinate axes, as specified in clause 8.15.1.1.2.

- If rotation is indicated, the sphere coordinates relative to the local coordinate axes are converted to sphere coordinates relative to the global coordinate axes, as specified in clause 8.15.1.1.3. Otherwise, the global coordinate axes are identical to the local coordinate axes.

The overall process for mapping of luma sample locations within a region-wise packed picture to sphere coordinates relative to the global coordinate axes is normatively specified in clause 8.15.1.1.5.

For each region-wise packed picture corresponding to a decoded picture, the following applies:

–	When an equirectangular projection SEI message with erp_cancel_flag equal to 0 that applies to the picture is present, ErpFlag is set equal to 1, and CmpFlag is set equal to 0.

–	When a generalized cubemap projection SEI message with gcmp_cancel_flag equal to 0 that applies to the picture is present, CmpFlag is set equal to 1, and ErpFlag is set equal to 0. [Ed. (YK): Consider making the wording in a generic way, such that this sphere rotation SEI message can also be used together with other projection SEI message, e.g., the cubemap projection SEI message specified in HEVC and AVC but not in this Specification.]

–	If a sphere rotation SEI message with sphere_rotation_cancel_flag equal to 0 that applies to the picture is present, RotationFlag is set equal to 1, and RotationYaw, RotationPitch, and RotationRoll are set equal to yaw_rotation ÷ 216, pitch_rotation ÷ 216, and roll_rotation ÷ 216, respectively.

–	Otherwise, RotationFlag is set equal to 0.

–	If a frame packing arrangement SEI message with frame_packing_arrangement_cancel_flag equal to 0 that applies to the picture is not present, StereoFlag, TopBottomFlag, and SideBySideFlag are all set equal to 0, HorDiv1 is set equal to 1, and VerDiv1 is set equal to 1.

–	Otherwise, the following applies:

–	StereoFlag is set equal to 1.

–	If the value of frame_packing_arrangement_type of the frame packing arrangement SEI message is equal to 3, TopBottomFlag is set equal to 0, SideBySideFlag is set equal to 1, HorDiv1 is set equal to 2 and VerDiv1 is set equal to 1.

–	Otherwise, if the value of frame_packing_arrangement_type of the frame packing arrangement SEI message is equal to 4, TopBottomFlag is set equal to 1, SideBySideFlag is set equal to 0, HorDiv1 is set equal to 1, and VerDiv1 is set equal to 2.

–	Otherwise, TopBottomFlag is set equal to 0, SideBySideFlag is set equal to 0, HorDiv1 is set equal to 1, and VerDiv1 is set equal to 1.

–	If a region-wise packing SEI message with rwp_cancel_flag equal to 0 that applies to the picture is not present, RegionWisePackingFlag is set equal to 0, and ConstituentPicWidth and ConstituentPicHeight are set to be equal to cropPicWidth / HorDiv1 and cropPicHeight / VerDiv1, respectively, where cropPicWidth and cropPicHeight are the width and height, respectively, of the cropped decoded picture.

–	Otherwise, RegionWisePackingFlag is set equal to 1, and ConstituentPicWidth and ConstituentPicHeight are set equal to proj_picture_width / HorDiv1 and proj_picture_height / VerDiv1, respectively.

##### Projection for one sample location

Inputs to this process are:

–	pictureWidth and pictureHeight, which are the width and height, respectively, of a monoscopic projected luma picture, in relative projected picture sample units, and [Ed. (GJS): What are these? They don’t seem properly specified.]

–	the centre point of a sample location (hPos, vPos) along the horizontal and vertical axes, respectively, in relative projected picture sample units, where hPos and vPos may have non-integer real values.

Outputs of this process are:

–	sphere coordinates (ϕ, θ) for the sample location in degrees relative to the local coordinate axes.

The projection for a sample location is derived as follows:

–	If ErpFlag is equal to 1, the following applies:

–	If RegionWisePackingFlag is equal to 0 and erp_guard_band_flag is equal to 1, the following applies:

hPos′ = hPos − erp_left_guard_band_width	(40)
pictureWidth = pictureWidth − erp_left_guard_band_width − erp_right_guard_band_width

–	Otherwise, the following applies:

hPos′ = hPos		(41)

–	The following applies:

ϕ = 180 − hPos′ * ( 360 ÷ pictureWidth )		(42)
θ = 90 − vPos * ( 180 ÷ pictureHeight )

–	Otherwise (CmpFlag is equal to 1), the outputs are derived by the following ordered steps:

- Clause 8.15.1.1.7 is invoked with pictureWidth and pictureHeight as inputs, and the output is assigned to faceWidth and faceHeight.

- Clause 8.15.1.1.8 is invoked with hPos, vPos, faceWidth, and faceHeight, where hPos and vPos are within a projected picture, and the output is assigned to hPosFace and vPosFace within a projected face.

- Clause 8.15.1.1.9 is invoked with hPosFace, vPosFace, faceWidth, and faceHeight, and the output is assigned to hPosRot and vPosRot.

- The following applies:

hPos′ = − ( 2 * hPosRot ÷ faceWidth ) + 1		(43)
vPos′ = − ( 2 * vPosRot ÷ faceHeight ) + 1

–	If gcmp_mapping_function_type is equal to 0, the following applies:

hPos′′ = hPos′		(44)
vPos′′ = vPos′

–	Otherwise, if gcmp_mapping_function_type is equal to 1, the following applies:

hPos′′ = Tan( hPos′ * π ÷ 4 )		(45)
vPos′′ = Tan( vPos′ * π ÷ 4 )

–	Otherwise (gcmp_mapping_function_type is equal to 2), the following applies:

coeffU[ n ] = ( gcmp_function_coeff_u[ n ] + 1 ) ÷ 128
coeffV[ n ] = ( gcmp_function_coeff_v[ n ] + 1 ) ÷ 128	(46)
hPos′′ = hPos′ ÷ ( 1 + coeffU[ n ] * ( 1 − gcmp_function_u_affected_by_v_flag[ n ] * vPos′2 ) *
			( 1 − hPos′2 ) )
vPos′′ = vPos′ ÷ ( 1 + coeffV[ n ] * ( 1 − gcmp_function_v_affected_by_u_flag[ n ] * hPos′2 ) *
			( 1 − vPos′2 ) )

–	The following applies:

if( gcmp_face_index[ n ]  = =  0 ) { /* positive x front face */
	x = 1.0
	y = hPos′′
	z = vPos′′
} else if( gcmp_face_index[ n ]  = =  1 ) { /* negative x back face */
	x = −1.0
	y = −vPos′′
	z = −hPos′′
} else if( gcmp_face_index[ n ]  = =  2 ) { /* positive z top face */	(47)
	x = −hPos′′
	y = −vPos′′
	z = 1.0
} else if( gcmp_face_index[ n ]  = =  3 ) { /* negative z bottom face */
	x = hPos′′
	y = −vPos′′
	z = −1.0
} else if( gcmp_face_index[ n ]  = =  5 ) { /* positive y left face */
	x = −hPos′′
	y = 1.0
	z = vPos′′
} else { /* ( gcmp_face_index[ n ]  = =  4 ), negative y right face */
	x = hPos′′
	y = −1.0
	z = vPos′′
}
ϕ = Atan2( y, x ) * 180 ÷ π
θ = Asinz ÷ x2 + y2 + z2 * 180 ÷ π

##### Conversion from the local coordinate axes to the global coordinate axes

Inputs to this process are:

–	rotation_yaw (αd), rotation_pitch (βd), rotation_roll (γd), all in units of degrees, and

–	sphere coordinates (ϕd, θd) relative to the local coordinate axes.

Outputs of this process are:

–	sphere coordinates (ϕ′, θ′) relative to the global coordinate axes.

The outputs are derived as follows:

ϕ = ϕd * π ÷ 180
θ = θd * π ÷ 180
α = αd * π ÷ 180
β = βd * π ÷ 180
γ = γd * π ÷ 180
x1 = Cos( ϕ ) * Cos( θ )
y1 = Sin( ϕ ) * Cos( θ )
z1 = Sin( θ )
x2 = Cos( β ) * Cos ( α ) * x1 − Cos( β ) * Sin( α ) * y1 + Sin( β ) * z1	(48)
y2 = ( Cos( γ ) * Sin( α ) + Sin( γ ) * Sin( β ) * Cos( α ) ) * x1 +
	( Cos( γ ) * Cos( α ) − Sin( γ ) * Sin( β ) * Sin( α ) ) * y1 −
	Sin( γ ) * Cos( β ) * z1
z2 = ( Sin( γ ) * Sin( α ) − Cos( γ ) * Sin( β ) * Cos( α ) ) * x1 +
	( Sin( γ ) * Cos( α ) + Cos( γ ) * Sin( β ) * Sin( α ) ) * y1 +
	Cos( γ ) * Cos( β ) * z1
ϕ′ = Atan2( y2, x2 ) * 180 ÷ π
θ′ = Asin( z2 ) * 180 ÷ π

##### Conversion of sample locations for rectangular region-wise packing

Inputs to this process are:

–	sample location (x, y) within the packed region, where x and y are in relative packed picture sample units, while the sample location is at an integer sample location within the packed picture,

–	the width and the height (projRegWidth, projRegHeight) of the projected region, in relative projected picture sample units,

–	the width and the height (packedRegWidth, packedRegHeight) of the packed region, in relative packed picture sample units,

–	transform type (transformType), and

–	offset values for the sampling position (offsetX, offsetY) in the range of 0, inclusive, to 1, exclusive, in horizontal and vertical relative packed picture sample units, respectively.

NOTE – offsetX and offsetY both equal to 0.5 indicate a sampling position that is in the centre point of a sample in packed picture sample units.

Outputs of this process are:

–	the centre point of the sample location (hPos, vPos) within the projected region in relative projected picture sample units, where hPos and vPos may have non-integer real values.

The outputs are derived as follows:

if( transformType  = =  0  | |  transformType  = =  1  | |  transformType  = =  2  | |  transformType  = =  3 ) {
	horRatio = projRegWidth ÷ packedRegWidth
	verRatio = projRegHeight ÷ packedRegHeight
} else if ( transformType  = =  4  | |  transformType  = =  5  | |  transformType  = =  6  | |
	transformType  = =  7 ) {
	horRatio = projRegWidth ÷ packedRegHeight
	verRatio = projRegHeight ÷ packedRegWidth
}
if( transformType  = =  0 ) {
	hPos = horRatio * ( x + offsetX )
	vPos = verRatio * ( y + offsetY )
} else if ( transformType  = =  1 ) {
	hPos = horRatio * ( packedRegWidth − x − offsetX )
	vPos = verRatio * ( y + offsetY )
} else if ( transformType  = =  2 ) {
	hPos = horRatio * ( packedRegWidth − x − offsetX )
	vPos = verRatio * ( packedRegHeight − y − offsetY )	(49)
} else if ( transformType  = =  3 ) {
	hPos = horRatio * ( x + offsetX )
	vPos = verRatio * ( packedRegHeight − y − offsetY )
} else if ( transformType  = =  4 ) {
	hPos = horRatio * ( y + offsetY )
	vPos = verRatio * ( x + offsetX )
} else if ( transformType  = =  5 ) {
	hPos = horRatio * ( y + offsetY )
	vPos = verRatio * ( packedRegWidth − x − offsetX )
} else if ( transformType  = = 6 ) {
	hPos = horRatio * ( packedRegHeight − y − offsetY )
	vPos = verRatio * ( packedRegWidth − x − offsetX )
} else if ( transformType  = =  7 ) {
	hPos = horRatio * ( packedRegHeight − y − offsetY )
	vPos = verRatio * ( x + offsetX )
}

##### Mapping of luma sample locations within a cropped decoded picture to sphere coordinates relative to the global coordinate axes

This clause specifies the semantics of luma sample locations within a cropped decoded picture to sphere coordinates relative to the global coordinate axes.

offsetX is set equal to 0.5 and offsetY is set equal to 0.5.

If RegionWisePackingFlag is equal to 1, the following applies for each packed region n in the range of 0 to NumPackedRegions − 1, inclusive:

–	For each sample location (xPackedPicture, yPackedPicture) belonging to the n-th packed region, the following applies:

–	The corresponding sample location (xProjPicture, yProjPicture) of the projected picture is derived as follows:

–	x is set equal to xPackedPicture − PackedRegionLeft[ n ].

–	y is set equal to yPackedPicture − PackedRegionTop[ n ].

–	Clause 8.15.1.1.4 is invoked with x, y, PackedRegionWidth[ n ], PackedRegionHeight[ n ], ProjRegionWidth[ n ], ProjRegionHeight[ n ], TransformType[ n ], offsetX and offsetY as inputs, and the output is assigned to sample location (hPos, vPos).

–	xProjPicture is set equal to ProjRegionLeft[ n ] + hPos.

–	When StereoFlag is equal to 0 or TopBottomFlag is equal to 1, and when xProjPicture is greater than or equal to proj_picture_width, xProjPicture is set equal to xProjPicture − proj_picture_width.

–	When SideBySideFlag is equal to 1, the following applies:

–	When ProjRegionLeft[ n ] is less than proj_picture_width / 2 and xProjPicture is greater than or equal to proj_picture_width / 2, xProjPicture is set equal to xProjPicture − proj_picture_width / 2.

–	When ProjRegionLeft[ n ] is greater than or equal to proj_picture_width / 2 and xProjPicture is greater than or equal to proj_picture_width, xProjPicture is set equal to xProjPicture − proj_picture_width / 2.

–	yProjPicture is set equal to ProjRegionTop[ n ] + vPos.

–	Clause 8.15.1.1.6 is invoked with xProjPicture, yProjPicture, ConstituentPicWidth, and ConstituentPicHeight as inputs, and the outputs indicating the sphere coordinates and the constituent picture index (for frame-packed stereoscopic video) for the luma sample location (xPackedPicture, yPackedPicture) belonging to the n-th packed region in the decoded picture.

Otherwise if RegionWisePackingFlag is equal 0 and CmpFlag is equal to 1, the following applies for each sample location (x, y) that is not a cubemap projection guard band sample within the cropped decoded picture:

–	xProjPicture is set equal to x + offsetX.

–	yProjPicture is set equal to y + offsetY.

–	Clause 8.15.1.1.6 is invoked with xProjPicture, yProjPicture, ConstituentPicWidth, and ConstituentPicHeight as inputs, and the outputs indicating the sphere coordinates and the constituent picture index (for frame-packed stereoscopic video) for the sample location (x, y) within the cropped decoded picture.

Otherwise (RegionWisePackingFlag is equal to 0, and CmpFlag is equal to 0), the following applies for each sample location (x, y) that is not an equirectangular projection guard band sample within the cropped decoded picture, where a sample location (x, y) is an equirectangular projection guard band sample when and only when ErpFlag is equal to 1, x is in the range of 0 to erp_left_guard_band_width − 1, inclusive, or ConstituentPicWidth − erp_right_guard_band_width to ConstituentPicWidth − 1, inclusive, and y is in the range of 0 to ConstituentPicHeight − 1, inclusive:

–	xProjPicture is set equal to x + offsetX.

–	yProjPicture is set equal to y + offsetY.

–	If ErpFlag is equal to 0, projPicWidth is set equal to ConstituentPicWidth. Otherwise (ErpFlag is equal to 1), projPicWidth is set equal to ConstituentPicWidth − ( erp_left_guard_band_width + erp_right_guard_band_width ).

–	Clause 8.15.1.1.6 is invoked with xProjPicture, yProjPicture, projPicWidth, and ConstituentPicHeight as inputs, and the outputs indicating the sphere coordinates and the constituent picture index (for frame-packed stereoscopic video) for the sample location (x, y) within the region-wise packed picture.

##### Conversion from a sample location in a projected picture to sphere coordinates relative to the global coordinate axes

Inputs to this process are:

–	the centre point of a sample location (xProjPicture, yProjPicture) within a projected picture, where xProjPicture and yProjPicture are in relative projected picture sample units and may have non-integer real values, and

–	pictureWidth and pictureHeight, which are the width and height, respectively, of a monoscopic projected luma picture, in relative projected picture sample units.

Outputs of this process are:

–	sphere coordinates (azimuthGlobal, elevationGlobal), in units of degrees relative to the global coordinate axes, and

–	when StereoFlag is equal to 1, the index of the constituent picture (constituentPicture) equal to 0 or 1.

The outputs are derived with the following ordered steps:

- constituentPicture, xProjPicture, and yProjPicture are conditionally set as follows:

–	If xProjPicture is greater than or equal to pictureWidth or yProjPicture is greater than or equal to pictureHeight, the following applies:

–	constituentPicture is set equal to 1.

–	When xProjPicture is greater than or equal to pictureWidth, xProjPicture is set to xProjPicture − pictureWidth.

–	When yProjPicture is greater than or equal to pictureHeight, yProjPicture is set to yProjPicture − pictureHeight.

–	Otherwise, constituentPicture is set equal to 0.

- Clause 8.15.1.1.2 is invoked with pictureWidth, pictureHeight, xProjPicture, and yProjPicture as inputs, and the output is assigned to azimuthLocal, elevationLocal.

- azimuthGlobal and elevationGlobal are set as follows:

–	If RotationFlag is equal to 1, clause 8.15.1.1.3 is invoked with azimuthLocal, elevationLocal, RotationYaw, RotationPitch, and RotationRoll as inputs, and the output is assigned to azimuthGlobal and elevationGlobal.

–	Otherwise, azimuthGlobal is set equal to azimuthLocal and elevationGlobal is set equal to elevationLocal.

##### Calculation of the cubemap face size for a projected picture

Inputs to this process are:

–	pictureWidth and pictureHeight, which are the width and height, respectively, of a monoscopic projected luma picture, in relative projected picture sample units.

Outputs of this process are:

–	faceWidth and faceHeight, which are the width and height, respectively, of a projected face, in relative projected picture sample units.

The outputs are derived as follows:

gcmpPicWidth = pictureWidth
gcmpPicHeight = pictureHeight

gcmpGuardBandSamples = gcmp_guard_band_samples_minus1 + 1
if( gcmp_guard_band_flag  &&  gcmp_guard_band_boundary_exterior_flag ) {
	cmpPicWidth = pictureWidth − 2 * gcmpGuardBandSamples
	cmpPicHeight = pictureHeight − 2 * gcmpGuardBandSamples
}
if( gcmp_packing_type  = =  0 ) {
	if( gcmp_guard_band_flag )
		gcmpPicHeight = gcmpPicHeight − 2 * gcmpGuardBandSamples
	faceWidth = gcmpPicWidth
	faceHeight = gcmpPicHeight / 6
} else if( gcmp_packing_type  = =  1 ) {
	if( gcmp_guard_band_flag )
		cmpPicWidth = cmpPicWidth − 2 * gcmpGuardBandSamples
	faceWidth = cmpPicWidth / 2
	faceHeight = cmpPicHeight / 3
} else if( gcmp_packing_type  = =  2 ) {
	if( gcmp_guard_band_flag )
		cmpPicHeight = cmpPicHeight − 2 * gcmpGuardBandSamples
	faceWidth = cmpPicWidth / 3
	faceHeight = cmpPicHeight / 2		(50)
} else if( gcmp_packing_type  = =  3 ) {
	if( gcmp_guard_band_flag )
		cmpPicWidth = cmpPicWidth − 2 * gcmpGuardBandSamples
	faceWidth = cmpPicWidth / 6
	faceHeight = cmpPicHeight
} else if( gcmp_packing_type  = =  4 ) {
	if( gcmp_guard_band_flag )
		cmpPicWidth = cmpPicWidth − 2 * gcmpGuardBandSamples
	faceWidth = cmpPicWidth / 3
	faceHeight = cmpPicHeight
} else if( gcmp_packing_type  = =  5 ) {
	if( gcmp_guard_band_flag )
		cmpPicHeight = cmpPicHeight − 2 * gcmpGuardBandSamples
	faceWidth = cmpPicWidth
	faceHeight = cmpPicHeight / 3 [Ed. (GJS): Why is this using a “/” type of division? Is some constraint needed here, such as making sure the cmpPicHeight is a multiple of 3 (or a multimple of 6 when ChromaFormatIdc is equal to 1)?]
}

The values of faceWidth and faceHeight are constrained as follows:

–	When ChromaFormatIdc is equal to 1 (4:2:0 chroma format) or 2 (4:2:2 chroma format), faceWidth shall correspond to an even number of luma samples, both within the decoded picture.

–	When ChromaFormatIdc is equal to 1 (4:2:0 chroma format), faceHeight shall correspond to an even number of luma samples, both within the decoded picture. [Ed. (GJS): What does it mean to “correspond to an even number of luma samples”, and why does this say “both”?]

It is a requirement of bitstream conformance that the following constraints apply:

–	If gcmp_packing_type is equal to 0, cmpPicHeight shall be a multiple of 6, and cmpPicWidth shall be equal to cmpPicHeight / 6.

–	Otherwise, if gcmp_packing_type is equal to 1, cmpPicWidth shall be a multiple of 2 and cmpPicHeight shall be a multiple of 3, and cmpPicWidth / 2 shall be equal to cmpPicHeight / 3.

–	Otherwise, if gcmp_packing_type is equal to 2, cmpPicWidth shall be a multiple of 3 and cmpPicHeight shall be a multiple of 2, and cmpPicWidth / 3 shall be equal to cmpPicHeight / 2.

–	Otherwise, if gcmp_packing_type is equal to 3, cmpPicWidth shall be a multiple of 6, and cmpPicWidth / 6 shall be equal to cmpPicHeight.

–	Otherwise, if gcmp_packing_type is equal to 4, cmpPicWidth shall be a multiple of 6, and cmpPicWidth / 3 shall be equal to cmpPicHeight.

–	Otherwise, if gcmp_packing_type is equal to 5, cmpPicHeight shall be a multiple of 6, and cmpPicWidth shall be equal to cmpPicHeight / 3.

##### Conversion from a sample location in a projected picture to a sample location in a projected cubemap face

Inputs to this process are:

–	sample location (hPos, vPos) within the projected picture in relative projected picture sample units, where hPos and vPos may have non-integer real values, and

–	faceWidth and faceHeight, which are the width and height, respectively, of the projected face, in relative projected picture sample units.

Outputs of this process are:

–	the sample location (hPosFace, vPosFace) within the projected face in relative projected picture sample units, where hPosFace and vPosFace may have non-integer real values.

The outputs are derived as follows:

gbSize = gcmpGuardBandSamples
tmpHorPos = hPos
tmpVerPos = vPos
if( gcmp_guard_band_flag ) {
	if( gcmp_guard_band_boundary_exterior_flag ) {
		tmpHorPos = hPos − gbSize
		tmpVerPos = vPos − gbSize
	}
	if( gcmp_packing_type  = =  0 )
		tmpVerPos = tmpVerPos < 3 * faceHeight ? tmpVerPos : tmpVerPos − 2 * gbSize
	else if( gcmp_packing_type  = =  1 )
		tmpHorPos = tmpHorPos < faceWidth ? tmpHorPos : tmpHorPos − 2 * gbSize
	else if( gcmp_packing_type  = =  2 )
		tmpVerPos = tmpVerPos < faceHeight ? tmpVerPos : tmpVerPos − 2 * gbSize	(51)
	else if( gcmp_packing_type  = =  3 )
		tmpHorPos = tmpHorPos < 3 * faceWidth ? tmpHorPos : tmpHorPos − 2 * gbSize
	else if( gcmp_packing_type  = =  4 )
		tmpHorPos = tmpHorPos < faceWidth / 2 ? tmpHorPos : tmpHorPos < 2.5 * faceWidth + gbSize ?
				tmpHorPos − gbSize : tmpHorPos − 2 * gbSize
	else if( gcmp_packing_type  = =  5 )
		tmpVerPos = tmpVerPos < faceHeight / 2 ? tmpVerPos : tmpVerPos < 2.5 * faceHeight + gbSize ?
				tmpVerPos − gbSize : tmpVerPos − 2 * gbSize
}
w = Floor( tmpHorPos ÷ faceWidth )
h = Floor( tmpVerPos ÷ faceHeight )
hPosFace = tmpHorPos − w * faceWidth
vPosFace = tmpVerPos − h * faceHeight

##### Rotation of sample locations for a projected cubemap face

Inputs to this process are:

–	sample location (hPosFace, vPosFace) within the n-th projected face in relative projected picture sample units, where hPosFace and vPosFace may have non-integer real values, and

–	faceWidth and faceHeight, which are the width and height, respectively, of the projected face, in relative projected picture sample units.

Outputs of this process are:

–	the rotated sample location (hPosRot, vPosRot) within the projected face in relative projected picture sample units, where hPosRot and vPosRot may have non-integer real values.

The outputs are derived as follows:

if( gcmp_face_rotation[ n ]  = =  0 ) {
	hPosRot = hPosFace
	vPosRot = vPosFace
} else if( gcmp_face_rotation[ n ]  = =  1 ) {
	hPosRot = vPosFace
	vPosRot = faceWidth − hPosFace		(52)
} else if( gcmp_face_rotation[ n ]  = =  2 ) {
	hPosRot = faceWidth − hPosFace
	vPosRot = faceHeight − vPosFace
} else if( gcmp_face_rotation[ n ]  = =  3 ) {
	hPosRot = faceHeight − vPosFace
	vPosRot = hPosFace
}

### Equirectangular projection SEI message

#### Equirectangular projection SEI message syntax

| equirectangular_projection( payloadSize ) { | Descriptor |
| --- | --- |
| erp_cancel_flag | u(1) |
| if( !erp_cancel_flag ) { |  |
| erp_persistence_flag | u(1) |
| erp_guard_band_flag | u(1) |
| erp_reserved_zero_2bits | u(2) |
| if( erp_guard_band_flag = = 1 ) { |  |
| erp_guard_band_type | u(3) |
| erp_left_guard_band_width | u(8) |
| erp_right_guard_band_width | u(8) |
| } |  |
| } |  |
| } |  |

#### Equirectangular projection SEI message semantics

The equirectangular projection SEI message provides information to enable remapping (through an equirectangular projection) of the colour samples of the projected pictures onto a sphere coordinate space in sphere coordinates (ϕ, θ) for use in omnidirectional video applications for which the viewing perspective is from the origin looking outward toward the inside of the sphere. The sphere coordinates are defined so that ϕ is the azimuth (longitude, increasing eastward) and θ is the elevation (latitude, increasing northward).

Use of this SEI message requires the definition of the following variable:

–	A chroma format indicator, denoted herein by ChromaFormatIdc, as described in clause 7.2.

When an equirectangular projection SEI message is present for any picture of a CLVS, an equirectangular projection SEI message shall be present for the first picture of the CLVS and no SEI message indicating a different type of projection shall be present for any picture of the CLVS.

When vui_aspect_ratio_idc is present and greater than 1 for a CLVS, there should be no equirectangular projection SEI messages applicable for any picture of the CLVS. [Ed. (YK): The interaction with the sample aspect ratio information SEI message needs to be clarified. (GJS) If there is one there, what happens?]

A frame packing arrangement SEI message for which all the following conditions are true is referred to as an effectively applicable frame packing arrangement SEI message:

–	The value of frame_packing_arrangement_cancel_flag is equal to 0.

–	The value of frame_packing_arrangement_type is equal to 3, 4, or 5.

–	The value of quincunx_sampling_flag is equal to 0.

–	The value of spatial_flipping_flag is equal to 0.

–	The value of field_views_flag is equal to 0.

–	The value of frame0_grid_position_x is equal to 0.

–	The value of frame0_grid_position_y is equal to 0.

–	The value of frame1_grid_position_x is equal to 0.

–	The value of frame1_grid_position_y is equal to 0.

When a frame packing arrangement SEI message with frame_packing_arrangement_cancel_flag equal to 0 that applies to the picture is present that is not an effectively applicable frame packing arrangement SEI message, an equirectangular projection SEI message with erp_cancel_flag equal to 0 that applies to the picture shall not be present. Decoders shall ignore equirectangular projection SEI messages when a frame packing arrangement SEI message with frame_packing_arrangement_cancel_flag equal to 0 that applies to the picture is present that is not an effectively applicable frame packing arrangement SEI message.

erp_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous equirectangular projection SEI message in output order. erp_cancel_flag equal to 0 indicates that equirectangular projection information follows.

erp_persistence_flag specifies the persistence of the equirectangular projection SEI message for the current layer.

erp_persistence_flag equal to 0 specifies that the equirectangular projection SEI message applies to the current decoded picture only.

erp_persistence_flag equal to 1 specifies that the equirectangular projection SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with an equirectangular projection SEI message is output that follows the current picture in output order.

erp_guard_band_flag equal to 1 indicates that the constituent picture contains guard band areas for which the sizes are specified by the syntax elements erp_left_guard_band_width and erp_right_guard_band_width. erp_guard_band_flag equal to 0 indicates that the constituent picture does not contains guard band areas for which the sizes are specified by the syntax elements erp_left_guard_band_width and erp_right_guard_band_width.

erp_reserved_zero_2bits shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for erp_reserved_zero_2bits are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of erp_reserved_zero_2bits.

erp_guard_band_type indicates the type of the guard bands as follows:

–	erp_guard_band_type equal to 0 indicates that the content of the guard band in relation to the content of the constituent picture is unspecified.

–	erp_guard_band_type equal to 1 indicates that the content of the guard band suffices for interpolation of sample values at sub-pel sample fractional locations within the constituent picture.

NOTE – erp_guard_band_type equal to 1 could be used when the boundary samples of a constituent picture have been copied horizontally to the guard band.

–	erp_guard_band_type equal to 2 indicates that the content of the guard band represents actual picture content at a quality that gradually changes from the picture quality of the constituent picture to that of the spherically adjacent region.

–	erp_guard_band_type equal to 3 indicates that the content of the guard bands represents actual picture content at a similar level of quality as the constituent picture.

–	erp_guard_band_type values greater than 3 are reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value of erp_guard_band_type when the value is greater than 3 as equivalent to the value 0.

erp_left_guard_band_width specifies the width of the guard band on the left side of the constituent picture in units of luma samples. When erp_guard_band_flag is equal to 0, the value of erp_left_guard_band_width is inferred to be equal to 0. When ChromaFormatIdc is equal to 1 (4:2:0 chroma format) or 2 (4:2:2 chroma format), erp_left_guard_band_width shall be an even number.

erp_right_guard_band_width specifies the width of the guard band on the right side of the constituent picture in units of luma samples. When erp_guard_band_flag is equal to 0, the value of erp_right_guard_band_width is inferred to be equal to 0. When ChromaFormatIdc is equal to 1 (4:2:0 chroma format) or 2 (4:2:2 chroma format), erp_right_‌guard_band_width shall be an even number.

### Generalized cubemap projection SEI message

#### Generalized cubemap projection SEI message syntax

| generalized_cubemap_projection( payloadSize ) { | Descriptor |
| --- | --- |
| gcmp_cancel_flag | u(1) |
| if( !gcmp_cancel_flag ) { |  |
| gcmp_persistence_flag | u(1) |
| gcmp_packing_type | u(3) |
| gcmp_mapping_function_type | u(2) |
| for( i = 0; i < ( gcmp_packing_type = = 4 ¦ ¦ gcmp_packing_type = = 5 ) ? 5 : 6; i++ ) { |  |
| gcmp_face_index[ i ] | u(3) |
| gcmp_face_rotation[ i ] | u(2) |
| if( gcmp_mapping_function_type = = 2 ) { |  |
| gcmp_function_coeff_u[ i ] | u(7) |
| gcmp_function_u_affected_by_v_flag[ i ] | u(1) |
| gcmp_function_coeff_v[ i ] | u(7) |
| gcmp_function_v_affected_by_u_flag[ i ] | u(1) |
| } |  |
| } |  |
| gcmp_guard_band_flag | u(1) |
| if( gcmp_guard_band_flag ) { |  |
| gcmp_guard_band_type | u(3) |
| gcmp_guard_band_boundary_exterior_flag | u(1) |
| gcmp_guard_band_samples_minus1 | u(4) |
| } |  |
| } |  |
| } |  |

#### Generalized cubemap projection SEI message semantics

The generalized cubemap projection SEI message provides information to enable remapping (through a generalized cubemap projection) of the colour samples of the projected pictures onto a sphere coordinate space in sphere coordinates (ϕ, θ) for use in omnidirectional video applications for which the viewing perspective is from the origin looking outward toward the inside of the sphere. The sphere coordinates are defined so that ϕ is the azimuth (longitude, increasing eastward) and θ is the elevation (latitude, increasing northward).

Use of this SEI message requires the definition of the following variable:

–	A chroma format indicator, denoted herein by ChromaFormatIdc, as described in clause 7.2.

When a generalized cubemap projection SEI message is present for any picture of a CLVS, a generalized cubemap projection SEI message shall be present for the first picture of the CLVS and no SEI message indicating a different type of projection shall be present for any picture of the CLVS.

When vui_aspect_ratio_idc is present and greater than 1 for a CLVS, there should be no generalized cubemap projection SEI messages applicable for any picture of the CLVS. [Ed. (YK): The interaction with the sample aspect ratio information SEI message needs to be clarified. (GJS) If there is one there, what happens?]

A frame packing arrangement SEI message for which all the following conditions are true is referred to as an effectively applicable frame packing arrangement SEI message:

–	The value of frame_packing_arrangement_cancel_flag is equal to 0.

–	The value of frame_packing_arrangement_type is equal to 3, 4, or 5.

–	The value of quincunx_sampling_flag is equal to 0.

–	The value of spatial_flipping_flag is equal to 0.

–	The value of field_views_flag is equal to 0.

–	The value of frame0_grid_position_x is equal to 0.

–	The value of frame0_grid_position_y is equal to 0.

–	The value of frame1_grid_position_x is equal to 0.

–	The value of frame1_grid_position_y is equal to 0.

When a frame packing arrangement SEI message with frame_packing_arrangement_cancel_flag equal to 0 that applies to the picture is present that is not an effectively applicable frame packing arrangement SEI message, a generalized cubemap projection SEI message with gcmp_cancel_flag equal to 0 that applies to the picture shall not be present. Decoders shall ignore generalized cubemap projection SEI messages when a frame packing arrangement SEI message with frame_packing_arrangement_cancel_flag equal to 0 that applies to the picture is present that is not an effectively applicable frame packing arrangement SEI message.

gcmp_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous generalized cubemap projection SEI message in output order. gcmp_cancel_flag equal to 0 indicates that cubemap projection information follows.

gcmp_persistence_flag specifies the persistence of the generalized cubemap projection SEI message for the current layer.

gcmp_persistence_flag equal to 0 specifies that the generalized cubemap projection SEI message applies to the current decoded picture only.

gcmp_persistence_flag equal to 1 specifies that the generalized cubemap projection SEI message applies to the current decoded picture and persists all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with a cubemap projection SEI message is output that follows the current picture in output order.

gcmp_packing_type specifies the packing type and the position index of the cubemap packing as specified in Table 10. When the value of gcmp_packing_type is in the range of 0 to 3, inclusive, cubemap packing with six faces is used. When gcmp_packing_type is 4 or 5, hemisphere cubemap packing with one full face and four half faces is used. The value of gcmp_packing_type shall be in the range of 0 to 5, inclusive. Other values for gcmp_packing_type are reserved for future use by ITU-T | ISO/IEC.

Table 10 – Specification of packing type and position index based on gcmp_packing_type

| gcmp_packing_type | Packing type and position index |
| --- | --- |
| 0 |  |
| 1 |  |
| 2 |  |
| 3 |  |
| 4 |  |
| 5 |  |

gcmp_mapping_function_type specifies the mapping function used to adjust the sample locations of the cubemap projection. gcmp_mapping_function_type equal to 0 indicates that the conventional cubemap projection as specified by the cubemap projection SEI message in Rec. ITU-T H.265 | ISO/IEC 23008-2 and Rec. ITU-T H.264 | ISO/IEC 14496-10, is used. gcmp_mapping_function_type equal to 1 indicates that the equi-angular mapping function is applied to adjust the sample locations of the projected face, as defined in clause 8.15.1.1.2. gcmp_mapping_function_type equal to 2 indicates that the coeffients of the mapping function applied to adjust the sample locations of the i-th projected face are specified by the syntax elements gcmp_function_coeff_u[ i ], gcmp_function_u_affected_by_v_flag[ i ], gcmp_function_coeff_v[ i ], and gcmp_function_v_affected_by_u_flag[ i ]. The value of gcmp_mapping_function_type shall be in the range of 0 to 2, inclusive.

When a region-wise packing SEI message with rwp_cancel_flag equal to 0 that applies to a picture is present, and a generalized cubemap projection SEI message with gcmp_cancel_flag equal to 0 that applies to the same picture is also present, the value of gcmp_mapping_function_type of the generalized cubemap projection SEI message shall be equal to 0.

gcmp_face_index[ i ] specifies the face index for position index i in gcmp_packing_type and the relationship between the global coordintes 3D (X, Y, Z) and the local coordinate 2D (u, v) as specified in clause 8.15.1.1.2.

gcmp_face_rotation[ i ] specifies the rotation to be applied to the face on position index i as specified in Table 11.

Table 11 – Specification of counterclockwise rotation angle based on gcmp_face_rotation[ i ]

| gcmp_face_rotation[ i ] | Rotation angle in degree (anticlockwise) |
| --- | --- |
| 0 | 0 |
| 1 | 90 |
| 2 | 180 |
| 3 | 270 |

gcmp_function_coeff_u[ i ] specifies the coefficient used in the cubemap mapping function of the u-axis of the i-th face. When gcmp_function_coeff_u[ i ] is not present, it is inferred to be equal to 0.

gcmp_function_u_affected_by_v_flag[ i ] equal to 1 indicates that the cubemap mapping function of the u-axis refers to the v position of the sample location. gcmp_function_u_affected_by_v_flag[ i ] equal to 0 indicates that the cubemap mapping function in u-axis does not refer to the v position of the sample location.

gcmp_fucntion_coeff_v[ i ] specifies the coefficient used in the cubemap mapping function of the v-axis of the i-th face. When gcmp_function_coeff_v[ i ] is not present, it is inferred to be equal to 0.

gcmp_function_v_affected_by_u_flag[ i ] equal to 1 indicates that the cubemap mapping function of the v-axis refers to the u position of the sample location. gcmp_function_v_affected_by_u_flag[ i ] equal to 0 indicates that the cubemap mapping function in v-axis does not refer to the u position of the sample location.

gcmp_guard_band_flag equal to 0 indicates that the coded picture does not contain guard band areas. cmp_guard_band_flag equal to 1 indicates that the coded picture contains guard band areas for which the sizes are specified by the syntax element gcmp_guard_band_samples_minus1.

gcmp_guard_band_type indicates the type of the guard bands as follows:

–	gcmp_guard_band_type equal to 0 indicates that the content of the guard bands in relation to the content of the coded face is unspecified.

–	gcmp_guard_band_type equal to 1 indicates that the content of the guard bands suffices for interpolation of sample values at sub-pel sample fractional locations within the coded face.

NOTE – gcmp_guard_band_type equal to 1 can be used when the boundary samples of a coded face have been copied horizontally or vertically to the guard band.

–	gcmp_guard_band_type equal to 2 indicates that the content of the guard bands represents actual picture content that is spherically adjacent to the content in the coded face at quality that gradually changes from the picture quality of the coded face to that of the spherically adjacent region.

–	gcmp_guard_band_type equal to 3 indicates that the content of the guard bands represents actual picture content that is spherically adjacent to the content in the coded face at a similar picture quality as within the coded face.

–	gcmp_guard_band_type values greater than 3 are reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value of gcmp_guard_band_type when the value is greater than 3 as equivalent to the value 0.

gcmp_guard_band_boundary_exterior_flag indicates which face boundaries contain guard bands, as specified in Table 12.

Table 12– Specification of guard band boundary location based on gcmp_packing_type and gcmp_guard_band_boundary_exterior_flag

| gcmp_packing_type | gcmp_guard_band_boundary_exterior_flag | Location of guard band |
| --- | --- | --- |
| 0 | 0 |  |
| 0 | 1 |  |
| 1 | 0 |  |
| 1 | 1 |  |
| 2 | 0 |  |
| 2 | 1 |  |
| 3 | 0 |  |
| 3 | 1 |  |
| 4 | 0 |  |
| 4 | 1 |  |
| 5 | 0 |  |
| 5 | 1 |  |

gcmp_guard_band_samples_minus1 plus 1 specifies the number of guard band samples, in units of luma samples, used in the cubemap projected picture. When ChromaFormatIdc is equal to 1 (4:2:0 chroma format) or 2 (4:2:2 chroma format), gcmp_guard_band_samples_minus1 plus 1 shall correspond to an even number of luma samples within the cropped decoded picture.

### Sphere rotation SEI message

#### Sphere rotation SEI message syntax

| sphere_rotation( payloadSize ) { | Descriptor |
| --- | --- |
| sphere_rotation_cancel_flag | u(1) |
| if( !sphere_rotation_cancel_flag ) { |  |
| sphere_rotation_persistence_flag | u(1) |
| sphere_rotation_reserved_zero_6bits | u(6) |
| yaw_rotation | i(32) |
| pitch_rotation | i(32) |
| roll_rotation | i(32) |
| } |  |
| } |  |

#### Sphere rotation SEI message semantics

The sphere rotation SEI message provides information on rotation angles yaw (α), pitch (β), and roll (γ) that are used for conversion between the global coordinate axes and the local coordinate axes.

Relative to an (x, y, z) Cartesian coordinate system, yaw expresses a rotation around the z (vertical, up) axis, pitch rotates around the y (lateral, side-to-side) axis, and roll rotates around the x (back-to-front) axis. Rotations are extrinsic, i.e., around x, y, and z fixed reference axes. The angles increase clockwise when looking from the origin towards the positive end of an axis.

sphere_rotation_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous sphere rotation SEI message in output order. sphere_rotation_cancel_flag equal to 0 indicates that sphere rotation information follows.

sphere_rotation_persistence_flag specifies the persistence of the sphere rotation SEI message for the current layer.

sphere_rotation_persistence_flag equal to 0 specifies that the sphere rotation SEI message applies to the current decoded picture only.

sphere_rotation_persistence_flag equal to 1 specifies that the sphere rotation SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with a sphere rotation SEI message is output that follows the current picture in output order.

When an equirectangular projection SEI message with erp_cancel_flag equal to 0 or a generelized cubemap projection SEI message with gcmp_cancel_flag equal to 0 is not present in the CLVS that applies to the current picture and precedes the sphere rotation SEI message in decoding order, a sphere rotation SEI message with sphere_rotation_cancel_flag equal to 0 shall not be present in the CLVS that applies to the current picture. Decoders shall ignore sphere rotation SEI messages with sphere_rotation_cancel_flag equal to 0 that do not follow, in decoding order, an equirectangular projection SEI message with erp_cancel_flag equal to 0 or a genearlized cubemap projection SEI message with gcmp_cancel_flag equal to 0 in the CLVS that applies to the current picture. [Ed. (YK): Consider making the wording in a generic way, such that this sphere rotation SEI message can also be used together with other projection SEI message, e.g., the cubemap projection SEI message specified in HEVC and AVC but not in this Specification.]

sphere_rotation_reserved_zero_6bits shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for sphere_rotation_reserved_zero_6bits are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of sphere_rotation_reserved_zero_6bits.

yaw_rotation specifies the value of the yaw rotation angle, in units of 2−16 degrees. The value of yaw_rotation shall be in the range of −180 * 216 (i.e., −11 796 480) to 180 * 216 − 1 (i.e., 11 796 479), inclusive. When not present, the value of yaw_rotation is inferred to be equal to 0.

pitch_rotation specifies the value of the pitch rotation angle, in units of 2−16 degrees. The value of pitch_rotation shall be in the range of −90 * 216 (i.e., −5 898 240) to 90 * 216 (i.e., 5 898 240), inclusive. When not present, the value of pitch_rotation is inferred to be equal to 0.

roll_rotation specifies the value of the roll rotation angle, in units of 2−16 degrees. The value of roll_rotation shall be in the range of −180 * 216 (i.e., −11 796 480) to 180 * 216 − 1 (i.e., 11 796 479), inclusive. When not present, the value of roll_rotation is inferred to be equal to 0.

### Region-wise packing SEI message

#### Region-wise packing SEI message syntax

| regionwise_packing( payloadSize ) { | Descriptor |
| --- | --- |
| rwp_cancel_flag | u(1) |
| if( !rwp_cancel_flag ) { |  |
| rwp_persistence_flag | u(1) |
| constituent_picture_matching_flag | u(1) |
| rwp_reserved_zero_5bits | u(5) |
| num_packed_regions | u(8) |
| proj_picture_width | u(32) |
| proj_picture_height | u(32) |
| packed_picture_width | u(16) |
| packed_picture_height | u(16) |
| for( i = 0; i < num_packed_regions; i++ ) { |  |
| rwp_reserved_zero_4bits[ i ] | u(4) |
| rwp_transform_type[ i ] | u(3) |
| rwp_guard_band_flag[ i ] | u(1) |
| proj_region_width[ i ] | u(32) |
| proj_region_height[ i ] | u(32) |
| proj_region_top[ i ] | u(32) |
| proj_region_left[ i ] | u(32) |
| packed_region_width[ i ] | u(16) |
| packed_region_height[ i ] | u(16) |
| packed_region_top[ i ] | u(16) |
| packed_region_left[ i ] | u(16) |
| if( rwp_guard_band_flag[ i ] ) { |  |
| rwp_left_guard_band_width[ i ] | u(8) |
| rwp_right_guard_band_width[ i ] | u(8) |
| rwp_top_guard_band_height[ i ] | u(8) |
| rwp_bottom_guard_band_height[ i ] | u(8) |
| rwp_guard_band_not_used_for_pred_flag[ i ] | u(1) |
| for( j = 0; j < 4; j++ ) |  |
| rwp_guard_band_type[ i ][ j ] | u(3) |
| rwp_guard_band_reserved_zero_3bits[ i ] | u(3) |
| } |  |
| } |  |
| } |  |
| } |  |

#### Region-wise packing SEI message semantics

The region-wise packing SEI message provides information to enable remapping of the colour samples of the cropped decoded pictures onto projected pictures as well as information on the location and size of the guard bands, if any.

Use of this SEI message requires the definition of the following variable:

–	A chroma format indicator, denoted herein by ChromaFormatIdc, as described in clause 7.2.

rwp_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous region-wise packing SEI message in output order. rwp_cancel_flag equal to 0 indicates that region-wise packing information follows.

rwp_persistence_flag specifies the persistence of the region-wise packing SEI message for the current layer.

rwp_persistence_flag equal to 0 specifies that the region-wise packing SEI message applies to the current decoded picture only.

rwp_persistence_flag equal to 1 specifies that the region-wise packing SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with a region-wise packing SEI message is output that follows the current picture in output order.

When an equirectangular projection SEI message with erp_cancel_flag equal to 0 and erp_guard_band_flag equal to 0 or a generalized cubemap projection SEI message with gcmp_cancel_flag equal to 0 is not present in the CLVS that applies to the current picture and precedes the region-wise packing SEI message in decoding order, a region-wise packing SEI message with rwp_cancel_flag equal to 0 shall not be present in the CLVS that applies to the current picture. Decoders shall ignore region-wise packing SEI messages with rwp_cancel_flag equal to 0 that do not follow, in decoding order, an equirectangular projection SEI message with erp_cancel_flag equal to 0 or a generalized cubemap projection SEI message with gcmp_cancel_flag equal to 0 in the CLVS that applies to the current picture. [Ed. (YK): Consider making the wording in a generic way, such that this sphere rotation SEI message can also be used together with other projection SEI message, e.g., the cubemap projection SEI message specified in HEVC and AVC but not in this Specification.]

[Ed. (RS): restrictions on the generalized cubemap projection SEI message for use with the region-wise packing SEI message should be added, e.g. cmp_packing_type shall be equal to 2, cmp_padding_enabled_flag shall be equal to 0, values of face_rotation[ i ] and face_index[ i ] should result in the AVC/HEVC cubemap layout.]

For the frame packing arrangement scheme indicated by a frame packing arrangement SEI message that applies to the current picture, if a region-wise packing SEI message with rwp_cancel_flag equal to 0 is present that applies to the current picture, the frame packing arrangement scheme applies to the projected picture, otherwise, the frame packing arrangement scheme applies to the cropped decoded picture.

If a frame packing arrangement SEI message with frame_packing_arrangement_cancel_flag equal to 0, frame_packing_arrangement_type equal to 3, 4, or 5, and quincunx_sampling_flag equal to 0 is not present that applies to the current picture, the variables StereoFlag, TopBottomFlag, SideBySideFlag, and TempInterleavingFlag are all set equal to 0, the variables HorDiv1 and VerDiv1 are both set equal to 1. Otherwise the following applies:

–	StereoFlag is set equal to 1.

–	When the frame_packing_arrangement_type is equal to 3, SideBySideFlag is set equal to 1, TopBottomFlag and TempInterleavingFlag are both set equal to 0, HorDiv1 is set equal to 2 and VerDiv1 is set equal to 1.

–	When the frame_packing_arrangement_type is equal to 4, TopBottomFlag is set equal to 1, SideBySideFlag and TempInterleavingFlag are both set equal to 0, HorDiv1 is set equal to 1 and VerDiv1 is set equal to 2.

–	When the frame_packing_arrangement_type is equal to 5, TempInterleavingFlag is set equal to 1, TopBottomFlag and SideBySideFlag are both set equal to 0, HorDiv1 and VerDiv1 are both set equal to 1.

constituent_picture_matching_flag equal to 1 specifies that the projected region information, packed region information, and guard band region information in this SEI message apply individually to each constituent picture and that the packed picture and the projected picture have the same stereoscopic frame packing format indicated by the frame packing arrangement SEI message. constituent_picture_matching_flag equal to 0 specifies that the projected region information, packed region information, and guard band region information in this SEI message apply to the projected picture.

When either of the following conditions is true, the value of constituent_picture_matching_flag shall be equal to 0:

–	StereoFlag is equal to 0.

–	StereoFlag is equal to 1 and frame_packing_arrangement_type is equal to 5.

rwp_reserved_zero_5bits shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for rwp_reserved_zero_56bits[ i ] are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of rwp_reserved_zero_5bits[ i ].

num_packed_regions specifies the number of packed regions when constituent_picture_matching_flag is equal to 0. The value of num_packed_regions shall be greater than 0. When constituent_picture_matching_flag is equal to 1, the total number of packed regions is equal to num_packed_regions * 2, and the information in each entry of the loop of num_packed_regions entries applies to each constituent picture of the projected picture and the packed picture.

proj_picture_width and proj_picture_height specify the width and height, respectively, of the projected picture, in relative projected picture sample units.

The values of proj_picture_width and proj_picture_height shall both be greater than 0.

packed_picture_width and packed_picture_height specify the width and height, respectively, of the packed picture, in relative packed picture sample units.

The values of packed_picture_width and packed_picture_height shall both be greater than 0.

It is a requirement of bitstream conformance that packed_picture_width and packed_picture_height shall have such values that packed_picture_width is an integer multiple of cropPicWidth and packed_picture_height is an integer multiple of cropPicHeight, where cropPicWidth and cropPicHeight are the width and height, respectively, of the cropped decoded picture.

rwp_reserved_zero_4bits[ i ] shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for rwp_reserved_zero_4bits[ i ] are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of rwp_reserved_zero_4bits[ i ].

rwp_transform_type[ i ] specifies the rotation and mirroring to be applied to the i-th packed region to remap to the i-th projected region. When rwp_transform_type[ i ] specifies both rotation and mirroring, rotation applies before mirroring. The values of rwp_transform_type[ i ] are specified in Table 13.

Table 13 – rwp_transform_type[ i ] values

| Value | Description |
| --- | --- |
| 0 | no transform |
| 1 | mirroring horizontally |
| 2 | rotation by 180 degrees (anticlockwise) |
| 3 | rotation by 180 degrees (anticlockwise) before mirroring horizontally |
| 4 | rotation by 90 degrees (anticlockwise) before mirroring horizontally |
| 5 | rotation by 90 degrees (anticlockwise) |
| 6 | rotation by 270 degrees (anticlockwise) before mirroring horizontally |
| 7 | rotation by 270 degrees (anticlockwise) |

rwp_guard_band_flag[ i ] equal to 0 specifies that the i-th packed region does not have a guard band. rwp_guard_band_flag[ i ] equal to 1 specifies that the i-th packed region has a guard band.

proj_region_width[ i ], proj_region_height[ i ], proj_region_top[ i ] and proj_region_left[ i ] specify the width, height, top sample row, and the left-most sample column, respectively, of the i-th projected region, either within the projected picture (when constituent_picture_matching_flag is equal to 0) or within the constituent picture of the projected picture (when constituent_picture_matching_flag is equal to 1).

proj_region_width[ i ], proj_region_height[ i ], proj_region_top[ i ], and proj_region_left[ i ] are indicated in relative projected picture sample units.

NOTE 1 – Two projected regions may partially or entirely overlap with each other.

packed_region_width[ i ], packed_region_height[ i ], packed_region_top[ i ], and packed_region_left[ i ] specify the width, height, the top luma sample row, and the left-most luma sample column, respectively, of the packed region, either within the region-wise packed picture (when constituent_picture_matching_flag is equal to 0) or within each constituent picture of the region-wise packed picture (when constituent_picture_matching_flag is equal to 1).

packed_region_width[ i ], packed_region_height[ i ], packed_region_top[ i ], and packed_region_left[ i ] are indicated in relative region-wise packed picture sample units. packed_region_width[ i ], packed_region_height[ i ], packed_region_top[ i ], and packed_region_left[ i ] shall represent integer horizontal and vertical coordinates of luma sample units within the cropped decoded pictures.

NOTE 2 – Two packed regions may partially or entirely overlap with each other.

rwp_left_guard_band_width[ i ] specifies the width of the guard band on the left side of the i-th packed region in relative region-wise packed picture sample units. When ChromaFormatIdc is equal to 1 (4:2:0 chroma format) or 2 (4:2:2 chroma format), rwp_left_guard_band_width[ i ] shall correspond to an even number of luma samples within the cropped decoded picture.

rwp_right_guard_band_width[ i ] specifies the width of the guard band on the right side of the i-th packed region in relative region-wise packed picture sample units. When ChromaFormatIdc is equal to 1 (4:2:0 chroma format) or 2 (4:2:2 chroma format), rwp_right_guard_band_width[ i ] shall correspond to an even number of luma samples within the cropped decoded picture.

rwp_top_guard_band_height[ i ] specifies the height of the guard band above the i-th packed region in relative region-wise packed picture sample units. When ChromaFormatIdc is equal to 1 (4:2:0 chroma format), rwp_top_guard_band_height[ i ] shall correspond to an even number of luma samples within the cropped decoded picture.

rwp_bottom_guard_band_height[ i ] specifies the height of the guard band below the i-th packed region in relative region-wise packed picture sample units. When ChromaFormatIdc is equal to 1 (4:2:0 chroma format), rwp_bottom_guard_band_height[ i ] shall correspond to an even number of luma samples within the cropped decoded picture.

When rwp_guard_band_flag[ i ] is equal to 1, rwp_left_guard_band_width[ i ], rwp_right_guard_band_width[ i ], rwp_‌top_‌guard_band_height[ i ], or rwp_bottom_guard_band_height[ i ] shall be greater than 0.

The i-th packed region as specified by this SEI message shall not overlap with any other packed region specified by the same SEI message or any guard band specified by the same SEI message.

The guard bands associated with the i-th packed region, if any, as specified by this SEI message shall not overlap with any packed region specified by the same SEI message or any other guard bands specified by the same SEI message.

rwp_guard_band_not_used_for_pred_flag[ i ] equal to 0 specifies that the guard bands may or may not be used in the inter prediction process. rwp_guard_band_not_used_for_pred_flag[ i ] equal to 1 specifies that the sample values of the guard bands are not used in the inter prediction process.

NOTE 3 – When rwp_guard_band_not_used_for_pred_flag[ i ] is equal to 1, the sample values within guard bands in cropped decoded pictures can be rewritten even if the cropped decoded pictures were used as references for inter prediction of subsequent pictures to be decoded. For example, the content of a packed region can be seamlessly expanded to its guard band with decoded and re-projected samples of another packed region.

rwp_guard_band_type[ i ][ j ] indicates the type of the guard bands for the i-th packed region as follows, with j equal to 0, 1, 2, or 3 indicating that the semantics below apply to the left, right, top, or bottom edge, respectively, of the packed region:

–	rwp_guard_band_type[ i ][ j ] equal to 0 indicates that the content of the guard bands in relation to the content of the packed regions is unspecified. When rwp_guard_band_not_used_for_pred_flag[ i ] is equal to 0, rwp_guard_band_type[ i ][ j ] shall not be equal to 0.

–	rwp_guard_band_type[ i ][ j ] equal to 1 indicates that the content of the guard bands suffices for interpolation of sample values at sub-pel sample fractional locations within the packed region and less than one sample outside of the boundary of the packed region.

NOTE 4 – rwp_guard_band_type[ i ][ j ] equal to 1 can be used when the boundary samples of a packed region have been copied horizontally or vertically to the guard band.

–	rwp_guard_band_type[ i ][ j ] equal to 2 indicates that the content of the guard bands represents actual picture content that is spherically adjacent to the content in the packed region and is on the surface of the packed region at quality that gradually changes from the picture quality of the packed region to that of the spherically adjacent packed region.

–	rwp_guard_band_type[ i ][ j ] equal to 3 indicates that the content of the guard bands represents actual picture content that is spherically adjacent to the content in the packed region and is on the surface of the packed region at a similar picture quality as within the packed region.

–	rwp_guard_band_type[ i ][ j ] values greater than 3 are reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value of rwp_guard_band_type[ i ][ j ] when the value is greater than 3 as equivalent to the value 0.

rwp_guard_band_reserved_zero_3bits[ i ] shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for rwp_guard_band_reserved_zero_3bits[ i ] are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of rwp_guard_band_reserved_zero_3bits[ i ].

The variables NumPackedRegions, PackedRegionLeft[ n ], PackedRegionTop[ n ], PackedRegionWidth[ n ], PackedRegionHeight[ n ], ProjRegionLeft[ n ], ProjRegionTop[ n ], ProjRegionWidth[ n ], ProjRegionHeight[ n ], and TransformType[ n ] are derived as follows:

–	For n in the range of 0 to num_packed_regions − 1, inclusive, the following applies:

- PackedRegionLeft[ n ] is set equal to packed_region_left[ n ].

- PackedRegionTop[ n ] is set equal to packed_region_top[ n ].

- PackedRegionWidth[ n ] is set equal to packed_region_width[ n ].

- PackedRegionHeight[ n ] is set equal to packed_region_height[ n ].

- ProjRegionLeft[ n ] is set equal to proj_region_left[ n ].

- ProjRegionTop[ n ] is set equal to proj_region_top[ n ].

- ProjRegionWidth[ n ] is set equal to proj_region_width[ n ].

- ProjRegionHeight[ n ] is set equal to proj_region_height[ n ].

- TransformType[ n ] is set equal to rwp_transform_type[ n ].

–	If constituent_picture_matching_flag is equal to 0, the following applies:

- NumPackedRegions is set equal to num_packed_regions.

–	Otherwise (constituent_picture_matching_flag is equal to 1), the following applies:

- NumPackedRegions is set equal to 2 * num_packed_regions.

- When TopBottomFlag is equal to 1, the following applies:

  - projLeftOffset and packedLeftOffset are both set equal to 0.

  - projTopOffset is set equal to proj_picture_height / 2 and packedTopOffset is set equal to packed_picture_height / 2.

- When SideBySideFlag is equal to 1, the following applies:

  - projLeftOffset is set equal to proj_picture_width / 2 and packedLeftOffset is set equal to packed_picture_width / 2.

  - projTopOffset and packedTopOffset are both set equal to 0.

- For n in the range of NumPackedRegions / 2 to NumPackedRegions − 1, inclusive, the following applies:

  - nIdx is set equal to n − NumPackedRegions / 2.

  - PackedRegionLeft[ n ] is set equal to packed_region_left[ nIdx ] + packedLeftOffset.

  - PackedRegionTop[ n ] is set equal to packed_region_top[ nIdx ] + packedTopOffset.

  - PackedRegionWidth[ n ] is set equal to packed_region_width[ nIdx ].

  - PackedRegionHeight[ n ] is set equal to packed_region_height[ nIdx ].

  - ProjRegionLeft[ n ] is set equal to proj_region_left[ nIdx ] + projLeftOffset.

  - ProjRegionTop[ n ] is set equal to proj_region_top[ nIdx ] + projTopOffset.

  - ProjRegionWidth[ n ] is set equal to proj_region_width[ nIdx ].

  - ProjRegionHeight[ n ] is set equal to proj_region_height[ nIdx ].

  - TransformType[ n ] is set equal to rwp_transform_type[ nIdx ].

For each value of n in the range of 0 to NumPackedRegions − 1, inclusive, the values of ProjRegionWidth[ n ], ProjRegionHeight[ n ], ProjRegionTop[ n ], and ProjRegionLeft[ n ] are constrained as follows:

–	ProjRegionWidth[ n ] shall be in the range of 1 to proj_picture_width, inclusive.

–	ProjRegionHeight[ n ] shall be in the range of 1 to proj_picture_height, inclusive.

–	ProjRegionLeft[ n ] shall be in the range of 0 to proj_picture_width − 1, inclusive.

–	ProjRegionTop[ n ] shall be in the range of 0 to proj_picture_height − 1, inclusive.

–	If ProjRegionTop[ n ] is less than proj_picture_height / VerDiv1, the sum of ProjRegionTop[ n ] and ProjRegionHeight[ n ] shall be less than or equal to proj_picture_height / VerDiv1. Otherwise, the sum of ProjRegionTop[ n ] and ProjRegionHeight[ n ] shall be less than or equal to proj_picture_height / VerDiv1 * 2.

For each value of n in the range of 0 to NumPackedRegions − 1, inclusive, the values of PackedRegionWidth[ n ], PackedRegionHeight[ n ], PackedRegionTop[ n ], and PackedRegionLeft[ n ] are constrained as follows:

–	PackedRegionWidth[ n ] shall be in the range of 1 to packed_picture_width, inclusive.

–	ProjRegionHeight[ n ] shall be in the range of 1 to packed_picture_height, inclusive.

–	PackedRegionLeft[ n ] shall be in the range of 0 to packed_picture_width − 1, inclusive.

–	PackedRegionTop[ n ] shall be in the range of 0 to packed_picture_height − 1, inclusive.

–	If PackedRegionLeft[ n ] is less than packed_picture_width / HorDiv1, the sum of PackedRegionLeft[ n ] and PackedRegionWidth[ n ] shall be less than or equal to packed_picture_width / HorDiv1. Otherwise, the sum of PackedRegionLeft[ n ] and PackedRegionWidth[ n ] shall be less than or equal to packed_picture_width / HorDiv1 * 2.

–	If PackedRegionTop[ n ] is less than packed_picture_height / VerDiv1, the sum of PackedRegionTop[ n ] and PackedRegionHeight[ n ] shall be less than or equal to packed_picture_height / VerDiv1. Otherwise, the sum of PackedRegionTop[ n ] and PackedRegionHeight[ n ] shall be less than or equal to packed_picture_height / VerDiv1 * 2.

–	When ChromaFormatIdc is equal to 1 (4:2:0 chroma format) or 2 (4:2:2 chroma format), PackedRegionLeft[ n ] shall correspond to an even horizontal coordinate value of luma sample units, and PackedRegionWidth[ n ] shall correspond to an even number of luma samples, both within the decoded picture.

–	When ChromaFormatIdc is equal to 1 (4:2:0 chroma format), PackedRegionTop[ n ] shall correspond to an even vertical coordinate value of luma sample units, and ProjRegionHeight[ n ] shall correspond to an even number of luma samples, both within the decoded picture.

### Omnidirectional viewport SEI message

#### Omnidirectional viewport SEI message syntax

| omni_viewport( payloadSize ) { | Descriptor |
| --- | --- |
| omni_viewport_id | u(10) |
| omni_viewport_cancel_flag | u(1) |
| if( !omni_viewport_cancel_flag ) { |  |
| omni_viewport_persistence_flag | u(1) |
| omni_viewport_cnt_minus1 | u(4) |
| for( i = 0; i <= omni_viewport_cnt_minus1; i++ ) { |  |
| omni_viewport_azimuth_centre[ i ] | i(32) |
| omni_viewport_elevation_centre[ i ] | i(32) |
| omni_viewport_tilt_centre[ i ] | i(32) |
| omni_viewport_hor_range[ i ] | u(32) |
| omni_viewport_ver_range[ i ] | u(32) |
| } |  |
| } |  |
| } |  |

#### Omnidirectional viewport SEI message semantics

The omnidirectional viewport SEI message specifies the coordinates of one or more regions of spherical-coordinate geometry, bounded by four great circles, corresponding to viewports recommended for display when the user does not have control of the viewing orientation or has released control of the viewing orientation.

When an effectively applicable frame packing arrangement SEI message, as specified in clause 8.15.2.2 or 8.15.3.2, that applies to the picture is present, the information indicated by the omnidirectional viewport SEI message applies to both views.

omni_viewport_id contains an identifying number that may be used to identify the purpose of the one or more recommended viewport regions.

omni_viewport_id equal to 0 indicates that the recommended viewports are per "director’s cut", i.e., a viewport suggested according to the creative intent of the content author or content provider. omni_viewport_id equal to 1 indicates that the recommended viewports are selected based on measurements of viewing statistics.

Values of omni_viewport_id from 2 to 511, inclusive, may be used as determined by the application. Values of omni_viewport_id from 512 to 1023 are reserved for future use by ITU-T | ISO/IEC. Decoders encountering a value of omni_viewport_id in the range of 512 to 1023, inclusive, shall ignore it.

omni_viewport_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous omnidirectional viewport SEI message in output order. omni_viewport_cancel_flag equal to 0 indicates that omnidirectional viewport information follows.

omni_viewport_persistence_flag specifies the persistence of the omnidirectional viewport SEI message for the current layer.

omni_viewport_persistence_flag equal to 0 specifies that the omnidirectional viewport SEI message applies to the current decoded picture only.

omni_viewport_persistence_flag equal to 1 specifies that the omnidirectional viewport SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with an omnidirectional viewport SEI message is output that follows the current picture in output order.

When an equirectangular projection SEI message with erp_cancel_flag equal to 0 or a generalized cubemap projection SEI message with gcmp_cancel_flag equal to 0 is not present in the CLVS that applies to the current picture and precedes the omnidirectional viewport SEI message in decoding order, an omnidirectional viewport SEI message with omni_viewport_cancel_flag equal to 0 shall not be present in the CLVS that applies to the current picture. Decoders shall ignore omnidirectional viewport SEI messages with omni_viewport_cancel_flag equal to 0 that do not follow, in decoding order, an equirectangular projection SEI message with erp_cancel_flag equal to 0 or a generalized cubemap projection SEI message with gcmp_cancel_flag equal to 0 in the CLVS that applies to the current picture. [Ed. (YK): Consider making the wording in a generic way, such that this sphere rotation SEI message can also be used together with other projection SEI message, e.g., the cubemap projection SEI message specified in HEVC and AVC but not in this Specification.]

omni_viewport_cnt_minus1 plus 1 specifies the number of recommended viewport regions that are indicated by the SEI message.

When omni_viewport_cnt_minus1 is greater than 0 and there is no information provided by external means not specified in this Specification on which recommended viewport is suggested to be displayed, the following applies:

- When omni_viewport_id is equal to 0 or 1, the 0-th recommended viewport is suggested to be displayed when the user does not have control of the viewing orientation or has released control of the viewing orientation.

- When omni_viewport_id is equal to 0, between any two recommended viewports per director's cut, the i-th recommended viewport has higher priority than the j-th recommended viewport for any values of i and j when i is less than j. The 0-th recommended viewport per director's cut has the highest priority.

- When omni_viewport_id is equal to 1, between any two recommended viewports, the i-th recommended viewport has higher popularity, among some selection of candidate viewports, than the j-th recommended viewport for any values of i and j when i is less than j. The 0-th most-viewed recommended viewport has the highest popularity. The selection of the candidate viewports is outside the scope of this Specification.

omni_viewport_azimuth_centre[ i ] and omni_viewport_elevation_centre[ i ] indicate the centre of the i-th recommended viewport region, in units of 2−16 degrees relative to the global coordinate axes. The value of omni_viewport_azimuth_centre[ i ] shall be in the range of −180 * 216 (i.e., −11 796 480) to 180 * 216 − 1 (i.e., 11 796 479), inclusive. The value of omni_viewport_elevation_centre[ i ] shall be in the range of −90 * 216 (i.e., −5 898 240) to 90 * 216 (i.e., 5 898 240), inclusive.

omni_viewport_tilt_centre[ i ] indicates the tilt angle of the i-th recommended viewport region, in units of 2−16 degrees. The value of omni_viewport_tilt_centre[ i ] shall be in the range of −180 * 216 (i.e., −11 796 480) to  216 − 1 (i.e., 11 796 479), inclusive.

omni_viewport_hor_range[ i ] indicates the azimuth range of the i-th recommended viewport region, in units of 2−16 degrees. The value of omni_viewport_hor_range[ i ] shall be in the range of 1 to 360 * 216 (i.e., 23 592 960), inclusive.

omni_viewport_ver_range[ i ] indicates the elevation range of the i-th recommended viewport region, in units of 2−16 degrees. The value of omni_viewport_ver_range[ i ] shall be in the range of 1 to 180 * 216 (i.e., 11 796 480), inclusive.

## Frame-field information SEI message

### Frame-field information SEI message syntax

| frame_field_info( payloadSize ) { | Descriptor |
| --- | --- |
| field_pic_flag | u(1) |
| if( field_pic_flag ) { |  |
| bottom_field_flag | u(1) |
| pairing_indicated_flag | u(1) |
| if( pairing_indicated_flag ) |  |
| paired_with_next_field_flag | u(1) |
| } else { |  |
| display_fields_from_frame_flag | u(1) |
| if( display_fields_from_frame_flag ) |  |
| top_field_first_flag | u(1) |
| display_elemental_periods_minus1 | u(4) |
| } |  |
| source_scan_type | u(2) |
| duplicate_flag | u(1) |
| } |  |

### Frame-field information SEI message semantics

The frame-field information SEI message may be used to indicate how the associated picture should be displayed (although this is merely a suggestion rather than a prescription, as the display process is outside the scope of this document), the source scan type of the associated picture, and whether the associated picture is a duplicate of a previous picture, in output order, of the same layer.

Use of this SEI message requires the definition of the following variables:

–	A fixed picture rate indicator associated with a temporal sublayer, denoted herein by FixedPicRateWithinCvsFlag, such that value 1 indicates that the temporal distance between the display times of consecutive pictures in output order is constrained and value 0 indicates no such constraint.

–	A display elemental period indicator, denoted herein by DisplayElementalPeriods, that indicates the number of elemental picture period intervals that the current coded picture occupies for the display model.

field_pic_flag equal to 1 indicates that the display model considers the current picture as a field, and field_pic_flag equal to 0 indicates that the display model considers the current picture as a frame.

bottom_field_flag equal to 1, when present, indicates that the current picture is a bottom field (i.e., that the parity of the current picture is bottom). bottom_field_flag equal to 0 indicates that the current picture is a top field (i.e., that the parity of the current picture is top). The two parities, bottom and top, are considered as opposite parities.

pairing_indicated_flag equal to 1, when present, indicates that the current picture is considered paired with the next picture in output order or with the previous picture in output order as the two fields of a frame. pairing_indicated_flag equal to 0, when present, indicates that a pairing of the current picture with another picture to form a frame is not expressed.

paired_with_next_field_flag equal to 1, when present, indicates that the current picture is considered paired with the next picture as the two fields of a frame. paired_with_next_field_flag equal to 0, when present, indicates that the current picture is considered paired with the previous picture as the two fields of a frame.

When paired_with_next_field_flag is present, the following constraints shall apply

–	If paired_with_next_field_flag is equal to 0, there shall be at least one picture in the CVS that precedes the current picture in output order and the picture that precedes the current picture in output order shall have the opposite parity and pairing_indicated_flag equal to 1 and the value of paired_with_next_field_flag for that preceding picture in output order shall be equal to 1.

–	Otherwise, there shall be at least one picture in the CVS that follows the current picture in output order and the picture that follows the current picture in output order shall have the opposite parity and pairing_indicated_flag equal to 1 and the value of paired_with_next_field_flag for that following picture in output order shall be equal to 0.

display_fields_from_frame_flag equal to 1, when present, indicates that the display model operates by sequentially displaying the individual fields of the frame with alternating parity. display_fields_from_frame_flag equal to 0, when present, indicates that the display model operates by displaying the current picture as a complete frame.

top_field_first_flag equal to 1, when present, indicates that the first field of the frame that is displayed by the display model is the top field. top_field_first_flag equal to 0, when present, indicates that the first field of the frame that is displayed by the display model is the bottom field.

display_elemental_periods_minus1 plus 1, when present and FixedPicRateWithinCvsFlag is equal to 1, indicates the number of elemental picture period intervals that the current coded picture occupies for the display model. The value of display_elemental_periods_minus1 shall be equal to DisplayElementalPeriods − 1 and is constrained as follows:

–	If display_fields_from_frame_flag is equal to 1, display_elemental_periods_minus1 shall be equal to 1 or 2.

–	Otherwise, when FixedPicRateWithinCvsFlag is equal to 0, display_elemental_periods_minus1 shall be equal to 0.

The interpretation of combinations of field_pic_flag, FixedPicRateWithinCvsFlag, bottom_field_flag, display_fields_from_frame_flag, top_field_first_flag, and display_elemental_periods_minus1 (through DisplayElementalPeriods) is specified in Table 14. , in which syntax elements that are not present are indicated by "–". Combinations of syntax elements that are not listed in Table 14 are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification.

NOTE 1 – When FixedPicRateWithinCvsFlag is equal to 1, the indicated display times are constrained to account for time duration for a display model that follows the display patterns indicated by the values of the syntax elements of the frame-field information SEI message (although the display process is outside the scope of this Specification). Although the video decoder model might be specified to only output the entire cropped decoded picture, the modelled display behaviour sometimes includes other steps, such as the repeated display of a frame for multiple time intervals when display_fields_from_frame_flag is equal to 0 or the sequential display of the individual fields of a frame when display_fields_from_frame_flag is equal to 1.

NOTE 2 – Frame doubling can be used to facilitate the display, for example, of 25 Hz progressive-scan video on a 50 Hz progressive-scan display or 30 Hz progressive-scan video on a 60 Hz progressive-scan display. Using frame doubling and frame tripling in alternating combination on every other frame can be used to facilitate the display of 24 Hz progressive-scan video on a 60 Hz progressive-scan display.

Table 14 – Interpretation of frame-field information syntax elements

| field_pic_flag | FixedPicRateWithinCvsFlag | bottom_field_flag | display_fields_from_frame_flag | top_field_first_flag | DisplayElementalPeriods |  | Indicated display of the picture by the display model |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | – | 0 | – | 1 |  | (progressive) Frame |
|  |  | – | 1 | 0 | 2 |  | Bottom field, top field, in that order |
|  |  | – | 1 | 1 | 2 |  | Top field, bottom field, in that order |
|  |  | – | 1 | 0 | 3 |  | Bottom field, top field, bottom field repeated, in that order |
|  |  | – | 1 | 1 | 3 |  | Top field, bottom field, top field repeated, in that order |
|  | 1 | – | 0 | – | n |  | (progressive) Frame displayed for n elemental periods of time |
|  |  | – | 1 | 0 | 2 |  | Bottom field, top field, in that order, each displayed for 1 elemental period of time |
|  |  | – | 1 | 1 | 2 |  | Top field, bottom field, in that order, each displayed for 1 elemental period of time |
|  |  | – | 1 | 0 | 3 |  | Bottom field, top field, bottom field repeated, in that order, each displayed for 1 elemental period of time |
|  |  | – | 1 | 1 | 3 |  | Top field, bottom field, top field repeated, in that order, each displayed for 1 elemental period of time |
| 1 | 0 | 0 | – | – | 1 |  | Top field |
|  |  | 1 | – | – | 1 |  | Bottom field |
|  | 1 | 0 | – | – | 1 |  | Top field displayed for 1 elemental period of time |
|  |  | 1 | – | – | – |  | Bottom field displayed for 1 elemental period of time |

source_scan_type equal to 1 indicates that the source scan type of the associated picture should be interpreted as progressive. source_scan_type equal to 0 indicates that the source scan type of the associated picture should be interpreted as interlaced. source_scan_type equal to 2 indicates that the source scan type of the associated picture is unknown or unspecified. source_scan_type equal to 3 is reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall interpret the value 3 for source_scan_type as equivalent to the value 2.

duplicate_flag equal to 1 indicates that the current picture is indicated to be a duplicate of a previous picture in output order. duplicate_flag equal to 0 indicates that the current picture is not indicated to be a duplicate of a previous picture in output order.

NOTE 3 – The duplicate_flag should be used to mark coded pictures known to have originated from a repetition process such as 3:2 pull-down or other such duplication and picture rate interpolation methods. This flag would commonly be used when a video feed is encoded as a field sequence in a "transport pass-through" fashion, with known duplicate pictures tagged by setting duplicate_flag equal to 1.

NOTE 4 – When field_pic_flag is equal to 1 and duplicate_flag is equal to 1, this should be interpreted as an indication that the AU contains a field that duplicates the content of the previous field in output order with the same parity as the current field.

## Sample aspect ratio information SEI message

### Sample aspect ratio information SEI message syntax

| sample_aspect_ratio_info( payloadSize ) { | Descriptor |
| --- | --- |
| sari_cancel_flag | u(1) |
| if( !sari_cancel_flag ) { |  |
| sari_persistence_flag | u(1) |
| sari_aspect_ratio_idc | u(8) |
| if( sari_aspect_ratio_idc = = 255 ) { |  |
| sari_sar_width | u(16) |
| sari_sar_height | u(16) |
| } |  |
| } |  |
| } |  |

### Sample aspect ratio information SEI message semantics

The sample aspect ratio information SEI message provides information about the sample aspect ratio of the samples of the associated decoded pictures. When vui_aspect_ratio_constant_flag is equal to 1, there shall be no sample aspect ratio information SEI messages present in the CLVS.

sari_cancel_flag equal to 1 indicates that the sample aspect ratio information SEI message cancels the persistence of any previous sample aspect ratio information SEI messages in output order that applies to the current layer. sari_cancel_flag equal to 0 indicates that sample aspect ratio information follows.

sari_persistence_flag specifies the persistence of the sample aspect ratio information SEI message for the current layer.

sari_persistence_flag equal to 0 specifies that the sample aspect ratio information applies to the current decoded picture only.

sar_persistence_flag equal to 1 specifies that the sample aspect ratio information SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with a sample aspect ratio information SEI message is output that follows the current picture in output order.

sari_aspect_ratio_idc, when not equal to 255, indicates the sample aspect ratio of the luma samples of the decoded output picture, with the same semantics as specified for the SampleAspectRatio parameter in Rec. ITU-T H.273 | ISO/IEC 23091-2. When the sari_aspect_ratio_idc syntax element is not present, the value of sari_aspect_ratio_idc is inferred to be equal to 0. Values of sari_aspect_ratio_idc that are specified as reserved for future use in Rec. ITU-T H.273 | ISO/IEC 23091-2 shall not be present in bitstreams conforming to this version of this Specification. Decoders shall interpret values of sari_aspect_ratio_idc that are reserved for future use in Rec. ITU-T H.273 | ISO/IEC 23091-2 as equivalent to the value 0.

sari_sar_width, when present, indicates the horizontal size of the sample aspect ratio (in an arbitrary unit).

sari_sar_height, when present, indicates the vertical size of the sample aspect ratio (in the same arbitrary unit as sari_sar_width).

When present, sari_sar_width and sari_sar_height shall be relatively prime or equal to 0. When sari_aspect_ratio_idc is equal to 0 or sari_sar_width is equal to 0 or sari_sar_height is equal to 0, the sample aspect ratio is unspecified in this Specification.

When vui_aspect_ratio_constant_flag in the VUI parameters is present and equal to 1, it is a requirement of bitstream conformance that all sample aspect ratio SEI messages in the CLVS shall have sari_aspect_ratio_idc equal to vui_aspect_ratio_idc and, when present, sari_sar_width equal to vui_sar_width and sari_sar_height equal to vui_sar_height.

## Reserved SEI message

### Reserved SEI message syntax

| reserved_message( payloadSize ) { | Descriptor |
| --- | --- |
| for( i = 0; i < payloadSize; i++ ) |  |
| reserved_message_payload_byte | u(8) |
| } |  |

### Reserved SEI message semantics

The reserved SEI message consists of data reserved for future backward-compatible use by ITU-T | ISO/IEC. Unless otherwise specified by a referencing specification, coded video bitstreams shall not contain reserved SEI messages and systems that make use of such coded video bitstreams shall not otherwise send reserved SEI messages until and unless the use of such messages has been specified by ITU-T | ISO/IEC. Decoders shall ignore reserved SEI messages.

reserved_message_payload_byte, when present, has values to be specified in the future by ITU-T | ISO/IEC.

# Parsing process for k-th order Exp-Golomb codes

## General

This process is invoked when the descriptor of a syntax element in the syntax tables is equal to ue(v) or se(v).

Inputs to this process are bits from the RBSP.

Outputs of this process are syntax element values.

Syntax elements coded as ue(v) or se(v) are Exp-Golomb-coded with order k equal to 0. The parsing process for these syntax elements begins with reading the bits starting at the current location in the bitstream up to and including the first non-zero bit, and counting the number of leading bits that are equal to 0. This process is specified as follows:

leadingZeroBits = −1
for( b = 0; !b; leadingZeroBits++ )		(53)

b = read_bits( 1 )

The variable codeNum is then assigned as follows:

codeNum = ( 2leadingZeroBits − 1 ) * 2k + read_bits( leadingZeroBits + k )	(54)

where the value returned from read_bits( leadingZeroBits ) is interpreted as a binary representation of an unsigned integer with most significant bit written first.

Table 15 illustrates the structure of the 0-th order Exp-Golomb code by separating the bit string into "prefix" and "suffix" bits. The "prefix" bits are those bits that are parsed as specified above for the computation of leadingZeroBits, and are shown as either 0 or 1 in the bit string column of Table 15. The "suffix" bits are those bits that are parsed in the computation of codeNum and are shown as xi in Table 15, with i in the range of 0 to leadingZeroBits − 1, inclusive. Each xi is equal to either 0 or 1.

Table 15 – Bit strings with "prefix" and "suffix" bits and assignment to codeNum ranges (informative)

| Bit string form | Range of codeNum |
| --- | --- |
| 1 | 0 |
| 0 1 x0 | 1..2 |
| 0 0 1 x1 x0 | 3..6 |
| 0 0 0 1 x2 x1 x0 | 7..14 |
| 0 0 0 0 1 x3 x2 x1 x0 | 15..30 |
| 0 0 0 0 0 1 x4 x3 x2 x1 x0 | 31..62 |
| ... | ... |

Table 16 illustrates explicitly the assignment of bit strings to codeNum values.

Table 16 – Exp-Golomb bit strings and codeNum in explicit form and used as ue(v) (informative)

| Bit string | codeNum |
| --- | --- |
| 1 | 0 |
| 0 1 0 | 1 |
| 0 1 1 | 2 |
| 0 0 1 0 0 | 3 |
| 0 0 1 0 1 | 4 |
| 0 0 1 1 0 | 5 |
| 0 0 1 1 1 | 6 |
| 0 0 0 1 0 0 0 | 7 |
| 0 0 0 1 0 0 1 | 8 |
| 0 0 0 1 0 1 0 | 9 |
| ... | ... |

Depending on the descriptor, the value of a syntax element is derived as follows:

- If the syntax element is coded as ue(v), the value of the syntax element is equal to codeNum.

- Otherwise (the syntax element is coded as se(v)), the value of the syntax element is derived by invoking the mapping process for signed Exp-Golomb codes as specified in clause 9.2 with codeNum as input.

## Mapping process for signed Exp-Golomb codes

Input to this process is codeNum as specified in clause 9.1.

Output of this process is a value of a syntax element coded as se(v).

The syntax element is assigned to the codeNum by ordering the syntax element by its absolute value in increasing order and representing the positive value for a given absolute value with the lower codeNum. Table 17 provides the assignment rule.

Table 17 – Assignment of syntax element to codeNum for signed Exp-Golomb coded syntax elements se(v)

| codeNum | syntax element value |
| --- | --- |
| 0 | 0 |
| 1 | 1 |
| 2 | −1 |
| 3 | 2 |
| 4 | −2 |
| 5 | 3 |
| 6 | −3 |
| k | (−1)k + 1 Ceil( k ÷ 2 ) |

# Bibliography

- Recommendation ITU-T H.264 (in force), Advanced video coding for generic audiovisual services.

ISO/IEC 144496-10 (in force), Information technology – Coding of audio-visual objects – Part 10: Advanced video coding

- Recommendation ITU-T H.265 (in force), High efficiency video coding.

ISO/IEC 23008-2 (in force), Information technology –High efficiency coding and media delivery in heterogeneous environments — Part 2: High efficiency video coding.

- Recommendation ITU-T H.VVC (in force), Versatile video coding.

ISO/IEC 23090-3 (in force), Information technology – Coded representation of immersive media – Part 3: Versatile video coding.

- Recommendation ITU-T H.271 (in force), Video back-channel messages for conveyance of status information and requests from a video receiver to a video sender.

- Recommendation ITU-R BT.1886 (in force), Reference electro-optical transfer function for flat panel displays used in HDTV studio production.

- Recommendation ITU-R BT.2020 (in force), Parameter values for ultra-high definition television systems for production and international programme exchange.

- Recommendation ITU-R BT.2035 (in force), A reference viewing environment for evaluation of HDTV program material or completed programmes.

- Recommendation ITU-R BT.2100 (in force), Image parameter values for high dynamic range television for use in production and international programme exchange.

- ANSI/CTA 861-G (2016), A DTV Profile for Uncompressed High Speed Digital Interfaces.

- CIE 15 (in force), Colorimetry.

- ISO 11664-3 (in force), Colorimetry – Part 3: CIE tristimulus values.

- SMPTE RDD 5 (2006), Film Grain Technology – Specifications for H.264/MPEG-4 AVC Bitstreams.

- SMPTE ST 2086 (2018), Mastering Display Color Volume Metadata supporting High Luminance and Wide Color Gamut Images.

[Ed. (GJS): Add bibliography citations for SMPTE RDD 5, SMPTE 2086, ANSI/CTA 861-G.]

